PASSCOM.DOC

Program  : PASSCOM.EXE

Author   : Kevin Kiley

Company  : INTERLINK TECHNOLOGIES

Address  : 1222 5th Street

Sarasota, Fla. 34236

(813) 366-3728

Released : 04/25/90

Terms    : Shareware. A $10 registration fee is expected if you

use the program on a regular basis. Registration entitles

you to phone support and free updates. Send check or

money order to INTERLINK.

Source   : Written completely in TURBO 'C'. No ASM code at all.

Fully commented source code is available from INTERLINK

TECHNOLOGIES.

WHAT IS PASSCOM?

PASSCOM is a PC DOS communications utility.

The program resides in memory and will pass ALL characters received or

transmitted on either COM1 or COM2 directly to the OPPOSITE COM PORT.

I have 4 PC's. Only ONE of them has an INTERNAL MODEM and I use this

program to SHARE that internal modem with ALL of the other PC's.

I never have to decide WHICH PC should have the modem and go through

the hassle of re-installing the modem card. The modem resides

permanently on COM2 in ONE PC and I use THIS PROGRAM and an inexpensive

ABC switch box connected to COM1 to use the modem from any one of the

other 3 PC's.

The program is a VERY SMALL Terminate and Stay Resident program (TSR)

which can be installed or removed from memory at any time from either

the DOS command line or from within a DOS batch file.

Since the program is a TSR you can still do anything you want in the

foreground on the PC with the modem in it while you are using the

modem from another PC.

This program can be used to SHARE ANY SERIAL device between a number

of PC's but the MODEM SHARING is perhaps the best example.

WHAT YOU NEED TO USE PASSCOM

IBMPC or compatible, DOS 3.1 or better, 2 com ports ( COM1 and COM2 ).

STARTING PASSCOM...

PASSCOM uses a syntax similar to the DOS MODE command. You never

need to supply the 'COM1:' or 'COM2:' parameter as you would with a MODE

command since PASSCOM already knows it will be dealing with BOTH COM PORTS.

You can, however, specify the PROTOCOL to be used for a session by supplying

the right parameters on the command line. Separate parameters with SPACES,

not COMMAS!...

PASSCOM             <-- Default protocol 2400 BAUD, NO   PARITY, 8 DATA, 1 STOP

PASSCOM 1200 N 8 1  <-- Use     protocol 1200 BAUD, NO   PARITY, 8 DATA, 1 STOP

PASSCOM 1200 E 7 1  <-- Use     protocol 1200 BAUD, EVEN PARITY, 7 DATA, 1 STOP

Etc...      | | |

| | |

USE SPACES

TO SEPARATE

PARAMETERS!

PASSCOM will complain if it doesn't like a parameter and will display an

error message and a reminder on how to the protocol should be specified.

If all is OK then PASSCOM will go memory resident and give you a message

telling you that the load was successful.

NOTE: PASSCOM will use the SAME BAUD RATE for both ports! If you need

to share a port and ( for some reason ) need to use different

BAUD rates then contact INTERLINK TECHNOLOGIES. I have a program

that uses BUFFERS and is much more complicated than PASSCOM but it

will UPSHIFT or DOWNSHIFT and allow 2 devices to communicate using

different BAUD RATES.

USING PASSCOM...

As long as PASSCOM is resident it will re-direct all COM I/O on COM1 and

COM2 to the opposite port. If you are using it to share an internal modem

then you can go to the OTHER PC and use ANY communications software like

PROCOMM or QMODEM to place phone calls and/or UPLOAD/DOWNLOAD files.

PASSCOM re-directs the communciations interrupt vectors for BOTH COM

PORTS to get the job done. You have to be careful about what you RUN

on the PC where PASSCOM is running because of this. Do NOT run any

other software that will also re-direct the communications interrupts

or PASSCOM may stop working. This includes PROCOMM, TELINK, etc.

STOPPING PASSCOM...

Just enter 'PASSCOM T' to 'TERMINATE' the program. If no other TSR's

were loaded after PASSCOM then the program will automatically remove

itself from memory. The communications interrupt vectors will be

restored to their values at the time PASSCOM was loaded and the ports

will be released.

If another TSR has been loaded on top of PASSCOM then all it can do

is SUSPEND. It will release the communciations interrupt vectors but

will NOT remove itself from memory. Use a standard TSR FLUSH setup to

remove PASSCOM if you're are forever loading other TSR's on top of it.

This will be the only way to actually REMOVE it from memory ( other than

a quck re-boot ) if there are other TSR's on top of it. This is true

for all TSR's that are asked to remove themselves from memory under such

circumstances.

NULL MODEM CABLE REQUIRED FOR ANY DEVICE CONNECTED TO HOST PC

Any device connected externally to the COM PORTS must use a NULL modem

for things to work as desired.

In my case where I'm using PASSCOM to share 1 INTERNAL MODEM on COM2...

The OTHER PC I have connected to COM1 uses a standard NULL MODEM cable

with pins 2 and 3 reversed. If you're confused about this just ask

your dealer for an RS232 NULL MODEM cable or go to RADIO SHACK and buy

one of their 'NULL MODEM ADAPTERS' and attach it to your 'STRAIGHT THRU'

serial cable at the point where it enters the HOST PC.

If the device you're SHARING happens to be external as well ( external

modem? ) then you may or may NOT need a NULL MODEM. You don't have

to do anything different for PASSCOM. If you can use it already on

COM1 or COM2 without PASSCOM then everything should be fine.

LIMITATION: HARDWARE SIGNALS ARE NOT PASSED!

Since PASSCOM is a bit of fakery there are a few things to keep in mind

about how it works with regard to HARDWARE SIGNALS. PASSCOM will re-direct

all TRANSMIT and RECEIVE DATA that arrives on pins 2 and 3 but it will

NOT PASS HARDWARE SIGNALS! This has never caused a problem for me since

I simply use it to share an INTERNAL SMARTMODEM but there's no telling

what someone might need it for so I felt obliged to mention this limitation.

If you run a piece of software on a PC that does NOT have the actual

device installed in it and that software is depending on the status of

certain RS232 pins to be HIGH or LOW then be aware that the pins on the

port you are communciating with will NOT go HIGH and LOW exactly the

way the pins would if the device were ACTUALLY connected to it.

DTR will always go HIGH and LOW on all ports since this is controllable

from PASSCOM but other signals like CD, RTS or CTS may be lost.

Confusing? Here's an example...

Let's say you have a TEMPEATURE SENSING DEVICE installed in 1 PC

on COM2 and you are trying to use that device from another PC by

attaching it to COM1 and running PASSCOM on the PC with the SENSING

DEVICE installed.

Let us also say that the TEMPERATURE SENSING SOFTWARE that came with

the device has been hardcoded to wait for the CD (CARRIER DETECT) pin

to go HIGH (ON) before it will run its main menu. This would be pretty

silly but you never know what's out there...

When you run the TEMPEATURE SENSING SOFTWARE on the OTHER PC attached to

COM1 then this software may wait forever since it's looking at a pin

on a COM PORT that is NOT actually attached to the device itself.

CD may be HIGH over on the PC with the actual device ( running PASSCOM )

but the state of the CD pin is NOT PASSED ALONG to the other RS232

connector. Only DTR and DATA are passed on a regular basis.

Don't despair! It would be unusual if problems like these could not

be solved with the right cable. Example: In the case above... STRAPPING

pin DTR to pin CD on the PC running the TEMPERATURE SENSING SOFTWARE would

succeed in fooling it and the MENU would appear normally. STRAPPING

pins to fool software is performed at even the highest levels of DP and

it is not difficult to do.

Consult your dealer or call me if you have an unusual piece of hardware

or software that you would like to use with PASSCOM.

Most hardware signals are controllable from software and if worst comes

to worst then either you or I can alter the source code to provide

the special signals that your device is looking for ( if DOS allows access

to the pin on the UART chip! ).

OTHER PUBLIC DOMAIN PROGRAMS BY INTERLINK TECHNOLOGIES...

1. ELSEWARE v2.2 - FREEWARE - TSR REMOTE ACCESS SOFTWARE

COMPUSERVE listing  : ELSEWA.ZIP

COMPUSERVE forum    : IBM COMMUNICATIONS ( GO IBMCOM )

COMPUSERVE library  : Communications programs

ELSEWARE is a FREEWARE REMOTE ACCESS PROGRAM that allows you to use

a remote PC as if you were sitting in front of it. It works the same

(if not better) than most of the commercial software you can buy but

it requires only 8K for the HOST TSR EXE file on the HOST PC. Some video

buffers are created dynamically and the total memory footprint is

no more than 16-17K. No commercial remote access software even comes

close to this small footprint.

The REMOTE session is a full TSR POP-UP session which will NOT TAKE OVER

the REMOTE PC used to call the HOST as most commercial software will.

Password security access is included.

Easy to use SETUP SCREENS with full HELP.

Full XMODEM file transfer and AUTOMATIC FILE TRANSFER are available

for ELSEWARE as well as a unique HOST MODEM SHARING option not offered

by any commercial software.

Many CUSTOMS versions of ELSEWARE have been written to solve specific

problems out in the field. These special versions can be supplied

QUICKLY by contacting INTERLINK TECHNOLOGIES at (813) 366-3728.

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 2,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

1-800-2424-PSL

MC/Visa/AmEx

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.