ELOGANAL

Copyright (c) 1990 H L Hendrick, all rights reserved.

ELOGANAL is a shareware microcomputer program  which  allows

the  user  to  perform  the  calculations  involved  in analyzing

electric logs run on prospective oil and gas wells.   The program

was written in the "C" language for reduced code size,  speed and

mathematical  accuracy.

If this program  is  useful  to  you,  please  register  the

program  with  the  author  by sending a check for $100.00 to H L

Hendrick P.O. Box 79530 Houston TX 77279. Program support and up-

date information will be furnished to registered users.

This package contains computer software and printed material

copyrighted by H L Hendrick.  You should carefully read the  fol-

lowing  terms  and conditions before using the software.   Use of

the software indicates your acceptance of the  software  and  all

terms and conditions.

This software product is licensed to one user for use on one

machine only.

Under  international  copyright laws there are penalties for

making unauthorized copies.   You may make and distribute  copies

of the program to others for their consideration.   Any copy that

you make must include reproduction of the  copyright  notice  and

warranty disclaimer.  You may not sell any copy that you make.

H  L  Hendrick  makes no express or implied warranty for the

software which is provided solely on an  "as  is"  basis.    H  L

Hendrick disclaims all conditions and warranties, whether express

or  implied  with  regard to the software,  including all implied

conditions or warranties of fitness for a particular purpose.   H

L  Hendrick shall not have any liability or responsibility of any

kind including special, indirect or consequential damages,  aris-

ing  out  of  or  resulting  from  such  software  or  the use or

modification thereof.

H L Hendrick is a member of  the  Association  of  Shareware

Professionals  (ASP).  ASP  wants to make sure that the shareware

principle  works  for  you.  If  you  are  unable  to  resolve  a

shareware-related  problem  with  an ASP member by contacting the

member directly,  ASP may be able to help.  The ASP Ombudsman can

help,  but  does  not  provide  technical  support  for  members'

products.   Please write to the ASP Ombudsman at  P.O.  Box  5786

Bellevue,  WA  98006 or send a CompuServe message via easyplex to

ASP Ombudsman 70007,3536

1

The design goals of the program were  to  create  a  program

which would be of use to the log analyst in working old type logs

as well as the new more modern type logs.  Empirical routines are

included  to  estimate the oil or gas content of the flushed zone

with an option to change it.  Porosity estimates  using  the  old

short normal technique are included with an option to re-enter if

the value is not deemed in an acceptable range.  Rw can be calcu-

lated from the SP curve or from an API water analysis as well  as

the  Rwa  method  or  by direct input of ppm of Cl or NaCl.   The

program uses standard oil field analysis methods, formula and al-

gorithms.   The statistical  and  empirical  routines  used  were

recommended  in  publications  by  Douglas Hilchie and Jim Brock.

These publications are available from the  Society  of  Petroleum

Engineers.    Chart books from Schlumberger,  Dresser and GO were

also used in designing and testing the program.

Estimates of critical water can be made from a list of  for-

mation  options or by entering core permeability.   Relative per-

meability is calculated for all reservoir  fluids  regardless  of

their presence.  Water cut is estimated using Reservoir Engineer-

ing  techniques  utilizing  relative permeability.   The critical

water is estimated from statistical data for the type  of  forma-

tion  selected  and  may  not  be correlative with the calculated

water cut.

The author of the program assumes the user is aware  of  the

uses  and  limitations  of electric logs run on drilled wells and

does not intend to present a tutorial in log analysis.  Petroleum

oriented literature should be reviewed for instructions regarding

the correct application of logging techniques.  Standard  logging

symbols are used in the screen presentation and printout.

HARDWARE REQUIREMENTS

Minimum requirements are a 128K,  one disk drive, 8086/8088,

MS or PC DOS 2.xx or greater microcomputer or compatible.

If a co-processor is present it will be  used.    If  a  co-

processor  is  not  present,  the  program  will  emulate the co-

processor using an internal 7K emulator.

In order to use the program,  format a new disk and  install

your operating system. Copy ELOGANAL.EXE to the new disk. Use the

new program disk you created and boot up.  Enter ELOGANAL and run

the program. If you prefer, Copy the program to your hard disk.

Follow the menu prompts.  Screen displays will be  presented

and remain on screen until you enter a requested key.

2

The program can create and maintain a data file for each log

analysis.  Provisions  to display to screen or print the file are

included.   The file name selected should relate to the  name  of

the  well or operator.  These files are stored in an ASCll format

and can be incorporated in a report prepared with a word  proces-

sor.

After the initial run through the program,  the UP ARROW and

DOWN ARROW keys will toggle the Main (underlined heading)  menus.

A new depth can be input by entering a RIGHT ARROW or LEFT ARROW.

When  a  Main  (underlined  heading)  menu  is  displayed,  a re-

calculation can be made by entering <RETURN>. If the shale volume

or the type of shale changes,  a new porosity will be  calculated

and  corrections  made  based  on the shale changes.  To exit the

program from a Main menu (underlined heading) enter END key.

Data entry begins  with  input  from  the  log  heading  and

proceeds  in  a logical manner to set flags and constants used in

the calculations for the specific log being analyzed.

Entered log heading data will be displayed to screen with an

option to change a value.   Most pre-1958 logs did not record the

Rmf and Rmc.   If this is the case, you will be prompted to enter

a zero.   The program will estimate both from Rm and the tempera-

ture will be set to the temperature of the mud.

TYPE OF MUD

1-Light weight fresh water muds sets density

of fluid to 1.0 and travel time to 189.

2-Heavy weight fresh water muds sets density

of fluid to 1.01 and travel time to 189.

3-Salt water base mud sets density of fluid to

1.1 and travel time to 185.

The  log matrix lithology flag must be set for all log types

requested even though a Density,  Neutron  or Sonic  log  is  not

available.  If one or all of these logs is not available, setting

these flags  will not affect the program.

LOG MATRIX LITHOLOGY

~~~~~~~~~~~~~~~~~~~~

1-Sand, sets matrix flags for density log to

2.65, Sonic log to 55.5 and neutron response

to a correction algorithm.

2-Lime, sets matrix flags for density log to

2.71, Sonic log to 47.5

3-Dolomite, sets matrix flags for density log to

3

2.87, Sonic log to 43.5 and neutron response

to a correction algorithm.

4-User input allows the user to input the desired

factor.

FORMATION OPTIONS

~~~~~~~~~~~~~~~~~

1- Soft Formations, sets MXP to 2.15,COER

to .62 and SXP to 2. (Humble).

2-Hard formations, sets MXP to 2, COER to 1

and SXP to 2. (Archie).

3-Low porosity carbonates, uses the Shell

developed method and sets SXP to 2, COER

to 1 with MXP being set based on porosity.

4-User input allows the user to input the

desired factors.

Rw OPTIONS

~~~~~~~~~~

1- Input Rw, allows direct input of known

water resistivity and temperature.

2-Compute from salinity, allows computation

of water resistivity from known ppm of NACL or CL.

3-Compute from Rwa, uses the method recommended

by the major logging companies for computation

of Rw from log data.

4-Compute from SSP, uses the method recommended

by the major logging companies for computation

of Rw from the SP curve.

5-Compute from analysis, allows computation

of Rw from a known API water analysis.

Factors used in the computation are set

for a water with a total solids content of

70000 ppm NACL.  The user has the option

to change the factors to the correct values

for the water being used.  These factors are

available in all logging company chart books.

SHALE CONTENT OPTIONS

~~~~~~~~~~~~~~~~~~~~~

4

Seven options are available for calculating VOLSH. Selection

of  any option other than no correction,  requires a selection of

the type of shale.

1-No corrections

Sets VOLSH to zero.

2-Direct input

VOLSH must be entered in a decimal fraction.

3-Gamma or SP index

Calculates VOLSH from the gamma or SP.

4-Tertiary age rocks

Calculates VOLSH using gamma index as proposed

by Larinov.

5-Older age rocks

Calculates VOLSH using gamma index as proposed

by Larinov.

6-Density-Neutron

7-Density-Sonic

8-Sonic-Neutron

Calculates VOLSH using a ratio of porosity

technique.

Five options are  available  for  calculating  porosity.  If

x-plot  is selected,  four combinations of x-plots are available.

If direct input is selected,  the porosity input will not be cor-

rected for fluid or shale content;  however,  the correction flag

must be set in order to output the water cut in  the  final  dis-

play.

POROSITY OPTIONS

~~~~~~~~~~~~~~~~

1-Direct input

2-Sonic

3-Density

4-X-plots

5-Short Normal

If the x-plots option is selected,  the x-plot menu will ap-

pear as follows:

1-CDL & CNL

2-Sonic & CNL

5

3-CDL & SNP

4-Sonic & SNP

The Short Normal option should be used only  when  no  other

option  is  available.   It is an empirical estimate and only ap-

plies where specific conditions of invasion exist.

ENTER SOURCE OF RESISTIVITY

~~~~~~~~~~~~~~~~~~~~~~~~~~~

1-Direct input, must be entered in decimal

fraction.

2-Induction Electrical Log, This option requires

input of the Induction and 16 inch normal. An

empirical correction is made as suggested by

Hilchie in his book "Old Electrical Log

Interpretation".

3-Dual Induction logs with SFL, this option requires

input of deep, medium and shallow resistivity.

A "Tornado Chart" correction is made mathematically.

4-Dual Induction logs with laterolog, this option

requires input of deep, medium and shallow

resistivity.  A "Tornado Chart" correction is made

mathematically.

5-Dual Laterolog (20' Sequential), correction made

on tool type.

6-Dual Laterolog (28' Simultaneous), correction made

on tool type.

A 16 inch normal will corrected for hole size.  If a

caliper survey is not available, the correction will

be made assuming a gauge hole.

SELECTION FOR CRITICAL WATER

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Option selection for several formations on  which  empirical

data is available can be made.  If the logged formation is not on

the  list  of  options  or  the  formation is not known or if one

prefers to enter a known core permeability,  option o  should  be

selected.   If the logged formation is not on the list of options

and a core permeability is not known,  options o must be selected

and  when the prompt ask for entry,  a zero must be entered.   If

this is the case,  the critical water cannot be estimated and the

permeabilities and water cut cannot be estimated.

After  all  data  is  entered,  the program calculations are

made.  All calculations are corrected for  depth,  shale  volume,

fluid content and temperature. Resistivity is corrected as stated

6

above.  An empirical estimate is made of critical water, absolute

permeability,  relative permeability,  effective permeability and

water cut.  Water cut for oil is reported in a fraction. Water to

gas ratio is reported as bbls. per mcf.

Printer output is optional.

After entering log data and calculating factors for a  zone,

recalculation  of other zones of interest can be made without en-

tering all new data.   The log heading data and type of mud  does

not change.  Use the arrow and entry key for rapid calculation of

additional zones of interest.

PLAYING "WHAT IF"

In order to play "what if",  after the initial  run  through

the  program,  use the arrow keys to toggle the main menus.  When

changes are made in the log matrix option or shale volume or type

of shale,  porosity calculations made from  Density,  Neutron  or

Sonic logs or x-plots are recalculated.

Each time the depth is changed,  the temperature of the for-

mation and the Rw is automatically  recalculated.  Any  parameter

that changes with depth i.e.,resistivity,  shale volume, porosity

must be recalculated.

To exit the program:

1-Enter the END key or

2-Enter control C or

3-Enter control break.

MISCELLANEOUS HINTS

Names of logs used in the program are based on Schlumberger

trade marks.  Logs from other companies which are compatible  may

be used in the program.

Neutron  logs  from each logging company are not compatible.

The program requires the log matrix for these logs to be  set  to

lime  and the charts from the logging company used to correct the

log reading to lime before using the cross-plot option.  The  CNL

is corrected for mud weight and temperature. The SNP is corrected

for hole size.  If a caliper survey is not available, the correc-

tion will be made assuming a gauge hole.

The gamma ray log entry will be corrected for mud weight and

hole size.   If a caliper survey is not available, the correction

will be made assuming a gauge hole.

When entering data from the log heading, operator name, well

name, location, county and state will accept 60 characters each.

7

Direct entry of porosity and  shale  content must    be  en-

tered  in  decimal  fractions  NOT  per-cent.  In  other  words a

porosity of 30% must be entered as .30 not 30.

When entering data from a density log, the entry may be made

in either a decimal fraction or the entry may be made  in  gm/cc.

The    mathematics    involved  requires  that  the   entries  be

consistent.   In other words,  if a shale correction is made  and

the    formation  density is entered in gm/cc,  the shale density

must also be entered in gm/cc.  If the formation density porosity

is entered in decimal fraction,  the shale density porosity  must

also  be  entered in decimal fraction.   Neutron porosity must be

entered in decimal fraction.

Porosity crossplots for density and neutron logs are made by

a crossplot  algorithm ,  after correcting for  fluid  and  shale

content.  Porosity    crossplots   using a  sonic log are made by

converting  all  data to  lime  matrix and    averaging,    after

correcting for fluid and shale content.

Factors  used  to  correct   porosity  for fluid content are

calculated assuming a normal pressure gradient and a natural  gas

gravity of 0.65.  The residual oil saturation in the flushed zone

is estimated based on empirical data and API gravity.    The  low

gravity cut off is 10 degrees API and the high gravity cut off is

50 degrees API.   In other words, if one enters an API gravity of

less than 10,  the value for 10 will be used.   If one  enters  a

gravity greater than 50, The value for 50 will be used.

If  the Induction Electric log option is selected and the 16

inch normal is greater than 10 times the mud resistivity,  an in-

termediate  evaluation is made for Sw,  Porosity and Ro using the

Rocky Mountain method.  If a shale option is selected,  the Sw is

reduced  by  10  per-cent and the porosity is adjusted to reflect

the decrease in Sw.   This information will be displayed  to  the

screen but will not be printed on the final print out.

In  order  to maintain maximum versatility and range of data

input methods and allow the user to play "what if" ,  error trap-

ping  is  kept at a minimum.   It is possible to enter data which

will cause math errors (divide by zero ,  etc).   Care should  be

used  when entering data and the menu prompts should be followed.

If one enters data which causes a math error,  the  program  will

notify the user and abort to the operating system.

Support  for  registered  users  is  provided by mail to H L

Hendrick at P.O. Box 79530, Houston,  TX 77279 or by telephone at

713-932-9408 or through CompuServe at address 76217,1171.

8