*------------------------------------------------------------*

W  A  Y  P  O  I  N  T      F O R      W  I  N  D  O  W  S

*------------------------------------------------------------*

Version 1.0

Ocean Routing & Navigation for  the Professional Mariner

User's Guide

*** ShareWare Version ***

WayPoint for Windows v1.0                                          Page 1

The software described in this document is provided under the terms of a

license Agreement.  Please read it carefully.  The software may be used or

copied only upon acceptance and in accordance with the terms of this

agreement.  Unauthorized use, duplication, distribution, or disassembly is

strictly prohibited by federal law.

The User Guide contains the necessary information to utilize the program.

However, ProMar Software reserves the right to make revisions and/or

periodic changes to the user documentation content without the direct or

inferred obligation of ProMar Software to notify any person of such revisions

or changes.  ProMar Software makes no warranty, expressed or implied, with

respect to the software and /or user  documentation, and specifically

disclaims the implied warranties of merchantability and fitness for a

particular purpose.  ProMar Software does not warrant that the software

and/or user documentation will meet your requirements or that the

operation/use of the software and/or user documentation will be uninterrupted

or error free.  You are solely responsible for the selection of the software

and/or user documentation to achieve your intended results and for the

results actually obtained.

This software, as with all navigation tools, is an aid to navigation.  The

prudent mariner will not solely rely on information obtained from this

software, but should use it as an aid with other navigation tools/methods

before making decisions.

Copyright 1993,1994 Michael D. Smith, ProMar Software.  All Rights Reserved.

Copyright, Trademark and Service Mark Notices:

---------------------------------------------

WayPoint is a trademark of ProMar Software.

MS-DOS and Windows are trademarks of Microsoft Corporation.

IBM is a registered trademark of International Business Machines, Inc.

Amver (v3.1) software is copyrighted by Ron Ristad and Richard Plant.

Trademarks of other companies mentioned in this documentation appear for

identification purposes only.

ProMar Software                 Fax:    (619) 464-5015

6161 El Cajon Blvd. #275        CIS:    71662,1015

San Diego, CA 92115             GEnie:  M.Smith106

WayPoint for Windows v1.0                                          Page 2

T A B L E   O F   C O N T E N T S

Introduction to WayPoint................................3

Chapter  1:  Installing & Running WayPoint..............5

Chapter  2:  Program Overview - Menu Commands...........7

Chapter  3:  File Menu Commands.........................13

Chapter  4:  Edit Menu Commands.........................17

Chapter  5:  Voyage Menu Commands.......................22

Chapter  6:  Utility Menu Commands......................25

Chapter  7:  Printing Menu Commands.....................27

Chapter  8:  Navigation Menu Commands...................29

Chapter  9:  Window Menu Commands.......................30

Chapter 10:  Tutorial **** REGISTERED USERS *****.......31

Appendix A:  Data Entry Requirements....................31

B:  Sample Reports *** REGISTERED USERS ****...32

C:  AMVER......................................32

D:  Limited Warranty and Disclaimers...........33

E:  Software License Agreement.................34

F:  Contacting ProMar Software.................35

WayPoint for Windows v1.0                                          Page 3

----------------

Introduction

----------------

What is WayPoint:

-----------------

WayPoint for Windows (1.0) is a powerful yet flexible navigation routing

program developed for the professional mariner.  The program is designed to

take full advantage of the Windows Graphical User Interface (GUI).  Multiple

options are available to the navigator for creating and maintaining Voyage

Sailing Plans, daily reports and AMVER messages, along with navigation

calculators for calculating the various sailings.  This is a full featured

application which provides the ship's navigator with a powerful navigation

program.

WayPoint is distributed as ShareWare.  WayPoint is not free or public domain

software.  ShareWare permits you to try WayPoint before you buy the software.

ShareWare provides a good channel for the distribution of a product without

incurring the high marketing costs involved in the normal commercial

distribution of software.  These cost savings are passed onto the user.  With

the registered version you receive technical support, a personalized serial

number, notices and discounts on future versions.

This User's Guide and Reference Manual is a comprehensive guide to all of the

features and procedures offered within the software WayPoint.  The registered

version contains additional features such as a detailed program tutorial

and sample reports.

Why You Should Use WayPoint for Windows:

---------------------------------------

WayPoint for Windows has the accessibility and power to increase productivity

in the planning and maintaining of a vessel's voyage.  If you know how to use

other Windows applications, you already know how to use WayPoint for Windows'

menus and commands. If you are new to the Windows environment you will find

that the menu system and command structure has been designed to follow the

program's features, thereby making it straightforward and easy to use.

WayPoint for Windows blends the work of voyage planning

with the power of computers and the flexibility of Windows. Computer users of

all levels will find WayPoint for Windows an excellent tool for voyage

planning.

Contents of package:

--------------------

Every registered user ordering the full WayPoint for Windows package will

receive:

* Software Diskette (3.5" 1.44m only)

* Sample sailing plans

* WayPoint for Windows User's Guide & Reference Manual

* Your personal serial number

* License Agreement

WayPoint for Windows v1.0                                          Page 4

System Requirements:

--------------------

You must already have Microsoft Windows v3.1 installed on your computer

before WayPoint for Windows can be installed.  The following are also

required to run the program:

* VGA monitor

* 1 MB of ram

* Hard disk; with at least 1 MB free space for program files

* Mouse

* 15% System Resources is utilized by WayPoint

*Optional:	Math-Coprocessor

Printer

Technical Support:

------------------

Technical support is available only to registered users.  At present,

ProMar Software may only be reached through the following methods:

* US Postal Service

* EMail Services (CompuServe, GEnie)

* Fax

Comments on improvements and noted 'bugs' are always appreciated.  Bear in

mind that the developers of ProMar are actively sailing in the maritime

industry and that there will be times of delayed responses -- However, all

inquiries from registered users are always answered.

How to Use This Manual:

----------------------

Using WayPoint for Windows can be straightforwared and easy to use if

approached correctly.  This user manual is organized to help you learn the

WayPoint for Windows quickly and efficiently.

The manual is organized into three parts:

Part   I:       WayPoint - Functions and Commands

Part  II:       Tutorial  (REGISTERED VERSION ONLY)

Part III:       Appendix A-F

****************************************************************************

****        S H A R E W A R E   U S E R S   P L E A S E   R E A D       ****

****************************************************************************

The text file called MANUAL.DOC, which is enclosed with the software,

is an abbreviated version of the printed User's Guide.  The printed manual is

supplied only to registered users at a cost of $10.00 ($15.00 on

international orders).

By ordering the printed manual you receive an added bonus of a detailed

program tutorial including pictures for an excellent step-by-step lesson.

Additional screen shots of WayPoint's numerous functions and voyage

planning capabilities are also included.

****************************************************************************

WayPoint for Windows v1.0                                          Page 5

----------------------------------------

Chapter 1: Installing & Running WayPoint

----------------------------------------

Software Installation:

----------------------

The installation program will automatically install WayPoint 1.0 onto your

hard disk.  It will first determine that there is enough disk space available

prior to installation.

To install WayPoint for Windows:

--------------------------------

1.      Start Windows by typing WIN at the DOS prompt and pressing [Enter]

2.      Insert the WayPoint for Windows diskette in drive A or B.

3.      From the Program Manager menu select:   File     Run.

If the floppy is in drive A, type:	A:Setup

or

If the floppy is in drive B, type:	B:Setup

4.      WayPoint will automatically install itself on your harddisk.  You

will be prompted for a directory to install the program in.  Setup

uses [ C:\Waypoint ] as a default.

If no changes are needed press the OK button to continue, otherwise

enter the path you wish Setup to use.

5.      The installation will create a new program group in the Program

Manager and install the program icons for use.  The program group

will be labeled WayPoint for Windows.

6.      You will be asked to register your version of WayPoint for Windows.

Enter your name, company name (optional), and serial number.  The

serial number must be entered exactly as shown on the disk you

received from ProMar Software.

Note:   If the registration process is not properly done, setup will

continue with the installation, however the program will

remain as the ShareWare version until the registration is

completed.  To  complete an aborted registration process,

access the registration form when starting WayPoint and

re-enter the correct serial number.

****************************************************************************

The WayPoint for Windows installation program for will not modify your

Win.ini, Autoexec.bat or Config.sys files in any way.

****************************************************************************

WayPoint for Windows v1.0                                          Page 6

Starting WayPoint for Windows:

------------------------------

To start WayPoint for Windows, follow these steps:

1.	Start Windows by typing WIN at the DOS prompt and pressing [Enter]

2.      Activate the group window that contains WayPoint for Windows.

3.      Start WayPoint for Windows with the mouse by double clicking on the

WayPoint for Windows program item icon.

Configuring WayPoint for Windows:

---------------------------------

The first time you start up WayPoint for Windows you will need to configure

the program with your vessel's particulars.  To properly run WayPoint the

following information is required:

* Vessel's name

* Vessel's call sign

* Operating speed (WayPoint uses 20.0 knots as a default speed)

* Radio/INMARSAT ID's for AMVER messages

* Printer setup to Laser or Dot Matrix.

(WayPoint uses the Dot Matrix as a default printer).

WayPoint for Windows v1.0                                          Page 7

----------------------------

Chapter 2: Program Overview

----------------------------

This chapter describes in detail the menu commands and toolbar functions of

WayPoint for Windows.  It has been divided into four parts:

Screen indentification

Menu Commands

Keyboard shortcut keys

Toolbar commands

The following diagrams show the various WayPoint screens with the locations

of the parts marked for easy identification.  Most of the parts labeled are

standard Windows terms and are used throughout this manual.  Please refer to

the appropriated sections throughout the manual for more details on the

commands.

WayPoint Application Screen:

----------------------------

WayPoint for Windows has a multiple document interface.  This means that more

than one window is housed within the application window.  Although

WayPoint 1.0 displays one waypoint list at a time, the sailings windows can

also be displayed along with the main waypoint list.  The program windows

which have the minimize and maximize buttons can be reduced to icon size or

enlarged to normal display.

WayPoint List:

--------------

The waypoint list is the main display for all route information, divided into

three display sections.  The first section contains the Departure and Arrival

port information.  The description of the highlighted waypoint is also

displayed in this section.  The second section is the main list of waypoints

entered for the route.  The third section is the Passage Summary display.

This portion displays information on the vessel's passage from departure to

arrival.

Passage Summary Display:

------------------------

Monitoring the passage for each route is within the Passage Summary display

portion of the waypoint list.  The departure and scheduled arrival times are

displayed once a departure time has been entered for the passage.  As each

daily position is entered the Passage Summary information is updated to

reflect the latest position.  The Summary displays the day's run from the

previous position update and total run from departure.  The projected

transit display is updated to show the total passage as projected from

departure to the current ETA for arrival.

WayPoint for Windows v1.0                                          Page 8

Main Menu:

---------

The menu command structure for WayPoint for Windows 1.0 is similar to that

of most of the popular Windows applications currently in use.  It is this

common menu structure and the flexibility of the Windows environment that

has made Windows so popular.  WayPoint 1.0 menus are designed to follow the

operation of creating and maintaining a route's sailing plan.

To activate the menus you have the option of using the mouse or keyboard.

When using the mouse the menu item is selected by clicking on the item.

To activate the menus with the keyboard the ALT key must be pressed first.

This activates the main menu for menu selection.  The menu items are selected

by pressing the letter that is underlined on the desired menu.  Pressing the

ESC key deactivates the menu.

A third option for menu selection is the use of short-cut keys.  These

commands are activated by holding down the CTRL key and pressing the

appropriate letter.  These shortcuts keys are shown on the menu next to the

items that are activated by them.

File Menu:

----------

The functions of the File Menu are used for opening and closing waypoint

files.  The exit command is also in this command structure for ending the

program.

File Command	Description

-----------------------------------------------------------------------------

New             Opens a blank list.  This command is used when creating a

new waypoint sailing plan.

Open            Retrieves a currently saved waypoint file from the disk.

Files are available as:

* Master Sail Plans - Unformatted sailing plans.

* Working Files -     Dep/Enroute files for tracking the

daily passage.

* Amver Files -       Conversion utility to import Amver 3.1

data files and format as WayPoint data

files.  This utility imports only

Master Sail Plan data from the Amver

data files.

Save            Saves waypoint list to file as Master or Working files.  A

third option is available for saving both formats at the

same time.  Whenever a Master Sail Plan is saved all

formatting attributes of the Working file are ignored.

Close           Closes the waypoint list.  If any changes to the file have

been made you will be prompted to save before the file is

closed.

Exit            Ends WayPoint and returns you to the Windows operating

system.  As with the Close menu item, if any changes have

occurred you will be prompted to save before the file is

closed.

WayPoint for Windows v1.0                                          Page 9

Edit Menu:

----------

The Edit menu provides the tools for editing the waypoint list.  Waypoints

can be added, deleted and changed.  Passage notes and setup information are

also obtained from the Edit menu.

Edit Command		Description

-----------------------------------------------------------------------------

Setup Port/Zone		Enter the name and zone descriptions for the

Departure and Arrival ports.

Passage Charts		Enter the charts to be used for the passage being

created.

Voyage Notes		Route description.

Add Wypt		Adds a new waypoint at the end of the waypoint list.

Edit Wypt		Edit a waypoint currently in the waypoint list

Insert Wypt		Adds a new waypoint between two listed waypoints.

Delete Wypt		Removes a waypoint from the list.

Reset Passage           Resets a working waypoint file back to a master

sail plan.

Reset Arrival           Resets the arrival status of a route back to an

ETA status.

Voyage:

------

The Voyage Menu provides all the tools for maintaining a passage from

departure to arrival.

Voyage Command			Description

--------------------------------------------------------------------------

Departure: Begin Passage        Activate the route with a departure date &

time and sets the passage enroute.

Departure times may be entered in GMT or LMT

format.

Daily Position Update           Calculate and maintain daily updates for

position reports.  Times are entered and

displayed in GMT format only.

Arrival: Waypoint Position      Date stamp a waypoint with an actual time as

it is reached or an ETA time to calculate

required speeds to reach that waypoint.

Times are entered and displayed in GMT

format only.

WayPoint for Windows v1.0                                          Page 10

Arrival: End Passage            Ends the passage and calculates an arrival

slip for the total run.  Arrival times may

be entered in GMT or LMT format.

ETA/Speed Calculations          Calculate a schedule of ETA's & speeds for

the arrival port.  Calculations are based

upon the most recent position update.  ETA's

are shown in LMT and GMT formats.

Utilities Menu:

---------------

The Utilities Menu is used to configure WayPoint for your vessel and for

managing the number of working files that reside in your Routes directory.

Utilities Command	Description

-----------------------------------------------------------------------------

Vessel Configuration	Configures the program with the vessel's name,

company, call sign, radio/comsat, operating speed,

and printer type.

File Management		Utility to manage the library of working files.

WayPoint 1.0 removes working files only.

Print Menu:

-----------

The Print Menu provides the commands for printing the many reports available

to the user.  Printed reports are available for waypoint lists, passage

reports and Amver messages.

Print Command		Description

-----------------------------------------------------------------------------

Waypoint List		Print out of the voyage sail plan

Position Report		Prints a daily position report.  Option to print 2 -

6 copies.

Arrival Report		Prints an end of voyage arrival slip; 2-6 copies.

AMVER                   Print options for AMVER messages.

WayPoint for Windows v1.0                                          Page 11

Navigate Menu:

--------------

The Navigate Menu provides the tools for calculating the various sailings.

From the menu you are able to calculate Mercator sailings, Great Circle

sailings and track points, and DR positions for either GC/RL calculations.

Navigate Command	Description

-----------------------------------------------------------------------------

Mercator		Calculate mercator course and distances

DR Position		Calculate a DR position for either Rhumb Line or

Great Circle

Great Circle		Calculate Great Circle track points; option to print

listing.

Window Menu:

------------

The Windows Menu provides the standard functions of Windows 3.x windowing

commands.  Also included in this menu are toggle commands for the WayPoint

Toolbar and Passage Summary screen.

Window Command		Description

----------------------------------------------------------------------------

Cascade			Arrange program windows

Tile                    Arrange program windows

Arrange                 Arrange iconized program windows

Toolbar                 Toggle to turn on/off the toolbar display.

View Summary            Toggle to turn on/off the voyage summary display

screen.

WayPoint for Windows v1.0                                          Page 12

------------------------------

Menu Shortcut Keys - Reference:

-------------------------------

The following table is a listing of the shortcut keys available to activate

the menu commands.  Shortcut keys are activated by holding down the CTRL key

and then pressing the appropriate letter.

File:					Edit:

CTRL-S  Save file                       CTRL-Z  Setup Port/Zone

CTRL-X	Exit program			CTRL-C	Passage Charts

CTRL-N	Voyage Notes

Voyage:					CTRL-A	Add Waypoint

CTRL-D	Departure			CTRL-E	Edit Waypoint

CTRL-U	Position Update			CTRL-I	Insert Waypoint

CTRL-Y	Arrival:  Waypoint Position

CTRL-R	Arrival:  End Passage

Utilities:				Print:

CTRL-G	Vessel Configuration		CTRL-W	Print Waypoint List

CTRL-M	File Management			CTRL-P	Print Position Report

Window:

CTRL-B	Toolbar toggle

CTRL-V	Summary display toggle

The WayPoint Toolbar:

---------------------

One of the powerful features in WayPoint is the WayPoint Toolbar.  A toolbar

contains a set of push buttons designed to provide direct access to many of

the commonly used commands with the mouse, bypassing the menus.  These tools

can be used to create and maintain your waypoint sailing plans.  By default,

the toolbar is displayed on program startup.  The toolbar display may be

toggled on/off by using the menu commands Window, Toolbar (shortcut key

CTRL-B).

The WayPoint Toolbar is grouped to follow the main menu command structure.

By clicking once on the toolbutton, program functions can be accessed

quickly.  Only active toolbuttons can be accessed. For a quick toolbutton

description, if the left mouse button is held down when clicking the button

a short command description is displayed in the lower left status bar of the

main screen.  All active toolbuttons on the WayPoint Toolbar are in color and

will depress when clicked on.  Inactive toolbuttons will have a metallic look

and will not react to the mouse -- the corresponding menu commands will also

be inactive.

**********  TOOLBAR ICON DISPLAYS AND DESCRIPTIONS **********

**********  INCLUDED WITH REGISTERED VERSION ONLY  **********

WayPoint for Windows v1.0                                          Page 13

------------------------

Chapter 3: File Commands

------------------------

Save File:

----------

By saving a waypoint list you store your work as a file on disk.  Until a

file is saved, your work exists only in the computer's memory.  What this

means is, if the electricity goes off, even briefly, your computer's memory

is lost -- along with any unsaved route information.

This section will detail the different file formats and procedures for saving

waypoint files.  Two file formats are utilized by WayPoint for saving files:

Master and Working files.

Master files are your master sailing plans, permanent routes created for

repeated use.  These files contain only the route information pertaining to

the ports and waypoints.

Working files are exactly that, files maintained and updated during a passage

from departure to arrival.  These files contain the master sailing plan along

with the passage times and ETA's.

The following commands can be used to open the Save File dialog box:

Menu Commands:	File,	Save File

Keyboard:	CTRL-S

Tool button

From the Save File dialog box you have the three save options:

(1)	Master files.

(2)	Working files.

(3)	Save both a Master and Working file.

To save the current waypoint list as a file:

* Enter the name of the file (or use the currently displayed name) in the

File Name text box.

* Choose the desired File Format option.

* Click on the OK command button to accept the file, or double click on the

filename if selected from the file list box.

The following sub sections will describe in more detail the file formats and

file naming procedures.

WayPoint for Windows v1.0                                          Page 14

Master Sailing Plans:

--------------------

Saving waypoint lists as master files creates a library of permanent routes.

Having a library of files readily available will help make the planning of a

passage an easier job.  A permanent record of established routes can be

quickly transferred over to the nautical charts to be used for the passage.

When creating a new waypoint file, the Save File dialog box will display

UNTITLED in the File Name text box.  This is because the new file has no

name.  All waypoint files use standard MS-DOS file name conventions:

* A file name can be from one to eight characters long .

* You can use letters A through Z (upper and lower case), numbers 0 through

9, hyphens (-), underscores (_), and exclamation marks (!).

* You cannot use periods (.) or blank spaces.  A period separates a file's

first name and extension.

* You cannot use characters such as asterisks (*), plus signs (+), equal

signs (=), and some punctuation marks.

* The file extensions used by WayPoint, [ .wpr ] and [ .wpw ], are

automatically added during the save process.

* For more information on naming of MS-DOS files please refer to your

DOS manual.

It is recommended that file names identify the passage being saved.  The

name should be easily identifiable, i.e., an Oakland to Honolulu passage

could be named as OAK-HON, O_HON-GC, etc.  Once a standard for file names

is created it should be maintained to minimize confusion in locating multiple

routes in a large library.

If the selected name already exists, all information in that file will be

erased and the new file will be saved in its place.  To avoid accidental

erasures WayPoint will always prompt for confirmation before overwriting

an existing file.

Working Files:

-------------

As explained earlier in this chapter, working files are used to maintain and

update a vessel's passage from departure to arrival.  When departure has been

entered the file needs to be saved as a working file.  WayPoint will

recognize when changes have been made to a Master file.  If departure has

been entered the Working File format option will be selected when the file

save dialog box is opened.

The naming of files is the same for Working files as it is for the Master

sail plans.  Additional options for filenames can be used, for example, the

same passage from Oakland to Honolulu could be named HON-35 to identify the

passage as voyage 35.  The master file name can also be used to name the

working file since both formats are tracked by WayPoint.

WayPoint for Windows v1.0                                          Page 15

Both Formats:  Master & Working Files

--------------------------------------

A third save option is available which permits both formats to be saved at

the same time.  When the Both formats option is selected, a Master and

Working file will be saved to file.  This option can be useful when a file

has been modified and a master version has not been saved.  Again, WayPoint

will prompt on any file names that are duplicated.

New File:

---------

When you first startup WayPoint for Windows, you will see a blank screen.

If a route is being created and a file for that route does not exist one

must be created.  The New File command will open and display a blank waypoint

list.

To create a new waypoint list, use one of the following commands:

Menu Commands:	File,	New File

Tool button

Close File:

-----------

The Close File command closes the waypoint file currently displayed without

closing the main program.  The waypoint screen will be blank except for any

navigation calculators that may be opened.  If any changes have occurred

since the last save, WayPoint will prompt with an option to save the latest

changes that have occurred.

Open File:

---------

When WayPoint is started the program screen is blank of any files.  If an

existing waypoint file is to be worked on then the Open File command is used

to access it.  The Open File dialog box is similar to the Save File dialog

except rather than saving a file the selected file is retrieved and displayed.

WayPoint will open three types of file formats:

* Master files          Permanent sailing plans created by WayPoint

for Windows.

* Working files         Working files for enroute use.

* Amver (v3.1) files    Data files created using Amver v3.1.

WayPoint for Windows imports Amver files

and converts the file to WayPoint format.

To Open an existing file, use the following commands:

Menu Commands:	File,	Open File

Tool button

WayPoint for Windows v1.0                                          Page 16

WayPoint always opens to the default directory called \Routes.

If data files reside in other directories locate the subdirectory by using

the Directory and Drive list boxes.  A file must first be selected before

it can be loaded into memory.

To select a file, first select the file format option:

(1)     Use the mouse and place the highlight scroll bar on the file by

clicking once and then pressing the OK command button

or

(2)	Double click on the file to be loaded.

Selecting the file format option changes the title on the Open File dialog

box so that the command operation to be made is always displayed.  By

default, the Master file option is selected when the dialog box is opened.

To display the appropriate files select the corresponding option button.

File extensions recognized by WayPoint are:

* WPR	Master sailing plans

* WPW	Working files

* DAT	Amver data files

Amver File Conversions:

-----------------------

For users who have accumulated a large library of routes using the DOS

program Amver (version 3.1), these files can be converted and used in

WayPoint.  The file conversion process will only convert Master sailing plan

information, therefore both Master and Working Amver files can be converted

into WayPoint sailing plans.

WayPoint for Windows maintains all working positions used throughout the

passage, whereas Amver files only keep the last two position updates

displayed.  Because WayPoint uses a different format in maintaining working

files, only the Master sailing information is extracted from the Amver files.

Passage information can then be updated through WayPoint's voyage functions.

To convert Amver data files:

----------------------------

(1)	Select the format option to Import Amver data.

(2)     If the data files are in a different directory you will have to

change to that directory.

(3)	Locate the file to be loaded and load into WayPoint.  The conversion

process is automatic.

(4)     Amver users should note that additional editing may be required to

the following items after a file has been converted:

* Zone description		* Port Names

* Waypoint descriptions		* Port Names

* List of passage charts	* Voyage description notes

WayPoint for Windows v1.0                                          Page 17

WayPoint uses a fixed length restriction of 12 characters for Departure and

Arrival ports and 30 characters for waypoint descriptions.  This may at times

conflict with the descriptions in the Amver data files.  The list of charts

may at times also be truncated during conversions depending on how they were

originally saved.

Exit WayPoint:

--------------

The Exit command closes WayPoint and returns you back to the Windows

operating system.  If any changes have occurred since the last save, WayPoint

will prompt with an option to save the latest changes that have occurred.

The following commands can be used to exit WayPoint:

Menu Commands:	File,	Exit

Keyboard:  CTRL-X

--------------------------

Chapter 4:  Edit Functions

--------------------------

Setup: Port/Zone

----------------

The port setup dialog box accepts the departure and arrival ports and zone

descriptions.  This information is required by WayPoint for passage

calculations from departure to arrival.

The setup dialog box may be accessed through different commands:

Menu Commands:     Edit,    Setup: Port/Zone

Shortcut keys:     CTRL-Z

Toolbar

The port setup dialog box will open showing the current port and zone

description settings.  If creating a new waypoint file then these settings

will be blank.  The valid range for port zone descriptions is:  -12 to +12.

Selecting the OK button will accept the modified entries and update the

waypoint list, while the Cancel button will ignore any changes made and

close the dialog box without updating.

The Port Setup function is active only when a waypoint list is displayed.

WayPoint for Windows v1.0                                          Page 18

Add Waypoint:

------------

The Add Waypoint function adds a new waypoint position to the end of the

waypoint list.  This is a primary function for creating new waypoint lists.

(See Insert Waypoint)

The Add waypoint dialog box may be accessed through different commands:

File Commands:     Edit,    Add Waypoint

Keyboard:          CTRL-A

Toolbar

When creating a new waypoint list the first two waypoints must be entered at

the same time.  Once the initial two waypoints have been added to the list

all remaining waypoints are added individually.

Add Waypoint command buttons:

Calculate       Calculates the coordinates and displays the course and

distance results in the Cse/Distance display.  This option

is useful when comparing the difference in tracks for the

passage leg.

Add             Adds the entered position to the end of the waypoint list.

All calculations for the waypoint list are updated with each

new waypoint.

Clear Entry     Clears the Add Waypoint position fields.

Close           Ignores the currently displayed position, closes the Add

Waypoint dialog and returns to the waypoint list without any

updating.

The Add Waypoint function is active only when a waypoint list is displayed.

WayPoint for Windows v1.0                                          Page 19

Insert Waypoint:

----------------

The Insert Waypoint function inserts a new waypoint between two existing

waypoints.  This function works similar to the Add Waypoint function,

utilizing the same commands except that the new waypoint is inserted at a

designated position.

To Insert a waypoint, place the waypoint scroll bar on the waypoint where the

new wypoint is to be inserted.  For example, if inserting a new waypoint

between waypoints (1) and (2), place the scroll bar on waypoint (2).

The Insert Waypoint dialog box may be accessed through different commands:

Menu Commands:   Edit,    Insert Waypoint

Keyboard:        CTRL-I

Toolbar

Insert Waypoint command buttons:

Calculate       Calculates the position and displays the course and distance

results between each waypoint in the two Cse/Distance display

areas.  This option is useful when comparing the difference

in tracks for the passage leg.

Insert Wypt     Inserts the position to between the two selected waypoints.

All calculations for the waypoint list are updated with each

inserted waypoint.

Clear Entry     Clears the waypoint position fields.

Close           Ignores the currently displayed position, closes the Insert

Waypoint dialog and returns to the waypoint list without any

updating.

The Insert Waypoint function is active only when waypoints are in a waypoint

list.

WayPoint for Windows v1.0                                          Page 20

Edit Waypoint:

-------------

Rather than create entire new waypoint files, the Edit function modifies

existing waypoints to update a current passage as waypoints are changed but

not removed from the list.

To edit a waypoint, place the scroll bar on the desired waypoint and access

using one of the following methods:

Menu Commands:   Edit,    Edit Waypoint

Mouse:           Double click on the waypoint

Shortcut keys:   CTRL-E

Toolbutton

Edit Waypoint command buttons:

OK      Accepts the entered information and updates the waypoint list.

The speed is reset to the end of the waypoint list.

Cancel  Ignores any changes made and returns back to the waypoint list.

Master Files may be edited without restrictions.  Working Enroute files

have a number of restrictions.

* The speed and distance calculations must remain within the program

parameters of 50 knots and passage duration of 120 days.

* Editing waypoints prior to position updates are restricted to the

description and track only.  It is assumed that before a daily position is

obtained that waypoints are current.

* Editing removes any waypoint arrival eta's that may be present by

recalculating the passage with the new speed.

The Edit Waypoint function is active only when waypoints are in a waypoint

list.

Delete Waypoint:

----------------

To remove a waypoint from the list the Delete Waypoint function is used.

Prior to removal WayPoint will prompt for confirmation.  There is no undo

feature in WayPoint.  The delete function will delete any waypoint selected

on both Master and Working files.  Deleting waypoints on a Working file will

recalculate passage information and reset the entire passage if the departure

or daily positions are removed.

To delete a file, place the scroll bar on the waypoint to be deleted and

select from the menu:  Edit,    Delete Waypoint

To accept, click the OK button.  You will be prompted for final confirmation

to remove the waypoint from the list.  Select yes if you are sure.

The Delete Waypoint function is active only when more than two waypoints are

in a waypoint list.

WayPoint for Windows v1.0                                          Page 21

Reset Passage:

--------------

The reset function is only available after departure has been taken and you

are editing a Working file.  By selecting the reset function the current

waypoint list is reset back to a Working sail plan.  All departure/arrival

times and daily positions are removed from the waypoint list.

To access the reset function select from the menu:  Edit,  Reset Passage

WayPoint will prompt for confirmation to remove all passage formatting from

the list.  Remember that a Working file can also be saved as a Master sailing

plan by utilizing the file format option when saving files.

The Reset Passage function is available only for Working Enroute files where

departure has been taken.

Passage Charts:

---------------

Each waypoint file can have the charts required for the route entered and

saved with the file.  There is no special formatting required for entering

the charts.  Each waypoint file has three text lines available to accommodate

the list of required charts.

The list of charts will be printed along with the waypoint list and can be

viewed using the Chart function commands:

Menu Commands:   Edit,    Passage Charts

Shortcut keys:   CTRL-C

Toolbutton

Voyage Notes:

------------

A voyage description can be entered by using the Voyage Notes function.  The

description is displayed in the voyage summary screen and printed along with

the waypoint list.

Menu Commands:   Edit,    Voyage Notes

Shortcut keys:   CTRL-N

Toolbutton

These functions are available only when a waypoint list is loaded.

WayPoint for Windows v1.0                                          Page 22

--------------------------

Chapter 5: Voyage Commands

--------------------------

Departure: Begin Voyage

-----------------------

To activate a route and calculate passage information a departure time must

first be entered.  Departure times may be entered in both LMT or GMT formats.

The zone descriptions have been entered into WayPoint and therefore dates and

times will be quickly converted over to GMT for the inter-waypoint

calculations.  All passage calculations are based upon a departure time for

the route, therefore most of the program's functions are unavailable until a

departure time is entered.

To access the departure function:

Menu Commands:   Voyage,  Departure: Begin Voyage

Keyboard:        CTRL-D

Toolbutton

First determine how the departure time is to be entered, LMT or GMT, by

selecting the proper format option.   All times will be converted on the

waypoint list to GMT formats, so it doesn't matter how the departure time is

entered.  WayPoint takes care of the conversions.  See Appendix A  for more

on date and time formats.

The projected speed is automatically entered from the file setup, but can be

changed voyage to voyage.  The voyage number and name of the watch officer

are optional items, but should be entered at this time as they are used in

the passage reports.

Entering a new departure time after a previous time has been entered will

reset the entire voyage of any formatting and recalculate the passage.

WayPoint will prompt for confirmation before resetting the departure times.

The Departure: Begin Passage function is available only when waypoints are

listed.

Daily Position Update:

----------------------

Once departure has been taken, the route must be monitored daily with

position updates if the reports and messages are to remain accurate.  The

Daily Position Update function updates the passage with the commonly known

'Noon Postion'.  With the use of modern electronics for navigating, the noon

position may now be taken at any given time that the vessel or company is

utilizing as a daily reporting time.

To access the Update function:

Menu Commands:   Voyage,  Daily Position Update

Keyboard:        CTRL-U

Toolbutton

WayPoint for Windows v1.0                                          Page 23

When first displayed the Passage Update dialog box will show the day and time

of the last update.  Dates and times are in GMT format only.  Using the Date

and Time text boxes, enter the day's reporting position time, then click on

the Calculate button to obtain a DR position.  Information on the day's run

and total run will be displayed in the appropriate boxes along with a DR

position for the entered time.  This position can be modified if a more

accurate one is available.  When all the information is correct select the

Accept button to update the waypoint list.  See Appendix A  for more on date

and time formats.

The Passage Summary screen will be updated to reflect the latest daily

position.

When the Calculate button is clicked the date and time can not be changed

except by using the Clear Entry button.  This will reset the Passage Update

dialog box so that a new time can be entered.

To ignore all entries, select the Cancel button to return to the waypoint

list without updating.

The Position Update function is available only after departure has been entered.

Arrival: Waypoint Position

--------------------------

When departure has been entered, every waypoint has a calculated ETA posted

to it.  These ETA's may be updated to keep a more accurate track of the

passage.  This function only updates the position's eta and recalculates the

speeds and times, it does not update the passage summary data.

This function can be used when a waypoint is reached and the time of arrival

is entered to update ETA's, or when a timed arrival is scheduled for a

certain waypoint and a required speed to make that arrival time is needed.

To access the Waypoint Arrival function, first place the scroll bar on the

waypoint to be updated.

Menu Commands:   Voyage,    Arrival:  Waypoint Position

Keyboard:        CTRL-Y

Toolbutton

If the scroll bar is on the departure or arrival points, the respective

dialog boxes will appear for those points with the option to use LMT or GMT.

All points in between use the standard ETA dialog box and require GMT times.

When the date and time are entered, select the OK button to update the

waypoint and return to the waypoint list.  The remainder of the list will be

recalculated to reflect any changes made to the waypoint's ETA.  See

Appendix A  for more on date and time formats.

The dialog box for the Arrival Waypoint is the same as used by the

Arrival:  End Passage dialog box.  The two functions use the end voyage

toggle switch to determine how the function is to be used.  For the waypoint

ETA/Arrival function the toggle is displayed in the unchecked mode.  Do not

check this option for the ETA/Arrival function.

The Arrival Waypoint function is available only after departure has been entered.

WayPoint for Windows v1.0                                          Page 24

Arrival: End Passage

--------------------

A passage ends when arrival is taken at the final waypoint.  The

Arrival: End Passage function finalizes the route with the arrival time.  As

with the departure setup, when the arrival time is entered, LMT or GMT

formats may be used.

To access the arrival function:

Menu Commands:   Voyage,    Arrival:  End Passage

Keyboard:        CTRL-R

Toolbutton

When displayed the Arrival dialog box will show the current ETA as shown on

the list, in addition, the End Passage toggle will be checked.  This tells

WayPoint that the passage has ended.  Once entered the passage summary

display receives a final update for arrival.

The Arrival:  End Passage function is available only after departure has been

entered.

ETA/Speed Calculations:

-----------------------

While a passage is underway a schedule of arrival ETA's for the Arrival

destination can be viewed and printed.  The display box shows each calculated

ETA, in LMT and GMT format, with a corresponding required speed through a

range of selected speeds.

To calculate arrival ETA's:

Menu Commands:   Voyage,    ETA/Speed Calculations

Printer Options Menu            ETA Calculations

Toolbutton

The following are non-changeable ETA displays:

Avg Spd          The most recent day's run average speed.  On departure this

value is the projected speed.

TAS              Total average speed calculated from departure to the most

recent position update.  On departure this value is 0 knots.

DTG              Distance to go is is the remaining passage distance based upon

the latest position update.

WayPoint for Windows v1.0                                          Page 25

The following settings are used to adjust the ETA calculations:

Min/Max          Sets the range for the ETA calculations.  When first

displayed this range will be set 1.0 knot below and above

the latest average speed.  The range is adjusted by entering

the desired range of speeds.

Increment        Sets the decimal increment on how the range of speeds

between the Min and Max values are to be displayed.  When

first displayed the initial increment is .5 knots.

Time             The time display can be shown in either exact minute

calculations or rounded to the nearest tenth of an hour.

After the range of speeds has been inputed, press the calculate button to

display the calculations.  To print the list of ETA's, press the print

command button.

The ETA/Speed Calculation function is available only after departure has been

entered.

---------------------------

Chapter 6: Utility Commands

---------------------------

Vessel Configuration:

--------------------

WayPoint must be configured to your vessel if it is to work properly.  After

installing WayPoint on your computer, the first time you start the program,

the Vessel Configuration dialog box will be displayed.  After the initial

configuration the dialog box can be accessed through menu commands.

The vessel configuration requires that the vessel's name, call sign and

proforma speed be entered.  The default speed is 20 knots.  Radio and

Inmarsat information is utilized by the AMVER message program.

Printer setup configures WayPoint for report printing.  Select the proper

option on the type of printer that WayPoint will be generating reports.

The following commands can be used to access Vessel Configuration:

Menu Commands:   Utility,   Vessel Configuration

Keyboard:        CTRL-G

Enter the vessel's name, company name (optional), call sign, communications

station, printer type and operating speed in the appropriate text boxes.

When all the information has been entered click on the Update button to save

the configuration.

WayPoint for Windows v1.0                                          Page 26

File Management:

---------------

It is possible that with a large number of data files being created that an

excess number of working files can be created.  WayPoint has a file

management utility to remove excess Working files.  Note that if the Master

sailing plan is always saved as a Working file then the old passage will be

overwritten thereby keeping the number of working files to a minimum.  This

utility is useful for when sailing plans are modified and a large number of

old working files reside in the data directory.

This utility will only delete the working data files.  The master sailing plans are not

effected by this utility.

The File Manager can be selected by the following commands:

Menu Commands:    Utility,    File Management

Keyboard:         CTRL-M

To delete a working waypoint file it must first be selected and entered

into the Selected File list.  This may be done by a number of options:

* Mouse select  Double click on the desired file name

* Single Select         Click once on the desired file and press the

Select button

* Multiple Select       Use the mouse and click once on each file to be

deleted.  Use the transfer [>>>>] button to transfer

all the selected files.

Pressing the Delete command will delete all listed files in the Selected File List.

You will be prompt for confirmation prior to deletion.

The Deselect command will remove the select highlight from all selected

highlights.

The procedures for transferring Selected Files back to the File list is done the same

way but with the commands listed on the Selected files side.

The Cancel command will ignore all selections and return back to WayPoint.

WayPoint for Windows v1.0                                          Page 27

----------------------------

Chapter 7: Printing Commands

----------------------------

Print Options Menu:

-------------------

WayPoint has two avenues for printing options; the main menu commands and the

printer toolbutton .  Using the toolbar will display a popup menu to open and

close the WayPoint reports.  The main menu accesses these reports

individually.  All the following command descriptions can be utilized by

using the printer toolbutton.

By clicking on the printer toolbutton the popup menu for printing options

will be displayed.

To select the desired printing option, click on the appropriate menu item.

The Waypoint List option will display the Print button whereas all other

items will display the Open button to open the reports for modifying.   This

function is useful when multiple reports are to be printed in one session.

When a report function is unavailable for printing, the option will not

activate when clicked on.  For example, if departure has not been entered

then the passage and AMVER reports will not be accessible.

Printing:  Waypoint List

------------------------

A report of the waypoint list can be printed showing detailed information of

each waypoint.  If a departure time has been entered the report will also

contain the scheduled ETA for each waypoint.

Additional commands to print a waypoint list are:

Menu Commands:   Print,   Waypoint List

Keyboard:        CTRL-W

Printing:  Position Report

--------------------------

After Position updates have been started on a passage the print option for

daily position reports becomes available.  The position report is a summary

report updated with each position udpate.

The daily report is broken down into four sections:

* Position      Latitude, Longitude and time of position.

* Day's Run     Distance gone, average speed and total hours run from

last position update.

* Total Run	Total distance gone, total average speed and total time

of passage (displayed in hours and time format)

* ETA summary	Distance to go, speed required and projected eta

WayPoint for Windows v1.0                                          Page 28

Additional commands to print a daily position report are:

Menu Commands:   Print,   Position Report

Keyboard:        CTRL-P

When opened the Position Report displays the date and time in GMT format as

taken from the waypoint list.  Using the assigned text boxes the report can

be modified to display the LMT format for printing purposes.  The Report

Title and Date Display automatically changes as the date and time text boxes

are modified.

The copies option sets the number of reports to be printed.

Because daily reports are printed in pairs, the choices available are 2, 4

and 6 copies, with 6 copies (3 pairs) printed per page.

The remarks and watch officer text boxes can be modified as needed on a daily

basis.

Printing:  Arrival Report

-------------------------

The Arrival Report is similar to the Position Report in that a summary of

the day's run and total run is generated.  The arrival report is designed to

end the passage with a final report.  To generate an Arrival Report the

passage must first be finalized by entering the arrival time through the

Arrival:  End Passage function.

Additional commands to print an Arrival Report are:

Menu Commands:   Print,   Arrival Report

The time display for the Arrival report is in LMT format.  An option is

available to change the format display for the date.

The copies option sets the number of reports to be printed.  As with the

position reports, arrival reports are printed in pairs.  The choices

available are 2, 4 and 6 copies, with 6 copies (3 pairs) printed per page.

The remarks and watch officer text boxes can be modified as needed.

Printing:  AMVER Messages

-------------------------

AMVER messages are available for printing after departure has been entered.

Of the five message formats specified for the AMVER program only four are

used by WayPoint.  The Sailing Plan message is combined with the Departure

Report message to make a single report.  AMVER is explained in more detail

in Appendix C.

The following message reports are provided by WayPoint:

* Departure Message     Combined with the Sailing Plan, provides the route

information, speeds and ETA's for the projected

passage.

WayPoint for Windows v1.0                                          Page 29

* Position Report       Provides information obtained from the last Position

Update

* Deviation Report      Used to report sailing plan and other changes

* Arrival Report        Reports vessel's arrival at destination port

Menu commands to access the AMVER messages are:

Menu Commands:   Print,   AMVER

The AMVER message is compiled automatically.  The optional X/, V/ and Y/

lines are added as needed.

------------------------------

Chapter 8: Navigation Commands

------------------------------

Sailings:

---------

WayPoint provides three calculators for calculating the Great Circle,

Mercator and DR sailings. These tools are independent of the waypoint list

and therefore may be opened and closed without effecting the waypoint files.

Great Circle & Track Points:

---------------------------

The great circle calculator will calculate the sailing information between

two known positions.  The distance, vertex points, initial and final course,

and adjustable track points are displayed with each calculation.  A option

to print the track points is also available.

To open the Great Circle calculator:

Menu Commands:      Navigate,    Great Circle Sailings

Toolbutton

The Longitude interval list selects the interval between track points.  The

range is from 1 to 10 degrees of longitude.

A print out of the calculations is available by utilizing the Print command

button.

Mercator Sailings:

-----------------

A mercator sailing can be calculated between two known positions by using the

Mercator Sailings function.  The distance final course between the two points

are displayed with each calculation.

To open the Mercator calculator:

Menu Commands:      Navigate,   Mercator Sailings

Toolbutton

WayPoint for Windows v1.0                                          Page 30

Enter the two positions in the appropriate text boxes.  Press the Calculate

button to calculate the mercator distance and course between the two points.

DR Position Calculator:

----------------------

Given a starting position and knowing a distance and initial course to the

second position, a DR point can be calculated for both great circle and

mercator (rhumb line) sailings.

To open the DR calculator:

Menu Commands:      Navigate,    DR Position

Toolbutton

Enter the first position, distance and initial course in the appropriate text

boxes.  Press the Calculate button to display the latitude and longitude of

the second position.

----------------------------

Chapter 9:  Window Functions

----------------------------

Toolbar - On/Off:

----------------

The WayPoint Toolbar is an excellent feature which makes WayPoint for

Windows so easy to use.  However, for users who wish not to use the toolbar,

WayPoint provides an option to toggle the display on and off.

To toggle the Toolbar:

Menu Commands:     Window,  Toolbar

Keyboard:          CTRL-B

View Summary - On/Off:

---------------------

As with the Toolbar option, WayPoint provides a toggle to turn on and off the

Passage Summary information dislayed in the lower half of the waypoint list.

This option is useful when creating new waypoint files and the summary

display is not needed.

To toggle the Passage Summary display:

Menu Commands:   Window  View Summary

Keyboard        CTRL-V

Cascade/Tile/Arrange:

--------------------

Windows commands similar to the Program Manager commands.  These commands

help to arrange how the program windows are displayed within WayPoint.

WayPoint for Windows v1.0                                          Page 31

-----------------------------

Chapter 10 - Program Tutorial

-----------------------------

*****  TUTORIAL IS AVAILABLE TO REGISTERED USERS ONLY ******

-----------------------------------------------------------------------------

APPENDIX A  -  Data Entry Requirements

-----------------------------------------------------------------------------

This section covers the data entry formats recognized in WayPoint for

Windows.  A wide variety of formats are accepted by WayPoint, however certain

rules must still be maintained to avoid error messages from being generated.

The following tables contain the accepted formats which can be used:

Latitude and Longitude Entries

Enter the position's coordinates using one of the following accepted formats.

Hyphens and periods are accepted by WayPoint as separators.  The quadrant

label of N/E/S/W must be entered, a space between the position and label is

optional.

Latitude	Longitude

--------------------------

12-30.0 N	012-30.0 W

2-30N            0-30W

12 S             125E

12.30.5 N         12.30.5E

Time Format Entries:

--------------------

The time format used by WayPoint is the 24 hour clock of 0000 to 2359 hours.

There are two accepted time formats for use in WayPoint, with or without a

colon (:).

12:30	or	1230

Date Format Entries:

-------------------

Date entries may be made in a number of ways.  If the computer's internal

clock is set to the current date then entering the year is optional.

1/2/94	2 Jan 94

1/2	2 Jan

1-2-94	Jan 2 94

Time Zone Entries:

-----------------

The accepted time zone descriptions are:	-12  to  +12

WayPoint for Windows v1.0                                          Page 32

Speed Entries:

-------------

The accepted range and format for speeds is:	1.0  to  50.0 knots

WayPoint can be used to track a series of coordinates for weather systems,

and in turn project a system's position for analysis.  As some weather

systems can often be fast moving, a maximum speed has been established in

WayPoint at 50 knots.

----------------------------------------------------------------------------

APPENDIX B  -  Sample Reports

-----------------------------

***** SAMPLE REPORTS MADE AVAILABLE TO REGISTERED USERS ONLY *****

-----------------------------------------------------------------------------

APPENDIX C  -  AMVER

-----------------------------------------------------------------------------

AMVER, Automated  Mutual-Assistance Vessel Rescue System, is operated by the

United States Coast Guard, Department of Transportation.  The following

information has been obtained from the AMVER User's Manual.

Participants:

------------

Any merchant vessel of  one thousand gross tons or more on a voyage of

greater than 24 hours to anywhere on the globe may participate in the AMVER

program.  International participation is voluntary regardless of the vessel's

or company's flag, country of origin, or destination.

U.S. flag vessels must report and regularly update their voyages to the

AMVER Center.

Information Reported:

--------------------

Information voluntarily provided by  vessel's to AMVER is kept strictly

confidential, and is protected by the Coast Guard.  It is released only for

safety purposes to recognized search-and-rescue authorities or to make

advance notice to the U.S. Coast Guard of arrival in U.S. ports as required

by certain sections of Title 33. Code of Federal Regulations.

What is Reported and When:

-------------------------

Sailing Plans           WayPoint incorporates the sailing plan route within

the Departure Report.

Departure Reports       Sent as soon as practicable upon leaving port.

Position Reports        Within 24 hours of departure, and no less frequently

than every 48 hours until arrival.

Arrival Reports         Sent immediately prior to or upon arrival at the port

of destination.

Position reports may be sent more frequently than the basic schedule, as in

the case of heavy weather or other adverse conditions.

WayPoint for Windows v1.0                                          Page 33

AMVER System Communication Network:

----------------------------------

A worldwide radio station communications network is in effect providing a

number of facilities which accept AMVER reports at no charge to the vessel.

These stations and hours of operation are listed in the AMVER Bulletin

publication, available from the following address:

Commander, Atlantic Area [As]

U.S. Coast Guard

AMVER Center

Governors Island

New York, NY 10004

AMVER and WayPoint:

-------------------

By maintaining regular daily positions on your routes, WayPoint for Windows

will provide the necessary reports as required by AMVER.  The messages can

then be transmitted to the appropriate stations.

-----------------------------------------------------------------------------

APPENDIX D  - Limited Warranty and Disclaimers

-----------------------------------------------------------------------------

The software, WayPoint for Windows, its data and accompanying written

material (including instructions for use) are provided "AS IS" without

warranty of any kind.  Further, ProMar Software does not warrant, guarantee,

or make any presentations regarding the use, or the results of the use, of

the software, data or written materials in terms of correctness, accuracy,

currentness, or otherwise and shall not include or extend to any claim for

or right to recover any other damages, including but not limited to, loss of

profit, data, or use of the software and data, or special, incidental, or

consequential damages or other similar claims, even if ProMar Software has

been specifically advised of the possibility of such damages.  In no event

will ProMar Software liability for any damages to you or any other person

ever exceed the lower of suggested list price or actual price paid for the

license to use the software and data, regardless of any form of claim.  The

entire risk as to the results and performance of the software and data is

assumed by you.

ProMar Software specifically disclaims all other warranties, express or

implied, including but not limited to, any implied warranty of

merchantability or fitness for a particular purpose.  Specifically, ProMar

Software makes no representation or warranty that the software is fit for

any particular purpose

The prudent mariner will not rely solely on this material to make decisions

regarding the safe navigation, but should be used in conjunction with other

aids to make the needed decisions.  The information in these materials has

been obtained from sources and formulae which are believed to be reliable,

but cannot be guaranteed as to its accuracy or completeness, whether supplied

as written materials, magnetically or electronically by any means.

The limited warranty gives you specific legal rights; you may have others

which may vary from state to state.  Some states do not allow the exclusion

of incidental or consequential damages, or the limitation on how long an

implied warranty, so some of the above may not apply to you.

The disclaimer of Warranty and Limited Warranty is governed by the laws of

the state of California.

WayPoint for Windows v1.0                                          Page 34

-----------------------------------------------------------------------------

APPENDIX E  -  Software License Agreement

-----------------------------------------------------------------------------

INSTALLATION OF WAYPOINT FOR WINDOWS ON YOUR COMPUTER SYSTEM IMPLIES

AGREEMENT WITH THE TERMS AND CONDITIONS SET FORTH BELOW.

1.  WayPoint for Windows (hereinafter WayPoint) is distributed as ShareWare.

It is not free, FreeWare, or in the Public Domain.  You may use WayPoint

for a trial period of sixty days, at no cost to you, to determine if it

fits your needs.  If you decide to use WayPoint regularly, you are

expected to register it and pay the applicable registration fee.

2.  Individual copies of the unregistered version of WayPoint may be given

to your friends and associates for the same sixty day free trial period.

You may also upload the unregistered version of WayPoint to the public

section of a public BBS, as long as all required files are enclosed in

accordance with the PACKING.LST file.

3.  You may not modify, adapt, translate, reverse engineer, decompile,

disassemble, or create derivative works based on WayPoint, nor

distribute any modified or disassembled versions of WayPoint.  WayPoint

may not be included with any other product without written permission

from ProMar Software.

4.  As the LICENSEE of a registered copy of WayPoint, you are permitted to

use the software on more than one computer at a time as long as no more

than one of these computers is running WayPoint at the same time. You

may make backup copies of WayPoint as necessary for archival purposes

only.

5.  The registered copy of WayPoint is licensed only to you, the LICENSEE,

and may not be transferred to anyone without the prior consent of ProMar

Software.  Any authorized transferee of the registered WayPoint shall be

bound by the terms and conditions of this Agreement.  In no event may you

transfer, assign, rent, lease, sell or otherwise dispose of the registered

copy, on a temporary or permanent basis except as expressly provided

herein.

6.  This License is effective until terminated.  This License will terminate

automatically without notice from ProMar Software if you fail to comply

with any provision of this License.  Upon termination you shall return

the written materials and original copies of the software, data and

destroy all copies of the written materials and all copies of WayPoint,

including modified copies, if any.

7.  ProMar Software may create, from time to time, update versions of

WayPoint.  ProMar Software will make such updates available to the

Licensee and transferee who have paid the update fee.

8. This Agreement is governed by the laws of the state of California.

WayPoint for Windows v1.0                                          Page 35

-----------------------------------------------------------------------------

APPENDIX F  -  Contacting ProMar Software

-----------------------------------------------------------------------------

Only registered users may receive technical support for WayPoint for Windows,

however comments and 'bug' finds are always appreciated from all users.

ProMar Software may only be reached through the following methods.

Registered users, please include your serial number with all correspondence.

US Postal:

---------

ProMar Software

6161 El Cajon Blvd  #275

San Diego, CA  92115

Telephone:

---------

Fax:    (619) 464-5015

EMail:

------

You can reach ProMar Software by EMail through the following online services:

CompuServe (CIS):       71662,1015

GEnie:       M.Smith106

ProMar Software

All rights reserved

--  Initial Release:   v1.0    February 1994