MICEPT INSTRUMENTS INC.                               MICEPT INSTRUMENTS INC.

Cross Assemblers (Series I)                             Cross Assemblers (Series I)

Version 2.20                                     Version 2.20

USER'S MANUAL                                    USER'S MANUAL

Micept Instruments Inc.

377 Julien St.

Cap De La Madeleine, PQ

Canada  G8T 6W6

DISCLAIMER          DISCLAIMER

Micept  Instruments  Inc.  disclaims all  warranties  as  to this

software  and the  documentation,  whether  express  or  implied,

including,   without  limitation,   any  implied   warranties  of

merchantability   and  of   fitness  for   any  purpose.   Micept

Instruments  Inc. assumes  no liability  for  damages, direct  or

consequential, which may results from the use of this software or

the documentation.

Information in this document is subject to change without notice.

Copyright (C) 1991, 1992 by Micept Instruments Inc.

COPYRIGHT INFORMATION          COPYRIGHT INFORMATION

This version of  the software is  distributed as shareware.  This

version  may  be  freely copied  and  distributed  for evaluation

purposes  as  long  as NO  FEE  IS  CHARGE  FOR  USE, COPYING  OR

DISTRIBUTION (EXCEPT FOR A NOMINAL DISTRIBUTION FEE PROVIDED THAT

IT IS NO MORE THAN $5 TOTAL) and  as long as THE SOFTWARE PACKAGE

IS NOT MODIFIED IN ANY WAY.

USE  OF THIS  SOFTWARE  BEYOND  A  REASONABLE  EVALUATION  PERIOD

REQUIRES REGISTRATION.

The   entire   package,   consisting   of   the   main  programs,

documentation  file and various  utility files are  all copyright

(C) 1991, 1992 by Micept Instruments Inc. All rights reserved.

Intel is a registered trademark of Intel Corporation.

Zilog and Z80 are registered trademarks of Zilog Inc.

MS-DOS is a trademark of Microsoft Corporation.

IBM, PC, XT, AT are trademarks of International Business Machines

Corporation.

All   brand  and  product  names  are  trademarks  or  registered

trademarks of their respective companies.

TABLE OF CONTENTS                                  TABLE OF CONTENTS

1. INTRODUCTION . . . . . . . . . . . . . . . . . . . . . . .   1

2. INSTALLATION . . . . . . . . . . . . . . . . . . . . . . .   1

3. SYSTEM REQUIREMENTS  . . . . . . . . . . . . . . . . . . .   1

4. FILES IN THE PACKAGE . . . . . . . . . . . . . . . . . . .   1

5. FORMAT OF COMMANDS . . . . . . . . . . . . . . . . . . . .   2

6. COMMAND LINE . . . . . . . . . . . . . . . . . . . . . . .   2

6.1 Invocation . . . . . . . . . . . . . . . . . . . . .   2

6.2 Options in more details  . . . . . . . . . . . . . .   3

6.3 Error codes returned to DOS  . . . . . . . . . . . .   5

6.4 Object code file formats . . . . . . . . . . . . . .   5

6.5 Environment variable . . . . . . . . . . . . . . . .   5

7. SOURCE LINE FORMAT . . . . . . . . . . . . . . . . . . . .   5

Label  . . . . . . . . . . . . . . . . . . . . . . . . .   6

Opcode . . . . . . . . . . . . . . . . . . . . . . . . .   6

Operand  . . . . . . . . . . . . . . . . . . . . . . . .   6

Comment  . . . . . . . . . . . . . . . . . . . . . . . .   6

8. EXPRESSIONS IN THE OPERAND FIELD . . . . . . . . . . . . .   7

8.1 Symbols  . . . . . . . . . . . . . . . . . . . . . .   7

8.2 Constants  . . . . . . . . . . . . . . . . . . . . .   7

Numeric constants . . . . . . . . . . . . . . . . .   7

Character constants . . . . . . . . . . . . . . . .   8

The location counter  . . . . . . . . . . . . . . .   8

8.3 Operators  . . . . . . . . . . . . . . . . . . . . .   8

8.4 How expressions are evaluated  . . . . . . . . . . .   9

9. ASSEMBLER DIRECTIVES . . . . . . . . . . . . . . . . . . .   9

ACLIST . . . . . . . . . . . . . . . . . . . . . . . . .   9

DB . . . . . . . . . . . . . . . . . . . . . . . . . . .  10

DW . . . . . . . . . . . . . . . . . . . . . . . . . . .  10

END  . . . . . . . . . . . . . . . . . . . . . . . . . .  10

EQU  . . . . . . . . . . . . . . . . . . . . . . . . . .  10

LIST . . . . . . . . . . . . . . . . . . . . . . . . . .  10

LLEN . . . . . . . . . . . . . . . . . . . . . . . . . .  11

NOACLIST . . . . . . . . . . . . . . . . . . . . . . . .  11

NOLIST . . . . . . . . . . . . . . . . . . . . . . . . .  11

ORG  . . . . . . . . . . . . . . . . . . . . . . . . . .  11

PAGE . . . . . . . . . . . . . . . . . . . . . . . . . .  11

PLEN . . . . . . . . . . . . . . . . . . . . . . . . . .  12

REDEF  . . . . . . . . . . . . . . . . . . . . . . . . .  12

RSB  . . . . . . . . . . . . . . . . . . . . . . . . . .  12

RSW  . . . . . . . . . . . . . . . . . . . . . . . . . .  12

SET  . . . . . . . . . . . . . . . . . . . . . . . . . .  12

TITLE  . . . . . . . . . . . . . . . . . . . . . . . . .  13

Micept Instruments Inc. - Cross Assemblers (Series I)         iii

10. SPECIAL SECTION ON THE MICROPROCESSORS  . . . . . . . . .  14

10.1 Intel 8044  . . . . . . . . . . . . . . . . . . . .  14

10.2 Intel 8048  . . . . . . . . . . . . . . . . . . . .  17

10.3 Intel 8051/52 . . . . . . . . . . . . . . . . . . .  18

10.4 Intel 8080  . . . . . . . . . . . . . . . . . . . .  22

10.5 Intel 8085  . . . . . . . . . . . . . . . . . . . .  23

10.6 Intel 8096  . . . . . . . . . . . . . . . . . . . .  23

10.7 Zilog Z80 . . . . . . . . . . . . . . . . . . . . .  23

11. BUG REPORTING PROCEDURE . . . . . . . . . . . . . . . . .  24

12. UPDATE POLICY . . . . . . . . . . . . . . . . . . . . . .  24

13. REGISTRATION  . . . . . . . . . . . . . . . . . . . . . .  25

APPENDIX A - EXPANDED VERSION . . . . . . . . . . . . . . . .  26

ORDERING INFORMATION  . . . . . . . . . . . . . . . . . . . .  27

ORDER FORM  . . . . . . . . . . . . . . . . . . . . . . . . .  28

INDEX . . . . . . . . . . . . . . . . . . . . . . . . . . . .  29

Micept Instruments Inc. - Cross Assemblers (Series I)           1

1. INTRODUCTION          1. INTRODUCTION

This  software are shareware distributed versions of high quality

Macro  Cross Assemblers  which you get  when you  register. These

versions  however do not support macros, conditional assembly and

included files like their expanded versions do. This software are

basic  two-pass assemblers  for the  Intel  8051/52, 8044,  8048,

8080, 8085, 8096 and the Zilog Z80 microprocessors.

These powerful cross  assemblers for the MS-DOS  operating system

support  the  complete instruction  set  and  a complete  set  of

directives.  They also  support  the  8051/52  and  8044  special

function registers and their assigned bit names. These assemblers

can produce formatted  listing files and object code  in both the

Intel Hex and the Motorola Hex formats.

The supported assembler directives are  the same as the ones used

by  the microprocessor manufacturers, but you can customized them

to your liking using the REDEF directive.

2. INSTALLATION          2. INSTALLATION

Make  a working  copy of  your disk, using  the COPY  or DISKCOPY

utility supplied with MS-DOS. Save the original disk for backup.

No  particular installation is  required except that  you have to

copy the executable files you will need to the disk you are going

to use.

3. SYSTEM REQUIREMENTS          3. SYSTEM REQUIREMENTS

- IBM PC/XT/AT or compatible computer

- DOS 2.0 or later

- 256K Bytes of RAM required (more for bigger symbol table)

4. FILES IN THE PACKAGE          4. FILES IN THE PACKAGE

README#1       The file containing the latest information

ORDER#1.FRM    The file containing the ready to print

registration form

MCASM#1.DOC    This file !

MA44.EXE       The executable file of the 8044 Cross

Assembler

MA48.EXE       The executable file of the 8048 Cross

Assembler

MA51.EXE       The executable file of the 8051/52 Cross

Assembler

Micept Instruments Inc. - Cross Assemblers (Series I)           2

MA80.EXE       The executable file of the 8080 Cross

Assembler

MA85.EXE       The executable file of the 8085 Cross

Assembler

MA96.EXE       The executable file of the 8096 Cross

Assembler

MAZ80.EXE      The executable file of the Z80 Cross

Assembler

5. FORMAT OF COMMANDS          5. FORMAT OF COMMANDS

The following rules are used in this manual:

1.   Words  in  capital  letters are  keywords  and  must be

entered as shown. They  may be entered in  uppercase or

lowercase characters.

2.   Items in square brackets ([]) are optional.

3.   Symbolic fields are enclosed in < >.

4.   An  ellipsis  (...)  indicates  that  an  item  may  be

repeated as many time as you wish.

5.   All punctuation, except the one mentioned  before, must

be included where shown.

6.   xxx is a two or three character string representing the

microprocessor identification.

This string is:

44  : for the Intel 8044 microprocessor

48  : for the Intel 8048 microprocessor

51  : for the Intel 8051/52 microprocessor

80  : for the Intel 8080 microprocessor

85  : for the Intel 8085 microprocessor

96  : for the Intel 8096 microprocessor

Z80 : for the Zilog Z80 microprocessor

6. COMMAND LINE          6. COMMAND LINE

6.1 Invocation

These assemblers can be invoked as follow:

MAxxx [<options>] <sourcefile> [<objfile> [<listfile>]]

Micept Instruments Inc. - Cross Assemblers (Series I)           3

Where  <options>  can  be  one  or  more  of  the  following

separated by spaces:

-l : Produce a listing file

-o : Produce NO object file

-t : Produce NO symbol table in the listing

-m : Produce object in Motorola Hex

-q : Quiet mode

-p<n> : Set listing page length (in lines per page)

-w<n> : Set listing width (in characters per line)

-r<n> : Set object record size (in bytes)

-x : Use extended special function register set for 8052

-c : Produce complete object in listing for long constants

<sourcefile>  : Source file name.  Default extension ".xxx".

<objfile>     : Object file name.  Default extension ".OBJ".

<listfile>    : Listing file name. Default extension ".LST".

If the  assembler is  invoked with an  error in  the command

line, a help screen is displayed on the console.

For <objfile>  and <listfile>, if no name  is specified, the

source file  name with the  default extensions will  be used

and the files will be created in the current directory.

6.2 Options in more details

c -  Complete  object code  in listing  for long  constants.

This is used to see  all object codes generated by long

constants,  like strings and  series of data.  A source

line  may  then  generate more  than  one  line  in the

listing because only a few  object codes can be written

on one line of listing.

l -  Listing  file.  This  option enables  the  output  to a

listing file. The listing produced has page headers and

footers inserted  at appropriate  intervals. Even  when

the option is invoked, the  output to the listing  file

will be controlled by the LIST and NOLIST directives.

m -  Motorola Hex format. This option causes the object file

to be produced in  the Motorola Hex format  rather than

the default Intel Hex format.

o -  Object file disabled. This option suppresses the output

to an object file.

Micept Instruments Inc. - Cross Assemblers (Series I)           4

p -  Page  length. This option sets  the number of lines per

page  in  the listing  file.  When  a listing  file  is

produced, since page headers and  footers are inserted,

a  page length is required. If this option, immediately

followed  by a numeric  constant, is invoked,  the page

length will  be set  to that value.  The value  must be

between 20  and 120  or it will  be ignored.  Even when

this  option is  invoked,  the page  length may  be set

again anywhere in  the file by the PLEN  directive. The

default page length is 66 lines.

q -  Quiet mode. This option turns  off the display of  line

numbers  during  assembly.  Normally  a running  count,

incremented  by  50s,  of the  numbers  of  lines being

assembled is displayed on the console.

r -  Record size  in the object  file. This option  sets the

number  of bytes in each object file record. The number

selected is the number of counted bytes in the selected

format. The number must be between 8 and 255 or it will

be ignored.  If the m  option is invoked, the  r option

must appear  after it in  the command line to  have any

effect. The default record size in Intel Hex is 24. The

default record size in Motorola Hex is 27.

t -  Table of symbols  suppressed in the listing  file. This

option suppresses the output of the symbol table in the

listing  file. Of course if  output to the listing file

is disabled at the end of the listing, the symbol table

will not appear at the end of the listing file anyway.

w -  Width of  the listing file. This option sets the number

of characters  per line  in the  listing file.  If this

option, immediately followed by  a numeric constant, is

invoked, the line length will be set to that value. The

value must be between 60 and 132 or it will be ignored.

Even  when this option is  invoked, the line length may

be  set  again  anywhere  in   the  file  by  the  LLEN

directive. The default line length is 80 characters.

x -  Extended  special function  register  set. This  option

allows  the  use  of  the  extended  special   function

register set and the extended special function register

bit set of the 8052 microprocessor.

Note:     Each option must be preceded  by a dash (-) in the

command line, to distinguish it from a file name.

Micept Instruments Inc. - Cross Assemblers (Series I)           5

6.3 Error codes returned to DOS

Condition                                    Error code

-------------------------------------------------------

Normal completion, without errors            0

Normal completion, with assembly errors      1

Abnormal completion, insufficient memory     2

Abnormal completion, file access error       3

6.4 Object code file formats

Each assembler supports two object file formats:

1. Intel Hex

2. Motorola Hex (S1-S9)

6.5 Environment variable

Each assembler  environment can  be customized by  the use  of an

environment  variable. The  variable, if  set,  will specify  the

default  command  line  options. The  options  specified  in that

variable  will not replace but be added to the options invoked in

the command line.  Options are specified in the same manner as if

they  were invoked  in  the  command line.  The  setting of  this

variable can be  automatically done if the set  command is placed

in the AUTOEXEC.BAT file.

The variable is called MAxxxOPT  and can be set by  the following

command:

set MAxxxOPT=-l -m -r10h

That command would invoked the l,  m and r options each time  the

assembler is invoked.

Note: Any error in this environment variable will be ignored.

7. SOURCE LINE FORMAT          7. SOURCE LINE FORMAT

Lines  in the  source  file  consist of  up  to  four fields,  as

follows:

[Label] [Opcode] [Operand] [Comment]

Each line  of the file may be up  to 256 characters long. An

arbitrary  number of  white  space  (spaces  or  tabs)  must

separate each field.

Micept Instruments Inc. - Cross Assemblers (Series I)           6

Label:

Labels are  symbols and  follow the same  rules. If  a label

starts in  the first column,  it must be terminated  with at

least one white  space (space or tab) or the end of the line

but it may follow the rules of a label starting in any other

column. If a label  starts in any  other column, it must  be

terminated with a colon (:) followed with at least one white

space (space  or tab)  or the  end of  the line. The  ending

colon is never part of the symbol.

A  line  may  consist  of  a  label  alone.  With  a  normal

instruction mnemonic  the assembler  assign a  value to  the

label equal to the current location counter.

Opcode:

The operand field  contains the instruction mnemonic  or the

assembler directive that is to be performed. This field must

not start in the first column.

Operand:

The operand field  may or may not be  required, depending on

the  opcode or directive  being assembled. If  present, this

field will contain a certain number of operand, separated by

commas. There  must not  be any white  space (space  or tab)

between these operand.

Comment:

The  optional comment field is used for annotation purposes.

This field is  ignored by the assembler, but  is included in

the listing.  A semicolon  (;) is used  by the  assembler to

designate  the beginning  of a  comment and  it can  be used

anywhere in  the line, including  in the first column  for a

stand-alone comment.

Micept Instruments Inc. - Cross Assemblers (Series I)           7

8. EXPRESSIONS IN THE OPERAND FIELD          8. EXPRESSIONS IN THE OPERAND FIELD

Expressions are combinations of constants, symbols, operators and

parentheses that are evaluated as an integer value.

8.1 Symbols

Symbols are strings of characters that contain letters (A-Z,

a-z), numbers (0-9), points (.) and underscores (_) but they

must begin with a letter. A symbol may be of any length, but

only the  first 8  characters are  significant  and will  be

listed in the symbol table.  Since all lowercase letter will

be converted  to uppercase,  symbols  which are  capitalized

differently will be the same. A symbol can not be a reserved

word.

examples:      LOOP

loop_2

key.buffer

The reserved words are the one representing:

- the mnemonics and assembler directives.

- the microprocessor's registers.

8.2 Constants

Numeric constants:

Numeric  constants must always  begin with a  decimal digit.

The radix is determine by a letter immediately following the

number according to the following table:

Format         Suffix

---------------------------

Binary         B

Octal          O or Q

Decimal        D or nothing

Hexadecimal    H

examples:      10010001b

7774Q

17O

18d

255

6fh

Micept Instruments Inc. - Cross Assemblers (Series I)           8

0FEH

Character constants:

A  character  constant  consists   of  one  ASCII  character

surrounded by single quote marks (').

Note:     To  put a  single  quote  mark  into  a  character

constant,  two  consecutive single  quotes  may be

used to represent one such character.

examples:      'a'

'*'

''''

The location counter:

The current value of  the location counter can be used in an

expression. It is represented by the dollar character ($).

8.3 Operators

The  operators  allowed  in expressions  are  shown,  in the

following table, in descending order of precedence:

Order  Operator   Type             Description

----------------------------------------------------------

1.     ()         expression       parentheses

2.     -          unary            unary negative

~                           one's complement

3.     &          binary           bitwise AND

!                           bitwise OR

^                           bitwise EXCLUSIVE OR

4.     *          multiplicative   multiplication

/                           division

%                           remainder

5.     +          additive         addition

-                           subtraction

6.     <<         shift            shift left

>>                          shift right

7.     =          relational       equal

<                           less than

>                           greater than

<=                          less than or equal

>=                          greater than or equal

Micept Instruments Inc. - Cross Assemblers (Series I)           9

8.4 How expressions are evaluated

In  an expression  parenthetical  expressions are  evaluated

first,  the  innermost  parentheses before  the  outer ones.

Next, operations  involving higher precedence  operators are

done first. Where several operators have the same precedence

they  are  evaluated from  left  to right  except  for unary

operators  which are  evaluated  from  right  to  left.  All

intermediate values are truncated to a 16-bit integer value.

The result of the expression is also a 16-bit integer value.

9. ASSEMBLER DIRECTIVES          9. ASSEMBLER DIRECTIVES

----------------------------------------------------------

Assembly Control

ORG          Set assembly origin

END          End of source

REDEF        Redefine an assembler directive

----------------------------------------------------------

Symbol Definition

EQU          Equate symbol value

SET          Set symbol value (temporary)

----------------------------------------------------------

Memory Allocation

DB           Define constant byte(s)

DW           Define constant word(s)

RSB          Reserve storage for byte(s)

RSW          Reserve storage for word(s)

----------------------------------------------------------

Listing Control

TITLE        Set title of page(s)

LLEN         Set line length (characters per line)

PLEN         Set page length (lines per page)

PAGE         Advance to top of next page

LIST         List the assembly

NOLIST       Do not list the assembly

ACLIST       List all object codes for long constants

NOACLIST     Do not list all object codes

----------------------------------------------------------

Note:     Some of  these directives  have a  different name  with

each microprocessor, so consult  the special section on

the microprocessor you are using.

ACLIST

Purpose:  This  directive  restores   the  listing  of   the

complete object code for long constants.

Micept Instruments Inc. - Cross Assemblers (Series I)          10

Format:   ACLIST

DB

Purpose:  This  directive allocates  and initializes  one or

more bytes.

Format:   [<label>] DB <expression>[,<expression>...]

Remarks:  <expression> must be  an 8-bit integer value  or a

string surrounded by single quote marks (').

DW

Purpose:  This directive  allocates and  initializes one  or

more words.

Format:   [<label>] DW <expression>[,<expression>...]

Remarks:  <expression> must be a 16-bit integer value.

END

Purpose:  This directive  designates the  end of  the source

file.

Format:   END

Remarks:  No line in the source file will be read after this

directive.

EQU

Purpose:  This directive  assigns the value  of <expression>

to <label>.

Format:   <label> EQU <expression>

Remarks:  The <label> entry can not be redefined.

LIST

Purpose:  This directive restores the output in the listing.

Format:   LIST

Micept Instruments Inc. - Cross Assemblers (Series I)          11

Remarks:  If a  listing is  not being  made, this  directive

will start one.

LLEN

Purpose:  This directive  sets the number  of characters per

line,   in   the   listing,  to   the   value   of

<expression>.

Format:   LLEN <expression>

Remarks:  The  number of characters per line must be between

60 and 132. The default is 80 characters per line.

NOACLIST

Purpose:  This  directive  suppresses  the  listing  of  the

complete object code for long constants.

Format:   NOACLIST

NOLIST

Purpose:  This  directive  suppresses   the  output  in  the

listing.

Format:   NOLIST

ORG

Purpose:  This directive  sets the location  counter to  the

value of <expression>.

Format:   ORG <expression>

Remarks:  <expression>   must  not   involved  any   forward

references.

PAGE

Purpose:  This directive starts a new page in the listing.

Format:   PAGE

Micept Instruments Inc. - Cross Assemblers (Series I)          12

PLEN

Purpose:  This  directive sets the number of lines per page,

in the listing, to the value of <expression>.

Format:   PLEN <expression>

Remarks:  The number  of lines per  page must be  between 20

and  120. The default  is 66 characters  per line.

The first  and the  last 3 lines  of the  page are

left blank. The fourth and fifth line are used for

the date, page number and page title.

REDEF

Purpose:  This directive redefines the <directive> assembler

directive to the <directive2> directive.

Format:   REDEF <directive>,<directive2>

Remarks:  This  allows you  to change the  name of  each and

every directive to satisfy your particular  source

file needs. You  can also use it to  make your own

set of universal directives.

RSB

Purpose:  This  directive reserves  a block  of <expression>

byte(s) for storage.

Format:   [<label>] RSB <expression>

Remarks:  The number of bytes reserved must be between 1 and

32767.

RSW

Purpose:  This directive  reserves a  block of  <expression>

word(s) for storage.

Format:   [<label>] RSW <expression>

Remarks:  The number of words reserved must be between 1 and

16383.

SET

Micept Instruments Inc. - Cross Assemblers (Series I)          13

Purpose:  This directive assigns  the value of  <expression>

to <label>.

Format:   <label> SET <expression>

Remarks:  The <label> entry can be redefined.

TITLE

Purpose:  This directive sets  the title of the  current and

following page(s) to the <text> string.

Format:   TITLE <text>

Remarks:  <text> is truncated to the first 60 characters.

Micept Instruments Inc. - Cross Assemblers (Series I)          14

10. SPECIAL SECTION ON THE MICROPROCESSORS          10. SPECIAL SECTION ON THE MICROPROCESSORS

10.1 Intel 8044

The reserved words in this assembler are:

A, AB, C, DPTR, R0, R1, R2, R3, R4, R5, R6, R7

Special Function Registers:

The 8044 has Special Function Registers. These constants are

words   representing  the   8044  microprocessor   registers

accessible through the use of their addresses in the on-chip

RAM.

These are predefined in this assembler:

ACC, B,  PSW, SP, DPL,  DPH, P0, P1,  P2, P3, IP,  IE, TMOD,

TCON, TH0, TL0,  TH1, TL1, FIFO, TBS, TBL,  TCB, NSNR, STAD,

RFL, RBS, RBL, RCB, SMD, STS, SMD

Special Function Register Bits:

These  Special  Function  Registers  have   bits  which  are

directly accessible. These constants  are words representing

the 8044 microprocessor register bits accessible through the

use of their addresses in the on-chip RAM.

These are predefined in the assembler:

SFR bit   Description

---------------------

P         PSW bit 0

OV        PSW bit 2

RS0       PSW bit 3

RS1       PSW bit 4

F0        PSW bit 5

AC        PSW bit 6

CY        PSW bit 7

EX0       IE bit 0

ET0       IE bit 1

EX1       IE bit 2

ET1       IE bit 3

ES        IE bit 4

ET2       IE bit 5

EA        IE bit 7

PX0       IP bit 0

PT0       IP bit 1

PX1       IP bit 2

PT1       IP bit 3

Micept Instruments Inc. - Cross Assemblers (Series I)          15

PS        IP bit 4

PT2       IP bit 5

IT0       TCON bit 0

IE0       TCON bit 1

IT1       TCON bit 2

IE1       TCON bit 3

TR0       TCON bit 4

TF0       TCON bit 5

TR1       TCON bit 6

TF1       TCON bit 7

ACC.0     Accumulator bit 0

ACC.1     Accumulator bit 1

ACC.2     Accumulator bit 2

ACC.3     Accumulator bit 3

ACC.4     Accumulator bit 4

ACC.5     Accumulator bit 5

ACC.6     Accumulator bit 6

ACC.7     Accumulator bit 7

B.0       Register B bit 0

B.1       Register B bit 1

B.2       Register B bit 2

B.3       Register B bit 3

B.4       Register B bit 4

B.5       Register B bit 5

B.6       Register B bit 6

B.7       Register B bit 7

PSW.0     PSW bit 0

PSW.1     PSW bit 1

PSW.2     PSW bit 2

PSW.3     PSW bit 3

PSW.4     PSW bit 4

PSW.5     PSW bit 5

PSW.6     PSW bit 6

PSW.7     PSW bit 7

P0.0      Port 0 bit 0

P0.1      Port 0 bit 1

P0.2      Port 0 bit 2

P0.3      Port 0 bit 3

P0.4      Port 0 bit 4

P0.5      Port 0 bit 5

P0.6      Port 0 bit 6

P0.7      Port 0 bit 7

P1.0      Port 1 bit 0

P1.1      Port 1 bit 1

P1.2      Port 1 bit 2

P1.3      Port 1 bit 3

P1.4      Port 1 bit 4

P1.5      Port 1 bit 5

P1.6      Port 1 bit 6

P1.7      Port 1 bit 7

Micept Instruments Inc. - Cross Assemblers (Series I)          16

P2.0      Port 2 bit 0

P2.1      Port 2 bit 1

P2.2      Port 2 bit 2

P2.3      Port 2 bit 3

P2.4      Port 2 bit 4

P2.5      Port 2 bit 5

P2.6      Port 2 bit 6

P2.7      Port 2 bit 7

P3.0      Port 3 bit 0

P3.1      Port 3 bit 1

P3.2      Port 3 bit 2

P3.3      Port 3 bit 3

P3.4      Port 3 bit 4

P3.5      Port 3 bit 5

P3.6      Port 3 bit 6

P3.7      Port 3 bit 7

IP.0      IP bit 0

IP.1      IP bit 1

IP.2      IP bit 2

IP.3      IP bit 3

IP.4      IP bit 4

IP.5      IP bit 5

IP.6      IP bit 6

IP.7      IP bit 7

IE.0      IE bit 0

IE.1      IE bit 1

IE.2      IE bit 2

IE.3      IE bit 3

IE.4      IE bit 4

IE.5      IE bit 5

IE.6      IE bit 6

IE.7      IE bit 7

TCON.0    TCON bit 0

TCON.1    TCON bit 1

TCON.2    TCON bit 2

TCON.3    TCON bit 3

TCON.4    TCON bit 4

TCON.5    TCON bit 5

TCON.6    TCON bit 6

TCON.7    TCON bit 7

RBP       STS bit 0

AM        STS bit 1

CPB       STS bit 2

BV        STS bit 3

SI        STS bit 4

RTS       STS bit 5

RE        STS bit 6

TBF       STS bit 7

SER       NSNR bit 0

NR0       NSNR bit 1

Micept Instruments Inc. - Cross Assemblers (Series I)          17

NR1       NSNR bit 2

NR2       NSNR bit 3

SES       NSNR bit 4

NS0       NSNR bit 5

NS1       NSNR bit 6

NS2       NSNR bit 7

STS.0     STS bit 0

STS.1     STS bit 1

STS.2     STS bit 2

STS.3     STS bit 3

STS.4     STS bit 4

STS.5     STS bit 5

STS.6     STS bit 6

STS.7     STS bit 7

NSNR.0    NSNR bit 0

NSNR.1    NSNR bit 1

NSNR.2    NSNR bit 2

NSNR.3    NSNR bit 3

NSNR.4    NSNR bit 4

NSNR.5    NSNR bit 5

NSNR.6    NSNR bit 6

NSNR.7    NSNR bit 7

Note:  Consult  a  data  book  on  the  8044  microprocessor

architecture for more details on the special function

registers.

Assembler directives:

For this microprocessor,  the standard assembler  directives

have been predefined as following:

DATA   instead of EQU

ORG

END

DB

DW

SET

DS     instead of RSB

DSW    instead of RSW

10.2 Intel 8048

The reserved words in this assembler are:

A, R0,  R1, R2, R3, R4,  R5, R6, R7, @R0, @R1,  BUS, P1, P2,

P4, P5, P6, P7,  C, F0, F1, I, TCNTI, CLK,  @A, PSW, T, MB0,

MB1, RB0, RB1, TCNT, CNT

Micept Instruments Inc. - Cross Assemblers (Series I)          18

Assembler directives:

For this  microprocessor, the standard  assembler directives

have been predefined as following:

DATA   instead of EQU

ORG

END

DB

DW

SET

DS     instead of RSB

RSW    is not supported, because it is not necessary

10.3 Intel 8051/52

The reserved words in this assembler are:

A, AB, C, DPTR, R0, R1, R2, R3, R4, R5, R6, R7

Special Function Registers:

The 8051 has Special Function Registers. These constants are

words   representing  the   8051  microprocessor   registers

accessible through the use of their addresses in the on-chip

RAM.

These are predefined in this assembler:

ACC,  B, PSW, SP,  DPL, DPH, P0,  P1, P2, P3,  IP, IE, TMOD,

TCON, TH0, TL0, TH1, TL1, SCON, SBUF, PCON

If  the extended  special function  register  set option  is

invoked  (x  option),  the  8052  microprocessor  additional

special  function  registers  are  also  predefined  in  the

assembler.

These are:

T2CON, TH2, TL2, RCAP2H, RCAP2L

Special Function Register Bits:

These   Special  Function  Registers  have  bits  which  are

directly accessible. These  constants are words representing

the 8051 microprocessor register bits accessible through the

use of their addresses in the on-chip RAM.

Micept Instruments Inc. - Cross Assemblers (Series I)          19

These are predefined in the assembler:

SFR bit   Description

---------------------

P         PSW bit 0

OV        PSW bit 2

RS0       PSW bit 3

RS1       PSW bit 4

F0        PSW bit 5

AC        PSW bit 6

CY        PSW bit 7

EX0       IE bit 0

ET0       IE bit 1

EX1       IE bit 2

ET1       IE bit 3

ES        IE bit 4

ET2       IE bit 5

EA        IE bit 7

PX0       IP bit 0

PT0       IP bit 1

PX1       IP bit 2

PT1       IP bit 3

PS        IP bit 4

PT2       IP bit 5

IT0       TCON bit 0

IE0       TCON bit 1

IT1       TCON bit 2

IE1       TCON bit 3

TR0       TCON bit 4

TF0       TCON bit 5

TR1       TCON bit 6

TF1       TCON bit 7

RI        SCON bit 0

TI        SCON bit 1

RB8       SCON bit 2

TB8       SCON bit 3

REN       SCON bit 4

SM2       SCON bit 5

SM1       SCON bit 6

SM0       SCON bit 7

RXD       Port 3 bit 0

TXD       Port 3 bit 1

INT0      Port 3 bit 2

INT1      Port 3 bit 3

T0        Port 3 bit 4

T1        Port 3 bit 5

WR        Port 3 bit 6

RD        Port 3 bit 7

ACC.0     Accumulator bit 0

ACC.1     Accumulator bit 1

Micept Instruments Inc. - Cross Assemblers (Series I)          20

ACC.2     Accumulator bit 2

ACC.3     Accumulator bit 3

ACC.4     Accumulator bit 4

ACC.5     Accumulator bit 5

ACC.6     Accumulator bit 6

ACC.7     Accumulator bit 7

B.0       Register B bit 0

B.1       Register B bit 1

B.2       Register B bit 2

B.3       Register B bit 3

B.4       Register B bit 4

B.5       Register B bit 5

B.6       Register B bit 6

B.7       Register B bit 7

PSW.0     PSW bit 0

PSW.1     PSW bit 1

PSW.2     PSW bit 2

PSW.3     PSW bit 3

PSW.4     PSW bit 4

PSW.5     PSW bit 5

PSW.6     PSW bit 6

PSW.7     PSW bit 7

P0.0      Port 0 bit 0

P0.1      Port 0 bit 1

P0.2      Port 0 bit 2

P0.3      Port 0 bit 3

P0.4      Port 0 bit 4

P0.5      Port 0 bit 5

P0.6      Port 0 bit 6

P0.7      Port 0 bit 7

P1.0      Port 1 bit 0

P1.1      Port 1 bit 1

P1.2      Port 1 bit 2

P1.3      Port 1 bit 3

P1.4      Port 1 bit 4

P1.5      Port 1 bit 5

P1.6      Port 1 bit 6

P1.7      Port 1 bit 7

P2.0      Port 2 bit 0

P2.1      Port 2 bit 1

P2.2      Port 2 bit 2

P2.3      Port 2 bit 3

P2.4      Port 2 bit 4

P2.5      Port 2 bit 5

P2.6      Port 2 bit 6

P2.7      Port 2 bit 7

P3.0      Port 3 bit 0

P3.1      Port 3 bit 1

P3.2      Port 3 bit 2

P3.3      Port 3 bit 3

Micept Instruments Inc. - Cross Assemblers (Series I)          21

P3.4      Port 3 bit 4

P3.5      Port 3 bit 5

P3.6      Port 3 bit 6

P3.7      Port 3 bit 7

IP.0      IP bit 0

IP.1      IP bit 1

IP.2      IP bit 2

IP.3      IP bit 3

IP.4      IP bit 4

IP.5      IP bit 5

IP.6      IP bit 6

IP.7      IP bit 7

IE.0      IE bit 0

IE.1      IE bit 1

IE.2      IE bit 2

IE.3      IE bit 3

IE.4      IE bit 4

IE.5      IE bit 5

IE.6      IE bit 6

IE.7      IE bit 7

TCON.0    TCON bit 0

TCON.1    TCON bit 1

TCON.2    TCON bit 2

TCON.3    TCON bit 3

TCON.4    TCON bit 4

TCON.5    TCON bit 5

TCON.6    TCON bit 6

TCON.7    TCON bit 7

SCON.0    SCON bit 0

SCON.1    SCON bit 1

SCON.2    SCON bit 2

SCON.3    SCON bit 3

SCON.4    SCON bit 4

SCON.5    SCON bit 5

SCON.6    SCON bit 6

SCON.7    SCON bit 7

If  the extended  special function  register  set option  is

invoked  (x  option),  the  8052  microprocessor  additional

special  function register bits  are also predefined  in the

assembler.

These are:

SFR bit   Description

---------------------

CPRL2     T2CON bit 0

CT2       T2CON bit 1

TR2       T2CON bit 2

EXEN2     T2CON bit 3

Micept Instruments Inc. - Cross Assemblers (Series I)          22

TCLK      T2CON bit 4

RCLK      T2CON bit 5

EXF2      T2CON bit 6

TF2       T2CON bit 7

T2CON.0   T2CON bit 0

T2CON.1   T2CON bit 1

T2CON.2   T2CON bit 2

T2CON.3   T2CON bit 3

T2CON.4   T2CON bit 4

T2CON.5   T2CON bit 5

T2CON.6   T2CON bit 6

T2CON.7   T2CON bit 7

Note:  Consult  a data book on the 8051, 8052 microprocessor

architectures  for   more  details  on   the  special

function registers.

Assembler directives:

For this  microprocessor, the standard  assembler directives

have been predefined as following:

DATA   instead of EQU

ORG

END

DB

DW

SET

DS     instead of RSB

DSW    instead of RSW

10.4 Intel 8080

The reserved words in this assembler are:

A, B, C, D, E, H, L, M, BC, DE, HL, SP

Assembler directives:

For this  microprocessor, the standard  assembler directives

have been predefined as following:

EQU

ORG

END

DB

DW

SET

DS     instead of RSB

RSW    is not supported, because it is not necessary

Micept Instruments Inc. - Cross Assemblers (Series I)          23

10.5 Intel 8085

The reserved words in this assembler are:

A, B, C, D, E, H, L, M, BC, DE, HL, SP

Assembler directives:

For this  microprocessor, the standard  assembler directives

have been predefined as following:

EQU

ORG

END

DB

DW

SET

DS     instead of RSB

RSW    is not supported, because it is not necessary

10.6 Intel 8096

There are no reserved words in this assembler.

Assembler directives:

For this microprocessor,  the standard assembler  directives

have been predefined as following:

EQU

ORG

END

DCB    instead of DB

DCW    instead of DW

SET

DSB    instead of RSB

DSW    instead of RSW

10.7 Zilog Z80

The reserved words in this assembler are:

A,  B, C, D, E, H, L, BC, DE,  HL, SP, IX, IY, M, NC, NZ, Z,

P, PE, PO, AF, I, R

Micept Instruments Inc. - Cross Assemblers (Series I)          24

Assembler directives:

For this  microprocessor, the standard  assembler directives

have been predefined as following:

EQU

ORG

END

DEFB   instead of DB

DEFW   instead of DW

DEFL   instead of SET

DEFS   instead of RSB

RSW    is not supported, because it is not necessary

11. BUG REPORTING PROCEDURE          11. BUG REPORTING PROCEDURE

Although each software is extensively tested prior to release, no

software  is  perfect  and may  contain  a few  bugs.  It  is the

intention  of  Micept  Instruments Inc.  to  correct  any genuine

problem  that will  be considered  as a  problem by  the company,

although  this  is  not  a  warranty that  any  problem  will  be

corrected.

If  you have any comments or suggestions, please send them to the

following address or mail them to us on CompuServe.

If you  think you have found a bug  in this software, please take

the time to report it. Any report will be helpful, but you should

follow these directions:

1.   Provide the name and version of the software.

2.   Provide  a brief description of the problem and provide

a copy or  listing of the  problem source file.  Please

try to isolate the problem  area in the source and only

provide that part of the source file.

3.   Send it to:  Micept Instruments Inc.  CIS:[72130,2513]

377 Julien St.

Cap De La Madeleine, PQ

Canada  G8T 6W6

12. UPDATE POLICY          12. UPDATE POLICY

Micept  Instruments Inc.  will not  notify  users of  new version

releases as improvements are made constantly. If registered users

would  like to  keep  current  they may  send  us their  original

distribution  disk (the one  with our  label) along  with payment

(see order form) to receive the most current version.

Micept Instruments Inc. - Cross Assemblers (Series I)          25

13. REGISTRATION          13. REGISTRATION

The  registration fee, for  each Macro  Cross Assembler  (one per

microprocessor) will  licence one  copy of  the expanded  version

that you  will receive, for  use on any  one computer at  any one

time. The printed manual is optional but recommended and includes

a special section on each of the microprocessors of the series.

Do not  be misled  by the very  low prices  of these  Macro Cross

Assemblers. They are of very high quality, fast and reliable. The

only reason why we can offer them to you at such a  low price, is

because of the distribution method we have chosen, shareware.

Site registrations  are available (see  ordering information  and

order form).

Micept Instruments Inc. - Cross Assemblers (Series I)          26

APPENDIX A - EXPANDED VERSION                            APPENDIX A - EXPANDED VERSION

Registered users get this expanded version of the software.

The version supports:

- up to 16 nested included files

- up to 16 nested conditional assembly

- up to 16 nested macros

- unlimited number of include files

- unlimited number of macros (limited by memory)

The expanded directive set includes the following:

----------------------------------------------------------

Conditional Assembly

IF        Assemble if not equal to zero

IFN       Assemble if equal to zero

IFC       Assemble if string1 equals string2

IFNC      Assemble if string1 not equal to string2

ELSE      Reverse condition

ENDIF     End of conditional assembly

FAIL      Print error message

----------------------------------------------------------

Include files

INCLUDE   Include file

----------------------------------------------------------

Macro Directives

MACRO     Define a macro

ENDM      End of macro definition

EXITM     Skip to the end of a macro definition

----------------------------------------------------------

Listing Control

FLIST     List false conditions

FNOLIST   Do not list false conditions

EXPAND    Expand macros

NOEXPAND  Do not expand macros

----------------------------------------------------------

ORDERING INFORMATION                                 ORDERING INFORMATION

Cat.      Description                                  License

No.                                                    Fee

==============================================================

Series I

--------------------------------------------

MA44      Macro Cross Assembler for the Intel 8044      $10.00

MA48      Macro Cross Assembler for the Intel 8048      $10.00

MA51      Macro Cross Assembler for the Intel 8051/52   $10.00

MA80      Macro Cross Assembler for the Intel 8080      $10.00

MA85      Macro Cross Assembler for the Intel 8085      $10.00

MA96      Macro Cross Assembler for the Intel 8096      $10.00

MAZ80     Macro Cross Assembler for the Zilog Z80       $10.00

SERIES-I  Complete series containing the Macro          $49.00

Cross Assemblers for the Intel 8044,

8048, 8051/52, 8080, 8085, 8096 and

the Zilog Z80 (manual not included)

MANUAL-I  Manual for the Macro Cross Assemblers of      $20.00

the series I and containing a special

section for each of the microprocessors

Series II

--------------------------------------------

MA03      Macro Cross Assembler for the Motorola 6803   $10.00

MA05      Macro Cross Assembler for the Motorola 6805   $10.00

MA09      Macro Cross Assembler for the Motorola 6809   $10.00

MA65      Macro Cross Assembler for the Rockwell 6502   $10.00

MA65C     Macro Cross Assembler for the Rockwell 65C02  $10.00

MA68      Macro Cross Assembler for the Motorola 6800   $10.00

SERIES-II Complete series containing the Macro          $49.00

Cross Assemblers for the Motorola 6800,

6803, 6805, 6809, and the Rockwell 6502

and 65C02 (manual not included)

MANUAL-II Manual for the Macro Cross Assemblers of      $20.00

the series II and containing a special

section for each of the microprocessors

Complete package

--------------------------------------------

MICRO-8   Package containing all the Macro Cross       $119.00

Assemblers of the series I and series II

(both manuals included)

-----------------------------------------------------------------

Note:     The manual is optional but recommended. Only one manual

is required by series.

ORDER FORM                                      ORDER FORM

#92-MC220-001

Qty  Cat. No.  Description              Unit Price     Price

---  --------  -----------------------  ----------     ----------

___  ________  _______________________  $_____.___     $_____.___

___  ________  _______________________  $_____.___     $_____.___

___  ________  _______________________  $_____.___     $_____.___

___  ________  _______________________  $_____.___     $_____.___

___  ________  _______________________  $_____.___     $_____.___

___  ________  _______________________  $_____.___     $_____.___

___  ________  _______________________  $_____.___     $_____.___

___  ________  _______________________  $_____.___     $_____.___

Subtotal  $_____.___

Add $6.00  Shipping + handling          $_____.___

(outside North America Add $10.00)

Subtotal  $_____.___

Canada residents Add GST        $_____.___

Subtotal  $_____.___

Quebec residents Add Provincial Sales Tax    $_____.___

Total  U.S. dollars $_____.___

Payments in Canadian Dollars: Total  Can. dollars $_____.___

Exchange rate ______

Send your check or money order to:      Micept Instruments Inc.

377 Julien St.

Specify disk format:                    Cap De La Madeleine, PQ

___ 5 1/4 or ___ 3 1/2                  Canada  G8T 6W6

Name:     _______________________________________

Company:  _______________________________________

Address:  _______________________________________

_______________________________________

*    Prices are in U.S. dollars. We accept only U.S. and Canadian

funds.  All other will  be refused. Prices  and availability

are subject to change without notice.

INDEX                                        INDEX

ACLIST  . . . . . . . . . .  9     Invocation  . . . . . . . .  2

Assembler directives  . . .  9     Label . . . . . . . . . . .  6

Binary  . . . . . . . . . .  7     Lines per page  . . . . . .  4

Bug reporting procedure . . 24     LIST  . . . . . . . . . . . 10

Character constants . . . .  8     Listing file  . . . . . . 3, 4

Characters per line . . . .  4     LLEN  . . . . . . . . . . . 11

Codes returned to DOS . . .  5     Location counter  . . . . .  8

Command line  . . . . . . .  2     Manual  . . . . . . . . . .  2

Comment . . . . . . . . . .  6     Microprocessors . . . . . . 14

Constants . . . . . . . . .  7          Intel 8044 . . . . . . 14

Copyright information . . .  i          Intel 8048 . . . . . . 17

DB  . . . . . . . . . . . . 10          Intel 8051/52  . . . . 18

Decimal . . . . . . . . . .  7          Intel 8080 . . . . . . 22

Default options . . . . . .  5          Intel 8085 . . . . . . 23

Directives  . . . . . . . .  9          Intel 8096 . . . . . . 23

ACLIST . . . . . . . .  9          Zilog Z80  . . . . . . 23

DB . . . . . . . . . . 10     Motorola Hex  . . . . . . 3, 5

DW . . . . . . . . . . 10     NOACLIST  . . . . . . . . . 11

END  . . . . . . . . . 10     NOLIST  . . . . . . . . . . 11

EQU  . . . . . . . . . 10     Numeric constants . . . . .  7

LIST . . . . . . . . . 10     Object

LLEN . . . . . . . . . 11          code . . . . . . . . .  3

NOACLIST . . . . . . . 11          code format  . . . . .  5

NOLIST . . . . . . . . 11          file . . . . . . . . .  3

ORG  . . . . . . . . . 11          file record size . . .  4

PAGE . . . . . . . . . 11     Octal . . . . . . . . . . .  7

PLEN . . . . . . . . . 12     Opcode  . . . . . . . . . .  6

REDEF  . . . . . . . . 12     Operand . . . . . . . . . .  6

RSB  . . . . . . . . . 12     Operators . . . . . . . . .  8

RSW  . . . . . . . . . 12     Options . . . . . . . . . .  3

SET  . . . . . . . . . 12     Order form  . . . . . . . . 28

TITLE  . . . . . . . . 13     Ordering information  . . . 27

Disclaimer  . . . . . . . .  i     ORG . . . . . . . . . . . . 11

DW  . . . . . . . . . . . . 10     PAGE  . . . . . . . . . . . 11

END . . . . . . . . . . . . 10     Page length . . . . . . . .  4

Environment variable  . . .  5     PLEN  . . . . . . . . . . . 12

EQU . . . . . . . . . . . . 10     Quiet mode  . . . . . . . .  4

Expanded version  . . . . . 26     Record size . . . . . . . .  4

Expressions . . . . . . . 7, 9     REDEF . . . . . . . . . . . 12

Files . . . . . . . . . . .  1     Registration  . . . . . . . 25

Hexadecimal . . . . . . . .  7     Reserved words  . . . . . .  7

Index . . . . . . . . . . . 29     RSB . . . . . . . . . . . . 12

Installation  . . . . . . .  1     RSW . . . . . . . . . . . . 12

Intel 8044  . . . . . . . . 14     SET . . . . . . . . . . . . 12

Intel 8048  . . . . . . . . 17     Site registration . . . . . 25

Intel 8051/52 . . . . . . . 18     Source line . . . . . . . .  5

Intel 8080  . . . . . . . . 22     Special sections  . . . . . 14

Intel 8085  . . . . . . . . 23     Symbol table  . . . . . . .  4

Intel 8096  . . . . . . . . 23     Symbols . . . . . . . . . .  7

Intel Hex . . . . . . . . .  5     System requirements . . . .  1

Introduction  . . . . . . .  1     Table of contents . . . . . ii

Micept Instruments Inc. - Cross Assemblers (Series I)          30

TITLE . . . . . . . . . . . 13

Update policy . . . . . . . 24

Zilog Z80 . . . . . . . . . 23