page 1      Masterful Disassembler - Intel 8086  version 1.00      page 1

1.0) Introduction

The  MD86 program is a powerful utility for examining and disassembling any

executable  program  or  any  series  of  machine  instructions (like a ROM

image). MD86 is designed to run on any IBM PC, XT, or AT or compatible with

at  least 128k of ram memory. Neither a graphics adaptor or a color monitor

is required. A hard disk is desirable but MD86 runs fine (actually a trifle

slower) on floppy based systems.

MD86  was developed with one goal in mind. Produce useable source code from

an executable program file. By useable, we mean that the resulting assembly

instructions  should  be understandable. This necessitates meaningful label

names  and  comments. Normally the disassembly of a large program is a time

consuming, laborious task. MD86 speeds this up as much as possible.

MD86 produces source files that are compatible with the Microsoft assembler

MASM  version  4.00  or reasonably compatible with the IBM assembler. While

this  is  not  the  easiest  assembler  to  use  (in  fact it is down right

difficult),  it  was  chosen  because  it is more "standard" than any other

assembler.   Eventhough   the   instruction   syntax   is  compatible,  the

organization  of  the segments may not be for some programs. After MD86 has

produced a source file, it is not uncommon that an editor is needed to make

some  minor  changes before it can be assembled without error. This will be

especially   true  with  EXE  type  programs  which  have  complex  segment

structures.

1.1) What MD86 Looks Like

MD86's  unique  video  display works very much like a full screen editor;

allowing  movement  within  the  disassembled source file with single key

ease.  Most  of  the  difficulties associated with other disassemblers is

gone.

When  executed,  MD86 presents the user with a full screen of information

that looks very similar to the printed output from an assembler. Figure I

shows  a  typical  screen  from  a  freshly disassembled program. This is

actually the file COMMAND.COM for PCDOS v3.1.

The  bold  line  towards  the  top of the display is the active line. The

cursor  (shown  as  an underline "_") is at the start of the label field.

Both the label field and the comment field may be edited.

Note how the display does not seem cluttered. The label field only has an

entry  if  the  address  is referenced. The comments shown here have been

automatically  inserted  by MD86  These help you remember the less common

instructions and MSDOS function calls.

page 2      Masterful Disassembler - Intel 8086  version 1.00      page 2

Figure I, Typical Display Of Freshly Disassembled Program

05DB:51           L05DBH   PUSH  CX            ;

05DC:1E                    PUSH  DS            ;

05DD:07                    POP   ES            ;

05DE:C536610A     _        LDS   SI,[L0A61H]   ;Load DS:reg with 32b pointr

05E2:57                    PUSH  DI            ;

05E3:BF5A08                MOV   DI,#L085AH    ;

05E6:B90B00                MOV   CX,#L000BH    ;

05E9:FC                    CLD                 ;Set forward direction for

05EB:F3A4                  REPZ MOVSB          ;Move byt. (SI)+- to (DX)+-

05EC:5F                    POP   DI            ;

05ED:06                    PUSH  ES            ;

05EE:1F                    POP   DS            ;

05EF:59                    POP   CX            ;

05F0:BA4708                MOV   DX,#L0847H    ;

05F3:B409CD21              MSDOS _OUTSTR       ;Display string at (DX)

05F7:BA9E08       L05F7H   MOV   DX,#L089EH    ;

05FA:E8A300                CALL  L06A0H        ;

05FD:F606D30AFF            TEST  [L0AD3H],0FFH ;

0602:7404                  JZ    SHORT L0683H  ;

0604:B403                  MOV   AH,#3         ;

0606:EB7B                  JMP   L0683H        ;

0608:B8010C       L0608H   MOV   AX,#L0C01H    ;Flush buffer, read keyboard

060B:CD21                  MSDOS               ;

060D:E88D00                CALL  L069DH        ;

CS:: Labels= 185/ 8%, Types=   0/  0%,   0 cmnts  No Edit 10/ 2/87  1:20:35

A  note  to  programmers familiar with the Microsoft assembler MASM. MD86

creates   compatible   data  files,  but  the  screen  display  has  been

simplified.  In  particular,  the  word  OFFSET  (as required by MASM) is

replaced  with the pound sign ("#") and all WORD PTR and BYTE PTR phrases

have  been  removed.  Generated  labels  are  not shown with an appending

colon.  When  a  source  file  is  generated,  the  source  code  will be

compatible.

The  line  at the bottom contains status information. This tells you that

the  code  being  viewed  is in the code segment, 185 address labels have

been  identified,  no  data  types  have been defined and no user entered

comment  records  exist. In this case we are not as yet editing any field

thus  "No  Edit"  is  displayed.  If  we  were,  then  either "INSERT" or

"REPLACE"  would  show  indicating  how characters are being added to the

field. At the far right corner, the current time and date are as shown.

The  comment  field  may extend past the right edge of the screen and the

active  line  scrolls horizontally as necessary to keep the cursor within

view.

The  function  keys  are  used to control MD86. A window "pops up" in the

upper  left  corner  for instructions. Inadvertently entered commands may

generally  be  aborted  by  a null response (ie, only pressing the RETURN

page 3      Masterful Disassembler - Intel 8086  version 1.00      page 3

key) to one of the questions.

2.0) Using MD86

To  disassemble  a  file it must first exist in the current directory. MD86

may  be  placed in any other directory as long as the PATH command includes

that  directory.  The  companion  file, MD86.CMT is only used to supply the

automatic  comments  and  is only needed when a program is disassembled for

the  first  time.  If  this  is  not  found,  MD86  will turn off automatic

commenting.  If  this  is not acceptable, then QUIT (see Section 2.2), move

MD86.CMT to the current directory and begin again.

To disassemble the program COMMAND.COM, use the following command.

C>MD86 COMMAND.COM

If MD86 cannot locate the associated data files, then MD86 will create them

(you  will  be asked for confirmation first just in case you misspelled the

program  name). MD86 will automatically determine the extent of the program

and  put the cursor on the first address of the program. Note that COM type

files  start at 100 (hex) and EXE type files start at 0000. See reference 1

for  a discussion of the dissection of EXE type files. The Alter Parameters

command may be used to override the choices made by MD86.

MD86  creates two data files when it disassembles a program. These have the

same  name  with  the  extensions  of  .001  and  .002 (ie, COMMAND.001 and

COMMAND.002). The first file contains the symbol table and other parameters

and the second file contains the comment records. If neither of these files

are  present,  then  MD86  assumes  this is a disassembly of a file for the

first  time. If they are both present (and readable) then MD86 will pick up

right  where  you left off. If only one of these files is present or one is

unreadable,  then  MD86  issues  an  error message and terminates. Refer to

Section 7 for a discussion of error causes and cures.

During  the  disassembly  process,  there are three groups of commands that

MD86  will  recognize.  Commands that require additional input will cause a

window  to  pop  up  in the upper left corner of the display. User dialogue

occurs within this window.

The  three  command  groups  consist of 1) editing commands, 2) non-editing

commands, and 3) general commands. The editing and non-editing commands are

mutually  exclusive. When editing, the non-editing commands are not allowed

and visa-versa. General commands are always valid.

MD86  will  "beep"  when an invalid command character is entered. Note that

some  keys  generate  more than one character and while the first character  may  be invalid, the others may not. So when you here the beep, examine the

characters  around  the  cursor  to  be  sure no extraneous characters were

inserted.

page 4      Masterful Disassembler - Intel 8086  version 1.00      page 4

2.1) General Command Keys

The  general  commands  can  be  typed  at  any time. They will always be

recognized.

o Left-Arrow

This  will  move  the cursor one position left within the current field

(either the label field or the comment field). It the cursor is already

at the beginning column, then a "beep" will be heard.

o Right-Arrow

This  will  move the cursor one position right within the current field

(either the label field or the comment field). Note that the fields are

always  filled  with blanks on the right. If the cursor is already past

the right hand column, then a "beep" is heard.

o Insert

This  will change the way editing character keys are entered. They will

either   replace   existing  characters  or  insert  in  front  of  the

characters.  In  editing mode, either INSERT or REPLACE is displayed in

the bottom status line.

2.2) Editing Command Keys

The  label  and  comment fields can be edited by moving the cursor to the

desired  location  and  just  typing;  similar  to a word processor. This

allows  label names and or comments to be associated with an address. The

current  line  may  be  in  the  code  segment  or data segment (EXE type

programs  only).  Once editing has begun, then only the ESCAPE key or the

RETURN key will revert to non-editing command mode.

When  a  temporary  label  field  is  edited, it is initially blanked out

eliminating  the  "L1234H" that was present. If the cursor was not at the

first  column  of  the label, then leading blanks will exist there. Since

labels must begin with a letter or underscore, this will be rejected when

a RETURN key is pressed.

Editing  the  automatic  comments causes MD86 to first ask if the comment

field should be blanked out or not. After this question is answered, your

key is processed.

page 5      Masterful Disassembler - Intel 8086  version 1.00      page 5

o Letters, Numbers, and Symbols

These  characters  are  entered  into the field. If the current mode is

INSERT  then  the  characters  to  the  right  are moved (the rightmost

character  is  lost)  to  make room. Otherwise, in REPLACE mode the new

character overwrites the current cursor character. Note that within the

label  field  only  the  characters  A-Z, a-z, 0-9, $, and _ are valid.

Other  characters  cause  a "beep" and are ignored. Note that a further

restriction  that labels not begin with a number is not checked until a

RETURN key is pressed.

o Escape

This  key will cancel any editing on the current field and its original

contents will be restored. This effectively returns to non-editing mode

without saving any changes to the current field.

o Return

Use  this  key  to tell MD86 that the editing changes you have made are

correct  and  should  be  remembered. Note that this is not the same as

saving the data as it is not actually written to the data files yet. If

the  field  contents are valid, then the cursor will be returned to the

starting   column  of  the  current  field  and  the  mode  is  set  to

non-editing.

o Backspace

If  the  cursor  is  not  at the left edge already, this will erase the

character  immediately  to the left of the cursor. The remainder of the

line  will  be  shifted  left  and  the  rightmost column will be blank

filled. If at the left, then this just "beeps".

o Delete

This  will  erase  the character immediately under the cursor and cause

the remainder of the line to be shifted left. The rightmost column will

be blank filled.

o End

This moves the cursor to the last column of the field.

o Home

This moves the cursor to the leftmost column of the field.

page 6      Masterful Disassembler - Intel 8086  version 1.00      page 6

2.3) Non-editing Command Keys

A  good  portion  of  time spent disassembling a program is spent rooming

around  various  areas  and other non-editing type functions. The simpler

cursor movement functions use a single key stroke for this work while the

more involved commands use the function keys and pop up windows.

The cursor movement keys are as follows.

o Down-Arrow

This  moves  the  cursor down one line and to the beginning of the same

field. You cannot move beyond the end of the current segment.

o Up-Arrow

Move  the  cursor  up  one line to the beginning of the same field. You

cannot move past the beginning of the current segment.

o Home

If  the  cursor  is  right of the leftmost column of the comment field,

then  it  is  moved  to  the  start of the comment field. Otherwise the

cursor  moves  to  the  first column in the label field within the same

line.

o End

This  moves  the cursor to the beginning of the comment field if it was

within  the  label  field. Otherwise the cursor moves to the end of the

comment field.

o Page-Up

This moves the cursor up approximately a full page. This may be more or

less  than  24  lines as this assumes there are three bytes per line on

the average. Note that no attempt is made to locate the beginning of an

instruction.  It  is  probable  that  the  first  line  or  so  will be

disassembled incorrectly. The cursor will be positioned at the start of

the label field in the top line.

o Page-Down

This  moves  the  cursor  to the top of the next page. The line that is

currently  at  the  bottom  of the screen will be at the top after this

command. The cursor will be positioned at the start of the label field.

page 7      Masterful Disassembler - Intel 8086  version 1.00      page 7

The  following  commands  utilize  the  function  keys on the PC keyboard

either  alone  or  in combination with the shift key. Remember these only

function in non-editing mode.

o F1 - Help Command

This  displays a one screen summary of the function keys. Press any key

to refresh the screen.

o Shift-F1 - Alter System Parameters

This  command  is  use  to  make  changes  (if possible) to the default

parameters.  For  COM  files, the beginning and ending addresses can be

modified. EXE files however, have ranges for the data and code segments

that are defined in the header. These cannot be changed.

For COM files, the following parameters can be changed.

o The  Start  Address. Normally this is 100 hex for COM files and 0000

hex  for EXE files. But for special work, like ROM disassembly, this

may be set to something else.

o The  End  Address. MD86 sets this to the physical end of the program

or  FFFE  hex  if  this is more than 64k. If you find that a smaller

value  is  correct, then change it here. This will prevent MD86 from

accessing garbage areas and contaminating the label table.

The  following  parameters  effect  how  MD86 displays the disassembled

lines. These are changeable at all times.

o Translate  MSDOS  Functions. MD86 normally tries to translate common

MSDOS functions into a pseudo instruction that has more meaning when

trying to understand the code. However, if you don't want this done,

then it can be disabled here.

o Enable  Automatic Comments. When MD86 finds certain instructions, it

tries  to  add  a comment line explaining the instruction in more or

less English. If you don't want to see these comments, then they can

be eliminated.

o F2 - Goto a Specified Address

This  allows  a quick jump to any valid address to begin disassembly. A

null  response  (only  the  RETURN  key pressed) causes MD86 to try and

return  to  the  location  of  the  last Goto command. A stack with the

page 8      Masterful Disassembler - Intel 8086  version 1.00      page 8

previous  16  locations  is maintained. This is handy in jumping to one

location and then returning without having to remember where you were.

Valid  destinations  are  anywhere  within the data segment or the code

segment.  Note  that  for  a COM type file, these are assumed to be the

same.

o Shift-F2 - Follow That Instruction

If  the  current  line contains a direct jump or call instruction, this

command will do an automatic Goto to the destination address. This does

not  apply  to  intra-segment  calls  or jumps or any indirect calls or

jumps.

MD86  saves  the current address on its internal stack so that a return

can  be made via the F2 command. With this you can conveniently examine

a subroutine and then continue from where you left off.

o F3 - Set Data Type

When  MD86  first  looks  at  a program, it thinks that all of the data

segment  is  made up of 8-bit data bytes and all of the code segment is

machine  instructions.  This more than likely is not 100% correct. When

disassembling a portion of the program, you may notice that the present

interpretation  does not make sense. Some other data type is necessary.

MD86 can recognize one of four data types. These are instructions (type

#0), 8-bit binary data (type #1), 8-bit ascii characters (type #2), and

16-bit addresses (type #3). See Section 2.6 for more details.

This  command  allows  any range of the program to be set to a specific

type.  You  will  be  asked  for  the  data type and the first and last

addresses. Addresses must be in the same segment of course.

The  internal type table has room for 512 entries. This is the total of

the  data and code segment types. The current total is displayed on the

bottom status line along with a percent used figure.

o F4 - Set Data Type for Unspecified Range

Often  the  extent of a different data type is not known. What is known

is the initial address and a suspected data type. This command uses the

current  line  as  the beginning address and will request the suspected

data  type (0 to 3). Then MD86 temporarily considers all data following

the  cursor  to  be this type. You would move the cursor down until you

reach an address that is not of this type and press F4 again to fix the

range  for the specified type. In this special mode, only one data item

per line is displayed.

page 9      Masterful Disassembler - Intel 8086  version 1.00      page 9

o F5 - Write Source to Disk

MD86 would be of limited use if you could not generate a disk file with

the  source  code.  Use this command and specify the file name and MD86

will  write out the data. If an extension of PRN is used, then the file

will have the address and binary code along with each instruction line.

This is the way the screen appears.

Before this file can be used by an assembler, some hand editing will be

required. Segments may have to be specified differently than MD86.

o F6 - Scan Code Segment

MD86  builds  the  label  table  as code is disassembled. At times, the

disassembly  is  not  correct  and  erroneous address references may be

entered into the label table. This function cleans this up.

When all of the code segment has been given the correct data type, then

this function should be used to properly build the label table. It will

remove  any  temporary  labels  and begin disassembling the entire code

segment.  When  this has completed, the label table will be correct and

erroneous references will be removed.

o F7 - Dump Program in Hexadecimal and ASCII

It  is  difficult to determine the location of data areas and character

strings  by  just  looking at a page of disassembled instructions. This

function  will  begin  in  the data segment (for EXE programs) and then

dump  the  code  segment.  The  data  is displayed 16 bytes per line in

hexadecimal and also in ascii (if possible). Pressing any key will halt

the  display so you can inspect the data and maybe write down addresses

of  obvious  data  areas.  Pressing  any key other than the ESCAPE will

continue the display. Press the ESCAPE key to end this segment and dump

the  next  segment  or  return  to  where  you  when  this  command was

initiated.

o F8 - Set Label Name

If  it is desired to associate a name with a particular address you can

either move the cursor to that address (if possible) and edit the label

field or use this command to set the name without having to move there.

Enter  any  valid  label  name and address to be set. A null label name

will  delete  a label from the tables. In valid names cause a "beep" to

be heard and is ignored. Valid names start with a non-digit and have no

imbedded spaces.

The  label  table  is  limited  to  2048  label  names  which should be      satisfactory for any reasonable size program.

page 10      Masterful Disassembler - Intel 8086  version 1.00      page 10

o F9 - Search for Address Reference

This  command  will allow any address to be searched for. Use this when

it  is  desired to find out how (or if) a particular area is referenced

within  the  code segment. The initial address to start is requested. A

null response causes the search to begin at the start address. You will

see  the  program  disassembled on the screen and it will stop when the

specified  address  is  referenced. During the search, press any key to

abort.

Note  that  the  search  is  limited  to the code segment. However, the

particular  address  may be in any other segment. For example to search

for  address 1234 within the extra segment, enter ES:1234 as the search

string.

o Shift-F9 - Search for Next Reference

Once  function  F9  has  been  used  to find the first occurrence of an

address,  use  this  command  to  locate the next. As with function F9,

press any key to abort the search.

o F10 - Save and/or Exit

Use  this command to save your current data tables (often!) and exit or

quit MD86  You are given the option to save the data or not and to exit

to MSDOS or not.

If  you wish to quit without saving any of the work you have done, then

respond No to saving the data and Yes to exiting.

2.4) Label Name Specification

MD86  allows,  even  encourages,  you to associate a label name with each

referenced  address.  Names are far more understandable than numbers. The

label  field within the display is either blank (the address has not been

referenced),  contains  a temporary label (the form is LnnnnH for address

nnnn),  or  contains  a user defined name. Label names can be up to eight

characters  long and may contain letters, digits, the dollar sign "$", or

the underscore "_" characters. Labels may not begin with a digit however.

Label  names  may  contain  upper  and lower case letters and the case is

maintained.  However, when searching for a name, MD86 ignores differences

in  case. Thus the name "HelpMsg" is perfectly valid and will appear this

way  in  the  output  file.  You could also jump to address "HELPMSG" and

"HelpMsg" would be found.

Upper  and  lower  case  letters make reading names easier, but you don't    have to remember the exact form to reference the name.

page 11      Masterful Disassembler - Intel 8086  version 1.00      page 11

2.5) Specification of Addresses

When  MD86  requests an address (like the destination of a Jump command),

the form the address must be entered as follows.

Address ?{ss:}nnnn

The  brackets  indicate  optional  qualifiers. If the address is within a

segment  other  than  the  current segment, then the segment name must be

included.  The  "ss:"  in  the line above is the segment name and it must

then  be either "CS:", "DS:", "ES:", or "SS:". The case of the letters is

not  important, but the segment name must precede the address (or offset)

portion.

If  the  actual address within the segment is entered as a number then it

must be in hexadecimal. In place of a number, a label name could be used.

This name must be resolvable within the segment.

For example, the following are valid addresses.

Address ?100

Address ?ds:HelpMsg

The  label table is stored internally and has room for 2048 entries. This

is  generally  enough to disassemble a 10,000 to 15,000 line program. For

larger  programs  it  is  recommended  that  they be divided into smaller

sections if at all possible.

2.6) Data Type Specification

MD86  initially  thinks the entire code segment contains instructions and

the data segment (for EXE type files) contains 8-bit binary data. This is

a  good  place  to start but there will be other data types mixed in with

these. Functions F3 and F4 can be used to tell MD86 to assume a different

data  type  for  a  specified address range. The types are specified by a

numeric code number and the ones recognized are:

0 - Machine instructions.

1 - 8-bit binary data.

2 - 8-bit ASCII character data.

3 - 16-bit address data.

1

page 12      Masterful Disassembler - Intel 8086  version 1.00      page 12

When  using function F3, MD86 must be told the first address of the newly

defined  type  and  the  last address with this type. For data types that

occupy  more  than  one  byte (type 0 or 3), the last address must be the

address  at  the  end  of  the  field  not the start. Thus if address 100

contains a single 16 bit address, then MD86 is given the first address as

100 and the last address as 101 (not 102 as you might think).

Function  F4 works a little differently. The start of the current line is

taken  as  the first address when this is initiated. When this is pressed

again,  then  the  start  of  the  now  current  line (if below the first

address)  is  assumed  to be just passed the type being defined. In other

words,  if the address range 120 to 140 is being defined as type 2 (ascii

character  data),  then the current line should be at address 120 when F4

is  pressed the first time and then moved to address 141 when pressed the

second time.

A  code  type  table  is  maintained internally by MD86 that contains the

beginning  and  ending addresses (with segment of course) and the type of

data  this  address range contains. Instructions are the default type and

(to  save  memory  space)  are not actually stored in the table. There is

room for 512 entries which should be plenty for most normal applications.

2.7) Output Source File Format

MD86  produces  a  standard  ASCII  text  file  as output. This should be

suitable  as  input to most any assembler and editor. Note that MD86 does

not  insert  tab  characters and thus the lines will contain many blanks.

This  causes the files to be quite large. The judicious insertion of tabs

would shrink the file size significantly.

When  MD86  disassembles  a  program,  it  remembers  how  addresses  are

referenced.  As a convenience, the output file will have separating lines

just  in front of all subroutines. That is, those addresses that were the

target of near call instructions.

3.0) The Inner Details of MD86

In  the  next  few  sections  we  will  describe  in  more  detail how MD86

functions.  It is not necessary that you remember or understand all of this

material, but when questions arise this will make a handy reference.

Many  choices  made during the construction of MD86 were ones of programmer

preference.  There are several ways to tackle the many problems encountered

in  creating  a  disassembler.  Which  one is better? If the choice was not  obvious, then personal preference would be the deciding factor.

MD86  was  written in TURBO Pascal and owes a lot of its speed to this fine

compiler.  Some  of the limitations of this compiler necessitated tradeoffs

in  the  design  of  MD86. In particular, to avoid the use of overlays, the

program  was  limited  to a 61k code segment. Not all of the "nice" options

could be included.

page 13      Masterful Disassembler - Intel 8086  version 1.00      page 13

3.1) Moving the Cursor Upward

When  MD86  displays a full screen of disassembled lines it remembers the

exact  starting  address  for  each  line. Thus to move upward within the

display  screen,  MD86 only has to display the new current line in bright

characters  again.  However,  when  the  cursor  moves off the top of the

screen  (either  by an Up Arrow or Up Page command), MD86 has a difficult

task  in  determining the starting address for the instruction. There are

times when more than one legitimate starting address is located. For this

situation, MD86 chooses the longest instruction. This may not be correct.

The  problem  comes  in  when  MD86  has moved up more than one time (you

pressed  Up  Arrow  more  than once when at the top of the screen) and it

finally  comes to a point where it cannot find any legitimate instruction

to  disassemble.  You here a "beep" and MD86 only backs up one byte. This

tells  you that the screen is not displaying correct instructions. One or

more of those at the top (where MD86 backed up) is not correct.

If  the  screen  is  not  correct,  then  how do you make it correct? The

easiest  way  is  to  back  up  a full page (Up Page command) and then go

forward  a  full  page  (Down  Page command). More than likely, MD86 will

correctly  synchronize  somewhere  on  the  screen  when  you  backed  up

(probably at a labeled instruction) and everything from there on downward

will be correct.

One  point  of  interest  will  surely  pop up. When a NOP instruction is

disassembled,  it  is  flaged  as  "questionable"  unless  the previously

disassembled instruction was a short JMP. However, if the cursor is moved

upward  to  a NOP instruction, MD86 marks this line incorrectly since the

instruction   disassembled   prior  to  this  was  actually  a  following

instruction.

A word of caution. When MD86 incorrectly disassembles a line for whatever

reason, erroneous address references may be added to the label pool. This

is  where  the  Scan  command  (F6 key) comes in. It erases all temporary

labels  (ones  that  have  not  been  given  names  by  you)  and  begins

disassembling  the  whole  program  from  the starting address. This will

correctly build the temporary label pool.

3.2) Questionable Instructions

When  MD86 disassembles an instruction line, it will check to see whether

or  not  this instruction makes sense. If it does not, a flag ("?" to the

left  of  the  label  field)  is  set  but  it  is disassembled anyway if

possible.  Valid  instructions  are considered "questionable" if they are

very rarely used or are "meaningless".

In  the  rarely  used catagory are the instructions "LOCK", "ESC", "INT",    "XLAT",  "WAIT",  "HLT", and far returns within a "COM" file. Further any

instructions  made  up of the exact same bytes (eg "ADD [BX+SI],AL" which

is two bytes of 00) is considered questionable.

page 14      Masterful Disassembler - Intel 8086  version 1.00      page 14

Meaningless instructions are "NOP" and "MOV destination,source" where the

source  and  destinations  are  the  same.  Note  that a "NOP" is allowed

following a two byte forward jump instruction.

Depending  on  the type of program being disassembled, there may be a few

or  a  lot of such "questionable" instructions that are actually supposed

to be there. Thus this "questionable" instruction flag is just a guide to

help you locate imbedded data areas.

3.3) Instructions prefix bytes

Like  the Intel 8086 processor, MD recognizes certain prefix bytes. These

are  the  segment  override instructions and the repeat instructions. The

bus  lock  instruction is considered separate as this is very rarely used

(it  is also flaged as "questionable"). When moving the cursor around, it

is  possible miss the prefix byte. This will occur most often when moving

upward.  MD  only  looks  at the previous six bytes to determine where an

instruction starts. If the seventh byte were a prefix byte, this would be

missed.  When  a  jump  is made (Function F2) MD does not check to see if

this is in the middle of an instruction. Here too, a prefix byte could be

missed.

The  effect  of  missing  a prefix byte could cause label addresses to be

associated  with  the wrong segment. The scan option (Function F6) can be

used to clean up this type of misinformation.

While  this  is  not  the  same  as  a  prefix  byte, MD86 checks for NOP

instructions  that  are preceeded by a short forward jump instruction. If

this  is  not  the  case,  then the NOP is marked as "questionable". This

logic  fails  when  the NOP is in the top line of the screen. There is no

preceeding instruction to check and the NOP is marked "questionable" even

though  it  may  be  perfectly valid. When moving the cursor upward, MD86

will  incorrectly  mark  a NOP as questionable. The logic only works when

moving the cursor downward.

3.4) Segment Handling

MD86  recognizes  references  to  the  four  segments  of  the Intel 8086

processor.  It  keeps  four  separate  tables for the labels within these

segments.  An  exception  is  with  COM  type  files. For these, the data

segment  and  the  code  segment are assumed to be the same. Data segment

references  are  forced  into  the  code segment space. This is generally

correct,  but  it  is  possible  that the program creates a separate data

segment.  In  this case the labels generated by MD86 will be put into the

code  segment  when they belong in the data segment. Not much can be done

about  this until after a source file has been produced. Use an editor to

fix these things.

page 15      Masterful Disassembler - Intel 8086  version 1.00      page 15

3.5) Known Compatibility Problems With MASM

MD86  was  designed  to  produce  source  files  to be used with the MASM

assembler  from  Microsoft.  This is the most common assembler and as you

probably know it is not the easiest assembler to use. MASM tries to guard

you  against  yourself.  When  variables  are defined as bytes, then MASM

checks  to  be  sure they are referenced as such. At times this is handy.

But  mostly  it is an annoyance. When disassembling a program it is often

times difficult to determine how labels are referenced. At the very least

it  would  consume  lots  of  time  which  would be better spent on other

aspects  of the code. To prevent MASM from generating numerous errors due

to  seemingly  inconsistent references, MD86 inserts WORD PTR or BYTE PTR

to  force  MASM into accepting these references. This results in the over

use of these override phrases (and a larger than necessary source file).

When MD86 notices a reference to an item within the data or code segments

that  is  outside of the limits of the actual program file, it inserts an

EQU  statement  to equate the label with its value. MD86 is thus assuming

the  the  reference  is  to  a constant value and not a variable address.

Under  some  assemblers  there is no difference between a constant and an

address  (or  offset). But MASM does make the distinction and flags as an

error  an  inconsistent reference. The following error message is typical

of this occurrence.

CMP.ASM(176) : error 56: No immediate mode

This  is MASM's way of telling you that line 176 in file CMP.ASM contains

a  reference  to a variable where the label has been defined as a number.

The  solution is to modify the source code file and change the definition

of this label from an EQU into a DB or DW. Thus the line

HELPTXT  EQU   00050H

should be change into the following lines.

ORG   00050H

HELPTXT: DB    0

You  won't  need  the  ORG statement in front of every definition line as

long as the data type is consistent.

EXE  programs  pose other problems that must be dealt with. MD86 does not

resolve  FAR  jumps and calls. You will have to determine the destination    address  and  equate  this  to a label. The absolute address is shown but

MD86 cannot find out where this ends up.

1

page 16      Masterful Disassembler - Intel 8086  version 1.00      page 16

Returns  from  FAR  procedures  are  flaged  by MD86, but the instruction

inserted is RET. MASM will determine if this is a NEAR or FAR return from

the  definition  of  the  procedure.  Thus,  you  will have to define the

procedure  containing  the  FAR  return as a FAR procedure. If you don't,

then  MASM will assume a NEAR return is to be generated. No error message

is displayed but this will not execute the same as a FAR return.

Another  problem  with  EXE  type  files concerns segment definition. The

output  file generated by MD86 should assemble and even link okay, but it

probably  won't execute correctly without being changed. You will need to

look  into  adding  ASSUME  and  GROUP  statements in the various segment

blocks.

EXE  files  can  be  constructed  in  many,  many ways. It will take some

persistence to resolve these differences.

4.0) A Short Course In Generating Source Code

The  process  of disassembling a program and recreating source code is more

art  than science. The more practice you have the easier this becomes. MD86

(as  well as other programs) have been written to make this process as easy

as  possible. As "smart" as they are, there is a long way to go before this

can be considered "automated".

Prior  to  starting  to  disassemble that super new program, there are some

questions you must ask your self.

o Do I really need the source code for this program?

o Is the program small enough for me to handle?

o Was the program written in a lower level language (C or assembler)?

o Do I really know how this program functions and all of what it does?

These  questions  are  important  as  the  answers  can give you an idea of

whether  you  can  finish  the  job once you start it. There is no point in

"cheating" on the answers either. You only have yourself to convenience.

Source code generation is a three step process.

There  are  three  distinct phases users go through when they disassemble a

program.  The  first  phase  is to identify the type of data the program is  composed  of.  Programs consist of machine instructions and data. But which

is  which? You must follow the logic to tell. The only technical difference

is  that  machine instructions are executed while data is referenced. It is

quite  possible  for  instructions  in  one part of a program to be data to

another  part.  With  the  different segments of the Intel 8086 this is not

very common. But it is certainly possible.

page 17      Masterful Disassembler - Intel 8086  version 1.00      page 17

Once  the  program  has  been  divided into instruction and data areas, the

second  phase  begins.  This  is  the  process of identifying the different

logical  parts. This is usually the most difficult and time consuming part.

It  is  not easy to understand what purpose a sequence of instructions has,

but with persistence this can be done.

The third stage involves generating an assembler source file and getting it

to  re-assemble  properly. Disassemblers are only "human". Their output may

assemble without error but it probably won't be a byte-for-byte copy of the

original  file.  Some "touch-up" will be required to rectify such things as

long  and short jumps. While you are at it, you could clean up the comments

and "pretty up" the source file.

4.1) Identifying Data Types

There  is  a  real knack to separating the code into data and instruction

areas. MD86 goes a long way my marking "questionable" instructions with a

question  mark  to  the left of the label field. This mark will appear on

the  screen  as  well  as  in  a  PRN type output source file. It will be

removed when a non-PRN output file is generated.

Initially  MD86  sees  the  entire  code  segment as instructions and the

entire  data  segment  (EXE  type  programs  only)  as 8-bit binary data.

However, most of the time this is not the case. It is very common to find

character  strings imbedded within the code as well as normal data areas.

When  MD86 marks an instruction line as "questionable", examine the lines

above  and below to determine where the instructions end and data begins.

Of course it is possible that MD86 was wrong in it judgement and the line

is correct.

MD86  assumes  that  memory  is made up of either machine instructions or

data.  The  data may be either 8-bit binary (numbers in the range 0-255),

8-bit  character  data,  or 16-bit address (or offset) data. As mentioned

above,  the  Intel 8086 processor sees instructions as data that it is to

execute  and other data is just referenced. In the sections below we will

assume  that  instruction  areas  are  sequences of instructions that are

executed in order and everything else is data.

There are five basic rules that can be used to determine data area types.

When  you identify data areas, make sure these rules have been satisfied.

If not, be very suspicious.

o rule 1

The  instruction preceeding a data area must be a transfer (jump, call,

interupt, or return). Conditional jumps would not be allowed unless the

condition was ALWAYS met.

page 18      Masterful Disassembler - Intel 8086  version 1.00      page 18

o rule 2

The  first  instruction in an instruction area must have a label unless

the  preceeding  data  area  was  an  argument  to  a  call or interupt

instruction.

o rule 3

An  absolute  transfer of control (jump or return) may be followed only

by a labeled instruction or a labeled data area.

o rule 4

For the type of data to change (from instructions to data or from ASCII

data  to  16-bit  address  data etc.), the first line of the newer type

must have a label.

o rule 5

ASCII  character  data  (including  carriage returns, line feeds, etc.)

must  either begin with a character count byte (or word) or it must end

with  with  a  special  (generally non-ASCII) byte. Is is common within

MSDOS  applications that character strings end with a dollar sign. This

is the way the console output and printer output functions know the end

of  a  string.  Assembly  programmers  also like to use null characters

(value  of zero) as an end of string mark. The Intel 8086 processor can

easily detect these.

For purposes of an example, Figures IIa through IIc will be used. This is

fairly typical of the kind of code you will encounter. But be forewarned,

by  its  very nature assembly code can be very obscure. If the programmer

wishes, it could be extremely difficult to decipher.

Refering  to  Figure  IIa,  note  how  several  lines have been marked as

"questionable".  Here  it  is  obvious  that the lines following the jump

instruction  at address 1283 cannot be instructions. The PUSH instruction

at  address  1286 is erroneous because of rule #1. Notice how most of the

bytes following address 1283 have a value in the range 20 to 7E (hex). It

is quite possible that this area consists mainly of ASCII characters. But

where  does this area end? Rule #2 says we should look for the next valid

instruction  line  containing  a  label.  In this example we find this at

address  12A2. A word of caution here. Since we may not have disassembled

the entire program, the label pool may be incomplete. It is then possible

that  at  this  time  an instruction does not have a label. We need to be

cautious in the application of rule #2.

page 19      Masterful Disassembler - Intel 8086  version 1.00      page 19

Figure IIa, Typical Display Of Partially Disassembled Program

127D:BE5011      _         MOV   SI,#L1150H    ;

1280:E822FC                CALL  L0EA5H        ;

1283:E9DB1B                JMP   L2E61H        ;

1286:55           L1286H   PUSH  BP            ;

1287:6E          ?         DB    06EH          ;

1288:6B          ?         DB    06BH          ;

1289:6E          ?         DB    06EH          ;

128A:6F          ?         DB    06FH          ;

128B:776E                  JA    L12FBH        ;

128D:207665                AND   [BP]+65H,DH   ;

1290:7273                  JC    L1305H        ;

1292:69          ?         DB    069H          ;

1293:6F          ?         DB    06FH          ;

1294:6E          ?         DB    06EH          ;

1295:206F66                AND   [BX]+66H,CH   ;

1298:205475                AND   [SI]+75H,DL   ;

129B:7262                  JC    L12FFH        ;

129D:6F          ?         DB    06FH          ;

129E:0D0A00                OR    AX,#L000AH    ;

12A1:FF          ?L12A1H   DB    0FFH          ;

12A2:9C           L12A2H   CBW                 ;Convert byte (AL) to word

12A3:2E803EA112FF          CMP   CS:[L12A1H],0FFH;

12A9:7404                  JZ    L12AFH        ;

12AB:9D                    CWD                 ;Convert word (AX) to dbl w

CS:: Labels= 492/23%, Types=  21/  4%,   0 cmnts  No Edit 10/ 3/87 10:20:35

As a first step, we use Function key F3 to set the area from 1286 to 12A1

as  characters (type #2). The code now makes more sense (see Figure IIb).

But  now  notice address 12B4. This instruction does not have a label and

yet  it  follows  an unconditional transfer (return) instruction. Rule #3

says  this  is  not correct. Now it could be that there should be a label

here  and  we  have  just  not  disassembled  the  section  of  code that

references  it,  but  the  instructions don't look right do they? The hex

sequences  06, 07, 08, 09, and C3, C4, C5, C6, C7, C8 would not likely be

instructions (although obviously possible). It looks more like numbers or

data.  In  fact the whole area from address 12AD up to 12B8 does not look

like  instructions  at  all.  Most  probably  this  is  just  a data area

containing  numerical  values.  And  8-bit  values  at that. If they were

16-bit  values (or addresses), they would be way beyond the bounds of our

code.

So  again  using  Function  key F3, we set this area to 8-bit binary data

(type  #1). Figure IIc shows what the screen looks like now. Compare this

with Figure IIa and you can see the improvement. In this way areas of the

program  are  disassembled one section at a time. Progress at first seems

slow  I  realize,  but after a while the pieces start to fit together. As    you begin to understand these small portions the remainder of the program

becomes  that  much  easier.  You are well on your way to a useful source

file.

page 20      Masterful Disassembler - Intel 8086  version 1.00      page 20

Figure IIb, Typical Display Of Partially Disassembled Program

127D:BE5011      _         MOV   SI,#L1150H    ;

1280:E822FC                CALL  L0EA5H        ;

1283:E9DB1B                JMP   L2E61H        ;

1286:556E6B6E6F77 L1286H   DB    'Unknown version ';

1296:6F6620547572          DB    'of Turbo',CR,LF,0;

12A1:FF           L12A1    DB    0FFH          ;

12A2:9C           L12A2H   CBW                 ;Convert byte (AL) to word

12A3:2E803EA112FF          CMP   CS:[L12A1H],0FFH;

12A9:7404                  JZ    L12ACH        ;

12AB:9D                    CWD                 ;Convert (AX) to dbl word

12AC:C3           L12ACH   RET                 ;

12AD:2EC606070809 L12ADH   MOV   CS:[L0807H],#09;

12B3:C3                    RET                 ;

12B4:C4C5                  LES   AX,BP         ;

12B6:C6C7C8                MOV   BH,C8         ;

12B9:E8C6FC       L12B8H   CALL  L0F82H        ;

12BE:8B4616                MOV   AX,[BP]+16H   ;

12C1:A38A01                MOV   [L018AH],AX   ;

12C4:8B4604                MOV   AX,[BP]+4     ;

12C7:A38C01                MOV   [L018CH],AX   ;

12CA:1E                    PUSH  DS            ;

12CB:C516AA11              LDS   DX,[L11AAH]   ;Load DS:DX with 32b pointe

12CF:B010                  MOV   AL,#10H       ;

12D1:B425CD21              MSDOS _SIVEC        ;Set vector.

CS:: Labels= 492/23%, Types=  21/  4%,   0 cmnts  No Edit 10/ 3/87 10:20:35

For  EXE  type programs, there is a separate data segment to worry about.

While  this probably does not contain instructions, it is still necessary

to  determine  if  there are any address references stored here. If there

are,  then  they should be identified as such so they can be entered into

the label pool.

In  some  cases tables of addresses can be spotted easily. If most of the

addresses  are  close  (within  a  few  pages)  then you will see similar

hexadecimal values every other byte. For example:

1234:017F097F0F7F L1234H   DB    1,7FH,9,7FH,0FH,7FH;

123A:137F4F7F1080          DB    13H,7FH,4FH,7FH,10H,80H;

When  these  areas  are  changed  into 16-bit address (type #3) then they

appear as follows.

1234:017F097F0F7F L1234H   DW    7F01H,7F09H,7F0FH;

123A:137F4F7F1080          DW    7F13H,7F4FH,8010H;

page 21      Masterful Disassembler - Intel 8086  version 1.00      page 21

Figure IIc, Typical Display Of Partially Disassembled Program

127D:BE5011      _         MOV   SI,#L1150H    ;

1280:E822FC                CALL  L0EA5H        ;

1283:E9DB1B                JMP   L2E61H        ;

1286:556E6B6E6F77 L1286H   DB    'Unknown version ';

1296:6F6620547572          DB    'of Turbo',CR,LF,0;

12A1:FF           L12A1    DB    0FFH          ;

12A2:9C           L12A2H   CBW                 ;Convert byte (AL) to word

12A3:2E803EA112FF          CMP   CS:[L12A1H],0FFH;

12A9:7404                  JZ    L12ACH        ;

12AB:9D                    CWD                 ;Convert (AX) to dbl word

12AC:C3           L12ACH   RET                 ;

12AD:2EC606070809 L12ADH   DB    2EH,0C6H,6,7,8,9,0C3H;

12B4:C4C5C6C7              DB    0C4H,0C5H,0C6H,0C7H;

12B8:C8                    DB    0C8H          ;

12B9:E8C6FC       L12B8H   CALL  L0F82H        ;

12BC:8B4616                MOV   AX,[BP]+16H   ;

12BF:A38A01                MOV   [L018AH],AX   ;

12C2:8B4604                MOV   AX,[BP]+4     ;

12C5:A38C01                MOV   [L018CH],AX   ;

12C8:1E                    PUSH  DS            ;

12C9:C516AA11              LDS   DX,[L11AAH]   ;Load DS:DX with 32b pointe

12CD:B010                  MOV   AL,#10H       ;

12CF:B425CD21              MSDOS _SIVEC        ;Set vector.

12D3:1F                    POP   DS            ;

CS:: Labels= 492/23%, Types=  21/  4%,   0 cmnts  No Edit 10/ 3/87 10:20:35

The  contents  of  these  areas are then added to the address label pool.

When  disassembled,  these  areas  will have a label to let you know that

they are referenced somewhere.

Notice  how  the  first  address  of  this  table has a reference. Rule 4

indicates that this is required. However this is not strictly true. It is

possible  that  the  beginning  of this area is implied by the end of the

previous  structure.  One  common  approach is to have a sequence of flag

bytes  that  is  followed  by  a corresponding address table. Because the

program  "knows"  how  long  the leading byte table is, is then knows the

start of the address table.

MD86  assumes  that  any  address  references present in the data segment

refer  to  offsets within the code segment. While this is generally true,

at  times  this  is  incorrect.  If  it  is  known  to  be  incorrect (by

examination  of  the  code that refers to the table of addresses), then a

choice  has  to  be  made.  Either these addresses must not be defined as

16-bit  addresses  (change  this  to 8-bit binary data), or the erroreous    references  to  the  code segment must be tolerated. It is suggested that

these  be  changed into 8-bit binary data. You could then add label names

to these references within the data segment to keep this correct.

page 22      Masterful Disassembler - Intel 8086  version 1.00      page 22

4.2) Understanding the Code

This is the part you have been waiting for. The real guts of the job! You

have   now   separated  all  data  from  instructions  but  what  do  the

instructions mean?

The Intel 8086 executes instructions in a logical order; the order chosen

by  the  programmer. To truly understand the function of the instructions

you  must  know  how  they  are  executed.  For example, just knowing the

instruction

123A:2C07                  SUB   AL,7

will  subtract  7  from  the contents of register AL is not very helpful.

However, if the surrounding instructions were

1234:8A07                  MOV   AL,[BX]       ;

1236:3C3A                  CMP   AL,':'        ;

1238:7202                  JC    L123BH        ;

123A:2C07                  SUB   AL,7          ;

123B:2C30          L123BH  SUB   AL,'0'        ;

123D:8807                  MOV [BX],AL         ;

you  then  have  the  feeling that register BX is pointing to one or more

bytes. And if these bytes are greater than the digit 9 (the character ":"

is  just  passed  the  digit  "9"  in  the ASCII character set) then 7 is

subtracted.  Looking  7  passed  the  "9"  digit  in  the  table of ASCII

characters  you find the letter "A". Then in either case the value of the

digit  "0" is subtracted. In other words, if register BX were pointing to

an  "8",  then  this  would  be  replaced with the binary value of 8. If,

however,  BX points to the letter "C", it will be replaced with the value

12.  So  this is just converting a hexadecimal digit or digits from ASCII

to  binary.  Well  of course! We "know" this program asks for hexadecimal

values and has to interpret them because in this case we are looking at a

DEBUG.

Because  the  processor executes instructions in a certain order, we must

examine  them  in  that  order. This might seem obvious (and in the above

example  it  is) but in many cases it is not easy to determine the way in

which instructions are executed. Consider the following code.

1234:E83033                CALL  L4567H       ;

1237:0130                  ADD   [BX+SI],SI   ;      1239:337200                XOR   SI,[BP+SI]+0 ;

page 23      Masterful Disassembler - Intel 8086  version 1.00      page 23

The  ADD  instruction following the CALL is not actually executed at all.

By looking at the routine at address 4567 we find that the byte following

the  initial CALL is just a parameter. This byte gets used and the return

will  be  to the following address (1238). We would not have been able to

tell  this  if we hadn't looked at the instructions in the same order the

processor does.

When  you  pick apart even a small section of code you should enter a few

comments and add a label name if you can. Then you won't have to reinvent

the  wheel  the  next time you look at this code (and you will look at it

more than once!).

This process is going to be very laborious. It takes many instructions in

assembly  language  to  accomplish  seemingly trivial functions. Like the

simple   BASIC   statement  "LET  A(1,2)=B+C^2"  may  take  thousands  of

instructions  and  involve many subroutines. But all is not lost. Because

you  know  how the program executes (at least in a gross sense), you will

be able to tackle small portions of it at a time.

Any  information  you  can  get  your  hands  on will help. User manuals,

especially  reference  manuals are a valuable source of information. Some

go  so  far  as  to include memory maps and descriptions of internal data

types. Take TURBO Pascal for example, the manual is a real gold mine!

A bottoms up approach has proven to be the most useful when disassembling

a  program.  Start  from  the lowest level. Look for the operating system

interface.  The reason is that these are well defined and have a specific

calling sequence. MD86 recognizes many of the MSDOS system calls and uses

more meaningful representations. For example the instructions

1234:B409                  MOV   AL,9      ;

1236:CD21                  INT   21H       ;

is replaced with a single macro instruction

1234:B409CD21              MSDOS _OUTSTR   ;Display string at (DX)

In  this way you can identify the lowest level routines. Those that write

characters  to  the  screen  or  read the keyboard. How about opening and

closing  files  and  input  and  output  from  the  communications ports?

Generally   these  are  short  subroutines  (<100  lines)  that  you  can

comprehend.  Try  to  find as many of these routines as possible and give

each one a name that will help you to remember what it does. Also toss in

as many comments as you can.

Once  the  lowest  routines  have  been  worked on, the next higher level    becomes  easier.  Now  you can find those routines that read and write to

files  buffers  without worrying about all those instructions required to

actually get the data out to the disk.

page 24      Masterful Disassembler - Intel 8086  version 1.00      page 24

In  this  way the program gradually starts to unravel and before you know

it you will actually understand how the programmer was able to write it.

Execute  files  (those  with  the extension EXE) introduce a whole set of

additional  problems.  Not  the  least  of  which  is  determining actual

physical address for instructions. You see, the Intel 8086 constructs the

physical  address  at run time from a segment register and an offset. The

relationship is:

physical address = segment*16 + offset

Because  each  register  is  16  bits  long,  there is the possibility of

tremendous  overlap.  An  offset  of 100 into segment 1234 is the same as

offset  110 into segment 1233. To further complicate matters, the segment

registers  can  be changed at will. Thus when an instruction is executed,

the  contents  of  the segment registers (which may have been defined who

knows  where)  are  of  vital  importance. The more segment registers are

modified within a program, the tougher the job of disassembly is.

As  an  example  of  a typical execute program, lets look at EXE2BIN.EXE.

Within the first few instructions we see the following code.

0000:1E          PUSH  DS

0001:33C0        XOR   AX,AX

0003:50          PUSH  AX

0004:B430CD21    MSDOS _GETVER

0008:3C02        CMP   AL,2

000A:7D13        JGE   L001FH

000C:BB3900      MOV   BX,#L0039H

000F:8EDB        MOV   DS,BX

0011:BA5B01      MOV   DX,#L015BH

0014:0E          PUSH  CS

0015:1F          POP   DS

0016:B409CD21    MSDOS _OUTST

001A:06          PUSH  ES

001B:33C0        XOR   AX,AX

001D:50          PUSH  AX

001E:CB          RET

001F:BE8100      MOV   SI,#L0081H

0022:BB3900      MOV   BX,#L0039H

0025:8EC3        MOV   ES,BX

Lets  look at this code for a second. We see that almost the first action

of this is to call MSDOS and find out what its version number is. If this

number  is  greater than or equal to 2 then this jumps to offset 001F. So    the  code between 000C and 001E is only executed if the version number is

less  than  2.  Following the jump instruction, the next two instructions

initialize  the  data  segment  register  (DS) to 39 hex. That means that

page 25      Masterful Disassembler - Intel 8086  version 1.00      page 25

further references into the data segment will get to physical address 390

hex  +  offset. The next instruction loads the DX register with the value

15B  hex. Now if we take a quick look at address 4EB hex (390+15B=4EB) in

our  code  we  will  find  the  start of the ascii message "Incorrect DOS

version$".  A  quick  note,  normally  these  addresses  (ie 4EB) will be

relative  to  the  start  of the data segment within the EXE file and the

code  segment  follows  this  immediately.  Thus we have to look at 4EB -

data_segment_size  within our code. But for EXE2BIN.EXE, the data segment

size  was  zero  so  we  can  look  directly  at address 4EB. Now the two

following instructions are very curious. By executing the PUSH CS and POP

DS  we  will  effectively  reset  the  data  segment register to the code

segment  register,  or  zero  within  our  file.  Thus  the call to MSDOS

function  to  display  an  ascii  character  string  will  try to get the

characters  from  offset  15B  instead  of 4EB. This is a definite bug in

EXE2BIN.EXE!  The PUSH and POP instructions should not be there. Even the

best  programs  can  contain bugs. Don't be too alarmed when you run into

one.

Moving  on, at addresses 22 and 25 we see that the extra segment register

(ES)  is being set to 39 hex just like the data segment register was set.

This  should  give us a real strong indication that address 390 hex (or a

few  bytes beyond) we will find the start of a data area within our code.

This will help us later on.

One further note, when MSDOS executes an EXE type program, it initializes

the  data  segment and extra segment registers to point to an area called

the  Program  Segment  Prefix (PSP). This area contains many useful items

that  the  program  will  need. So prior to changing these registers, the

program will examine this area for those items it needs. Figure III lists

those  items  that are of most interest to us. Refer to reference 1 for a

more complete discussion of this area.

page 26      Masterful Disassembler - Intel 8086  version 1.00      page 26

Figure III, The Program Segment Prefix Summary

Offset  |                           Contents

------  | ------------------------------------------------------------

0002   | System memory size in paragraphs (16 byte blocks). This is a

| 16 bit integer.

|

000E   | Control-C exit address. First 2 bytes are offset and second

| 2 bytes are the segment.

|

0012   | Hard error exit. 2 byte offset and 2 byte segment.

|

005C   | Unopened file control block for first file specified after

| command. Only valid if a path is not specified.

|

006C   | Unopened file control block for second file specified after

| command. Only valid if a path is not specified.

|

0080   | Entire text string the follows the command. The first byte

| is a character count. Note redirection information is not

| passed on to the program (it is stripped first).

4.3) Polishing the Source Code

Sooner  or  later  you  will come to the point where you must abandon the

disassembler.  It  has  done  its  job  but now an editor would be better

suited to working on the files.

Once  you  get  a source file out of MD86 then you can try assembling it.

There  will  undoubtedly be many areas where MASM will complain. Segments

may  be  defined  in  the wrong order or some external references are not

defined at all.

Get  yourself  a  good  screen  oriented  editor. One with virtual memory

support  is vital. Assembly programs tend to be very large and it will be

a real pain if you have to break it into small pieces because your editor

limits  the  code  to 64k. You are going to especially need global search

and replace functions. WordStar, although rather slow, does work fine for

this type of work as long as you don't use document mode.

MD86  always  inserts  data type pointer override instructions. These are

the WORD PTR and BYTE PTR sequences you see all over the place. MASM does

not  require  an override if the types already match. That is, a value is

referenced  as  a  16-bit word and it has previously been defined as this

type,  then  an override is not required. Since MD86 does not know enough

to be sure these conditions have been met, WORD PTR will be inserted. One

of  the first things you will want to do is to remove these phrases where    they are not needed. They just clutter the code.

EXE type files pose the biggest challenge to MD86 and MASM will certainly

page 27      Masterful Disassembler - Intel 8086  version 1.00      page 27

complain about some aspect of the way the different segments are handled.

MD86  rather  simplemindedly inserts tables for each segment that has any

labels  defined at the start of the program. Although careful use of MD86

will limit the number of erroneous labels, some extra ones will exits and

these tables will end up being quite long.

When  MD86 encounters an instruction that references a 16-bit quantity it

assumes  that  this  is  an  address  (or  more properly an offset into a

segment).  This address is put into the label pool. It is not possible to

distinguish  an address reference from a pure constant. Thus you will see

many  labels  in the segment tables (mainly the data segment) with values

line  0,  1, 2, 7, etc. Now these may be valid addresses, but most likely

they  are  just constants. A worth while exercise is to eliminate as many

of  these  as possible. Change the address reference into a constant (ie,

change  "MOV  AX,OFFSET  L03E8H"  into  MOV  AX,OFFSET  1000") so you can

eliminate the "L03E8H:" definition from the data segment table.

4.4) Deciphering More Obscure Code

In  the  good  old  days  when  memory was expensive and processors had a

limited  address range, assembly programmers delighted in seeing how much

they could squeeze into small spaces. This tendency has lessened somewhat

with  the  newer processors and cheap memory but you will still find some

real funny looking code.

Consider  the  following which was found at the start of a disk input and

output routine.

1234:F9                  STC

1235:73F8                JNC   L122FH

1237:B80100              MOV   AX,1

123A:7304                JNC   L4567H

123B:7304                JC    L7654H

Wait  a  minute,  you say. How can you have a set carry instruction (STC)

immediately  followed  by  a  jump  on  no  carry  (JNC)?.  There must be

something  wrong.  No  one  writes  code like that! Actually this code is

correct.  Since  the  jump on no carry is never executed, the destination

byte  is  always  skipped  if  the instructions are executed in the order

shown.  However,  the programmer sometimes jumps directly to address 1236

which  is  in  the  middle  of  the  jump  instruction. In this case, the

displacement  is executed and this becomes a clear carry instruction (the

F8  byte).  What  happens  is that the routine has two functions that are

very similar (like keyboard input with and without echo) and the state of

the  carry flag is used to determine which function is desired. A jump to

address  1234  does  one  thing  and  a jump to 1236 does the other. Very    sneaky!

Or how about this piece of code.

page 28      Masterful Disassembler - Intel 8086  version 1.00      page 28

1234:40                  INC   AX

1235:40                  INC   AX

1236:40                  INC   AX

1237:40                  INC   AX

1238:40                  INC   AX

1239:E82B33              CALL  L4567H

Surely  it doesn't make sense to have that many increment instructions in

a  row.  Or  does it? Actually this is part of an error handling routine.

The  idea  is  to  load the AX register with an error number and call the

routine  at  4567  to  print  out a message based on the error number. To

display error number 1, then the programmers writes the code

2345:31C0                XOR   AX,AX

2347:E8EEEE              CALL  L1238H

To  display  error  message  number 4, then the call goes to address 1235

instead. For this particular procedure, the AX register always contains a

zero  (it  is used as an error flag) and so the XOR AX,AX instruction can

be  eliminated.  Then this requires only a three byte call instruction to

flag  an  error  condition  (instead  of  the  usual  five  bytes).  Some

programmers go to great lengths to save a few bytes of code!

5.0) Examples

A   couple   of  example  disassembly  files  have  been  included  on  the

distribution  disk.  These  give  you an idea of how a typical (if there is

such a thing) disassembly proceeds.

The  first  example  is  the complete disassembly of a disk file comparison

utility  program  called  CMP.COM. This is a short (1/2k) program that took

about an hour or so to disassemble. Using MD86 to examine the progress, you

will  note  that  all  labels  have been given names that more or less make

sense.  In  addition,  numerous comments have been entered. You can write a

source file to the disk and try to assemble it or print it. If you use MASM

to assemble this file, you will run into error 56 (No immediate mode) a few

times. Refer to section 3.5 as to why this happens and how to correct it.

The second example was included to show the results of a basic disassembly.

Here  the program EXE2BIN.EXE has been disassembled but only the first step

has been completed. Only the data areas have been separated. Note that this

EXE  program  does  not  have  a separate data segment. When MD86 reaches a

statement like "MOV DX,L0582H" it notes that offset 582 hex within the data

segment  has  been  referenced.  Since  there isn't any data segment in the  file,  an equate statement is inserted when a source file is generated. But

note  that  the  code internally sets the data segment address to be within

the  code  segment. Thus the reference to DS:582 is really somewhere within

page 29      Masterful Disassembler - Intel 8086  version 1.00      page 29

the  code  segment.  MD86  does not know this and the corresponding address

within  the  code segment does not appear to be referenced. This is all too

typical of EXE programs. They are a real bear to disassemble.

6.0) MD86 Limitations

MD86 is designed to provide as much functionality as is reasonably possible

without  requiring  any  special  equipment.  There  are  some restrictions

imposed on the user although the disassembly of a normal file should not be

hampered. These are:

o 2048 Address References.

o 512 Entries in the Data Type Tables.

o 2048 Comment Strings.

o 64k Maximum Data Segment Size.

o 64k Maximum Code Segment Size.

These  parameters  have  been  chosen  such that a program up to 30k can be

disassembled  as a single file. A 30k program would result in a 15,000 line

assembly  file.  When  disassembling a larger file, it should be broken up.

This  can  be  very  difficult  for  EXE  type programs. Even if MD86 could

process  larger  files,  MASM  has its own restrictions which would require

smaller sections.

7.0) MD86 Error Messages

During  the  process  of initializing its internal tables, MD86 may display

one  of  a  few error messages. At other times, MD86 just beeps to indicate

that  some  process  could  not be completed properly. After the beep, MD86

just waits for a "correct" response or the error to be corrected.

The error messages that may appear are:

o Help, file filename does not exist.

MD86  tried  to locate the file with the name "filename" and it did not

exist.  You are requested to enter another filename. A leading path may

be included if the file is under another directory or on another drive.

o Help, error reading the auto comment file. Cannot continue.

While  reading  the  file  MD86.CMT  (which  did exist), MSDOS or TURBO

Pascal  returned an error code. Try again. If this error persists, then

page 30      Masterful Disassembler - Intel 8086  version 1.00      page 30

re-copy  this  file from the master distribution disk. If this does not

help,  then  send  a  copy  of MD86.CMT and MD86.COM to CC Software for

analysis. You will be contacted with the solution as soon as possible.

o Help,  auto  comment  file  MD86.CMT  cannot  be  found  on the current

directory.

Automatic comment generation is disabled.

The  file MD86.CMT, which contains the automatic comment strings, could

not  be located. MD86 looks only at the current drive and directory. It

does  not  use  the PATH variable. Under most circumstances, you should

quit (F10) without saving the data. Then copy this file from the master

distribution  disk  over  to the correct directory. Re-execute MD86 and

this error message should not appear.

In  the  event  that  you do not need these comments, you may continue.

MD86 will just ignore attempts to insert the comment strings.

o Help, data file filename.001 does not exist!

Help, Data file filename.002 does not exist!

One  or  both  of  the  required data files could not be found. Both of

these  files  are  used  by  MD86  for  label and comment storage. When

disassembling  file  "filename",  MD86 creates files "filename.001" and

"filename.002"  to  store  related  parameters. These files are created

under  the  same  directory  as  "filename.com"  or  "filename.exe" was

located. More than likely you are trying to disassemble another copy of

this  program  under a different directory. Another possibility is that

the  filename  was  not  in  a correct form (ie C>MD86 MYFILE..COM). If

neither of these situations are the cause, then contact CC Software for

additional help.

o Help, one or more data files cannot be read properly.

One  of the data files (either filename.001 or filename.002) was not in

the  correct form for MD86 to read. This can occur when an I/O error or

Run-time  error  aborts  the  writing of these files. Also one of these

files  could just have a bad block. If there has not been a lot of work

already  invested in these files, the safest procedure is to erase them

and  start  over. Or use DEBUG to read these files into memory to check

that they are at least readable. If they are, then send copies of these

files to CC Software for analysis.

7.1) Error Beep While Editing a Field

If  you hear a beep while you are editing a field MD86 is saying that the    last  command  could  not  be completed or it was an illegal command. The

status  line  at  the  bottom  of  the display will show either INSERT or

REPLACE  if  you  are  editing.  The following are the sources of editing

page 31      Masterful Disassembler - Intel 8086  version 1.00      page 31

errors.

o Trying  to  move  the cursor beyond the edges of the field. Left and

right arrow keys.

o Entering  a  non-editing  command  (a  function key or an up or down

arrow).

o Trying  to  update  a  field  that  contains an illegal character. In

particular,  labels  are  restricted  to a leading alphabetic key and

imbedded spaces are not allowed.

If  you  are  editing,  the  ESCAPE  key can always be used to cancel and

restore  the  field to its original content. Often times a key is pressed

by  mistake  which  causes edit mode to be entered but is itself illegal.

For  example,  pressing  the  backspace key (<X) at the first column of a

field. MD86 enters edit mode but rejects the key because there is nothing

to  delete.  However,  MD86 remains in edit mode. Press the ESCAPE key to

cancel.

7.2) Error Beep While Not Editing a Field

If  the  last  line  of the displays contains "No Edit", then you are not

editing  a  field.  In  this  case  MD86 beeps when an illegal command is

entered.  This could be an unbound function key, one of the key pad keys,

or the ESCAPE key. MD86 ignores these keys. No harm is done.

References

1) "MS-DOS  Developer's Guide", John Angermeyer and Kevin Jaeger, Howard W.

Sams & Co, 1986

2) "Peter Norton's Assembly Language Book for the IBM PC", Peter Norton and

John Socha, Prentice Hall Press, 1986

page 32      Masterful Disassembler - Intel 8086  version 1.00      page 32

C O M M A N D   K E Y   S U M M A R Y

Key     Mode                  Description

---------- ---- -----------------------------------------------------------

Left-Arrow   3  Move left one space.

Right-Arrow  3  Move right one space.

Up-Arrow     2  Move up one line.

Down-Arrow   2  Move down one line.

Page-Up      2  Move up about one page.

Page-Down    2  Move down one page.

Home         1  Move to beginning of field.

Home         2  Move to beginning of label field.

End          1  Move to end of field.

End          2  Move to beginning of comment field.

Insert       3  Switch between insert and replace modes for editing.

Delete       1  Delete character under the cursor.

Backspace    1  Delete the character to the left of the cursor.

Escape       1  Cancel editing changes and return cursor to start of field.

Return/Enter 1  Make editing changes permanent.

Return/Enter 2  Move down one line (same as Down-Arrow).

F1           2  Display a one page help summary.

Shift-F1     2  Alter system parameters.

F2           2  Goto a specified address or return from previous Goto.

Shift-F2     2  Follow current instruction (jump or call only).

F3           2  Set data type for given address range.

F4           2  Set data type for unspecified address range.

F5           2  Write source file to disk.

F6           2  Scan code segment to build label table.

F7           2  Dump program in hex and ascii.

F8           2  Set label name for specified address.

F9           2  Search for an address reference.

Shift-F9     2  Search for next reference.

F10          2  Save and/or exit.

Modes: 1= Editing, 2=Non-editing, 3=Either editing or non-editing.

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

The Public (Software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  The P(s)L cannot de-

bug programs over the telephone.

Disks in the P(s)L are updated monthly, so if you did not get

this disk  directly from the P(s)L,  you should be aware that

the files in this set may no  longer be the current versions.

For a copy of the latest monthly software library newsletter

and a list of the 1,000+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

(713) 665-7017