TASM USER'S MANUAL

TASM - A Table Driven Cross Assembler for the MSDOS* Environment

Thomas N. Anderson

Speech Technology Incorporated

837 Front Street South

Issaquah, WA   98072

November,  1987

Version 2.5

[Speech Technology  Incorporated  manufactures electronic devices to

aid  the  visually  impaired  employing   digital  speech  synthesis

technology.]

(C) Copyright 1985,  1986,  1987 by Speech Technology  Incorporated.

All rights  reserved.  Permission  is  granted to copy this document

and  related software except for the source code.   The source code,

distributed  to registered users,  may be copied for the sole use of

the registered user only.

* MSDOS is a trademark of Microsoft Corporation.

TASM - Table Driven Assembler                                Page 2

TABLE OF CONTENTS

SECTION                                        PAGE

____________________________________________________

INTRODUCTION....................................3

INVOCATION......................................4

SOURCE FORMAT...................................7

EXPRESSIONS.....................................9

ASSEMBLER DIRECTIVES............................12

OBJECT FILE FORMATS.............................19

LISTING FILE FORMAT.............................21

PROM PROGRAMMING................................22

ERROR MESSAGES..................................24

BUGS AND LIMITATIONS............................26

6502 INSTRUCTIONS AND ADDRESSING MODES..........27

8048 INSTRUCTIONS AND ADDRESSING MODES..........30

8051 INSTRUCTIONS AND ADDRESSING MODES..........34

8085 INSTRUCTIONS AND ADDRESSING MODES..........38

Z80  INSTRUCTIONS AND ADDRESSING MODES..........41

TASM DISTRIBUTION FILES.........................46

PORTING TASM TO OTHER ENVIRONMENTS..............47

TASM INSTRUCTION SET TABLE DEFINITION...........49

SUMMARY.........................................53

APPENDIX A - SAMPLE LISTING FILE................54

APPENDIX B - SAMPLE INSTRUCTION SET TABLE ......56

APPENDIX C - ORDERING INFORMATION...............61

TASM - Table Driven Assembler                                Page 3

INTRODUCTION

TASM is a set of  table  driven  cross  assemblers  for  the  MS-DOS

environment.     Currently  versions  for  the  6502,   8048,  8051,

8080/8085  and Z80  microprocessors are supported by STI,   but  the

user so inclined  may build versions for other 8 bit microprocessors

(see  sections on TASM DISTRIBUTION  FILES  and   TASM   INSTRUCTION

SET  TABLE  DEFINITION  for notes on reconfiguring  TASM  for  other

processors).

TASM characteristics include:

1.  Powerful expression parsing (17 operators).

2.  Supports a subset of the 'C' preprocessor commands.

3.  Macro capability (through use of DEFINE directive).

4.  Multiple statements per line.

5.  Supports three object file formats (Intel Hex, MOS

Technology Hex, and binary).

6.  Absolute code generation only.

7.  Source code available (in C).

8.  Uniform syntax  across  versions  for  different  target

machines.

9.  Features in support  of  PROM  programming  (preset  all

bytes  to  specified value,  output object code in a

contiguous block).

10.  Supports extended instructions of the Rockwell R65C02.

11.  Tables  can  be  generated  for  other  microprocessors

without having to modify the TASM executable module.

TASM - Table Driven Assembler                                Page 4

INVOCATION

TASM can be invoked as  follows  (optional fields shown in brackets,

symbolic fields enclosed in <>):

tasm -<pn> [-bcdefhlmopqtx] source_file [object_file [list_file]]

Where the option flags are defined as follows:

-<pn>           Specify version (<pn> = part number)

-t<table>       Table (alternate form of above)

-c              Object file written as a contiguous block

-e              Show source lines after macro expansion

-f<xx>          Fill entire memory space with <xx> (hex)

-h              Produce hex table of the assembled code

-l              Produce a label table in the listing

-m              Produce object in MOS Technology format

-b              Produce object in binary (.COM) format

-p              Page the listing file

-q              Quite, disable the listing file

-x[<m>]         Enable extended instruction set (if any)

-d<macro>       Define a macro (or just a macro label)

-o<bb>          Bytes per object record (hex)

TASM has no built in instruction set tables,  and so  it  must  read

them  at  run time.  TASM determines which table to use based on the

'-<pn>' field shown above.  For example,  to assemble the code in  a

file called 'source.asm' one would enter:

tasm -48  source.asm            for an 8048 assembly

tasm -65  source.asm            for a  6502 assembly

tasm -51  source.asm            for an 8051 assembly.

tasm -85  source.asm            for an 8085 assembly.

tasm -80  source.asm            for a  Z80  assembly.

The file name that the tables are read from is formed by taking  the

digits  specified  after  the  '-'  and  appending it to 'TASM' then

appending the '.TAB' extension.  Thus,  the '-48' flag  would  cause

the  tables  to  be  read from the file 'TASM48.TAB' (See section on

TASM INSTRUCTION SET DEFINITION).

It  is  possible to designate tables by non numeric part numbers  if

the '-t' flag is used.  For example,  if a user built a table called

TASMF8.TAB then TASM could be invoked as follows:

tasm -tf8 source.asm

The  source file must be specified.  If not,  some usage information

is displayed.  If the object file is not specified then  the  object

filename  is  formed  by taking the source filename and changing the

extension  to  '.OBJ'.  Similarly,  if  the  list  filename  is  not

specified  it is formed by changing the source filename extension to

'.LST'.

TASM - Table Driven Assembler                                Page 5

Each option flag must be  preceded  by  a  dash.  Options  need  not

precede  the file names, however.  The various options are described

below:

c - Contiguous Block Output. If this option is specified,  then  all

bytes  in the range from the lowest used byte to the highest will be

defined in the object file.  Normally,  with the default  Intel  Hex

object  format  enabled,  if  the Program Counter (PC) jumps forward

because of an .ORG directive,  the bytes skipped over will not  have

any  value  assigned  them  in  the  object  file.  With this option

enabled,  no output to the object file occurs until the end  of  the

assembly  at  which time the whole block is written.  This is useful

when using TASM to generate code that will be put  into  a  PROM  so

that  all  locations  will have a known value.  This option is often

used in conjunction with the -f option to ensure  all  unused  bytes

will have a known value.

e  -  Expand Source.   Normally TASM shows lines in the listing file

just  as  they are in the source file.   If the DEFINE directive  is

used,  however,  it is sometimes desirable to see the source  lines

after expansion.  Use the '-e' flag to accomplish this.

f  -  Fill  Memory.  This  option  causes the memory image that TASM

maintains to be initialized to the value specified by  the  two  hex

characters  immediately  following the 'f'.  TASM maintains a memory

image that is a full 64K bytes in size (even if the target processor

cannot  utilize  that  memory  space).  Invocation  of  this  option

introduces a 2 second delay at start up (time required to initialize

all 64K bytes).   See Appendix A for an example.

h - Hex Table. This option causes a hex table of the produced object

code to appear in the listing file.  Each line of  the  table  shows

sixteen bytes of code.  The format is shown in the sample listing in

Appendix A.

l - Label Table. This option causes a label table to appear  in  the

listing  file.  Each  label is  shown  with its corresponding value.

Macro labels (as established  via  the  DEFINE  directives)  do  not

appear.  The format is shown in the sample listing in Appendix A.

m - MOS Technology Object Format. This option causes the object file

to be written in MOS Technology hex format rather than  the  default

Intel  hex  format.  See  section  on  OBJECT  FILE  FORMATS  for  a

description of the format.

b  -  Binary Object Format. This option causes the object file to be

written in binary - one byte for each byte of code/data.  Note  that

no  address  information  is  included  in  the  object file in this

format.  The contiguous block (-c) output mode is forced  when  this

option is invoked.

p - Page Listing File.  This option causes the listing file to  have

top of page headers and form feeds inserted at appropriate intervals

(every sixty lines of output).

q - Disable Listing File.  This option  causes  all  output  to  the

TASM - Table Driven Assembler                                Page 6

listing  file  to  be  suppressed,   unless  a  .LIST  directive  is

encountered in the source file (see LIST/NOLIST directives).

x - Enable Extended Instruction  Set.  If  a  processor  family  has

instructions  that  are valid for only certain members,  this option

can be used to enable those beyond the  basic  standard  instruction

set.  Presently,  this  option  only  has  significance for the 6502

and  8048  versions of TASM.   The  6502  version  (TASM  -65 )  has

extended instructions for the  Rockwell R65C02  and  the  R65C00/21.

The 8048  version (TASM -48) has extended instructions for the 8041,

8022,  and 8021. A   hex  digit  may  follow  the 'x'  to indicate a

mask value used in selecting the appropriate  instruction set.   Bit

0   of the  mask selects  the  basic instruction set,   thus a '-x1'

would have no effect.   A  '-x3'  would   enable  the basic set plus

whatever instructions have bit 1 set in their  class  mask.  A  '-x'

without  a  digit  following is equivalent to a '-xf' which sets all

four of the  mask  bits.  (See  section  on  6502  INSTRUCTIONS  AND

ADDRESSING MODES for details on its extended instructions).

d - Define a Macro. Macros  are  be  defined  on  the  command  line

generally to control the assembly of various IFDEF's that are in the

source  file.  This is a convenient way to generate various versions

of object code from a single source file.

o - Set Number of Bytes per Object Record.  When  generating  object

code in either the MOS Technology format or the Intel hex format,  a

default of 24 (decimal) bytes of object are defined on each  record.

This can be altered by invoking the '-o' option immediately followed

by  two  hex digits defining the number of bytes per record desired.

For example,  if 32 bytes per record are desired,  one might  invoke

TASM as:

TASM -48 -o20 source.asm

t  - Table Name.    As an alternative to specifying the table to use

(in the case it starts with a non-numeric).   Thus a table for an F8

might be selected as:

TASM -tf8 source.asm

TASM - Table Driven Assembler                                Page 7

SOURCE FORMAT

Statements in the source file must conform to a  format  as  follows

(except  for assembler directive statements which are described in a

subsequent section):

<label>  <operation>    <operand>       <comment>

All of the fields are optional, under appropriate circumstances.  An

arbitrary amount of white space (space and tabs) can  separate  each

field  (as  long as the maximum line length of 255 characters is not

exceeded).  Each of fields are described below:

Label Field. If the first character of the line is alphabetic, it is

assumed  to  be  the  start  of  a label.  Subsequent characters are

accepted as part of that  label  until  a  space,  tab,  or  ':'  is

encountered.   The   assembler   assigns   a   value  to  the  label

corresponding to the current  location  counter.  Labels  can  be  a

maximum  of  13 characters long.  Labels can contain upper and lower

case letters, digits, underscores,  and periods (the first character

must  be alphabetic).  Labels are case sensitive - the label 'START'

is a different label from 'start'.

Operation  Field.  The  operation  field  contains  an   instruction

(opcode)  mnemonic  which  specifies the action to be carried out by

the  target  processor  when  this  instruction  is  executed.   The

interpretation  of each mnemonic is dependent on the version of TASM

being used (see  section  on  OPCODES  AND  ADDRESSING  MODES).  The

operation  field  may  begin  in  any  column except the first.  The

operation field is case insensitive.

Operand Field. The operand field specifies the data to  be  operated

on  by  the  instruction.  It may include expressions and/or special

symbols describing the  addressing  mode  to  be  used.  The  actual

format and interpretation is dependent on the target processor.  For

a description of the format for currently supported processors,  see

the section on OPCODES AND ADDRESSING MODES  for  the  processor  of

interest.

Comment  Field.  The  comment  field always begins with a semicolon.

The rest of the line from the semicolon to the end of  the  line  is

ignored  by  TASM,  but passed on to the listing file for annotation

purposes.  The comment field must be the last field on a  line,  but

it may be the only field, starting in column one, if desired.

Multiple  Statement Lines. If the backslash character is encountered

on a source line,  it is treated as a newline.  The remainder of the

line  following  the  backslash  will be processed as an independent

line of source code.  This allows one to put multiple statements  on

a line.  This facility is not so useful of itself,  but when coupled

with the capability  of  the  DEFINE  directive,  powerful  multiple

statement  macros  can  be  constructed  (see  section  on ASSEMBLER

DIRECTIVES).  Note that when  using  the  statement  separator,  the

character  immediately  following  it should be considered the first

character of a new line,  and thus must either be a start of a label

TASM - Table Driven Assembler                                Page 8

or  white space (not an instruction).  As the examples show, a space

is put between the backslash and the start of the next instruction.

Some  examples  of  valid  source  statements follow (6502 mnemonics

shown):

label1  lda     byte1   ;get the first byte

dec     byte1

jne     label1

;

label2

sta     byte2,X

; a multiple statement line follows

lda     byte1\ sta byte1+4\ lda byte2\ sta byte2+4

TASM - Table Driven Assembler                                Page 9

EXPRESSIONS

Expressions are made  up  of  various  syntactic  elements  (tokens)

combined  according  to  a set of syntactical rules.  The tokens are

summarized as follows:

1.  Labels

2.  Constants

3.  Location Counter Symbol

4.  Operators

5.  Parenthesis

Labels. Labels are strings of characters that have a  numeric  value

associated with them, generally representing an address.  Labels can

contain  upper  and  lower case letters,  digits,  underscores,  and

periods.  The first character must be a letter  (to  distinguish  it

from a numeric constant).  The value of a label is limited to 16 bit

precision.  Labels can contain up to 13 characters, all of which are

significant (none are ignored when looking at a label's value, as in

some assemblers).

Constants.  Numeric constants must always begin with a decimal digit

(thus hexadecimal  constants  that  start  with  a  letter  must  be

prefixed by a '0').  The radix is determined by a letter immediately

following the digit string according to the following table:

Radix           Suffix                  Prefix

---------------------------------------------------

2               B or b                    %

8               O or o                    @

10              D or d (or nothing)

16              H or h                    $

Decimal is the default radix, so decimal constants need no suffix or

prefix.

The following representations are equivalent:

1234H           or      $1234

100d            or      100

177400O         or      @177400

01011000b       or      %01011000

The  prefixes  are provided for compatibility with some other source

code formats but introduce a problem of ambiguity.  Both '%' and '$'

have alternate uses  ('%'  for  modulo,  '$'  for  location  counter

symbol).  To  resolve this,  some simple rules are employed.  If the

first character following a '%' is a '0' or '1', it is assumed to be

a radix specifier and not the modulo  operator.  Similarly,  if  the

first character following a '$' is a valid hexadecimal digit,  it is

assumed to be a radix specifier and not the location  counter.  This

can  cause problems,  however.  Suppose you wanted to find the value

of the low byte of the label 'PNTR_TABLE', you might do this:

(PNTR_TABLE%0100h)

TASM - Table Driven Assembler                                Page 10

Here the '%' would mistakenly be taken for  a  radix  specifier.  To

correct  the  problem one need only insert a space in an appropriate

spot:

(PNTR_TABLE % 0100h)

Character  constants  are  single  characters  surrounded  by single

quotes (following  quote  is  optional).  The  ASCII  value  of  the

character  in the quotes is returned.  No escape provision exists to

represent non-printable characters within the quotes,  but  this  is

not  necessary  since  these  can  be  just as easily represented as

numeric constants.

String constants are one or more  characters  surrounded  by  double

quotes.  Note  that string constants are not allowed in expressions.

They are only allowable following the 'TITLE' and  'TEXT'  assembler

directives.

Location  Counter  Symbol. The current value of the location counter

(PC) can be used in expressions by placing  a  '$'  in  the  desired

place.  The  Location Counter Symbol is allowable anywhere a numeric

constant is.  (Note that if the '$' is followed by a  decimal  digit

then  it  is  taken to be the hexadecimal radix indicator instead of

the Location Counter symbol,  as mentioned above).  The '*' may also

be  used  to  represent the location counter,  but is less preferred

because of its ambiguity with the multiplicative operator.

Operators. Expressions can optionally contain operators  to  perform

some   alterations   or  calculations  on  particular  values.   The

operators are summarized as follows:

Operator  Type          Description

__________________________________________

+        Additive       addition

-                       subtraction

*        Multiplicative multiplication

/                       division

%                       modulo

<<                      logical shift left

>>                      logical shift right

~        Unary          bit inversion (one's complement)

-                       unary negation

=        Relational     equal

==                      equal

!=                      not equal

<                       less than

>                       greater than

<=                      less than or equal

>=                      greater than or equal

&       Binary          binary 'and'

|                       binary 'or'

^                       binary 'exclusive or'

TASM - Table Driven Assembler                                Page 11

The syntax is much the same as in 'C' with the following notes:

1.  No operator precedence is in effect.  Evaluation is from

left to right unless grouped by  parenthesis  (  see

example below).

2.  All evaluations are done with 16 bit precision.

3.  Both '=' and '==' are allowable equality checkers.  This

is allowed   since   the  syntax  does  not  provide

assignment capability (as '=' would normally imply).

The relational operators return a value of 1 if the relation is true

and 0 if it is false.  Sixteen bit signed arithmetic is used.

It is always a good idea to explicitly indicate the desired order of

evaluation with  parenthesis,  especially  to  maintain  portability

since  TASM does not evaluate expressions in the same manner as many

other assemblers.  To understand how it does arrive  at  the  values

for expressions, consider the following example:

1 + 2*3 + 4

TASM would start at the left and read the first token '1'  and  then

the  operator  '+'.  To  determine  what  to  add  to  the '1',  the

expression evaluator would be called recursively on the remainder of

the expression.  The next pass would read the '2' and '*'  and  then

call  itself again to evaluate the rest.  Another level of recursion

would take place in evaluating the '4'.  Since it is not followed by

any more operators, the recursion would start undoing itself and the

final expression would be evaluated as:

1 + (2 * (3 + (4))) = 15

If  the user had desired the '*'  to take precedence,  the following

could have been done:

1 + (2*3) + 4

Use parenthesis liberally.

Here are some examples of valid expressions:

(0f800H + tab)

(label_2 >> 8)

(label_3 << 8) & $f000

$ + 4

010010000100100b + 'a'

(base + ((label_4 >> 5) & (mask << 2))

TASM - Table Driven Assembler                                Page 12

ASSEMBLER DIRECTIVES

Most of the assembler  directives  have  a  format  similar  to  the

machine   instruction   format.   However,   instead  of  specifying

operations for the processor to carry out,  the directives cause the

assembler  to perform some function related to the assembly process.

TASM has two types of assembler directives - those  that  mimic  the

'C'  preprocessor  functions,  and  those  that  resemble  the  more

traditional assembler directive functions.  Each of  these  will  be

discussed.

The  'C' preprocessor style directives are invoked with a '#' as the

first   character   of   the  line  followed  by   the   appropriate

directive  (just  as in 'C').  Thus,  these directives cannot have a

label preceding  them (on the same line).  Note that in the examples

directives are shown in upper case,  however,  either upper or lower

case is acceptable.

INCLUDE. The INCLUDE directive reads in and assembles the  indicated

source  file.  INCLUDEs can be nested up to six levels.  This allows

a convenient means to  keep  common  definitions,  declarations,  or

subroutines  in  files  to  be included as needed.  The format is as

follows:

#INCLUDE        <filename>

The  <filename>  must  be  enclosed in double quotes.  Here are some

examples:

#INCLUDE        "macros.h"

#include "equates"

#include "subs.asm"

DEFINE.  The  DEFINE  directive  is  one of the most powerful of the

directives and allows string substitution  with  optional  arguments

(macros).  The format is as follows:

#DEFINE  <macro_label>[(<arg_list>)]      [<macro_definition>]

<macro_label> := character string to be expanded when found

in the source file.

<arg_list>    := optional argument list for variable

substitution in macro expansion.

<macro_def>   := character string to replace the occurrences

of <macro_label> in the source file.

The simplest form of the DEFINE directive might look like this:

#DEFINE         MLABEL

Notice  that no substitutionary string is specified.  The purpose of

TASM - Table Driven Assembler                                Page 13

a statement like this would typically be to define a label  for  the

purpose  of  controlling some subsequent conditional assembly (IFDEF

or IFNDEF).

A more complicated example,  performing simple  substitution,  might

look like this:

#DEFINE         VAR1_LO         (VAR1 & 255)

This statement would cause all occurrences of the  string  'VAR1_LO'

in the source to be substituted with '(VAR1 & 255)'.

As  a  more  complicated  example,   using  the  argument  expansion

capability, consider this:

#DEFINE         ADD(xx,yy)      clc\ lda xx\ adc yy\ sta xx

If the source file then contained a line like this:

ADD(VARX,VARY)

It would be expanded to:

clc\ lda VARX\ adc VARY\ sta VARX

The above example shows the use of the backslash ('\') character  as

a  multiple  instruction  statement delimiter.  This approach allows

the definition of fairly powerful,  multiple statement  macros.  The

example shown generates 6502 instructions to add one memory location

to another.

Some rules associated with the argument list:

1.  Use a maximum of 10 arguments.

2.  An argument in the DEFINE statement  must  be  a  unique

string  (unique  on  that line) occurring only as an

argument  elsewhere  in  the  line.   TASM  does   a

straight  forward  search  for the occurrence of the

specified argument strings in  the  macro_definition

field  and  if  the  string  is  found somewhere not

intended,  bad things  will  happen.  In  the  above

example,  if  a  simple '(a,b)' had been used as the

argument list instead '(xx,yy)',  then each  'a'  in

the  definition  would be expanded (e.g.  the 'a' in

'lda').

3.  Each argument should be a maximum of 15 characters.

Note that macros can be defined on the TASM command line, also, with

the '-d' option flag.

DEFCONT.   This  directive  can be used to add to  the  last  macro

started with a DEFINE directive.   This provides a convenient way to

define long macros without running off the edge of the page. The ADD

macro shown above could be defined as follows:

TASM - Table Driven Assembler                                Page 14

#DEFINE         ADD(xx,yy)      clc

#DEFCONT                      \ lda xx

#DEFCONT                      \ adc yy

#DEFCONT                      \ sta xx

IFDEF.  This directive can be used to optionally assemble a block of

code.  It has the following form:

#IFDEF  <macro_label>

When invoked,  the list of  macro  labels  (established  via  DEFINE

directives) is searched.  If the label is found, the following lines

of  code  are  assembled.  If  not found,  the input file is skipped

until an ENDIF or ELSE directive is found.

Lines that are skipped over still appear in the listing file,  but a

'~'  will appear immediately after the current PC and no object code

will be generated (this is applicable to IFDEF, IFNDEF, and IF).

IFNDEF. This directive is the opposite of the IFDEF  directive.  The

block   of  code  following  is  assembled  only  if  the  specified

macro_label is undefined.  It has the following form:

#IFNDEF  <macro_label>

When invoked,  the list of  macro  labels  (established  via  DEFINE

directives)  is searched.  If the label is not found,  the following

lines of code are assembled.  If it is  found,  the  input  file  is

skipped until an ENDIF or ELSE directive is found.

IF.  This  directive  can  be used to optionally assemble a block of

code dependent on the value of a given expression.  The format is as

follows:

#IF     <expr>

If the expression <expr> evaluates to non-zero, the following block

of code is assembled (until an ENDIF or ELSE is encountered).

ENDIF. This directive must always follow an  IFDEF,  IFNDEF,  or  IF

directive and signifies the end of the conditional block.

ELSE. This directive can optionally be used with IFDEF,  IFNDEF  and

IF  to  delineate  an alternate block of code to be assembled if the

block  immediately  following  the  IFDEF,   IFNDEF  or  IF  is  not

assembled.

Here are some examples of the use of IFDEF, IFNDEF,  IF,  ELSE,  and

ENDIF:

#IFDEF  label1

lda     byte1

sta     byte2

#ENDIF

TASM - Table Driven Assembler                                Page 15

#ifdef  label1

lda     byte1

#else

lda     byte2

#endif

#ifndef label1

lda     byte2

#else

lda     byte1

#endif

#if ($ >= 1000h)

; generate an invalid statement to cause an error

;  when we go over the 4K boundary.

!!! PROM bounds exceeded.

#endif

ORG. This directive  provides  the  means  to  set  the  Instruction

Pointer  (a.k.a.  Program Counter) to the desired value.  The format

is:

[<label>] .ORG    <expr>

The <label> is optional.  The Instruction pointer  is  assigned  the

value  of  the  expression,  <expr>.  For example,  to generate code

starting at address 1000H, the following could be done:

start   .ORG    1000H

The expression  (<expr>)  may  contain  references  to  the  current

Instruction Pointer, thus allowing various manipulations to be done.

For  example,  to align the Instruction Pointer on the next 256 byte

boundary, the following could be done:

.ORG  (($ + FFH) & FF00H)

ORG can also be used to reserve space without assigning values:

.ORG    $+8

An alternate form of ORG is '*=' or '$='.  Thus  the  following  two

examples as exactly equivalent to the previous example:

*=*+8

$=$+8

EQU. This directive can be used to  assign  values  to  labels.  The

labels  can  then  be  used  in  expressions in place of the literal

constant.  The format is:

<label> .EQU  <expr>

Here is an example:

TASM - Table Driven Assembler                                Page 16

MASK    .EQU    F0H

;

lda     IN_BYTE

and     MASK

sta     OUT_BYTE

An  alternate  form  of  'EQU'  is  '='.  The  previous  example  is

equivalent to:

MASK    =       F0H

or

MASK    =FOH

MASK    =$FO

White  space must exist after the label,  but none is required after

the '='.

BYTE.  This directive allows a value assignment to the byte pointed

to by the current Instruction Pointer.  The format is:

[<label>] .BYTE   <expr>

Only  the lower eight bits of <expr>  are used.   Multiple bytes may

be assigned by separating them with commas. Here are some examples:

label1  .BYTE   10010110B

.byte   'a'

.byte   0

.byte   100010110b,'a',0

DB.  This is alternate form of the BYTE directive.

WORD. This directive allows a value assignment to the next two bytes

pointed to by the current Instruction Pointer.  The format is:

[<label>] .WORD  <expr>

The  least  significant  byte  of  <expr>  is  put  at  the  current

Instruction  Pointer  with  the  most  significant  byte at the next

sequential location.  Here are some examples:

data_table      .WORD   (data_table + 1)

.word   $1234

.Word   (('x' - 'a')  << 2)

.Word   12,55,32

DW.  This is alternate form of the WORD directive.

TEXT.  This  directive  allows  an ASCII string to be used to assign

values  to  a  sequence  of  locations  starting  at   the   current

Instruction Pointer.  The format is:

[<label>] .TEXT   "<string>"

The  ASCII value of each character in <string> is taken and assigned

to the next sequential location.  Some escape sequences are

TASM - Table Driven Assembler                                Page 17

supported as follows:

Escape

Sequence        Description

------------------------------

\n              Line Feed

\r              Carriage return

\b              Backspace

\t              Tab

\f              Formfeed

\\              Backslash

\"              Quote

\000            Octal value of character

Here are some examples:

message1  .TEXT  "Disk I/O error"

message2  .text  "Enter file name "

.text  "abcdefg\n\r"

.text  "I said \"NO\""

TASM  only  shows  the first six  bytes of the string in the listing

file, but all bytes are included in the object file.

BLOCK. This directive causes the Instruction Pointer to advance  the

specified  number  of  bytes without assigning values to the skipped

over locations.  The format is:

[<label>] .BLOCK        <expr>

Some valid examples are:

word1   .BLOCK  2

byte1   .block  1

buffer  .block  80

TITLE. This directive allows the user to define a title string  that

appears  at the top of each page of the list file (assuming the PAGE

mode is on).  The format is:

.TITLE  "<string>"

The string should not exceed 80 characters.  Here are some examples:

.TITLE  "Controller version 1.1"

.title  "This is the title of the assembly"

.title  ""

LIST/NOLIST.  These directives can be used to alternately turn the

output to the list file on (LIST) or off (NOLIST).  The format is:

.LIST

.NOLIST

PAGE/NOPAGE. These directives can be used to  alternately  turn  the

TASM - Table Driven Assembler                                Page 18

paging mode on (PAGE) or off (NOPAGE).  If paging is in effect, then

every  sixty  lines  of  output  will  be  followed by a Top of Form

character and a two line header containing  page  number,  filename,

and the title.  The format is:

.PAGE

.NOPAGE

EJECT.  This  directive  can  be used to force a Top of Form and the

generation of a page header on the list file.  It has no  effect  if

the paging mode is off (see PAGE/NOPAGE).  The format is:

.EJECT

ADDINSTR.  This  directive  can  be  used   to   define   additional

instructions for TASM to use in this assembly.  The format is:

[<label>]  .ADDINSTR <inst> <args> <opcode> <nbytes> <modop> <class>

<shift> <binor>

The fields are separated by white space just as they would appear in an

instruction definition file as described in TASM INSTRUCTION SET

TABLE DEFINITION.

END.   This   directive   should  follow  all  code/data  generating

statements in the source file.  It forces  the  last  record  to  be

written to the object file.  The format is:

[<label>]       .END

TASM - Table Driven Assembler                                Page 19

OBJECT FILE FORMATS

TASM supports three object file formats:

1.  Intel Hex (default).

2.  MOS Technology Hex.

3.  Binary

Each are described below:

Intel Hex Object Format. This is the default format.  This format is

line  oriented  and  uses only printable ASCII characters except for

the carriage return/line feed at the end of each line.  Each line in

the file assumes the following format:

:NNAAAARRHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHCCTT

Where:

All fields marked 'hex' consist of two  or  four  ASCII  hexadecimal

digits  (0-9,  A-F).  A maximum of 24 data bytes will be represented

on each line.

:       = Record Start Character

NN      = Byte Count (hex)

AAAA    = Address of first byte (hex)

RR      = Record Type (hex, 00 except for last record which is 01)

HH      = Data Bytes (hex)

CC      = Check Sum (hex)

TT      = Line Terminator (carriage return, line feed)

The last line of the file will be a record conforming to  the  above

format with a byte count of zero (':00000001FF').

The checksum is defined as:

sum      =  byte_count + address_hi + address_lo +

record_type + (sum of all data bytes)

checksum =  ((-sum) & ffh)

TASM - Table Driven Assembler                                Page 20

MOS  Technology  Hex Object Format. This format is line oriented and

uses  only  printable  ASCII  characters  except  for  the  carriage

return/line  feed  at  the  end of each line.  Each line in the file

assumes the following format:

;NNAAAAHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHHCCCCTT

Where:

All fields marked 'hex' consist of two  or  four  ASCII  hexadecimal

digits  (0-9,  A-F).  A maximum of 24 data bytes will be represented

on each line.

;       = Record Start Character

NN      = Byte Count (hex)

AAAA    = Address of first byte (hex)

HH      = Data Bytes (hex)

CCCC    = Check Sum (hex)

TT      = Line Terminator (carriage return, line feed)

The last line of the file will be a record with a byte count of zero

(';00').

The checksum is defined as:

sum      =  byte_count + address_hi + address_lo +

record_type + (sum of all data bytes)

checksum =  (sum & ffffh)

Binary   Object   Format.   This  file  format  has  only  a  binary

representation of each data byte with no address, checksum or format

description,  whatsoever.  It is often a convenient format to use to

pass  the  data to other programs on your PC (like a PROM programmer

package) but because of the  non-printability and  lack  of  address

information,  it  is  not  often  used to transmit the code to other

systems.

Note that when this object format is selected (-b  option),  the  -c

option  is  forced.  This  is  done so that no ambiguity arises as a

result of the lack of address information in the file.  Without  the

-c  option,   discontinuous  blocks  of  object  code  would  appear

contiguous.

TASM - Table Driven Assembler                                Page 21

LISTING FILE FORMAT

Each line of source code generates one (or more) lines of output  in

the listing file.  The fields of the output line are as follows:

1.  Current  source  file  line  number  (4 decimal digits).

2.  An optional '+' appears if this is  an  'INCLUDE'  file.

(One '+' for each level of INCLUDE invoked).

3.  Current Instruction Pointer (4 hex digits).  An optional

'~' follows the Instruction Pointer if the  line  of

source  code  is  not  being assembled because of an

IFDEF, IFNDEF, or IF directive.

4.  Resulting code/data generated from this source line (two

hex digits per byte, each byte separated by a space,

up to four bytes per line).

5.  The  source  line  exactly  as  it appears in the source

file.

If  paging  is  enabled (by either the '-p' option flag or the .PAGE

directive) some additional fields will be inserted into the  listing

file every 60 lines.  These fields are:

1.  Top of Form (form feed).

2.  Assembler identifier (e.g. "TASM 6502 Assembler").

3.  Initial source file name.

4.  Page number.

5.  Title.

For an example of the listing file format, see appendix A.

TASM - Table Driven Assembler                                Page 22

PROM PROGRAMMING

A wide variety of PROM programming equipment is available  that  can

use  object  code  in  one or more of the formats TASM supports.  We

will not try to list all such  compatible  systems  here,  but  will

mention one configuration that has worked well for us.

We have used the Apparat "IBM PROM Blaster,  24 Pin" (available from

Apparat Inc.,  4401 Tamarac Parkway,  Denver,  Colorado 80237, $129)

for programming 24  pin  EPROMs  with  much  success.  The  software

supplied  with  this  product  will accept a TASM object file in the

binary  format  (-b  option  flag  on  TASM   command   line).   The

combination   of  TASM  and  the  PROM  Blaster  make  for  a  truly

inexpensive solution to the problem of generating  PROMed  code  for

target microprocessor systems.  (STI is not affiliated or associated

with Apparat in any way).

Some additional notes about generating code to be put in PROMs:

1.  It  is  often  desirable  to  have all bytes in the PROM

programmed even if not explicitly assigned  a  value

in the source code (e.g.  the bytes are skipped over

with a .ORG statement).  This can be accomplished by

using the -c (contiguous block) and  the  -f  (fill)

command  line option flags.  The -c will ensure that

every byte from the lowest byte assigned a value  to

the the highest byte assigned a value will be in the

object  file  with no gaps.  The -f flag will assign

the specified value to all bytes before the assembly

begins so that when the object file is written,  all

bytes  not  assigned a value in the source code will

have a known value.  As an  example,  the  following

command  line  will  generate  object  code  in  the

default Intel Hex format with all bytes not assigned

a value in the source set to EA (hex, 6502 NOP):

tasm -65 -c -fEA test.asm

2.  To ensure that TASM generates object code to  cover  the

full  address  range of the target PROM,  put a .ORG

statement at the end of the source file set  to  the

last address desired.  For example, to generate code

to  be  put  in  a  2716  EPROM  (2 Kbytes) from hex

address $1000 to $17ff,  do something like  this  in

the source file:

;start of the file

.ORG    $1000

;rest of the source code follows

<source code>

;end of the source code

.ORG    $17ff

TASM - Table Driven Assembler                                Page 23

.BYTE   0

.END

Now,  to  invoke  TASM  to  generate the code in the

binary format with all unassigned bytes  set  to  00

(6502 BRK), do the following:

tasm -65 -b -f00 test.asm

Note that -b forces the -c option.

TASM - Table Driven Assembler                                Page 24

ERROR MESSAGES

Error Message                           Description

--------------------------------------------------------

unrecognized directive..................A statement starting  with a

'.'  or  '#'  has a mnemonic

that is  not  defined  as  a

directive.

unrecognized  instruction...............A  statement  has  an opcode

mnemonic   that    is    not

defined.

unrecognized argument...................A statement has  an  operand

format that is not defined.

label value misaligned..................The value of a label appears

to have a different value on

the  second pass then it was

computed  to  have  on   the

first    pass.    This    is

generally due to  Zero  Page

Addressing   mode   problems

with  the  6502  version  of

TASM.  Labels  that are used

in operands  for  statements

that   could  utilize   Zero

Page addressing mode  should

always   be  defined  before

used as an operand.

label table overflow....................To  many  labels  have  been

encountered.

No END directive before EOF.............The source file did not have

an   END  directive  in  it.

This is not fatal,  but  may

cause  the  last object file

record to be lost.

No  files  specified....................TASM  was  invoked  with  no

source file specified.

Unknown  option  flag...................TASM  was  invoked  with  an

undefined option flag on the

command line.

Source file open error..................TASM was not  able  to  open

the specified source file.

List file open error....................TASM was not  able  to  open

the specified list file.

Object file open error..................TASM was not  able  to  open

the specified object file.

TASM - Table Driven Assembler                                Page 25

Unknown token...........................Unexpected  characters  were

encountered while parsing an

expression.

maximum number of macros exceeded.......To  many  macros   (DEFINEs)

have been encountered.

macro buffer overflow...................The buffer from which  space

is   allocated   for   macro

definitions is exhausted.

range of relative branch exceeded.......A branch instruction exceeds

the  maximum   range   (6502

Version).

macro expansion too long................The  expansion  of  a  macro

resulted  in  a  line  that

exceeded the maximum length.

TASM - Table Driven Assembler                                Page 26

BUGS AND LIMITATIONS

Limitations and Specifications

----------------------------------------------------------------

Maximum number of labels                2000

Maximum length of labels                13 characters

Maximum address space                   64 Kbytes (65536 bytes)

Maximum number of nested INCLUDES       4

Maximum length of TITLE string          79 characters

Maximum source line length              255 characters

Maximum length after macro expansion    255 characters

Maximum length of expressions           255 characters

Maximum length of pathnames             79 characters

Maximum length of command line          127 characters

Maximum number of instructions          600

Maximum number of macros                1000

Maximum number of macro arguments       10

Maximum length of macro argument        16 characters

Heap size (for labels and macros)       20000 bytes

Memory requirements                     160K (32K for code, 64K for

data and stack, 64K for

opcode memory image).

Bugs

1.  The 8048 version of TASM does not check for use of memory beyond

any  reasonable  bounds (e.g.  an 8048 has a maximum address

space of 4 Kbytes but TASM will let  you  pretend  that  you

have 64 Kbytes).

2.  Expression evaluation has no operator precedence in effect which

can  make  for  unexpected results if not explicitly grouped

with parenthesis.

3.  First  page  of  listing file will not show a user defined title

(defined via TITLE directive).

4.  TASM sometimes does not generate error messages  for  improperly

formed expressions.

5.  TASM expands macros in comments at the end of a line (but not in

lines that are all comment).

6.  TASM does not generate an error message when a EQU directive has

an undefined label on the right hand side.

TASM - Table Driven Assembler                                Page 27

6502 INSTRUCTIONS AND ADDRESSING MODES

The acceptable 6502 opcode mnemonics for TASM are as follows:

ADC  AND  ASL  BCC  BCS  BEQ  BNE  BMI  BPL  BVC  BVS  BIT

BRK  CLC  CLD  CLI  CLV  CMP  CPX  CPY  DEC  DEX  DEY  EOR

INC  INX  INY  JMP  JSR  LDA  LDX  LDY  LSR  NOP  ORA  PHA

PHP  PLA  PLP  ROL  ROR  RTI  RTS  SBC  SEC  SED  SEI  STA

STX  STY  TAX  TAY  TSX  TXA  TXS  TYA

TASM  also  supports the following instructions that are part of the

Rockwell  R65C02  and  R65C00/21  microprocessor  instruction  sets.

Those  that  are  marked  as  set A are applicable to the R65C02 and

those marked as set B are  applicable  to  the  R65C00/21  (A+B  for

both):

Mnemonic        Description                Address Mode  Set

---------------------------------------------------------------

ADC             Add with carry             (IND)         A

AND             And memory with A          (IND)         A

BIT             Test memory bits with A    ABS,X         A

BIT             Test memory bits with A    ZP,X          A

BIT             Test memory bits with A    IMM           A

CMP             Compare memory with A      (IND)         A

DEC             Decrement A                A             A

EOR             Exclusive OR memory with A (IND)         A

INC             Increment A                A             A

JMP             Jump                       (ABS,X)       A

LDA             Load A with memory         (IND)         A

ORA             OR A with memory           (IND)         A

SBC             Subtract memory form A     (IND)         A

STA             Store A in memory          (IND)         A

STZ             Store zero                 ABS           A

STZ             Store zero                 ABS,X         A

STZ             Store zero                 ZP            A

STZ             Store zero                 ZP,X          A

TRB             Test and reset memory bit  ABS           A

TRB             Test and reset memory bit  ZP            A

TSB             Test and set memory bit    ABS           A

TSB             Test and set memory bit    ZP            A

BRA             Branch Always              REL           A+B

BBR0            Branch on Bit 0 Reset      ZP,REL        A+B

BBR1            Branch on Bit 1 Reset      ZP,REL        A+B

BBR2            Branch on Bit 2 Reset      ZP,REL        A+B

BBR3            Branch on Bit 3 Reset      ZP,REL        A+B

BBR4            Branch on Bit 4 Reset      ZP,REL        A+B

BBR5            Branch on Bit 5 Reset      ZP,REL        A+B

BBR6            Branch on Bit 6 Reset      ZP,REL        A+B

BBR7            Branch on Bit 7 Reset      ZP,REL        A+B

BBS0            Branch on Bit 0 Set        ZP,REL        A+B

BBS1            Branch on Bit 1 Set        ZP,REL        A+B

BBS2            Branch on Bit 2 Set        ZP,REL        A+B

BBS3            Branch on Bit 3 Set        ZP,REL        A+B

TASM - Table Driven Assembler                                Page 28

BBS4            Branch on Bit 4 Set        ZP,REL        A+B

BBS5            Branch on Bit 5 Set        ZP,REL        A+B

BBS6            Branch on Bit 6 Set        ZP,REL        A+B

BBS7            Branch on Bit 7 Set        ZP,REL        A+B

MUL             Multiply                   Implied       B

PHX             Push Index X               Implied       A+B

PHY             Push Index Y               Implied       A+B

PLX             Pull Index X               Implied       A+B

PLY             Pull Index Y               Implied       A+B

RMB0            Reset Memory Bit 0         ZP            A+B

RMB1            Reset Memory Bit 1         ZP            A+B

RMB2            Reset Memory Bit 2         ZP            A+B

RMB3            Reset Memory Bit 3         ZP            A+B

RMB4            Reset Memory Bit 4         ZP            A+B

RMB5            Reset Memory Bit 5         ZP            A+B

RMB6            Reset Memory Bit 6         ZP            A+B

RMB7            Reset Memory Bit 7         ZP            A+B

SMB0            Set   Memory Bit 0         ZP            A+B

SMB1            Set   Memory Bit 1         ZP            A+B

SMB2            Set   Memory Bit 2         ZP            A+B

SMB3            Set   Memory Bit 3         ZP            A+B

SMB4            Set   Memory Bit 4         ZP            A+B

SMB5            Set   Memory Bit 5         ZP            A+B

SMB6            Set   Memory Bit 6         ZP            A+B

SMB7            Set   Memory Bit 7         ZP            A+B

Note that correct assembly of these extended  instructions  has  not

been tested on a target system.

Addressing  modes  are denoted as follows:

ABS               Absolute

ZP                Zero Page

ABS,X             Absolute X

ZP,X              Zero Page X

ABS,Y             Absolute Y

ZP,Y              Zero Page Y

A                 Accumulator

(IND,X)           Indirect X

(IND),Y           Indirect Y

(IND)             Indirect

#IMM              Immediate

REL               Relative (Branch instructions only)

ZP,REL            Zero Page, Relative

Implied           Implied

Note  that Zero Page addressing can not be explicitly requested.  It

is used if the value of the operand is  representable  in  a  single

byte for the applicable statements.

The '-x' command line option can be  used  to  enable  the  extended

instructions.  A  '-x'  with  no  digit  following  will  enable the

standard set plus both extended sets.  The 6502 version of TASM uses

TASM - Table Driven Assembler                                Page 29

three bits in the instruction class  mask  to  determine  whether  a

given  instruction  is enabled or not.  Bit 0 enables the basic set,

bit 1 enables set A (R65C02) and bit 2 enables  set  B  (R65C00/21).

The following table shows various options:

Class Mask        Enabled Instructions

BASIC   R65C02  R65C00/21

--------------------------------------------

1               yes     no      no

2               no      yes     no

3               yes     yes     no

4               no      no      yes

5               yes     no      yes

6               no      yes     yes

7               yes     yes     yes

Thus,  to enable the basic set plus the R65C02 instructions,  invoke

the '-x3' command line option.

See manufacturer's data for  a  more  complete  description  of  the

meaning of the mnemonics and addressing modes.

TASM - Table Driven Assembler                                Page 30

8048 INSTRUCTIONS AND ADDRESSING MODES

The following list shows the acceptable opcode mnemonics  and  their

corresponding  operand  formats for the 8048 version of TASM.  Where

'Rn' is seen,  R0 through R7  may  be  substituted.  Other  symbolic

fields are as follows:

SYMBOLIC            DESCRIPTION

-----------------------------------------------

<addr8>             Absolute address (8 bits)

<addr11>            Absolute address (11 bits)

<immed>             Immediate data

Any valid TASM expression can appear in the  place  of  any  of  the

above symbolics.

The lines that are marked with an (8041),  (8022),  or (8021) on the

far  right are extended instructions that are available only if a -x

option has  been  invoked  on  the  command  line.  The  classes  of

instructions  (and their bit assignment in the class mask) are shown

below:

BIT     PROCESSOR

-------------------------------

0       8X48, 8035, 8039, 8049

1       8X41A

2       8022

3       8021

Thus, to enable the basic 8048 set plus the 8022 set, a -x5 could be

used on the command line.

Note that some of the base instructions should be disabled  for  the

8041, 8022, and 8021, but are not.

OPCODE  OPERANDS        DESCRIPTION

-------------------------------------------------------------------

ADD     A,Rn            Add Register to Acc

ADD     A,@R0           Add Indirect RAM to Acc

ADD     A,@R1           Add Indirect RAM to Acc

ADD     A,#<immed>      Add Immediate data to Acc

ADDC    A,Rn            Add Register to Acc with carry

ADDC    A,@R0           Add Indirect RAM to Acc with carry

ADDC    A,@R1           Add Indirect RAM to Acc with carry

ADDC    A,#<immed>      Add Immediate data to Acc with carry

ANL     A,Rn            AND Register to Acc

ANL     A,@R0           AND Indirect RAM to Acc

ANL     A,@R1           AND Indirect RAM to Acc

ANL     A,#<immed>      AND Immediate data to Acc

ANL     BUS,#<immed>    AND Immediate data to BUS

ANL     P1,#<immed>     AND Immediate data to port P1

ANL     P2,#<immed>     AND Immediate data to port P2

ANLD    P4,A            AND Acc to Expander port P4

TASM - Table Driven Assembler                                Page 31

ANLD    P5,A            AND Acc to Expander port P5

ANLD    P6,A            AND Acc to Expander port P6

ANLD    P7,A            AND Acc to Expander port P7

CALL    <addr11>        Call subroutine

CLR     A               Clear Acc

CLR     C               Clear Carry

CLR     F0              Clear Flag 0

CLR     F1              Clear Flag 1

CPL     A               Complement Acc

CPL     C               Complement Carry

CPL     F0              Complement Flag F0

CPL     F1              Complement Flag F1

DA      A               Decimal adjust Acc

DEC     A               Decrement Acc

DEC     Rn              Decrement Register

DIS     I               Disable Interrupts

DIS     TCNTI           Disable Timer/Counter Interrupt

DJNZ    Rn,<addr8>      Decrement Register and Jump if nonzero

EN      DMA             Enable DMA                           (8041)

EN      FLAGS           Enable Flags                         (8041)

EN      I               Enable External Interrupt

EN      TCNTI           Enable Timer/Counter Interrupt

ENT0    CLK             Enable Clock Output

IN      A,DBB           Input Data Bus to Acc                (8041)

IN      A,P0            Input Port 0 to Acc                  (8021)

IN      A,P1            Input Port 1 to Acc

IN      A,P2            Input Port 2 to Acc

INC     A               Increment Acc

INC     Rn              Increment Register

INC     @R0             Increment Indirect RAM

INC     @R1             Increment Indirect RAM

INS     A,BUS           Strobed Input of Bus to Acc

JB0     <addr8>         Jump if Acc bit 0 is set

JB1     <addr8>         Jump if Acc bit 1 is set

JB2     <addr8>         Jump if Acc bit 2 is set

JB3     <addr8>         Jump if Acc bit 3 is set

JB4     <addr8>         Jump if Acc bit 4 is set

JB5     <addr8>         Jump if Acc bit 5 is set

JB6     <addr8>         Jump if Acc bit 6 is set

JB7     <addr8>         Jump if Acc bit 7 is set

JMP     <addr11>        Jump

JC      <addr8>         Jump if Carry is set

JF0     <addr8>         Jump if Flag F0 is set

JF1     <addr8>         Jump if Flag F1 is set

JNC     <addr8>         Jump if Carry is clear

TASM - Table Driven Assembler                                Page 32

JNI     <addr8>         Jump if Interrupt input is clear

JNIBF   <addr8>         Jump if IBF is clear                 (8041)

JNT0    <addr8>         Jump if T0 is clear

JNT1    <addr8>         Jump if T1 is clear

JNZ     <addr8>         Jump if Acc is not zero

JOBF    <addr8>         Jump if OBF is set                   (8041)

JTF     <addr8>         Jump if Timer Flag is set

JT0     <addr8>         Jump if T0 pin is high

JT1     <addr8>         Jump if T1 pin is high

JZ      <addr8>         Jump if Acc is zero

JMPP    @A              Jump Indirect (current page)

MOV     A,PSW           Move PSW to Acc

MOV     A,Rn            Move Register to Acc

MOV     A,T             Move Timer/Counter to Acc

MOV     A,@R0           Move Indirect RAM to Acc

MOV     A,@R1           Move Indirect RAM to Acc

MOV     A,#<immed>      Move Immediate data to Acc

MOV     PSW,A           Move Acc to PSW

MOV     Rn,A            Move Acc to Register

MOV     Rn,#<immed>     Move Immediate data to Register

MOV     STS,A           Move Acc to STS                      (8041)

MOV     T,A             Move Acc to Timer/Counter

MOV     @R0,A           Move Acc to Indirect RAM

MOV     @R1,A           Move Acc to Indirect RAM

MOV     @R0,#<immed>    Move Immediate data to Indirect RAM

MOV     @R1,#<immed>    Move Immediate data to Indirect RAM

MOVD    A,P4            Move half-byte Port 4 to Acc (lower nibble)

MOVD    A,P5            Move half-byte Port 5 to Acc (lower nibble)

MOVD    A,P6            Move half-byte Port 6 to Acc (lower nibble)

MOVD    A,P7            Move half-byte Port 7 to Acc (lower nibble)

MOVD    P4,A            Move lower nibble of Acc to Port 4

MOVD    P5,A            Move lower nibble of Acc to Port 5

MOVD    P6,A            Move lower nibble of Acc to Port 6

MOVD    P7,A            Move lower nibble of Acc to Port 7

MOVP    A,@A            Move Indirect Program data to Acc

MOVP3   A,@A            Move Indirect Program data to Acc (page 3)

MOVX    A,@R0           Move Indirect External RAM to Acc

MOVX    A,@R1           Move Indirect External RAM to Acc

MOVX    @R0,A           Move Acc to Indirect External RAM

MOVX    @R1,A           Move Acc to Indirect External RAM

NOP                     No operation

ORL     A,Rn            OR Register to Acc

ORL     A,@R0           OR Indirect RAM to Acc

ORL     A,@R1           OR Indirect RAM to Acc

ORL     A,#<immed>      OR Immediate data to Acc

ORL     BUS,#<immed>    OR Immediate data to BUS

ORL     P1,#<immed>     OR Immediate data to port P1

ORL     P2,#<immed>     OR Immediate data to port P2

ORLD    P4,A            OR lower nibble of Acc with P4

ORLD    P5,A            OR lower nibble of Acc with P5

TASM - Table Driven Assembler                                Page 33

ORLD    P6,A            OR lower nibble of Acc with P6

ORLD    P7,A            OR lower nibble of Acc with P7

OUTL    BUS,A           Output Acc to Bus

OUT     DBB,A           Output Acc to DBB                    (8041)

OUTL    P0,A            Output Acc to Port P0                (8021)

OUTL    P1,A            Output Acc to Port P1

OUTL    P2,A            Output Acc to Port P2

RAD                     Move A/D Converter to Acc            (8022)

RET                     Return from subroutine

RETI                    Return from Interrupt w/o PSW restore(8022)

RETR                    Return from Interrupt w/  PSW restore

RL      A               Rotate Acc Left

RLC     A               Rotate Acc Left through Carry

RR      A               Rotate Acc Right

RRC     A               Rotate Acc Right through Carry

SEL     AN0             Select Analog Input 0                (8022)

SEL     AN1             Select Analog Input 1                (8022)

SEL     MB0             Select Memory Bank 0

SEL     MB1             Select Memory Bank 1

SEL     RB0             Select Register Bank 0

SEL     RB1             Select Register Bank 1

STOP    TCNT            Stop Timer/Counter

STRT    CNT             Start Counter

STRT    T               Start Timer

SWAP    A               Swap nibbles of Acc

XCH     A,Rn            Exchange Register with Acc

XCH     A,@R0           Exchange Indirect RAM with Acc

XCH     A,@R1           Exchange Indirect RAM with Acc

XCHD    A,@R0           Exchange lower nibble of Indirect RAM w/ Acc

XCHD    A,@R1           Exchange lower nibble of Indirect RAM w/ Acc

XRL     A,Rn            Exclusive OR Register to Acc

XRL     A,@R0           Exclusive OR Indirect RAM to Acc

XRL     A,@R1           Exclusive OR Indirect RAM to Acc

XRL     A,#<immed>      Exclusive OR Immediate data to Acc

See manufacturer's data for  a  more  complete  description  of  the

meaning of the mnemonics and addressing modes.

TASM - Table Driven Assembler                                Page 34

8051 INSTRUCTIONS AND ADDRESSING MODES

The following list shows the acceptable opcode mnemonics  and  their

corresponding  operand  formats for the 8051 version of TASM.  Where

'Rn' is seen,  R0 through R7  may  be  substituted.  Other  symbolic

fields are as follows:

SYMBOLIC            DESCRIPTION

-----------------------------------------------

<addr11>            Absolute address (11 bits)

<addr16>            Absolute address (16 bits)

<bit>               Bit address

<immed>             Immediate data

<direct>            Direct RAM address

<rel>               Relative address

Any valid TASM expression can appear in the  place  of  any  of  the

above symbolics.

OPCODE  OPERAND             DESCRIPTION

--------------------------------------------------------------------

ACALL   <addr11>            Absolute Call

ADD     A,Rn                Add Register to Acc

ADD     A,@R0               Add Indirect RAM to Acc

ADD     A,@R1               Add Indirect RAM to Acc

ADD     A,#<immed>          Add Immediate data to Acc

ADD     A,<direct>          Add Direct RAM to Acc

ADDC    A,Rn                Add Register to Acc with carry

ADDC    A,@R0               Add Indirect RAM to Acc with carry

ADDC    A,@R1               Add Indirect RAM to Acc with carry

ADDC    A,#<immed>          Add Immediate data to Acc with carry

ADDC    A,<direct>          Add Direct RAM to Acc with carry

AJMP    <addr11>            Absolute Jump

ANL     A,Rn                AND Register and Acc

ANL     A,@R0               AND Indirect RAM and Acc

ANL     A,@R1               AND Indirect RAM and Acc

ANL     A,#<immed>          AND Immediate data and Acc

ANL     A,<direct>          AND Direct RAM and Acc

ANL     C,/<direct>         AND Complement of direct bit to Carry

ANL     C,<direct>          AND direct bit to Carry

ANL     <direct>,A          AND Acc to direct RAM

ANL     <direct>,#<immed>   AND Immediate data and direct RAM

CJNE    A,#<immed>,<rel>    Compare Immediate to Acc   and JNE

CJNE    A,<direct>,<rel>    Compare direct RAM to Acc and JNE

CJNE    Rn,#<immed>,<rel>   Compare Immediate to Register and JNE

CJNE    @R0,#<immed>,<rel>  Compare Immediate to Indirect RAM and JNE

CJNE    @R1,#<immed>,<rel>  Compare Immediate to Indirect RAM and JNE

CLR     A                   Clear Accumulator

CLR     C                   Clear Carry

CLR     <direct>            Clear Direct RAM

TASM - Table Driven Assembler                                Page 35

CPL     A                   Complement Accumulator

CPL     C                   Complement Carry

CPL     <direct>            Complement Direct RAM

DA      A                   Decimal Adjust Accumulator

DEC     A                   Decrement Acc

DEC     Rn                  Decrement Register

DEC     @R0                 Decrement Indirect RAM

DEC     @R1                 Decrement Indirect RAM

DEC     <direct>            Decrement Direct RAM

DIV     AB                  Divide Acc by B

DJNZ    Rn,<rel>            Decrement Register and JNZ

DJNZ    <direct>,<rel>      Decrement Direct RAM and JNZ

INC     A                   Increment Acc

INC     Rn                  Increment Register

INC     @R0                 Increment Indirect RAM

INC     @R1                 Increment Indirect RAM

INC     DPTR                Increment Data Pointer

INC     <direct>            Increment Direct RAM

JB      <bit>,<rel>         Jump if Bit is set

JBC     <bit>,<rel>         Jump if Bit is set & clear Bit

JC      <rel>               Jump if Carry is set

JMP     @A+DPTR             Jump indirect relative to Data Pointer

JNB     <bit>,<rel>         Jump if Bit is clear

JNC     <rel>               Jump if Carry is clear

JNZ     <rel>               Jump if Acc is not zero

JZ      <rel>               Jump if Acc is zero

LCALL   <addr16>            Long Subroutine Call

LJMP    <addr16>            Long Jump

MOV     A,Rn                Move Register to Acc

MOV     A,@R0               Move Indirect RAM to Acc

MOV     A,@R1               Move Indirect RAM to Acc

MOV     A,#<immed>          Move Immediate data to Acc

MOV     A,<direct>          Move direct RAM to Acc

MOV     C,<bit>             Move bit to Acc

MOV     DPTR,#<immed>       Move immediate data to Data Pointer

MOV     Rn,A                Move Acc to Register

MOV     Rn,#<immed>         Move Immediate data to Register

MOV     Rn,<direct>         Move Direct RAM to Register

MOV     @R0,A               Move Acc to Indirect RAM

MOV     @R1,A               Move Acc to Indirect RAM

MOV     @R0,#<immed>        Move Immediate data to Indirect RAM

MOV     @R1,#<immed>        Move Immediate data to Indirect RAM

MOV     @R0,<direct>        Move Direct RAM to Indirect RAM

MOV     @R1,<direct>        Move Direct RAM to Indirect RAM

MOV     <direct>,A          Move Acc to Direct RAM

MOV     <bit>,C             Move Carry to Bit

MOV     <direct>,Rn         Move Register to Direct RAM

MOV     <direct>,@R0        Move Indirect RAM to Direct RAM

TASM - Table Driven Assembler                                Page 36

MOV     <direct>,@R1        Move Indirect RAM to Direct RAM

MOV     <direct>,#<immed>   Move Immediate data to Direct RAM

MOV     <direct>,<direct>   Move Direct RAM to Direct RAM

MOVC    A,@A+DPTR           Move code byte relative to DPTR to Acc

MOVC    A,@A+PC             Move code byte relative to PC to Acc

MOVX    A,@R0               Move external RAM to Acc

MOVX    A,@R1               Move external RAM to Acc

MOVX    A,@DPTR             Move external RAM to Acc (16 bit addr)

MOVX    @R0,A               Move Acc to external RAM

MOVX    @R1,A               Move Acc to external RAM

MOVX    @DPTR,A             Move Acc to external RAM (16 bit addr)

MUL     AB                  Multiply Acc by B

NOP                         No operation

ORL     A,Rn                OR Register and Acc

ORL     A,@R0               OR Indirect RAM and Acc

ORL     A,@R1               OR Indirect RAM and Acc

ORL     A,#<immed>          OR Immediate data and Acc

ORL     A,<direct>          OR Direct RAM and Acc

ORL     C,/<direct>         OR Complement of direct bit to Carry

ORL     C,<direct>          OR direct bit to Carry

ORL     <direct>,A          OR Acc to direct RAM

ORL     <direct>,#<immed>   OR Immediate data and direct RAM

POP     <direct>            Pop  from Stack and put in Direct RAM

PUSH    <direct>            Push from Direct RAM to Stack

RET                         Return from subroutine

RETI                        Return from Interrupt

RL      A                   Rotate Acc left

RLC     A                   Rotate Acc left through Carry

RR      A                   Rotate Acc right

RRC     A                   Rotate Acc right through Carry

SETB    C                   Set the Carry Bit

SETB    <bit>               Set Direct Bit

SJMP    <rel>               Short jump

SUBB    A,Rn                Subtract Register from Acc with Borrow

SUBB    A,@R0               Subtract Indirect RAM from Acc w/ Borrow

SUBB    A,@R1               Subtract Indirect RAM from Acc w/ Borrow

SUBB    A,#<immed>          Subtract Immediate data from Acc w/ Borrow

SUBB    A,<direct>          Subtract Direct RAM from Acc w/ Borrow

SWAP    A                   Swap nibbles of Acc

XCH     A,Rn                Exchange Acc with Register

XCH     A,@R0               Exchange Acc with Indirect RAM

XCH     A,@R1               Exchange Acc with Indirect RAM

XCH     A,<direct>          Exchange Acc with Direct RAM

XCHD    A,@R0               Exchange Digit in Acc with Indirect RAM

TASM - Table Driven Assembler                                Page 37

XCHD    A,@R1               Exchange Digit in Acc with Indirect RAM

XRL     A,Rn                Exclusive OR Register and Acc

XRL     A,@R0               Exclusive OR Indirect RAM and Acc

XRL     A,@R1               Exclusive OR Indirect RAM and Acc

XRL     A,#<immed>          Exclusive OR Immediate data and Acc

XRL     A,<direct>          Exclusive OR Direct RAM and Acc

XRL     <direct>,A          Exclusive OR Acc to direct RAM

XRL     <direct>,#<immed>   Exclusive OR Immediate data and direct RAM

Note  that  the above tables do not automatically define the various

mnemonics that may be  used  for  addressing  the  special  function

registers  of  the  8051.  The  user  may  wish  to set up a file of

equates (EQU's) that can be included in the  source  file  for  this

purpose.  The following illustrates some of the appropriate equates:

P0      .equ    080H    ;Port 0

SP      .equ    081H    ;Stack pointer

DPL     .equ    082H

DPH     .equ    083H

PCON    .equ    087H

TCON    .equ    088H

TMOD    .equ    089H

TL0     .equ    08AH

TL1     .equ    08BH

TH0     .equ    08CH

TH1     .equ    08DH

P1      .equ    090H    ;Port 1

SCON    .equ    098H

SBUF    .equ    099H

P2      .equ    0A0H    ;Port 2

IEC     .equ    0A8H

P3      .equ    0B0H    ;Port 3

IPC     .equ    0B8H

PSW     .equ    0D0H

ACC     .equ    0E0H    ;Accumulator

B       .equ    0F0H    ;Secondary Accumulator

;Now some bit addresses

P0.0    .equ    080H    ;Port 0 bit 0

P0.1    .equ    081H    ;Port 0 bit 1

P0.2    .equ    082H    ;Port 0 bit 2

P0.3    .equ    083H    ;Port 0 bit 3

P0.4    .equ    080H    ;Port 0 bit 4

P0.5    .equ    081H    ;Port 0 bit 5

P0.6    .equ    082H    ;Port 0 bit 6

P0.7    .equ    083H    ;Port 0 bit 7

ACC.0   .equ    0E0H    ;Acc bit 0

ACC.1   .equ    0E1H    ;Acc bit 1

ACC.2   .equ    0E2H    ;Acc bit 2

ACC.3   .equ    0E3H    ;Acc bit 3

ACC.4   .equ    0E4H    ;Acc bit 4

ACC.5   .equ    0E5H    ;Acc bit 5

ACC.6   .equ    0E6H    ;Acc bit 6

ACC.7   .equ    0E7H    ;Acc bit 7

See the manufacturer's data sheets for more information.

TASM - Table Driven Assembler                                Page 38

8085 INSTRUCTIONS AND ADDRESSING MODES

The following list shows the acceptable opcode mnemonics  and  their

corresponding  operand  formats for the 8085  version of TASM.   The

following symbols are used in the table:

SYMBOLIC            DESCRIPTION

-----------------------------------------------

<addr>              Absolute address (16 bits)

<data>              Immediate data (8 bits)

<data16>            Immediate data (16 bits)

<reg>               Register (A,B,C,D,E,H,L)

<rp>                Register pair (B,D,H,SP)

<port>              Port address (0-255)

<int>               Interrupt level (0 - 7)

Any valid TASM expression can appear in the  place  of  any  of  the

above symbolics except <reg>, <rp> and <int>.

OPCODE  OPERAND             DESCRIPTION

--------------------------------------------------------------------

ACI      <data>         Add immediate to A with carry

ADC      <reg>          Add <reg> to A with carry

ADC      M              Add indirect memory (HL) with carry

ADD      <reg>          Add <reg> to A

ADD      M              Add indirect memory (HL) to A

ADI      <data>         Add immediate to A

ANA      <reg>          And register with A

ANA      M              And indirect memory (HL) to A

ANI      <data>         And immediate to A

CALL     <addr>         Call subroutine at <addr>

CC       <addr>         Call subroutine if carry set

CNC      <addr>         Call subroutine if carry clear

CZ       <addr>         Call subroutine if zero

CNZ      <addr>         Call subroutine if non zero

CP       <addr>         Call subroutine if positive

CM       <addr>         Call subroutine if negative

CPE      <addr>         Call subroutine if even parity

CPO      <addr>         Call subroutine if odd  parity

CMA                     Complement A

CMC                     Complemennt carry

CMP      <reg>          Compare register with A

CMP      M              Compare indirect memory (HL) with A

CPI      <data>         Compare immediate data with A

DAA                     Decimal adjust A

DAD      <rp>           Add register pair to HL

DCR      <reg>          Decrement register

DCR      M              Decrement indirect memory (HL)

DCX      <rp>           Decrement register pair

TASM - Table Driven Assembler                                Page 39

DI                      Disable interrupts

EI                      Enable interrupts

HLT                     Halt

IN       <port>         Input on port

INR      <reg>          Increment register

INR      M              Increment indirect memory (HL)

INX      <rp>           Increment register pair

JMP      <addr>         Jump

JC       <addr>         Jump if carry set

JNC      <addr>         Jump if carry clear

JZ       <addr>         Jump if zero

JNZ      <addr>         Jump if not zero

JM       <addr>         Jump if minus

JP       <addr>         Jump if plus

JPE      <addr>         Jump if parity even

JPO      <addr>         Jump if parity odd

LDA      <addr>         Load A direct from memory

LDAX     B              Load A indirect from memory using BC

LDAX     D              Load A indirect from memory using DE

LHLD     <addr>         Load HL direct from memory

LXI      <rp>,<data16>  Load register pair with immediate data

MOV      <reg>,<reg>    Move register to register

MOV      <reg>,M        Move indirect memory (HL) to register

MVI      <reg>,<data>   Move immediate data to register

NOP                     No operation

ORA      <reg>          Or register with A

ORA      M              Or indirect memory (HL) with A

ORI      <data>         Or immediate data to A

OUT      <port>         Ouput to port

PCHL                    Jump to instruction at (HL)

POP      <rp>           Pop  register pair (excluding SP) from stack

PUSH     <rp>           Push register pair (excluding SP) onto stack

POP      PSW            Pop  PSW from stack

PUSH     PSW            Pop  PSW onto stack

RAL                     Rotate A left  with carry

RAR                     Rotate A right with carry

RLC                     Rotate A left  with branch carry

RRC                     Rotate A right with branch carry

RET                     Return from subroutine

RZ                      Return if zero

RNZ                     Return if non zero

RC                      Return if carry set

RNC                     Return if carry clear

RM                      Return if minus

TASM - Table Driven Assembler                                Page 40

RP                      Return if plus

RPE                     Return if parity even

RPO                     Return if parity odd

RIM                     Read interrupt mask

RST      <int>          Restart at vector <int>

SBB      <reg>          Subtract <reg> from A         with borrow

SBB      M              Subtract indirect memory (HL) with borrow

SBI      <data>         Subtract immediate from A     with borrow

SUB      <reg>          Subtract <reg> from A

SUB      M              Subtract indirect memory (HL) from A

SUI      <data>         Subtract immediate from A

SHLD     <addr>         Store HL

SIM                     Store Interrupt mask

SPHL                    Exchange SP with HL

STA      <addr>         Store A direct memory

STAX     B              Store A indirect using BC

STAX     D              Store A indirect using DE

STC                     Set carry

XRA      <reg>          Exclusive OR A with register

XRA      M              Exclusive Or A with indirect memory (HL)

XRI      <data>         Exclusive Or A with immediate data

XCHG                    Exchange DE with HL

XTHL                    Exchange HL with top of stack

See the manufacturer's data sheets for more information.

TASM - Table Driven Assembler                                Page 41

Z80 INSTRUCTIONS AND ADDRESSING MODES

The following list shows the acceptable opcode mnemonics  and  their

corresponding   operand  formats for the Z80  version of TASM.   The

following symbols are used in the table:

SYMBOLIC            DESCRIPTION

-----------------------------------------------

<addr>              Absolute address (16 bits)

<bit>               Bit address

<data>              Immediate data (8 bits)

<data16>            Immediate data (16 bits)

<disp>              Relative address

<reg>               Register (A, B, C, D, E, H, or L)

<rp>                Register pair (BC, DE, HL, or SP)

<port>              Port (0 - 255)

<cond>              Condition

NZ - not zero

Z  - zero

NC - not carry

C  - carry

PO - parity odd

PE - parity even

P  - positive

M  - minus

Any valid TASM expression can appear in the  place  of the  <addr>,

<data>, <data16>, or <disp> symbolics.

OPCODE  OPERAND             DESCRIPTION

--------------------------------------------------------------------

ADC  A,<data>           Add immediate with carry to accumulator

ADC  A,<reg>            Add register with carry to accumulator

ADC  A,(HL)             Add indirect memory with carry to accumulator

ADC  A,(IX+<disp>)      Add indirect memory with carry to accumulator

ADC  A,(IY+<disp>)      Add indirect memory with carry to accumulator

ADC  HL,<rp>            Add register pair with carry to HL

ADD  A,<data>           Add immediate to accumulator

ADD  A,<reg>            Add register to accumulator

ADD  A,(HL)             Add indirect memory to accumulator

ADD  A,(IX+<disp>)      Add indirect memory to accumulator

ADD  A,(IY+<disp>)      Add indirect memory to accumulator

ADD  HL,<rp>            Add register pair to HL

ADD  IX,<rp>            Add register pair to index register

ADD  IY,<rp>            Add register pair to index register

AND  <data>             And immediate with accumulator

AND  <reg>              And register  with accumulator

AND  (HL)               And memory with accumulator

AND  (IX+<disp>)        And memory with accumulator

AND  (IY+<disp>)        And memory with accumulator

BIT  <bit>,<reg>        Test <bit> in register

BIT  <bit>,(HL)         Test <bit> in indirect memory

BIT  <bit>,(IY+<disp>)  Test <bit> in indirect memory

TASM - Table Driven Assembler                                Page 42

BIT  <bit>,(IX+<disp>)  Test <bit> in indirect memory

CALL <addr>             Call the routine at <addr>

CALL <cond>,<addr>      Call the routine if <cond> is satisfied

CCF                     Complement carry flag

CP   <data>             Compare immediate data with accumulator

CP   <reg>              Compare register with accumulator

CP   (HL)               Compare indirect memory with accumulator

CP   (IX+<disp>)        Compare indirect memory with accumulator

CP   (IY+<disp>)        Compare indirect memory with accumulator

CPD                     Compare accumulator with memory and

decrement address and byte counters

CPDR                    Compare accumulator with memory and

decrement address and byte counter,

continue until match is found or

byte counter is zero

CPI                     Compare accumulator with memory and

increment address and byte counters

CPIR                    Compare accumulator with memory and

increment address and byte counter,

continue until match is found or

byte counter is zero

CPL                     Complement the accumulator

DAA                     Decimal adjust accumulator

DEC  <reg>              Decrement register contents

DI                      Disable interrupts

DJNZ <disp>             Decrement reg B and jump relative if reg B zero

EI                      Enable interrupts

EX   AF,AF'             Exchange program status and alternate program stat

EX   DE,HL              Exchange DE and HL contents

EX   (SP),HL            Exchange contents of HL and top of stack

EX   (SP),IX            Exchange contents of IX and top of stack

EX   (SP),IY            Exchange contents of IY and top of stack

EXX                     Exchange register pairs and alternate reg pairs

HALT                    Program execution stops

IM   0                  Interrupt mode 0

IM   1                  Interrupt mode 1

IM   2                  Interrupt mode 2

IN   A,<port>           Input port to accumulator

INC  <reg>              Increment contents of register

INC  <rp>               Increment contents of register pair

INC  IX                 Increment IX

INC  IY                 Increment IY

INC  (HL)               Increment indirect memory

INC  (IX+<disp>)        Increment indirect memory

INC  (IY+<disp>)        Increment indirect memory

IND                     Input to memory and decrement pointer

INDR                    Input to memory and decrement pointer until

byte counter is zero

INI                     Input to memory and increment pointer

INIR                    Input to memory and increment pointer until

byte counter is zero

IN   <reg>,(C)          Input to register

TASM - Table Driven Assembler                                Page 43

JP   <addr>             Jump to location

JP   <cond>,<addr>      Jump to location if condition satisifed

JP   (HL)               Jump to location pointed to by HL

JP   (IX)               Jump to location pointed to by IX

JP   (IY)               Jump to location pointed to by IY

JR   <disp>             Jump relative

JR   C,<disp>           Jump relative if carry is set

JR   NC,<disp>          Jump relative if carry bit is reset

JR   NZ,<disp>          Jump relative if zero flag is reset

JR   Z,<disp>           Jump relative if zero flag is set

LD   A,I                Move interrupt vector contents to accumulator

LD   A,R                Move refresh reg contents to accumulator

LD   A,(<addr>)         Load accumulator indirect from memory

LD   A,(<rp>)           Load accumulator indirect from memory by <rp>

LD   <reg>,<reg>        Load source register to destination register

LD   <rp>,(<addr>)      Load register pair indirect from memory

LD   IX,(<addr>)        Load IX indirect from memory

LD   IY,(<addr>)        Load IY indirect from memory

LD   I,A                Load interrup vector from accumulator

LD   R,A                Load refresh register from accumulator

LD   <reg>,<data>       Load register with immediate data

LD   <rp>,<data16>      Load register pair with immediate data

LD   IX,<data16>        Load  IX  with immediate data

LD   IY,<data16>        Load  IY  with immediate data

LD   <reg>,(HL)         Load register indirect from memory

LD   <reg>,(IX+<disp>)  Load register indirect from memory

LD   <reg>,(IY+<disp>)  Load register indirect from memory

LD   SP,HL              Load contents of HL to stack pointer

LD   SP,IX              Load contents of IX to stack pointer

LD   SP,IY              Load contents of IY to stack pointer

LD   (addr),A           Load contents of A to memory

LD   (<addr>),HL        Load contents of HL to memory

LD   (<addr>),<rp>      Load contents of register pair to memory

LD   (<addr>),IX        Load contents of IX to memory

LD   (<addr>),IY        Load contents of IY to memory

LD   (HL),<data>        Load immediate into indirect memory

LD   (IX+<disp>),<data> Load immediate into indirect memory

LD   (IY+<disp>),<data> Load immediate into indirect memory

LD   (HL),<reg>         Load register  into indirect memory

LD   (IX+<disp>),<reg>  Load register  into indirect memory

LD   (IY+<disp>),<reg>  Load register  into indirect memory

LD   (<rp>),A           Load accumulator into indirect memory

LDD                     Transfer data between memory and decrement

destination and source addresses

LDDR                    Transfer data between memory until byte

counter is zero,

decrement destintation and source addresses

LDI                     Transfer data between memory and increment

destination and source addresses

LDIR                    Transfer data between memory until byte

counter is zero,

increment destination and source addresses

NEG                     Negate contents of accumulator

NOP                     No operation

OR   <data>             Or immediate with accumulator

TASM - Table Driven Assembler                                Page 44

OR   <reg>              Or register with accumulator

OR   (HL)               Or indirect memory with accumulator

OR   (IX+<disp>)        Or indirect memory with accumulator

OR   (IY+<disp>)        Or indirect memory with accumulator

OUT  (C),<reg>          Output from registor

OUTD                    Output from memory, decrement address

OTDR                    Output from memory, decrement address

continue until reg B is zero

OUTI                    Output from memory, increment address

OTIR                    Output from memory, increment address

continue until reg B is zero

OUT  <port>,A           Output from accumulator

POP  <rp>               Load register pair from top of stack

POP  IX                 Load IX from top of stack

POP  IY                 Load IY from top of stack

PUSH <rp>               Store resister pair on top of stack

PUSH IX                 Store IX on top of stack

PUSH IY                 Store IY on top of stack

RES  <bit>,<reg>        Reset register bit

RES  <bit>,(HL)         Reset bit at indirect memory location

RES  <bit>,(IX+disp)    Reset bit at indirect memory location

RES  <bit>,(IY+<disp>)  Reset bit at indirect memory location

RET                     Return from subroutine

RET  <cond>             Return from subroutine if condition is satisfied

RETI                    Return from interrupt

RETN                    Return from non-maskable interrupt

RL   <reg>              Rotate left through carry register contents

RL   (HL)               Rotate left through carry indirect memory

RL   (IX+<disp>)        Rotate left through carry indirect memory

RL   (IY+<disp>)        Rotate left through carry indirect memory

RLA                     Rotate left through carry accumulator

RLC  <reg>              Rotate left branch  carry register contents

RLC  (HL)               Rotate left branch  carry indirect memory

RLC  (IX+<disp>)        Rotate left branch  carry indirect memory

RLC  (IY+<disp>)        Rotate left branch  carry indirect memory

RLCA                    Rotate left accumulator

RLD                     Rotate one BCD digit left between the

accumulator and memory

RR   <reg>              Rotate right through carry register contents

RR   (HL)               Rotate right through carry indirect memory

RR   (IX+<disp>)        Rotate right through carry indirect memory

RR   (IY+<disp>)        Rotate right through carry indirect memory

RRA                     Rotate right through carry accumulator

RRC  <reg>              Rotate right branch  carry register contents

RRC  (HL)               Rotate right branch  carry indirect memory

RRC  (IX+<disp>)        Rotate right branch  carry indirect memory

RRC  (IY+<disp>)        Rotate right branch  carry indirect memory

RRCA                    Rotate right branch  carry accumulator

RRD                     Rotate one BCD digit right between the

accumulator and memory

RST                     Restart

SBC  A,<data>           Subtract data            from A with borrow

SBC  A,<reg>            Subtract register        from A with borrow

SBC  A,(HL)             Subtract indirect memory from A with borrow

SBC  A,(IX+<disp>)      Subtract indirect memory from A with borrow

SBC  A,(IY+<disp>)      Subtract indirect memory from A with borrow

SBC  HL,<rp>            Subtract register pair from HL with borrow

TASM - Table Driven Assembler                                Page 45

SCF                     Set carry flag

SET  <bit>,<reg>        Set register bit

SET  <bit>,(HL)         Set indirect memory bit

SET  <bit>,(IX+<disp>)  Set indirect memory bit

SET  <bit>,(IY+<disp>)  Set indirect memory bit

SLA  <reg>              Shift register left arithmetic

SLA  (HL)               Shift indirect memory left arithmetic

SLA  (IX+<disp>)        Shift indirect memory left arithmetic

SLA  (IY+<disp>)        Shift indirect memory left arithmetic

SRA  <reg>              Shift register right arithmetic

SRA  (HL)               Shift indirect memory right arithmetic

SRA  (IX+<disp>)        Shift indirect memory right arithmetic

SRA  (IY+<disp>)        Shift indirect memory right arithmetic

SRL  <reg>              Shift register right logical

SRL  (HL)               Shift indirect memory right logical

SRL  (IX+<disp>)        Shift indirect memory right logical

SRL  (IY+<disp>)        Shift indirect memory right logical

SUB  <data>             Subtract immediate from accumulator

SUB  <reg>              Subtract register from accumulator

SUB  (HL)               Subtract indirect memory from accumulator

SUB  (IX+<disp>)        Subtract indirect memory from accumulator

SUB  (IY+<disp>)        Subtract indirect memory from accumulator

XOR  <data>             Exclusive or immediate with accumulator

XOR  <reg>              Exclusive or register with accumulator

XOR  (HL)               Exclusive or indirect memory with accumulator

XOR  (IX+<disp>)        Exclusive or indirect memory with accumulator

XOR  (IY+<disp>)        Exclusive or indirect memory with accumulator

See the manufacturer's data sheets for more information.

TASM - Table Driven Assembler                                Page 46

TASM DISTRIBUTION FILES

TASM  is  distributed  in  two  different  packages  - an executable

package and a source package.  The files  associated  with  each  of

these are described below:

EXECUTABLE (SHAREWARE) PACKAGE

------------------------------------------------------------

1.  TASM.EXE          - TASM Assembler, executable

2.  TASM48.TAB        - 8048 Instruction definition table

3.  TASM51.TAB        - 8051 Instruction definition table

4.  TASM65.TAB        - 6502 Instruction definition table

5.  TASM85.TAB        - 8085 Instruction definition table

6.  TASM80.TAB        - Z80  Instruction definition table

7.  TASM.DOC          - TASM Documentation

8.  README            - Brief Explanation of Disk contents

SOURCE PACKAGE

------------------------------------------------------------

1.  TASM.EXE          - TASM Assembler, executable

2.  TASM48.TAB        - 8048 Instruction definition table

3.  TASM51.TAB        - 8051 Instruction definition table

4.  TASM65.TAB        - 6502 Instruction definition table

5.  TASM85.TAB        - 8085 Instruction definition table

6.  TASM80.TAB        - Z80  Instruction definition table

7.  TASM.DOC          - TASM Documentation

8.  README            - Brief Explanation of Disk contents

9.  TASM.C            - TASM mainline source code

10. STRLIB.C          - String library

11. IOLIB.C           - I/O Library

12. MACRO.C           - Assembler directive support library

13. CMAIN.C           - Command line parser

14. CT.ASM            - MSDOS/C Interface module (startup)

15. FUNC.ASM          - Assembly language functions

16. UBYTE.H           - Header file defining unsigned types

17. TASM.H            - Header file defining TASM constants

18. TASMBLD.BAT       - Batch file to build TASM.EXE

The  'C'  modules  can  be compiled with the Microsoft 'C' compiler,

version 3.0   using  the  small  memory  model.  It  should  not  be

difficult to use other 'C' compilers, however, because no library is

needed and typical portability problems have been avoided.

The  assembler  files  can  be  assembled  using the Microsoft Macro

Assembler (MASM) or  equivalent.  The  standard  link  utility  that

comes  with  MSDOS (Microsoft LINK) can be used to perform the link.

The TASMBLD.BAT file demonstrates  steps  necessary  for  compiling,

assembling and linking.

TASM - Table Driven Assembler                                Page 47

PORTING TASM TO OTHER ENVIRONMENTS

TASM source code is fairly portable.  We,  however, have only tested

it in two environments,  Berkeley UNIX (BSD 4.2) and MSDOS 2.1 (UNIX

is  a  trademark  of  AT&T,  MSDOS  is  a  trademark  of Microsoft).

Portability is enhanced by:

1.  All identifiers are unique to 6 characters.

2.  No floating point arithmetic.

3.  Limited use of Long (32 bit) arithmetic.

4.  No bit fields.

5.  Self contained code (no library routines are necessary).

It  should  be  noted that a number of the modules that are included

with TASM are not  necessary  in  some  other  environments  because

identical  facilities  are  part of the standard library.  TASM does

not use any libraries from other vendors (including the Microsoft  C

library  that comes with the compiler that was used to compile TASM)

to ensure  no  copyrights  are  violated.  The  modules  that  would

generally not be needed include:

1.  iolib.c     This includes read(), write(), open(),

close(), sprintf(), etc.

2.  strlib.c    This includes strcpy(), strcmp(), isdigit(),

isalnum(), etc.

3.  cmain.c     Parses command line into argc, argv format.

4.  ct.asm      Interface between MSDOS and Microsoft C.

5.  func.asm    DOS interface functions.

For example, the following command can be used to make an executable

version of TASM for the BSD 4.X environment:

%cc -DUNIX tasm.c macro.c  -o tasm

The header files tasm.h and ubyte.h should also be available.

Note  that the UNIX flag is being defined on the command line.  This

causes the following things to happen:

1.  Tasm.h includes sys/file.h instead of defining  its  own

file open constants.

2.  TASM declares a 64 Kbyte array  in  which  to  hold  the

assembled   opcodes   and   data.   In   the   MSDOS

environment  the  'far'  keyword  is  used   in  the

declaration  of that large array.   The Microsoft  C

compiler  causes that array to be put in its own  64

Kbyte segment.  This is done so a small  data  model

can  be used  by  the compiler (since many compilers

for  the MSDOS environment do not have   any   other

option   -done   for  portability  to other MSDOS  C

compilers).  In the BSD UNIX environment,  64  Kbyte

arrays are no problem,   and  if you are porting the

code  to  a like environment (whether UNIX or  not),

TASM - Table Driven Assembler                                Page 48

the UNIX flag may be  useful. Also,  if you are only

interested  in  the 8048 version,  which really only

needs a  4  Kbyte  memory  space,  the  local  array

approach  would  suffice  and eliminate the need for

the assembly language calls.

For information on building TASM in the MSDOS environment,  see  the

TASMBLD.BAT file.

TASM - Table Driven Assembler                                Page 49

TASM INSTRUCTION SET TABLE DEFINITION

The tables that control TASM's interpretation of the source file are

read from a file at run time.  The table file name is determined  by

taking  the  numeric option field specified on the TASM command line

and appending it to the string "TASM", then a  ".TAB"  extension  is

added.  Thus, if the following command line is entered:

tasm -51  test.asm

then TASM would read the table file named "TASM51.TAB".

The following rules apply to the structure of the table file:

1.  The  first  line  of  the  file  should contain a string

surrounded by double quotes that should identify the

version of the assembler  table.  This  string  will

appear at the top of each page in the list file.  It

should be limited to 24 characters.

2.  Any  line  whose  first  character  is not alphabetic is

considered to be a comment and is discarded.

3.  Any  line  that has an alphabetic character as the first

character is assumed to be an instruction definition

record  and  is  parsed  to   build   the   internal

representation   of   the  instruction  set  tables.

Eight  fields   (separated  by  white  space)    are

expected,  as follows:

Field Name      Description

--------------------------------------------

INSTRUCTION     Instruction Mnemonic

ARGS            Argument definition

OPCODE          Opcode value

NBYTES          Number of bytes

MODOP           Modifier operation

CLASS           Instruction class

SHIFT           Argument left shift count

OR              Argument bitwise OR mask

INSTRUCTION.  The  INSTRUCTION  field should contain

the string to be  used  as  the  mnemonic  for  this

instruction.  Upper case letters should be used (the

source statements are converted to upper case before

comparison).

ARGS.   The  ARGS  field  should  contain  a  string

describing  the  format  of  the operand field.  All

characters are taken literally except the '*'  which

denotes  the  presence  of  a valid TASM expression.

Multiple '*'s can be used,  but all but the last one

must  be  followed  by  a  comma.  If  a  single '*'

TASM - Table Driven Assembler                                Page 50

appears in the ARGS field,  then the default  action

of  TASM  will  be  to  determine  the  value of the

expression that matches the field and insert one  or

two  bytes  of  it into the object file depending on

the NBYTES field.  If multiple '*'s are  used,  then

special  operators  (MODOP)  must  be  used  to take

advantage of them (see the examples below).  An ARGS

field of a pair  of  double  quotes  means  that  no

arguments are expected.

OPCODE.  The OPCODE field should contain the  opcode

value    (two  to  six  hex   digits)   for   this

instruction  and  address mode.   Each pair  of  hex

digits  represent  a  single  byte  of  the  opcode,

ordered with the right most pair being placed in the

lowest memory location.

NBYTES.  The NBYTES field should specify the  number

of  bytes  this  instruction  is to occupy (a single

decimal  digit).   This number includes both  opcode

bytes and argument bytes, thus,  the number of bytes

of argument is computed by subtracting the number of

bytes  of  opcode  (dictated by the  length  of  the

OPCODE field) from NBYTES.

MODOP.  The MODOP field determines  if  any  special

operations   need   to  be  performed  on  the  code

generated for this  instruction.  For  example,  the

zero-page  addressing  mode of the 6502 is a special

case of the absolute addressing mode, and is handled

by a special MODOP code (see appendix B).  The  list

of operators is as follows:

MODOP       DESCRIPTION

---------------------------------------------------

NOTOUCH     Do nothing to instruction or args

JMPPAGE     Put bits 8-10 of first arg into

bits 5-7 of opcode (8048 JMP)

ZPAGE       If arg < 256 then use zero-page (6502)

R1          Make arg relative to PC (single byte)

R2          Make arg relative to PC (two byte)

CREL        Combine LS bytes of first two args

making the second one relative to PC

SWAP        Swap bytes of first arg

COMBINE     Combine LS bytes of first two args into

first arg (arg1 -> LSB, arg2 ->MSB).

CSWAP       Combine LS bytes of first two args into

first arg and swap.

Note  that the reason for the combining of arguments

(COMBINE and CSWAP) is that TASM  assumes  that  all

object  bytes  to be inserted in the object file are

derived from a variable representing  the  value  of

TASM - Table Driven Assembler                                Page 51

the  first  argument (argval).  If two arguments are

in the  ARGS  field,  then  one  of  the  previously

mentioned  MODOP`s  must  be  used.  They  have  the

effect of combining the low bytes of the  first  two

arguments  into the variable (argval) from which the

object  code  will  be  generated.  TASM`s  argument

parsing   routine  can  handle  a  large  number  of

arguments,  but the code that generates  the  object

code is less capable.

CLASS.  The  CLASS  field is used to specify whether

this instruction is part of the standard instruction

set or a member of a set of  extended  instructions.

Bit 0 of this field should be set to denote a member

of  the standard instruction set.  Other bits can be

used as needed  to  establish  several  classes  (or

sets)   of  instructions  that  can  be  enabled  or

disabled via  the  '-x'  command  line  option  (see

section on 6502 INSTRUCTIONS AND ADDRESSING MODES).

SHIFT (optional).   The SHIFT field is used to cause

the  first  argument of the given instruction to  be

shifted left the specified number of bits.

OR  (optional).   The OR field is used to perform  a

bitwise  OR  with  the first argument of  the  given

instruction.  Specified as hex digits.

For  example  the  following  table   shows   possible   instruction

definition  records,  followed  by  possible  source statements that

would match it,  followed by the resulting object code that would be

generated (in hex):

EXAMPLE

INSTRUCTION DEFINITION                    SOURCE          OBJECT

-------------------------------------------------------------------

XYZ  *      FF   3  NOTOUCH 1             xyz 1234h       FF 34 12

XYZ  *      FF   2  NOTOUCH 1             xyz 1234h       FF 34

ZYX  *      FE   3  SWAP    1             zyx 1234h       FE 12 34

ZYX  *      FE   3  R2      1             zyx $+4         FE 01 00

ABC  *,*    FD   3  COMBINE 1             abc 45h,67h     FD 45 67

ABC  *,*    FD   3  CSWAP   1             abc 45h,67h     FD 67 45

ADD  A,#*   FC   2  NOTOUCH 1             add A,#'B'      FC 42

RET  ""     FB   1  NOTOUCH 1             ret             FB

LD   IX,*   21DD 4  NOTOUCH 1             ld  IX,1234h    DD 21 34 12

LD   IX,*   21DD 4  NOTOUCH 1 1 0         ld  IX,1234h    DD 21 68 24

LD   IX,*   21DD 4  NOTOUCH 1 0 1         ld  IX,1234h    DD 21 35 12

LD   IX,*   21DD 4  NOTOUCH 1 1 1         ld  IX,1234h    DD 21 69 24

LD   IX,*   21DD 4  NOTOUCH 1 8 12        ld  IX,34h      DD 21 12 34

Note that the only table in the standard TASM distribution that uses

the SHIFT/OR fields is the Z80 table.

See Appendix B for more examples.

The  order  of  the  entries for various addressing modes of a given

instruction is important.  Since the wild card matches anything,  it

is  important to specify the ARGS for the addressing modes that have

TASM - Table Driven Assembler                                Page 52

the  most  qualifying  characters  first.   For   example,   if   an

instruction  had  two  addressing  modes,   one  that  accepted  any

expression,  and another that required a pound sign in front  of  an

expression,  the  pound  sign  entry  should  go first otherwise all

occurrences of the instruction would match  the  more  general  ARGS

expression   that  it  encountered  first.   The  following  entries

illustrate the proper sequencing:

ADD #*  12 3 NOTOUCH 1

ADD *   13 3 NOTOUCH 1

TASM - Table Driven Assembler                                Page 53

SUMMARY

--------------------------------------------------------------------------------------------------------------------------

COMMAND LINE:                                               | DIRECTIVES:

tasm -vv [-bcdefhlmopqtx] src_file [obj_file [list_file]] |   #INCLUDE <filename> Read <filename> as source

|   #DEFINE  <label> [<macro_def>] Define a macro

-<vv>   Select version (-48, -51, -65, -85, -80)          |   #IFDEF   <label>    Assemble following if <label> DEFINEd

-o<nn>  Specify number of obj bytes per record (hex)      |   #IFNDEF  <label>    Assemble following if <label> not DEFd

-c      object file written as a contiguous block         |   #IF      <expr>     Assemble following if <expr> nonzero

-d<mac> define a macro                                    |   #ELSE               Alternate block to assemble

-f<xx>  Fill entire memory space with <xx> (hex)          |   #ENDIF              End of conditional block

-h      Produce hex table of the assembled code           |   .ORG     <expr>     Set Instruction Pointer

-l      Produce a label table in the listing              |   .BYTE    <expr>     Define a byte

-m      Produce object in MOS Technology format           |   .WORD    <expr>     Define a word (lsb, msb)

-b      Produce object in binary (.COM) format            |   .EQU     <expr>     Assign a value to label

-e      Expand the source lines in listing file           |   .DB      <expr>     Same as BYTE

-t<tab> Select version (if non numeric)                   |   .DW      <expr>     Same as WORD

-p      Page the listing file                             |   .TEXT    <string>   Define bytes as ASCII characters

-x<m>   Enable extended instruction set (m = class mask)  |   .BLOCK   <expr>     Block space reserve

-q      Quite, disable listing file                       |   .TITLE   <string>   Set a title to appear on listing

------------------------------------------------------------|   .EJECT              Do Top of Form on listing

OPERATORS:                                                  |   .LIST               Turn listing on

Operator  Type          Description                       |   .NOLIST             Turn listing off

__________________________________________                |   .PAGE               Turn paging on

+        Additive       addition                          |   .NOPAGE             Turn paging off

-                       subtraction                       |   .ADDINSTR <instdef> Add instruction

*        Multiplicative multiplication                    |   .END                End of source code

/                       division                          |   =        <expr>     Alternate form of EQU

%                       modulo                            |   *=       <expr>     Alternate form of ORG

<<                      logical shift left                |------------------------------------------------------

>>                      logical shift right               | CONSTANTS:

~        Unary          bit inversion (one's complement)  |   Numeric constants: <digits><suffix>

-                       unary negation                    |     <digits> = digit string taken from (0-9,a-f,A-F)

=        Relational     equal                             |     <suffix> = radix indicator defined as follows:

==                      equal                             |

!=                      not equal                         |     Suffix              Radix

<                       less than                         |     ------------------------

>                       greater than                      |     b or B               2   (or % prefix)

<=                      less than or equal                |     o or O               8   (or @ prefix)

>=                      greater than or equal             |     d or D (or nothing)  10

&       Binary          binary 'and'                      |     h or H               16  (or $ prefix)

|                       binary 'or'                       |

^                       binary 'exclusive or'             |   Character constants: '<char>' (printable ASCII

Note: 16 bit values used.  Relational Ops yield 1 or 0.     |                         character surrounded by single

|                         quotes, e.g. 'a'.

|

------------------------------------------------------------|   String constants: "<characters>" (printable ASCII

NOTES:                                                      |                      characters surrounded by double

1. Statement format:<label> <operation> <operand> <comment>|                      quotes, e.g. "This is a string".

2. The backslash can be used to separate multiple          |                      Used only for TEXT and TITLE.

statements on a line (e.g. lda byte1 \ sta byte2).  |

3. '$' or '*' represent the current Instruction Counter.   |

4. Comments preceded by semicolon (';').                   |

5. TASM is case insensitive for mnemonics, but not labels. |

6. The default object file format is Intel Hex.            |

7. 6502: -x3 enables R65C02 opcodes, -x5 enables R65C00/21 |

--------------------------------------------------------------------------------------------------------------------------

TASM - Table Driven Assembler                                Page 54

APPENDIX A - SAMPLE LISTING FILE

The listing was generated by issuing the command:

tasm -65 -p -l -h -f00 t.asm

TASM 6502 Assembler.            t.asm                         page 1

Speech Technology Incorporated.

0001   0000             ;This is a simple example of an assembly

0002   0000             ;include a file just for fun

0003   0000             #include        "include.h"

0001+  0000             ;this is just a simple include file

0002+  0000             exit    .equ    7e00h

0003+  0000             ;

0004+  0000             ; That's all

0004   0000             ;

0005   0000             ; define two byte increment macro

0006   0000             #define INCZ(p) ldx #p\ inc 0,x\ bne $+4\ inc 1,x

0007   0000             ;

0008   0000             start   .org    $0

0009   0000             mask    .equ    $5678

0010   0000

0011   0000 0A          byte1   .byte   10

0012   0001 D2 04       word1   .word   1234

0013   0003 34 12       word2   .word   $1234

0014   0005             ;skip past a few locations to start code

0015   0020                     .org    $0020

0016   0020             begin

0017   0020             ;     let's just increment word1 the number

0018   0020             ;     of times indicated by byte1 (use the

0019   0020             ;     double byte increment macro INCZ).

0020   0020             loop1

0021   0020 A2 01               ldx #word1\ inc 0,x\ bne $+4\ inc 1,x

0021   0022 F6 00

0021   0024 D0 02

0021   0026 F6 01

0022   0028 C6 00               dec     byte1   ;decrement the count

0023   002A D0 F4               bne     loop1   ;loop until zero

0024   002C

0025   002C             ; Now  let's just do some simple arithmetic

0026   002C A5 56               lda     (mask >> 8)

0027   002E AD 00 56            lda     (mask & 0ff00h)

0028   0031 A5 57               lda     ((mask >> 8) & 0ffh) + word1

0029   0033 4C 00 7E            jmp     exit

0030   0036                     .end

TASM - Table Driven Assembler                                Page 55

Label        Value      Label        Value      Label        Value

------------------      ------------------      ------------------

exit          7E00      start         0000      mask          5678

byte1         0000      word1         0001      word2         0003

begin         0020      loop1         0020

ADDR  00 01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F

-----------------------------------------------------

0000  0A D2 04 34 12 00 00 00 00 00 00 00 00 00 00 00

0010  00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00

0020  A2 01 F6 00 D0 02 F6 01 C6 00 D0 F4 A5 56 AD 00

0030  56 A5 57 4C 00 7E 00 00 00 00 00 00 00 00 00 00

tasm: Number of errors = 0

TASM - Table Driven Assembler                                Page 56

APPENDIX B - SAMPLE INSTRUCTION DEFINITION TABLE (TASM65.TAB)

"TASM 6502 Assembler.    "

/* This is the instruction set definition table

/*   for the 6502 version of TASM.

/*      Thomas N. Anderson, Speech Technology Incorporated, Feb 1986.

/*  First line of this file is a banner that will appear at the

/*  top of each page of the TASM listing file (not the same as

/*  the TITLE).  Should be limited to 24 characters.

/*  Any other line that does not start with an upper case letter is

/*  ignored.

/*  See TASM manual for info on table structure.

/*  Note that there are two classes of extended instructions beyond

/*    the standard set.  The classes are assigned bits as follows:

/*      bit 0 = standard set

/*      bit 1 = extended instructions for R65C02

/*      bit 2 = extended instructions for R65C00/21

/*

/*INSTR ARGS OPCODE BYTES MOD CLASS SHIFT OR */

/*-------------------------------------------*/

ADC  #*     69    2  NOP 1

ADC  (*,X)  61    2  NOP 1

ADC  (*),Y  71    2  NOP 1

ADC  (*)    72    2  NOP 2   /* R65C02 */

ADC  *,X    7D    3  ZP  1

ADC  *,Y    79    3  NOP 1

ADC  *      6D    3  ZP  1

AND  #*     29    2  NOP 1

AND  (*,X)  21    2  NOP 1

AND  (*),Y  31    2  NOP 1

AND  (*)    32    2  NOP 2   /* R65C02 */

AND  *,X    3D    3  ZP  1

AND  *,Y    39    3  NOP 1

AND  *      2D    3  ZP  1

ASL  A      0A    1  NOP 1

ASL  *,X    1E    3  ZP  1

ASL  *      0E    3  ZP  1

BCC  *      90    2  R1  1

BCS  *      B0    2  R1  1

BEQ  *      F0    2  R1  1

BNE  *      D0    2  R1  1

BMI  *      30    2  R1  1

BPL  *      10    2  R1  1

BVC  *      50    2  R1  1

BVS  *      70    2  R1  1

BIT  #*     89    2  NOP 2

BIT  *,X    3C    3  ZP  2

BIT  *      2C    3  ZP  1

BRK  ""     00    1  NOP 1

CLC  ""     18    1  NOP 1

TASM - Table Driven Assembler                                Page 57

CLD  ""     D8    1  NOP 1

CLI  ""     58    1  NOP 1

CLV  ""     B8    1  NOP 1

CMP  #*     C9    2  NOP 1

CMP  (*,X)  C1    2  NOP 1

CMP  (*),Y  D1    2  NOP 1

CMP  (*)    D2    2  NOP 2

CMP  *,X    DD    3  ZP  1

CMP  *,Y    D9    3  NOP 1

CMP  *      CD    3  ZP  1

CPX  #*     E0    2  NOP 1

CPX  *      EC    3  ZP  1

CPY  #*     C0    2  NOP 1

CPY  *      CC    3  ZP  1

DEC  A      3A    3  NOP 2

DEC  *,X    DE    3  ZP  1

DEC  *      CE    3  ZP  1

DEX  ""     CA    1  NOP 1

DEY  ""     88    1  NOP 1

EOR  #*     49    2  NOP 1

EOR  (*,X)  41    2  NOP 1

EOR  (*),Y  51    2  NOP 1

EOR  (*)    52    2  NOP 2

EOR  *,X    5D    3  ZP  1

EOR  *,Y    59    3  NOP 1

EOR  *      4D    3  ZP  1

INC  A      1A    3  NOP 2

INC  *,X    FE    3  ZP  1

INC  *      EE    3  ZP  1

INX  ""     E8    1  NOP 1

INY  ""     C8    1  NOP 1

JMP  (*,X)  7C    3  NOP 2

JMP  (*)    6C    3  NOP 1

JMP  *      4C    3  NOP 1

JSR  *      20    3  NOP 1

LDA  #*     A9    2  NOP 1

LDA  (*,X)  A1    2  NOP 1

LDA  (*),Y  B1    2  NOP 1

LDA  (*)    B2    2  NOP 2

LDA  *,X    BD    3  ZP  1

LDA  *,Y    B9    3  NOP 1

LDA  *      AD    3  ZP  1

LDX  #*     A2    2  NOP 1

LDX  *,Y    BE    3  ZP  1

LDX  *      AE    3  ZP  1

TASM - Table Driven Assembler                                Page 58

LDY  #*     A0    2  NOP 1

LDY  *,X    BC    3  ZP  1

LDY  *      AC    3  ZP  1

LSR  A      4A    1  NOP 1

LSR  *,X    5E    3  ZP  1

LSR  *      4E    3  ZP  1

NOP  ""     EA    1  NOP 1

ORA  #*     09    2  NOP 1

ORA  (*,X)  01    2  NOP 1

ORA  (*),Y  11    2  NOP 1

ORA  (*)    12    2  NOP 2

ORA  *,X    1D    3  ZP  1

ORA  *,Y    19    3  NOP 1

ORA  *      0D    3  ZP  1

PHA  ""     48    1  NOP 1

PHP  ""     08    1  NOP 1

PLA  ""     68    1  NOP 1

PLP  ""     28    1  NOP 1

ROL  A      2A    1  NOP 1

ROL  *,X    3E    3  ZP  1

ROL  *      2E    3  ZP  1

ROR  A      6A    1  NOP 1

ROR  *,X    7E    3  ZP  1

ROR  *      6E    3  ZP  1

RTI  ""     40    1  NOP 1

RTS  ""     60    1  NOP 1

SBC  #*     E9    2  NOP 1

SBC  (*,X)  E1    2  NOP 1

SBC  (*),Y  F1    2  NOP 1

SBC  (*)    F2    2  NOP 2

SBC  *,X    FD    3  ZP  1

SBC  *,Y    F9    3  NOP 1

SBC  *      ED    3  ZP  1

SEC  ""     38    1  NOP 1

SED  ""     F8    1  NOP 1

SEI  ""     78    1  NOP 1

STA  (*,X)  81    2  NOP 1

STA  (*),Y  91    2  NOP 1

STA  (*)    92    2  NOP 2

STA  *,X    9D    3  ZP  1

STA  *,Y    99    3  NOP 1

STA  *      8D    3  ZP  1

STX  *,Y    96    2  ZP  1

STX  *      8E    3  ZP  1

TASM - Table Driven Assembler                                Page 59

STY  *,X    94    2  NOP 1

STY  *      8C    3  ZP  1

TAX  ""     AA    1  NOP 1

TAY  ""     A8    1  NOP 1

TSX  ""     BA    1  NOP 1

TXA  ""     8A    1  NOP 1

TXS  ""     9A    1  NOP 1

TYA  ""     98    1  NOP 1

/* Here are the extended instructions that are totally new */

BRA  *      80    2  R1  6

BBR0 *,*    0f    3  CR  6

BBR1 *,*    1f    3  CR  6

BBR2 *,*    2f    3  CR  6

BBR3 *,*    3f    3  CR  6

BBR4 *,*    4f    3  CR  6

BBR5 *,*    5f    3  CR  6

BBR6 *,*    6f    3  CR  6

BBR7 *,*    7f    3  CR  6

BBS0 *,*    8f    3  CR  6

BBS1 *,*    9f    3  CR  6

BBS2 *,*    af    3  CR  6

BBS3 *,*    bf    3  CR  6

BBS4 *,*    cf    3  CR  6

BBS5 *,*    df    3  CR  6

BBS6 *,*    ef    3  CR  6

BBS7 *,*    ff    3  CR  6

MUL  ""     02    1  NOP 4  /* R65C00/21 only*/

PHX  ""     da    1  NOP 6

PHY  ""     5a    1  NOP 6

PLX  ""     fa    1  NOP 6

PLY  ""     7a    1  NOP 6

RMB0 *      07    2  NOP 6

RMB1 *      17    2  NOP 6

RMB2 *      27    2  NOP 6

RMB3 *      37    2  NOP 6

RMB4 *      47    2  NOP 6

RMB5 *      57    2  NOP 6

RMB6 *      67    2  NOP 6

RMB7 *      77    2  NOP 6

SMB0 *      87    2  NOP 6

SMB1 *      97    2  NOP 6

SMB2 *      a7    2  NOP 6

SMB3 *      b7    2  NOP 6

SMB4 *      c7    2  NOP 6

SMB5 *      d7    2  NOP 6

SMB6 *      e7    2  NOP 6

SMB7 *      f7    2  NOP 6

TASM - Table Driven Assembler                                Page 60

/* The following extended instructions are available on the

R65C02 but not the R65C00/21 */

STZ  *,X    9e    3  ZP  2

STZ  *      9c    3  ZP  2

TRB  *      1c    3  ZP  2

TSB  *      0c    3  ZP  2

TASM - Table Driven Assembler                                Page 61

APPENDIX C - ORDERING INFORMATION

TASM  is  distributed as shareware.  If you find it useful,  you are

encouraged to become a registered  user.  Registered  users  receive

the following benefits:

1.  The recent version of TASM.

2.  TASM source code (in C).

3.  Bound TASM manual.

4.  Knowledge that they are supporting  the  development  of

useful but inexpensive software.

DESCRIPTION                             UNIT PRICE            PRICE

--------------------------------------------------------------------

TASM Registration                            30.00           _______

TASM User's Manual (included above)          10.00           _______

TASM update for registered users             10.00           _______

(latest source, executable, and manual)

TASM shareware disk                          10.00           _______

Subtotal                                                     _______

Tax (Washington state residents add 7.9%)                    _______

Foreign orders (outside North America) add $10.00            _______

TOTAL (post paid)                                            _______

Shipping Address:

______________________________________________

______________________________________________

______________________________________________

Send check or money order (no credit cards) to:

Speech Technology Incorporated

Software Division

837 Front Street South

Issaquah,  WA  98027

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

The Public (Software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  The P(s)L cannot de-

bug programs over the telephone.

Disks in the P(s)L are updated monthly, so if you did not get

this disk  directly from the P(s)L,  you should be aware that

the files in this set may no  longer be the current versions.

For a copy of the latest monthly software library newsletter

and a list of the 1,000+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

(713) 721-6104 or (713) 721-5205