VGA TEXT ATTRIBUTES AND SPECIAL EFFECTS

by Chris Dunford

While working on a project recently, I realized that I didn't know very

much about how VGA text attributes work.  I knew that there wasn't a

simple one-to-one relationship between attribute numbers and colors, but

that's about it.  So, I did a little research and fooling around, and

found that lots of interesting effects are possible.  You can draw

people's attention to important screen information in ways more (and

less!) subtle than simply flashing it on and off or using intensified

colors.  For example, you can have colors that gradually shade from

normal to high intensity and back, "pulsing" on the screen; you can

display text that changes from one color to another (gradually shading

from blue to red and back, for example); you can soften blinking by

having the color "fade" rather than just disappear.  And, there are some

things you can do under software control that can't be done with the

usual hardware attributes; for example, you can use flashing

backgrounds, or you can using flashing foregrounds without sacrificing

high intensity backgrounds.  These can be done very efficiently, without

the need to change the attributes of individual characters.

This document is an attempt to share some of what I learned; it

explains how VGA text attributes work, and how these effects are

possible.  The archive that contains this document should also contain

a demo program and a linkable object module (with source) that allow

you to use the effects in your own programs.  You might want to do a

quick run of DEMO1 to see some of the possible effects in action.

What's different with the VGA?

------------------------------

The CGA implemented a direct relationship between a 4-bit attribute and

a color.  Each attribute mapped to one color, and the color could not be

changed.

That changed with the EGA (which had palette registers to play with),

and changed again, radically, with the VGA.  The attribute number

specifies the displayed color very indirectly; it is combined with bits

and pieces of 5 different internal VGA registers before it appears on

the screen as one of the 262,144 possible colors.  There are therefore

several ways to change a character's color other than by changing its

attribute.  And, of course, if you change the way an attribute is

displayed, all characters with that attribute are affected.  Thus, it is

possible to "design" attributes with interesting effects, and to alter

the way entire screen fields are displayed very efficiently.

Mapping attributes to colors

----------------------------

Before we start, let me note that we are working here with 4-bit

attributes.  The 8-bit attribute that's associated with a character is,

of course, two 4-bit attributes: one for the foreground, and one for the

background.  This document discusses how these 4-bit attributes become

colors; everything here applies equally to foreground and background

attributes (thus it is possible to use flashing backgrounds, etc.)

Screen colors in text modes are controlled by the VGA's attribute

controller.  In one of its two modes (the one that the PS/2 BIOS sets

by default), here is how a color is selected by an attribute:

AND

attribute |-------| color plane enable register

|

| selects

|

palette register 0-15     color select register

|                         |

|                         |

| bits 0-5       bits 6-7 |

+------------+------------+

|

|      AND

+-------+--------video DAC mask register

|

| selects

|

video DAC color register 0-255

This appears complex, but it can be simplified.  First, notice the two

logical AND operations.  Since both the color plane enable register and

the video DAC mask register normally contain all ones, they tend to drop

out of the picture, resulting in:

attribute 0-15

|

| selects

|

palette register 0-15     color select register

|                         |

|                         |

| bits 0-5       bits 6-7 |

+------------+------------+

|

| selects

|

video DAC color register 0-255

The 4-bit attribute (0-15) selects one of the 16 palette registers.  The

palette register contains a number.  This number is combined with the

contents of the color select register to form another number.  For

example, suppose:

attribute = 13

palreg[13] = 1Bh

CSR = 3

Attribute 13 selects palette register 13, which contains 1BH.  The color

select register contains 3. We put the contents of the color select

register in bits 6-7 of the final number, and the contents of palreg[13]

in bits 0-5:

3  1B

-  --

11 011011

The resulting number is 11011011 binary, 219 decimal.  This number

selects one of the 256 video DAC color registers.

A C-like formula for the video DAC color resgister selection is:

r = (p[a] & 0x3F) | ((c << 6) & 0xC0)

where r is the DAC color register number, p[] is the array of 16 palette

registers, a is the attribute number (0..15), and c is the current

contents of the color select register.

The color that will be displayed is the color defined by the selected

video DAC color register (DAC stands for Digital-to-Analog Converter).

Each video DAC register is an 18-bit register that contains three 6-bit

values: one for each of the red, green, and blue primary colors.  Each

value specifies the intensity of that primary color on the screen.  A

zero for a primary means that it's not included in the color; 63 is

maximum intensity.

Suppose, in our example, that video DAC register 219 contains the

numbers 0, 3FH, and 3FH (it's convenient to think of the register as

three 6-bit values rather than one 18-bit value).  This means that red

will be off, and green and blue will be at maximum intensity.  The color

displayed by attribute 13 will be bright cyan.

Notice that you could change the color of a character with FG attribute

N in four different ways:

1. Change the attribute

2. Change the contents of palreg[N]

3. Change the contents of the video DAC register specified by

palreg[N] and the color select register

4. Change the contents of the color select register

These all have different effects on the total screen display.  Changing

an attribute affects a single character; changing a palette register

affects all characters with that attribute; changing a video DAC

register affects any character whose attribute, when combined with color

select, selects that register--this could be several attributes;

changing the color select register could very well affect everything on

the screen.

Another way to visualize color selection

----------------------------------------

There is a simpler way to look at the process.  The two bits used from

the color select register always end up in the high two bits of the

video DAC color register number.  Thus, the selected DAC color register

will always be one of:

CSR=0: 00xx xxxx  (register range 0-63)

CSR=1: 01xx xxxx  (64-127)

CSR=2: 10xx xxxx  (128-191)

CSR=3: 11xx xxxx  (192-255)

The remaining 6 bits come directly from the selected palette register.

If we break the 256 video DAC color registers into four palettes of 64

colors each, as follows:

DAC registers   Palette

0-63          0

64-127          1

128-191          2

192-255          3

then the color select register selects one of the four 64-color

palettes, and the palette register (which is selected by the attribute)

selects one of the 64 colors of the CSR-selected palette.

Thus, it's simply stated:  the attribute selects one of the 64 colors

available from the palette selected by the color select register.

Mode 1

------

I mentioned that the above represents one of the two modes supported by

the attribute controller; call it "mode 0".  The other mode (mode 1) is

very similar; the only difference is in which bits are combined from the

palette register and the color select register.  The simplified diagram

is as follows:

attribute 0-15

|

| selects

|

palette register 0-15     color select register

|                         |

|                         |

| bits 0-3       bits 4-7 |

+------------+------------+

|

| selects

|

video DAC color register 0-255

The difference is that there are four bits each from the palette

register and the color select register (instead of 6 and 2 under the

mode 0).

A C-like formula for the video DAC color resgister selection in mode 1:

r = (p[a] & 0x0F) | ((c << 4) & 0xF0)

The effect of mode 1 is that the 256 video DAC color registers are

broken down into sixteen 16-color palettes instead of four 64-color

palettes:

DAC registers   Palette

0-15          0

16-31          1

32-47          2

...

240-255         15

The palette register (still selected by the attribute) selects one of

the 16 colors available in the palette selected by the current color

select register.

The more complex effects (graded color changes, pulsing, etc.) of the

demo program DEMO1 are generated under mode 1. This is because all 256

colors defined by the video DAC color registers can be accessed by

simply changing the contents of the color select register, which is very

efficient (it takes about 1/100th of a second).

Under mode 0, only 75% of the available colors can be accessed by just

changing the color select register (this is because there are 64 colors

per palette, but only 16 attributes).  To get to the remaining colors,

you have to change the palette registers.  But there are 16 palette

registers; if you need to alter more than one displayed color, this is

less efficient than using the color select register.

A mode 1 technique

------------------

As mentioned, there are a number of ways to alter displayed colors.  I

will concentrate on one technique: creating a series of palettes and

then switching palettes by systematically changing the contents of the

color select register.  This document concentrates on mode 1; some of

the same effects (such as periodic intensification and simulated

blinking) can be obtained in mode 0.

Note that the use of mode 1 largely dictates that you must reinitialize

the palette registers.  The VGA BIOS initializes some of these registers

with values greater than 15; these will be masked in mode 1 to the low

4 bits, yielding values in the range 0..15.  These values will probably

not result in the colors you intend (two attributes will yield red

foreground, for example).  The simplest solution is to just fill the

palette registers sequentially with numbers from 0..15; this largely

causes the palregs to drop out of the picture:  attribute N will map

directly to color N within the palette selected by the color select

register.

Assuming that the palette registers are set as described, create a

"base" palette in palette 0. That is, fill palette 0 (video DAC color

registers 0-15) with 16 base color definitions.  It makes sense to set

the standard color combinations for the 16 CGA-like colors that people

will expect to see (color intensity values in hex):

DAC regs  Attr  R   G   B   Color

0-2      0    0   0   0   Black

3-5      1    0   0  2A   Blue

6-8      2    0  2A   0   Green

9-11     3    0  2A  2A   Cyan

12-14    4   2A   0   0   Red

15-17    5   2A   0  2A   Magenta

18-20    6   2A  2A   0   Brown

21-23    7   2A  2A  2A   White

24-26    8    0   0  15   "Gray" (not really)

27-29    9    0   0  3F   Bright blue

30-32   10    0  3F   0   Bright green

33-35   11    0  3F  3F   Bright cyan

36-38   12    3F  0   0   Bright red

39-41   13    3F  0  3F   Bright magenta

42-44   14    3F 3F   0   Yellow

45-47   15    3F 3F  3F   Intense white

(In practice, I use 33h in place of 3Fh.  This leaves some room to flash

even the bright colors by intensification.)

The second step is to duplicate palette 0 into palettes 1-15, and then

vary the color definitions for attributes of interest in some systematic

way.  For example, let's specify that attribute 1 is not going to be

"blue"; it's going to be a green that continuously cycles from normal

intensity to high intensity and back.  The effect is similar to flashing

text, but more subtle.  To do this, set the colors for attribute 1 in

each of the palettes as follows:

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

G     2A 2C 2E 31 33 35 36 37 38 39 3A 3B 3C 3D 3E 3F

B      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

Note that the intensity of the green primary slowly increases from

normal (2A) in palette 0 to maximum intensity (3F) in palette 1. Also

note that attribute 1 no longer has anything to do with blue--it's

green.  (See below for information on how to set a video DAC color

register definition.)

If these values are loaded into the video DAC color registers, and you

then sequence through the palettes in a cyclical pattern (0->15->0),

characters with attribute 1 will slowly "pulse" from green to bright

green.  The effect is more subtle--and readable--than simply switching

the characters off and on.

The palette change is easily and efficiently accomplished by setting the

color select register--say, in a timer tick intercept routine.  Using a

timer intercept is an ideal way to accomplish this sort of special

effect.  Unlike hardware flashing, the special effects described in this

document must be accomplished by your software.  Using an externally

triggered routine allows this to occur in background; you simply set up

the attributes the way you want them and then go about your work.  The

special effects will continue to be generated even when you're in DOS or

BIOS, say, waiting for a keystroke.  The skeleton for a typical timer

routine might be (assume that DELTA, COUNT, and RESET are all

initialized to 1):

count = count - 1

if count = 0 then

palette = palette+delta

if palette > 15 then

palette = 15

delta = -delta

else if palette < 0 then

palette = 0

delta = -delta

end

set color select register to palette

count = reset

end

This is both efficient and flexible:  the blink rate can be adjusted by

altering the values of either DELTA and RESET (or both).  Increasing

DELTA causes the blink rate to speed up by skipping palettes; increasing

RESET slows the blinking by skipping clock ticks.  Be sure that PALETTE

and DELTA are treated as signed numbers.  (Note that FLASH1 uses a

slughtly fancier technique.)

Effects

-------

Many "special effects" are possible using this technique.  "Pulsing" has

already been discussed; here are some others:

PERIODIC INTENSIFICATION

Pulsing is a gradual change from a low intensity color to a high

intensity color.  The effect can be hardened by using the normal color

for half of the palettes and an intnsified version for the other half.

For example, the following palettes flash cyan to bright cyan:

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

G     2A 2A 2A 2A 2A 2A 2A 2A 3F 3F 3F 3F 3F 3F 3F 3F

B     2A 2A 2A 2A 2A 2A 2A 2A 3F 3F 3F 3F 3F 3F 3F 3F

The effect can be varied by altering the ratio of low intensity palettes

to high intensity palettes ("beacons", below, are simply extreme cases

of periodic intensification).

COLOR CHANGES

Another effect is to flash text from one color to another.  This set of

palettes flashes bright green text to bright red.  It's quite an eye

catcher:

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

G     3F 3F 3F 3F 3F 3F 3F 3F  0  0  0  0  0  0  0  0

B      0  0  0  0  0  0  0  0 3F 3F 3F 3F 3F 3F 3F 3F

GRADED COLOR CHANGES

An interesting effect is to "grade" one color into another.  Rather than

simply flashing green to red, change it gradually by fading out the

green primary and fading in the red:

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R      0  2  4  6  9 0C 0F 12 15 18 1B 1E 21 24 27 2A

G     2A 27 24 21 1E 1B 18 15 12 0F 0C  9  6  4  2  0

B      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

This attribute will still change from green to red, but the change is

not instantaneous; it slowly fades from one color to the other, passing

through various other unnamed colors (all combinations of red and green)

on the way.

SIMULATED FLASHING

Regular hardware flashing can be simulated via a variant of the above

scheme; simply use the background color for half of the palettes (or the

foreground color, if you want to flash the background):

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R     3F 3F 3F 3F 3F 3F 3F 3F  0  0  0  0  0  0  0  0

G     3F 3F 3F 3F 3F 3F 3F 3F  0  0  0  0  0  0  0  0

B      0  0  0  0  0  0  0  0 2A 2A 2A 2A 2A 2A 2A 2A

The example flashes yellow text if displayed on a blue background.

SOFTENED FLASHING

By placing the background attribute in half of the palettes (as for

flashing) and fading out the foreground in the other half, a softer

version of flashing is possible:

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

G     2A 24 1E 19 14 0F 0A  5  0  0  0  0  0  0  0  0

B     2A 24 1E 19 14 0F 0A  5  0  0  0  0  0  0  0  0

The example shows softened flashing of cyan text on a black background.

If any of the background primaries are also components of the

foreground color, do not fade those:

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

G     2A 24 1E 19 14 0F 0A  5  0  0  0  0  0  0  0  0

B     2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A

The example shows softened flashing of cyan on blue.

FADING

This is an even softer version of flashing text.  It fades the foregound

over the full 16 palettes by grading the foregound color into the

background color.

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

G     2A 27 24 21 1E 1B 18 15 12 0F 0C  9  6  4  2  0

B      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

The example fades a green foreground into a black background.  If the

background is a color other than black, just use a graded color change

from the FG color to the BG color.

STROBES AND BEACONS

These are two final sample effects.  A strobe is a color that's

invisible most of the time (BG=FG) but flashes briefly to maximum

intensity.  The follow palettes strobe yellow when used with a blue

background:

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0 3F

G      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0 3F

B     2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A 2A  0

A beacon is a low intensity color that briefly flashes to maximum

intensity.  The following palletes create a blue beacon:

Palette  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

R      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

G      0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0

B     20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 3F

Note to programmers:  if you use the routines provided in FLASH1.ASM,

note that only palettes that are a multiple of DELTA are ever used.

E.g., if DELTA is 2, palette 15 will not be used; put the strobe or

beacon color in palette 14 instead.

Advantages/disadvantages

------------------------

Creating special effects in this manner has several advantages over

using hardware effects (such as flashing), and some disdvantages.  Among

the advantages are:

- MANY more effects are possible; I have mentioned only a few of the

possibilities.

- Transition rates are under your control; the hardware blink rate

is fixed.

- Effects can be mixed on one screen (one attribute could be a

strobe, another a graded color change).

- Any effect can be applied to background as well as foreground.

Among the disadvantages are:

- Additional programming effort is required, along with increased

code and data storage.

- A background timer routine, if used, has some effect on system

efficiency (but it's small).

- Effects are associated with attribute numbers, not with bit

positions.

The last one bears some explanation.  Hardware blinking is associated

with a single bit (bit 7) of the FG/BG attribute byte.  Thus, you have

16 FG colors, all of which can be flashed.  Software-controlled special

effects, however, are associated with attributes.  If you set, say,

attribute 6 to be pulsed green, you no longer have a brown attribute

available.  Thus, the number of effective "base" colors is reduced for

each special effect attribute you create.

Mode 0

------

I have covered attribute control mode 1 in some detail, but some of the

same effects can be accomplished in mode 0.  The primary difference is

that you have only four palettes to work with.  The simpler effects such

as color changes, periodic intensification, and simulated blink can

easily be accomplished in mode one by putting the second color in

palette 1, and then alternating between palettes 0 and 1.

More complex effects would require periodic alterations to the palette

registers and/or color plane enable register.

Programming

-----------

The programming required to accomplish these special effects is

relatively straightforward.  The VGA BIOS provides all necessary

services; no register-level progamming is required (but see the next

section). Useful BIOS services are described below.

The only element that might need clarification is how to alter a color

definition in a video DAC color register.  BIOS provides services to set

a single color (BIOS video function 10h, subfunction 10h) and to set a

whole block of colors (function 10h, subfunction 12h).  If you are

altering a few colors only, setting them individually is efficient, but

it's more efficient to update larger numbers of colors by reloading the

entire set of registers.

Video function 10h, subfunction 17h reads a block of DAC color registers

into memory allocated by your program.  Each 18-bit register is split

into three bytes, one for each of the red, green, and blue primaries (in

that order).  If you read all 256 registers, 3*256 or 768 bytes of

storage will be required.

To obtain the video DAC register number for a particular color in a

particular palette, the formula is:

r = 64*p + pr[a]  (mode 0)

r = 16*p + pr[a]  (mode 1)

where r=register number, p=palette number, pr[] is the array of 16

palette registers, and a=attribute number.

The array of palette registers can be read and set via video function

10, subfunctions 9 and 2 respectively.  Note that if you have set the

palette registers sequentially (pr[0]=0, pr[1]=1,...,pr[15]=15), then

they drop out of the picture for calculation purposes, and the formula

becomes:

r = 64*p + a  (mode 0)

r = 16*p + a  (mode 1)

To locate the offset of the 3-byte color definition for a specific

video DAC color register within the table described above:

o = r * 3

where r is the register number described above.

A combined formula that locates an absolute address of the definition

of a particular attribute/palette combination:

address = base + 64*p + pr[a] (mode 0)

address = base + 16*p + pr[a] (mode 1)

where base is the base address of the table.  The function GET_DAC_PTR

in FLASH1.ASM performs this calculation.

To alter the displayed color for a palette/register combination,

calculate the address of its definition, alter the R/G/B bytes as

desired, and use video BIOS function 10h, subfunction 12h.  Because

this sunfunction can reload the entire set of DAC color registers,

you can alter as many registers as you want in one shot.

To alter the definitions for a particular attribute in all 16 palettes,

just locate the first one (palette 0).  The definitions for subsequent

palettes will follow at intervals of 16*3 or 48 (30h) bytes in mode 1,

or at intervals of 64*3 bytes in mode 0. For example, if the mode 1

definition for attribute 8 in palette 0 is at 1000h, attribute 8 in

palette 1 will be at 1030h, in palette 2 at 1060h, etc.

Changing the current palette (color select register)

----------------------------------------------------

A palette is selected by setting the contents of the color select

register to the desired palette number.  This can be done by using a

BIOS video service as described below (int 10H, function 10H,

subfunction 13h, BL=1).  This takes about 1/100th of a second and can

easily be done from within a timer intercept routine without any real

problems.

However, you may want to set the color select register using register

level programming, for one reason:  programs such as screen blankers may

be watching for BIOS video activity.  If you use BIOS services to change

the palette 18 times a second, your user's screen will never blank.  The

technique is shown in the timer routine in FLASH1.ASM.  It is assembled

only if the equate USE_BIOS is zero during assembly.  If USE_BIOS is

nonzero, FLASH1 is assembled to use the BIOS service for palette

switching.  There is no significant difference in efficiency between

using BIOS and programming the registers yourself; most of the time is

spent in waiting for a video retrace (and you thought you were through

with that when you got rid of your CGA, didn't you).

The demo (DEMO1.ASM) was assembled with USE_BIOS off.

The linkable object module

--------------------------

FLASH1.OBJ(ASM) provides a linkable object module that can be used to

generate all of the special effects described above, and others.

The module is organized for COM files; adjusting for other models should

be relatively painless.  The primary assumption is that DS=ES; there is

no assumption that CS=DS (except in the timer intercept), or any

explicit use of the stack.

The module prologue describes the services available.  Be sure to call

FLASH_INIT when you start and FLASH_TERM when you exit.

The demo program (DEMO1.ASM) demonstrates use of the module.

BIOS services

--------------

The VGA BIOS supports all necessary manipulation of VGA registers,

including the ability to read register contents (for state restoration

when you are done).  All of the following are accessed through the

standard video BIOS service call (int 10h), function 10h.  To use video

BIOS:

; Set reg AL and others as needed

mov ah,10h

int 10h

Most of these functions are reasonably efficient.  An operation such as

setting the color select register can easily be accomplished within an

interrupt service routine.

PALETTE REGISTERS

Read one palette register

Entry:

AL=7

BL=palette register number (0-15)

Return:

BH=palette register value

Set one palette register

Entry:

AL=0

BH=color value

BL=palette register number (0-15)

Return:

none

Read all palette registers

Entry:

AL=9

ES:DX -> 17-byte buffer

Return:

ES:DX[0..15] = current palette regs

ES:DX[16] = current overscan (border) color value

Set all palette registers

Entry:

AL=2

ES:DX -> 17-byte buffer (formatted as for function 9)

Return:

None

VIDEO DAC REGISTERS

Read one video DAC register

Entry:

AL=15h

BX=register number (0..255)

Return:

CH=green intensity (0..63)

CL=blue intensity (0..63)

DH=red intensity (0..63)

Set one video DAC register

Entry:

AL=10h

BX=register number (0..255)

CH=green intensity (0..63)

CL=blue intensity (0..63)

DH=red intensity (0..63)

Return:

none

Read a block of video DAC registers

Entry:

AL=17H

BX=first register (0..255)

CX=number of registers

ES:DX -> buffer

Return:

On return, the buffer is filled with video DAC color values.

The first 3 bytes contain the R/G/B values for the first

specified register, etc.  The buffer should be of size 3*CX

or more.  Ensure that BX+CX <= 256.

Set#a block of video DAC color registers

Entry:

AL=12H

BX=first register (0..255)

CX=number of registers

ES:DX -> buffer

The buffer is formatted as for function 12H.  Ensure

that BX+CX <= 256.

Return:

None

ATTRIBUTE CONTROL MODE/COLOR SELECT REGISTER

Read attribute control mode and color select register

Entry:

AL=1AH

Return

BL=current attribute control mode (0/1)

BH=color select register contents

Set attribute control mode

Entry:

AL=13H

BL=0

BH=attribute control mode (0/1)

Return:

None

Set color select register

Entry:

AL=13H

BL=1

BH=value for color select register (0..3 for attribute

control mode 0, 0..255 for mode 1).

Return:

None

Software version history

------------------------

FLASH1.ASM

10/29/89 1.01   Switched to "official" test for VGA presence

10/09/89 1.00   Initial release

DEMO1.ASM

10/09/89 1.00   Initial release

Author and reference

--------------------

This document and the DEMO1 and FLASH1 modules are by:

Chris Dunford

The Cove Software Group

PO Box 1072

Columbia MD 21044

301/992-9371

CompuServe 76703,2002

The document is copyright (C) 1989 by the author.  Permission is granted

to distribute freely by electronic or other means, but it may not be

republished without permission.

Reference (and a darn good one, too): Programmer's Guide to IBM PC and

PS/2 Video Systems; Richard Wilton, Microsoft Press 1987.

10/09/89

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

The Public (Software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  The P(s)L cannot de-

bug programs over the telephone.

Disks in the P(s)L are updated monthly, so if you did not get

this disk  directly from the P(s)L,  you should be aware that

the files in this set may no  longer be the current versions.

For a copy of the latest monthly software library newsletter

and a list of the 2,000+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

(713) 665-7017