Cport

Serial Communications Library for C/C++

Version 2.0

Copyright (c) 1993  Bri Productions - all rights reserved

Cport is a registered trademark of Bri Productions. All other product and

brand names are trademarks or registered trademarks of their respective

holders.

i

Contents

========

Introduction

------------

General Description ............................. 1-1

Disclaimer ...................................... 1-2

Evaluation Copy ................................. 1-2

Purchase Version ................................ 1-2

Contacting  Bri Productions ..................... 1-2

Terminology ..................................... 1-3

Using Cport

-----------

Basics .......................................... 2-1

Buffered Input .................................. 2-3

Buffered Output ................................. 2-5

Handshaking ..................................... 2-7

Direct Transmit Mode ............................ 2-10

Theory of Operation

-------------------

The UART ........................................ 3-1

The Transmitter and Receiver .................... 3-1

The Transmit and Receive Queues ................. 3-2

Input and Output Functions ...................... 3-2

Control and Status Functions .................... 3-2

Direct Transmit mode ............................ 3-2

C Reference

-----------

Typedefs and Structures ......................... 4-1

COM .......................................... 4-2

CPARAM ....................................... 4-2

byte ......................................... 4-3

Global Variables ................................ 4-4

comopen_errno ................................ 4-4

Control Functions ............................... 4-5

ComOpen ...................................... 4-6

ComOpenS ..................................... 4-8

ComClose ..................................... 4-8

ComCloseAll .................................. 4-9

ComHandshake ................................. 4-9

ComParam ..................................... 4-10

ComBaud ...................................... 4-11

ComMode ...................................... 4-12

ComRxQ ....................................... 4-13

ComTxQ ....................................... 4-14

ComTx ........................................ 4-15

ComNS550 ..................................... 4-15

ComTurbo ..................................... 4-16

ii

Input Functions ................................. 4-18

ComGetc ...................................... 4-19

ComGets ...................................... 4-19

ComIn ........................................ 4-20

ComFlushRx ................................... 4-21

ComLenRx ..................................... 4-22

ComPeek ...................................... 4-22

ComRxScan .................................... 4-23

Output Functions ................................ 4-25

ComPutc ...................................... 4-26

ComPuts ...................................... 4-26

ComOut ....................................... 4-27

ComFlushTx ................................... 4-28

ComLenTx ..................................... 4-29

ComTxWait .................................... 4-29

Status Functions ................................ 4-31

ComError ..................................... 4-32

ComStatus .................................... 4-33

ComRts ....................................... 4-33

ComDtr ....................................... 4-34

ComOut1 ...................................... 4-35

ComUart ...................................... 4-35

Data Integrity .................................. 4-37

ComChecksum .................................. 4-38

ComCrc16 ..................................... 4-38

ComCrc32 ..................................... 4-39

Misc Functions .................................. 4-41

ComSetBreak .................................. 4-42

ComClrBreak .................................. 4-42

ComPutScrtch ................................. 4-43

ComGetScrtch ................................. 4-43

C++ Reference

-------------

Control Functions ............................... 5-1

Cport::Cport ................................. 5-2

Cport::operator int .......................... 5-4

Cport::Handshake ............................. 5-5

Cport::Param ................................. 5-6

Cport::Baud .................................. 5-6

Cport::Mode .................................. 5-7

Cport::RxQ ................................... 5-8

Cport::TxQ ................................... 5-9

Cport::Tx .................................... 5-10

Cport::NS550 ................................. 5-10

Cport::Turbo ................................. 5-11

iii

Input Functions ................................. 5-13

Cport::Get ................................... 5-14

Cport::In .................................... 5-15

Cport::FlushRx ............................... 5-16

Cport::LenRx ................................. 5-17

Cport::Peek .................................. 5-17

Cport::RxScan ................................ 5-18

Output Functions ................................ 5-19

Cport::Put ................................... 5-20

Cport::Out ................................... 5-21

Cport::FlushTx ............................... 5-22

Cport::LenTx ................................. 5-22

Cport::TxWait ................................ 5-23

Status Functions ................................ 5-24

Cport::Error ................................. 5-25

Cport::Status ................................ 5-25

Cport::Rts ................................... 5-26

Cport::Dtr ................................... 5-27

Cport::Out1 .................................. 5-28

Cport::Uart .................................. 5-28

Data Integrity .................................. 5-29

Cport::Checksum .............................. 5-30

Cport::Crc16 ................................. 5-30

Cport::Crc32 ................................. 5-31

Misc Functions .................................. 5-33

Cport::SetBreak .............................. 5-34

Cport::ClrBreak .............................. 5-34

Cport::Scratch ............................... 5-35

File Transfer

-------------

Xmodem .......................................... 6-1

XmodemTx ........................................ 6-2

XmodemRx ........................................ 6-3

Xcallback ....................................... 6-4

Callback Function ............................... 6-5

Xmodem Tables of Values ......................... 6-7

Appendix

--------

Appendix A - Tables of Values ................... A-1

Appendix B - Compiling and Linking .............. B-1

Appendix C - Order form ......................... C-1

1-1

Introduction

============

General Description

-------------------

Cport is a general purpose C/C++ serial communications library for

80x86 Intel processor-based systems and the National Semiconductor

family of UARTs. This includes IBM and compatible computers and

potentially 80x86 processor-based embedded applications. The latter

is usually possible with few no modifications.

Cport features:

Simultaneous serial communications.

Built-in hardware and software handshaking.

Adjustable transmit and receive queues.

Transmit and receive queues are handled internally.

Developed in assembly language for optimum speed and efficiency.

Cport provides a middle level C/C++ interface for general purpose

serial communications. The library is lean and fast, so as to

provide programs with high performance serial communications without

significantly increasing its size. Cport can be used to develop

higher level interfaces for specific types of serial devices or as

is for more generic types of serial communications applications. An

example of a higher level interface might be a set of functions

that communicate with a serial printer. This interface would have

more knowledge of the type of data, controls codes, protocol, etc.

that relate to a serial printer. An example of a more generic

application would be a terminal program since, like Cport, a

terminal program is indifferent to the actual significance of the

data.

The design philosophy of Cport is to provide as many features

as possible without hindering its most important attributes: speed

and size. Many of the details, in addition to the actual hardware

interface itself, are provided in order to facilitate serial

communications software development. This includes the internal

buffering (queues), hardware and software handshaking and different

flavors of input/output functions. These features would either need

to be implemented externally anyway, or do not add overhead unless used.

Many features, such as timeouts, can additionally be easily implemented

as needed at the middle, C/C++ level. Many techniques and features are

demonstrated in the next section (Using Cport) and in examples

throughout this document. Examples and techniques will continue

to be compiled and made available in the future.

1-2

DISCLAIMER

----------

Cport is provided AS IS. Bri Productions specifically disclaims any

and all warranties, expressed or implied, including but not limited

to any implied warranties including fitness for a particular purpose.

Use this product at your own risk.

Evaluation Copy

---------------

This is an evaluation copy of Cport. This evaluation copy is

provided to you at no charge* so that you may properly evaluate Cport

prior to purchasing it. This evaluation copy has a fully functional

small model library and contains no cripples. Redistribution of this

evaluation copy, unaltered and without charge, is encouraged.

We feel it is important that you be able to try a product prior

to paying for it. This ensures that you will be able to determine

the quality of a product and if it fits your needs. It avoids

the hassle of money back guarantees which would otherwise waste

your time and ours.

We also feel that the producers of a product are entitled to be

compensated for their hard work. This enables them to continue to

provide a high quality product. If you intend to use Cport beyond

the purpose evaluation, you are obligated to purchase a license.

* Disk distributors may charge a minimum (reasonable) fee for

distribution of this evaluation copy.

Licensing

---------

When you purchase a license for Cport, you will receive the small,

medium, compact, large and huge model libraries; Library source code;

Printed manual; and examples. You will also be entitled to

technical support and low cost upgrades. When you license Cport you

may incorporate any part of it into your own executable programs

without additional royalties as long as they bear your own valid

Copyright notice. Each lisense covers usage by a single person at a

time. An order form is provided in Appendix C.

Contacting  Bri Productions

---------------------------

Bri Productions may be contacted by any of the following means:

Telephone: (510) 794-0616

CompuServe: 76635,2246

U.S. mail: Bri Productions

P.O.Box 7121

Fremont, CA 94537-7121

1-3

Terminology

-----------

This section defines some of the terminology used in this and other

Cport documentation.

The 'computer' refers to the machine on which Cport is running.

'Remote device' refers to the device the computer is communicating

with. The actual device may be a modem, serial printer or other

communications device.

If a serial port is, 'open' it is currently performing or ready to

perform serial communications. All 'open' serial ports have a 'handle'

which is assigned when it is 'opened'. This 'handle' is passed to Cport

functions in order to identify the 'open' serial port the function is to

act upon. (In C++, instances of the Cport class are used instead of

handles).

Individual units of 'data' that are transmitted and received may be

referred to as 'characters' or 'bytes'. Technically, there is no

difference between 'characters' and bytes, however; 'characters'

usually refer to 7 or 8 bit ASCII values and 'bytes' typically refer to

8 bit binary data. 'Data' refers to 'characters'/'bytes' in general.

A 'signal' may refer to software (i.e. control characters) as well

as actual hardware lines. This mainly refers to XON (ctrl-Q) and

XOFF (ctrl-S), which are used in software handshaking. 'Signals' may

be either inputs or outputs. A 'signal' is said to be 'asserted' if

it is in a true/on state and 'inhibited' if it is in a false/off

state. This stems from the idea that a 'signal' can be used to allow

(assert) or preclude (inhibit) something such as the transmission of

'data'.

2-1

Using Cport

===========

Basics

------

Before a serial port can be accessed, it must be opened. The open

functions identify the serial port by it's port address and

interrupt request line. The parameter id contains the port base

address in the lower 12 bits and the irq number in the upper 4 bits

of id. The open functions also take parameters that describe the

baud rate, word length, number of stop bits, parity check, and

transmit and receive queue sizes. If a serial port is successfully

opened, a handle to the newly opened serial port is returned to the

caller. This handle is used to reference the serial port in

subsequent calls to Cport functions. (C++ uses instances of the

Cport class instead of handles). If the serial port cannot be

opened, the open function returns NULL and sets the global variable

'comopen_errno' to an error code. (C++ uses an int typecast instead

of the global variable to hold the error code).

Before a program terminates, the serial port must be closed to make

certain restorations. Failing to close a serial port will leave

these restorations undone and almost certainly cause the system to

lock up. In C++, the serial port is closed automatically when it goes

out of scope (destructor).

2-2

C Example:

#include <stdio.h>

#include "cport.h"

int main(void)

{

COM com;      /* declare a serial port handle */

/* Open the serial port and get/store the serial port handle

*/

com = ComOpen(COM1, B9600, W8|S1|NONE, 1024, 512);

/* Check for a NULL handle. If the handle is NULL check

// comopen_errno for the cause

*/

if(com == NULL)

{

puts("ERROR: unable to open serial port");

return(comopen_errno);

}

/* Put a string to the newly opened serial port

*/

ComPuts(com, "Hello Cport");

... The rest of the program here

/* Close the serial port before exiting

*/

ComClose(com);

return(0);

}

C++ Example:

#include <iostream.h>

#include "cport.h"

int main(void)

{

Cport com(COM1, B9600, W8|S1|NONE, 1024, 512);

if((int)com != NO_ERR)   // uses int typecast

{

cout << "ERROR: unable to open serial port" << endl;

return(com);

}

com.Put("Hello Cport");

... The rest of the program here

// The serial port is closed automatically by

// the destructor.

return(0);

}

2-3

Buffered Input

--------------

When data is received, it is placed into the receive queue and stays

there until an input function retrieves it. Input functions will

only fetch whatever data is existing in the receive queue at

the time the input function is called. If a specified length of data

is expected, the receive queue can be queried to determine if enough

data is available to call an input function. This is a common case

with serial devices that send specific length messages terminated

by a specific character (often '\n' or '\r'). In the examples, the

function fetches a message only if one is available. If the message

is not available, it returns a zero, If a message is available, the

measurement is copied to the caller's buffer and a 1 is returned.

C Example:

#define   MSG_LEN      24

#define   TERM_C      '\n'

int GetMessage(COM com, char *buf)

{

if(ComLenRx(com) > MSG_LEN)

{

ComGets(com, buf, MSG_LEN, TERM_C);

return(1);

}

return(0);

}

C++ Example:

#define   MSG_LEN   24

#define   TERM_C   '\n'

int GetMessage(Cport& com, char *buf)

{

if(com.LenRx() > MSG_LEN)

{

com.Get(buf, MSG_LEN, TERM_C);

return(1);

}

return(0);

}

If the messages are not of a specific length, the receive queue can

be scanned for a particular character, namely the termination

character. Scanning for a character is also possible with specific

length messages but is slower than checking the length of the

receive queue. In the following examples, the function fetches a

message only if one is available. If the message is not available it

returns a zero, If a message is available, the measurement is

copied to the caller's buffer and a 1 is returned. If the message

is longer that the specified maximum message length, a -1 is

returned.

2-4

C Example:

#define   MAX_MSG_LEN      80

#define   TERM_C            '\n'

int GetMessage(COM com, char *buf)

{

int len;

len = ComRxScan(com, TERM_C);

if(len > MAX_MSG_LEN)

return(-1);

else if(len > 0)

ComGets(com, buf, MAX_MSG_LEN, TERM_C);

return(len != 0);

}

C++ Example:

#define   MAX_MSG_LEN      80

#define   TERM_C            '\n'

int GetMessage(Cport& com, char *buf)

{

int len;

len = com.RxScan(TERM_C);

if(len > MAX_MSG_LEN)

return(-1);

else if(len > 0)

com.Get(buf, MAX_MSG_LEN, TERM_C);

return(len != 0);

}

The previous examples are ideal for event driven routines. They do

not wait for a message to be received if one is not ready.

This allows a program to process other tasks instead of holding up

program execution while waiting for a message. On the contrary, one

may want to wait for a message, perhaps within a certain time frame.

Revisiting the first example, we can add a timeout using the ANSI C

compatible function, clock.

2-5

C Example:

#include <time.h>

#define   MSG_LEN      24

#define   TERM_C      '\n'

int GetMessage(COM com, char *buf, clock_t timeout)

{

timeout += clock();

while(ComLenRx(com) <= MSG_LEN)

{

if(clock() > timeout)

return(0);

}

ComGets(com, buf, MSG_LEN, TERM_C);

return(1);

}

C++ Example:

#include <time.h>

#define   MSG_LEN   24

#define   TERM_C   '\n'

int GetMessage(Cport& com, char *buf, clock_t timeout)

{

timeout += clock();

while(com.LenRx() <= MSG_LEN)

{

if(clock() > timeout)

return(0);

}

com.Get(buf, MSG_LEN, TERM_C);

return(1);

}

Buffered Output

---------------

When an output function inserts data into the transmit queue, it

does not wait for the data to be transmitted before the function

returns. Instead the transmitter will subsequently remove the data,

one byte/character at a time, in the order in which it was inserted and

transmit them in the background. This allows the program to continue

with more important tasks instead of waiting for the data to

transmit at a relatively slow rate (compared to the processor).

At certain points in a program, one may want to ensure that all the

data thus far has been transmitted. To do this the transmit queue

can be queried to determine how many bytes/characters still reside

there. The following examples make sure all data finishes

transmitting before the program is terminated.

2-6

C Example:

int main(void)

{

COM com;

com = ComOpen(COM2,B9600,W8|S1|NONE,512,2048);

if(com == NULL)

return(comopen_errno);

ComPuts(com, "Welcome to Bri Productions");

while(ComLenTx(com) != 0);

ComClose(com);

return(0);

}

C++ Example:

int main(void)

{

Cport com(COM2, B9600, W8|S1|NONE, 512, 2048);

if((int)com != NO_ERR)

return(com);

com.Put("Welcome to Bri Productions");

while(com.LenTx() != 0);

// The serial port is closed automatically by

// the destructor.

return(0);

}

Querying the transmit queue can also be useful in determining if

there is enough room for new data. In the following, example a

message is sent only if there is enough room for the whole message.

The value returned should always be 0 (if not enough room) or the

length of the message (number of characters put by ComPuts).

C Example:

#include <string.h>

#define   TXSIZE   1024

int PutMessage(COM com, const char *buffer)

{

if(TXSIZE - ComLenTx(com) < strlen(buffer))

return(0);

return(ComPuts(com, buffer));

}

2-7

C++ Example:

#include <string.h>

#define   TXSIZE   1024

int PutMessage(Cport& com, const char *buffer)

{

if(TXSIZE - com.LenTx() < strlen(buffer))

return(0);

return(com.Put(buffer));

}

If the transmit queue length is not checked and an output function

is called without enough room for all the data, the number of

byte/characters that did fit is returned. This value can be used

as an offset into the data to determine the first byte/character

that there was no room for. The following examples will not return

until all the characters in the message are put to the queue.

C Example:

void PutMessage(COM com, const char *buffer)

{

const char *_buffer = buffer;

while(*_buffer != '\0')

_buffer += ComPuts(com, _buffer);

}

C++ Example:

void PutMessage(Cport& com, const char *buffer)

{

const char *_buffer = buffer;

while(*_buffer != '\0')

_buffer += com.Put(_buffer);

}

Handshaking

-----------

There are number of built-in options available for implementing

handshaking. The options can be divided into two categories. One is

hardware handshaking and the other is software handshaking. In

hardware handshaking, one or more of the hardware lines is used.

In software handshaking, XON and XOFF characters are used.

2-8

Hardware Handshaking

--------------------

In hardware handshaking, any combination of the hardware signals

RTS, DTR, DSR, CTS, and DCD can be used to achieve handshaking. RTS

and/or DTR are used by the receiver to signal the remote device to

suspend or resume transmitting. Similarly, DSR, CTS, and/or DCD can

be used by the remote device's receiver to signal the transmitter to

suspend or resume transmitting.

Software Handshaking

--------------------

In software handshaking, the control characters XON (ctrl-Q) and

XOFF (ctrl-S) are used to achieve handshaking. When receiver

software handshaking (S_RX) is enabled, XON and XOFF characters are

transmitted to signal the remote device to suspend or resume it's

transmission. Notice that even though the XON and XOFF characters

are sent by the transmitter, it is the receiver that exercises the

handshaking in order to preclude a receive queue overflow. In

transmit software handshaking (S_TX), the remote device sends XON

and XOFF characters to suspend and resume the computer's

transmission. Notice that even though the XON and XOFF characters

are received, it is the transmitter that is affected. Both receive

and transmit handshaking can be enabled simultaneously.

The Receiver

------------

When receive software (S_RX) and/or hardware handshaking (DTR and/or

RTS) are enabled, the receiver monitors the length of the receive

queue. If the length exceeds the specified threshold, the receiver

will take the appropriate action to signal the remote device to

suspend transmitting. When the length of the receive queue

subsequently falls below half of the set threshold, the appropriate

action is taken to signal the remote device to resume transmitting.

The Transmitter

---------------

When transmit software (S_TX) and/or hardware handshaking (DSR, CTS,

and/or DCD) are enabled and the computer is signaled to suspend

transmitting with the corresponding handshaking enabled, the

transmitter will stop transmitting. Subsequently, when the computer

is signaled to resume transmitting, the transmitter will pick up

where it left off.

User Handshaking

----------------

Another option for implementing handshaking is for the programmer to

implement their own handshaking scheme. To accomplish this, Cport

provides the function ComTx. ComTx allows the programmer to turn the

transmitter on and off. When the transmitter is turned off,

transmitting halts immediately. If any characters still reside in

the transmit queue, they will remain there until the transmitter is

turned back on or the transmit queue is flushed. More characters may

be put into the transmit queue while the transmitter is turned off.

2-9

Cooperation

-----------

In any handshaking scheme, it is imperative that the computer and

the remote device recognize the same handshaking scheme. For

instance, if the remote device recognizes hardware handshaking and

the computer does not, the computer will ignore any requests by the

remote device to suspend transmitting. If the remote device

recognized software handshaking and the computer did not, bogus XON

and XOFF characters might corrupt the received data.

In the first example, the transmitter will not transmit unless CTS

and DSR are asserted. The receiver will inhibit RTS if the receive

queue exceeds 768 bytes/characters and assert it when it

subsequently falls below 768 / 2. In the second example, the

transmitter will stop if an XOFF character is received and restart

when an XON character is received. If the receive queue length

exceeds 768 bytes/characters, an XOFF character will be transmitted

and when the length subsequently falls below 768 / 2, an XON

character is transmitted.

C Example 1

int main(void)

{

COM com;

com = ComOpen(COM1, B9600, W7|S1|EVEN, 1024,256);

if(com == NULL)

return(comopen_errno);

ComHandshake(com, CTS | DSR | RTS, 768);

/* rest of the program */

ComClose(com);

return(0);

}

C++ Example 1

int main(void)

{

Cport com(COM1, B9600, W7|S1|EVEN, 1024, 256, CTS|DSR|RTS, 768);

if((int)com)

return(com);

// rest of the program

return(0);

}

2-10

C Example 2

int main(void)

{

COM com;

com = ComOpen(COM1, B9600, W7|S1|EVEN, 1024, 256);

if(com == NULL)

return(comopen_errno);

ComHandshake(com, SOFT, 768); /* SOFT == S_RX | S_TX */

/* rest of the program */

ComClose(com);

return(0);

}

C++ Example 2

int main(void)

{

// SOFT == S_RX | S_TX

Cport com(COM1, B9600, W7|S1|EVEN, 1024, 256, SOFT, 768);

if((int)com)

return(com);

// rest of the program

return(0);

}

Direct Transmit Mode

--------------------

In direct transmit mode, data is sent directly to the serial port.

The output functions will only return after all the data has been

transmitted or when a handshaking signal is inhibited. Direct mode

is ideal for transmit intensive routines such as file transfers, where

the main task is transmitting data. Otherwise direct transmit mode

is not normally preferred.

There are two ways to get into direct transmit mode. The first is to

set the transmit queue size in the call to the open function (or

constructor in C++) to zero. This method sets direct transmit mode

permanently. To set direct transmit mode temporarily, turn the

transmitter off with ComTx with the TX_DIRECT option.

2-11

C Example 1

int main(void)

{

COM com;

/* transmit queue 0 = direct transmit mode

*/

com = ComOpen(COM1, B9600, W8 | S1 | NONE, 4096, 0);

if(com == NULL)

return(comopen_errno);

/* continued   */

ComClose(com);

return(0);

}

C++ Example 1

int main(void)

{

// transmit queue 0 = direct transmit mode

Cport com(COM1, B9600, W8 | S1 | NONE, 4096, 0);

// continued

return(0);

}

In this examples, the transmitter is put into direct mode while a

block of data is transmitted. The transmitter is subsequently

restored to its previous state. The block of data is completely

transmitted before the function returns (assumes no transmit

handshaking is enabled).

C Example 2

#define BLOCK_SIZE   1024

int SendBlock(COM com, byte *block)

{

int tx, rv;

tx = ComTx(com, OFF | TX_DIRECT);

rv = ComOut(com, block, BLOCK_SIZE);

ComTx(com, tx);

return(rv);

}

2-12

C Example 2

#define BLOCK_SIZE   1024

int SendBlock(Cport& com, byte *block)

{

int tx, rv;

tx = com.Tx( OFF | TX_DIRECT);

rv = com.Out(block, BLOCK_SIZE);

com.Tx(tx);

return(rv);

}

Handshaking in Direct Transmit Mode

-----------------------------------

Transmit handshaking works a bit differently in direct transmit

mode. In normal transmit handshaking, output functions will always

succeed in putting the data as long as there is room in the transmit

queue, even if transmit handshaking is in an inhibited state. In

direct transmit mode, there is no transmit queue to buffer the output

data during the inhibited state of transmit handshaking. Therefore,

in direct transmit mode, an output function may not complete a

transmission of the data if transmit handshaking changes to an

inhibited state.

The return value of the output functions should always be tested when

transmit handshaking is used in direct transmit mode to ensure that all

of the data has been transmitted. The return value can subsequently be

used to transmit the remaining data when the transmit handshaking

returns to an asserted state. This is demonstrated in the following

example. In this example, it is assumed that DSR handshaking is enabled

and the transmitter is in direct mode.

C Example:

void SendData(COM com, char *buf)

{

char *_buf = buf;

while(*_buf != '\0')

{

_buf += ComPuts(com, _buf);

if(*_buf != '\0')

while(!ComStatus(com) & DSR);   /* wait for DSR */

}

}

2-13

C++ Example

void SendData(Cport& com, char *buf)

{

char *_buf = buf;

while(*_buf != '\0')

{

_buf += com.Put(_buf);

if(*_buf != '\0')

while(!com.Status() & DSR);   // wait for DSR

}

}

Receive handshaking will always function the same regardless of the

current mode of the transmitter.

3-1

Theory of Operation

===================

The UART

--------

The UART (Universal Asynchronous Receiver/Transmitter) is a single

integrated circuit peripheral chip, which is specially designed for

asynchronous serial communications. It is responsible for many of

the electronic details of serial communications. The UART converts

parallel data (bytes/characters) to and from serial signals at the

correct baud rate, and with the assigned word length, stop bits and

parity. It also monitors and controls the hardware status lines.

Cport interfaces intimately with the serial port's UART.

The name UART is a generic name used to describe a type of

integrated circuit. serial communications in IBM and compatible

computers is based on the National Semiconductor family of UARTs.

These chips include:

INS8250

INS8250-B

INS8250A

INS82C50A

NS16450

NS16C450

NS16550A

The latest and greatest is the NS16550A, which features transmitter

and receiver FIFO's. If a NS16550 UART is detected by Cport when a

serial port is opened, Cport will automatically take advantage of

it's advanced features. These feature greatly improve performance.

The Transmitter and Receiver

----------------------------

The transmitter and receiver interact intimately with the serial

port's UART to coordinate data exchanges between it and the transmit

and receive queues. They also coordinate and control any enabled

handshaking. The transmitter and receiver together constitute an

interrupt service routine that the program will branch to any time

the serial port's UART generates an interrupt.

When the UART is ready for another byte/character, the transmitter

will transfer another byte/character (if available) to the UART. In

addition, it sends XON/XOFF characters at the request of the receiver

when software handshaking is enabled, and monitors the status of the

input hardware lines when hardware handshaking in enables.

When the UART receives a new byte/character, the receiver fetches

it from the UART and stores it in the receive queue. It also

monitors the amount of data in the receive queue when handshaking is

enabled, in order to perform the enabled handshaking and avoid a

receive queue overflow. In software handshaking, requests to the

transmitter are made to send the XON/XOFF characters. In hardware

handshaking, the receiver manipulates the  hardware line(s) directly.

3-2

The Transmit and Receive Queues

-------------------------------

The receive and transmit queues provide the interface/buffering

between the Cport input and output functions and the receiver and

transmitter respectively. This allows the transmitter/receiver to

operate independently from the input/output functions and vice

versa. The transmitter is concerned only with the head of the

transmit queue while the output functions are only concerned with

the tail of the transmit queue. Similarly, the receiver is only

concerned with the tail of the receive queue while the input

functions are only concerned with the head of the receive queue.

Input and Output Functions

--------------------------

Input functions retrieve data from the receive queue in the order in

which it is placed into the receive queue by the receiver. Since the

data is buffered, the program does not need to worry about the

incoming data until it is ready to fetch it. The output functions

insert data into the transmit queue and will be transmitted by the

transmitter in the order in which the data is inserted.

Control and Status Functions

----------------------------

Control functions manipulate both the transmitter/receiver and the

UART. The queues are independent of the control functions except

when the queues are created and destroyed when the serial port is

opened and close respectively. Status functions retrieve information

from the transmitter/receiver and the UART. They also manipulate the

UART to set status (to the remote unit).

Direct Transmit Mode

--------------------

In direct transmit mode, the transmitter and transmit queue are

removed. The output functions write directly to the UART. The output

functions will return only after all it's data is written to the

UART, or transmit handshaking is enabled and in an inhibited state.

This holds up program execution but may be desirable when there are

no other tasks to perform, since otherwise, the transmitter and

receiver share available background time. An example of when direct

transmit mode might be advantageous is a dedicated file transfer

routine where the only interest is to transmit data.

4-1

C Reference

===========

Typedefs and Structures

-----------------------

Basic

COM is a typedef used for storing a serial port handle.

CPARAM is a structure type used for storing a serial port's parameters.

byte is a typedef userd for storing a byte size value.

4-2

COM

------------------------------------------------------------------

Function    Data type for storing a serial port handle.

Syntax      #include "cport.h"

typedef void* COM;

Remarks     COM is a type used to store a serial port handle. The open

functions (ComOpen and ComOpenS) return values of type COM and

all serial port specific functions take a parameter of type COM.

Example     Open a serial port.

int main(void)

{

COM com1;

com1 = ComOpen(COM1, B9600, W8|S1|NONE, 1024, 512);

if(com1 == NULL)

{

/* error */

}

...continued

CPARAM

------------------------------------------------------------------

Function    Data type for storing a serial port's parameters.

Syntax      #include "cport.h"

typedef struct

{

unsigned   id;

int        baud;

byte       mode;

unsigned   rxQ;

unsigned   txQ;

byte       htype;

unsigned   thresh;

}CPARAM;

id  -    Value that identifies the serial port. The upper 4 bits define

the interrupt request line while the lower 12 bits define the

port address. This technique allows for the possibility

of custom configurations. See Table A-1 in Appendix A for a list

of predefined values.

baud   - Value that defines the baud rate. The actual value is the baud

rate divisor, which allows for the possibility of custom baud

rates. The baud rate divisor = f / (16 * baud rate) where f is

1.8432e+6. See Table A-2 in Appendix A for a list of predefined

baud rate divisors.

mode   - Bit ORed value defining the word length, number of stop bits

and the parity. See Table A-3 in Appendix A for a list of mode

components.

4-3

* rxQ  - Size, in bytes, of the receive queue.

* txQ  - Size in bytes, of the transmit queue.

* Limited to 32K in small and medium memory models.

htype  - Type of handshaking to be used. See Table A-5 in Appendix A

for a list of handshaking components.

thresh - Receive queue threshold.

Remarks     CPARAM is a structure type that holds the parameters of a

serial port. This structure is mainly used with the alternate

open function ComOpenS and with the function ComParam.

Example     Open COM2 at 9600 baud, 8 bits, 1 stop bit, no parity, a

receive queue of 1024 bytes, a transmit queue of 512 bytes and

software handshaking (XON/XOFF) with a receive queue threshold of

3072 bytes.

int main(void)

{

CPARAM param= {COM2, B9600, W8|S1|NONE, 4096, 512, SOFT, 3072};

COM com2;

com2 = ComOpenS(&param);

if(com2 == NULL)

{

/* error */

}

...continued

byte

------------------------------------------------------------------

Function    Data type used for storing byte size values.

Syntax      #include "cport.h"

typedef unsigned character byte;

Remarks     The byte data type is used for storing byte size values. Some

of the Cport functions take parameters of type byte and some

return values of type byte.

Example     Use a byte data type for the mode parameter.

static byte mode = W8|S1|NONE;

byte Mode(COM terminal, byte newmode)

{

byte temp;

ComMode(terminal, newmode);

temp = mode;

mode = newmode;

return(temp);      /* return previous mode */

}

4-4

Global Variables

----------------

comopen_errno

------------------------------------------------------------------

Function    Holds the error code from the last attempt to open a serial

port.

Syntax      #include "cport.h"

extern int comopen_errno;

Remarks     Comopen_errno holds the error code from the last attempt to

open a serial port via a call to ComOpen or ComOpenS. If the

last opened serial port was successfully opened, comopen_errno

will be 0. If an error occurred while attempting to open the

serial port the value of comopen_errno will be non-zero. See

Table A-4 in Appendix A for a list of possible values for

comopen_errno.

Example     Attempt to open COM1. If an error occurs, check comopen_errno

for the cause.

int main(void)

{

COM com1;

com1 = ComOpen(COM1, B9600, W8|S1|NONE, 1024, 512);

if(com1 == NULL)

{

switch(comopen_errno)

{

case RX_ALC:

case TX_ALC:

case GEN_ALC:

/* memory allocation error */

break;

case NO_UART:

/* no UART at that address */

break;

default:

/* all other errors */

break;

}

}

...continued

4-5

Control Functions

-----------------

Basic

ComOpen opens a serial port using discrete parameters.

ComOpenS opens a serial port using a structure of parameters.

ComClose closes a currently open serial port.

ComCloseAll closes all open serial ports.

ComHandshake determines if and what type of handshaking scheme will be

enabled and sets the receiver threshold.

Intermediate

ComParam reports on the current parameters and handshaking under which

the specified serial port is currently operating.

ComBaud changes the serial port's baud rate.

ComMode changes the serial port's word length, number of stop bits and

parity.

Advanced

ComRxQ resizes the receive queue.

ComTxQ resizes the transmit queue.

ComTx allows advanced control over the Cport transmitter.

ComNS550 sets the receive fifo threshold of an NS16550 UART.

ComTurbo temporarily disables/re-enables selected high priority

interrupts that may impede high speed transfers.

4-6

ComOpen

------------------------------------------------------------------

Function    Opens a serial port for communications.

Syntax      #include "cport.h"

COM ComOpen(unsigned id, int baud, byte mode,

unsigned rxQ, unsigned txQ);

Parameters

id     - Value that identifies the serial port. The upper 4 bits define

the interrupt request line, while the lower 12 bits define the

port address. This technique allows for possibility of custom

configurations. See Table A-1 in Appendix A for a list of

predefined values.

baud   - Value that defines the baud rate. The actual value is the baud

rate divisor, which allows for the possibility of custom baud

rates. The baud rate divisor = f / (16 * baud rate) where f is

1.8432e+6. See Table A-2 in Appendix A for a list of predefined

baud rate divisors.

mode   - Bit ORed value defining the word length, number of stop bits

and the parity. See Table A-3 in Appendix A for a list of mode

components.

* rxQ  - Size, in bytes, of the receive queue.

* txQ  - Size, in bytes, of the transmit queue.

* Limited to 32K in small and medium memory models.

Remarks     ComOpen opens the specified serial port and returns a handle

used to identify the newly opened serial port. If ComOpen fails

to open the specified serial port, it returns NULL and sets

comopen_errno to an error code. The 4 most significant bits in

id represent the interrupt vector, while the lower 12 bits

represent the port address. Constants are provided in cport.h

to facilitate the formulation of the id parameter. The COMx

values will work in most cases; however, the IRQx values can

be ORed with the PORTx or BIOSx values to produce other

combinations. The PORTx values are actual port addresses where

the serial ports are normally located. The BIOSx values are

actually macros, which fetch the port address from the BIOS data

area. Usually the BIOSx and PORTx values will be the same, but

it is not guaranteed. Quite often the values at BIOS2 and BIOS3

(com 3 and com 4) will be set to 0 regardless if com3 or com4

exist. See Table A-1 in Appendix A for a list of predefined

values for formulating the id parameter.

The parameter baud is the baud rate divisor that determines the

baud rate. Achieving baud rates above 19200 is depends on many

variables such as machine speed, cable length, handshaking

activity, degree bidirectional transfers, number of open serial

ports, the type UART(s) used and activity of other hardware

interrupts.

4-7

The parameter mode determines the word length, number of stop

bits and parity. Bits 0 and 1 determine the word length, bit 2

determines the number of stop bits, bit 3 enables parity and

bits 4 and 5 determine the type of parity check.

RxQ and txQ set the size of the receive and transmit queues

respectively. RxQ should be at least four times larger than the

largest string or block of data the programmer intends to fetch

from the queue at once. TxQ can be set as small as 1, but is

not recommended. In the small and medium memory models, where

all data is limited to 64K, the size queues are limited. In the

compact and large memory models, each queue can be as large as

64K-1 (65534 bytes).

Return value   If successful, ComOpen returns a handle to the newly opened

serial port. In the event of an error, it returns NULL and

sets the global variable comopen_errno to an  error code.

See Table A-4 in Appendix A for a list of possible values

for comopen_errno.

See also    ComOpenS, ComClose, ComCloseAll, ComBaud, ComMode, COM,

comopen_errno

Example     Open COM1 at 9600 baud with 8 bit words, 1 stop bit, no parity,

a 1024 byte receiver and a 512 byte transmitter.

int main(void)

{

COM com1;

com1 = ComOpen(COM1, B9600, W8|S1|NONE, 1024, 512);

if(com1 == NULL)

{

switch(comopen_errno)

{

case RX_ALC:

case TX_ALC:

case GEN_ALC:

/* allocation error */

break;

case NO_UART:

/* no UART */

break;

default:

/* all other errors */

break;

}

}

...continued

4-8

ComOpenS

------------------------------------------------------------------

Function    Opens a serial port for communications.

Syntax      #include "cport.h"

COM ComOpenS(const CPARAM *param);

Parameters

param  - Pointer to a type CPARAM that contains the serial port

parameters.

Remarks     ComOpenS is an alternative to ComOpen for opening a serial port.

It takes a pointer to a type CPARAM. CPARAM is a structure

that contains the serial port parameters.

Return value   If successful, ComOpenS returns a handle to the newly opened

serial port. In the event of an error, it returns NULL and

sets the global variable comopen_errno to an error code.

See Table A-4 in Appendix A for a list of possible values

for  comopen_errno.

See also    ComOpen, ComClose, ComCloseAll, ComParam, COM, comopen_errno

Example     Open COM2 at 9600 baud, 8 bits, 1 stop bit, no parity, a

receive queue of 1024 bytes, with a transmit queue of 512

bytes and software handshaking (XON/XOFF) with a receive queue

threshold of 3072 bytes.

int main(void)

{

CPARAM param = {COM2, B9600, W8|S1|NONE, 4096, 512, SOFT, 3072};

COM com2;

com2 = ComOpenS(&param);

if(com2 == NULL)

{

/* error */

}

...continued

ComClose

------------------------------------------------------------------

Function    Closes the specified serial port.

Syntax      #include "cport.h"

void ComClose(COM);

Parameters

COM    - Handle of the serial port to be closed.

4-9

Remarks     ComClose closes the specified serial port. Failing to close all

serial ports before a program is terminated will cause the

system to lock up.

See also    ComCloseAll, ComOpen, ComOpenS, COM

Example     Close the serial port prior to terminating.

int main(void)

{

COM com1;

com1 = ComOpen(COM1, B9600, W8|S1|NONE, 1024, 512);

if(com1 == NULL)

/* error */

...later

ComClose(com1);

return(0);

}

ComCloseAll

------------------------------------------------------------------

Function    Closes all currently opened serial ports.

Syntax      #include "cport.h"

void ComCloseAll(void);

Remarks     ComCloseAll closes all the currently opened serial ports.

Failing to close all serial ports before a program is

terminated will cause the system to lock up. ComCloseAll is

convenient for ensuring all serial ports are closed prior to a

program's termination.

See also    ComClose, ComOpen, ComOpenS

Example     Make sure all serial ports are closed prior to terminating.

void ErrorExit(void)

{

ComCloseAll();

exit(-1);

}

ComHandshake

------------------------------------------------------------------

Function    Enables or disables handshaking (flow control) for the

specified serial port.

Syntax      #include "cport.h"

void ComHandshake(COM, byte htype, unsigned thresh);

4-10

Parameters

COM    - Handle of the serial port.

htype  - Type of handshaking to be used. See Table A-5 in Appendix A

for a list of handshaking components.

thresh - Receive queue threshold.

Remarks     ComHandshake enables or disables software and/or hardware

handshaking for the specified serial port. In software

handshaking, XON and XOFF characters are used to suspend and

resume communications. Hardware handshaking uses any or all of

the DTR, DSR, RTS, CTS and DCD lines to suspend and resume

communications. Thresh determines when the receiver will

assert or inhibit the corresponding handshaking signals. If

no receive handshaking is specified (RTS, DTR or S_RX), the

threshold is not used and may be set to zero.

See also    ComParam, ComOpenS, ComTx

Example     If COM1 is successfully opened, turn on some hardware

handshaking and set the receiver threshold to 3/4 of the

receiver's full size. The transmitter will not transmit unless

both DSR and DCD are asserted. The receiver will inhibit DTR if

the receive queue length exceeds the threshold.

int main(void)

{

COM com1;

com1 = ComOpen(COM1, B9600, W8|S1|NONE, 1024, 512);

if(com1 == NULL)

{

/* error */

}

ComHandshake(com1, DSR|DTR|DCD, 768);

...continued

ComClose(com1);

return(0);

}

ComParam

------------------------------------------------------------------

Function    Retrieves the parameters of the specified serial port.

Syntax      #include "cport.h"

void ComParam(COM, CPARAM *param);

Parameters

COM    - Handle of the serial port from which to get the parameters.

4-11

param  - Pointer to a type CPARAM where the parameters will be stored.

Remarks     ComParam retrieves the parameters and handshaking status of the

specified serial port and stores them in a type CPARAM.

See also    CPARAM, ComOpen, ComOpenS, ComMode, ComBaud

Example     Open a serial port with the same configuration as one that is

already opened.

COM ComOpenCopy(unsigned id, COM copyme)

{

CPARAM param;

ComParam(copyme, &param);

param.id = id;

return(ComOpenS(&param));

}

ComBaud

------------------------------------------------------------------

Function    Changes the baud rate of the specified serial port.

Syntax      #include "cport.h"

void ComBaud(COM, int baud);

Parameters

COM    - Handle of the serial port.

baud   - Value that defines the baud rate. The actual value is the baud

rate divisor, which allows for possibility of custom baud rates.

The baud rate divisor = f / (16 * baud rate) where f is

1.8432e+6. See Table A-2 in Appendix A for a list of predefined

baud rate divisors.

Remarks     ComBaud changes the baud rate of the specified serial port. The

parameter baud is the baud rate divisor that determines the

baud rate.

See also    ComOpen, ComOpenS, ComParam, ComMode

Example     Set the specified serial port to a new baud rate.

4-12

#define NBAUD   4

int SetBaud(COM com, unsigned n)

{

int baud[NBAUD] = {    B9600, B2400, B1200, B300 };

if(n < NBAUD)

{

ComBaud(com, baud[n]);

return(0);

}

else

return(-1);

}

ComMode

------------------------------------------------------------------

Function    Changes the word length, number of stop bits and parity of the

specified serial port.

Syntax      #include "cport.h"

void ComMode(COM, byte mode);

Parameters

COM    - Handle of the serial port.

mode   - Bit ORed value defining the word length, number of stop bits

and the parity. See Table A-3 in Appendix A for a list of mode

components.

Remarks     ComMode changes the word length, number of stop bits and parity

of the specified serial port. The parameter mode determines the

word length, number of stop bits and parity of the specified

serial port. Bits 0 and 1 determine the word length, bit 2

determines the number of stop bits, bit 3 enables parity and

bits 4 and 5 determine the type of parity check.

See also    ComOpen, ComOpenS, ComParam, ComBaud

Example     Set the serial port to a new mode.

#define NMODE   2

int SetMode(COM com, unsigned n)

{

byte mode[NMODE] = { W8|S1|NONE, W7|S1|EVEN };

if(n < NMODE)

{

ComMode(com, mode[n]);

return(0);

}

return(-1);

}

4-13

ComRxQ

------------------------------------------------------------------

Function    Resizes the specified serial port's receive queue.

Syntax      #include "cport.h"

unsigned ComRxQ(COM, unsigned size);

Parameters

COM    - Handle of the serial port.

size   - New receive queue size.

Remarks     ComRxQ resizes the specified serial port's receive queue. The

memory currently allocated for the receive queue is freed, and

new memory is allocated. If ComRxQ is unable to allocate the

specified amount memory, it will try to reallocate memory for

the previous size. ComRxQ returns the new queue size. If

memory was successfully allocated for the new size, the return

value will be size. If the previous size is reallocated, the

return value will be the previous size. If no memory could be

allocated, ComRxQ returns zero.

No data can be received while the new receive queue is being

resized. However, data in the transmit queue will continue to

be transmitted normally.

Return value   ComRxQ returns the new receive queue size.

See also    ComTxQ, ComOpen, ComOpenS

Example     Enlarge the receive queue for a file transfer.

#define LARGE_RX   4096

int ReceiveFile(COM com, const char *filename)

{

CPARAM param;

ComParam(com, &param);

if(ComRxQ(com, LARGE_RX) != LARGE_RX)

return(-1);

... transfer file here

/* Restore original size   */

if(ComRxQ(com, param.rxQ) != param.rxQ)

return(-2);

else

return(0);

}

4-14

ComTxQ

------------------------------------------------------------------

Function    Resizes the specified serial port's transmit queue.

Syntax      #include "cport.h"

unsigned ComTxQ(COM, unsigned size);

Parameters

COM    - Handle of the serial port.

size   - New transmit queue size.

Remarks     ComTxQ resizes the specified serial port's transmit queue. The

memory currently allocated for the transmit queue is freed, and

new memory is allocated. If ComTxQ is unable to allocate the

specified amount memory, it will try to reallocate memory for

the previous size. ComTxQ returns the new queue size. If memory

was successfully allocated for the new size, the return value

will be size. If the previous size is reallocated, the return

value will be the previous size. If no memory could be

allocated, ComTxQ returns zero.

No data can be transmitted while the new transmit queue is

being resized. However, data to the receive queue will

continue to be received normally.

Return value   ComTxQ returns the new transmit queue size.

See also    ComRxQ, ComOpen, ComOpenS

Example     Enlarge the transmit queue for a file transfer.

#define LARGE_TX   4096

int TransmitFile(COM com, const char *filename)

{

CPARAM param;

ComParam(com, &param);

if(ComTxQ(com, LARGE_TX) != LARGE_TX)

return(-1);

... transfer file here

/* Restore original size   */

if(ComTxQ(com, param.txQ) != param.txQ)

return(-2);

else

return(0);

}

4-15

ComTx

------------------------------------------------------------------

Function    Transmitter control.

Syntax      #include "cport.h"

int ComTx(COM, int cmnd);

Parameters

COM    - Handle of the serial port.

cmnd   - Determines the action to be taken on the transmitter See Table

A-9 in Appendix A for a list of transmitter options.

Remarks     ComTx  allows advanced control of the Cport transmitter The

transmitter can be turned on and off to suspend and resume

transmissions. This function can be very useful when

implementing your own handshaking scheme. In addition, it can

inform the output functions to transmit directly when the

transmitter is turned off.

Return value   ComTx returns the previous status of the transmitter

See also    ComHandshake, ComOpenS, ComParam

Example     Put a priority character directly  to the  serial port.

void PriorityPutc(COM com, char c)

{

int tx;

tx = ComTx(com, TX_DIRECT | OFF);

ComPutc(com, c);

ComTx(com, tx);   /* restore original state */

}

ComNS550

------------------------------------------------------------------

Function    NS16550 UART setup.

Syntax      #include "cport.h"

void ComNS550(COM, int trigger);

Parameters

COM    - Handle of the serial port.

trigger   - Trigger threshold for the UART receive fifo. See Table A-12 in

Appendix A for a list of trigger thresholds.

Remarks     If Cport detects an NS16550 UART when a serial port is opened,

Cport automatically takes advantage of it's advanced

capabilities.

4-16

The receive fifo threshold is the number of bytes in the

receive fifo that will cause an interrupt to be generated. By

default, the receive fifo threshold is set  to 14 bytes.

ComNS550 allows this receive fifo trigger threshold to be

changed.

See Also    ComUart

Example     If the specified serial port is an NS16550 set the trigger

level to 1 byte.

void SetThreshIf550(COM com)

{

if(ComUart(com) == NS16550)

ComNS550(com, T550_1);

}

ComTurbo

------------------------------------------------------------------

Function    Disables/re-enables high priority interrupts.

Syntax      #include "cport.h"

void ComTurbo(int options);

Parameters

options - Bit mask of options. See Table A-6 in Appendix A for possible

options.

Remarks     During high speed data exchanges, certain events that cause

high priority interrupts can inhibit serial communications and

possibly cause (over run) errors. This is especially true of

the timer, which periodically uses extra time and the

keyboard, if keys are pressed.

ComTurbo disables selected high priority interrupts or

restores all of them. The high priority interrupts should

never be disabled permanently or for long periods of time. The

timer interrupt is not actually disabled but diverted,

allowing the system clock to continue to update. All other

options are actually disabled when specified.

Example     Disable the keyboard and divert the timer while a block is

transferred at high speed.

#include <time.h>

#define BLK_SZ 128

4-17

int GetBlock(COM com, byte* block, clock_t timeout)

{

int rv;

/* assume 115200 baud

*/

ComTurbo(TIMER | KEYBOARD);

timeout += clock();

while(ComLenRx(com) < BLK_SZ)

{

if(clock() > timeout)

{

ComTurbo(OFF);

return(0);

}

}

rv = ComIn(com, block, BLK_SZ);

ComTurbo(OFF);

return(rv);

}

4-18

Input Functions

---------------

Basic

ComGetc gets a character from the receive queue.

ComGets gets a string of characters from the receive.

ComIn gets a block of data from the receive queue.

ComFlushRx flushes the receive queue and receiver.

ComLenRx calculates the number of characters/bytes in the receive queue.

Intermediate

ComPeek returns a copy of the head character/byte without removing it

from the receive queue.

ComRxScan scans the receive queue for a specified character.

4-19

ComGetc

------------------------------------------------------------------

Function    Fetches a character from the specified serial port's receive

queue.

Syntax      #include "cport.h"

char ComGetc(COM);

Remarks     ComGetc fetches the head character from the specified serial

port's receive queue. If there are no characters in the

receive queue, ComGetc returns a NULL character ('\0').

Parameters

COM    - Handle of the serial port.

Return value   If successful, ComGetc returns the character from the receive

queue. If it fails, it returns a NULL character ('\0').

See also    ComPutc, ComGets, ComIn, ComPeek

Example     Get a character from the modem and test if it is a start of

header control character.

#define SOH 1

int CheckForSOH(COM modem)

{

char c;

c = ComGetc(modem);

return(c == SOH);

}

ComGets

------------------------------------------------------------------

Function    Fetches a string of characters from the specified serial

port's receive queue.

Syntax      #include "cport.h"

char *ComGets(COM, char *str, int maxc, char termc);

Parameters

COM    - Handle of the serial port.

str    - Pointer to the buffer where the string of characters will be

stored.

maxc   - Maximum number of characters to fetch.

termc  - Termination character. ComGets will return if this character

is encountered.

Remarks     ComGets fetches a string of characters from the specified

serial port's receive queue. The string will be stored in str.

Str must be at least maxc + 1 characters long to allow for the

4-20

NULL terminator. ComGets will return on the first occurrence

of one of three conditions: 1) the character termc is

encountered; 2) maxc characters are reached; or 3) no more

characters are available in the receive queue.

Return value   ComGets returns a pointer to the NULL terminated string.

See also    ComPuts, ComGetc, ComIn, ComLenRx

Example     Get a measurement from the specified instrument if one is

ready.

#define MEAS_SIZ   24

char* GetMeasurment(COM instrument, char* buf)

{

if(ComLenRx(instrument) > MEAS_SIZ)

return(ComGets(instrument, buf, MEAS_SIZ, '\n'));

else

return(NULL);

}

ComIn

------------------------------------------------------------------

Function    Fetches a block of bytes from the specified serial port's

receive queue.

Syntax      #include "cport.h"

unsigned ComIn(COM, void *abyte, unsigned nbyte);

Parameters

COM    - Handle of the serial port.

abyte  - Pointer to a buffer where the block of bytes will be stored.

nbyte  - Number of bytes to get from the receive queue.

Remarks     ComIn fetches a block of bytes from the specified serial

port's the receive queue and stores it in abyte. ComIn will

always fetch num_byte bytes unless not enough bytes are

available. ComIn is well suited for receiving binary data.

Return value   ComIn returns the number of bytes actually fetched.

See also    ComOut, ComGetc, ComGets, ComLenRx

4-21

Example     Get a block of 1024 bytes from the serial port.

#include <time.h>

#define BLK_SZ    1024

#define TIMEOUT   -1

int GetBlock(COM datalink, byte* buf, clock_t timeout)

{

timeout += clock();

while(ComLenRx(datalink) < BLK_SZ)

{

if(clock() > timeout)

return(TIMEOUT);

}

return(ComIn(datalink, buf, BLK_SZ));

}

ComFlushRx

------------------------------------------------------------------

Function    Flushes the specified serial port's receiver and receive

queue.

Syntax      #include "cport.h"

void ComFlushRx(COM);

Parameters

COM    - Handle of the serial port.

Remarks     ComFlushRx deletes the specified serial port's receive queue

and the UART of any existing characters/bytes.

See also    ComLenRx, ComFlushTx

Example     If a receive flush command is in order, flush the receiver.

void RemoteCommand(COM com, int command)

{

switch(command)

{

case TX_FLUSH:

ComFlushTx(com);

break;

case RX_FLUSH:

ComFlushRx(com);

break;

...continued

}

}

4-22

ComLenRx

------------------------------------------------------------------

Function    Calculates the number of characters/bytes in the specified

serial port's receive queue.

Syntax      #include "cport.h"

unsigned ComLenRx(COM);

Parameters

COM    - Handle of the serial port.

Remarks     ComLenRx calculates the number of characters/bytes in the

specified serial port's receive queue.

Return value   ComLenRx returns the number of characters/bytes residing

in the receive queue.

See also    ComFlushRx, ComLenTx

Example     Get a measurement from the specified instrument if one is

ready.

#define MEAS_SIZ   24

char* GetMeasurment(COM instrument, char* buf)

{

if(ComLenRx(instrument) > MEAS_SIZ)

{

ComGets(instrument, buf, MEAS_SIZ, '\n');

return(buf);

}

else

return(NULL);

}

ComPeek

------------------------------------------------------------------

Function    Fetches a copy of the head character from the specified

serial port's receive queue.

Syntax      #include "cport.h"

char ComPeek(COM);

Parameters

COM    - Handle of the serial port.

Remarks     ComPeek fetches a copy of the head character from the

specified serial port's receive queue. ComPeek will not

remove the character from the queue.

Return value   ComPeek returns the copy of the head character.

4-23

See also    ComGetc, ComRxScan

Example     Get the head character, if it is a control character.

#include <ctype.h>

char GetIfControl(COM com)

{

char peek;

peek = ComPeek(com);

if(iscntrl(peek))

return(ComGetc(com));

else

return('\0');

}

ComRxScan

------------------------------------------------------------------

Function    Scans the receive queue of the specified serial port for a

specific character.

Syntax      #include "cport.h"

unsigned ComRxScan(COM, char c);

Parameters

COM    - Handle of the serial port.

c      - Character to scan for.

Remarks     ComRxScan searches for the character c in the specified serial

port's receive queue. If the character is not found, ComRxScan

returns a zero. Otherwise it returns the offset into the

receive queue of the first occurrence of c starting at 1 for

the first character. This is the number of characters that

must be fetched to get all data up to and including the

character scanned for.

Return value   ComRxScan returns the offset into the receive queue to the

first occurrence of c. If c does not exist, ComRxScan

returns a zero.

See also    ComPeek

4-24

Example     Get the next line terminated with a '\n'.

char *GetLine(COM com, char *buffer)

{

int n;

n = ComRxScan(com, '\n');

if(n > 0 && n < 80)

return(ComGets(com, buffer, n, '\n'));

else

return(NULL);

}

Example     Get the next message terminated with a EOT control character.

#define EOT   0x4

int GetMessage(COM com, byte *buffer)

{

int n;

n = ComRxScan(com, EOT);

if(n > 0 && n < 80)

return(ComIn(com, buffer, n));

else

return(0);

}

4-25

Output Functions

----------------

Basic

ComPutc puts a character to the transmit queue.

ComPuts puts a string of characters to the transmit queue.

ComOut puts a block of data to the transmit queue.

ComFlushTx flushes the transmit queue and transmitter.

ComLenTx calculates the number of characters in the transmit queue.

Advanced

ComTxWait pauses program execution until the transmitter has emptied.

4-26

ComPutc

------------------------------------------------------------------

Function    Puts a character into the specified serial port's transmit

queue.

Syntax      #include "cport.h"

int ComPutc(COM, char c);

Parameters

COM    - Handle of the serial port.

c      - Character to be put into the transmit queue.

Remarks     ComPutc puts a character into the specified serial port's

transmit queue to be transmitted. If the transmit queue is

full, the character is not put and ComPutc returns a 0.

Return value   On success, ComPutc returns a 1. If it fails, it returns 0.

See also    ComGetc, ComPuts, ComOut

Example     Echo the keyboard to the serial port when fetching a keystroke.

#include <conio.h>

static COM echo;

char GetKey(void)

{

char c;

if(kbhit())

{

c = getch();

ComPutc(echo, c);

return(c);

}

else

return('\0');

}

ComPuts

------------------------------------------------------------------

Function    Puts a string of characters into the specified serial port's

transmit queue.

Syntax      #include "cport.h"

int ComPuts(COM, const char *str);

4-27

Parameters

COM    - Handle of the serial port.

str    - Pointer to the string to be put into the transmit queue.

Remarks     ComPuts puts the string str into the specified serial port's

transmit queue to be transmitted. If the transmit queue

becomes full, ComPuts will not attempt to put any more

characters in the queue.

Return value   ComPuts returns the actual number of characters put into

the transmit queue.

See also    ComGets, ComPutc, ComOut

Example   Dial a number on the modem.

#include <string.h>

void ModDial(COM modem, const char* number)

{

char dial[24] = "ATDT";

strcat(dial, number);

strcat(dial, "\r");

ComPuts(modem, dial);

}

ComOut

------------------------------------------------------------------

Function    Puts a block of bytes into the specified serial port's

transmit queue.

Syntax      #include "cport.h"

unsigned ComOut(COM, const void *abyte, unsigned nbyte);

Parameters

COM    - Handle of the serial port.

abyte  - Pointer to the block of data to be put  into the transmit

queue.

nbyte  - Number of bytes to be put into the transmit  queue.

Remarks     ComOut puts the block of bytes pointed to by abyte into the

specified serial port's transmit queue to be transmitted.

Return value   ComOut returns the number of bytes actually put into the

transmit queue.

See also    ComIn, ComPutc, ComPuts

4-28

Example     Send a block of 128 bytes to the transmitter.

#define BLK_SZ 128

int SendBlock(COM com, const byte* block)

{

int rv;

rv = ComOut(com, block, BLK_SZ);

if(rv < BLK_SZ)

return(-1);

else

return(0);

}

ComFlushTx

------------------------------------------------------------------

Function    Flushes the specified serial port's transmit queue.

Syntax      #include "cport.h"

void ComFlushTx(COM);

Parameters

COM    - Handle of the serial port.

Remarks     ComFlushTx deletes any existing characters in the specified

serial port's transmit queue.

See also    ComFlushRx, ComLenTx

Example     If a transmit flush command is in order, flush the transmit

queue.

void RemoteCommand(COM com, int command)

{

switch(command)

{

case TX_FLUSH:

ComFlushTx(com);

break;

case RX_FLUSH:

ComFlushRx(com);

break;

...continued

}

}

4-29

ComLenTx

------------------------------------------------------------------

Function    Calculates the number of characters/bytes in the specified

serial port's transmit queue.

Syntax      #include "cport.h"

unsigned ComLenTx(COM);

Parameters

COM    - Handle of the serial port.

Remarks     ComLenTx calculates the number of characters/bytes in the

specified serial port's transmit queue.

Return value   ComLenTx returns the number of characters/bytes in the

transmit queue.

See also    ComLenRx, ComFlushTx

Example     Check if the transmitter has reached 3/4 of its capacity.

#define TX_SIZE      1024

int IsAboveThresh(COM sprinter)

{

return(ComLenTx(sprinter) > (TX_SIZE * 3 / 4));

}

ComTxWait

------------------------------------------------------------------

Function    Waits for the specified serial port's transmitter to empty

Syntax      #include "cport.h"

void ComTxWait(COM);

Parameters

COM    - Handle of the serial port.

Remarks     When a transmit queue becomes empty, the UART may not have

actually transmitted the last of the bytes. A program may need

to know when all the data has actually been transmitted so

that it may, for example, change the state of the hardware

lines during half duplex communications. ComTxWait will return

after the UART has been emptied of data.

See Also    ComLenTx

4-30

Example     Send a string to a half duplex device. This examples assumes

that the device is monitoring our RTS line to determine a

transmit or receive state.

void SendMessage(COM device, const char* msg)

{

ComRts(device, ON);

ComPuts(device, msg);

while(ComLenTx(device));

ComTxWait(device);

ComRts(device, OFF);

}

4-31

Status Functions

----------------

Basic

ComError detects hardware and software errors.

Intermediate

ComStatus returns the status of the software and input hardware lines.

ComRts controls the RTS output line.

ComDtr controls the DTR output line.

Advanced

ComOut1 controls the general purpose OUT1 output line.

ComUart determines what type of UART chip is residing at a serial port.

4-32

ComError

------------------------------------------------------------------

Function    Determines if any errors have occurred on the specified

serial port since the last call to ComError.

Syntax      #include "cport.h"

unsigned ComError(COM);

Parameters

COM    - Handle of the serial port.

Remarks     ComError determines if any errors are pending on the specified

serial port. After a call to ComError, the errors are cleared.

ComError returns a bit ORed error word. See Table A-7 in

Appendix A for a list of bit definitions.

Return value   If an error has occurred, ComError returns an error word.

If no errors have occurred, it returns a 0.

See also    ComStatus

Example     Determine the source of the error if one has occurred.

#define HARDWARE   (PARITY | FRAMING | BREAK | RX_FIFO)

void CheckForErrors(COM dvm)

{

unsigned err;

if((err = ComError(dvm)) == 0)

return;

else if(err & OVERUN)

{

/* over run error */

}

else if(err & HARDWARE)

{

/* hardware errors */

}

else if(err & (TXFULL | RXFULL))

{

/* queue errors */

}

}

4-33

ComStatus

------------------------------------------------------------------

Function    Retrieves the status of the specified serial port's software

and hardware lines.

Syntax      #include "cport.h"

unsigned ComStatus(COM);

Parameters

COM    - Handle of the serial port.

Remarks     ComStatus retrieves the status of the specified serial port's

software and hardware lines. ComStatus returns a bit ORed

status word. See Table A-8 in Appendix A for a list of bit

definitions.

Return value   ComStatus returns the status word of the software and

hardware lines.

See also    ComError, ComRts, ComDtr

Example     Test if the modem is powered up and ready.

#include <dos.h>

int CheckModem(COM modem)

{

ComDtr(modem, ON);

delay(40);

return((ComStatus(modem) & DSR) != 0);

}

ComRts

------------------------------------------------------------------

Function    Asserts or inhibits the specified serial port's Request to

Send line.

Syntax      #include "cport.h"

void ComRts(COM, byte on_off);

Parameters

COM    - Handle of the serial port.

on_off - Determines whether RTS is to be asserted or inhibited. See

Table A-10 in Appendix A.

Remarks     ComRts asserts or inhibits the specified serial port's RTS

line depending on the value of on_off. If the value of on_off

is ON, RTS is asserted. If the value of on_off is OFF, RTS is

inhibited. In many cases, this signal will be looked at by the

4-34

remote device to determine if it is safe to transmit. This

gives the programmer the ability to turn off the remote

device's transmitter.

NOTE: ComRts should never be called when RTS hardware

handshaking is enabled.

See also    ComDtr, ComStatus, ComHandshake

Example     Put the device in an on or off line state. (Assumes the

remote device will not send data without our RTS asserted)

void OffLine(COM device)

{

ComRts(device, OFF);

}

void OnLine(COM device)

{

ComRts(device, ON);

}

ComDtr

------------------------------------------------------------------

Function    Asserts or inhibits the specified serial port's  Data

Terminal Ready line.

Syntax      #include "cport.h"

void ComDtr(COM, byte on_off);

Parameters

COM    - Handle of the serial port.

on_off - Determines whether DTR is to be asserted or inhibited. See

Table A-10 in Appendix A.

Remarks     ComDtr asserts or inhibits the specified serial port's DTR

line depending on the value of on_off. If the value of on_off

is ON, DTR is asserted. If the value of on_off is OFF, DTR is

inhibited. In many cases, this signal will be looked at by the

remote device to determine if it is safe to transmit. This

gives the programmer the ability to turn off the remote

device's transmitter.

NOTE: ComDtr should never be called when DTR hardware

handshaking is enabled.

See also    ComRts, ComStatus, ComHandshake

4-35

Example     Put the device in an on or off line state. (Assumes the remote

device will not send data without our DTR asserted).

void OffLine(COM device)

{

ComDtr(device, OFF);

}

void OnLine(COM device)

{

ComDtr(device, ON);

}

ComOut1

------------------------------------------------------------------

Function    Asserts or inhibits the specified serial port's Out1 line.

Syntax      #include "cport.h"

void ComOut1(COM, byte on_off);

Parameters

COM    - Handle of the serial port.

on_off - Determines whether OUT1 is to be asserted or inhibited. See

Table A-10 in Appendix A.

Remarks     ComOut1 asserts or inhibits the specified serial port's general

purpose OUT1 line depending on the value of on_off. If the

value of on_off is ON, OUT1 is asserted. If the value of

on_off is OFF, OUT1 is inhibited. OUT1 is a general purpose

output sometimes utilized by a device for a special feature

or reset.

See also    ComRts, ComDtr

Example     Reset a smart modem.

void SmartReset(COM modem)

{

ComOut1(modem, ON);

delay(50);

ComOut1(modem, OFF);

}

ComUart

------------------------------------------------------------------

Function    Determines the type of UART at the specified serial port.

Syntax      #include "cport.h"

int ComUart(COM);

4-36

Parameters

COM    - Handle of the serial port.

Remarks     ComUart determines the type of UART chip that resides at the

specified serial port. See Table A-11 in Appendix A for a list

of UART types.

Return value   ComUart returns a value representative of the type of UART

that was detected.

See Also    ComNS550

Example     If the specified serial port has an NS16550 UART, set the

trigger level to 1 byte.

void SetThreshIf550(COM com)

{

if(ComUart(com) == NS16550)

ComNS550(com, T550_1);

}

4-37

Data Integrity

--------------

Intermediate

ComChecksum calculates a byte checksum of a block of data.

ComCrc16 calculates a 16 bit cyclic redundancy check (CRC) on a block

of data.

ComCrc32 calculates a 32 bit cyclic redundancy check (CRC) on a block

of data.

4-38

ComChecksum

------------------------------------------------------------------

Function    Performs a byte checksum calculation on a block of data.

Syntax      #include "cport.h"

byte ComChecksum(const void *abyte, unsigned nbyte);

Parameters

abyte  - Pointer to the block on which the checksum is to be performed.

nbyte  - Number of bytes in the block.

Remarks     ComChecksum performs a byte checksum on the block of bytes

pointed to by abyte. The checksum is calculated by summing all

the bytes in the block.

Return value   ComChecksum returns a one byte checksum.

See also    ComCrc16, ComCrc32, ComIn, ComOut

Example     Verify that the actual checksum matches the received checksum.

struct Xblock{

byte num;

byte _num;

byte data[128];

byte checksum;

}block;

int GetXblock(COM com, Xblock *block)

{

ComIn(com, block, sizeof(Xblock));

if((ComChecksum(block->data, 128) - block->checksum) != 0)

return(-1);

else

return(0);

}

ComCrc16

------------------------------------------------------------------

Function    Performs a 16 bit cyclic redundancy check on a block of data.

Syntax      #include "cport.h"

unsigned ComCrc16(const void *abyte, unsigned nbyte);

Parameters

abyte  - Pointer to the block on which the 16 bit cyclic redundancy

check is to be performed.

nbyte  - Number of bytes in the block.

4-39

Remarks     ComCrc16 performs a 16 bit cyclic redundancy check on the

block of data pointed to by abyte.

Return value   ComCrc16 returns the 16 bit CRC value.

See also    ComCrc32, ComChecksum, ComIn, ComOut

Example     Verify that the actual CRC matches the received CRC.

struct Xblock{

byte num;

byte _num;

byte data[128];

unsigned crc;

}block;

int GetXblock(COM com, Xblock *block)

{

ComIn(com, block, sizeof(Xblock));

/* Check the calculated CRC against the block's CRC.

*/

if(ComCrc16(block->data, 128) != block->crc)

return(-1);

else

return(0);

}

ComCrc32

------------------------------------------------------------------

Function    Performs a 32 bit cyclic redundancy check on a block of data.

Syntax      #include "cport.h"

unsigned long ComCrc32(const void *abyte, unsigned nbyte);

Parameters

abyte  - Pointer to the block on which the 32 bit cyclic redundancy

check is to be performed.

nbyte  - Number of bytes in the block.

Remarks     ComCrc32 performs a 32 bit cyclic redundancy check on the

block of data pointed to by abyte.

Return value   ComCrc32 returns the 32 bit CRC value.

See also    ComCrc16, ComChecksum, ComIn, ComOut

4-40

Example     Verify that the actual CRC matches the received CRC.

struct Packet

{

byte num;

byte data[1024];

unsigned long crc;

}packet;

int GetBlock(COM com, Packet* packet)

{

ComIn(com, packet, sizeof(Packet));

/* Check the calculated CRC against the packet's CRC.

*/

if(ComCrc32(packet->data, 1024) != packet->crc)

return(-1);

else

return(0);

}

4-41

Misc Functions

--------------

Advanced

ComSetBreak sets a break condition.

ComClrBreak clears a break condition.

ComPutScrtch writes to the UART's scratch register.

ComGetScrtch reads the UART's scratch register.

4-42

ComSetBreak

------------------------------------------------------------------

Function    Set a break condition on the specified serial port.

Syntax      #include "cport.h"

void ComSetBreak(COM);

Parameters

COM  - Handle of the serial port.

Remarks     ComSetBreak sets a break condition on the specified serial

port. The break condition will stay in effect until a

subsequent call to ComClrBreak is made.

See also    ComClrBreak

Example     If the an abort condition has been declared, set a break

condition. When it clears, release the break condition.

void Abort(COM com, int abrt)

{

static int brkflg;

if(abrt && !brkflg)

ComSetBreak(com);

else if(!abrt && brkflg)

ComClrBreak(com);

}

ComClrBreak

------------------------------------------------------------------

Function    Clears a previously set break condition on the specified

serial port.

Syntax      #include "cport.h"

void ComClrBreak(COM);

Parameters

COM    - Handle of the serial port.

Remarks     ComClrBreak clears a previously set break condition on the

specified serial port.

See also    ComSetBreak

4-43

Example     If the an abort condition has been declared, set a break

condition. When it clears, release the break condition.

void Abort(COM com, int abrt)

{

static int brkflg;

if(abrt && !brkflg)

ComSetBreak(com);

else if(!abrt && brkflg)

ComClrBreak(com);

}

ComPutScrtch

------------------------------------------------------------------

Function    Writes a byte to the specified serial port's scratch register.

Syntax      #include "cport.h"

void ComPutScrtch(COM, byte abyte);

Parameters

COM    - Handle of the serial port.

abyte  - The byte to be written to the scratch register.

Remarks     ComPutScrtch writes the byte value abyte to the specified

serial port's UART scratch register. Not all UARTs have a

scratch register.

See also    ComGetScrtch

Example     Exchange a value with the scratch register.

byte ScratchExchange(COM com, byte value)

{

byte rv;

rv = ComGetScrtch(com);   /* read the scratch register */

ComPutScrtch(com, value);   /* write to the scratch register */

return(rv);

}

ComGetScrtch

------------------------------------------------------------------

Function    Fetches the byte stored in the specified serial port's scratch

register.

Syntax      #include "cport.h"

byte ComGetScrtch(COM);

4-44

Parameters

COM    - Handle of the serial port.

Remarks     ComGetScratch reads the byte in the specified serial port's

UART scratch register. Not all UARTs have a scratch register.

Return value   ComGetScrtch returns the byte read from the UART scratch

register.

See also    ComPutScrtch

Example     Exchange a value with the scratch register.

byte ScratchExchange(COM com, byte value)

{

byte rv;

rv = ComGetScrtch(com);   /* read the scratch register */

ComPutScrtch(com, value);   /* write to the scratch register */

return(rv);

}

5-1

C++ Reference

=============

Control Functions

-----------------

Basic

Cport::Cport constructs serial port instance with either discrete

parameters or a structure of parameters.

Cport::operator int returns the results of constructing a serial port

instance.

Cport::Handshake determines if and what type of handshaking scheme

will be enabled and sets the receive queue threshold.

Intermediate

Cport::Param reports on the current parameters and handshaking under

which a serial port is currently operating.

Cport::Baud changes the baud rate.

Cport::Mode changes the word length, number of stop bits and parity.

Advanced

Cport::RxQ resizes the receive queue.

Cport::TxQ resizes the transmit queue.

Cport::Tx allows advanced control over the  transmitter.

Cport::NS550 sets the receive fifo threshold of an NS16550 UART.

Cport::Turbo temporarily disables/re-enables selected high priority

interrupts that may impede high speed communications.

5-2

Cport::Cport

------------------------------------------------------------------

Function    Constructor.

Syntax      #include "cport.h"

Cport::Cport(unsigned id, int baud, byte mode = W8|S1|NONE,

unsigned rxQ = 1024, unsigned txQ = 512, byte htype = OFF);

or

Cport::Cport(unsigned id, int baud, byte mode, unsigned rxQ,

unsigned txQ, byte htype, unsigned thresh);

or

Cport::Cport(const CPARAM& param)

Parameters

id     - Value that identifies the serial port. The upper 4 bits define

the interrupt request line, while the lower 12 bits define the

port address. This technique allows for possibility of custom

configurations. See Table A-1 in Appendix A for a list of

predefined values.

baud   - Value that defines the baud rate. The actual value is the

baud rate divisor, which allows for the possibility of custom

baud rates. The baud rate divisor = f / (16 * baud rate) where

f is 1.8432e+6. See Table A-2 in Appendix A for a list of

predefined baud rate divisors.

mode   - Bit ORed value defining the word length, number of stop bits

and the parity. See Table A-3 in Appendix A for a list of mode

components.

* rxQ  - Size, in bytes, of the receive queue.

* txQ  - Size, in bytes, of the transmit queue.

* Limited to 32K in small and medium memory models.

htype  - Type of handshaking to be used. See Table A-5 in Appendix A

for a list of handshaking components.

thresh - Receive queue threshold.

Remarks     The constructor creates an instance of the specified serial

port. If the constructor is unable to open the specified

serial port, the instance of the serial port will return an

error code when casted to an integer. If the port is

successfully opened, the instance will return a zero when

casted to an integer.

The 4 most significant bits in id represent the interrupt

vector. The lower 12 bits represent the port address.

Constants are provided to facilitate the formulation of the id

parameter. The COMx values will work in most cases, however,

5-3

the IRQx values can be ORed with the PORTx or BIOSx values to

produce other combinations. The PORTx values are actual port

addresses where the serial ports are normally located. The

BIOSx values are actually macros, which fetch the port address

from the BIOS data area.

The parameter baud is the baud rate divisor, that determines

the baud rate. Achieving baud rates above 19200 is largely

dependent on many variables, such as machine speed, cable

length, handshaking activity, degree bidirectional transfers,

number of open serial ports, the type UART(s) used and

activity of other hardware interrupts.

The parameter mode determines the word length, number of stop

bits and parity. Bits 0 and 1 determine the word length, bit 2

determines the number of stop bits, bit 3 enables parity and

bits 4 and 5 determine the type of parity check.

RxQ and txQ set the size of the receive and transmit queues

respectively. RxQ should be at least 4 times larger than the

largest string or block of data expected. TxQ can be set as

small as 1, but it is not recommended. In the small and medium

memory models where all data is limited to 64K, the size of

the queues are limited. In the compact and large memory

models, each queue can be as large as 64K -1 (65534 bytes).

See Also    Cport::operator int

Example     Open COM1 at 9600 baud with 8 bit words, 1 stop bit, no

parity, a 1024 byte receiver and a 512 byte transmitter.

int main(void)

{

Cport com1(COM1, B9600, W8|S1|NONE, 1024, 512);

if((int)com1 != NO_ERR)

{

switch(com1)

{

case RX_ALC:

case TX_ALC:

case GEN_ALC:

// allocation error

break;

case NO_UART:

// no UART

break;

default:

// all other errors

break;

}

}

...continued

5-4

Cport::operator int

------------------------------------------------------------------

Function    Gets the error code from when the instance of the serial port

was constructed.

Syntax      #include "cport.h"

Cport::operator int()

Remarks     When an instance of a serial port is constructed, the

specified serial port is not necessarily successfully opened.

Cport::operator int is an integer typecast used for procuring

the error code from when an instance of the serial port was

constructed. The actual typecasting may or may not be

necessary, but is recommended.

Return value   Cport::operator int returns an error code from when the

instance of the serial port was constructed. See Table A-4

in Appendix A for a list of possible return values.

See Also    Cport::Cport

Example     Check for errors after instance of a serial port is

constructed.

int main(void)

{

Cport com1(COM1, B9600, W8|S1|NONE, 1024, 512);

// The instance 'com1' is casted to an int to procure the

// error code.

//

if((int)com1 != NO_ERR)

{

switch(com1)

{

case RX_ALC:

case TX_ALC:

case GEN_ALC:

// allocation error

break;

case NO_UART:

// no UART

break;

default:

// all other errors

break;

}

}

...continued

5-5

Cport::Handshake

------------------------------------------------------------------

Function    Enables or disables handshaking (flow control ) for a serial

port.

Syntax      #include "cport.h"

void Cport::Handshake(byte htype, unsigned thresh);

Parameters

htype  - Type of handshaking to be used. See Table A-5 in Appendix A

for a list of handshaking components.

thresh - Receive queue threshold.

Remarks     Cport::Handshake enables or disables software and/or hardware

handshaking for the serial port instance. In software

handshaking, XON and XOFF characters are used to suspend and

resume communications. Hardware handshaking uses any or all of

the DTR, DSR, RTS, CTS and DCD lines to suspend and resume

communications. Thresh determines when the receiver will

assert and inhibit the corresponding handshaking. If no

receive handshaking is specified (RTS, DTR or S_RX), the

threshold is not used and may be set to zero.

See also    Cport::Param, Cport::Cport, Cport::Tx

Example     If COM1 is successfully opened, turn on some hardware

handshaking and set the receiver threshold to 3/4 of the

receiver's full size. The transmitter will not transmit unless

both DSR and DCD are asserted. The receiver will inhibit DTR

if the receive queue length exceeds the threshold.

int main(void)

{

Cport com1(COM1, B9600, W8|S1|NONE, 1024, 512);

if((int)com1 != NO_ERR)

{

// error

}

com1.Handshake(DSR|DTR|DCD, 768);

...continued

return(0);

}

5-6

Cport::Param

------------------------------------------------------------------

Function    Retrieves the parameters of a serial port.

Syntax      #include "cport.h"

Cport::Param(CPARAM& param)

Parameters

param  - Reference to a type CPARAM where the parameters will be

stored.

Remarks     Cport::Param retrieves the parameters and handshaking status

of the serial port instance and stores them in a type CPARAM.

See also    CPARAM, Cport::Cport

Example     Create an instance of a serial port with the same

configuration as one that already exists.

Cport OpenCopy(int id, Cport& copyme)

{

CPARAM param;

copyme.Param(param);

param.id = id;

return(Cport(param));

}

Cport::Baud

------------------------------------------------------------------

Function    Changes the baud rate of a serial port.

Syntax      #include "cport.h"

void Cport::Baud(int baud);

Parameters

baud   - Value that defines the baud rate. The actual value is the

baud rate divisor, which allows for possibility of custom baud

rates. The baud rate divisor = f / (16 * baud rate) where

f is 1.8432e+6. See Table A-2 in Appendix A for a list of

predefined baud rate divisors.

Remarks     Cport::Baud changes the baud rate of the serial port instance.

The parameter baud is the baud rate divisor that determines

the baud rate.

See also    Cport::Cport, Cport::Param, Cport::Mode

5-7

Example     Set the specified serial port to a new baud rate.

#define NBAUD   4

int SetBaud(Cport& com, unsigned n)

{

int baud[NBAUD] = { B9600, B2400, B1200, B300 };

if(n < NBAUD)

{

com.Baud(baud[n]);

return(0);

}

else

return(-1);

}

Cport::Mode

------------------------------------------------------------------

Function    Changes the word length, number of stop bits and parity of a

serial port.

Syntax      #include "cport.h"

void Cport::Mode(byte mode);

Parameters

mode   - Bit ORed value defining the word length, number of stop bits

and the parity. See Table A-3 in Appendix A for a list of mode

components.

Remarks     Cport::Mode changes the word length, number of stop bits and

parity of the serial port instance. The parameter mode

determines the word length, number of stop bits and parity of

the serial port instance. Bits 0 and 1 determine the word

length, bit 2 determines the number of stop bits, bit 3 enables

parity and bits 4 and 5 determine the type of parity check.

See also    Cport::Cport, Cport::Param, Cport::Baud

Example     Set the serial port to a new mode.

#define NMODE   2

int SetMode(Cport& com, unsigned n)

{

int mode[NMODE] = { W8|S1|NONE, W7|S1|EVEN };

if(n < NMODE)

{

com.Mode(mode[n]);

return(0);

}

return(-1);

}

5-8

Cport::RxQ

------------------------------------------------------------------

Function    Resizes the receive queue of a serial port.

Syntax      #include "cport.h"

unsigned Cport::RxQ(unsigned size);

Parameters

size   - New receive queue size.

Remarks     Cport::RxQ resizes the receive queue of the serial port

instance. The memory currently allocated for the receive queue

is freed and new memory is allocated. If Cport::RxQ is unable

to allocate the specified amount memory, it will try to

reallocate memory for the previous size. Cport::RxQ returns

the new queue size. If memory was successfully allocated for

the new size, the return value will be size. If the previous

size is reallocated, the return value will be the previous

size. If no memory could be allocated, Cport::RxQ returns

zero.

No data can be received while the new receive queue is being

resized. However, data in the transmit queue will continue to

be transmitted normally.

Return value   Cport::RxQ returns the new receive queue size.

See also    Cport::TxQ, Cport::Cport

Example     Enlarge the receive queue for a file transfer.

#define LARGE_RX   4096

int ReceiveFile(Cport& com, const char *filename)

{

CPARAM param;

com.Param(param);

if(com.RxQ(LARGE_RX) != LARGE_RX)

return(-1);

... transfer file here

// Restore original size

if(com.RxQ(param.rxQ) != param.rxQ)

return(-2);

else

return(0);

}

5-9

Cport::TxQ

------------------------------------------------------------------

Function    Resizes the transmit queue of a serial port.

Syntax      #include "cport.h"

unsigned Cport::TxQ(unsigned size);

Parameters

size   - New transmit queue size.

Remarks     Cport::TxQ resizes the transmit queue of the serial port

instance. The memory currently allocated for the transmit

queue is freed and new memory is allocated. If Cport::TxQ is

unable to allocate the specified amount memory, it will try

to reallocate memory for the previous size. Cport::TxQ returns

the new queue size. If memory was successfully allocated for

the new size, the return value will be size. If the previous

size is reallocated, the return value will be the previous

size. If no memory could be allocated, Cport::TxQ returns zero.

No data can be transmitted while the new transmit queue is

being resized. However, data in the receive queue will

continue to be received normally.

Return value   ComTxQ returns the new transmit queue size.

See also    Cport::RxQ, Cport::Cport

Example     Enlarge the transmit queue for a file transfer.

#define LARGE_TX   4096

int TransmitFile(Cport& com, const char *filename)

{

CPARAM param;

com.Param(param);

if(com.TxQ(LARGE_TX) != LARGE_TX)

return(-1);

... transfer file here

// Restore original size

if(com.TxQ(param.txQ) != param.txQ)

return(-2);

else

return(0);

}

5-10

Cport::Tx

------------------------------------------------------------------

Function    Transmitter control.

Syntax      #include "cport.h"

int Cport::Tx(int cmnd);

Parameters

cmnd   - Determines the action to be taken on the transmitter See

Table A-9 in Appendix A for a list of transmitter options.

Remarks     Cport::Tx  allows advanced control of the Cport transmitter

The transmitter can be turned off and on to suspend and resume

transmissions. This function can be very useful when

implementing your own handshaking scheme. In addition, it can

inform the output functions to transmit directly when the

transmitter is turned off.

Return value   Cport::Tx returns the previous status of the transmitter.

See also    Cport::Handshake, Cport::Cport, Cport::Param

Example     Put a priority character directly  to the  serial port.

void PriorityPutc(Cport& com, char c)

{

int tx;

tx = com.Tx(TX_DIRECT | OFF);

com.Put(c);

com.Tx(tx);

}

Cport::NS550

------------------------------------------------------------------

Function    NS16550 UART setup.

Syntax      #include "cport.h"

void Cport::NS550(int trigger);

Parameters

trigger - Trigger threshold for the UART receive fifo. See Table A-12

in Appendix A for a list of trigger levels.

Remarks     If Cport detects an NS16550 UART when a serial port is opened,

Cport automatically takes advantage of it's advanced capabilities.

The receive fifo threshold is the number of bytes in the receive

fifo that will cause an interrupt to be generated. By default,

the receive fifo threshold is set to 14 bytes. Cport::NS550

allows this receive fifo trigger threshold to be changed.

5-11

See Also    Cport::Uart

Example     If the specified serial port is an NS16550, set the trigger

level to 1 byte.

void SetThreshIf550(Cport& com)

{

if(com.Uart() == NS16550)

com.NS550(T550_1);

}

Cport::Turbo

------------------------------------------------------------------

Function    Disables/re-enables high priority interrupts.

Syntax      #include "cport.h"

static void Cport::Turbo(int options);

Parameters

options - Bit mask of options. See Table A-6 in Appendix A for a list

of turbo options.

Remarks     During high speed data exchanges certain events that cause

high priority interrupts can impede serial communications and

possibly cause (over run) errors. This is especially true of

the timer, which periodically uses extra time, and the

keyboard, if keys are pressed.

Cport::Turbo disables selected high priority interrupts or

restores all of them. The high priority interrupts should

never be disabled permanently or for long periods of time. The

timer interrupt is not actually disabled but divert, allowing

the system clock to continue to update. All other options are

actually disabled when specified. Cport::Turbo is a static

function. Although not very useful, it can be called prior to

and/or independent of any instance of a serial port by

specifying it's full name: Cport::Turbo(options).

Example     Disable the keyboard and divert the timer while a block is

transferred at high speed.

5-12

#include <time.h>

#define BLK_SZ 128

int GetBlock(Cport& com, byte* block, clock_t timeout)

{

int rv;

// assume 115200 baud

//

com.Turbo(TIMER | KEYBOARD);

timeout += clock();

while(com.LenRx() < BLK_SZ)

{

if(clock() > timeout)

{

com.Turbo(OFF);

return(0);

}

}

rv = com.In(block, BLK_SZ);

com.Turbo(OFF);

return(rv);

}

5-13

Input Functions

---------------

Basic

Cport::Get gets a character or string of characters from the receive

queue.

Cport::In gets a block of data from the receive queue.

Cport::FlushRx flushes the receive queue and receiver.

Cport::LenRx calculates the number of characters/bytes in the receive

queue.

Intermediate

Cport::Peek returns a copy of the head character/byte without removing

it from the receive queue.

Cport::RxScan scans the receive queue for a specified character.

5-14

Cport::Get

------------------------------------------------------------------

Function    Fetches a character or string of characters from a serial

port's receive queue.

Syntax      #include "cport.h"

char* Cport::Get(char *str, int maxc, char termc = '\n');

or

char Cport::Get();

Parameters

str    - Pointer to the buffer where the string of characters will be

stored.

maxc   - Maximum number of characters to fetch.

termc  - Termination character. Cport::Get will return if this

character is encountered.

Remarks     Cport::Get fetches a character or string of characters from

the serial port instance's receive queue.

If Cport::Get is called with no parameters, it will fetch one

character from the head of the serial port instance's receive

queue. If there are no characters in the receive queue,

Cport::Get returns a NULL character ('\0').

If Cport::Get is called with parameters, it fetches a string

of characters from the serial port instance's receive queue.

The string will be stored in str. Str must be at least

maxc + 1 characters long to allow for the NULL terminator.

Cport::Get will return on the first occurrence of one of

three conditions: 1) the character termc is encountered;

2) maxc characters are reached; or 3) no more characters are

available in the receive queue.

Return value   When Cport:Get is called with no parameters, on success

it returns the character from the receive queue. If it

fails, it returns a NULL character ('\0'). When it is called

with parameters, Cport::Get returns a pointer to the NULL

terminated string.

See also    Cport::In, Cport::LenRx, Cport::Put

5-15

Example     Get a measurement from the specified instrument if one is

ready.

#define MEAS_SIZ   24

char* GetMeasurment(Cport& instrument, char* buf)

{

if(instrument.LenRx() > MEAS_SIZ)

{

instrument.Get(buf, MEAS_SIZ, '\n');

return(buf);

}

else

return(NULL);

}

Example     Get a character from the modem and test if it is a start of

header control character.

#define SOH 1

int CheckForSOH(Cport& modem)

{

char c;

c = modem.Get();

return(c == SOH);

}

Cport::In

------------------------------------------------------------------

Function    Fetches a block of bytes from a serial port's receive queue.

Syntax      #include "cport.h"

unsigned Cport::In(void *abyte, unsigned nbyte);

Parameters

abyte  - Pointer to a buffer where the block of bytes will be stored.

nbyte  - Number of bytes to get from the receive queue.

Remarks     Cport::In fetches a block of bytes from the serial port

instance's receive queue and stores it in abyte. Cport::In

will always fetch num_byte bytes unless not enough bytes are

available. Cport::In is well suited for receiving binary data.

Return value   Cport::In returns the number of bytes actually fetched.

See also    Cport::Out, Cport::Get, Cport::LenRx

5-16

Example     Get a block of 1024 bytes from the serial port.

#include <time.h>

#define BLK_SZ    1024

#define TIMEOUT   -1

int GetBlock(Cport& datalink, byte* buf, clock_t timeout)

{

timeout += clock();

while(datalink.LenRx() < BLK_SZ)

{

if(clock() > timeout)

return(TIMEOUT);

}

return(datalink.In(buf, BLK_SZ));

}

Cport::FlushRx

------------------------------------------------------------------

Function    Flushes a serial port's receiver and receive queue.

Syntax      #include "cport.h"

void Cport::FlushRx();

Remarks     Cport::FlushRx deletes all existing characters/bytes in the

serial port instance's receive queue and receiver.

See also    Cport::LenRx, Cport::FlushTx

Example     If a receive flush command is in order, flush the receiver.

void RemoteCommand(Cport& com, int command)

{

switch(command)

{

case TX_FLUSH:

com.FlushTx();

break;

case RX_FLUSH:

com.FlushRx();

break;

...continued

}

}

5-17

Cport::LenRx

------------------------------------------------------------------

Function    Calculates the number of characters/bytes in the receive

queue of a serial port.

Syntax      #include "cport.h"

unsigned Cport::LenRx();

Remarks     Cport::LenRx calculates the number of characters/bytes

in the receive queue of the serial port instance.

Return value   Cport::LenRx returns the number of characters/bytes

residing in the receive queue.

See also    Cport::FlushRx, Cport::LenTx

Example     Get a measurement from the specified instrument if one is

ready.

#define MEAS_SIZ   24

char* GetMeasurment(Cport& instrument, char* buf)

{

if(instrument.LenRx() > MEAS_SIZ)

{

instrument.Get(buf, MEAS_SIZ, '\n');

return(buf);

}

else

return(NULL);

}

Cport::Peek

------------------------------------------------------------------

Function    Fetches a copy of the head character from a serial port's

receive queue.

Syntax      #include "cport.h"

char Cport::Peek();

Remarks     Cport::Peek fetches a copy of the head character from the

serial port instance's receive queue. Cport::Peek will not

remove the character from the queue.

Return value   Cport::Peek returns the copy of the head character.

See also    Cport::Get, Cport::RxScan

5-18

Example     Get the head character if it is a control character.

#include <ctype.h>

char GetIfControl(Cport& com)

{

char peek;

peek = com.Peek();

if(iscntrl(peek))

return(com.Get());

else

return('\0');

}

Cport::RxScan

------------------------------------------------------------------

Function    Scans the receive queue of a serial port for a specific

character.

Syntax      #include "cport.h"

unsigned Cport::RxScan(char c);

Parameters

c      - Character to scan for.

Remarks     Cport::RxScan searches for the character c in the serial port

instance's receive queue. If the character is not found,

Cport::RxScan returns a zero. Otherwise, it returns the offset

into the receive queue of the first occurrence of c starting

at 1 for the head character. This is the number of characters

that must be fetched to get all data up to and including the

character scanned for.

Return value   Cport::Peek returns the offset into the receive queue to

the first occurrence of c. If c does not exist,

Cport::RxScan returns a zero.

See also    Cport::Peek, Cport::Get

Example     Get the next line terminated with a '\n'.

char *GetLine(Cport& com, char *buffer)

{

int n;

n = com.RxScan('\n');

if(n > 0 && n < 80)

return(com.Get(buffer, n, '\n'));

else

return(NULL);

}

5-19

Output Functions

----------------

Basic

Cport::Put puts a  character or a string of characters to the transmit

queue.

Cport::Out puts a block of data to the transmit queue.

Cport::FlushTx flushes the transmit queue and transmitter.

Cport::LenTx calculates the number of characters/bytes in the transmit

queue.

Advanced

Cport::TxWait pauses program execution until the transmitter has emptied.

5-20

Cport::Put

------------------------------------------------------------------

Function    Puts a character or a string of characters into the serial

port's transmit queue.

Syntax      #include "cport.h"

int Cport::Put(const char *str);

or

int Cport::Put(char c);

Parameters

str    - Pointer to the string to be put into the transmit queue.

c      - Character to be put into the transmit queue.

Remarks     Cport::Put puts a character (c) or a string of characters

(str) into the serial port a instance's transmit queue to be

transmitted. If the transmit queue becomes full, Cport::Put

will not attempt to put any more characters in the queue.

Return value   Cport::Put returns the actual number of characters put

into the transmit queue. For single characters, this value

will be either 0 or 1.

See also   Cport::Out, Cport::Get, Cport::LenTx

Example     Echo the keyboard to the serial port when fetching a

keystroke.

#include <conio.h>

static Cport echo(COM2, B9600, W8|S1|NONE ,1024 , 64);

char GetKey(void)

{

char c;

if(kbhit())

{

c = getch();

echo.Put(c);

return(c);

}

else

return('\0');

}

5-21

Example   Dial a number on the modem.

#include <string.h>

void ModDial(Cport& modem, const char* number)

{

char dial[24] = "ATDT";

strcat(dial, number);

strcat(dial, "\r");

modem.Put(dial);

}

Cport::Out

------------------------------------------------------------------

Function    Puts a block of bytes into a serial port's transmit queue.

Syntax      #include "cport.h"

unsigned Cport::Out(const void *abyte, unsigned nbyte);

Parameters

abyte  - Pointer to the block to be put into the transmit queue.

nbyte  - Number of bytes to be put into the transmit  queue.

Remarks     Cport::Out puts the block of bytes pointed to by abyte into

the serial port instance's transmit queue to be transmitted.

Return value   Cport::Out returns the number of bytes actually put into

the transmit queue.

See also    Cport::In, Cport::Put, Cport::LenTx

Example     Send a block of 128 bytes to the transmitter.

#define BLK_SZ 128

int SendBlock(Cport& com, byte* block)

{

int rv;

rv = com.Out(block, BLK_SZ);

if(rv < BLK_SZ)

return(-1);

else

return(0);

}

5-22

Cport::FlushTx

------------------------------------------------------------------

Function    Flushes a serial port's transmit queue.

Syntax      #include "cport.h"

void Cport::FlushTx();

Remarks     Cport::FlushTx deletes any existing characters in the serial

port instance's transmit queue.

See also    Cport::FlushRx, Cport::LenTx

Example     If a transmit flush command is in order, flush the transmit

queue.

void RemoteCommand(Cport& com, int command)

{

switch(command)

{

case TX_FLUSH:

com.FlushTx();

break;

case RX_FLUSH:

com.FlushRx();

break;

...continued

}

}

Cport::LenTx

------------------------------------------------------------------

Function    Calculates the number of characters/bytes in a serial

port's transmit queue.

Syntax      #include "cport.h"

unsigned Cport::LenTx();

Remarks     Cport::LenTx calculates the number of characters/bytes

in the serial port instance's transmit queue.

Return value   Cport::LenTx returns the number of characters/bytes

in the transmit queue.

See also    Cport::LenRx, Cport::FlushTx

Example     Check if the transmitter has reached 3/4 of capacity.

#define TX_SIZE      1024

int IsAboveThresh(Cport& sprinter)

{

return(sprinter.LenTx() > (TX_SIZE * 3 / 4));

}

5-23

Cport::TxWait

------------------------------------------------------------------

Function    Waits for a serial port's transmitter to empty

Syntax      #include "cport.h"

void Cport::TxWait();

Remarks     When a transmit queue becomes empty, the UART may not have

actually transmitted the last of the bytes. A program may need

to know when all the data has actually been transmitted so

that it may, for example, change the state of the hardware

lines. Cport::TxWait will return after the UART has been

emptied of data.

See Also    Cport::LenTx

Example     Send a string to a half duplex device. This examples assumes

that the device is monitoring our RTS line to determine a

transmit or receive state.

void SendMessage(Cport device, const char* msg)

{

device.Rts(ON);

device.Put(msg);

while(device.LenTx());

device.TxWait();

device.Rts(OFF);

}

5-24

Status Functions

----------------

Basic

Cport::Error detects hardware and software errors.

Intermediate

Cport::Status returns the status of the software and input hardware lines.

Cport::Rts  controls the RTS output line.

Cport::Dtr controls the DTR output line.

Advanced

Cport::Out1 controls the general purpose OUT1 output line.

Cport::Uart determines what type of UART chip is residing at a serial port.

5-25

Cport::Error

------------------------------------------------------------------

Function    Determines if any errors have occurred on a serial port since

the last call to Cport::Error.

Syntax      #include "cport.h"

unsigned Cport::Error();

Remarks     Cport::Error determines if any errors are pending on the

serial port instance. After a call to Cport::Error, the errors

are cleared. Cport::Error returns a bit ORed error word. See

Table A-7 on Appendix A for a list of bit definitions.

Return value   If an error has occurred, Cport::Error returns an error

word. If no errors have occurred, it returns a 0.

See also    Cport::Status

Example     Determine the source of the error if one has occurred.

#define HARDWARE   (PARITY | FRAMING | BREAK | RX_FIFO)

void CheckForErrors(Cport& dvm)

{

unsigned err;

if((err = dvm.Error()) == 0)

return;

else if(err & OVERUN)

{

// over run error

}

else if(err & HARDWARE)

{

// hardware errors

}

else if(err & (TXFULL | RXFULL))

{

// queue errors

}

}

Cport::Status

------------------------------------------------------------------

Function    Retrieves the status of a serial port's software and hardware

lines.

Syntax      #include "cport.h"

unsigned Cport::Status();

5-26

Remarks     Cport::Status retrieves the status of the serial port

instance's software and hardware lines. Cport::Status returns

a bit ORed status word. See Table A-8 in Appendix A for a list

of bit definitions.

Return value   Cport::Status returns the status word of the software and

hardware lines.

See also    Cport::Error, Cport::Rts, Cport::Dtr

Example     Test if the modem is powered up and ready.

#include <dos.h>

int CheckModem(Cport& modem)

{

modem.Dtr(ON);

delay(40);

return((modem.Status() & DSR) != 0);

}

Cport::Rts

------------------------------------------------------------------

Function    Sets or clears a serial port's Request to Send line.

Syntax      #include "cport.h"

void Cport::Rts(byte on_off);

Parameters

on_off - Determines whether RTS is to be set or cleared. See Table A-10

in Appendix A.

Remarks     Cport::Rts sets or clears the serial port instance's RTS line,

depending on the value of on_off. If the value of on_off is

ON, RTS is asserted. If the value of on_off is OFF, RTS is

inhibited. In many cases, this signal will be looked at by the

remote device to determine if it is safe to transmit. This

gives the programmer the ability to turn off the remote

device's transmitter.

NOTE: Cport::Rts should never be called when RTS hardware

handshaking is enabled.

See also    Cport::Dtr, Cport::Status, Cport::Handshake

5-27

Example     Put the device in an on or off line state. (Assumes the remote

device will not send data without our RTS asserted)

void OffLine(Cport& device)

{

device.Rts(OFF);

}

void OnLine(Cport& device)

{

device.Rts(ON);

}

Cport::Dtr

------------------------------------------------------------------

Function    Sets or clears a serial port's Data Terminal Ready line.

Syntax      #include "cport.h"

void Cport::Dtr(byte on_off);

Parameters

on_off - Determines whether DTR is to be asserted or inhibited. See

Table A-10 in Appendix A.

Remarks  Cport::Dtr set or clears the serial port instance's DTR line,

depending on the value of on_off. If the value of on_off is

ON, DTR is asserted. If the value of on_off is OFF, DTR is

inhibited. In many cases, this signal will be looked at by the

remote device to determine if it is safe to transmit. This

gives the programmer the ability to turn off the remote

device's transmitter.

NOTE: Cport::Dtr should never be called when DTR hardware

handshaking is enabled.

See also    Cport::Rts, Cport::Status, Cport::Handshake

Example     Put the device in an on or off line state. (Assumes the remote

device will not send data without our DTR asserted).

void OffLine(Cport& device)

{

device.Dtr(OFF);

}

void OnLine(Cport& device)

{

device.Dtr(ON);

}

5-28

Cport::Out1

------------------------------------------------------------------

Function    Sets or clears a serial port's Out1 line.

Syntax      #include "cport.h"

void Cport::Out1(byte on_off);

Parameters

on_off - Determines whether OUT1 is to be asserted or inhibited. See

Table A-10 in Appendix A.

Remarks     Cport::Out1 sets or clears the serial port instance's general

purpose OUT1 line depending on the value of on_off. If the

value of on_off is ON, OUT1 is asserted. If the value of

on_off is OFF, OUT1 is inhibited. OUT1 is a general purpose

output sometimes utilized by a device for a special feature or

reset.

See also    Cport::Rts, Cport::Dtr

Example     Reset a smart modem.

void SmartReset(Cport& modem)

{

modem.Out1(ON);

delay(50);

modem.Out1(OFF);

}

Cport::Uart

------------------------------------------------------------------

Function    Determines the type of UART at a serial port.

Syntax      #include "cport.h"

int Cport::Uart();

Remarks     Cport::Uart determines the type of UART chip that resides

at the instance's serial port. See Table A-11 for a list of

UART types.

Return value   Cport::Uart returns a value representative of the type of

UART that was detected.

See Also    Cport::NS550

Example     If the serial port instance has an NS16550 UART, set the

trigger level to 1 byte.

void SetThreshIf550(Cport& com)

{

if(com.Uart() == NS16550)

com.NS550(T550_1);

}

5-29

Data Integrity

--------------

Intermediate

Cport::Checksum calculates a byte checksum of a block of data.

Cport::Crc16 calculates a 16 bit cyclic redundancy check (CRC) on a

block of data.

Cport::Crc32.calculates a 32 bit cyclic redundancy check (CRC) on a

block of data.

5-30

Cport::Checksum

------------------------------------------------------------------

Function    Performs a byte checksum calculation on a block of data.

Syntax      #include "cport.h"

static byte Cport::Checksum(const void *abyte, unsigned nbyte);

Parameters

abyte  - Pointer to the block on which the checksum is to be performed.

nbyte  - Number of bytes in the block.

Remarks     Cport::Checksum performs a byte checksum on the block of bytes

pointed to by abyte. The checksum is calculated by summing all

the bytes in the block.

Return value   Cport::Checksum returns a one byte checksum.

See also    Cport::Crc16, Cport::Crc32, Cport::In, Cport::Out

Example     Verify that the actual checksum matches the received checksum.

struct Xblock

{

byte num;

byte _num;

byte data[128];

byte checksum;

}block;

int GetXblock(Cport& com, Xblock& block)

{

com.In(&block, sizeof(Xblock));

if((com.Checksum(block.data, 128) - block.checksum) != 0)

return(-1);

else

return(0);

}

Cport::Crc16

------------------------------------------------------------------

Function    Performs a 16 bit cyclic redundancy check on a block of data.

Syntax      #include "cport.h"

static unsigned ComCrc16(const void *abyte, unsigned nbyte);

Parameters

abyte  - Pointer to the block on which the 16 bit cyclic redundancy

check is to be performed.

5-31

nbyte  - Number of bytes in the block.

Remarks     Cport::Crc16 performs a 16 bit cyclic redundancy check on the

block of data pointed to by abyte.

Return value   Cport::Crc16 returns the 16 bit CRC value.

See also    Cport::Crc32, Cport::Checksum, Cport::In, Cport::Out

Example     Verify that the actual CRC matches the received CRC.

struct Xblock{

byte num;

byte _num;

byte data[128];

unsigned crc;

}block;

int GetXblock(Cport& com, Xblock& block)

{

com.In(&block, sizeof(Xblock));

// Check the calculated CRC against the block CRC.

//

if(com.Crc16(block.data, 128) != block.crc)

return(-1);

else

return(0);

}

Cport::Crc32

------------------------------------------------------------------

Function    Performs a 32 bit cyclic redundancy check on a block of data.

Syntax      #include "cport.h"

static unsigned long ComCrc32(const void *abyte, unsigned nbyte);

Parameters

abyte  - Pointer to the block on which the 32 bit cyclic redundancy

check is to be performed.

nbyte  - Number of bytes in the block.

Remarks     Cport::Crc32 performs a 32 bit cyclic redundancy check on the

block of data pointed to by abyte.

Return value   Cport::Crc32 returns the 32 bit CRC value.

See also    Cport::Crc16, Cport::Checksum, Cport::In, Cport::Out

5-32

Example     Verify that the actual CRC matches the received CRC.

struct Packet {

byte num;

byte data[1024];

unsigned long crc;

}packet;

int GetPacket(Cport& com, Packet& packet)

{

com.In(&packet, sizeof(Packet));

// Check the calculated CRC against the packet's CRC.

//

if(com.Crc32(packet.data, 1024) != packet.crc)

return(-1);

else

return(0);

}

5-33

Misc Functions

--------------

Advanced

Cport::SetBreak sets a break condition.

Cport::ClrBreak clears a break condition.

Cport::Scratch writes to or reads from the UART's scratch register.

5-34

Cport::SetBreak

------------------------------------------------------------------

Function    Set a break condition on a serial port.

Syntax      #include "cport.h"

void Cport::SetBreak();

Remarks     Cport::SetBreak sets a break condition on the serial port

instance. The break condition will stay in effect until a

subsequent call to Cport::ClrBreak is made.

See also    Cport::ClrBreak

Example     If the an abort condition has been declared, set a break

condition. When it clears, release the break condition.

void Abort(Cport& com, int abrt)

{

static int brkflg;

if(abrt && !brkflg)

com.SetBreak();

else if(!abrt && brkflg)

com.ClrBreak();

}

Cport::ClrBreak

------------------------------------------------------------------

Function    Clears a previously set break condition on a serial port.

Syntax      #include "cport.h"

void Cport::ClrBreak();

Remarks     Cport::ClrBreak clears a previously set break condition on the

serial port instance.

See also    Cport::SetBreak

Example     If the an abort condition has been declared, set a break

condition. When it clears, release the break condition.

void Abort(Cport& com, int abrt)

{

static int brkflg;

if(abrt && !brkflg)

com.SetBreak();

else if(!abrt && brkflg)

com.ClrBreak();

}

5-35

Cport::Scratch

------------------------------------------------------------------

Function    Writes a byte to, or reads a byte from the serial port's

scratch register.

Syntax      #include "cport.h"

void Cport::Scratch(byte abyte);

or

byte Cport::Scratch();

Parameters

abyte  - The byte to be written to the scratch register.

Remarks     If a parameter is specified, Cport::Scratch writes the byte

value abyte to the serial port instance's UART scratch

register. If Cport::Scratch is called with no parameters, it

reads the byte in the UART scratch register. Not all UARTs

have a scratch register.

Example     Exchange a value with the scratch register.

byte ScratchExchange(Cport& com, byte value)

{

byte rv;

rv = com.Scratch();      // read the scratch register

com.Scratch(value);   // write to the scratch register

return(rv);

}

6-1

File Transfer

=============

Xmodem

------

XmodemTx transmits a file using the Xmodem file transfer protocol.

XmodemRx receives a file using the Xmodem file transfer protocol.

Xcallback is the default callback function for XmodemTx and XmodemRx.

6-2

XmodemTx

------------------------------------------------------------------

Function    Transmits a file with the Xmodem file transfer protocol.

Syntax      #include "xmodem.h"

int XmodemTx(COM, const char* file,

int (*cb)(int msg, XPARAM param));

or

int XmodemTx(Cport&, const char* file,

int (*cb)(int msg, XPARAM param));

Parameters

COM    - Handle of the serial port (C).

Cport  - Reference to a serial port instance (C++).

file   - Path/name of the file to be transferred.

cb     - Pointer to the callback function.

Remarks     XmodemTx transmits the file file via the serial port specified

by COM or Cport. It will make calls back to the function cb if

one is specified. If you specify NULL for cb, a default

callback function will be used. See the following discussion

on the callback function for a complete explanation of the

callback function.

Return value   XmodemTx returns zero if the file was transferred

successfully. Otherwise, it returns non-zero. See Table

6-1 for a list of return codes.

See also    XmodemRx, Xcallback

Example     Transfer the file cport.zip via the modem. (C)

/* callback function prototype */

int myCallback(int msg, XPARAM param);

int main(void)

{

COM modem;

int rv;

modem = ComOpen(COM1, B2400, W8|S1|NONE, 1024, 512);

if(comopen_errno != NO_ERR)

return(-comopen_errno);

rv = XmodemTx(modem, "cport.zip", myCallback);

ComClose(modem);

return(rv);

}

6-3

Example     Transfer the file cport.zip via the modem. (C++)

// callback function prototype

int myCallback(int msg, XPARAM param);

int main(void)

{

Cport modem(COM1, B2400);  // uses default parameters

if((int)modem != NO_ERR)

return(-((int)modem));

return(XmodemTx(modem, "cport.zip", myCallback));

}

XmodemRx

------------------------------------------------------------------

Function    Receives a file with the Xmodem file transfer protocol.

Syntax      #include "xmodem.h"

int XmodemRx(COM, const char* file,

int (*cb)(int msg, XPARAM param));

or

int XmodemRx(Cport&, const char* file,

int (*cb)(int msg, XPARAM param));

Parameters

COM    - Handle of the serial port (C).

Cport  - Reference to a serial port instance (C++).

file   - Path/name of the file to be transferred.

cb     - Pointer to the callback function.

Remarks     XmodemRx receives the file file via the serial port specified

by COM or Cport. It will make calls back to the function cb

if one is specified. If you specify NULL for cb, a default

callback function will be used. See the following discussion

on the callback function for a complete explanation of the

callback function.

Return value   XmodemTx returns zero if the file was transferred

successfully. Otherwise, it returns non-zero. See Table

6-1 for a list of return codes.

See also    XmodemTx, Xcallback

6-4

Example     Receive the file cport.zip via the modem. (C)

/* callback function prototype */

int myCallback(int msg, XPARAM param);

int main(void)

{

COM modem;

int rv;

modem = ComOpen(COM1, B2400, W8|S1|NONE, 1024, 512);

if(comopen_errno != NO_ERR)

return(-comopen_errno);

rv = XmodemRx(modem, "cport.zip", myCallback);

ComClose(modem);

return(rv);

}

Example     Receive the file cport.zip via the modem. (C++)

// callback function prototype

int myCallback(int msg, XPARAM param);

int main(void)

{

Cport modem(COM1, B2400);  // uses default parameters

if((int)modem != NO_ERR)

return(-((int)modem));

return(XmodemRx(modem, "cport.zip", myCallback));

}

Xcallback

------------------------------------------------------------------

Function    Default callback function.

Syntax      #include "xmodem.h"

int Xcallback(int msg, XPARAM param);

Parameters

msg    - Callback message number. (See Table 6-2).

param  - Message specific parameter. (See Table 6-2 and 6-3)

Remarks     Xcallback is the default Xmodem callback function. When NULL

is specified as the callback function in a call to XmodemTx

or XmodemRx, Xcallback will be used. Xcallback only keeps

track of errors to decide if the transfer should be aborted.

You can chain callback functions by specifying a callback

function and then calling Xcallback directly. You would do

this if you were only interested in the information provided

6-5

to the callback function and did not want to affect the

functionality of the transfer.

Return value   When a callback function returns zero, the transfer

continues. If it returns non-zero, the transfer is

canceled, and XmodemRx or XmodemTx will return with a

non-zero return code.

See also    XmodemTx, XmodemRx

Example     Use the default callback when receiving the file cport.zip

via the modem. This results in a blind transfer.

int main(void)

{

COM modem;

int rv;

modem = ComOpen(COM1, B2400, W8|S1|NONE, 1024, 512);

if(comopen_errno != NO_ERR)

return(-comopen_errno);

rv = XmodemRx(modem, "cport.zip", NULL);

ComClose(modem);

return(rv);

}

Example     Establish a callback function for procuring information for

screen updates only.

int myCallback(int msg, XPARAM param)

{

switch(msg)

{

case XM_BLOCKCHECK:

/* Update the screen */

break;

case XM_BLOCK:

/* Update the screen */

break;

case...

}

/* chain to the default callback for functionality

*/

return(Xcallback(msg, param));

}

Callback Function

------------------------------------------------------------------

Since it usually takes time to transfer a file, there must be a mechanism

for monitoring and optionally canceling the transfer. To accomplish this,

the Xmodem transmit and receive functions accept a parameter, which is a

pointer  to a callback function. This function will be called at certain

6-6

points during the file transfer. Your callback function can be used to

update the screen, cancel the transfer, etc. Your callback function must

have the exact prototype:

int function_name(int msg, XPARAM param);

Msg is the message number that identifies why the callback was made.

The value of param depends on the message. It provides additional

information for a specific message. Anytime param is not used, it is set

to zero. The following is a list of messages and their descriptions.

XM_IDLE (0) - Idle time. This message is sent during idle time to give

the callback the opportunity to do something, such as checking to see

if the user wishes to cancel the transfer.

XM_START (1) - Start of transfer. This message is sent, at most, one time.

It signals that the transfer has been successfully prepared and will

now attempt to make contact with the remote station.

XM_BLOCKCHECK (2) - Block check determined. Once the block check has

been determined, this message is sent to inform the callback if the

block check will be checksum (param == 0) or CRC16 (param != 0). This

message is sent, at most, one time during a transfer. It is mostly

informative .

XM_BLOCK (3) - New block transferred. This message is sent each time a new

block (128 bytes) is successfully transferred. For this message, param

contains the block number. This message is primarily an informative

message.

XM_EOT (4) - End of transmission. This message is sent when an end of

transmission is being established. Usually there will be only one of

these messages, however, if problems arise, the EOT may be retried.

XM_DONE (5) - Transfer done. This message is sent at the end of a

transmission whether the transmission was successful or canceled. It

is sent exactly one time if XM_START was previously sent. Otherwise,

it will not be sent.

XM_ERROR (6) - Error detected. This message is sent when an error is

detected in the data. Param contains the error code. The lower 12 bits

of param is the bit ORed errors returned by the function ComError.

The upper 4 bits is the extended Xmodem error code (not bit ORed).

The extended Xmodem error code (upper 4 bits) should always be

non-zero. The ComError bits (lower 12 bits) may or may not indicate

errors. Usually the ComError portion of the error code takes precedence

over the extended Xmodem code, since it is likely to be the source of

the error. The extended Xmodem errors are explained here. See ComError

or Cport::Error for information on the lower 12 bits of the error code:

XE_BADBLOCK (0x1000) - Bad block. This error occurs when a block was

determined to be corrupted. Occasional bad block errors may occur. If

the transfer is not canceled by the callback function, Xmodem will

attempt to recover from this error (usually succeeding) by retrying

the block.

6-7

XE_BADCHECK (0x2000) - Bad block check. This error occurs when the block

is not detected as bad (XE_BADBLOCK) but the block check (CRC or

checksum) does not match the data. This error can only be detected

when receiving a file.

XE_TIMEOUT (0x3000) - Timeout. Xmodem timed out waiting for a block or

for protocol exchange. If the callback function does not cancel the

transfer, Xmodem will retry whatever timed out. Successful recovery

from this error depends on the reason (usually unknown) for the timeout.

XE_CANCEL (0x4000) - Transmission canceled. This message is sent when

the transmission is canceled by either the callback function, by the

remote station or for other internal reasons.

Xmodem Tables of Values

------------------------------------------------------------------

name        value       description

-----------------------------------------------

XR_NOERR    (0)         No error.

XR_FILEERR  (1)         File error.

XR_CANCEL   (2)         Transfer canceled.

XR_ALLOC    (3)         Memory allocation error.

Table 6-1 - XmodemRx/XmodemTx return values.

name           value    description             XPARAM

-------------------------------------------------------------------------

XM_IDLE        (0)      Idle time.              0.

XM_START       (1)      Start of transfer       0.

XM_BLOCKCHECK  (2)      Block check determined  0 if checksum, 1 if CRC16.

XM_BLOCK       (3)      New block transferred   Block number.

XM_EOT         (4)      End of transmission     0.

XM_DONE        (5)      Transfer complete       0.

XM_ERROR       (6)      Error has been detected Error code.

Table 6-2 - Xmodem callback messages.

name           value       description

------------------------------------------------------------

XE_BADBLOCK    (0x1000)    Bad block.

XE_BADCHECK    (0x2000)    Bad block check (CRC or checksum).

XE_TIMEOUT     (0x3000)    Timeout.

XE_CANCEL      (0x4000)    Transfer canceled.

Table 6-3 - Extended Xmodem error codes.

A-1

Appendix A

==========

name     value

-----------------------

* COM1   (PORT0 | IRQ4)

* COM2   (PORT1 | IRQ3)

* COM3   (PORT2 | IRQ4)

* COM4   (PORT3 | IRQ3)

PORT0    (0x03F8)

PORT1    (0x02F8)

PORT2    (0x03E8)

PORT3    (0x02E8)

IRQ2     (0x2000)

IRQ3     (0x3000)

IRQ4     (0x4000)

IRQ5     (0x5000)

IRQ6     (0x6000)

IRQ7     (0x7000)

BIOS0    (*(int far *)0x400000l)

BIOS1    (*(int far *)0x400002l)

BIOS2    (*(int far *)0x400004l)

BIOS3    (*(int far *)0x400006l)

* The COMx constants are usually sufficient.

Table A-1 - Serial port id.

name     value

---------------

B115200  (1)

B57600   (2)

B38400   (3)

B19200   (6)

B9600    (12)

B7200    (16)

B4800    (24)

B3600    (32)

B2400    (48)

B2000    (58)

B1800    (64)

B1200    (96)

B600     (192)

B300     (384)

B150     (768)

B110     (1047)

B75      (1536)

B50      (2304)

Table A-2 - Baud rate divisors.

A-2

name     value    description

----------------------------------------

W8       (0x03)   8 bit words.

W7       (0x02)   7 bit words.

W6       (0x01)   6 bit words.

W5       (0x00)   5 bit words.

S1       (0x00)   1 stop bit.

S2       (0x04)   2 stop bits.

NONE     (0x00)   No parity.

ODD      (0x08)   Odd parity.

EVEN     (0x18)   Even parity.

MARK     (0x28)   Mark parity (logic 1).

SPACE    (0x38)   Space parity (logic 0).

WMASK    (0x03)   Word mask.

SMASK    (0x04)   Stop bit mask.

PMASK    (0x38)   Parity mask.

Table A-3 - Bit ORed mode components.

name     value    description

---------------------------------------------------------------

NO_ERR   (0)      No errors.

OPENED   (1)      Serial port is already opened.

BAD_ID   (2)      Invalid id parameter.

NO_UART  (3)      No UART chip was detected.

RX_ALC   (4)      Receive queue allocation error.

TX_ALC   (5)      Transmit queue allocation error.

MAX_PORT (6)      The maximum number of serial ports are opened.

IRQ_CTN  (7)      The irq contends with another open serial port.

GEN_ALC  (8)      General memory allocation error.

Table A-4 - Open error values.

name     value    description

---------------------------------------------------

OFF      (0x00)   No handshaking.

DTR      (0x01)   The receiver uses DTR.

RTS      (0x02)   The receiver uses RTS.

S_RX     (0x04)   The receiver uses XON/XOFF.

CTS      (0x10)   CTS is checked by the transmitter.

DSR      (0x20)   DSR is checked by the transmitter.

DCD      (0x80)   DCD is checked by the transmitter.

S_TX     (0x08)   The transmitter honors XON/XOFF.

SOFT     (0x0C)   Same as  (S_RX | S_TX).

HARD1    (0x21)   Same as  (DTR | DSR).

HARD2    (0x12)   Same as  (RTS | CTS).

Table A-5 - Bit ORed handshaking options.

A-3

name     value    description

-----------------------------------

TIMER    (0x01)   divert timer (irq0).

KEYBOARD (0x02)   disable keyboard (irq1).

CASCADE  (0x04)   disable irq2.

OFF      (0x00)   restore all interrupts.

Table A-6 - Turbo options.

name     value    description

-----------------------------------------

OVERUN   (0x0002) Overrun error.

PARITY   (0x0004) Parity error.

FRAMING  (0x0008) Framing error.

BREAK    (0x0010) Break detect.

RX_FIFO  (0x0080) Rx fifo error.

TXFULL   (0x0100) Transmit queue overflow.

RXFULL   (0x0200) Receive queue overflow.

(unused) (0xF000) Designated as unused.

Table A-7 - Bit ORed error code.

name     value    description

-----------------------------------------------

DCTS     (0x001)  Change in clear to send.

DDSR     (0x002)  Change in data set ready.

TERI     (0x004)  Trailing edge ring indicator.

DDCD     (0x008)  Change in data carrier detect.

CTS      (0x010)  Clear to send asserted.

DSR      (0x020)  Data set ready asserted.

RI       (0x040)  Ring indicator asserted.

DCD      (0x080)  Data carrier detect asserted.

S_TX_OFF (0x100)  Transmitter off (XOFF state).

U_TX_OFF (0x200)  Transmitter off (ComTx).

S_RX_OFF (0x400)  Receiver off (XOFF state).

Table A-8 - Bit ORed status word.

name        value    description

----------------------------------------------

ON          (1)      Turns the transmitter on.

OFF         (0)      Turns the transmitter off.

TX_DIRECT   (0x2)    Use direct mode when off.

Table A-9 - Transmitter options.

A-4

name        value    description

-------------------------------------

ON          (1)      Signal asserted.

OFF         (0)      Signal inhibited.

Table A-10 - On/off values.

name     value    description

--------------------------------------

INS8250  (1)      INS8250 or INS8250-B.

NS16450  (2)      NS16450 or INS8250A.

NS16550  (3)      NS16550.

I82510   (4)      82510 (Intel).

Table A-11 - UART types.

name     value    description

--------------------------------

T550_1   (0x00)   1 byte.

T550_4   (0x400   4 bytes.

T550_8   (0x80)   8 bytes.

T550_14  (0xC0)   14 bytes.

Table A-12- NS550 trigger thresholds.

B-1

Appendix B

==========

Compiling

---------

1) Make sure the header file(s) are included in any modules (source

files) which make calls to Cport library functions.

Example: #include "cport.h"

2) Make sure your compiler knows where to find the header file(s).

Borland C/C++: bcc -c -Ic:\cport\include your_file

Microsoft C/C++: cl /c /Ic:\cport\include your_file

NOTE: cl will also look at the INCLUDE environment variable for

the include directories.

Example: set INCLUDE=c:\c600\include;c:\cport\include;

3) Make sure your compiler is generating byte alignments when generating

structures. This is only a concern when using Cport functions, which

involve structures such as CPARAM. For Borland and Microsoft

compilers, this is taken care of. If you are using another compiler,

consult your user's manual on how to generate byte alignments.

Borland C/C++: (uses byte alignment by default)

Microsoft C/C++: cl /c /Zp1 /Ic:\cport\include your_file

Linking

-------

1) Make sure the appropriate Cport library is included in the list of

libraries. The last letter in the file name of the libraries

indicates the memory model:

cports.lib - small model

cportm.lib - medium model

cportc.lib - compact model

cportl.lib - large model

cporth.lib - huge model

2) Make sure the linker knows where to find the library files.

Borland C/C++: tlink /Lc:\cport\lib c0s objs, exe_name, map_name, cports cs

Microsoft C/C++: link  objs, exe_name, map_name, cports slibce;

NOTE: link looks at the LIB environment variable for the library

directories:

Example: set LIB=c:\c600\lib;c:\cport\lib;

C-1

Appendix C

==========

Cport v2.0 Order Form

Name: ___________________________________________________________

Company: ________________________________________________________

Address: ________________________________________________________

City: _________________________________  State: _________________

Zip Code: _________________  Phone # : __________________________

CompuServe # : __________________________________________________

How did you acquire Cport ? _____________________________________

Compiler(s): ____________________________________________________

Computer(s): ____________________________________________________

Operating system(s): ____________________________________________

Single Licence .... $65.00

2 Licenses ........ $59.00 each

3 to 4 Licenses ... $54.00 each

5 to 9 licenses ... $48.00 each

10+ licenses ...... $41.00 each

Quantity          price each

Subtotal      _________    X    __________   =    $________

California residents add sales tax    ( 8.25% )   $________

Shipping outside continental U.S. add ( $ 5.00)   $________

Total:   $________

All Payments Must be in U.S. Dollars

Make check or money order payable to:    Bri Productions

P.O. Box 7121

Fremont, CA 94537-7121

Visa/Master Card#:___________________  Expiration Date:_________

Signature (Credit card only):___________________________________

C-2

We want to hear from you!

-------------------------

In order to improve Cport and add valuable features, we need to know what

features you need most. This information is important in order to provide

you with the best possible product. Please take a moment to answer the

following questions.

Indicate your need for the following features by checking the choice

which best describes your need for the feature:

very    |  nice   |  don't

important | to have | need it

-----------  -------- ---------

Multi port Serial Boards:   ____      ____      ____

Board name(s) ______________

Modem support:   ____      ____      ____

Zmodem file transfer:   ____      ____      ____

Other file transfer __________:   ____      ____      ____

Other file transfer __________:   ____      ____      ____

Windows support:   ____      ____      ____

Other ________________________:   ____      ____      ____

Other ________________________:   ____      ____      ____

Other ________________________:   ____      ____      ____

What other types of libraries would you find useful ? _____

___Game Port   ___Expanded Memory   ___Sound effects

Others______________________________________________________

Other Comments______________________________________________

____________________________________________________________

____________________________________________________________

Bri Productions, P.O. Box 7121, Fremont, CA  94537, (510) 794-0616

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 4,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705

Houston, TX 77235-5705

Orders only:

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394