MAPX.WK1, the Worksheet v1.3

A tool to find latitude and longitude

from a known reference on a map.  A

handy GPS companion.

Copyright 1994, 1995, John Ceccherelli

************************  WARNING  **************************

* Be advised that MAPX is essentially a linear interpolator *

* and should not be used for spans greater than 2 degrees   *

* of latitude or longitute.                                 *

* This program has been tested to the best of my ability,   *

* but ...  If you plan to risk life, limb, or property on   *

* this program, it's at YOUR OWN RISK. You are responsible  *

* for its application and consequences.  The author is not  *

* liable for damages arising from the use of this product.  *

************************  WARNING  **************************

Quid Pro Quo:

You get what you pay for and MAPX is free for personal use!

Commercial use is prohibited without written consent from the

author.  So feel free to copy, distribute, post, or purge as

you see fit.  Just make sure that this document accompanies

the worksheet.

Introduction:

I've spent many late nights trying to figure out latitude and

longitude of points on USGS Quadrangles.  It's a chore.  I've

spent many dollars for rulers, overlays, and similar contraptions

that never work all that well.  Conversely, I've been hiking

deep in the backwoods with $500 of GPS satellite technology and

have yet to find an easy way to translate positional coordinates

back to the map.  This is why MAPX was written.  With a simple

(and cheap) metric ruler, MAPX solves both problems.

MAPX was developed to solve these problems with USGS 7.5'

quadrangles.  It works well with scales down to 1/500,000 used

for Sectional Aeronautical Charts.  Smaller than this and the

results get dicey.  By the way, the bigger the denominator,

the smaller the scale of the map.  For example, a USGS 7.5'

quad has a scale of 1/24,000.  This is a larger number than

1/10,000,000 - a scale you might find on a wall map of the USA.

Remember: large scale-great detail, small scale-no detail.

Rules:

A few rules apply here.  First, North and West are positive (+),

East and South are negative (-).  I live and travel north

of the Equator and west of Greenwich so that's the way it is.

(Ceccherelli's Convention).  This applies to latitude, longitude

and the distances you will measure with your metric ruler.

Notice I said METRIC ruler, no English distance units allowed

because I'm a serious engineer (translation - the math is easier).

Measurements from the metric ruler are entered as millimeters.

Do not use MAPX for spans greater than 2 degrees from the

reference, map scale errors will begin to be noticeable.  MAPX

does not handle polar regions greater than +/- 88 degrees latitude.

A Real Life Example:

I recently went camping in the Catskills with some buddies of

mine.  Due to scheduling constraints and minor commitments

(like, my job), my friend Tom and I wouldn't be able to hit the

trailhead until dark.  It would be a 3 mile hike, just me, Tom,

and Coleman the lantern.  With a little luck and some expert

navigation, we would meet the others on the south shore of Tunis

Pond.  There was no room for error in this mission.  Rumor had

it that beverages made from fermented grains were being served

at the camp site.  The supply was sure to dwindle rapidly with

each passing minute.  Time was critical, we had to execute

flawlessly.

There's a hitch though.  Tunis Pond is not on the trail.  It's

a 300 meter bushwack north from a non-descript portion of the

trail.  Fortunately, I'm equipped with a GPS receiver.  All we

have to do is enter the lat and long of the point on the trail

(waypoint) where we depart due north.  Dead reckoning 300 meters

to the south shore of Tunis Pond should get us within shouting

distance of the others.

Tunis Pond is on Map 43 published by the New York-New Jersey

Trail Conference.  These maps are OK except they have precious

little positional information on them.  If you're lucky, there

will be one latitude and longitude intersection labeled.  I'm

lucky and this point is designated "the reference".

Here's where MAPX comes in (finally).  First we have to enter

the scale of the map.  This map is 1:63,360.

So enter 63360 in cell B11

The reference I'm using is 42 30' 00" west longitude. Enter the lat in B16, ' in C16 and " in D16.

Now get out the metric ruler and measure the vertical distance

from 4200'N

74    '     "                 Ellipsoid

14     REF Latitude      42     0   0.0             a:  6E+06

15    REF Longitude      74    30   0.0           e^2: 0.0068

16   -----------------------------------------------------------------

17   To find the Lat/Long of a point on the map (target), enter the

18   X,Y distance to the target from the reference here:

19   Distance, Ref to Target:

20      VERTICAL mm    27.5

21    HORIZONTAL mm    80.5

22

23                      Deg   Min   Sec    Deg    Min

24          Target:             '

25         Latitude      42     0  56.5    42     0.94

26        longitude      74    33  41.7    74    33.69

27   -----------------------------------------------------------------

The waypoint latitude and longitude appear in row 25 and 26.

Cool eh?  I plugged that data into my GPS receiver.

We're hiking along wondering how much progress we've made

toward that waypoint.  So I whip out the map and the GPS

receiver.  A couple of minutes later, GPS tells me we're at

4133'32"W long.  Great!?  Where's that on the

map?  No big deal with MAPX, just enter 4133'32"

on lines 34 and 35.  Cells B36 and B37 give you the Y and X

distance from the reference to the point on the map (where I

am now).

ref

x---

I'm here *                   ---  -0.5mm

|------ 77mm ------|

I pencil in that point and see there should be a spring on

my left... aaahhh, there it is.  Several gulps later, we continue

on the trail knowing we're more than half way there.

Well, we hit the waypoint with the aid of the GPS receiver.

Switched over to geomagnetic mode (used a compass, GPS sucks

for short distances), and dead reckoned due north 300 meters.

We ended up at the south shore of Tunis Pond about 50 meters west

of our friends!  The last drop of the aforementioned beverage had

been consumed about 30 minutes prior to our arrival :-(.

Polar Coordinates (Range/Bearing):

The polar (azimuth and distance) coordinates from the reference

to the position are also available in cells D36 and D37.  The

azimuth is referenced to true north so north = 0,

south = 180 (0 are both north).

Chuck Hawley of West Marine, has described a very useful method

to find your position on the map.  His "range/bearing" method

is simple and all it requires is a protractor or plotter.

Using a protractor with the example above, the position is 269.6

degrees and 77 millimeters from the reference.  Some people find

this measurement system easier than the X,Y rectangular system.

Enter the map corner (or other convenient) latitude and longitude

as a waypoint.  You then use the GPS range and bearing information

to plot your position.  I find it is easiest to plot from the

reference so you need to calculate the back azimuth (bearing)

by adding or subtracting 180 degrees which ever is appropriate.

Chuck suggests jotting down the distance conversion factor

in the map margin.  The math is easy to do by hand and you

don't need to bring a palmtop computer or similar device with

you.  I find this method beautifully simple and elegant.

If you are not comfortable with calculating in your head, I've

included the Range/Bearing method in MAPX.

The bearing from the reference to the waypoint is automatically

displayed in cell D36.  The range in millimeters is in cell D37

and the true ground distance is in cell D38.

If you want to determine the range and bearing from the

reference given the range and bearing to the reference,

use cells B45 and B46 for input.  The back azimuth from the

reference is displayed in cells E45 and E46.  The X/Y

coordinates are also displayed in cell G45 and G46.

True to Magnetic Conversion:

MAPX will also adjust for magnetic deviation.  Magnetic

deviation for the local area is entered in cell G10.

Following the rules, west deviation is (+) and east is (-).

Cell H10 will automatically display E for east or W for west.

If you forget the rules, cell H10 will remind you.  Input

the true azimuth in cell B45 (range is not required).  Cell

C45 will display the magnetic azimuth.

Ellipsiod:

The default ellisiod supplied with the worksheet is Clarke

1866.  This is the ellipsiod used for NAD27.  You can change

it to WGS84 by entering a, 6,378,137 in cell G14 and e^2,

0.00669438 in cell G15.  The Clarke 1866 values are a=

6,378,249.1, e^2=0.006768658.  You can use any other reference

ellipsiod to correspond with the datum of your map providing

you can find the appropriate values for equatorial radius (a)

and eccentricity squared (e^2).  See Appendix.

The ellisiod differences are so minor, they will not be

noticable.  The feature is included for psychological comfort

and for those trying to squeek out that last tenth of an arc

second.

Accuracy:

How accurate is MAPX?  It depends on the map scale, map

projection, printing errors, and humidity.  Humidity?  Yup,

paper can expand and contract +/- 1% due to humidity.  And,

it's not necessarily uniform across the sheet.  This error

is twice that incurred by using a spherical model for the

Earth Vs. an ellipsoid.

You can compensate for humidity and printing error by calculating

the map scale yourself.  I use the bar scale on the bottom

margin of USGS quadrangles to do this along with digital

calipers.  For example, spanning 4km of bar scale, I measure

166mm or .166 meters. 4000 meters / .166 meters = a scale of

24,096.  The published scale is 24,000.  So this map has

0.4% error.  Enter 24096 instead of 24000 for scale (B11).

The smaller the map scale, the more distortion you get from

placing the curved Earth on a flat sheet.  This will introduce

errors.  For large scale USGS Quads like 24,000 and 63,360 scales,

you won't be able to measure the effect (well... maybe on

63,360 scales).  Minimize the error by using the closest

reference available.  This is usually the corners of the map,

use the closest corner for the reference.  Note that there are

"crosshairs" every 2.5' on 7.5' quads.  They are often very hard

to pick out through the clutter but they are handy for drawing

in supplimental grid lines (only 4!).  This is much easier than

drawing in the UTM grid.  These crosshairs appear every 7.5' on

USGS 1:100,000 scale maps also.  You can punch in the lat/long

of the crosshairs and MAPX will tell you where to find them!

After you adjust for all the above, you still have to deal with

the fact that the mapped feature may be wrong.  National Map

Accuracy Standards used by the USGS say that no more than 10%

of well defined features will be off by more than 1/50 inch.

What about not so well defined features?  What about the 10%

allowed to be off by more than 1/50 inch (.5mm)?

MAPX displays distances to .1mm precision from the reference.

That translates to 2.4 meters on the ground.  A .3mm pencil point

is a 7.2 meter circle on the ground (1:24,000 scale).  GPS

with SA on has CEP of 100 meters.  That's a 8.4 mm circle on

the map.  Don't get wrapped around the precision axle.

Thanks:

Thanks to Ron Harris for his suggestions and critique of MAPX

computations.  Thanks to Chuck Hawley for the "Range/Bearing"

method.  And last, special thanks to Tom Cowell for for prodding

me into camping/hiking/canoeing situations where navigation turned

out to be a matter of life and death rather than an interesting

intellectual exercise.

References:

Snyder, John P., "Map Projections - A Working Manual", USGS

Professional Paper 1395, second printing, 1989.

Bowditch,N., "American Practical Navigator", U.S. Navy Hydrographic

Office, Washington D.C., 1966.

Duffet-Smith,P., "Practical Astronomy with Your Calculator",

third edition, Cambridge University Press, 1988.

Clarke,B., "Aviator's Guide to GPS", Tab Books, McGraw-Hill, 1994.

Moulton,F.,"An Introduction to Celestial Mechanics", Dover

Publications, 1970, originally Macmillan Company, 1914.

Greenhood,D., "Mapping", The University of Chicago Press, 1964.

Robinson et al,"Elements of Cartography", fifth edition,

John Wiley & Sons, 1978.

Thompson, M.,"Maps for America", third edition, U.S. Department

of the Interior, U.S. Printing Office, 1988.

Revision History:

Version 1.23 - Release

Version 1.24 - Documentation typos corrected

Version 1.30 - Fixed latitude display error occuring from 30'

to 59.99' south.

- Fixed X/Y sign error when crossing reference

and waypoint over the prime meridian.

- Fixed longitude modulus error when crossing

reference and waypoint over International

Date Line.

- Added user configurable ellipsiod.

- Added polar (range/bearing) input and output.

- Added true to magnetic heading conversion.

Appendix:

Some Official Ellipsiods -

Name:                  a           e^2          Use:

GRS80              6,378,137     .006694380     WGS84

Clarke 1866        6,378,204.4   .006768658     NAD27

International 1924 6,378,388     .00672267      world

Austrailian 1965   6,378,160     .00669454      Austrailia

Krasovsky 1940     6,378,245     .00669342      Soviet Union

Airy 1830          6,377,563.4   .00667065      Great Britain

Bessel 1841        6,377,397.2   .00667443      Central Europe

Everest 1830       6,377,276.3   .00663788      India, So. Asia

******************************************************************

*  Questions or comments?  Send them to me, John Ceccherelli on  *

*  CompuServe, 71011.3424  Internet: 71011.3424@CompuServe.com   *

*  January 1995 - Wappingers Falls, NY                           *

******************************************************************