PREFLIGHT PLANNING CALCULATOR FOR LIGHT AND ULTRALIGHT AIRCRAFT

Alan Christian

9394 Premier Way

Sacramento, CA 95826

INTRODUCTION

There are two programs in this archive.  PREFLITE.EXE is a preflight

planning aid for light and ultralight aircraft.  OPCOST.EXE will

calculate the actual per-hour cost of operating your aircraft.

I assume you already know how to successfully plan a flight with the

conventional tools (chart, plotter, E6B, etc.) and have a grasp of the

terminology.  If you are unfamiliar with preflight planning

techniques, you should plan to attend an approved ground school.  The

PREFLITE program is NOT a substitute for a solid understanding of the

subject of safe flight planning.

OPCOST.EXE simply inputs all the things that cost you money on your

plane, asks for the number of hours per year that you fly, and tells

you what it really costs to fly your bird.  This may also be of help

to those of you who are contemplating aircraft ownership, and would

like to figure out what your per-hour cost will be.  Use of this

program is described in the Appendix.

PREFLITE.EXE is an excellent flight planning tool for very low speed

operations.  An E6B computer does not indicate airspeeds below 30 MPH,

nor accurately determine wind correction angles at very low airspeeds.

The trig functions in the program provide correct results at any

airspeed, allowing you to plan ultralight, helicopter, or other low-

speed operations accurately.

DISCLAIMER

I have tested this program on day VFR flights in a single engine Cessna,

and have found no errors or bugs.  However, there is no guarantee that

there are no errors in the program.  As Pilot-In-Command, you are

responsible for the safe operation of your aircraft, which includes

proper preflight planning.  This program is a tool to help you plan your

flight.  Don't use it blindly, with your brain up and locked, and don't

leave your judgement, experience and training in your flight case.

Check your planning against actual flight performance, and convince

yourself that this program will work satisfactorily for you.

The program is provided "as-is", without warranty of any kind

regarding its fitness or performance for any application.  All risk of

use of this software rests with the user.  The author is not

responsible for loss incurred through use of the program, and in no

event is he liable for direct, indirect, incidental, or consequential

damages arising from any defect in the software.

However, the author is open to comments and suggestions on ways to

improve the program, and certainly welcomes any and all bug reports.

The goal is to provide useful, reliable, easy-to-use software for

pilots.

USER SUPPORTED SOFTWARE CONCEPT

This program is made available primarily on electronic bulletin boards,

but is also available from the author via direct mail.  The price is low

to encourage all who find it useful to be able to make the requested

contribution of $10.  Mail orders must include an additional $2 for

postage and handling.  California residents please add sales tax.  If

you have downloaded this from a bulletin board, please include the

release number from the sign-on message with your check and if the

bulletin board SYSOP has registered his copy with me (free registration

for him/her), I will send him/her a 10% commission as payment for

placing my program on the board.

Technical assistance with the program is available by mail to users who

have sent the $10 contribution.  They will also be placed on a mailing

list, so they can be notified of future updates and releases of the

program, or of additional programs that may become available.

Whether you use this program or not, and whether you pay for it or

not, please copy it intact and unmodified and pass it along to friends

who may have need for it.  Also, please feel free to upload

PREFLITE.ARC to bulletin boards.

The requested donation is valid for private (non-commercial)

applications only.  Commercial operators who use this program in their

operations are required to register it.  Commercial registration is $10

for each system the program is running on.

Please correspond with the author, either to send a contribution,

comment, suggestion, or bug report.   Send all correspondence to:

Alan Christian

9394 Premier Way

Sacramento, CA 95826

Programs and documentation copyright (c) 1984, 1987 by Alan Christian.

PROGRAM DESCRIPTION

The program named PREFLITE.EXE should be on your default disk drive.

Type  PREFLITE  and press return.  The program will load and you will

see the following message and menu displayed on the screen:

PREFLIGHT CALCULATION PROGRAM          Release nnnn             V1.0

(c) Copyright 1984,1986 - Alan Christian

- 9394 Premier Way, Sacramento, CA  95826

If you find this program of value, your $10 contribution will be

appreciated.  Comments or suggestions for improvement are welcome.

AS PILOT IN COMMAND, YOU ARE RESPONSIBLE FOR THE ACCURACY AND COMPLETENESS OF

YOUR PREFLIGHT PLANNING.   THIS PROGRAM IS AN AID  ---  IT IS NOT INTENDED TO

REPLACE YOUR JUDGEMENT.  IF YOU DOUBT THE ACCURACY OF A RESULT, CHECK IT OUT!

SELECT DESIRED FUNCTION

WEIGHT AND BALANCE -------------------- (1)

VERTICAL SPEED ------------------------ (2)

GROUND SPEED/WIND CORRECTION ANGLE ---- (3)

DENSITY ALTITUDE ---------------------- (4)

OPTIMUM ALTITUDE ---------------------- (5)

GREAT CIRCLE COURSE ------------------- (6)

FUEL CONSUMPTION ---------------------- (7)

CONVERSIONS --------------------------- (8)

RETURN TO SYSTEM ---------------------- (9)

Enter your choice ---------------------  __

The sign-on message only appears when you first load the program.

Subsequent returns to the main menu will display only the menu.

In the following descriptions of each program feature, I have used

examples to illustrate its use.  Try them yourself as you read the

instruction manual.  The answers follow each example, and are outlined

and written as you see them on your screen.

WEIGHT AND BALANCE CALCULATIONS

An important calculation for mose flights is weight and balance.  The

program allows for an unlimited number of passengers, cargo stowage

locations, and fuel tanks.  You are required to enter weight and arm for

the airframe (empty), the engine oil, at least one fuel tank, and the

pilot.  If you are flying a sailplane, you can enter a zero value for

fuel and oil weight.

You have the choice of entering data in INCH-POUNDS (a-la Cessna), or

in INCHES or CENTIMETERS forward or aft of the reference point.

When you have no (or no additional) data to enter for a category, enter

a zero (0).  When all categories have been entered, the program will

display total weight and the arm.  Compare these numbers with the

weight and balance information in your owner's manual to determine if

you are within the aircraft envelope.

You will notice that entry in the fuel, passenger and cargo categories

continues indefinitely until you enter a zero.  A program that allows

for a maximum of four passengers, one baggage compartment and one fuel

tank will not work for an 8-passenger twin with four cargo compartments

and several fuel cells.  The reverse is true also.  You shouldn't have

to enter data for an 8-passenger twin when you're planning a flight in

your Super Cub or your ultralight.

Example 1:

You are flying a friend to a nearby town.  Your aircraft weighs 1455

pounds empty with a moment of 41.2 inch-pounds/1000.  Your engine holds

12 quarts of oil with a moment of -1.5 inch-pounds/1000 and there will

be 70 gallons of aviation gasoline aboard when you start.  The fuel

moment is 22.5 inch-pounds/1000.  (Note that the computer figures the

weight of oil and fuel for you).  You weigh 195 pounds and the pilot

seat moment is 15 inch-pounds/1000.  Your passenger weighs 160 pounds

and the passenger seat moment is 17 inch-pounds/1000.  He is bringing a

suitcase weighing 25 pounds and the baggage area moment is 30 inch-

pounds/1000.  Calculate takeoff weight and balance.  (Hint: Select '1'

when asked how you will enter arms.)

-----------------------------

| WILL YOU ENTER ARMS IN    |

| INCH-POUNDS ------- (1)   |

| INCHES OR CM ------ (2)   |

| Enter your choice --_1    | <--- Select '1'

|                           |

|                           |

| A/C EMPTY WEIGHT ? 1455   | <--- Enter values from owner's

| A/C EMPTY ARM    ? 41.2   |      manual.

| QTS OF OIL IN ENG? 12     |

| ENGINE OIL ARM   ? -1.5   |

| ENTER ZERO TO SKIP        |

| FUEL GALLONS     ? 70     |

| FUEL ARM         ? 22.5   |

| ENTER ZERO TO SKIP        |

| FUEL GALLONS     ? 0      | <--- Enter '0' to end fuel weight

| PILOT WEIGHT     ? 195    |      data entry.

| PILOT ARM        ? 15     |

| ENTER ZERO TO SKIP        |

| PASSENGER WEIGHT ? 160    |

| PASSENGER ARM    ? 17     |

| ENTER ZERO TO SKIP        |

| PASSENGER WEIGHT ? 0      | <--- Enter '0' to end passenger

| ENTER ZERO TO SKIP        |      weight data entry.

| CARGO WEIGHT     ? 25     |

| CARGO ARM        ? 30     |

| ENTER ZERO TO SKIP        |

| CARGO WEIGHT     ? 0      | <--- Enter '0' to end cargo weight

|                           |      data entry.

| GROSS WEIGHT= 2277.5      | <--- Compare with owner's manual

| TOTAL MOMENT= 124.2       | <--- to determine if aircraft will

-----------------------------      be within W&B limits.

Example 2:

You are taking a vacation in your plane.  You will be flying solo, with

just one suitcase and a flight bag.  Your aircraft weighs 1215 pounds

empty with an arm of 34 inches.  The engine holds 8 quarts of oil with

an arm of -1 inch.  The fuel tank holds 22 gallons of avgas with an arm

of 30 inches.  You weigh 190 pounds and sit at an arm of 36 inches.

Your baggage weighs 45 pounds and will be strapped down at an arm of 51

inches.  Find your weight and balance.  (Hint: Select '2' when asked how

you will enter arms.)

-----------------------------

|  WILL YOU ENTER ARMS IN   |

|  INCH-POUNDS ------- (1)  |

|  INCHES OR CM ------ (2)  |

|  Enter your choice --_2   | <--- Select '2'

|                           |

|                           |

|   A/C EMPTY WEIGHT ? 1215 | <--- Enter values from owner's

|   A/C EMPTY ARM    ? 34   |      manual.

|   QTS OF OIL IN ENG? 8    |

|   ENGINE OIL ARM   ? -1   |

|   ENTER ZERO TO SKIP      |

|   FUEL GALLONS     ? 22   |

|   FUEL ARM         ? 30   |

|   ENTER ZERO TO SKIP      |

|   FUEL GALLONS     ? 0    | <--- Enter '0' to end fuel weight

|   PILOT WEIGHT     ? 190  |      data entry.

|   PILOT ARM        ? 36   |

|   ENTER ZERO TO SKIP      |

|   PASSENGER WEIGHT ? 0    | <--- Enter '0' to skip passenger

|   ENTER ZERO TO SKIP      |      weight data entry.

|   CARGO WEIGHT     ? 45   |

|   CARGO ARM        ? 51   |

|   ENTER ZERO TO SKIP      |

|   CARGO WEIGHT     ? 0    | <--- Enter '0' to end cargo weight

|                           |      data entry.

|   GROSS WEIGHT= 1597      | <--- Compare with owner's manual

|   TOTAL MOMENT= 54390     | <--- to determine if aircraft is

-----------------------------      within W&B limits.

VERTICAL SPEED CALCULATIONS

This feature is used when you need to climb or descend a given vertical

distance (such as climbing to clear an obstacle) while flying a track

over the ground.  You enter your ground speed in either knots or MPH.

You can then calculate your vertical distance travelled in feet per

mile, given rate of climb in feet per minute, or you can calculate the

rate of climb given the horizontal distance to travel and the

difference in altitude desired.

Example 3:

You are departing an airport that is situated in a natural bowl in the

mountains.  After takeoff you need to climb an additional 2500 feet to

safely clear a mountain pass.  The pass is located 11 miles from the end

of the runway and your groundspeed during the climb is 120 MPH.  What

rate of climb do you have to maintain after takeoff to clear the pass

without circling?

Answer: You must maintain at least 454 FPM R-O-C.

-----------------------------------------------------------------

|                                                               |

| SPEED WILL BE ENTERED IN:                                     |

|                      KNOTS (1)                                |

|                      MPH   (2)                                |

|          Enter your choice _2                                 |

|                                                               |

| ENTER GROUNDSPEED IN MPH? 120                                 |

|                                                               |

| DO YOU WANT TO .....                                          |

|      Enter FEET PER MILE and calculate FEET/MINUTE (1)        |

|      Enter FEET PER MINUTE and calculate FEET/MILE (2)        |

|                                  Enter your choice  _1        |

|                                                               |

| ENTER ALTITUDE TO BE GAINED OR LOST (IN FT)? 2500             |

| ENTER DISTANCE TRAVELED FOR ALTITUDE CHANGE (IN MILES)? 11    |

| VERTICAL SPEED = 454 FT PER MINUTE                            |

|                                                               |

-----------------------------------------------------------------

Example 4:

You want to file for a standard instrument departure that requires you

to climb 4000 feet along an airway between two intersections, which are

10 miles apart.  At climb power and gross takeoff weight, your plane

will maintain a steady 550 feet per minute rate of climb.  Your ground

speed during climbout will be 105 knots.  Can you file for this

departure, or must you take a longer route that offers a flatter climb

angle?

Answer: 273 feet per mile over a 10-mile path = 2730 feet, which is

not enough.  You must file for the longer route.

-----------------------------------------------------------------

|                                                               |

| SPEED WILL BE ENTERED IN:                                     |

|                      KNOTS (1)                                |

|                      MPH   (2)                                |

|          Enter your choice _1                                 |

|                                                               |

| ENTER GROUNDSPEED IN KNOTS? 105                               |

|                                                               |

| DO YOU WANT TO .....                                          |

|      Enter FEET PER MILE and calculate FEET/MINUTE (1)        |

|      Enter FEET PER MINUTE and calculate FEET/MILE (2)        |

|                                  Enter your choice  _2        |

|                                                               |

| ENTER VERTICAL SPEED IN FEET PER MINUTE? 550                  |

|                                                               |

| VERTICAL SPEED = 273 FT PER MILE                              |

|                                                               |

-----------------------------------------------------------------

GROUND SPEED AND WIND CORRECTION ANGLE

You've obtained the winds aloft forecast from FSS, and you know your

TAS at cruising altitude.  Now to draw those pencilled wind triangles

on the E6B and figure out your groundspeed for each leg, and your wind

correction angle.  OR, you can enter the wind speed and direction, and

your aircraft TAS and heading, and the computer will figure the WCA and

ground speed for you.

Example 5:

Forecast wind is 17 knots at 260 degrees.  Your TAS will be 144 knots on

a true course of 185 degrees.  Find your ground speed and wind

correction angle.

-----------------------------------------------------------------

|                                                               |

|  GROUNDSPEED/WIND CORRECTION ANGLE CALCULATION                |

|                                                               |

|  WIND SPEED             (KNOTS)? 17                           |

|  WIND DIRECTION       (DEGREES)? 260                          |

|  AIRCRAFT TRUE AIRSPEED (KNOTS)? 144                          |

|  AIRCRAFT TRUE COURSE (DEGREES)? 185                          |

|                                                               |

|  GROUNDSPEED= 140 KNOTS                                       |

|  WIND CORRECTION = 7 DEGREES RIGHT                            |

|                                                               |

-----------------------------------------------------------------

Example 6:

Forecast wind is 38 knots at 020 degrees.  Your TAS will be 106 knots on

a true heading of 185 degrees.  Find your ground speed and WCA.

-----------------------------------------------------------------

|                                                               |

|  GROUNDSPEED/WIND CORRECTION ANGLE CALCULATION                |

|                                                               |

|  WIND SPEED             (KNOTS)? 38                           |

|  WIND DIRECTION       (DEGREES)? 020                          |

|  AIRCRAFT TRUE AIRSPEED (KNOTS)? 106                          |

|  AIRCRAFT TRUE COURSE (DEGREES)? 185                          |

|                                                               |

|  GROUNDSPEED= 143 KNOTS                                       |

|  WIND CORRECTION = 4 DEGREES LEFT                             |

|                                                               |

-----------------------------------------------------------------

DENSITY ALTITUDE

Elevated air temperatures reduce the density of the air, which causes

your plane to fly as if it were at a higher altitude than the altimeter

indicates.  Sea level departures rarely require calculations for density

altitude since its effects are not as hazardous at sea level as they are

at higher-altitude airports.  A 2000-foot density altitude difference at

sea level translates into a slightly longer takeoff run and slightly

reduced rate of climb.  That same density altitude difference at an

8000-foot-high mountain strip can spell disaster.

Example 7:

Your 4-place single has two adults, two children, and some suitcases on

board, and you are leaving a mountain resort airport in the Rocky

Mountains for a trip back home to St. Louis.  The air temperature is 17

degrees C.  Your altimeter, when set to 29.92, reads a pressure altitude

of 8800 feet.  Given the temperature today, what is the Density Altitude

at this field?  The airplane's rate of climb is less than 200 FPM at

altitudes above 10,000 feet.  Can you hope to realize better than a 200

FPM rate of climb from this field at today's temperature?

Answer:

NO. Your rate of climb will be less than 200 FPM.

-----------------------------------------------------------------

|                                                               |

|  DENSITY ALTITUDE CALCULATION                                 |

|                                                               |

|  PRESSURE ALTITUDE? 8800                                      |

|  AIR TEMP (DEGR.C)? 17                                        |

|                                                               |

|  DENSITY ALTITUDE= 11020                                      |

|                                                               |

-----------------------------------------------------------------

Example 8:

You are leaving a mountain strip and will need to climb to 12,000 feet

to safely clear a mountain pass.  The pressure altitude is 8000 feet and

the surface temperature is 20 degrees C.  FAA says that the air

temperature decreases 2 degrees C. for each 1000 feet of altitude.  Your

service ceiling is 13,500 feet.  Can you make the pass?

First calculate temperature at the altitude of the pass.

Temperature at 8000 feet = 20 degrees

Difference in altitude = 12000 - 8000 feet = 4000 feet.

Temperature drops 2 degrees per 1000 feet = 8 degrees.

Estimated temperature at 12000 feet is 12 degrees.

Then calculate density altitude (at the mountain pass).

-----------------------------------------------------------------

|                                                               |

|  DENSITY ALTITUDE CALCULATION                                 |

|                                                               |

|  PRESSURE ALTITUDE? 12000                                     |

|  AIR TEMP (DEGR.C)? 12                                        |

|                                                               |

|  DENSITY ALTITUDE= 14430                                      |

|                                                               |

-----------------------------------------------------------------

Answer:

The density altitude is above your service ceiling. You will not be

able to climb over the pass.

OPTIMUM ALTITUDE

This feature allows you to enter the FAA forecast winds aloft for 3000,

6000, 9000, and 12000 feet and then enter your TAS and heading.  The

computer will display your groundspeed for each of these altitudes.

This feature can help you select a potential cruising altitude when the

winds are coming from different directions at 3000-foot intervals.

Naturally you will need to consider terrain clearance at the optimum

altitude.

Example 9:

FAA forecasts: 3000 feet  12 knots   210 degrees

6000 feet  17 knots   260 degrees

9000 feet  23 knots   300 degrees

12000 feet  27 knots   360 degrees

Your heading will be 275 degrees at a TAS of 110 knots.  What is your

optimum altitude?

---------------------------------------------------

|                                                 |

| ENTER WIND SPEED  IN KNOTS AT 3000  FT.? 12     |

| ENTER WIND DIR. IN DEGREES AT 3000  FT.? 210    |

| ENTER WIND SPEED  IN KNOTS AT 6000  FT.? 17     |

| ENTER WIND DIR. IN DEGREES AT 6000  FT.? 260    |

| ENTER WIND SPEED  IN KNOTS AT 9000  FT.? 23     |

| ENTER WIND DIR. IN DEGREES AT 9000  FT.? 300    |

| ENTER WIND SPEED  IN KNOTS AT 12000  FT.? 27    |

| ENTER WIND DIR. IN DEGREES AT 12000  FT.? 360   |

| AIRCRAFT TRUE AIRSPEED (KTS)? 110               |

| AIRCRAFT TRUE COURSE (DEGR.)? 275               |

| ALTITUDE     GROUNDSPEED                        |

|  3000          105 KTS       <------------------------ 2nd choice

|  6000          93 KTS                           |

|  9000          89 KTS                           |

|  12000         110 KTS       <------------------------ Best altitude

|                                                 |

---------------------------------------------------

Example 10:

FAA forecasts: 3000 feet   5 knots   160 degrees

6000 feet  12 knots   190 degrees

9000 feet  20 knots   220 degrees

12000 feet  31 knots   260 degrees

Your heading will be 200 degrees at a TAS of 95 knots.  What is your

optimum altitude?

---------------------------------------------------

|                                                 |

| ENTER WIND SPEED  IN KNOTS AT 3000  FT.? 5      |

| ENTER WIND DIR. IN DEGREES AT 3000  FT.? 160    |

| ENTER WIND SPEED  IN KNOTS AT 6000  FT.? 12     |

| ENTER WIND DIR. IN DEGREES AT 6000  FT.? 190    |

| ENTER WIND SPEED  IN KNOTS AT 9000  FT.? 20     |

| ENTER WIND DIR. IN DEGREES AT 9000  FT.? 220    |

| ENTER WIND SPEED  IN KNOTS AT 12000  FT.? 31    |

| ENTER WIND DIR. IN DEGREES AT 12000  FT.? 260   |

| AIRCRAFT TRUE AIRSPEED (KTS)? 95                |

| AIRCRAFT TRUE COURSE (DEGR.)? 200               |

| ALTITUDE     GROUNDSPEED                        |

|  3000          91 KTS    <------------------------- 1st choice

|  6000          83 KTS    <------------------------- 2nd choice

|  9000          76 KTS                           |      -OR-

|  12000         83 KTS    <------------------------- 2nd choice

|                                                 |

---------------------------------------------------

Example 11:

You are planning a triangular course in your ultralight.  The three

true courses are 090 degrees, 180 degrees, and 285 degrees.  Your

ultralight flies at a TAS of 33 knots.  You call the nearby FSS and get

the winds aloft forecast.

FAA forecasts: 3000 feet   4 knots   100 degrees

6000 feet   6 knots   130 degrees

9000 feet   9 knots   160 degrees

12000 feet  12 knots   180 degrees

For each leg of your flight, at which forecast altitude will you

achieve the best groundspeed?   (Hint: Run the calculations 3 times).

---------------------------------------------------

|                                                 |

| ENTER WIND SPEED  IN KNOTS AT 3000  FT.? 4      |

| ENTER WIND DIR. IN DEGREES AT 3000  FT.? 100    |

| ENTER WIND SPEED  IN KNOTS AT 6000  FT.? 6      |

| ENTER WIND DIR. IN DEGREES AT 6000  FT.? 130    |

| ENTER WIND SPEED  IN KNOTS AT 9000  FT.? 9      |

| ENTER WIND DIR. IN DEGREES AT 9000  FT.? 160    |

| ENTER WIND SPEED  IN KNOTS AT 12000  FT.? 12    |

| ENTER WIND DIR. IN DEGREES AT 12000  FT.? 180   |

| AIRCRAFT TRUE AIRSPEED (KTS)? 33                |

| AIRCRAFT TRUE COURSE (DEGR.)? 090    180  285   |

| ALTITUDE     GROUNDSPEED                        | ANSWER:

|  3000          KNOTS ---->     29     32   36   | Fly 12000 on leg 1

|  6000                          28     29   38   | Fly 3000 on leg 2

|  9000                          31     24   38   | Fly 6/9000 on leg 3

|  12000                         35     21   37   |

|                          LEG #  1      2    3   |

---------------------------------------------------

GREAT CIRCLE COURSE CALCULATION

A great circle is the shortest distance between two points on the Earth.

Trans-Pacific jets fly great circles between the West Coast and Japan,

for example.  In a light plane, you rarely need to calculate a great

circle heading between airports, but just in case...... here's the

program.  You simply enter the latitude and longitude of the departure

and destination airports and the program calculates the distance between

the fields along a great circle, plus calculates the initial heading to

fly.

A latitude south of the equator is entered as a negative number, and a

longitude between the Greenwich meridian and the international date line

is entered as a negative number.  Maximum latitude is +/- 89 degrees.

Maximum longitude is +/- 180 degrees. Do not enter a course where

departure and destination longitude are exactly the same or a program

error will occur.

Be sure to enter both degrees and minutes of latitude and longitude as

dd/mm, where dd = degrees and mm = minutes, and they are separated by a

slash.  Enter both numbers, even if one or both are zero.

Example 12:

You are departing from at an airport located at 34 degrees 11 minutes

North, 126 degrees 19 minutes West.  Your destination is a field at 25

degrees 50 minutes North, 97 degrees 42 minutes West.  What is your

initial great circle heading, and the distance between airports?

-------------------------------------------------------

|                                                     |

| ENTER LATITUDE AND LONGITUDE IN THIS FORMAT:        |

|                          dd/mm  d=DEGREE m=MINUTES  |

|                                                     |

| STARTING LATITUDE ---------------------? 34/11      |

| STARTING LONGITUDE --------------------? 126/19     |

| DESTINATION LATITUDE ------------------? 25/50      |

| DESTINATION LONGITUDE -----------------? 97/42      |

| FLIGHT DISTANCE = 1562 NAUTICAL MILES               |

| INITIAL HEADING = 100  DEGREES                      |

|                                                     |

-------------------------------------------------------

Example 13:

You are departing from at an airport located at 46 degrees 15 minutes

North, 75 degrees 30 minutes West.  Your destination is a field at 30

degrees 00 minutes North, 83 degrees 05 minutes West.  What is your

initial great circle heading, and the distance between airports?

-------------------------------------------------------

|                                                     |

| ENTER LATITUDE AND LONGITUDE IN THIS FORMAT:        |

|                          dd/mm  d=DEGREE m=MINUTES  |

|                                                     |

| STARTING LATITUDE ---------------------? 46/15      |

| STARTING LONGITUDE --------------------? 75/30      |

| DESTINATION LATITUDE ------------------? 30/00      |

| DESTINATION LONGITUDE -----------------? 83/05      |

| FLIGHT DISTANCE = 1037 NAUTICAL MILES               |

| INITIAL HEADING = 202  DEGREES                      |

|                                                     |

-------------------------------------------------------

FUEL CONSUMPTION CALCULATIONS

This feature asks for your usable fuel on board, then asks you to enter

your estimated fuel consumed before takeoff, during climb, cruise and

descent.  It displays fuel used and remaining available after each

entry, plus zero-reserve time and distance to go when a cruise leg is

entered.

A WORD ABOUT THE FUEL CONSUMPTION CALCULATIONS.

The examples shown are for simple VFR depart-climb-cruise-descend-land

flight plans.  It is possible to profile a more complex flight, with

climbs and descents enroute (as is often the case when flying IFR).

Simply select the operation for the NEXT leg of the flight and enter the

data.  The computer will keep track of your cumulative fuel used, and

amount of fuel remaining.  On cruise legs, it will display the flight

time and distance remaining (ZERO reserve) if you continued cruising at

the last-entered speed and fuel consumption rate.

Example 14:

Your aircraft is filled with 45 gallons of fuel.  You estimate pre-

takeoff consumption of 2 gallons, a climb consumption of 1.8 gallons

per 1000 feet of altitude gained in a climb from sea level to 9500

feet.  You plan to cruise 220 miles at a groundspeed of 90 knots with a

cruise consumption of 4.5 gph.  Your descent will cover 24 miles at a

groundspeed of 95 knots, with a descent consumption of 4.5 gph.  How

much fuel will remain in your tank when you reach your destination?

1st Screen

------------------------------------

| FUEL CONSUMPTION PLANNER         |

|                                  |

| STARTING USABLE FUEL  (GAL):? 45 | <--- Enter fuel quantity

| PRETAKEOFF FUEL USED  (GAL):? 2  | <--- Enter pretakeoff fuel used

| SELECT OPERATION                 |

|      CLIMB -----------(1)        | <--- Select CLIMB

|      CRUISE ----------(2)        |

|      DESCEND ---------(3)        |

|      QUIT ------------(4)        |

|     Enter your choice  __        |

|                                  |

| STARTING ALTITUDE    ? 0         | <--- Sea Level Departure

| ENDING ALTITUDE      ? 9500      | <--- Cruising altitude

| FUEL USED PER 1000 FT? 1.8       | <--- Climb fuel consumption

|                                  |

------------------------------------

2nd Screen

------------------------------------

|                                  |

| FUEL USED      TO GO             |

|  19.1 GAL      25.9 GAL          | <--- Fuel used/remaining after

|                                  |      climb

| SELECT OPERATION                 |

|      CLIMB -----------(1)        |

|      CRUISE ----------(2)        | <--- Select CRUISE

|      DESCEND ---------(3)        |

|      QUIT ------------(4)        |

|     Enter your choice  __        |

|                                  |

| FUEL CONSUMPTION (GPH)? 4.5      | <--- Enter cruise fuel consumption

| DISTANCE (MILES)      ? 220      | <--- Enter distance at cruise

| GROUNDSPEED (MPH/KTS) ? 90       | <--- Enter groundspeed

|                                  | (Use MPH: Distance in statute mi.)

------------------------------------ (Use KTS: Distance in nautical mi.)

3rd Screen

------------------------------------

| FUEL USED      TO GO             |      Fuel used, and remaining, at

|  30.1 GAL      14.9 GAL          | <--- end of cruise leg.

| TIME TO GO     DIST TO GO        |

|  3.3 HRS       297 MILES         | <--- No-reserve time and distance

|                                  |      remaining at cruise.

| SELECT OPERATION                 |

|      CLIMB -----------(1)        |

|      CRUISE ----------(2)        |

|      DESCEND ---------(3)        | <--- Select DESCEND

|      QUIT ------------(4)        |

|     Enter your choice  __        |

|                                  |

| FUEL CONSUMPTION (GPH)? 4.5      | <--- Enter descent fuel consumption

| DISTANCE (MILES)      ? 24       | <--- Enter distance to descend

| GROUNDSPEED (MPH/KTS) ? 95       | <--- Enter groundspeed

|                                  | (Use MPH: Distance in statute mi.)

------------------------------------ (Use KTS: Distance in nautical mi.)

4th Screen

------------------------------------

| FUEL USED      TO GO             |

|  31.2 GAL      13.8 GAL          | <--- Total fuel used for trip,

|                                  |       and fuel remaining.

|                                  |

| SELECT OPERATION                 |

|      CLIMB -----------(1)        |

|      CRUISE ----------(2)        |

|      DESCEND ---------(3)        |

|      QUIT ------------(4)        | <--- Select QUIT to return to menu

|     Enter your choice  __        |

|                                  |

------------------------------------

Example 15:

You have 90 gallons on board.  You will burn 2.5 gallons during taxi and

run-up, then will burn 2 gallons per 1000 feet in a climb from 2400 feet

to 12,500 feet.  You plan to cruise 595 miles at 170 knots groundspeed,

burning 14 gph at cruise.  Your descent will cover 30 miles at 185

knots, with a consumption of 13.5 gph.  How much fuel will remain in

your tanks when you reach your destination?

1st Screen

------------------------------------

| FUEL CONSUMPTION PLANNER         |

|                                  |

| STARTING USABLE FUEL  (GAL):? 90 | <--- Enter fuel quantity

| PRETAKEOFF FUEL USED  (GAL):? 2.5| <--- Enter pretakeoff fuel used

| SELECT OPERATION                 |

|      CLIMB -----------(1)        | <--- Select CLIMB

|      CRUISE ----------(2)        |

|      DESCEND ---------(3)        |

|      QUIT ------------(4)        |

|     Enter your choice  __        |

|                                  |

| STARTING ALTITUDE    ? 2400      | <--- Departure altitude

| ENDING ALTITUDE      ? 12500     | <--- Cruising altitude

| FUEL USED PER 1000 FT? 2         | <--- Climb fuel consumption

|                                  |

------------------------------------

2nd Screen

------------------------------------

|                                  |

| FUEL USED      TO GO             |

|  22.7 GAL      67.3 GAL          | <--- Fuel used/remaining after

|                                  |      climb

| SELECT OPERATION                 |

|      CLIMB -----------(1)        |

|      CRUISE ----------(2)        | <--- Select CRUISE

|      DESCEND ---------(3)        |

|      QUIT ------------(4)        |

|     Enter your choice  __        |

|                                  |

| FUEL CONSUMPTION (GPH)? 14       | <--- Enter cruise fuel consumption

| DISTANCE (MILES)      ? 595      | <--- Enter distance at cruise

| GROUNDSPEED (MPH/KTS) ? 170      | <--- Enter groundspeed

|                                  | (Use MPH: Distance in statute mi.)

------------------------------------ (Use KTS: Distance in nautical mi.)

3rd Screen

------------------------------------

| FUEL USED      TO GO             |      Fuel used, and remaining, at

|  71.7 GAL      18.3 GAL          | <--- end of cruise leg.

| TIME TO GO     DIST TO GO        |

|  1.3 HRS       220 MILES         | <--- No-reserve time and distance

|                                  |      remaining at cruise.

| SELECT OPERATION                 |

|      CLIMB -----------(1)        |

|      CRUISE ----------(2)        |

|      DESCEND ---------(3)        | <--- Select DESCEND

|      QUIT ------------(4)        |

|     Enter your choice  __        |

|                                  |

| FUEL CONSUMPTION (GPH)? 13.5     | <--- Enter descent fuel consumption

| DISTANCE (MILES)      ? 30       | <--- Enter distance to descend

| GROUNDSPEED (MPH/KTS) ? 185      | <--- Enter groundspeed

|                                  | (Use MPH: Distance in statute mi.)

------------------------------------ (Use KTS: Distance in nautical mi.)

4th Screen

------------------------------------

| FUEL USED      TO GO             |

|  73.9 GAL      16.1 GAL          | <--- Total fuel used for trip,

|                                  |       and fuel remaining.

|                                  |

| SELECT OPERATION                 |

|      CLIMB -----------(1)        |

|      CRUISE ----------(2)        |

|      DESCEND ---------(3)        |

|      QUIT ------------(4)        | <--- Select QUIT to return to menu

|     Enter your choice  __        |

|                                  |

------------------------------------

CONVERSIONS

These are several conversion routines that allow you to convert between

different units of measure.   The following conversions are available:

DISTANCE/SPEED: Statute Miles (MPH), Nautical Miles (Knots), Kilometers

(KPH)

ALTITUDE: Feet, meters.

TEMPERATURE: Degrees F. and degrees C.

LIQUID MEASURE: US Gallons, Imperial Gallons, Quarts, Litres

COURSE: Magnetic, True

WEIGHTS: Pounds, Kilograms

RETURN TO SYSTEM

This menu selection exits the program and returns to the disk operating

system (DOS).

APPENDIX: Calculating your hourly operating cost with OPCOST.EXE

OPCOST.EXE combines annualized costs, such as insurance, inspections,

repairs, depreciation, and licensing fees with monthly costs (payment,

tiedown) and operations costs (fuel, oil, RON, landing fees).  It asks

you to enter the number of hours you fly each year, and tells you your

actual cost to fly each hour.

To run the program, be sure that OPCOST.EXE is on the default disk

drive.  Enter OPCOST, press return and follow the program prompts.

If an expense item does not apply to you (such as no hull insurance, no

100-hour inspections, or no avionics maintenance), enter a '0' (zero).

At the end of the program, you may enter a different value for annual

flight hours to let you evaluate your costs for different amounts of

flying.  You can also restart the program and enter a different set of

cost numbers, or you can quit and return to DOS.

OPCOST Examples:

Hourly operational cost calculations for your single engine and your

ultralight aircraft:

(* = Ultralight fuel cost includes mixed 2-cycle oil cost.)

CATEGORY                      SINGLE            ULTRALIGHT

----------------------------------------------------------------------

Liability Insurance                 $ 600             $ 1100

Hull Insurance                        950                  0

Other Insurance Premiums              350                  0

Depreciation                         2100                400

Annual Inspection                     400                 60

Property Tax                          880                  0

Registration Fee                       25                  0

Avionics Maintenance                  375                  0

Airframe Maintenance                  400                450

Engine Maintenance                    225                 70

Instrument Maintenance                 75                  0

Miscellaneous Maintenance             300                200

Landing Fees                          200                  0

Transient Parking Fees (RON)          100                  0

Monthly Payment                      1450                205

Monthly Tiedown                        55                 25

100-hour inspection                     0                  0

Fuel consumption rate                  12 gph              1.4 gph

Fuel cost                               2.10/gal           1.80/gal*

Quarts of Oil used per 100 hours        4                  0

Oil cost per quart                      3.65               0

Hours flown per year                 1500                120

-------------------------------------

HOURLY COST                       $ 42.03             $ 44.52

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

The Public (Software) Library

Disks in the P(s)L are updated monthly, so if you did not get

this disk  directly from the P(s)L,  you should be aware that

the files in this set may no longer be the current versions.

For a copy of the latest monthly software library newsletter

and a list of the 800+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

(713) 721-6104 or (713) 721-5205

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.