THE CODE MACHINE

Version 3.0

* Lloyd J. Wilson, N6IKH

1530 W. Ranch Rd

San Bernardino, CA  92407

TABLE OF CONTENTS

I.  Introduction .......................................  2

II.  Getting Started ....................................  2

III.  Menus ..............................................  3

IV.  Transmit ...........................................  3

V.  Words Per Minute (WPM) .............................  4

VI.  Call Sign ..........................................  6

VII.  Messages ...........................................  6

VIII.  Practice ...........................................  7

IX.  Help ...............................................  8

X.  Quit ...............................................  8

XI.  Special Characters .................................  8

XII.  Connection to Your Transmitter .....................  9

XIII.  Date and Time ......................................  9

XIV.  System Speed .......................................  9

XV.  Disk Files ......................................... 10

* Note: At some time in the future this address

may become obsolete.  If you need to

contact me and can't reach me at this

address, you might find my address

in a current Call Book.

��������������������������ͻ

� SECTION I - INTRODUCTION �

��������������������������ͼ

This program is distributed as Public Domain Software.  The

Copyright is retained by the author.  The program may be freely

copied, placed on any bulletin board system, or otherwise

freely distributed.  Shareware distributors and bulletion board

services may charge their standard rates.

No fees are solicited for use of this program, and none will be

accepted.

������������������������������ͻ

� SECTION II - GETTING STARTED �

������������������������������ͼ

The Code Machine requires and IBM PC, XT, AT, or compatible, at

least 256K of RAM, and PC-DOS or MS-DOS 2.0 or higher.

Floppy Disk Systems

To run The Code Machine on a floppy-based system, copy all

files on The Code Machine distribution disk to a boot disk (a

disk formatted with the /s option).  After booting up with this

disk, enter `TCM` (for The Code Machine) to start the program.

Hard Disk Systems

Create a new directory called "CODE". Copy all files on the

distribution disk into \CODE.  Log into this directory and

enter `TCM` to start the program.  This sequence would be as

follows:

C>MD \CODE

C>COPY A:*.* \CODE

C>CD \CODE

C\CODE>TCM

Batch Files

A batch file will allow you to enter the program without

changing directories.  The following example of a batch file

named TCM.BAT (located in the root directory or a directory for

which a path has been established) will let you start The Code

Machine by entering `TCM`:

CD\CODE

TCM

\CD

-2-

Manual

To print this manual, align your paper to the top of a page, then

at the DOS prompt type in the following command:

PRINT TCM.DOC

���������������������ͻ

� SECTION III - MENUS �

���������������������ͼ

The Code Machine's menus work like most standard program menus

today.  You may either use the arrow keys to identify the

selection you want and then press the [ENTER] key, or you may

simply press the first character of your selection.  The second

method is generally preferred as it requires only one keystroke

to obtain the desired menu selection.

�����������������������ͻ

� SECTION IV - TRANSMIT �

�����������������������ͼ

Once you enter the transmit mode, you will see the cursor

located at the beginning of the transmit buffer.  Code will be

transmitted as soon as you begin typing characters into the

buffer.  You will hear a series of clicks on your speaker as you

type in characters.  This is the sound of a D.C. signal being sent

to your speaker port, this is also the signal that will drive your

transmitter (see Section XII, "Connection to Your Transmitter").

If you want to hear an audible tone from your speaker, select

"Practice" then "Use Transmit Buffer" from the menu.

The characters will be transmitted at the WPM previously selected

(see Section V, "Words Per Minute").  If you select a character for

which a message exists in your message library, that message will

be placed into the transmit buffer (see Section VII, "Messages").

As the code is transmitted, the character that is currently being

sent will blink.  The transmit buffer is 13 lines long which allows

you to enter over 1000 characters (13 X 80 = 1040) ahead of what is

being transmitted.  If more space is needed, the buffer will scroll

to make room for the additional characters.  The buffer will not

scroll until all the characters in the top line have been

transmitted.  For messages, four lines must be available before a

scroll will occur.

The [ESC] escape key will (1) suspend transmission, (2) clear the

transmit buffer, and (3) return control to the main menu.  It is

not necessary to leave the transmit buffer between copying and

sending sessions, as the program waits when it reaches the cursor.

Transmission will start again when additional characters are

entered.

-3-

The [CTRL]-[S] keys pause transmission until another key is

touched.  If you select to pause the transmission in the middle of

a character, the program will resume transmission with the next

full character, thus preventing transmission of a partial

character.

������������������������������������ͻ

� SECTION V - WORDS PER MINUTE (WPM) �

������������������������������������ͼ

The Code Machine offers an extremely versatile selection of code

speeds and modified code speeds.  The speeds and modifications

selected under menu choice WPM control both the code transmission

speed and the speed of the practice programs.  Selection of code

speeds falls into three categories: (1) true speeds, (2) modified

speeds, and (3) space-elongated speeds.

True Speeds

True code speed is just what the name implies. True speed complies

strictly with the guidelines outlined in the ARRL Handbook: dit = 1

unit dah = 3 units space between character elements = 1 unit space

between characters = 3 units space between words = 7

units average units per word = 50 units

To enter the desired true code speed simply select the WPM option

off the Main Menu, enter any speed between 1 and 100, and press the

[ENTER] key.  The speed you selected will be displayed at the top

right-hand corner of your monitor, followed by a comma and "True".

Modified Speeds

Working at slower speeds can be somewhat distracting because the

lengths of the characters increase excessively.  For example, at 1

WPM the true length of a dah is 3.6 seconds.  Even at 5 WPM the dah

is almost 3/4 of a second long (.72 seconds).  With the modified

speed option, all characters are transmitted at 12 WPM, however,

the spaces between the characters and words are increased to obtain

the overall speed selected. To select the modified code, enter a

comma then a `M`  (for modified) following the overall speed you

select.  For example, if you want 8 WPM modified code you will

select WPM off the Main Menu and enter the following:

8,M   (then press the [ENTER] key)

The speed you selected will be displayed at the top right-hand

corner of your monitor followed by a comma and "Mod".  Of course,

this option only works at speeds under 12 WPM.

-4-

Space-Elongated Speeds

If you listen closely to a station working CW, you will notice that

amateurs often leave more space between characters than the three

units (dah length) of true code, especially when working at the

slower speeds.  The Code Machine will let you recreate this type of

code transmission by increasing the space between characters.  You

can increase the space from 1.1 to 9.9 times what the true length

would be.  The way to select this option is choose WPM from the

Main Menu, select your WPM then enter a comma followed by the

increase desired (elongation factor).  For example, if you want to

double the space between characters and send characters at a rate

of 10 WPM, enter the following at the WPM prompt: 10,2

If you only wished to increase the length by 1 1/2 times (a 50%

increase) you would enter:

10,1.5

1 3/4 would be:

10,1.75

2 1/2 would be:

10,2.5

and so on.

With the space-elongated selection, the characters will be sent at

the true rate, however, the increased length between characters

will decrease your overall transmission rate.  Your actual rate can

be calculated with the following formula:

WPM = WPM - ((WPM X .38)(X-1))

where X = elongated factor

So, for example, if you selected 13 WPM and doubled your space

between characters (13,2) your actual code speed would be:

WPM = 13-((13 X .38)(2-1))

WPM = 13-(4.94 X 1)

WPM = 13-4.94

WPM = 8.06

-5-

������������������������ͻ

� SECTION VI - CALL SIGN �

������������������������ͼ

The call sign option is a very power and useful capability of The

Code Machine.  To use the option, simply select `Call Sign` from

the Main Menu and at the prompt enter the call sign of the station

you are working.  The call sign will be displayed in the upper left

corner of your monitor.  Once you have entered the call sign press

[ENTER] to return to the Main Menu.

Now when you are entering data in the transmit buffer, you can

simply select the colon [:] key and the station's call sign will be

entered into the transmit buffer.

Additionally, this command can be embedded into your messages-more

on this in the Messages Section.

������������������������ͻ

� SECTION VII - MESSAGES �

������������������������ͼ

You may store 22 messages of up to 255 characters each to use

during transmission.  Then by selecting only one key, that message

will be displayed in your buffer and transmitted.  There are no

special requirements and the messages can be inserted while you are

typing just as if you were entering another character.

Messages are automatically saved to your disk as you originally

enter them or change them.  All previously entered messages are

loaded when you start The Code Machine.  The messages are stored on

your disk in a file named TCM.MES.  If your program does not find

the TCM.MES file when it starts up, it creates a new one.  Ensure

that the disk containing The Code Machine has at least 6K of

available space in order to store your messages.  If you do not

have available space on your disk your TCM.MES file will be lost.

To enter new messages or view previously stored messages, select

`Messages` from the Main Menu.  You will be prompted to select the

key that corresponds to the message you want.  After selecting the

key you will be shown the current message stored for that key and

asked if you want to change it.  Select `Y` to enter a new message

and `N` to return to the Main Menu.  After selecting `Y`, you are

prompted to enter a message.  Each message can contain between 1

and 255 characters.

The Code Machine provides a very powerful method for entering

messages containing the call sign of the station being worked.

Enter the colon [:] in any message and the call sign of the station

your are working will replace the colon when that message is

called.

-6-

Remember to select one key to represent your own call sign.  For

convenience, select a key that does not require you to use the

[SHIFT] key.

�������������������������ͻ

� SECTION VIII - PRACTICE �

�������������������������ͼ

The Code Machine's practice programs use your computer's speaker to

output code at the rate previously selected with the WPM option.

Selecting `Practice` from the Main Menu drops down the Practice

Menu and allows you to select from seven different practice

programs.

Choose Characters

This option allows you to select individual characters you need to

practice.  You will be prompted to enter the characters you want to

practice.  You may select as many characters as you wish (up to

255). The characters will be randomly selected and placed in the

transmit buffer in groups of five.  You may select characters more

than once--their random selection will be proportional to the

number of times you select them.

Alpha Characters

All twenty-six letters will be randomly selected and placed in the

transmit buffer in groups of five.

Numeric Characters

Numbers 0 thru 9 will be randomly selected and placed in the

transmit buffer in groups of five.

Special Characters

The special characters (see Section XI, "Special Characters") will

be randomly selected and placed in the transmit buffer in groups of

five.

Letters and Numbers

All letters and numbers will be randomly selected and placed in the

transmit buffer in groups of five.

Total Character Set

All letters, numbers, and special characters will be randomly

selected and placed in the transmit buffer in groups of five.

-7-

Use Transmit Buffer

This selection works identical to the Transmit Mode except that an

audible frequency is sent to the computer's speaker.  Text can be

entered directly or sample tests and sample QSO's can be stored as

messages and then called up in this practice mode.   Note: Users of

Versions below 2.0 will find no difference between this selection

and `Transmit` on the Main Menu.

�������������������ͻ

� SECTION IX - HELP �

�������������������ͼ

Selecting help from the Main Menu will drop down an ON/OFF menu

that will let you toggle the help screen on and off.  If ON is

selected, the help screen will be displayed when you are in the

Transmit Mode.  This provides a quick reference to the special

characters while transmitting.

������������������ͻ

� SECTION X - QUIT �

������������������ͼ

Selecting `Quit` from the Main Menu returns control of your machine

to DOS.

���������������������������������ͻ

� SECTION XI - SPECIAL CHARACTERS �

���������������������������������ͼ

The following special characters and functions are supported by the

keys indicated:

[CONTROL] - [S] = Pauses transmission of characters

[-] dash = error

[\] backslash = KN

[/] slash = DN

[+] plus = AR

[=] equals = BT (double dash)

[;] semicolon = SK

[.] period = period

[,] comma = comma

[?] question mark = question mark

[SPACE BAR] = space between words

[ESC] escape = stops transmission, clears buffer, Main

[BACKSPACE] = moves cursor one space left and clears the

character in that space from the buffer

-8-

����������������������������������������������ͻ

� SECTION XII - CONNECTION TO YOUR TRANSMITTER �

����������������������������������������������ͼ

Keying your transmitter with The Code Machine Version  is a simple

matter of connecting two components, an optocoupler and a resister

or a relay and a diode, between the computers speaker port and the

transmitter.  Parts for either circuit are available at Radio Shack

for about two dollars.  Either the optocoupler or the relay will

completely isolate your computer from your transmitter. While there

are probably thousands of ways to connect your equipment, I would

suggest that any you come up with provide similar isolation

protection.  One other point, if you use a relay in the circuit, be

sure to use a diode to protect your computer from the back voltage

associated with inductive loads.

I have put together schematics and directions for three different

circuits (including Radio Shack parts numbers). I would be more

than happy to send you a copy.  In fact, if you are not completely

familiar with the components I have been speaking about, I highly

encourage you to send for the schematics for these coupling

circuits.  Just send a self addressed stamped envelope (SASE)

(letter size with the minimum postage is fine) to:

Lloyd J. Wilson

TCM Connections

1530 W. Ranch Rd

San Bernardino, CA  92407

Note: There is no charge--please do not send money.

������������������������������ͻ

� SECTION XIII - DATE AND TIME �

������������������������������ͼ

The current date and time are shown at the top of your monitor. The

clock is suspended during code transmission.  Once you return to

the Main Menu the clock will update itself and resume.  You may use

the suspended time to mark the beginning of a QSO.

����������������������������ͻ

� SECTION XIV - SYSTEM SPEED �

����������������������������ͼ

The Code Machine relies on system timing to accurately calculate

code speeds.  Because of the many different system clock

frequencies used by various compatible computers, the program does

a test when started to determine the speed of your computer.  This

results of this test are then used as the basis for the timing of

your code.  For this reason, users with selectable clock

frequencies (often called turbo) should not change their turbo

setting during operation of the program.  If you change your turbo

option while running the program, it will be necessary to restart

The Code Machine so the program's timing can be adjusted to the

computer's new clock frequency.

-9-

��������������������������ͻ

�  SECTION XV - DISK FILES �

��������������������������ͼ

The following files should be on your The Code Machine disk:

TCM.EXE  (The Code Machine Program) TCM.SYS  (The Code

Machine Program) TCM.DOC  (The Code Machine Manual)

TCM.MES  (Message file - Created by the program if not on

the disk)

-10-

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 4,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.