B A L L I S T I C

Version  3.01

(C) Copyright 1988 by William R. Frenchu

DISCLAIMER

Reloading  small arms ammunition can be a DANGEROUS pastime.  The

purpose of this program is NOT to teach the fundamentals of  this

fascinating hobby but rather to aid the hobbyist in assessing the

performance of the ammunition he has produced. It is assumed from

the outset that the user of this program is acquainted with basic

reloading  techniques. For review, a list of  references  wherein

may be found the necessary instruction is included in Appendix  B

of this document. DO NOT ATTEMPT TO PRODUCE AMMUNITION FOR  YOUR-

SELF  OR  OTHERS UNTIL YOU ARE COMPLETELY FAMILIAR WITH  ALL  THE

FUNDAMENTALS OF RELOADING. ALWAYS START WITH THE MINIMUM LOAD AND

WORK SLOWLY UP TO (NEVER EXCEED) THE MAXIMUM LISTED LOAD.

Although  every reasonable care was taken to insure the  accuracy

of this program, it is provided "AS IS" and without warranties as

to performance of merchantability or any other warranties whether

expressed  or implied, including, but not limited to warranty  of

fitness for a particular purpose. Good data processing  procedure

dictates that any program be tested with non-critical data before

relying on it. The user must assume the entire risk of using  the

program. The author will assume NO responsibility for any  damage

or injury that may occur.

User Supported Software

This  program may be copied and freely distributed for  any  non-

commercial use provided all accompanying files and  documentation

are  included  WITHOUT MODIFICATION. See Appendix E  for  further

information (and the usual pitch for cash).

TABLE OF CONTENTS

-----------------

1. About the Program ....................................... 1

A. General ............................................... 1

B. Program files ......................................... 1

C. System requirements ................................... 1

D. Program limitations ................................... 1

E. Notation .............................................. 2

F. What's new ............................................ 2

G. The future ............................................ 3

H. Acknowledgements ...................................... 3

2. Using the Program ....................................... 4

A. Starting the program .................................. 4

B. Command line arguments ................................ 4

C. Calculating the ballistic coefficient ................. 5

D. The ballistic table ................................... 5

E. Modifying the ballistic coefficient ................... 6

F. Changing the cross wind value ......................... 6

G. Changing the input function table ..................... 6

H. Miscellaneous functions menu .......................... 6

I. Calculating recoil .................................... 6

J. Calculating minimum twist ............................. 7

K. Calculating the point blank range ..................... 7

L. DOS Shell ............................................. 8

3. The Configuration File .................................. 9

A. Using the configuration file .......................... 9

B. Allowable parameters .................................. 9

4. Sources of Data ......................................... 11

A. Altitude .............................................. 11

B. Ballistic coefficient ................................. 11

C. Bullet weight, length and caliber ..................... 11

D. Gun and powder weight ................................. 11

E. Specific gravity of bullet alloys ..................... 12

F. Temperature, atmospheric pressure and wind ............ 12

G. Velocity .............................................. 12

5. Using GENTABLE and READTABLE ............................ 14

A. Using the pre-programmed functions .................... 14

B. Creating custom function tables ....................... 14

C. Checking the function file ............................ 15

Figures .................................................... 16

Appendix A, Glossary ....................................... 20

Appendix B, Information Sources ............................ 23

Appendix C, Coefficients for Several Factory Cartridges .... 24

Appendix D, Specific Gravities of Pb/Sn/Sb Alloys .......... 27

Appendix E, User Supported Software ........................ 28

Registration Form .......................................... 29

1.  ABOUT THE PROGRAM

A. General

BALLISTIC  is a Turbo C (v1.5) program written  to  generate

ballistic  tables for shooters. Given certain  easily  obtainable

data,  it  calculates such useful information  as  the  ballistic

coefficient,  remaining  velocity, energy, momentum,  drop,  wind

deflection, maximum height, and time of flight at a given  range.

In  addition,  it can be used to determine the amount  of  recoil

generated  by  a cartridge and gun, estimate  the  minimum  twist

necessary for bullet stabilization, calculate point blank  ranges

and produce trajectory tables for estimating "holdover". It  does

all  this by comparing the bullet in question to actual  compiled

firing data. As this data is calculated before use by a  separate

program  (GENTABLE),  the  ballistic  tables  are  produced  very

quickly.

B. Program files

The  following files should be included in any  distribution

of BALLISTIC:

-README.1ST  -Describes version, etc.

BAL-GEN.EXE  -GENTABLE  :creates function table for BALLISTIC

BAL-READ.EXE -READTABLE :optionally checks function table

BALISTIC.EXE -BALLISTIC :executable file

BALISTIC.DOC -This file

BALISTIC.REV -BALLISTIC revision history

BALREMOV.COM -Removes graphics characters from READTABLE.

C. System Requirements

BALLISTIC  should run on most IBM BIOS-compatible  computers

using  PC/MS-DOS 3.0 or above. It requires 150k and at least  one

disk  drive.  Non-graphic functions will be directed to  the  de-

fault display. Graphic functions (for Shareware versions) require

a  VGA,  EGA, CGA, ATT or Hercules compatible display  card.  (At

this  time  the Toshiba 3100 series is NOT supported due  to  in-

compatibilities between their display board and the ATT board  it

is supposed to emulate.) Registered versions can be supplied that

additionally  support  any  one of MCGA,  IBM  8514  or  PC-3270.

Starting  with version 3.00, CP/M support has  been  discontinued

due to a lack of interest.

BALLISTIC  has been tested and found to operate properly  on

the following systems:

PC/XT with Hercules monochrome or "EGA Wonder" adapter

PC/AT with CGA or EGA adapter

NEC MultiSpeed Portable

BIOS Model G (V40 CPU)

ATT 6300

Compaq Plus with Compaq CGA-type adapter

Compaq DeskPro 386 with Compaq EGA-type adapter

D. Program limitations

Any  program of this type will have certain limitations  re-

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 1

garding its use and accuracy and BALLISTIC is no exception. These

discrepancies  between  actual and calculated  performance  arise

primarily  because of the inability of the user to measure  input

values exactly and certain approximations the program uses in its

calculations. Assuming that the input values are known to 1%  the

average user should find agreement with published data to be 1-2%

for all velocity and trajectory calculations. The recoil section,

due to the more approximate nature of the calculation,  shouldn't

be considered more than 10-15% accurate.

While Ingalls' ballistic table is the most commonly used for

this  sort of work, several others are likely to be  encountered.

For the types of bullets most often used by reloaders the results

will  be very similar regardless of the table used.  Indeed,  the

variation  between handloads or factory ammunition randomly  sel-

ected  from the same lot will probably be much greater  than  the

discrepancies noted between different tables over normal shooting

ranges.  In any case, while the numbers produced by  the  program

are  believed to be accurate enough for all  practical  purposes,

they  are meant only as a guide to ammunition performance. How  a

particular load performs with a given gun can ONLY be  determined

by actual firings.

For best results, one should try to use the same input func-

tion  to produce the ballistic table that was initially  used  to

calculate  the coefficient. For instance, if the  Ingalls'  table

was used to calculate a ballistic coefficient, the Ingalls' table

should  also be used to produce the ballistic table.  This  isn't

always possible, as in the case where a published coefficient was

calculated  using a table to which the program doesn't  have  ac-

cess.

E. Notation

Throughout  this document, keys to be pushed in response  to

program  prompts  are enclosed in angle  brackets.  For  example,

"press  <1>" means to press number 1 on the keyboard. For  "press

<RETURN>",  use the key labeled either "return" or "enter".  "En-

tering"  a  value will require that the <RETURN> key  be  pressed

when finished, i.e., to "enter 10" press <1><0><RETURN>.

F. What's New

There  have  been many additions to the program  since  ver-

sion 2.30. Some of these include:

1. The  addition  of "G1" and "extended"  Ingalls'  tables  to

GENTABLE.

2. Addition of graphics for representation of trajectory.

3. Calculation of I.P.S.C. power factor.

4. Increased speed during table loading and program operation.

5. User configurable color for text and tables.

6. Calculation of point-blank range.

7. Calculation of minimum twist.

8. Increased accuracy in all calculations.

9. Addition of DOS shell and directory commands.

10. Improved user interface.

11. Addition of form feed option to print menus.

12. No longer necessary to choose targeted distance from table.

13. Automatic detection and support of math co-processor.

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 2

14. Calculation  of  recoil has been changed  to  reflect  more

recent findings.

15. Calculation  of  standard temperature and  pressure  for  a

given altitude.

16. Default function file to be loaded may now be specified  on

the command line.

17. Entering  out of range data now prompts the user with  cor-

rect input range.

G. The Future

BALLISTIC  isn't perfect (or even necessarily finished).  It

is  hoped, for instance, that future releases will incorporate  a

"Powley  Computer"  type computation for  estimating  the  powder

charge necessary for a given velocity. The ability to make calcu-

lations based on distances given in meters or yards would also be

a welcome improvement, as would an on-line database of  ballistic

data  relating to commercially loaded cartridges.  Several  users

have  asked  for the ability to display more  than  ten  velocity

intervals and the author is examining this possibility as well.

The  author is interested in adding new function  tables  if

reliable data can be found. He is currently on the lookout for  a

copy  of  BRITISH TEXTBOOK OF SMALLARMS or listings  of  the  "J"

tables. Also of interest is additional specific gravity data  for

commonly  used  bullet casting alloys. All things  will  come  to

those  users who wait (and support BALLISTIC with  the  voluntary

contribution!)

All in all, BALLISTIC seems likely to fill the needs of many

shooting buffs. Also included in the documentation  is a  section

listing  sources for further reading on the mathematics and  his-

tory of small arms ballistics. The local gun shop or public  lib-

rary  may have others. READ some of them!.. AND the  disclaimers.

Then enjoy the program.

H. Acknowledgements

The  author  would like to thank the  following  people  for

their help, support, and the many suggestions they made.  Without

dedicated  users  such  as these, the program  would  never  have

reached its current level of sophistication.

Bart Carfizzi

Jim Kreiser

James Roberts

Donald Terwilliger

Dave Zarodnansky

Special  thanks of course, are due to the author's wife  and

family,  Susan, Billy and Karen, for understanding (or  at  least

putting  up  with!) the author's wild mood  swings,  ranting  and

raving  as  various parts of BALLISTIC (which were  working  just

fine yesterday!) stopped working  for no particular reason. Thank

you.

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 3

2.  USING THE PROGRAM

The goal in writing BALLISTIC was to produce a program which

was at once versatile and relatively easy to use. While there  is

still room for improvement, some success in both of these  direc-

tions has been achieved. The program is primarily menu driven and

prompts  whenever input data is required. Additionally, data  en-

tered  (or calculated) in one section of the program need not  be

re-entered  unless the user wants to change the value.  Data  the

program  knows about are displayed during the prompts. To  accept

the displayed value press <RETURN>. To change it, simply type the

new  data over the old, then press <RETURN>. (Table  titles  must

ALWAYS  be  entered explicitly if they are desired.)  To  correct

typing  errors on the current line, use the <BACKSPACE> key,  NOT

the <DELETE> key. Improper entries on previous lines of the  same

menu can be corrected by using the <ESC> key to "back up" to  the

bad  line, then retyping the data. In most cases,  obviously  im-

proper  data will be refused and input filenames will be  checked

for existence before they are accepted.

For  now, an example is perhaps the best way  to  demonstrate

the use of BALLISTIC. (The figures referred to in the text follow

immediately after section 5.)

A. Running the Program for the First Time

Prior  to  running BALLISTIC for the first  time,  the  file

containing  the  function table required by the program  must  be

generated  using GENTABLE (v3.00 or above, see Section  5).  This

file  will  contain the version number of the  GENTABLE  used  to

create it, the function name, some atmospheric data, the  maximum

and minimum velocity and S and T value pairs. Once this file  has

been  created (and optionally checked using READTABLE)  BALLISTIC

can be invoked.

B. Command Line Arguments

BALLISTIC  is  started by simply typing its filename  on  the

command line, optionally followed by one or both of the following

arguments.  "-BIOS" will cause all video output to be done  using

BIOS calls rather than direct screen writes. The second argument,

"-f"  followed (with NO spaces) by the name of a BALLISTIC  func-

tion file, causes the named function file to be loaded instead of

the default file BALISTIC.TAB.

Only  BALISTIC.EXE and a function file need actually be  pre-

sent  on the disk when running the program. BALLISTIC will  first

attempt  to  read  the configuration file  (BALISTIC.CFG,  if  it

exists),  then  search  for the  default  function  file  (either

BALISTIC.TAB  or a user specified file), prompting the  user  for

the name of an alternate if it's not found. The default file type

is ".TAB" which need not be entered. (To enter a filename with no

extension enter "filename." with the trailing period.)  BALLISTIC

will  now read the function file, make a check to  determine  its

validity (a simple checksum procedure), and present the user with

the main menu. (Fig. 1)

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 4

C. Calculating a Ballistic Coefficient

The first step in developing a ballistic table is to  deter-

mine  the  ballistic coefficient. If this value cannot  be  found

explicitly  in one of the sources described in section  3B,  main

menu  choice  2 may be used to calculate it. Press  <2>  and  the

program will prompt for an initial and final velocity, the  range

and some atmospheric data. After this has been entered, the  cal-

culated value of the effective ballistic coefficient (as well  as

the  coefficient at standard conditions for the function  chosen)

will be displayed on the screen (Fig. 2). Having found the  coef-

ficient,  BALLISTIC  can  now  calculate  the  table.  Press  the

<RETURN> key to redisplay the main menu then <1> to bring up  the

input section.

D. The Ballistic Table

Figure  3 shows the input data for main menu choice 1,  cal-

culating  the ballistic table. Since the initial velocity,  range

and  ballistic coefficient have been entered or calculated  on  a

prior  screen, these values needn't be re-entered. In this  case,

entering  a range value of 500 will create a table to  500  yards

based  on  the performance noted over the first 100.  Entering  a

value  of zero for the range will cause the program to switch  to

metallic silhouette mode. The user will then be prompted for  his

choice  of standard silhouette rifle or pistol ranges rounded  to

the  nearest yard. As long as the input value for  range  remains

zero these alternate ranges will be used.

Figure 4 shows the table itself and the program's prompt for

generation  of a sight table. At this point, should it be  desir-

able to get a printed copy, press <1>. The program will  redirect

ballistic table output to the system's LST: device and return  to

the  prompt. A form feed (page eject) can be sent to the  printer

by pressing <2>. In both cases, the user will be informed if  the

printer isn't ready and may abort output with the <ESC> key.

Sight (trajectory) tables can be produced by pushing <3>  in

response to the table prompt. The program will require that  dis-

tance  for which the weapon is "sighted in" (this value need  NOT

have  appeared in the range column of the original ballistic  ta-

ble)  and the height of the front sight above the  centerline  of

the  bore. In silhouette mode, entering a value of zero  for  the

targeted  range will cause the prompt to take the form  of  which

silhouette  (chicken,  javelina,  turkey or ram)  the  weapon  is

zeroed for. The user should respond with the first letter of  the

appropriate  target.  Output  of this table can  be  diverted  in

exactly the same manner as the ballistic table.

Pressing <4> at the sight table prompt will present the user

with a graphic representation of the bullet's trajectory on  sys-

tems  with an appropriate graphics card. Range, in yards will  be

indicated along the bottom, with tic marks appearing at distances

corresponding  to  the entries in the ballistic table.  A  dotted

vertical line will appear at the distance for which the weapon is

zeroed  and  a horizontal dotted line will indicate the  line  of

sight.  The  trajectory  (in inches above or below  the  line  of

sight)  is indicated on the left scale. Hardcopy of the graph  is

currently  available only through the <PRINT SCREEN>  key.  (This

may require first running the program GRAPHICS.COM, usually found

on the DOS system diskette. Not all printers or graphics adapters

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 5

will be supported.) Pressing any key will return the user to  the

sight table prompt.

From  the sight table press <5> (or <ESC>) to return to  the

ballistic  table menu (Figure 4 again) then a <4> (or  <ESC>)  to

return to the main menu.

E. Modifying the Ballistic Coefficient

Those  living at high altitude or in areas of extreme  temp-

erature  will want to adjust published (sea level) values of  the

ballistic  coefficient to more accurate local values.  Main  menu

choice  3 (figure 5) may be used to accomplish this. The  program

will  need to know the altitude, barometric pressure and  temper-

ature  in addition to the value of the ballistic  coefficient  at

STP  (Standard  Temperature and Pressure, the  published  value).

Bear in mind that these meteorological values (especially temper-

ature)  ALSO  affect the initial velocity of a given  load.  This

effect  is  beyond the ability of this program to  calculate  but

descriptions of it may be found in many reloading manuals.

F. Changing the Cross Wind Value

Menu choice 3 also allows for adjustment of the speed of the

cross  wind  used to calculate wind deflection. By  default,  the

program  sets this value to 10 miles per hour,  blowing  directly

across  the  path of the bullet (0 degrees). It should  be  noted

that  deflection  is a linear function, i.e., a 5 mph  wind  will

deflect a bullet by exactly half the amount of a 10 mph wind.

G. Changing the Input Function Table

Starting  with version 2.00, it is possible to switch  input

function tables without restarting the program. Pressing <4> from

the  main menu will cause a prompt for the new file name.   If  a

filename  containing the wildcard character "*" is  entered,  the

directory  of all matching filenames will be displayed. As  noted

in section A, the default table type is ".TAB" which need not  be

entered.  Simply press <RETURN> to abort and return to  the  main

menu.

This  option doesn't affect the fact that  BALISTIC.TAB  (or

the filename supplied with the "-f" command line option) will  be

searched  for (and loaded if present) when the program  is  first

started.  It  is provided only for users who routinely  use  more

than one function table. Even so, program startup will be simpli-

fied if the table most commonly used is copied to BALISTIC.TAB.

It  should  be noted here that loading a new  function  file

will  not reset the temperature and pressure to the defaults  for

the  new table. Thus, some care should be taken if switching  be-

tween two tables that use different standard conditions.

H. Miscellaneous Functions Menu

Pressing  <5> at the main menu prompt will present the  user

with  the miscellaneous functions menu. From this menu  the  user

may  elect to calculate recoil, the minimum necessary  twist  for

bullet stabilization, or the point blank range for a given target

size.

I. Calculating Recoil

The recoil energy and velocity of a weapon can be calculated

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 6

using  miscellaneous menu choice 1, provided its weight  and  the

weight of the powder charge are known (Figure 6). This number can

be deceptive as the perception of recoil by different shooters is

VERY subjective. In addition to the physics involved, there  must

be some consideration given to the fit of the gun to the individ-

ual,  shape  and  material  of the  stock  or  grips,  etc.  Many

shooters,  for  instance, fear the recoil of a 12  gauge  shotgun

even with target loads while thinking nothing of using a  6 pound

30-06 despite the fact that the "calculated" recoil can be  quite

similar.  Such  recoil is often in the eye (or shoulder)  of  the

beholder.

This  section also calculates whether a "muzzle brake"  type

device  will  be useful in reducing felt recoil by  allowing  the

user  to determine the rough percentage of recoil caused  by  the

bullet  itself  (which a muzzle brake cannot affect).  The  value

indicated is the level to which recoil energy would be reduced if

there  were no powder at all. This is a crude estimate of what  a

shooter  might achieve with a perfect brake. (Bear in mind,  how-

ever,  that  NO  muzzle brake is 100% effective  and  the  actual

amount of reduction will be less.)

The  recoil  calculation used by BALLISTIC is a  very  rough

approximation. It would be extremely challenging to write a  com-

pletely accurate description of the recoil process. It would also

be  very  difficult  to use as many variables  are  involved  in-

cluding, but not limited to bore cross sectional area, length  of

barrel,  atmospheric  pressure and temperature, type  of  powder,

loading  density, etc. The values calculated here should  not  be

considered  accurate to more than 10-15%. Barrel/powder  combina-

tions that produce excessive muzzle flash will be even less accu-

rate.

J. Calculating Minimum Twist

The slowest twist that will stabilize a bullet in flight may

be calculated using miscellaneous menu choice 2. The formula used

here is the approximation credited to Sir Alfred George Greenhill

in 1879. Necessary input data includes the bullet's  diameter and

length  in  inches and specific gravity. The  default  value  for

specific  gravity  is the average density of a  typical  jacketed

bullet.  Also listed on the input screen (for reference)  is  the

specific gravity of pure lead. Specific gravities (or  densities)

of  other  bullet  alloys seem difficult to find.  It  should  be

pointed out that these specific gravities can NOT be added algeb-

raically. That is, an alloy containing 50% lead and 50% tin  will

NOT  have a specific gravity that is the average of the two  pure

metals.  Appendix  D of this document contains  specific  gravity

data for a small selection of common alloys.

K. Calculating the Point Blank Range

Given the velocity, sight height and ballistic  coefficient,

a  point blank range may be determined for any target  size.  The

point blank range is the maximum range for which a bullet can  be

counted upon to strike a target by simply aiming the weapon  dir-

ectly  at it. For example, given a 6 inch target, a  point  blank

range may be calculated so that the bullet is always less than  6

inches  from  the line of sight. BALLISTIC  will  calculate  this

distance for a given target size and tell the user for what range

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 7

the weapon should be sighted and what trajectory the bullet  will

have at 100 yards. (If the targeted range is less than 100 yards,

the  trajectory at 25 yards will be given instead.) This 100  (or

25)  yard  trajectory may be used to sight the  weapon  when  the

targeted range is either unrealistically far away, or an  unusual

number  of yards. For instance, in the example  screen,  sighting

the weapon 5.0 inches high at 100 yards will allow the shooter to

hit  a 6 inch target out to 335 yards, without allowing  for  any

holdover. However, the bullet will only strike the CENTER of  the

target at 287 yards.

L. DOS Shell (Escape to DOS)

Users operating under MS/PC-DOS version 3.00 or higher  have

the option of shelling out to DOS from either the main or miscel-

laneous menus. The only other requirement for this option is that

there  be  a copy of COMMAND.COM in the place  indicated  by  the

COMSPEC path variable.

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 8

3.  THE CONFIGURATION FILE

Version  3.01  of BALLISTIC will allow certain  program  de-

faults  to be overridden with the aid of the  configuration  file

BALISTIC.CFG.  This option was originally provided for  users  in

the CP/M environment.

A. Using the configuration file

The  configuration file is an ascii text file each  line  of

which  contains  the  name of the parameter to  be  changed,  ONE

space, and the new value. Parameters listed as "must be  integer"

may  NOT  contain a decimal point. Certain other  parameters  may

only accept ON and OFF (or similar string choices) as values.  Be

careful not to include leading or trailing blanks in ANY values.

The following file would turn off the bell, set the  default

altitude to 2000 feet and set the video write mode to BIOS:

BELL OFF

Altitude 2000

video bios

B. Allowable parameters

The  following parameters are allowable for all  implementa-

tions of BALLISTIC v3.01 and above:

ALTITUDE - The  altitude to be used in the ballistic  table

and  ballistic coefficient calculations. May  be  REAL.

The default value is zero.

BELL - Controls  whether the console bell is rung  for  I/O

errors detected by the program. Allowable values are ON

and OFF. The default is BELL ON.

BKG - Determines  the background text screen color.  Allow-

able  values are 0 through 7. Must be INTEGER. The  de-

fault value is 0. This parameter is illegal for systems

having only a monochrome text/graphics card.  Following

are the colors corresponding to each value:

0 - Black         4 - Red

1 - Blue          5 - Magenta

2 - Green         6 - Brown

3 - Cyan          7 - Lt. Gray

COLOR - The  text color. Default is 7. See BKG for  a  des-

cription of allowable values and restrictions.

PRESSURE - The atmospheric pressure used for the  ballistic

table  and ballistic coefficient calculations.  May  be

REAL.  The default value is dependent on  the  function

file chosen.

PRINTER - Line  width in characters of the system  printer.

Must be an INTEGER value from 80 to 200. Default is 80.

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 9

SCREEN - The  output screen width. This is for  future  re-

visions of BALLISTIC. Currently, only 80 is  supported.

Must be an INTEGER value.

TEMPERATURE - The  temperature  to be  used  for  ballistic

table  and ballistic coefficient calculations.  May  be

REAL.  The default value is dependent on  the  function

file chosen.

VIDEO - Determines the method used to write to the  screen.

Allowable  values are BIOS and DIRECT. BIOS causes  all

output to be done through the use of system BIOS calls.

This mode may be useful for computers/graphics adapters

that are not 100% compatible with the IBM PC family. It

does,  however,  significantly slow output.  Using  the

DIRECT  option causes all screen output to  be  written

directly to video memory. Default is DIRECT.

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 10

4.  SOURCES OF DATA

Much  of  the data necessary for ballistic  calculations  is

readily available. Even the ballistic coefficient can usually  be

calculated from the information at hand. Following are  suggested

sources for specific input data to the program.

A. Altitude

While the calculation isn't particularly sensitive to  alti-

tude, if the calculations will be routinely used in an area  well

above sea level the corrected numbers are more accurate. The best

suggestion  is to take a guess at the altitude then run the  pro-

gram both with and without correcting. If the difference will  be

noticeable over the anticipated firing range, look up the  actual

altitude  in an almanac or one of the U.S. Department of the  In-

terior  "Quadrangle  Maps" available at many  sporting  goods  or

camping stores. Either jot it down near the computer so it can be

entered  when the program is run or add it to  the  configuration

file as described in section 3.

B. Ballistic Coefficient

This will probably be the most difficult number to find.  In

many cases, those who reload will be able to get this value  from

the  manufacturer of their bullets. (Speer, for instance,  prints

this  data in their reloading manual for all of  their  bullets.)

The  value for other bullets may sometimes be estimated  by  com-

paring bullet shapes, provided the bullet is of the same caliber.

Comparing the bullet in question to a specially prepared chart of

shapes  is  often more accurate. The DuPont Company at  one  time

produced  a  series of these and other useful charts,  called  "A

Short  Cut  To  Ballistics". The complete set  is  supplied  with

Ackley's handbook and the specific chart for estimating ballistic

coefficients is reprinted in HATCHER'S NOTEBOOK.

The program itself will calculate the ballistic  coefficient

in  certain cases. Both the initial and remaining velocity  at  a

given  range  must be known. This data is available  for  factory

loaded  ammunition in manufacturers' literature and sources  such

as  SHOOTERS BIBLE. (A short section of coefficients for  several

factory loadings calculated using this program and Ingalls' table

may be found in Appendix C.)

C. Bullet Weight, Length, and Caliber

These are easy. On factory-loaded ammunition OR  components,

bullet weight (in grains) and caliber will be written on the box.

Those who cast their own bullets will need to weigh several  sam-

ples  prepared  with the actual alloy used. Take the  average  of

half  a dozen or so and use that value. A  sufficiently  accurate

value for bullet length may be found with a ruler.

D. Gun, Powder Weights

These two are needed only to calculate the amount of  recoil

generated  by a given load. The weight of a rifle can usually  be

found accurately enough on a bathroom-type scale. Actual  weights

of  most  firearms can be found in manufacturers'  literature  as

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 11

well as in such sources as SHOOTER'S BIBLE, GUN DIGEST, and  pro-

duct reviews in many magazines. Don't forget to add in the weight

of scopes, slings, custom stocks or any other accessories.

Powder weights will be known if performing calculations  for

handloads. If using factory loads, try estimating by comparing to

available  reloading  data. Find a load that moves  a  bullet  of

similar  construction  (jacketed, soft lead, etc.) to  about  the

same muzzle velocity.

E. Specific Gravity

This  value  is only required for  calculating  the  minimum

necessary  twist  for stabilization. Two more  or  less  standard

values are 11.34 for pure lead, and 10.90 for a typical  jacketed

bullet.  Other values are very difficult to find in  the  litera-

ture.  Hatcher,  describes a fairly simple way  to  measure  this

quantity,  but the two values above should be sufficiently  accu-

rate for most practical purposes.

Appendix D of this document contains a short compilation  of

specific  gravity data for some common Lead/Tin/Antimony  alloys.

These were taken primarily from volume one of the American  Soci-

ety of Metals METALS HANDBOOK.

F. Temperature, Atmospheric Pressure, Wind Speed and Direction

Suitably accurate values for temperature and pressure can be

obtained  from a weather report or newspaper. As  with  altitude,

the  calculation  isn't terribly sensitive to these  factors,  so

unless  the range is in the Sahara Desert or Antarctica they  can

probably be ignored. (It CAN be interesting to set up a ballistic

table and vary them just to see what happens... Experiment!)  The

same  suggestion applies; try the program with and  without  cor-

recting and see if the difference is big enough to  realistically

notice.  Bear  in mind that this program examines the  effect  of

these  variables only on the ballistic coefficient.  Temperature,

for instance, will also change the initial velocity of a load,  a

factor  beyond the ability of this program to calculate.  (Tables

describing  this effect can be found in both the Speer  reloading

manual and Ackley's handbook.) Temperature and pressure are  also

valid parameters for use in the configuration file. (Section 3)

Wind direction can only be estimated at the actual  shooting

site.  Zero degrees indicates a wind blowing across the  bullet's

path to either the right or left. Ninety degrees indicates a wind

blowing  either  directly behind or directly  into  the  bullet's

path. As this value is only used for calculating wind deflection,

a signed value isn't necessary. Also, since deflection is  linear

with  wind  speed, leaving the value set for the default  10  mph

allows for easy interpolation at the range.

G. Velocities

Muzzle velocities of most factory ammunition can be found in

periodicals  such  as the SHOOTER'S BIBLE. Those who  reload  are

aware  that estimated or reported velocities are  usually  listed

with the loading data for a particular cartridge.

The most effective way to find the velocity for a completely

unknown  load is, of course, the chronograph. Unfortunately,  few

people  have access to one and their price puts them  beyond  the

reach of the casual buyer. (The author once built an  inexpensive

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 12

attachment  that allowed a Commodore-64 home computer to be  used

as  a simple chronograph, but its usefulness was limited since  a

source of house current was still needed nearby... also, the C-64

isn't exactly portable. The technique SHOULD be adaptable to many

other home computers and the author would be happy to discuss the

project with registered users.) Ackley, in his book HANDBOOK  FOR

SHOOTERS  AND RELOADERS, describes a simple "ballistic  pendulum"

that  can be used to estimate projectile velocities. No  external

power is necessary; it relies on the kinetic energy of the bullet

to swing a heavy mass.

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 13

5.  USING GENTABLE AND READTABLE

A. Using the pre-programmed functions

Before  running  BALLISTIC for the first time  a  file  con-

taining  the chosen set of function values the program  will  use

during  the calculations must be produced. GENTABLE Version  3.00

makes the following functions available:

FUNCTION           VELOCITY         ATMOSPHERIC

max.   min.     temp.  pressure

British 1909          4000    0        59      29.53

"G1"                  4094   300       59      29.53

Ingalls'              3600    0        60      30.00

Ingalls' (extended)   4094    0        60      30.00

The  British  1909 and Ingalls'  functions  are  practically

identical until low velocities are reached. Both are well  suited

to  calculations  involving typical hunting  or  pistol  bullets,

i.e.,  rounded or slightly pointed (roughly two  caliber  radius)

spitzers. Results for sharply pointed spitzers and boattails will

not  be  as accurate with these functions as they might  be  with

some others. Less accurate results will also be obtained if  bal-

listic coefficients calculated with one set of functions are used

with  another, i.e., using the coefficients listed in Appendix  C

(calculated with Ingalls' table) with the British 1909 tables.

The  extended  Ingalls' table is simply the  standard  table

continued to the maximum allowable program velocity following the

method  used in Hatcher. As values above 3600  are  extrapolated,

they may not be as accurate as those below.

After  starting GENTABLE, the user will be presented with  a

short  menu  to which he should respond with the  number  of  the

function he desires. The program will then prompt the user for  a

filename to contain the new data. Finally the function table will

be  created, overwriting any previous file with that name.  Users

that  plan to routinely use more than one table  should  probably

create  them both ahead of time, renaming them to something  more

descriptive.  Either can then be loaded later on through  use  of

BALLISTIC  main  menu choice 4. Copy the one used most  often  to

BALISTIC.TAB.

GENTABLE will accept only one argument on the command  line.

If "-BIOS" is used when invoking GENTABLE, all video output  will

be  sent  to the screen via BIOS calls. The default is  to  write

directly to screen memory.

B. Creating custom function tables

The  user is not limited to using the functions produced  by

GENTABLE but may produce his own files according to the following

format:

1.  Truncated  revision number for version of  GENTABLE  used

multiplied by -1. (int)

2.  The table name. (char[32])

3.  Standard temperature for table data. (double)

4.  Standard pressure for table data. (double)

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 14

5.  Maximum velocity in table (<=4094). (unsigned)

6.  Minimum velocity in table. (unsigned)

7.  S_table, T_table pairs as described in HATCHER'S NOTEBOOK

for velocities (max >= v >= 0). (float)

8.  Checksum  calculated  as the sum of (S_table  -  T_table)

values. (float)

C. Checking the function file

After  creating  the  function file  it  may  be  optionally

checked  with  READTABLE. READTABLE was originally written  as  a

debugging aid and is included only to satisfy the users curiosity

as  to what is contained in a ballistic function table. It  might

also be of some use to those contemplating creation of their  own

function files. Once started, it prompts the user for the name of

the  function file to be examined, displays the  header  informa-

tion,  then  prints the values to the screen. The  speed  of  the

display  may be altered by pressing either the <-> or  <+>  keys.

Pressing  any  other key will cause the display to pause  at  the

next  value.  Once  paused, pressing <ESC>  causes  execution  to

abort. Any other key will cause the program to resume.

As  with  GENTABLE, READTABLE will accept the  command  line

argument  "-BIOS"  to cause it to perform all writes  using  BIOS

calls. This will significantly slow output.

For those systems that don't support the IBM graphics  char-

acter set, a provision has been made to remove the graphics char-

acters from READTABLE and replace them with standard ASCII  char-

acters. Simply put both BAL-READ.EXE and BALREMOV.COM in the same

directory and run BALREMOV. A backup of the existing copy of BAL-

READ.EXE will be created (BAL-READ.BAK) and a new version will be

created without the offending characters. BALREMOV is quite  sim-

ple minded, however. NEVER use a version of BALREMOV that  didn't

come with the version of BAL-READ to be altered.

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 15

--==<  B A L L I S T I C  v3.01  >==--

(C) Copyright 1988 by W.R. Frenchu

- MAIN MENU -

1 ................... Calculate Ballistic Table.

2 ............. Calculate Ballistic Coefficient.

3 ............... Adjust Atmospheric Conditions.

4 ................... Select New Function Table.

5 ..................... Miscellaneous Functions.

6 ................................... DOS Shell.

7 ............................ Return to System.

Your Choice ? 1

---------------------( Fig. 1 - MAIN MENU )----------------------

--==<  B A L L I S T I C  v3.01  >==--

(C) Copyright 1988 by W.R. Frenchu

- CALCULATE BALLISTIC COEFFICIENT -

Please input:

Initial velocity ....(fps)  2820

Final velocity ......(fps)  2507

Range .............(yards)  100

Altitude ............(Ft.)  0

Temperature ......(Deg. F)  59.0

Atmos. Pressure ..(In. Hg)  29.53

Effective Bal. Coeff. =  0.288

(Bal. Coeff. at STP = 0.288

-------( Fig 2. - Calculating the Ballistic Coefficient )-------

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 16

--==<  B A L L I S T I C  v3.01  >==--

(C) Copyright 1988 by W.R. Frenchu

- CALCULATE BALLISTIC TABLE -

Please input:

Initial velocity ........(fps)  2820

Bullet weight ........(grains)  165

Effective Bal. Coeff. ........  0.288

Number of intervals .(10 max.)  10

Range .................(yards)  500

Chart title ?

.308 Winchester - 165 gr. Spire pt.

-----------------( Fig. 3 - MAIN MENU choice 1 )-----------------

.308 Winchester - 165 gr. Spire pt.

(calculated using G1 table)

Bullet weight ......... 165 grains  IPSC Power Factor ..... 465.3

Cross Wind ............ 10.0 m.p.h. Altitude .............. 0    Ft.

Atmospheric Pressure .. 29.53 In.   Temperature ........... 59.0 F

Effective Bal. Coeff... 0.288       Bal. Coeff. at STP .... 0.288

Range   Velocity  Energy  Momentum   Max. Ht.  Defl.   Drop    Time

Yards    f.p.s.   ft-lbs  lb.-secs     in.      in.     in.    secs

0      2820    2913.2   2.0661      0.0      0.0     0.0    0.000

50      2661    2593.5   1.9495      0.1      0.3     0.6    0.055

100      2507    2302.4   1.8368      0.6      1.1     2.4    0.113

150      2358    2037.4   1.7278      1.5      2.6     5.5    0.175

200      2215    1797.1   1.6228      2.8      4.8    10.3    0.240

250      2076    1579.4   1.5213      4.7      7.8    16.8    0.310

300      1942    1382.2   1.4231      7.2     11.6    25.3    0.385

350      1814    1205.8   1.3292     10.5     16.3    36.2    0.465

400      1693    1050.3   1.2406     14.7     22.0    49.7    0.550

450      1578     912.5   1.1563     20.0     28.8    66.2    0.642

500      1470     791.2   1.0767     26.7     36.7    86.2    0.741

1=Print  2=Form Feed  3=Sight table  4=Main menu  ? 4

------------( Fig. 4 - The completed ballistic table )-----------

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 17

--==<  B A L L I S T I C  v3.01  >==--

(C) Copyright 1988 by W.R. Frenchu

- ADJUST ATMOSPHERIC CONDITIONS -

(Modify ballistic coefficient)

Please input:

Bal. Coeff. at STP ........ 0.288

Altitude .............(Ft.) 5000

Temperature .......(Deg. F) 41.2

Atmos. Pressure ...(In. Hg) 25.44

Wind Speed ........(M.P.H.) 10.0

Angle from Broadside (deg.) 0

Effective Ballistic Coefficient = 0.322

--------( Fig. 5 - Modifying the ballistic coefficient )--------

--==<  B A L L I S T I C  v3.01  >==--

(C) Copyright 1988 by W.R. Frenchu

- CALCULATE RECOIL -

Please input:

Initial velocity .....(fps) 2820

Weight of gun ........(lbs) 9.50

Weight of bullet ..(grains) 165

Weight of powder ..(grains) 42.0

Recoil Velocity =  9.5 fps

Recoil Energy   = 13.4 ft-lbs

(Recoil due to bullet =  7.2 ft-lbs or 54%)

-----------------( Fig. 6 - Recoil calculation )----------------

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 18

--==<  B A L L I S T I C  v3.01  >==--

(C) Copyright 1988 by W.R. Frenchu

- CALCULATE POINT BLANK RANGE -

Please input:

Initial Velocity ............(fps)  2820

Sight Height .............(inches)  0.90

Max. Dist. from Sight Line ..(in.)  6.00

Effective Bal. Coeff. ............  0.288

Targeted range for +/-6.00 inch path is 287 yds.

Point-blank range is 335 yds.

(100 yd. trajectory = +5.0 in.)

-----------( Fig. 7 - Calculating Point Blank Range )-----------

--==<  B A L L I S T I C  v3.01  >==--

(C) Copyright 1988 by W.R. Frenchu

- CALCULATE TWIST -

Please input:

Bullet Diameter ......(in.)  0.308

Bullet Length ........(in.)  1.20

Spec. Gravity of Bullet ...  10.90

Slowest twist allowing for stabilization is

1 turn in 11.9 inches.

------------------( Fig. 8 - Calculating twist )----------------

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 19

Appendix A

Glossary

BALLISTICS - The  study of moving projectiles. Commonly, what  is

meant is "exterior" ballistics, that portion of the bullet's

flight  between  muzzle and  target.  "Interior"  ballistics

refers  to  the  portion between primer  ignition  and  free

flight  and "terminal" ballistics to what happens  when  the

bullet strikes its target.

BALLISTIC COEFFICIENT - The  ratio of the sectional density of  a

projectile to its coefficient of form. A measure of how well

a  bullet  retains its velocity. Commonly  supplied  by  re-

loading manufacturers, or it may be derived from their  bal-

listic tables.

BULLET - The actual projectile in small arms ammunition. Commonly

corrupted to mean the entire cartridge.

BULLET WEIGHT - Weight of the actual projectile. Usually given in

grains.

CALIBER - Technically, the diameter of the barrel measured across

the lands. In practice, the diameter of the bullet.

CARTRIDGE - Fixed ammunition, including case, powder, primer, and

bullet, for small arms.

COEFFICIENT OF FORM - A number relating the ballistic  efficiency

of  a  given shape to the shape of the  projectile  used  to

calculate the ballistic table.

CROSS WIND ANGLE - The  angle  between the wind and line  of  de-

parture.

DRIFT - Technically,  the distance a bullet will travel  horizon-

tally  due to its spin. This effect is usually quite  small,

amounting to about 7 inches at 1000 yards for a military  M2

(30-06) cartridge. Often confused with wind deflection.

DEFLECTION - See wind deflection.

DROP - The  distance a bullet will fall due to the  influence  of

gravity.  Measured from the line of departure, not the  line

of sight.

ENERGY - Here,  the  kinetic energy of a moving  bullet,  usually

given  in ft-lbs. Equal to one half the mass of  the  bullet

multiplied by the square of the velocity.

F.P.S. - Feet per second. A unit of velocity.

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 20

FT.-LBS. - Foot  pounds.  A unit of energy equal  to  the  effort

required to raise one pound to a height of one foot.

GRAINS - A unit of weight equal to 1/7000 of a pound.

INGALLS' TABLES - A  set of ballistic tables first calculated  by

Col.  J. M. Ingalls in 1918. Probably the most  widely  used

tables  for small arms calculations. Other  tables  commonly

used include the British tables of 1909, 1929,  Winchester's

G tables and the Ordnance Department's J table.

I.H.M.S.A. - International  Handgun  Metallic  Silhouette  Assoc-

iation.

I.P.S.C - International Practical Shooting Confederation.

LINE OF DEPARTURE - An  imaginary  line formed by  extending  the

muzzle  of the weapon. It coincides with the  bullet's  path

only while the bullet is actually in the barrel.

LINE OF SIGHT - An  imaginary line from the center of the  sights

to the point of aim. The bullet typically crosses this  line

only twice; once, a few yards from the muzzle, and again  at

the targeted distance.

MAXIMUM HEIGHT - The highest vertical distance above the line  of

sight attained by a bullet. Sometimes confused with the mid-

range  trajectory due to the fact that it  typically  occurs

about  halfway between muzzle and target. Maximum height  is

usually somewhat higher and later than the MRT.

M.O.A. - Minute of Angle. Equal to 1/60 of an angular degree.  At

100 yards, 1 M.O.A. is approximately 1.047 inches.

MOMENTUM - Equal to the mass of a bullet multiplied by its  velo-

city.  Indicative  of the striking or knockdown power  of  a

projectile.

N.R.A - National  Rifle  Association. If you  shoot,  you  SHOULD

belong to this organization.

POINT-BLANK  RANGE - That  range  for  which  the  bullet's  path

doesn't  vary  from the line of sight by more than  a  given

amount.

POWER FACTOR - A  number  used in IPSC competition  to  determine

whether  a handgun cartridge may be scored as a  "major"  or

"minor"  caliber. Defined as the bullet weight  (in  grains)

divided  by 1000 and multiplied by the velocity.  Cartridges

with  a power factor equal to or greater than 175  are  con-

sidered "major". Cartridges ranking between 125 and 175  are

"minor"  and those below 125 are not legal for  IPSC  compe-

tition.

S.A.A.M.I - Sporting Arms Ammunition Manufacturers Institute.

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 21

SECTIONAL DENSITY - The  weight of a projectile (in  pounds)  di-

vided by the square of its diameter (in inches).

SIGHT HEIGHT - The  measured distance between the centerlines  of

the sights and muzzle of a gun.

SPECIFIC GRAVITY - The ratio of the mass of a material to that of

an equal volume of water.

TARGETED RANGE - That  distance  where  the path  of  the  bullet

crosses the line of sight. (The "sighting in" distance.)

TIME OF FLIGHT - The  time necessary for a bullet to travel  from

the muzzle of a gun to a given range.

TRAJECTORY - The path of a moving projectile. Here, the number of

inches above or below the line of sight.

TWIST - Here,  the rate of twist in the rifling of a gun  barrel.

The minimum amount of twist necessary to stabilize a  bullet

in  flight can be determined from its length,  density,  and

caliber. Usually expressed as "1 turn in xx inches" where xx

may range from about 8 inches up to over 30. "Faster"  twist

translates  to  a  smaller  number.  Minimum  twist  is  the

"slowest" twist that will stabilize a given bullet.

WIND DEFLECTION - The amount of horizontal motion attributable to

the action of the wind. Often mistakenly called drift.

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 22

Appendix B

Information Sources

BALLISTICS:

Any intermediate physics (mechanics) text.

BALLISTICS ON THE HOME COMPUTER. American Rifleman, June

1983

BALLISTICS ON YOUR POCKET CALCULATOR. American Rifleman,

June 1987

BALLISTIC TABLES AND HOW TO USE THEM. American Rifleman,

December 1963

EXPLORING BALLISTICS WITH YOUR COMPUTER, Byte Magazine

September 1980

EXTERIOR BALLISTICS, McShane, Kelly and Reno, University of

Denver Press, 1953

HATCHER'S NOTEBOOK, Julian S. Hatcher, Harrisburg: Stackpole

Books, 1962

TRAJECTORY COMPUTATION SYSTEM FOR DIGITAL COMPUTER, SAAMI

publications, 1976

RELOADING:

CARTRIDGES OF THE WORLD, J.T. Amber, Northfield: Digest

Books, 1972

COMPLETE GUIDE TO HANDLOADING, Philip B. Sharpe, New York:

Funk & Wagnells Co., 1953

HORNADY HANDBOOK OF CARTRIDGE RELOADING, RIFLE-PISTOL, Grand

Island: Hornady Manufacturing Co., 1973

HANDBOOK FOR SHOOTERS AND RELOADERS, VOL I & II, P.O. Ackley,

Salt Lake City: Publisher's Press, 1965

LYMAN RELOADING HANDBOOK #46, Middlefield: Lyman Products

for Shooters, 1970

NRA HANDLOADER'S GUIDE, Wash. D.C.: The National Rifle

Association of America, 1969

PRINCIPLES AND PRACTICE OF HANDLOADING, Georgetown: Small

Arms Technical Publishing Co., 1954

SIERRA BULLETS RELOADING MANUAL, Santa Fe Springs: Sierra

Bullets, 1971

SPEER RELOADING MANUAL #10, Lewiston: Speer, Inc., 1979

ASSOCIATIONS:

THE NATIONAL RIFLE ASSOCIATION - 1600 Rhode Island Ave. NW,

Washington, D.C. 20036

S. A. A. M. I. (SPORTING ARMS AMMUNITION MANUFACTURERS

INSTITUTE) - PO Box 838, Branford, Conn. 06405

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 23

Appendix C

Ballistic Coefficients for Factory Bullets

The  coefficients  listed here were calculated  from  tables

found  in  SHOOTER'S BIBLE No. 75 (1984) using  the  program  and

Ingalls'  function.  No attempt was made to include  every  load,

instead, representative loads for each caliber were chosen.

Part 1 - Centerfire Pistol and Revolver

Cartridge      Bullet       Coefficient    Manufacturer

================================================================

.25 ACP        50 FMC          .174            F

.25 ACP        50 FMC          .097            R,W

.380 ACP       85 STHP         .105            W

.380 ACP       90 JHP          .073            F

.380 ACP       95 FMC          .082            F,R,W

9 mm          95 JHP          .101            F,R,W

9 mm         115 FMC          .128            W

9 mm         115 JHP          .131            R,F

9 mm         115 STHP         .131            W

9 mm         123 FMC          .140            F

.38 SPEC.     110 STHP         .147            W

.38 SPEC.     148 WC           .062            F,R,W

.38 SPEC.     158 LRN          .162            F,R,W

.357 MAG.     110 JHP          .093            F,R,W

.357 MAG.     125 JHP          .122            F,R,W

.357 MAG.     158 JSP          .135            F,R,W

.44 MAG.      180 JHP          .120            F,R

.44 MAG.      210 STHP         .125            W

.44 MAG.      220 MCP          .193            F

.44 MAG.      240 JHP          .156            R

.44 MAG.      240 LGC          .139            R

.45 ACP       185 MCWC         .082            R,W

.45 ACP       185 JHP          .152            F

.45 ACP       185 STHP         .137            W

.45 ACP       230 FMC          .165            R,W

.45 COLT      225 STHP         .167            W

.45 COLT      225 SWCHP        .172            F

.45 COLT      250 LRN          .157            R

---------------

ABBREVIATIONS

Manufacturers: F=Federal, R=Remington, W=Winchester

Bullets:  FMC=Full  Metal  Case, STHP=Silver  Tip  Hollow  Point,

JHP=Jacketed  Hollow Point, MCWC=Metal Case Wadcutter,  WC=Wad-

cutter,  LRN=Lead Round Nose, LGC=Lead Gas Check,  JSP=Jacketed

Soft Point, MCP=Metal Case Profile, SWCHP=Semiwadcutter  Hollow

Point

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 24

Part 2 - Centerfire Rifle

Cartridge      Bullet       Coefficient    Manufacturer

=================================================================

.22-250 REM.     55 PSP          .231            F,W

.22-250 REM.     55 PSP          .253            R

.223 REM.        55 FMC          .207            R

.223 REM.        55 FMC          .278            W

.223 REM.        55 MCBT         .350            F

.243 WIN.        80 SP           .259            F,R,W

.243 WIN.       100 PSP          .371            F,R,W

.270 WIN.       130 PSP          .378            F,W

.270 WIN.       130 PSP          .345            R

.270 WIN.       150 SP           .267            F,R

.270 WIN.       150 PSP          .356            W

7mm MAUSER     140 PSP          .433            F,R

7mm MAUSER     175 SP           .280            F,W

7mm REM.MAG.   150 PSP          .354            F,R,W

7mm REM.MAG.   175 PSP          .453            F,R,W

.30 CARBINE     110 FMC          .180            F,W

.30 CARBINE     110 FMC          .164            R

30-30 WIN.     150 SP           .221            F,W

30-30 WIN.     150 SP           .195            R

30-30 WIN.     170 SP           .259            F,R,W

30-06          150 PSP          .329            F,R,W

30-06          180 PSP          .398            F,R,W

30-06          200 BTSP         .592            F

30-06          220 SP           .299            R,W

.300 WIN.MAG.   180 PSP          .462            F,R,W

.308 WIN.       150 PSP          .323            F,R,W

.308 WIN.       180 PSP          .390            F,R,W

.35 REM         200 SP           .193            F,R,W

.45-70          300 JHP          .302            F,W

.45-70          405 SP           .269            R

---------------

ABBREVIATIONS

Manufacturers: F=Federal, R=Remington, W=Winchester

Bullets:  FMC=Full  Metal Case, PSP=Pointed Soft  Point,  SP=Soft

Point, BTSP=Boat Tail Soft Point, MCBT=Metal Case Boat Tail

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 25

Part 3 - Rimfire

Cartridge              Bullet   Coefficient  Manufacturer

==================================================================

.22 LR "VIPER"           36 TC       .110           R

.22 LR "YELLOW JACKET"   33 TCHP     .101           R

.22 LR HV                40 LRN      .128           R

.22 LR HV                36 LHP      .115           R

.22 LR TARGET            40 LRN      .136           R

.22 LONG                 29 LRN      .095           R

.22 SHORT                29 LRN      .098           R

.22 SHORT                27 LHP      .091           R

.22 SHORT TARGET         29 LRN      .097           R

.22 WIN. MAG.            40 JHP      .108           W

.22 WIN. MAG.            40 FMC      .108           W

.22 LR "XPEDITER"        29 LHP      .079           W

.22 LR "H.P."            37 LRN      .117           W

.22 LR "DYNAPOINT"       40 LDP      .127           W

.22 LR "T22"             40 LRN      .135           W

.22 LR                   40 LRN      .127           W

.22 LONG                 29 LRN      .095           W

.22 SHORT "H.P."         27 LRN      .091           W

.22 SHORT "T22"          29 LRN      .097           W

.22 SHORT                29 LRN      .098           W

.22 LR "HI POWER"        40 LRN      .126           F

.22 LR "HI POWER"        38 LHP      .122           F

.22 LR "CHAMPION"        40 LRN      .135           F

.22 LONG "HI POWER"      29 LRN      .094           F

.22 SHORT                29 LRN      .100           F

.22 SHORT                29 LHP      .092           F

---------------

ABBREVIATIONS

Manufacturers: F=Federal, R=Remington, W=Winchester

Bullets:  LRN=Lead  Round Nose, LHP=Lead Hollow  Point,  LDP=Lead

Dynapoint, FMC=Full Metal Case, TC=Truncated Cone, TCHP=  Trun-

cated Cone Hollow Point

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 26

Appendix D

Specific Gravities of Pb/Sn/Sb Alloys

Percent Composition      Specific Gravity      Trade or

Pb      Sn       Sb         of alloy          Common Name

=================================================================

100                           11.340        pure lead

100                    7.298        pure tin

100           6.62         pure antimony

30      70                    8.32         soft solder

37      63                    8.42         eutectic solder

50      50                    8.89         50-50 solder

75      10       15           9.73         lead babbit-SAE 14

80      20                   10.20         20-80 solder

80       5       15          10.04         alloy 8

83       2       15          10.09         alloy 10

85               15          10.28         alloy 11

85       5       10          10.24         lead babbit-SAE 13

91                9          10.66         9% antimonial lead

92                8          10.74         8% antimonial lead

94                6          10.88         hard lead (bullets)

95       5                   11.00         5-95 solder

96                4          11.04         hard lead

99                1          11.27         1% antimonial lead

92        8           7.28         white metal

---------------

SOURCES:

Metals Handbook, Vol. 1, "Properties and Selection of Metals"

(8th ed.), American Society for Metals

Metal and Alloys Data Book, Samuel L. Hoyt

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 27

Appendix E

User Supported Software

User  Supported Software is based on the premise that  if  a

user  gets a chance to try out a program, likes it, and finds  he

uses it on a regular basis, he will voluntarily contribute to the

support  of that program. (Where else do you get the  opportunity

to  check out a product BEFORE you buy it?) Users are  encouraged

to copy and distribute User Supported Software for non-commercial

purposes. Commercial users should be aware that the author of the

program often retains commercial rights.

BALLISTIC  is being distributed as User Supported  Software.

If you're using BALLISTIC and you like it, or if you just want to

contribute  to  its further development, please send  the  regis-

tration form and $15.00 along with a blank, formatted 5 1/4" DSDD

diskette (or $20.00 without diskette) to:

William R. Frenchu

79 Taylor Terrace

Hopewell, NJ  08525

(Please label the diskette with your name and disk format!)

Contributors  will receive a copy of the latest revision  of

the  program,  as well as the TURBO C source code.  In  addition,

they  will be informed by mail of any major updates  or  improve-

ments. Major revisions will probably not be undertaken without  a

positive  user response. If you have any questions,  problems  or

suggestions about the program please contact the author.

All  commercial  rights  to BALLISTIC are  reserved  by  the

author. Anyone wishing to make any commercial use of this program

is  urged  to contact the author at the above address.  This  in-

cludes people who use the program in any commercial  environment.

Non-commercial  users  are encouraged to make and  distribute  as

many UNMODIFIED copies as they like, provided the following files

and documentation are included:

-README.1ST

BAL-GEN.EXE

BAL-READ.EXE

BALREMOV.COM

BALISTIC.EXE

BALISTIC.DOC

BALISTIC.REV

Comments  or  suggestions may be left for the  author  (Bill

Frenchu) at any of the following places:

Bullet 'n Board BBS    (703)-971-4491

Road To Utopia BBS     (609)-799-5418

CompuServe                   74575,61

BALLISTIC v3.01 - (C) Copyright 1988  W.R. Frenchu         Page 28

REGISTRATION FORM FOR BALLISTIC v3.01

Name _____________________________________   Date _______________

Mailing Address:

Street __________________________________   Apartment No. ______

City  ________________________  State ___   Zip ________________

Computer type ________________________ Display Adapter __________

Where did you get your copy of BALLISTIC? _______________________

Please register my copy of BALLISTIC and send the latest version

(as well as the TURBO-C source code) to the above address. (If

the version being registered IS the latest, the next upgrade will

be sent at half price.) Check:

_____ I have enclosed $15.00 and a 5 1/4" DSDD (360k) diskette.

_____ I have enclosed $20.00.

The author is always trying to improve BALLISTIC. You can help!

Comments about BALLISTIC v3.01:

Comments about the documentation:

Suggestions for future versions of BALLISTIC:

Types of shooting you do:

Mail to: William Frenchu, 79 Taylor Terrace, Hopewell, NJ  08525

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

The Public (Software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  The P(s)L cannot de-

bug programs over the telephone.

Disks in the P(s)L are updated monthly, so if you did not get

this disk  directly from the P(s)L,  you should be aware that

the files in this set may no  longer be the current versions.

For a copy of the latest monthly software library newsletter

and a list of the 1,000+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

(713) 721-6104 or (713) 721-5205