PMDial - copyright M.H.Plungjan 1994 - PostCardWare - send me a postcard

Platform         : OS/2 2.1 and higher (I hope)

Development tools: David C. Morrill's DRDialog

Acknowledgements : Ken Kavanagh for idea and modem commands; My office,

who's introduction of pincodes to dial outside the

city, for making it necessary.

Functions        : Phone book, access code prompting, translation of

free-phone numbers, makes your phone+modem a

hands-free phone with volume control, creates icons

of often used numbers - reach out and click someone,

you can paste a number to be called, you can drag and

drop a phonebook to be used

Known problems   : Some modems do not allow voice - maybe a voice/data

setting or switch needs to be changed - I cannot help that

- Dial the number, lift the handle and click hangup.

DRDialog doesn't like XGA or vice versa - it can give

screen refresh problems, sorry about that.

This little utility which I wrote in my spare time, uses any modem that

understands the AT commands (almost all modems) DT, M0, L1,2,3.

To use it, first create an ascii file containing the following:

COMPORT COMx

# FirstName LastName PhoneNumber

# FirstName LastName PhoneNumber

...

...

# FirstName LastName PhoneNumber

where x is the comport where the modem is attached.

The default file name the program will look for is PMDIAL.TXT in the

same directory PMDIAL.EXE is called from, but if you create an object

for the program, any file name can be specified as a parameter as long

as the file has the above format. This means that you can have several

phone books with an icon for each.

Phone number examples

Phone Number                 Comments

3798                         Inter-office

555-123                      Interstate

1-800-GO-LOTUS               Free phone

0,0045,31123456              International on outside line

#02,$,0,0045,31123456        International with $ prompting for pin code

0,555-123,,,,,,,,,,,,,,312   Outside line, Interstate, wait for a while

then dial extension 312

The following rules apply:

$ anywhere in the phone number will prompt you for a pin- or access code.

, is interpreted by the modem as a pause.

No spaces in phone number allowed, use - for readability and , for pause.

# and * are allowed.

You can dial the number, pick up the handle and click the hang-up

to disconnect the modem, the line will be maintained. If you leave the

modem connection, you can use your phone as a hands free phone depending

on the quality of the speaker in your modem and on the situation described in

Known Problems above.

If you like the program or have suggestions, drop me a note on

plungjm.plungjan@ibmmail.ibmx400.nl

or n1epo9sp@ibmmail.com

If you don't like the program, use the shredder - also on your mail to me.