SSFAXER.INI in the name of the file that is included with SSFAXER.  The INI

file can be named any filename you wish.  We have also included a file named

SHORT.INI so you can see how to use the INI file without all the programming

and explanation included.

The INI file is an initilization file for use with SSFAXER.  The INI file

allows you to control some of the parameters as you are faxing.  Variables are

stored in the INI file so the programmer can send faxes.

The INI file also allows you to use a cover sheet with your faxes.  The cover

sheet is a separate file and can be named whatever you wish.  Just make sure

it is included in the INI file.  (SEE COVER.DOC)

The first line of the INI file must be as follows:

[SSFAXER 7.00]

The following is a list of the variables stored in the INI file.

VARIABLE                        EXAMPLE

AnswerRing                      AnswerRing=3

AutoDelete                      AutoDelete=True

ComPort                         ComPort=2

CoverPage                       CoverPage=C:\SSFAXER\COVER.DOC

DialTime                        DialTime=45

DialPrefix                      DialPrefix=9,,

ErrorCorrection                 ErrorCorrection=False

FaxClass                        FaxClass=2

FaxFile                         FaxFile=C:\AUTOEXEC.BAT

FaxRate                         FaxRate=9600

FaxTitle                        FaxTitle=Info Request

FontName                        FontName=Compressed

FontFile                        FontFile=C:\SSFAXER\FONTS\WINDOWS.FNT

FromName                        FromName=M. Bates

Header                          Header=$D  $T   Title: $S  $F     Page $P of $N

HighResolution                  HighResolution=True

International                   International=False

IrqNum                          IrqNum=5

LastFaxFile                     LastFaxFile=FAX0002.FAX

LeftMargin                      LeftMargin=50

LogFaxFile                      LogFaxFile=SSFAXER.LOG

ModemInit                       ModemInit=W0&K4S7=90

PageLength                      PageLength=66

PhoneNumber                     PhoneNumber=2835871

PortRate                        PortRate=19200

StationID                       StationID=817-283-5871

ToName                          ToName=SS Faxer User

TopMargin                       TopMargin=20

TouchTone                       TouchTone=True

UseINI                          UseINI=True

The following is a more detailed explanation of each variable used in

SSFAXER.INI.  Use an editor or word processor to edit the SSFAXER.INI file.

You will notice in the file SSFAXER.INI, most lines are preceeded by a ";".

These lines are considered remarks, and have nothing to do with the actual

running of the program.  These remarks are there only for explanation purposes

and can be deleted.  (SEE SHORT.INI)

*** AnswerRing=3

This is the ring which SSRECER answers. Valid values are 1 - 20.

*** AutoDelete=True

If AutoDelete is TRUE then the FAX file is deleted after a successful

transmission.

*** ComPort=2

This is Communications Port to use. Valid choices are 1 to 8. Set IrqNum for

non-standard comports.

*** CoverPage=C:\SSFAXER\COVER.DOC

SSFAXER can send a cover page along with each fax.  A cover page is a text

file that contains text and replacement tags. By default, no cover

is used.  You can change the 'CoverPage' option to specify a cover page.

Use the full pathname of an appropiate coversheet. See COVER.DOC or Header.

*** DialTime=45

The 'DialTime' option sets the maximum number of seconds that SSFAXER will

wait for a carrier from the remote fax device.  By default, this is set to 45

seconds, which should be adequate for most situations.

*** DialPrefix=9,,

The 'DialPrefix' option is used to define any characters that need to be

prepended to a phone number.  For instance, if you need to dial '9' to get an

outside line on your phone system, you'll need to set this option to '9,' for

SSFAXER to be able to dial. Use commas for delays.

*** ErrorCorrection=False

Some faxmodems are capable of doing automatic ErrorCorrection.  If your

faxmodem is such a device, you'll want to set the 'ErrorCorrection' option

below to 'True'. ECM is the buzzword.

*** FaxClass=2

SSFAXER can send faxes using Class 1,Class 2  or CAS.  By default, SSFAXER

uses a Class 2 device.  You can change the device type by setting the

'FaxClass' option defined below.  By using Auto, SSFAXER will determine

the best setting. (If you are not sure, choose A - Auto.)

Valid settings for this option are:

1       Class 1

2       Class 2

C       CAS

A       Auto Detect

*** FaxFile=C:\AUTOEXEC.BAT

Enter the filename want faxed. Use a complete pathname if the file is in

another directory.

*** FaxRate=9600

The 'FaxRate' option is the rate at which data is transmitted to

the remote fax device.  Many faxmodems are capable of transmitting

at 14400 baud, but SSFAXER defaults to 9600 for compatibility with

other devices.

*** FaxTitle=Info Requestor

The 'FaxTitle' option is used by SSFAXER for the $S replacement tag in

header lines and cover pages.  You'll want to set this as a subject matter of

the fax.

*** FontName=Normal

Use Normal or Compressed for different CPIs.

****FontFile=C:\SSFAXER7\FONTS\WINDOWS.FNT

The filename of the font you want to use. Enter the complete path if

necessary.

*** FromName=S. Simon

The 'FromName' option is used by SSFAXER for the $F replacement tag in

header lines and cover pages.  You'll want to set this to your company name or

your own name.

*** Header=$D  $T   Title: $S  $F                Page $P of $N

SSFAXER can print a line of header text at the top of each fax page.  You can

change this header text by setting the 'Header' option.

A header line can contain one or more "replacement tags."  Replacement tags

are tokens in the string, prefaced by a '$', which SSFAXER expands at send

time into meaningful data.  Valid replacement tags are:

$D  today's date in MM/DD/YY format, always 8 characters (see

International)

$I  your station ID (see Station ID)

$N  total number of pages, variable size

$P  current page number, variable size

$R  recipient's name (see ToName)

$S  title of Fax (see FaxTitle)

$F  sender's name (see FromName)

$T  current time in HH:MMpm format, always 7 characters (see

International)

*** HighResolution=True

Faxes can be sent in either standard resolution or high resolution.  SSFAXER

defaults to standard resolution.  If you're sending complex graphics or very

busy pages, you might want to set this to high resolution.  Valid settings for

this option are:

False     standard resolution

True      high resolution

*** International=False

By default use Dates in the MM/DD/YY format and Times in HH:MMpm formats.

If International is TRUE then Dates are YYYY-MM-DD format and time is in

HH:MM:SS format.

*** IrqNum=5

Each comport has a default interrupt setting. If you have a modem which

requires a non-standard interrupt then set it here along with the comport

setting.

*** LastFaxFile=FAX0002.FAX

This the last fax file SSRECER has successfully received. SSRECER will change

this filename automatically.

*** LeftMargin=50

This is the pixel count for the left margin of the fax document.

*** LogFaxFile=SSFAXER.LOG

By default fax info is stored in the file SSFAXER.LOG. You may change this

setting to any valid full pathname.

*** ModemInit=W0&K4S7=90

Some faxmodems require a special modem initialization string, in addition to

the default string that SSFAXER uses.  If your modem requires a special

initialization string, you can define that in the 'ModemInit' option below.

The default string should be adequate for most faxmodems.

*** PageLength=66

When SSFAXER converts text files, it can break the text file up into multiple

pages of a specified length, or it can send the file as one long page.

By default, SSFAXER will insert page breaks every 60 lines of text.  You can

change this behavior by setting the 'PageLength' option. By setting a page

length of '0', SSFAXER will not insert any page breaks.

*** PhoneNumber=2835871

This the phone number to send the fax.

*** PortRate=19200

The 'PortRate' option tells SSFAXER the rate at which to talk to your serial

port. Most of the time, you'll want to set this to 19200, but it varies from

modem to modem.

*** StationID=817-283-6691

A fax device can identify itself to another fax device with a 20 character

name, called the station ID. The Class 1 and Class 2 specifications indicate

that the station ID should contain just a phone number, therefore the limit is

digits 0 to 9 and space.  However, the station ID is frequently

used to store an alphabetic name.  Most faxmodems support this convention by

adding upper and lower case letters as well as other special characters in the

station ID.  This can cause problems for some fax machines, though, since they

cannot print these characters.

The station ID (which defaults to a blank string) can be changed by setting

the 'StationID' option.  SSFAXER does not attempt to validate this string in

any way.

*** ToName=SS Faxer User

The 'ToName' option is used by SSFAXER for the $R replacement tag in

header lines and cover pages.  You'll want to set this to the person or

company receiving the fax.

*** TopMargin=20

This is the top margin created on a fax. The value is in pixels.

*** TouchTone=True

Faxmodems are capable of pulse dialing and tone dialing.  Most phone systems

can handle tone dialing.  If your phone system can't handle tone dialing, set

the 'ToneDial' option to 'False'