EEDRAW - Electrical Engineering Drawings (ver 2.0)

--------------------------------------------------

BECAUSE EEDRAW/EED-PS/EEDEPSON ARE LICENSED FREE OF CHARGE, I PROVIDE

ABSOLUTELY NO WARRANTY, TO THE EXTENT PERMITTED BY APPLICABLE STATE LAW.

EXCEPT WHEN OTHERWISE STATED IN WRITING, I GERSHON ELBER PROVIDE EEDRAW/

EED-PS/EEDEPSON PROGRAMS "AS IS" WITHOUT WARRANTY OF ANY KIND, EITHER

EXPRESSED OR IMPLIED, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED

WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE. THE

ENTIRE RISK AS TO THE QUALITY AND PERFORMANCE OF THE PROGRAM IS WITH

YOU. SHOULD THE EEDRAW/EED-PS/EEDEPSON PROGRAMS PROVE DEFECTIVE, YOU ASSUME

THE COST OF ALL NECESSARY SERVICING, REPAIR OR CORRECTION.

IN NO EVENT UNLESS REQUIRED BY APPLICABLE LAW WILL GERSHON ELBER,

BE LIABLE TO YOU FOR DAMAGES, INCLUDING ANY LOST PROFITS, LOST MONIES,

OR OTHER SPECIAL, INCIDENTAL OR CONSEQUENTIAL DAMAGES ARISING OUT OF THE

USE OR INABILITY TO USE (INCLUDING BUT NOT LIMITED TO LOSS OF DATA OR A FAILURE

OF THE PROGRAM TO OPERATE WITH PROGRAMS NOT DISTRIBUTED BY GERSHON ELBER)

THE PROGRAM, EVEN IF YOU HAVE BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES,

OR FOR ANY CLAIM BY ANY OTHER PARTY.

1. INTRODUCTION

---------------

EEDraw is a simple parametric drawing program, specifically designed for

EE (Electrical Engineering) type of drawing. The program fully allows new

parametric types, through library mechanisms, and in fact all the EE types

are defined this way. This library mechanism is extremely useful in defining

new types and can be used for other applications other than EE.

The following graphic devices are supported for display:

Hercules, CGA, ATT, EGA, VGA and SVGA (through user provided BGI driver).

The following printing devices are supported (via the printing drivers):

EPSON compatible printers (8 pins), PostScript printers.

Usage: EEDraw [-z] *[FileName[.EED]]

Options: [-z] Print version number and usage.

Quick start: The program comes with quite common setup. If you have a mouse

and a VGA/EGA/Hercules/CGA then you can simply start eedraw. Typing

'eedraw 82720-1.eed 82720-2.eed' will restart eedraw and prompt you for two

windows for the above two files. Click on left button (return on keyboard)

to pick window corners. Click on right button (space bar on keyboard) if you

want to default size (maximum size). Alt-Q, by default, will take you out.

2. INSTALATION

--------------

For correct execution of the main program (EEDraw), the following files must

exist in one of the path directories:

1. EEDraw.exe

2. EEDraw.cfg

3. EEDraw.bnd

4. EEDraw.hlp

5. *.lib

For correct execution of the printer drivers (EED-PS, EED-Epsn), the following

files must exists in one of the path directories (see also Appendix B):

1. EED-PS.exe				1. EED-Epsn.exe

2. EED-PS.cfg				2. EED-Epsn.cfg

3. *.lib				3. *.lib

The library files (*.lib) are the same for the printer drivers and eedraw

itself and should exist probably only once.

See SETUP below shows how to change the setup of this programs.

The default values should be enough to start and learn this program.

3. SETUP

--------

EEDraw set up is done through two files: EEDraw.cfg and EEDraw.bnd.

EEDraw.cfg

----------

This files contain global parameters that should be selected and set only

once, in final installation stage. Empty lines or anything after a semicolon

(';') is ignored. This file is read once with eedraw is being started.

Parameters that can be set are:

1. AutoPan - Boolean flag. If TRUE, any operation involves in the drawing

space (such as move, copy or draw) will auto-pan if cursor exists screen.

If FALSE, Panning option must be used manually to perform this operation.

This Boolean flag can be set from the Status main menu.

For example:

AutoPan	TRUE

2. WindowName - Boolean flag. If TRUE each EEDraw data file name is printed

in the head of the file. If FALSE no name is printed and this makes some

more space for the drawing itself.

This Boolean flag can be set from the Window main menu.

For example:

WindowName	TRUE

3. ATKeyboard - Boolean flag. If TRUE, it is assumed AT style keyboard bios

calls (also known as Enhanced keyboard - if you have F11/F12 on your

keyboard, good chance you can set this to TRUE). This will enable using

more complex scan codes, not supported by regular keyboard (such as

F11/F12). Note however it may HANG your system if it is not so.

For example:

ATKeyboard	FALSE

4. MouseSensitivity - Integer flag. Default is set to 10 which is a good

start. Making this number bigger makes the mouse less sensitive.

For example

MouseSensitivity 10

5. HVLines - Boolean flag. If TRUE, lines are coerced to be vertical/horizontal

only. This Boolean flag can be set from the Status main menu.

For example

HVLines	TRUE

6. SnapDistance	- Integer value. Sets the distance in drawing space of points

to be snapped. You probably do not want to change this, especially not in

the middle of a drawing. This Integer value can be set from Status main

menu.

For example:

SnapDistance	16

7. Allow256Colors - Boolean flag. 256 colors BGI drivers will have a richer

set of colors. If FALSE such a driver will be forced to use only 16 colors

effectively  having same colors are EGA or VGA default colors (EGAVGA.BGI).

For example:

Allow256Colors	FALSE

8. AllowAsyncEvents - Boolean flag. The user interface supports async event

handling. This means that one can pop up something while something else

is being poped up already for example. This has the advantage of the

ability to arbitrary "jump" between commands. However this may be VERY

confusing as well. Leave it FALSE at least at the beginning!

For example:

AllowAsyncEvents	FALSE

9. SaveBackMethod - Integer. Underlaying windows are being saved for short

life time pop up items such as queries. This integer selects where to

save this data. Options are:

1 - Conventional memory. This obviously reduces the amount of memory

for the EE drawings themselves.

2 - Expanded memory.

3 - XMS Extanded memory (currently not supported).

4 - Disk file. For reasonable response this better be a ram disk. See

also SaveBackPath below.

The device, this data is saved into, should be able to hold ~100k for

16 colors BGI drivers and twice that for 256 colors BGI drivers.

For example:

SaveBackMethod	1

10. SaveBackPath - String. This string specifies where back windows should

be saved. This full path should have a postfix '\' in it.

For example:

SaveBackPath		"f:\temp\"

11. Mouse - Boolean flag. Inform the system if a Mouse exists or not.

If this is commented out, auto detection will be used.

For example:

Mouse	TRUE

12. Joystick - Boolean flag. Inform the system if a joystick exists or not.

Must be TRUE for a joystick to be used. Personally the joystick was not

found accurate enough for this type of drawing.

Joystick	FALSE

13. BGIDriverPath - specifies where to look for the *.BGI drivers. Note the

drivers are supplied with this program (type 'dir *.bgi') and you should

update this path to the exact place you had moved them.

For example:

BGIDriverPath  "C:/TC/BGI"

14. GraphDriver - Set what type of display device to use. The relative driver

*.BGI are looked in the directory specified in BGIDriverPath above.

The following values are valid (same as TC 2.0/TC++ 1.0 graphics.h file

if that helps):

0 - Auto detect (default, so you can delete this one...)

1 - CGA (Hires two colors mode)

3 - EGA

4 - EGA64

5 - EGAMONO

7 - Hercules

8 - ATT (Not tested)

9 - VGA

999 - SVGA (super VGA - see SVGANameMode below)

For example:

GraphDriver	0

15. SVGANameMode - select a BGI driver to use. This driver is searched for only

if GraphDriver above is set to 999. Thus driver is assumed to be 800 by

600 in resolution but any driver with 4:3 aspect ratio will probably be

o.k.. This driver is searched in the directory specified by BGIDriverPath

above. For example if the driver you want to use is ATI.BGI and in mode 2:

SVGANameMode "ATI.2"

16. Libraries - String value. A list of libraries (neither file type, nor full

path is required - only the name) to automatically load on restart,

separated with commas (','). Libraries can also be loaded using the libs

main menu, and/or will be automatically loaded when a data files needs them.

For example:

Libraries	"TTL4LS,EPROM,GENERAL,LINEAR"

17. FrameWidth - Integer value. Sets the border width for all windows.

18. Colors control. Below are all colors values to select. Each can be one of

the following 8 colors:

0 - WHITE

1 - BLACK

2 - RED

3 - GREEN

4 - BLUE

5 - YELLOW

6 - CYAN

7 - MAGENTA

The colors are self explanatory and grouped into the following groups:

Root controls colors of the back ground root window. Actv sets the active

drawing window colors. Psv sets the passive window(s) colors. PopUp

controls all pop ups like queries colors. HighLigh sets the highlight

objects color. Below are the color groups themselves.

;Root window - the backgound.

RootFrameolor	7

RootForeColor	3

RootBackColor	5

RootXorColor	6

;Pop up menus/queries.

PopUpFrameColor	2

PopUpForeColor	5

PopUpBackColor	5

PopUpXorColor	6

;The currently active file/window if any.

ActvFrameColor	2

ActvForeColor	5

ActvBackColor	4

;All other passive files/windows if any.

PsvFrameColor	3

PsvForeColor	5

PsvBackColor	4

;High light color - selected items.

HighLightColor	2

If this configuration file is not found in any of the path directories,

a message is printed to the screen, and internal defaults will be used

instead. This is NOT recommended - fix that as soon as you can, as the

internal defaults are not guaranteed to be anything specific.

EEDraw.bnd

----------

This file allow you to bind any of the program internal functions (as can be

selected from any of the menus) to a specific key. Note some keys are

hard-bound and can not be used to rebind function (such as arrow keys - see

Functionality below). The format of the file is following.

Empty lines, or lines begin with a semicolon (';') are ignored.

Every line consists of three items: "FunctionName KeyName ScanCode"

seperated by white spaces. FunctionName are the fixed names this program

supports (see blow). KeyName is the name of the key ("AltK" for example) this

FunctionName will be bound to. Up to 4 characters are allowed for KeyName, and

this name will appear in the main menu which contains this function. ScanCode

is the scan code of the bounded key. If the key is regular ascii code, this

ascii code is the scan code. If it is extended code (such as F? keys are), add

256 to this value. All numbers should be in decimal.

Currently available FunctionName(s) and their associated main menu (see

Functionality section below for their description):

File menu:

LoadEEDFile

NewEEDFile

SaveAllEED

SaveEEDFile

SaveEEDAsOld

SaveEEDAsNew

CloseEEDFile

ClearAll

Directory

ChangeDir

ShellToDos

ExitEEdraw

Window menu:

PopWindow

PushWindow

MoveWindow

ResizeWindow

FullSize

ShowName

MakeActive

NameActive

PanWindowLeft

PanWindowRight

PanWindowUp

PanWindowDown

Display menu:

ZoomOut

ZoomIn

ZoomReset

RedrawAll

Libs menu:

LoadLibrary

FreeLibrary

ViewLibrary

DirLibrary

Draw menu:

DrawLine

DrawBus

DrawConnect

DrawText

DrawLibItem

Modify menu:

ModifyMove

ModifyCopy

EditLibItem

ModifyDelete

ModifyUnDel

ModifyCut

ModifyPaste

ModifyDrop

Status menu:

Help

GetMemoryFree

GetZoomFactor

SetDrawText

SetTextSize

SetHVLines

SetSnapFactor

Other (not in any menu):

SameAgain

4. Functionality

----------------

This section describes the currently available function set, and main modes.

Although most of the keys may be bound to any function as described in the

Setup section (EEDraw.bnd description), some of them are hard-bound and should

not be used. Note that in order for enhanced Keyboard scan codes to be used,

such as F11 and F12, 'ATKeyboard' must be set to TRUE in eedraw.cfg file.

See 'Mouse' and 'Joystick' in eedraw.cfg for mouse and joystick setup.

Below are the "hard-bound" keys:

A. All the numeric keypad (arrow keys, home, PhUp, End, PgDn). These keys

are used to move to cursor (even if mouse exists).

B. All the numeric keys but 0 (used by shift-arrows).

C. Enter (or Return) key. this is the SELECT key.

D. Space bar. This is the ABORT key.

E. Tab. This is equivalent to MIDDLE button key.

The arrow keys are used to move the cursor (co-exist with mouse), and if

shifted, move cursor faster. In addition to the cursor movement, two

important keys are defined as well:

A. SELECT key, hard-bound to the Enter (or Return), left button on mouse,

if such exists, and button 1 on the joystick, if exists.

B. ABORT key, hard-bound to the Space bar, right button on mouse if such

exists, and button 2 on the joystick, if exists.

C. MIDDLE key, hard-bound to the Tab, middle button on mouse if such

exists (or pressing both mouse buttons), and both buttons on the joystick.

All the eedraw functions described below uses these hard-bound keys.

This version supports multi file editing via multi window system (running

on regular DOS). Each time a file is being open, a new window is created

for it. If window header is to be shown (see ShowName function below) the

file name the window is associated with is displayed in addition to its

status as:

O)riginal - this file is exactly the same as first loaded in.

M)odified - the current file state should be saved. Data has been modified.

S)aved    - file has been modified since it was first loaded, but current

state is being saved.

There is no limit to number on the number of windows/files that may be open

at once, aside from DOS and memory limits. The libraries are being shared

between all files, so reduce memory usage. This means that even if two

(or more) files needs LINEAR.LIB it will be loaded once into memory.

However, there is no way for the program to detect which file uses which

library when they are being saved and it is assumed ALL libraries currently

loaded are being used by all files.

Few type of interactors are defined to query the user:

A. Yes/No question - prompt the question, with Yes and No buttons. The SELECT

key will select the answer. ABORT key is ignored. One may use the Y/N

keys as well to signal Yes/No answer.

B. Continue question - prompt a message, and wait for SELECT on the continue

button. ABORT key is ignored. One may use the C key to continue as well.

C. String question - prompt a message, and waits for string input. This

facilitates a full line editor as follows (independent of key bindings):

* Right and Left arrows will move right and left.

* Insert toggles Insert/Overwrite modes (toggles cursor shape).

* Delete deletes current character.

* Backspace deletes character to the back.

* Home and End will move to beginning/end of string respectively.

* Esc clears the current string.

* Return will terminate editing.

D. List question - prompt with a list of items (files names for example).

An item will be picked by the SELECT key if applicative (when viewing

a directory nothing needs to be picked). ABORT key will usually abort

the operation, but may have some special operation as well or may be

ignored.

E. Menu(s) question. Two types of menus are used - top down from the main menu,

and pop up on special occasions. Menus behave very much like List questions.

SELECT key selects menu item while ABORT key usually abort the menu with no

selection at all.

A. File menu:

LoadEEDFile - Prompts for a file name (List question), and reads it

from the current directory, if SELECT key, abort if ABORT key.

Note file type must be '.eed'. A new window is created for

it while the user is prompted to set the window size (see

ResizeWindow).

NewEEDFile - Creates a new drawing window not assosicated with any

file. This is the way to create new drawings. The user is

prompted to set the window size (see ResizeWindow).

SaveAllEED - Saves all unsaved drawings. This is probably useful

before quiting eedraw.

SaveEEDFile - Saves currently edited file. This routine can be used

only after a new file was saved at list once, or it was loaded.

In both cases, it will be saved on the same name, after

renaming old one to have '.bak' extension.

Ask for verification (Yes/No question).

SaveEEDAsOld - Same as SaveEEDFile, but prompts with list of all '.eed'

files in current directory, and saved by the name selected if

SELECT key, or abort if ABORT key.

SaveEEDAsNew - Same as SaveEEDFile, but prompts for a new file using

String question.

CloseEEDFile - Closes a drawing window associated with a file. Note

that non of the above saving functions close the window,

and this function is the only way to close a window.

Ask for verification if data modified (Yes/No question).

ClearAll - Clears all eedrawing data in a picked window. The user is

requested to "click" on the window to clear.

Ask for verification (Yes/No question).

Directory - Display content of current directory as in List question.

both SELECT key and ABORT key can be used to exit this mode.

Unlike LoadEEDFile, all files are displayed.

ChangeDir - Prompts with String question for a new directory. Old

directory is provided to begin with.

ShellToDos - "Fork"s (I wish this was true...) to the DOS O.S. . Type

'exit' at the DOS prompt to return to EEDraw.

ExitEEdraw - Exits the Program.

Ask for verification (Yes/No question) and issue a warning

if not all files were saved.

Window menu:

PopWindow - Pops up a window the user picks. The window is being

picked by "clicking" on it. ABORT key aborts this operation.

PushWindow - Pushes down a window the user picks. The window is being

picked by "clicking" on it. ABORT key aborts this operation.

MoveWindow - Moves a window the user picks. The window is being

picked by "clicking" on it. ABORT key aborts this operation.

The picked window frame is poped up so current position is

all visible. The cursor prompt changes to a box in the window

size that the user needs to place as the new position.

ResizeWindow - Same as MoveWindow but for resizing the window.

FullSize - Blows up a window the user picks. The window is being

picked by "clicking" on it. ABORT key aborts this operation.

This make the window as big as possible in the current

display device. Calling this function again on the window

will recover its original size.

This function may be activated on drawing windows only.

ShowName - A toggle to show/hide window name. The window name is

simply the filename the window is associated with. Also

displayed is O)riginal, M)odified, or S)aved file status

as O/M/S letters.

MakeActive - All drawing operations are performed on the active

window. If only one drawing window exists it is the active

one. If more, exactly one of them is the active one. This

function selects the active window by a user pick. The window

is being picked by "clicking" on it. ABORT key aborts this

operation.

NameActive - All drawing operations are performed on the active

window. If only one drawing window exists it is the active

one. If more, exactly one of them is the active one. This

function selects the active window by name. A list query of

all drawing window (file) names is being poped up and the

selected becomes the active one.

PanWindowLeft - Pan the ACTIVE window a half window to the left.

PanWindowRight - Pan the ACTIVE window a half window to the right.

PanWindowUp - Pan the ACTIVE window a half window up.

PanWindowDown - Pan the ACTIVE window a half window down.

Display menu:

ZoomOut - Zoom out (center of screen remains at center) by factor of 2.

Note that if you zoom out from regular zoom factor, text will

disappear as it can not be shrinked. You can enable text from

the Status menu manually.

ZoomIn - Zoom in (center of screen remains at center) by factor of 2.

ZoomReset - Reset Zoom value to the regular zoom factor.

RedrawAll - Redraw all screen.

Libs menu:

LoadLibrary - Prompts for a file name (List question), and reads it

from the current directory, if SELECT key, abort if ABORT key.

Note file type must be '.lib'.

FreeLibrary - Prompts with all loaded libraries with List question,

and frees the selected one if SELECT key, abort if ABORT.

ViewLibrary - Prompts with all loaded libraries with List question,

If a library has been selected via SELECT key, prompt with

all parts defined in this library again with a List question.

Selection of a part with SELECT key will display it. ABORT will

go back to library List question, which can be aborted again

via the ABORT key.

DirLibrary - Displays content of current directory library files

(those with '.lib' extension) as in List question.

both SELECT key and ABORT key can be used to exit this mode.

Draw menu:

DrawLine - Enters drawing mode. If HVLines is set (see SETUP,

EEDraw.cfg section), only horizontal/vertical lines are

allowed.

In this mode, pressing SELECT key will insert a new point

in the created polyline and pressing ABORT will delete the

last entered point.

If however, there are no points at all in the current polyline,

ABORT key will abort this mode.

Also, pressing SELECT twice (at the SAME place as the last

point entered) will complete the drawn polyline.

MIDDLE key will also complete the line drawing.

DrawBus - Same as DrawLine, but draw thicker lines.

DrawConnect - Draw a connection junction, if SELECT key is pressed,

or abort if ABORT key is pressed.

DrawText - Prompts for a string with String question, and then position

the text horizontally or vertically. If the positioning is

terminated with ABORT it is ignored, SELECT will update.

DrawLibItem - Prompts for a part name with String question. If this is

not the first invocation to this mode, last part will be

used as default string (remember you can always use Esc to

clear to a fresh line). If part is found (name is exactly the

same as	shown in the ViewLibrary function above), a pop up

menu will allow to rotate it 90 degree CW or CCW or mirror it.

Successive SELECT will transform the library item to the

desired orientation, while ABORT will abort this DrawLibItem

mode. SELECT PlaceIt (top menu entry) will allow to position

the part in the exact place.

The user is also being prompted for Chip (i.e. 74LS00) and

Part (i.e. IC7) names if they are required/allowed (see

Appendix on library file definition) for this part.

These are positioned similar to the way DrawText position

its text, except that Chip name	is already known and can not

be modified.

Modify menu:

This menu is heavily based on Picking objects. a single object is

being picked if the cursor is on an object. The cursor is considered

on an object if one of the following:

A. Line and Bus items: the cursor is on one of the Line/Bus end points

(any point that was set by SELECT when it was created, including

internal points).

B. Connect item: the cursor is on the Connect center.

C. Text item: the cursor is inside the smallest axes parallel bounding

box holding this text.

D. Library item: the cursor is on any polyline end point (including

internal points).

The picked object will blink. SELECT key will select it, ABORT key

will cause the next object, if such exists at this position, to blink

until one is selected via the SELECT key, or no more objects can be

picked.

If no object at all is found at the cursor point, the point is used

as first corner of a box to pick a set of objects - a second SELECT

key will pick the second corner of the box, ABORT key will abort this

mode. All objects intersect with the defined box, will be considered

picked, with no more verification.

ModifyMove - Pickes object(s), and moves it (them) to a new position.

Once the object(s) i s(are) picked, SELECT key will set

its(their) new	position, ABORT will leave it unchanged.

ModifyCopy - Pickes object(s), and copies it (them) to a new position.

Once the object(s) is (are) picked, SELECT key will place

it(them) in new position, ABORT will abort the copy operation.

EditLibItem - if the picked object(s) is a single library item, allows

editing any of the following (via a pop up menu):

A. Editing the part name. This is the only way to add a part

name to a library item if the library item has none by

default.

B. Toggle to Draw/Undraw chip name (if this chip is allowed

to have one).

C. Reposition Chip name (if this chip have one).

D. Reposition Part name (if this chip is allowed to have one).

E. Reorient the lib part.

Picking one of the above via SELECT will allow text

positioning or text editing (via String question). ABORT or

selecting Done (first menu entry) will abort this EditLibItem

mode.

ModifyDelete - Deletes the picked object(s). The deleted object(s) is

(are) saved in a special 10 places deep stack, and deleted

items can be undeleted using the ModifyUnDel function.

Each of the 10 placed stack can hold any result of a pick

no matter how many objects are inside.

This Delete/Undelete mechanism can be used to cut and paste

even between different files/windows - this stack is never

being cleared.

ModifyUnDel - Undelete the last delete operation (see ModifyDelete),

from the delete stack (LIFO).

ModifyCut - Same as ModifyDelete, but original is NOT deleted. The

object is being pushed onto the undelet stack and can then be

ModifyUnDel, ModifyPaste etc.

ModifyPaste - Same as ModifyUnDel, but the undelete stack is NOT

modified. This function enables pasting and placing the same

object few times.

ModifyDrop - Drops the top of stack from the undelete stack. May be

used to drop a cut/paste object sequence from stack.

Status menu:

Help - Pops up an help window and displays the content of eedraw.hlp.

If the help window is already poped up, this function will

close it (acts like a toggle.).

GetMemoryFree - Prompts with a continue question, on the current free

memory. Getting close to zero is unsafe!

GetZoomFactor - Allows you to see the current zoom factor

SetDrawText - Sets drawing of text smaller than minimal size (8x8).

This will draw text at 8x8 size which is too big, but may be

sometimes useful.

SetTextSize - Sets the scaling (up only) of the text size. By default

this is equal to 1, but up to scaling up by 7 is allowed.

SetAutoPan - If TRUE, auto panning will occur when placing

objects that have been moved off the active WINDOW. The

object will be place so it is centered in the window.

SetHVLines - Allows drawing of diagonal lines as well if set to FALSE.

Uses Yes/No question.

SetSnapFactor - Sets the invisible grid the cursor position is

snapped to. You probably do not want to change this,

especially not in the middle of file editing.

Uses String question, and display current Snap factor to

begin with.

Other (not in any menu):

SameAgain - Executes the last function again. This is hard bounded

to MIDDLE key, although in may be bounded to another key

as well.

5. Spacial Notes

----------------

Designing using A4 page size does not require much memory even when few

libraries are loaded. My system with 640k (-70k of the O.S) has almost 300k

free to use (Use the GetMemoryFree function to query free memory), when all the

provided libraries are loaded. I guess you will need to work hard to get down

to less than 100k. If you succeed and/or you system has less than 640k memory,

make yourself a rule: never to go below 64k of free memory. The program need

about 32k of temporary memory for the different operations, and getting close

to that is dangerous! When you approach 32k of free memory, you will be

issued a warning (Continue question):

Free memory too small - dangerous to continue.

Every 30 second (if you do any operation in that time). Continuing at this

point may result with loss of all your work - the program will exit when no

more memory will be available! This is especially true if 'SaveBackMethod'

in eedraw.cfg is set to conventional memory.

Yes, I know this is ugly, but you eat what you cook, right? I used this

program quite a bit, and never succeeded to go below 200k, so why bather.

6. Acknowledgement

------------------

I would like to thank skh@eng.sun.com (Steve Howell) for his great comments

during the last stages of bringing version 2.0 up.

-------------------------------------------------------------------------------

Appendix A. Glossary

--------------------

SELECT Key - The key used for the select operation. Currently hard-bound to

Enter (or Return), the Left mouse button if mouse exists, and to

Button 1 in a joystick.

ABORT Key - The key used for the abort operation. Currently hard-bound to

Space bar, the Right mouse button if mouse exists and to Button 2

in a joystick.

MIDDLE key - Another general purpose key. Currently hard-bound to

Tab keyr, the Middle mouse button if mouse exists and has middle

button (or both right and left buttons pressed simultaneously)

and to both Buttons of the joystick.

Screen Space - the space of the display device. Depends on the graphic

device coordinates on the selected mode.

Drawing Space - the space in which the drawing objects exist. This space

is bounded by a wide box (zoom out twice to see this box on an

empty drawing) so it will fit into an A4 (8.5" by 11") page in both

the EPSON and the PostScript printer drivers (see Appendix B).

Appendix B. Printer drivers

---------------------------

The following printer drivers are available (see also installation section):

1. EPSON printer.

Usage: EEDEpson [-g] [-r] [-1] [-2] [-3] [-d] [-z] FileName[.EED]

Options: [-g] Output goes to standard output as 1 bit per pixel (B&W)

GIF file.

[-r] Output goes to standard output as raw data.

[-1], [-2], [-3] Output goes to LPT1:, LPT2:, LPT3:

respectively.

[-d] if Output goes to Epson printer - print in double the

density.

[-z] Print version number and usage.

* if neither [-g], not [-r] are specified, output is in Epson

compatible format (default).

* If non of [-1], [-2], [-3] are specified, output goes to

standard output.

* [-1], [-2], [-3], [-d] are ignored if [-g] or [-r] are

specified.

2. PostScript Printer.

Usage: EED-PS [-f FontName] [-z] FileName[.EED]

Options: [-z] Print version number and usage.

[-f FontName] Specify the font name to use instead of the

default name (Times-Roman).

Appendix C. Library file format

-------------------------------

This appendix describes the format of the library file. It contains

all the information required to create your own libraries. The libraries are

designed to be parametric only. I.e. no bitmaps are supported. You can look

at the given libraries while reading this appendix for better understanding.

Empty lines and lines starts with '#' are ignored. In order to identify

a library as such the first line must start with 'EEDRAW-LIB'. Following in

the same line is version number as 'Version X.X'. Currently 'X.X' equal '1.0'

and is ignored. However it may be used in future versions, to ensure backward

compatibility. Following are the parts themselves. Each part definition begins

as follows:

DEF Name Prefix #Pins TextInside DrawNum #Units #PinsPerUnit

Were	1. Name is the Part name, i.e. "74LS00". If Name is prefixed with "~"

(for example "~74LS00") then the name is not drawn, nor it will be

accessible to the user.

2. Prefix is the prefix to be used in chip name (IC, R, C etc.). If

prefix is "~", then no prefix is drawn.

3. #Pins is the same as number of entries in PINS (see below),

i.e 14 for 74LS00.

4. TextInside is 0 if pin text is to be put outside as well (as the

pin numbers are), otherwise inside. You may put any positive

integer there to scale the distance inside. 10 is a good start.

5. DrawNum is 1 if pin numbers are to be drawn, 0 otherwise.

6. #Units holds number of multiple entries. For example 74LS00 has

4 units (Nand gates). For no multiple units put 0 here.

7. #PinsPerUnit holds number of entries for one unit if more than

one unit per part. If #Units is 0, it is ignored.

The part definition follows by the following blocks, in any order:

1. DRAW/ENDDRAW - defines how to draw the part. This block is optional, and

a simple box with half the pins on each side is drawn if this block is

not defined. The following commands are supported:

A x y r t1 t2		(Define ARC at x, y, radius r, angles t1 to t2)

P n x y x y x y... [F]	(Define a POLYLINE of (any) length n)

C x y r			(Define a CIRCLE at x, y, radius r)

S x1 y1 x2 y2		(Define a square (Not filled))

T x y h str			(Define a text string at x, y (see below))

L x1 y1 x2 y2 [I]		(Define terminal line to part - see PINS)

Notes:

A. a text string (S) with be drawn horizontally if h is 1, vertically if 0.

Also '~' characters will be replaced by spaces, or this provides a

mechanism to have spaces in the string.

B. If the polyline (P) is postfixed by a F, the polyline should define a

closed shape that will be filled. The first point of the polyline must

be identical to last one in this case.

C. If the line (L) is postfixed by an I, this line is assumed to define

negative logic, and a small circle will be drawn to express that.

D. The number of L definitions should be exactly the same as #Pins in the

DEFS line if #Units = 0, and equal #PinsPerUnit if #Units != 0.

The order will match to the order in the PINS/ENDPINS block if #Units = 0

and to the order in the MULTI/ENDMULTI if #Units != 0.

E. Arcs (A) should not exceed 180 degrees, are CCW, and 0 degrees is at

3 o'clock.

2. PINS/ENDPINS - defines the names (strings) associated with each pin. This

block must always exists, and it is the minimum required to define a part.

The block holds #Pins lines each hold the string defines that pin. This

order must be the same as define in the DRAW/ENDDRAW as L commands

(if DRAW/ENDDRAW block is defined), from pin 1 to #Pins. Empty lines

are allowed and defined with only '~' in them. Not sign for X (vertical bar

above it) is defined by ~X.

3. MULTI/ENDMULTI - This block must exists iff #Unit != 0. Each line in this

provides a mapping from L terminal definitions in the DRAW/ENDDRAW block

to the pin numbers in a multi unit part. This is required as same L

terminal will have different pin number in different Units.

Each line in this block holds the pin numbers of a unit.

Number of lines should be equal to number units in part.

Part definition terminates with the ENDDEF line.

General Notes:

1. All parts are scaled up to easy the access of a single pin (by 16, but you

should not wonder about it too much - its internal mapping to drawing

space).

2. Library file can hold any number of parts (at list one though...).

3. Library file must have '.lib' extension, to be usable by the EED* programs.

Gershon Elber

Email: gershon@cs.utah.edu

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 3,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

Orders only:

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.