Instructions for

MAPLPT -- LPT to COM port mapper

Copyright (C) Patrick Swayne 1988.  All Rights Reserved.

** Introduction

MAPLPT is a program that allows you to use a serial printer on an

IBM compatible computer.  It maps output intended for a  parallel

(LPT)  port to one of the serial (COM) ports.  Like MODE, it  in-

stalls  a portion of itself into memory as a resident (TSR)  pro-

gram  to perform the mapping.  It goes beyond MODE,  however,  in

that  it supports all forms of handshaking likely to be  required

by a printer, and it supports baud rates to 38,400.

MAPLPT was written by a user of Zenith computer equipment,  which

has the ability to map an LPT port to a COM port with handshaking

support  built  into the ROM BIOS.  This  capability  was  sorely

missed  when  I  aquired some non-Zenith equipment,  so  I  wrote

MAPLPT originally for my own benefit, and it has served me well.

** Distribution

This program is distributed under the "Shareware" concept.   Feel

free  to use this program and give copies to others, as  long  as

you  include this documentation with the program, and as long  as

you do not modify either the program or this documentation.

You may not sell this program for any amount, but you may  charge

a nominal fee (e.g. $5.00) for the disk on which it is distribut-

ed.   If you are a shareware distributor, it is recommended  that

you  combine this program with other utilities on a disk to  pro-

vide your customer a greater service.

If you feel that this program is of value to you, please register

your copy by sending a contribution of $15 to

Patrick Swayne

155 Baker St.

Coloma, MI 49038

Registered users will receive a disk containing a copy of  MAPLPT

without the "Please register" message, and a copy of the assembly

source code for MAPLPT.

Warranty

This program is provided AS IS without any warranty, expressed or

implied,  including but not limited to fitness for  a  particular

purpose.

Disclaimer

In no event will Patrick Swayne be liable to you for any damages,

including any lost profits, lost savings, or other incidental  or

consequential  damages arising out of the use of or inability  to

1

use this program, even if Patrick Swayne has been advised of  the

possibility of such damages, or for any claim by any other party.

** Operation

To  use MAPLPT, you must first copy MAPLPT.COM from this disk  to

your system disk, in the root directory or a directory pointed to

by  your  PATH description.  The syntax for operating  MAPLPT  is

nearly identical to the one for MODE, when it is used for mapping

an  LPT port to a COM port.  The differences are that  there  are

more items you can specify, you do not use a colon (:) after  the

LPT  port designation, and only one command line is used  to  map

the  port  and  set the baud rate and  other  settings.   To  use

MAPLPT, enter a command in this form.

MAPLPT LPTn=COMn,baud,handshake,parity,data bits,stop bits,time out

Here  is a description of the command line arguments.  The  n  in

LPTn  and COMn is a number that designates the ports you want  to

use,  as  in LPT1 or COM2.  The defaults, if you do  not  specify

anything, are LPT1 and COM1.  MAPLPT supports printer ports  LPT1

to  LPT4 (regardless of how many you actually have),  and  serial

ports  COM1  and COM2.  It also supports COM3 and COM4  if  their

port  addresses  are recorded at memory locations 40:2  and  40:3

(hex).

The  baud  rates supported are 110, 150, 300,  600,  1200,  2400,

4800, 9600, 19200 and 38400.  The default baud rate is 9600.

The handshaking modes supported are

RTS[-] -- hardware handshaking via the RTS/CTS line.  Include the

- after RTS if negative polarity is used to indicate ready.

DTR[-] -- hardware handshaking via the DTR/DSR line.  Include the

- after DTR if negative polarity is used to indicate ready.

CMP -- compatibility mode.  Both the RTS and DTR lines monitored.

ETX(burst)  -- ETX/ACK software handshaking.  An  optional  burst

count  from 1 to 32 can be specified in parentheses,  as  in

ETX(20).   The count you specify is multiplied by 32 to  ob-

tain the actual count used.  The default count is 1  (making

an  actual count of 32).  Diablo printers that  use  ETX/ACK

handshaking  will print bi-directionally if you specify  the

largest burst count the printer can handle.

XON --  XON/XOFF software handshaking.  This kind of  handshaking

is also called DC1/DC3.

NONE -- no handshaking is used.

The default handshaking mode is RTS positive.  You should consult

your  printer manual to determine the handshaking mode(s) it  re-

quires.

Parity  checking is specified by indicating E for even parity,  O

for odd parity, or N for no parity.  The default is no parity.

You can specify 7 or 8 data bits.  The default is 8.

2

You can specify 1 or 2 stop bits.  The default is 1.

The  time out can be any number from 1 to 255.  It indicates  ap-

proximately  the number of seconds before MAPLPT returns  a  time

out error if the printer should signal that it is not ready for a

long time.  The default time out value is 20.

As with MODE, you can leave out any of the command line arguments

by  typing  a comma in its place.  Here are  some  sample  MAPLPT

command entries.

MAPLPT ,,DTR

In  this  example, all of the defaults are used  except  for  the

handshaking, which is set to DTR positive.

MAPLPT LPT1=COM2,,,,,,60

In this example, LPT1 is mapped to COM2, and the time out is  set

to 60.  The defaults are used for the other settings.

MAPLPT ,4800,RTS-,,,,40

In this example, the baud rate is set to 4800, the handshaking is

set to RTS negative, and the time out is set to 40.  The defaults

are used for the other settings.

MAPLPT  installs its resident portion into memory the first  time

you  run it after booting your operating system.  If you  run  it

again  later, it will make any changes required to  the  resident

part.   In addition to the settings described above, you can  use

these commands.

MAPLPT D     To disable LPT to COM mapping.

MAPLPT       To  enable LPT to COM mapping (using current  set-

tings).

MAPLPT  U    To  uninstall MAPLPT.  You can only  uninstall  if

MAPLPT is the last TSR loaded.

Each time you enter a MAPLPT command (except for the D or U  com-

mands), MAPLPT echoes the current settings so that you can verify

that they are correct.  For example, if you had just entered  the

third sample command line, MAPLPT would print

LPT1=COM1,4800,RTS-,N,8,1,40

You  can  take  advantage of this feature to  check  the  current

MAPLPT  settings  by entering MAPLPT with no  argument.   If  you

enter invalid information in a command line, MAPLPT will print

Error in command line.

It  will  accept any correct information up to the point  of  the

error,  and print the current settings so that you can  see  what

was accepted.

3

If you forget the command syntax for MAPLPT, and you do not  want

to read these instructions again, just enter

MAPLPT ?

and MAPLPT will print a screen of instructions.

Using Printer Software

Once  you have installed MAPLPT, any software that you have  that

uses  your printer should be configured to use the LPT  port  you

have  mapped,  not COM1 or COM2.  If your LPT port is  LPT1,  the

software  may  also be configured to use PRN.  However,  you  may

find that printing via LPT1 is faster than printing via PRN.

4

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

The Public (Software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  The P(s)L cannot de-

bug programs over the telephone.

Disks in the P(s)L are updated monthly, so if you did not get

this disk  directly from the P(s)L,  you should be aware that

the files in this set may no  longer be the current versions.

For a copy of the latest monthly software library newsletter

and a list of the 1,900+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

(713) 665-7017