UNIBAR01.DOC    4-16-90

UNIBAR BAR CODE 39 LASER PRINTING PERFECTION

Nine Different Variable Size Bar Code Densities

Use Any Bar Code Layout Anywhere on the Page

UNIBAR is for use with HP and Compatible laser printers and IBM

PC, XT, AT, 386, PS/2 and Compatibles running DOS.  The memory

resident routines work with almost any programming language.

Subroutine versions are available by special order.  Site,

distribution, program inclusion and single user licensing are

available.

UNIBAR offers the full accuracy of the 300 dots per inch Bar Code

positioning, line width, line space and line length printing.  In

contrast, much existing software uses the "easy-to-program" 1/120

positioning and introduces significant errors in some bar widths

and bar spacings.  This is caused by laser printer plus and minus

compensation when converting from 120 to 300 dots per inch.  You

can easily verify this with a magnifier.  In fact, many so called

"excellent" commercially produced bar code labels are ridden with

these errors.  No wonder users often think their bar code readers

are faulty!

Smallest bar codes start at one dot for small bar and space

widths (0.0045").  The largest bar code has nine dots for small

bar and space widths (0.030").  The large bars and spaces are

dimensioned to a ratio of at least 2.5 times the small bars and

spaces.  Due to the fact that most people set their laser printer

toner control for extra-black output, the bars usually are wider

than they should be.  UNIBAR provides compensation to help

counteract these over-width lines.

There are nine different bar code densities.  Density is a trade

term that refers to the number of characters per inch.  This term

can be slightly misleading because Code 39 requires a start and a

stop character in addition the valid alphanumeric characters.

Bars can be printed in any height from one dot up to 999 dots

tall (3.3 inches).

The optional SWFTE International Glyphix font generation package

will produce 6 to 60 point printer soft fonts in infinite

variations of short, tall, condensed, wide, slanted, light, bold,

etc. in Helvette style.  Other options include many types of

variable fonts for artistic and form uses.  Another package will

produce only the characters you need.  This is an important

printer memory saving tool, especially when you start using large

fonts on labels and forms.  Glyphix programs are easy to use and

very fast at loading the created characters.  The font quality is

excellent. Converters are available to enable Glyphix fonts to

work with the proportional spacing of word processors and desktop

publishing programs.

A documented source code program is included.  It is written in

BASIC to be more universally useable.  The code can be used by

programmers to get them up and running quickly.  They can also

use much of the code with slight modifications, or study it to

pick up some of the tricks that are very costly to develop.

There are provisions for most of the complicated conversions and

code structuring that will be needed.  Bar codes and associated

text can be accurately dot positioned to any of the printable

locations, including a number of areas that are not accessible by

many programs.  Much competitive software does not offer true

variable dot positioning and size.  Usually the bar codes are

located with row and column placement instructions.  That system

quite often requires special labels or forms to fit the limited

placement accuracy.  The BASIC program also gives you the code to

do row and column placement for use when accuracy is not needed.

Functions are included to produce ESC codes, CR without LF codes,

any of the nine bar code densities, 001 to 999 dot bar height,

deactivate the routine, etc.  These can all be generated by short

ASCII printable sequences from Word Processors, Basic, C, Cobol,

Pascal, Fortran, etc. programs.  Many programs cannot easily

generate these control and escape sequences, so UNIBAR includes

them.  Once a function is set, such as bar density or bar height,

it will stay the same until changed by the program.  This also

means you can place any density or height of bar code anywhere

you choose.  If you are limited at printer control, you can only

do part of the job.

A short manual is included.  The manual is written to minimize

confusion and to try to relate code and techniques to the laser

printer manuals.  To try to find a simple solution in a laser

printer manual can sometimes defeat all but the most hardy

people.  Other help is available by phone.  It is best to have

some programming knowledge of the laser printer and a technical

manual in addition to the users manual.  A working knowledge of

Basic or the ability to learn it is necessary for best use of the

programs.  Laser printers are not easy to master.  Some programs

contain more features and codes than UNIBAR, but they are more

difficult to learn to use effectively.  There are easier programs

to work with, but they do not offer the accuracy, versatility and

flexibility of UNIBAR. Many options are available, and others

will be considered on user demand.

The UNIBAR Single User License is $75.  This license includes

either a TSR version and control source code, or a linkable OBJ

module and control source code.  The SWFTE Glyphix variable font

generator and Helvette master font is $35.  Other font and design

generators by quotation.

UNIKEY SYSTEMS, 20210 Laceyland, Katy, TX 77449. Tel 713-578-

5522, Fax 713-579-2160.

UNIBAR Documentation Updated 4-9-90

UNIBAR Control Source Code and Object Module for printing Bar

Code 39 with variable bar height and width.

We have eliminated the UNIKEY plug-in software protection.  Also,

we reduced the linking complexity by combining the link object

modules.

UNIKEY SYSTEMS, Div. of Gordon Company, KATY, TX 77449  Tel 713-

578-5522, Fax 713-579-2160

This write-up is in the UNIBARxx.DOC file on the disk.  That is

where most of the documentation goes for all our software

packages.

You no longer have to insert your serial number in the source

code.  It is supplied when we generate the object modules.

You should also have a Technical Manual, which is more advanced

than your User's manual, available before you even try to program

a laser printer.  Kindly keep in mind that we released the basic

source code so users could actually learn laser printer control.

Most other groups want to keep you in the dark and furnish you

with menu driven or string driven laser software.  These are

usually easier to learn to use, but we believe you are better off

if you learn to do your own formatting and control.  The program

is much longer and more detailed than most people will need for

their printer control.  If you are a customer of ours, it is OK

with us if you just build on it for your own needs.  Later you

can cut out the code you do not need.  For now, there are enough

functions to get most people into production with the code as is.

If you see something that we should add, please let us know.

The trick to getting a lot more production from your laser

printer is to, where practical, set up your page so that all

exactly the same data can be printed on one pass.  The variable

data should be set up to print on the last pass of the page or

labels thru the printer.  This way you can set the printer, via

the control panel or program, to print multiple copies (up to 99

at the panel) to print at the highest speed possible.  It takes

almost no setup time to print pages that are alike, but takes a

lot of formatting time to set up a page with any variable data.

It has to re-set the entire page.  You will have to learn how to

hand curl the pages opposite to any curl they come out of the

printer with.  If you have data that can be reversed, usually you

can run the pages back thru tail end first without de-curling.

In many cases we make a master with the laser printer and then

run the repetitive copy material on a good copier with accurate

registration capability.  This is a less costly and much faster

process than laser printers.  Many copiers enhance the print

quality of the laser master.

If you can, study the control source code of UNIBARxx.BAS to

absorb some of the methods we have developed.  It will save you a

lot of time.  Our customers are authorized to use any of this

code they need.

We have included a simplified version called UNIBTEST.BAS.  Work

with it first, then tackle the UNIBARxx.BAS.  We will be glad to

give you a hand for a moderate fee.

We include UNIBARxx.EXE and UNIBTEST.EXE on the disk so you can

get into production.  These have a lot of features.  If you need

modifications you can do so, or we can help.

We can produce the OBJ modules for linking with many other

programming languages.  For now we only provide the source

control code in BASIC.  Most people can handle this.  Many

programmers are also finally recognizing that BASIC as an

economical language to work with.  Debugging and getting a

working version of software is QUICK indeed.  You have to use a

compiled version if you want speed.

You will have to compile the code you use with a BASIC compiler

compatible with those sold by Microsoft, and then link the

resulting OBJ module with UNIBARxx.OBJ to get a running EXE

program.  Compiling within QuickBasic will not work.  You will

have to use a compile line such as: QB PRONAME /O/E; when you use

a QuickBasic compiler.

We no longer include ON ERROR GOTO xxxx.  You can code error

messages to suit your needs.  Another addition is the printer

status check routine at line 2010.  We now gosub to this before

each print command and each call bars(xxx$).  The calls print

from within the UNIBAR.OBJ assembly language routine.  We try to

keep you from the loss of print data caused by printer errors or

time-outs.  This is not perfect in that we do not check status

between each character of a print string, only just before a

string of characters is to be printed.  For perfection, it is

necessary to link in an assembly language print routine that

checks status before each character.

You can debug in BASIC without compiling if you the following to

eliminate the actual call functions.  We change CALL BARS(xxx$)

to LPRINT"CALL BARS(xxx$)".  This will give you test printing at

the location, but not bar codes.  This method does not require

the program to be compiled and linked with the needed object

modules each time you want to test a portion of the code.  We do

the same, or jump around similar coding while working on program

changes.

Normal text and large fonts build up and to the right from the

cursor location, while bar codes build down and to the right.  An

easy way to get started is to move the top margin and left margin

around.  This will let you move repeat printed patterns, we call

them step and repeat, around the page in a way you can visualize

more easily.

For general testing, you do not have to download fonts.  Some of

the output text will be missing using menu choice 6.  This menu

choice will give you the code for changing fonts while printing,

and accurate bar code and text placement.  The missing fonts can

be downloaded with GORLOD.BAT.  You can change them or the code

to suit your format needs.

We display a number of the control settings while in the main

menu.  The more complete display and control programming is via

menu choice 5.  This addition will save you hundreds of hours in

development time.  The section is self explanatory by menu

descriptions, but in short it lets you construct a format to use

with single or multiple placement labels.  Also, you can save the

control data to a disk file with menu choice 23.  You can reload

or/and change it to fit your application.  We set up a format

initially that you can use for testing.  For simplicity, we have

included some test files.  The control file LAB3X10 can be loaded

with menu choice 22.  It sets 3 columns wide by 10 labels high.

When using internal label generation,  main menu choices 7 & 8,

no data files are required.

When using file input data, you will have to name a data file.

The data file DATA3X26 has 26 records of 3 items each, while The

file DATA5X2 has 2 records of 5 items each, etc.  File print

programs will ask for the data file name in their sub-menus.

Data files can be generated in many ways, but are usually

selected data fields generated from master files by database

programs.  You can also look at, change or generate test files

with a text editor or word processor.  Be sure your editor does

not add in hidden formatting codes.  You can use our DFA5.EXE

disk file analyzer program to display or print data or text files

and at the same time showing where the hidden control codes are.

The quickest way to learn laser printer control is to play with

this entire program.  For instance you will learn that you can

print to within 0.2" from the top, 0.3" from the bottom.  The

main control is the TM, top margin, and the LM, left margin.  All

dimensioning is done from these values.  TR, top row, will

control only when you use row and column placement, and then it

will be in relation to TM.

You cannot use a LF, line feed, after a print at the last bottom

line if you do not want a form feed.  Only a CR will work well

down there to eliminate the forced form feed.  The CR can be

generated via the bar code routine using CALL BARS(CR$).  Note

that the CR$ constant is set up in line one.

The X & Y dimensions are in 300 dots per inch.  They are relative

to the XS & YS which start from TM & LM.  Bar code density can be

changed with the code at UNIBARxx.BAS line 120.  Height at line

130.  The main menu causes jumps to these entry points when you

enter menu choices 2 or 3.  The other code can be found the same

way, via menu choices and jumps.  The main menu starts at line

30.

If you enter  GORLOD  the batch file should load all the

necessary the Swfte Glyphix Soft Fonts that are required for

complete printing of bar codes and text via menu choice 6 and the

DATA5xxx test files on the disk.  Resetting the printer after

loading fonts is the best approach.  You will have to know how to

reset the printer and what exactly happens with control panel

functions to use the printer effectively.  If you reset via the

program, a label sheet will get ejected.  The sheet front may

curl from heat and be difficult to feed thru again.  If you get

good at de-curling sheets, you can overcome this problem.  The

other approach is to put a plain setup sheet at the top.  Best

you print and look at the GORLOD.BAT file.  Also you may have to

reset the printer if the wrong fonts are printing or if some

other problem pops up.  We use permanent download fonts so they

will not get erased if you reset the printer.

You can transfer the files to any disk or directory you choose,

however, the program files will have to be on the default drive

disk and in the same directory to run.  Also, the computer will

have to be set via the Change Directory, command to that

directory.

When you get involved with multiple placement of bar codes on a

sheet, you will soon run into a bug in HP printers and possibly

some others.  This will be kindly referred to as a bug, but we

believe it was put in the logic control to keep you from using

bar code programs other than those you would have to buy from

them.  It also appears that most other laser printers were

designed by the same group of people.  What happens is, that when

the printer detects you are printing bar codes, it will place

some of them, or/and text, at a position other than that which

you have programmed.  To get around this, all you have to do is

add some CR and LF codes in the troubling area of your program.

By the way, the HP technical people told us we could not get

around this.  Our FIX is used in all the programs, and is 100%

reliable.

If you ordered the SWFTE variable font generator option, you will

have to read the manual to find out what is going on.  Call if

you get hung up.  You should notice that we use SWFTE Glyphix

routine DOWNLOAD.COM to load fonts.  This is most convenient once

you have settled on the font types and sizes to use.  For testing

you can generate and load fonts by hand.  Later put them in a

library and download them automatically with a batch file similar

to GORLOD.BAT.  Keep in mind that large fonts take up a lot of

memory.  It is best to store them on disks rather than use up

your hard disk memory.  We can provide a selective loader that

will load only the groups of characters you need.  It is a bit

more expensive, but it is an alternative to filling up your disk

memory or buying additional memory.

We have to treat SWFTE right.  They are very nice people and have

made variable size font generation and control affordable to us

all.  NO bootlegging their programs.  Their disk and program

license are assigned to you, our customer, via our invoice.  If

you want more of their fonts, or some of the others we carry,

give us a call.  Font test disks are available that print font

designs that can be generated with the variable font generators.

There are no provisions to set MODE commands to print via serial

ports.  That will have to be done by you.  The serial port should

work OK if set properly, but it will be a lot slower.  One of our

customers reports that the programs work fine in landscape mode

when set at the control panel.  A word of caution, the accuracy

of the bar codes and placement will be less in that mode, so use

the largest density you can to reduce any paper movement errors

that cause bar widths and spacings that are out of tolerance.

Try the straight-thru paper path when printing thick labels.  The

paper stacking top return path can cause excess curling of the

label stock and feeding problems.  A good side effect is, by

keeping the rear door open, the printer has a better chance to

get rid of excess heat in the fuser section.  Do the same for all

thick or tuff stock.  The curling problem can cause paper jams

when you try to run the sheets again.  This can be corrected by

hand opposite curling.  You can also run paper thru backwards

with very few problems, as the curl is usually at the front of a

sheet.

You might want to avail yourself of our services in the area of

toners, label stock and sheet overlay protection.  You could

spend years in this area and still have a lot to learn.  Possibly

for other equipment, services and software too.

We have added PRERSUB.BAS to the disk.  It is one of the most

valuable programs ever written for BASIC programmers that use

printers.  A more sophisticated and expanded version is in

UNIBARxx.BAS starting at line 2000.  If you feel rich, send us a

few thousand for the convenience.  The main problem with Basic is

that it uses the BIOS print routines and On Error Goto routines

to handle printing.  When the world crashes down upon you as your

printer runs out of paper, or when your laser printer has a bad

feed, is busy, off line, out of paper, not turned on, etc., the

BIOS routines will usually time out and kick you back into DOS

before you can correct the situation.  Also, you quite often lose

data in the process and have to start over.  You will note that

we include the ability to start or restart any place in the file

printing routines in order to alleviate this problem somewhat.

Many programs do not include provisions for you to start in the

middle of a print routine to recover from this problem.  The on

error routines in Basic are helpful to a point, but, to the best

of our knowledge, there is no easy way to get around the timeout

problem completely, except to check printer status before each

character goes to the printer.  In many cases this slows down the

printer data transfer.  Try it, study it, learn printer control,

modify it and be happy ever after.

You can test the status check subroutine by using the simplified

version on the disk, PRERSUB.BAS.  It has direct printing via

printer ports.  Entry at line 2050 will add a CR and LF to the

print string DA$, line 2052 just adds the CR and line 2054 just

prints the string DA$ as is.  This routine is set up via line

2000.  It can be a very valuable tool for learning printer

control.  There is more documentation at the front of the code.

When we added this subroutine, we put in variable time out

control.  You may see SC1=xx  SC2=xx  TDL!=x on the lower right

of the screen.  The counts for these can be set from the main

menu.  We set SC1 to 1000 and SC2 to 1 at the start.  Try doing

some printing with the printer off line, etc. and you will see

the counters work before they shut you down.

We have tried to add a minimum of comments to the control code to

avoid messy confusion.  If any of your people see something that

we should change, kindly let us know.  We very much appreciate

feedback and suggestion.

We have found a bug which seems to cause unpredictable operation

when you try to put comments following  a GOSUB command.  If the

last instruction is a GOSUB, put the comments on the entire line

above and refer to the GOSUB on the next line.

Kindly send us any suggestions or critical comments you have.

GOOD LUCK from us all

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 2,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

1-800-2424-PSL

MC/Visa/AmEx

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.