******************************************************************

DESIGN CENTER - 6.0 RELEASE

******************************************************************

This file describes enhancements and modifications made to the 6.0

release of the Design Center programs.

Note that this file provides information for all platforms supported by

the Design Center programs.  For this reason, some of the information

may not apply to your system.

******************************************************************

CONTENTS

******************************************************************

Part 1.  New Features	2

Part 2.  General	3

2.1.  File Name Changes	3

2.2.  Environment Variable Name Change	3

2.3.  Version Compatibility -- Schematics	3

2.4.  Cautions for PC and Windows Network License Users	3

2.5.  Command Line Option: -i	3

2.6.  Using Extended DOS Versions Under Microsoft Windows	4

Part 3.  Installation Considerations	4

3.1.  Extended DOS Versions	4

3.2.  Network Versions	4

3.2.1.  Definitions	4

3.2.2.  Specifying Local Networks with HASPSERV.NLM

(i.e., on file servers)	4

3.2.3.  Specifying Local Networks with HASPSERV.EXE

(i.e., on local network PCs)	5

Part 4.  PSpice and PLogic	5

4.1.  New PLogic Digital Simulator	5

4.2.  Using FILE Specification in PWL Forms	6

4.3.  Noise Immunity Behavior Modeling for Digital Devices	6

4.4.  Digital Library Updates	6

4.5.  Intel Pentium(TM) IBIS Models	7

Part 5.  Probe	7

5.1.  Probe Accelerator Keys (Windows and OpenWindows only)	7

5.2.  Marching Waveforms (Windows and OpenWindows)	7

5.3.  Timing Violations and Error Tracking (Windows only)	7

Part 6.  Schematics	8

6.1.  User Interface Changes (Windows and OpenWindows)	8

6.1.1.  Menu Bar Changes	8

6.1.2.  Setup Block and View Functionality	8

6.1.3.  Edit/Model Changes	9

6.2.  PLogic Digital Simulator Integration	9

6.3.  The VIEWPOINT Symbol	9

6.4.  Creating Symbols/Netlisting--Illegal Characters	9

6.5.  App Settings Dialog for Application Settings	9

6.6.  Windows Stimulus Editor Integration (Windows only)	9

6.7.  Reconfiguring Symbol and Package Libraries	9

6.8.  Symbols	9

6.8.1.  New "misc.slb" Symbol - "555alt"	10

6.8.2.  Breakout Symbol Name Change - "DBreak3"	10

6.8.3.  Analog Devices with Visible Pin Numbers	10

6.8.4.  The "dig_prim.slb" Symbol Library File	10

6.8.5.  Obsolete Symbols	10

6.9.  Hierarchical Symbols and Blocks	10

6.10.  Pin Name Translation Changes	10

6.11.  The "msim.ini" File	10

6.11.1.  New [Schematics Colors] Entries	10

6.11.2.  New [MicroSim] Entries	11

6.11.3.  New [Schematics] Entries	11

6.11.4.  "Marker.slb"	12

6.12.  Using European Parts	12

6.13.  Font-Related Issues	12

6.13.1.  Using the RASTERFONT=ON/OFF Switch	12

6.13.2.  Using Adobe Type Manager with Schematics	12

Part 7.  Control Shell for Extended DOS Versions	12

7.1.  New Features	12

7.2.  Features Not Supported in the 6.0 Control Shell	13

7.3.  Known Anomalies	13

Part 8.  Stimulus Editor	14

8.1.  Stimulus Editor for Windows	14

8.1.1.  Creating a STIM	14

8.1.2.  Adding a STIM	14

8.2.  New Stimulus Editor Circuit File Commands	15

8.2.1.  Stimulus Library Files (.STIMLIB)	15

8.2.2.  Stimulus Definition(.STIMULUS)	16

Part 9.  Polaris Signal Integrity	16

Part 10.  Intel IBIS Model Support	16

Part 11.  New Libraries and Devices	16

11.1.  New Transmission Line Models	16

11.2.  New Comlinear Model	17

11.3.  Library Name Changes - "nom.lib"	17

Part 12.  Parts Program	17

12.1.  New Voltage Reference Template	17

12.2.  Miscellaneous Changes	17

12.3.  Mouse Support	17

12.4.  Preservation of the MDT file in case of error	17

Part 13.  MODTOMDT Utility	18

Part 14.  Filter Designer	19

******************************************************************

Part 1.  New Features

******************************************************************

DESIGN CENTER

* Mentor integration for HP and Sun systems

* Expanded libraries now with over 9000 devices, including over 1,700

Japanese devices

* New PLogic digital-only circuit simulator

* PSpice availability for analog-only and mixed-signal circuit

simulations

SCHEMATICS (Microsoft Windows and Sun OpenWindows platforms)

* Extended support for the SCICARDS external board layout package and

EDIF 2 0 0 netlist format

* Stand-alone Schematics evaluation package

* Global and local configuration of symbol and package library files

* Support for the PLSyn Programmable Logic Synthesis package

* Improved pan and zoom functions

* Attribute editing on a global or multiple-part basis

* Integrated stimulus editing within the Schematics environment (Windows

only)

* New symbols for external input, output, and bi-directional ports

to identify peripheral circuit connections, and facilitate persistent

hazard identification and tracking, which is a new feature of Probe

(Windows only)

* Menu naming conventions consistent with Probe and PSpice

PSPICE/PLOGIC

* Simulation status display showing the three fastest transmission lines

* New .EXTERNAL command which provides a means of identifying and

distinguishing those nets representing the outermost (or peripheral)

connections to the circuit

* The .STMLIB command which makes stimulus library files available to

PSpice and PLogic

* The .STIMULUS command which allows you to define the source devices

(V, I, or STIM).  The .STIMULUS definition encompasses only the

Transient specification portion of what is allowed in the V, I, or

STIM device syntax

* Improved noise immunity modeling for digital devices

CONTROL SHELL

* New Control Shell user interface for Extended DOS platforms

* Integrated access to PSpice, PLogic, Probe, Parts, and StmEd

* Improved printing functions

* Interactive analysis specification through dialogs and prompts

PROBE

* Append function for combining trace data from multiple data files

* Marching waveforms on the OpenWindows platform

* Improved pan and zoom functions

* Timing Violations and Error Tracking (Windows only)

STIMULUS EDITOR

* New native Windows version

PARTS

* New Voltage Reference Template

* Improvements to the MOSFET template

POLARIS

* Support for TangoPro board layout package

******************************************************************

Part 2.  General

******************************************************************

2.1.  File Name Changes

The file "pspice.iff" used in previous versions has been replaced by the

file "msim.iff" in this release.

The Windows PSpice executable file name has been changed from

"wpspice.exe" to "pspice.exe."

2.2.  Environment Variable Name Change

The environment variable PSPICELIB used in previous versions has been

replaced by the environment variable SIMLIBPATH in this release.

2.3.  Version Compatibility -- Schematics

Warning:  Schematic and/or Symbol Library files which are created and/or

modified using the 6.0 version of Schematics may not be backward

compatible with previous versions of Schematics.  That is, if you have a

5.3 schematic that you modify using the 6.0 version of Schematics, you

may not be able to read that schematic back into 5.3 or 5.4 Schematics.

Similarly, if you create new symbols using the 6.0 Symbol Editor, these

symbols may not be compatible with previous versions of Schematics.

2.4.  Cautions for PC and Windows Network License Users

If you try to invoke PSpice/PLogic, but a license is not available, the

simulator may delete or inadvertently overwrite any previous output

(.out) and data (.dat) files you may have had for that circuit.

Please be aware of this and copy any required output and/or data files

to other names before subsequently simulating.

2.5.  Command Line Option: -i

PSpice, Probe, the Stimulus Editor (Windows version) and Schematics

support a -i command line option which allows you to specify a

configuration file other than "msim.ini."

To use this option, you can change the command line associated with the

program's icon to be:

<program name>  -i <new_file_name>.ini

If no path is specified for the new ".ini" file, it is assumed to be in

your Windows directory (on the PC) or in your home directory (on the

Sun).

From Schematics, an initialization file can be configured through the

Options/Editor Configuration/App Settings dialog.  The initialization

file configured here will only be used by programs invoked from within

Schematics.  (The file specified by the Options/Editor Configuration/App

Settings dialog is written to the "inifile" specification within the

[Schematics] section of the "msim.ini" file.)

2.6.  Using Extended DOS Versions Under Microsoft Windows

The Probe "Mid-Analysis Snoop" feature will not work if PSpice (PLogic)

is being run from within a DOS "Window" (under Microsoft Windows).  The

symptom is an error message,

"DOS/16M error: [32] DPMI host error (possibly insufficient memory)"

To avoid this problem, run PSpice (or PLogic) directly from the Control

Shell or the DOS command line, not under Windows.

******************************************************************

Part 3.  Installation Considerations

******************************************************************

3.1.  Extended DOS Versions

The environment variable PSPICELIB used in previous releases has been

replaced by the environment variable SIMLIBPATH in this release.  This

variable can be set from the DOS command line, in your "autoexec.bat"

file, or from within the Control Shell using the Analysis/Analysis

Options dialog.

3.2.  Network Versions

The following information applies to the use of NetHASP on Novell

networks consisting of several interconnected networks.

3.2.1.  Definitions

Internetwork:  a series of two or more networks that are linked via

internal or external routers.

Local network:  a single cabling scheme identified by a unique network

number to which one or more stations are attached.

By default, when HASPSERV is loaded, it advertises its name to the

entire internetwork and serves all of its networks and stations.

However, you can instruct HASPSERV to serve only a specific local

network.

3.2.2.  Specifying Local Networks with HASPSERV.NLM (i.e., on file

servers)

To instruct HASPSERV.NLM to serve a local network, use the following

command:

LOAD HASPSERV LOCALNET

If the file server is a "bridge," then HASPSERV will serve only those

stations which are part of the networks connected directly to this

"bridge."

To specify the network numbers of those networks with which HASPSERV

should advertise its name:

LOAD HASPSERV NET m [NET n ...]

where "m" and "n" are network numbers on the internetwork.

Note:  Although HASPSERV advertises its name with specific networks, it

will serve all of the stations in the entire internetwork.  To instruct

HASPSERV to only serve the specified networks, use the LOCALNET switch:

HASPSERV NET 111 NET 222 LOCALNET

SET NETHASPPROTOCOL=LOCALNET ensures that the station will access only

those HASPSERVs which advertise their names with the station's network

number.  The station will access a HASPSERV.NLM which was loaded with

the "NET n" switch, where "n" is the local network number of the

station.

3.2.3.  Specifying Local Networks with HASPSERV.EXE (i.e., on local

network PCs)

To instruct HASPSERV.EXE to serve a local network:

HASPSERV -LOCALNET

SET NETHASPPROTOCOL=LOCALNET ensures that the station will access only

the HASPSERV.EXE which advertises its name with that station's local

network.

******************************************************************

Part 4.  PSpice and PLogic

******************************************************************

4.1.  New PLogic Digital Simulator

PLogic is available for the Windows and OpenWindows schematic capture

environments, and for the Extended DOS with Control Shell environment.

PLogic is a digital-only circuit simulator that analyzes timing behavior

during a transient analysis.  Detailed analysis is conducted subject to

the timing constraints (e.g., setup/hold times and min/max propagation

delay ambiguities) specified for individual devices.

PLogic operates identically to PSpice.  In the Design Center environment

with Schematics, a schematic is prepared for simulation from which

Schematics generates a circuit file set.  In circuit file entry

environments, the circuit file is created directly through the Control

Shell or a text editor.  The circuit file set (or circuit file)

containing the circuit netlist and analysis commands is read by PLogic

for simulation.  PLogic calculates the states of the nodes connected to

digital devices.  Any timing violations or hazards are reported by

PLogic in the form of messages written to the simulation output file and

to the Probe data file.

With the Design Center for Windows, the results are formulated into

meaningful graphical traces in Probe which can be marked for display

directly from your schematic.  Probe's messaging feature (Windows only)

allows you to automatically display digital signals and buses along with

detailed message text associated with a given timing violation or

hazard;  trace segments with diagnostics are visually tagged.

PLogic analyses can be classified as standard and simple multi-run.  For

more information on running a particular analysis as well as specifying

analysis parameters and output, refer to the Circuit Analysis User's

Guide provided with your system.

4.2.  Using FILE Specification in PWL Forms

When using the FILE keyword within a PWL (Piece-Wise Linear)

specification, be aware that anything following the file name is

ignored.  For example, if you are using the FILE form with a

REPEAT...ENDREPEAT loop, the ENDREPEAT must appear on a continuation

line if it follows the file name.

The following examples illustrate correct and incorrect forms:

Correct:

I3   5,6   PWL   REPEAT FOR 5 FILE DATA1.TAB

+ ENDREPEAT

Incorrect:

I3   5,6   PWL   REPEAT FOR 5 FILE DATA1.TAB ENDREPEAT

Correct:

I4   7 8   PWL   TIME_SCALE_FACTOR=0.1

+   REPEAT FOREVER

+   REPEAT FOR 5 (1,0) (2,1) (3,0) ENDREPEAT

+   REPEAT FOR 5 FILE DATA1.TAB

+   ENDREPEAT

+   ENDREPEAT

Incorrect:

I4   7 8   PWL   TIME_SCALE_FACTOR=0.1

+   REPEAT FOREVER

+   REPEAT FOR 5 (1,0) (2,1) (3,0) ENDREPEAT

+   REPEAT FOR 5 FILE DATA1.TAB ENDREPEAT

+   ENDREPEAT

NOTE:  The REPEAT ... ENDREPEAT specifications may be nested to any

depth.

4.3.  Noise Immunity Behavior Modeling for Digital Devices

PSpice A/D and PLogic now model the noise immunity behavior in digital

devices.  By setting the Pulse Width Rejection Threshold (PPWRT)

parameter in a digital device's I/O Model, the simulator will

appropriately generate a glitch when the pulse width is greater than or

equal to TPWRT and the pulse width is less than the propagation delay.

Refer to the Circuit Analysis User's Guide for additional information.

4.4.  Digital Library Updates

The Digital component libraries, DIG_1.LIB, DIG_2.LIB, DIG_3.LIB,

DIG_4.LIB and DIG_5.LIB, have been modified to support the new "Embedded

Constraint" form of PINDLY device syntax (in those models that use both

PINDLY and CONSTRAINT devices).  These changes do not affect the logic

operation of the models, but they support the new Probe "Error Tracking"

functionality (offered with Windows-based packages at this time).

The new syntax, as of release 6.0, represents the preferred modeling

style for devices of this type; the old syntax (i.e., "separate"

PINDLY/CONSTRAINT devices) will still work (logically), but will

generally not provide correct results with respect to "Error Tracking"

behavior.

4.5.  Intel Pentium(TM) IBIS Models

A library of Intel's Pentium (TM) 82430 (Mercury) PCIset IBIS models is

now available.  These highly accurate driver/receiver buffer models

include effects such as switching times, nonlinear transfer

characteristics, and package parasitics.  These allow you to perform

accurate signal integrity simulation of I/O interconnects on printed

circuit boards.  The models are located in the "buffer.lib" file.

******************************************************************

Part 5.  Probe

******************************************************************

5.1.  Probe Accelerator Keys (Windows and OpenWindows only)

The following accelerator keys have been added to Probe:

<CTRL X>    EDIT/CUT

<CTRL C>    EDIT/COPY

<CTRL V>    EDIT/PASTE

5.2.  Marching Waveforms (Windows and OpenWindows)

Marching Waveforms was supported for Microsoft Windows in release 5.4,

and remains unchanged for 6.0.  Support for OpenWindows is now included

in 6.0 as well.

5.3.  Timing Violations and Error Tracking (Windows only)

When there are problems with your design, such as setup/hold violations,

pulse-width violations, or worst-case timing hazards, PSpice and PLogic

will log messages to the simulation output file and/or Probe data file.

Using Probe (Microsoft Windows only), messages can be selected causing

associated waveforms and detailed message text to be automatically

displayed.

In the digital domain, the types of problems identified by the simulator

are generally classified as either timing violations or timing hazards.

Timing violations include SETUP, HOLD, and minimum pulse WIDTH

violations of component specifications.  The occurrence of such a

violation may result in a change in the state behavior of the design,

and potentially the answer.  However, the effects of many of these

errors are short-lived and don't influence the final circuit results.

PSpice and PLogic now identify any digital net that may carry a possibly

invalid state value due to a timing hazard; Probe identifies digital

traces of these nets by highlighting them with a different color.  The

color difference exists for the duration of the potential logic

difference due to the timing hazard.  Changes in a trace's color may or

may not coincide with actual state changes, depending on the nature of

the timing violation.  In Probe, when a digital trace is highlighted in

this manner, double-click on the actual highlighted portion of the trace

to see a Simulation Message pop-up dialog; clicking on "OK" will then

bring up a new window containing both the message text and a plot of the

signals involved in the hazard.

Timing hazards are most easily identified by simulating a design in

worst-case timing mode, usually close to its critical timing limits.

Under such conditions, the simulator will report things such as

AMBIGUITY CONVERGENCE hazards.  Again, these may or may not pose a

problem to the operation of the design.  However, there are identifiable

cases that cause major problems.

The types of error conditions reported by the simulator are summarized

in the "Simulation Condition Messages" section of the "PSpice Analyses"

chapter in the Circuit Analysis User's Guide.

******************************************************************

Part 6.  Schematics

******************************************************************

6.1.  User Interface Changes (Windows and OpenWindows)

6.1.1.  Menu Bar Changes

To maintain consistency with the other Design Center programs, some

minor changes have been made to the commands in the Menu Bar:

Old Command            New Command

--------------------------------------------

Zoom                   View

Zoom/Normal            View/Fit(*)

Configure              Options

Edit/Search            Edit/Find

Analysis/Run PSpice    Analysis/Simulate(**)

(*)The View/Fit command replaces the Zoom/Normal command, although the

functionality is somewhat different.  Refer to the Schematic Capture

User's Guide for details.

(**)The simulator (PSpice or PLogic) is selected through the

Options/Editor Configuration/App Settings dialog.

Opening a schematic (either via File/Open or Navigate/Push) now does an

automatic View/Fit to adjust the zoom factor so that all wires, parts,

etc. on the schematic are visible.

The View/Area command now deselects the viewed area after zooming.

6.1.2.  Setup Block and View Functionality

The Edit/Setup Block command was removed and functionality combined with

the Edit/Views command.

Thus, for 6.0, to change the schematic associated with a block, the

Edit/Views command should be used.  The value of the DEFAULT view is the

default schematic name.  The command can also be used to add, delete, or

display all views and schematics associated with a block.

If a hierarchical symbol is chosen (as opposed to a block), Edit/Views

will display the list of views but not allow editing.  Views for

hierarchical symbols can only be changed in the Symbol Editor.

6.1.3.  Edit/Model Changes

When editing a model definition, <ENTER> should now be used instead of

<CTRL-ENTER> to go to the next line.

6.2.  PLogic Digital Simulator Integration

The Analysis/Setup command will bring up a different dialog depending on

whether PSpice or PLogic is configured as the simulator.  For PSpice

(analog and mixed A/D), the Analysis Setup dialog provides a new Digital

Setup button.  For PLogic, the Analysis/Setup command displays only the

digital analysis options dialog.

6.3.  The VIEWPOINT Symbol

If you are using PLogic, the VIEWPOINT symbol will not show any

information.

6.4.  Creating Symbols/Netlisting--Illegal Characters

Do not use the "%" character in names that will appear in the template

for PSpice.  Also do not use "%" in the value of an attribute which must

be substituted in the template by the netlister (e.g., DEVTOL=@DEVTOL

where DEVTOL=10%).  This will cause the netlister to emit erroneous "No

such pin" errors.

6.5.  App Settings Dialog for Application Settings

The Options/Editor Configuration dialog has been changed to include

another button, labeled "App Settings...".  This button displays a

dialog which allows the user to specify the simulator (PSpice or PLogic)

and the command lines used to invoke the simulator, Probe, and StmEd.

(The SUN version of Schematics will not have the StmEd command line

field).

6.6.  Windows Stimulus Editor Integration (Windows only)

The Analysis/Library and Includes dialog now includes Stimulus Library

files.

Using the symbols VSTIM, ISTIM, and DIGSTIM (from the "source.slb"

symbol library) on your schematic allows you to define voltage, current

and digital stimulus using the new Windows Stimulus Editor.  Refer to

the Schematic Capture User's Guide, and the Circuit Analysis User's

Guide for complete details.

If you edit a stimulus using the Stimulus Editor, be sure that any

changes are saved (using the File/Save command in the Stimulus Editor)

prior to doing your simulation if you want these changes included in the

simulation.

6.7.  Reconfiguring Symbol and Package Libraries

Prior to 6.0, if the list of libraries was changed via Options/Editor

Configuration, then all libraries in the list would be re-read.  For 6.0

this has been sped up by recognizing which libraries were previously

loaded and only reading new libraries.  This change means that you

should never replace symbol and package libraries manually (using the

DOS copy command, for example) while Schematics is running.

6.8.  Symbols

Local symbol library support has been added.  You can now associate

symbol libraries with a particular schematic.  These symbol libraries

will be loaded when that particular schematic is loaded.  This is done

by selecting the Options/Editor Configuration/Library settings and

selecting the new Add Local button.  Local libraries are always searched

prior to global libraries (those loaded for all schematics).  For a

hierarchical schematic, only the local libraries associated with the

topmost schematic are used.

6.8.1.  New "misc.slb" Symbol - "555alt"

The 555alt symbol has been added to "misc.slb."  This symbol is intended

to provide a more functional pin configuration than the other 555

symbols (i.e. 555B, 555C, and 555D).  The 555alt symbol references the

555C model, which is currently available for use with the Mixed A/D and

Analog-only products from MicroSim.

6.8.2.  Breakout Symbol Name Change - "DBreak3"

The name of D3Break symbol has been changed to DBreak3.

6.8.3.  Analog Devices with Visible Pin Numbers

A symbol library file, "analog_p.slb," is shipped for those who wish to

have visible pin numbers on the analog devices R, L, C, R_VAR, and

C_VAR.  The "analog_p.slb" file must be configured by adding it to the

list of configured symbol library files before or in place of the file

"analog.slb."

6.8.4.  The "dig_prim.slb" Symbol Library File

The symbol library file "dig_prim.slb" is provided primarily for users

of PLSyn Programmable Logic Synthesis.  The IMPL attribute for each of

these symbols is set to PLSYN, and the timing delay is set to zero.  If

you are not using PLSyn but want to use these symbols, Use the Edit

Attributes command to remove the PLSYN assignment from the IMPL

attribute.  (In other words, set the IMPL= value to a blank.)

6.8.5.  Obsolete Symbols

The "dig_old.slb" symbol library file is included for compatibility with

earlier releases. The symbols in this file are considered obsolete and

have been replaced by new symbols in other library files.  These symbols

are provided for compatibility, only, and should not be used for new

schematics.

6.9.  Hierarchical Symbols and Blocks

Hierarchical symbols and blocks now use a new default reference

designator (REFDES) to avoid conflicts with packaging.

The default REFDES for hierarchical blocks is now HB?.

The default REFDES for hierarchical symbols is now HS?.

Pin numbers are now P1, P2, etc. instead of 1, 2, etc.

6.10.  Pin Name Translation Changes

Within pin names, the characters "<," ">," and "=" are now replaced by

"GT," LT," and "EQ," respectively.  (In 5.4 and prior, these characters

were replaced by "G," "L," and "E" respectively.)

6.11.  The "msim.ini" File

Note: You cannot use tabs within the "msim.ini" file.

6.11.1.  New [Schematics Colors] Entries

ENTRY     DESCRIPTION

-------------------------------------------------------------------

plsyn     specifies the color of PLSyn parts and blocks

(IMPL=PLSYN or blocks with a DSL view)

stimulus  specifies the color of StmEd stimulus parts

(those with a STIMULUS attribute)

6.11.2.  New [MicroSim] Entries

ENTRY       DESCRIPTION                         DEFAULT

-------------------------------------------------------------------

plsyncmd    Command to invoke PLSyn.           plsyn

stmedcmd    Command to invoke StmEd.           stmed

polariscmd  Command to invoke Polaris.         polaris.exe /FT /RC

plogiccmd   Command to invoke PLogic.          plogic

backup      Previously in [Schematics] section

6.11.3.  New [Schematics] Entries

ENTRY              DESCRIPTION                             DEFAULT

-------------------------------------------------------------------

viewfitpercent     See description of View Fit below

lastfile1-4        Most-recently used Schematic files

lastslb1-4         Most-recently used Symbol Editor

files

plsyn              ON adds Tools/PLSyn command to           OFF

menu, and shows PLSyn Setup button

on the Analysis Setup dialog.

polaris            ON adds Tools/Polaris to menu.           OFF

simulator          PSPICE or PLOGIC.  Set using Options/

Editor Configuration/App Settings...

inifile            Name of initialization file to use when invoking

other Design Center Apps (PSpice, PLogic, StmEd,

Probe).  If this is left blank, the default

"msim.ini" file is used.  Can be changed through

the Options/Editor Configuration/App Settings.

A file specified through one of the program-

specific commands in the [MicroSim] section

(such as pspicecmd, probecmd, stmedcmd, etc.)

will take precedence over the file specified

with the inifile command setting.

usescicardsp2      Flag to govern use of P2 record when       ON

netlisting in SCICARDS format.

pcbhierpathpsep    Path separator character for PCB           " - "

netlists;  the hierarchical path is added

to named nets of subschematics to

identify them uniquely.  The character

"." had been used previously.

6.11.4.  "Marker.slb"

"Marker.slb" should not be included in the list of libraries.  If it is,

you will get a warning when you start Schematics:

"marker libraries should not be in configured list."

This is just a warning; you can continue to use Schematics.

6.12.  Using European Parts

There are symbol and simulation model libraries available for European

manufacturers' components.  If you want to use these parts, you need to

add "europe.lib" to the list of globally available libraries AHEAD of

"nom.lib" using the Analysis/Library and Include Files menu command.

You also need to reconfigure the symbol and package libraries using

Options/Editor Configuration/Library Settings to add "ebipolar,"

"ediode," and "epwrbjt" AHEAD of the other libraries.  Remember to check

the "plb" setting as well as "slb."

6.13.  Font-Related Issues

6.13.1.  Using the RASTERFONT=ON/OFF Switch

If the line RASTERFONT=ON is included in your "msim.ini" file, it causes

the Helvetica font to be used instead of the modern font for display in

Schematics.  This will cause rotated text to appear vertically versus

being rotated on its side.  If your symbols and schematics use rotated

text, setting this item to ON may cause problems with positioning text.

For this reason, we suggest using the RASTERFONT=OFF switch setting.

6.13.2.  Using Adobe Type Manager with Schematics

If you have Adobe Type Manager installed and are using Schematics, you

may encounter problems with how rotated text is displayed.  We recommend

that you edit the "ATM.ini" file and comment out the line

Modern=Helvetica

in the [ALIASES] section.  If this is not done, rotated text may be

positioned differently on the display versus on hard copies.  We also do

not recommend using the FONTTYPE=ADOBE setting in your "msim.ini" file

at this time as it has the same problem.

******************************************************************

Part 7.  Control Shell for Extended DOS Versions

******************************************************************

7.1.  New Features

The 6.0 Control Shell provides the following features:

*  A menu-driven environment for setting up your netlists, circuit

files, and the analysis parameters

*  32-bit implementation

*  Integrated Help, with entries for the commands supported

by PSpice and PLogic

*  Command entry through function keys, pull-down menus, or buttons

for the most frequently used functions

*  Availability of multiple editing and browse windows

*  Context-sensitive help for menus and dialogs

*  Syntax checker for circuit files

*  Printing functions, including multiple-copy output and page

customization options

*  File size is no longer limited to 32KB (All of the available free

RAM and disk space may be used.)

The above features are described in detail in the Circuit Analysis

User's Guide and the Circuit Analysis Reference Manual.

7.2.  Features Not Supported in the 6.0 Control Shell

1.  The External Editor command (e.g. for calling OrCAD's SDT) is not

currently supported.  Use the File/DOS Shell menu command to "call"

other DOS-based programs.

2.  The use of the ";" character for user comments on a command line is

not supported.  Comments must appear on a separate line which begins

with the "*" character.

3.  Command pairs such as .ac/.noise and .tran/.four will appear

together when set up or altered using the Control Shell Analysis Setup

dialogs.  These command pairs cannot be separated by comment lines.

4.  As with previous versions of the Control Shell, you cannot have more

than one circuit description in a circuit file.

5.  Global parameters (.param) cannot be edited through dialogs.  You

must manually define .param parameters in the edit window.

7.3.  Known Anomalies

The following problems and/or restrictions are inherent in the software

tools used to create the Control Shell, and are currently beyond our

control.

1.  Positioning of the text within the edit window is spurious when some

of the dialogs are invoked, used, or closed.

2.  Cursor positioning on a line in the edit window drifts to the right

when the mouse is used.  The farther to the right on the line that the

mouse pointer is clicked, the more the cursor position drifts.

3.  If a line in the circuit file contains a space character in the

first column, spurious "garbage" characters may be inserted when new

characters are entered in column one. These characters may be manually

deleted by the user.

4.  During a Replace operation, the target text is not highlighted.

Instead, a solid block character (similar to a cursor) is placed in

front of the selected text.

5.  Various anomalies have been observed when using the Control Shell in

a DOS window under Microsoft Windows.  For best performance under

Windows, use the ".pif" file provided with the Control Shell.  If you

encounter problems, try running the Control Shell directly from DOS.

******************************************************************

Part 8.  Stimulus Editor

******************************************************************

8.1.  Stimulus Editor for Windows

The Stimulus Editor (StmEd) program has been rewritten as a native

Windows-based program for the Microsoft Windows platform.  The 6.0

version is tightly integrated with Schematics through new voltage,

current, and digital stimulus symbols.

The following sections provide information on digital stimulus editing

which is not included in the Circuit Analysis User's Guide:

8.1.1.  Creating a STIM

When creating a STIM, a dialog appears which allows you to define

the following parameters of the STIM:

1.  Bus Width - the number of individual signals which comprise the bus.

STIMs that are one bit wide are refered to as "signals" while those that

are more than one bit wide are refered to as "buses."

2.  Time Step - allows the STIM to be rescaled in time by changing a

single value.  This is analogous to the TIME_SCALE_FACTOR in PWLs.  All

transtions added to the STIM when a Time Step has been defined will

automatically be recorded internally in terms of the Time Step.

3.  The format for the bus - "Binary," "Octal," "Hex," and "Other."

This defines how the values for the bus appear in the plot and any

applicable dialogs.  For "Binary," "Octal," and "Hex," the user selects

the Bus Width and StmEd determines the proper Format Array.  (See

Section 5.1.2.1 of the Circuit Analysis Reference Manual for more

information on Format Arrays.)

When "Other" is selected, the user specifies the Format Array and StmEd

determines the Bus Width.  Additionally, selecting "Other" causes the

bus to display in decimal.

8.1.2.  Adding a STIM

After a STIM has been created (or whenever it is being edited) the Add

option on the Edit menu is enabled and given a submenu.  The selections

on the submenu vary depending on whether the STIM is a signal or a bus.

The following selections are available for a signal:

1.  To 0/1 - add a single transition to either zero or one.  If the

value is 0 or it is X or Z and the following value is 0, then the new

transition is to one.  Otherwise the new transition is to 0.  Repeatedly

clicking while moving the mouse along the X-axis will create a digital

pulse.

2. To 0R1F - adds a single transition, cycling through the states 0, R,

1 and F.  The value for the new transition is determined from the values

of the states on either side.  Repeatedly clicking while moving the

mouse along the X-axis will create a digital pulse with rise/fall times.

3.  To X - inserts a transition setting the STIM to the unknown

state (X).

4.  To Z - inserts a transition setting the STIM to the high impedance

state (Z).

The following selections are available for a bus:

1.  To Value - add a transition to a specified value.  When this option

is selected, a dialog pops up so the user may set the value of the

transition that will be added.  Additional transitions will have the

same value until the user right-clicks the mouse.  The initial default

is zero and StmEd remembers the last value used in a given session of

editing a STIM.

2.  Incr by - add an INCR BY x command.  When this option is selected, a

dialog pops up so the user may set the amount of the increment.

Additional transitions will use the same amount until the user right-

clicks the mouse.  The initial default is one and StmEd remembers the

last value used for INCR/DECR in a given session of editing a STIM.

3.  Decr by - add an DECR BY x command.  When this option is selected, a

dialog pops up so the user may set the amount of the decrement.

Additional transitions will use the same amount until the user right-

clicks the mouse.  The initial default is one and StmEd remembers the

last value used for INCR/DECR in a given session of editing a STIM.

4.  To X - inserts a transition setting the bus to the unknown

state (X).

5.  To Z - inserts a transition setting the bus to the high impedance

state (Z).

Pressing Alt-A repeats the most recent Add command for a STIM.  For

example, to make an eight bit bus that varies between the two values 00

and FF, add all the transitions to 00 and then right-click.  Next, type

Alt-A and set the new value to FF and add the other transitions.

If Alt-A is pressed before any Add command is selected from the menu,

after going into edit mode on a given STIM, it will act as a "To 0/1"

for a signal, and as a "To Value" for a bus.

8.2.  New Stimulus Editor Circuit File Commands

8.2.1.  Stimulus Library Files (.STIMLIB)

General Form:     .STMLIB <file name[.stl]>

Examples

.STMLIB mylib.stl

.STMLIB volts.stl

.STMLIB dgpulse

Stimulus library files created by StmEd are made available to PSpice and

PLogic via the .STMLIB command.  The <file name> specification

identifies a file containing .STIMULUS statements.  The file extension

is not defaulted to ".stl."  If you specify a file name, you must

include its extension.

8.2.2.  Stimulus Definition(.STIMULUS)

General Form:

.STIMULUS <stimulus name> <type> <type-specific parameters>*

Examples:

.STIMULUS  InputPulse PULSE  (-1mv 1mv 2ns 2ns 50ns 100ns

.STIMULUS  DigitalPulse  STIM  (1,1)

+  0S    1

+  10NS   0

+  20NS   1

.STIMULUS 50KHZSIN  SIN  (0 5 50KHX 0 0 0 )

.STIMULUS statements generally appear within stimulus libraries created

by StmEd.  These define <stimulus name>s which are referred to by the

source devices (V or I), or by the digital STIM device.  The .STIMULUS

definition encompasses only the Transient specification portion of what

is allowed in the V, I, or STIM device syntax.

******************************************************************

Part 9.  Polaris Signal Integrity

******************************************************************

Polaris now supports Version 2.0 of the TangoPro PCB layout package with

the ASCII Database Translation option.

******************************************************************

Part 10.  Intel IBIS Model Support

******************************************************************

A library of Intel's Pentium (TM) 82430 (Mercury) PCIset IBIS models is

now available.  These highly accurate driver/receiver buffer models

include effects such as switching times, nonlinear transfer

characteristics, and package parasitics.  These allow you to perform

accurate signal integrity simulation of I/O interconnects on printed

circuit boards.  The models are located in "buffer.lib."

******************************************************************

Part 11.  New Libraries and Devices

******************************************************************

This section lists the new libraries and devices added to the 6.0

release which are not documented in the documentation set.

11.1.  New Transmission Line Models

Four new twisted pair transmission line subcircuits have been added to

"tline.lib." The names are as follows:

TP19AWG     TP22AWG     TP24AWG    and TP26AWG

These models use a .SUBCKT named ABMlossyTline which implements a

general lossy transmission line with R, L, C, and G, as functions of

frequency.

The symbol library "tline.slb" has also been updated to reflect these

changes.  A base part, TWSTPAIR, has also been added.  The coax symbols

now automatically choose between the .MODEL and .SUBCKT implementations,

depending on whether the user specified an operating frequency with the

FRQ parameter.

11.2.  New Comlinear Model

One new model from Comlinear Corporation has been added to the

"comlinr.lib" library since publication of the manuals.  It is a CLC532

2:1 analog multiplexer.

11.3.  Library Name Changes - "nom.lib"

The file "nom.lib" now references different files depending on the

simulator that was installed.  If, for example, you are using the PLogic

digital simulator, the file "nom.lib" will reference the files in the

Digital Library.  If you have the PSpice mixed-signal simulator, the

file "nom.lib" will reference files in both the Analog and the Digital

library.  In the latter case, this is done through a reference to the

file "nom_dig.lib" which in turn references the Digital Library files.

******************************************************************

Part 12.  Parts Program

******************************************************************

12.1.  New Voltage Reference Template

A new voltage reference template has been added for 6.0, with 5 screens.

A subckt-based model will be generated.  It is added as menu item 9 in

the Parts program main menu.  The voltage reference template is

described in the Circuit Analysis Reference Manual.

12.2.  Miscellaneous Changes

The power MOSFET template in the Rev. Drain Current screen has been

changed and the extraction algorithm was modified to yield a better fit.

Minor changes to the BJT template for some model parameter ranges were

made.

12.3.  Mouse Support

The use of the mouse is not supported for commands in Device_curve

submenus (such as Exit, Add, Change, or Delete).

12.4.  Preservation of the MDT file in case of error

Under the following error conditions, the original MDT file will not be

over-written (as would happen with release 5.4 and previous releases):

i)    A corrupted MDT file is read - the MDT file is corrupted.

ii)   A non-compatible MDT file for BJT and MOSFET is read - the

existing MDT file was created using Parts 5.3 or before.

iii)  A wrong device type MDT file is read - the existing MDT file was

created with a different device type (e.g., user trying to run

Parts for a diode but using a BJT MDT file).

An error message is displayed and Parts will return to the Main menu.

******************************************************************

Part 13.  MODTOMDT Utility

******************************************************************

MODTOMDT is a utility program to convert PSpice model files or library

files containing PSpice models to MDT files in Parts format.  The MDT

files generated will be compatible with other MDT files generated by

Parts.

Currently, the following device types are supported:

DIODE        JFET

BJT          MOSFET (LEVEL 3)

The following lists features and usage of this utility:

1.  Allow users to load a model file into Parts and see the performance

curves of the model.

2.  Allow users to perform a "what-if" analysis when they change the

model parameters in Parts.

3.  Allow our users to modify/update the model by entering screen data

into the Parts program and re-fit some model parameters

Usage:      modtomdt [options] <infile>

An input file must be provided.  This could be a single model file, or

it could be a .LIB file that contains many .MODEL statements.

Command line options:

-h   provide a summary of usage

-n   force all answers to program queries to 'No'

-y   force all answers to program queries to 'Yes'

Each .MODEL statement in a file will result in an individual MDT file.

If the output MDT file already exists, the user is prompted to confirm

overwriting the existing file.

An example:

A user wants to use a device, "A," for PSpice simulation, but cannot

find the model in the library.  However, a similar device, "B," is in

the library.

Here is the procedure for creating the A.MDT file using Parts and the B

device:

1.  Copy the model for device B from the appropriate library file to a

file <infile>.  (The model begins with a .MODEL statement.)

2.  Run MODTOMDT <infile> to create B.MDT.

3.  Rename B.MDT to A.MDT.

4.  Run Parts, and load A.MDT.

5.  Modify/Add data using the Parts program, then generate the file

A.MOD.

******************************************************************

Part 14.  Filter Designer

******************************************************************

Filter Designer is no longer supported on the Macintosh Platform.