CW is a Morse Code send/receive program designed to run on the IBM PC and

it's clones.  The program expects to find the Color Graphics Adapter present.

It will not function correctly with a Mono board or the EGA.  A split screen

format is used for the visual interface and on-line help is available to the

operator.  On-line help is provided by the file HELP.CW.  If that file is not

on the default drive, the program will still function, but without help.

The program is user configurable and creates a file which supports the

screen images for subsequent use.  The user has the option of changing the

operating characteristics and input/output environment recognized by the pro-

gram.  These may then be saved by CW in a file called INIT.CW, an ASCII file

which can be viewed/modified using a word processor.  The default character-

istics provide for Morse Input and Output on communications port COM1 using

the RTS and CTS signal lines.   If INIT.CW is not on the default disk then

the default values are assumed.

CW.COM is a user supported program which may be transferred and/or copied

as often as you like.  No monetary fees (other than disk copying) may be

charged as a part of this service.  The program is a product of

Dave Freese, W1HKJ

c/o Clermont Computer Consultants

29 N Ravenwood Drive

Cape May Court House, NJ  08210

(609) 624 0076

The program is written in TURBO (tm) PASCAL and demonstrates how well

that particular product allows full use of the PC's hardware. Despite the

disparaging reviews from some self proclaimed keepers of the PASCAL standard,

this program demonstrates the efficacy of TURBO Pascal.

It is not anticipated that the source code for this program will be

placed in the public domain.

Have fun with CW.  If you enjoy using it as much as I enjoyed creating

it, you will have a ball.  Happy DX and bcnu.

73's   Dave   W 1 H K J

INSTALLATION PROCEDURE

Create a bootable disk with CW.COM and HELP.CW installed in the root

directory.  CW.COM is compatible with ancilliary programs such as SIDEKICK(tm)

and in fact is much enhanced when used in conjunction with that product.

When CW is run for the first time, the Logo, and Help screens are created

and stored in a new file called IMAGE.CW.  This new file is used on subsequent

program executions to speed the screen graphic response time.

Press <F10> to review the HELP screen and then <SPACE> followed by <F7>

to enter the configuration procedures.  CW is distributed with presets for

screen colors to suit the authors tastes.  Colors may be altered, reviewed and

eventually stored in an initialization file called INIT.CW.

The internal tone generator is configured to ON by the default configur-

ation parameters.  Try typing a few lines of text and observe the operation of

the type ahead keyboard buffer/display, the character transmit indication and

the morse code generated on the internal speaker.

PROGRAM TERMINATION

The <ctrl>C and <Break> keys are disabled by the program.  The program is

exited by the use of the <alt><F10> keystroke combination.  This allows a

graceful termination of any open files in use by the program.

MESSAGE BUFFERS

<F1> - Transmitting a Message Buffer

CW maintains 10 message buffers (0..9) which may be transmitted by press-

ing <F1># where # is  0..9 .  The <F1># sequence may be pressed at any time

which automatically inserts the specified message buffer into the outgoing

character buffer.

<F2> - Modifying the Message Buffers

The contents of the ten message buffers may be observed and modified by

pressing <F2>.  The ten message buffers are displayed on a full screen and

the operator prompted to enter a number (0..9) to modify a buffer, or <ESC>

to return to the split screen display.  If a buffer is selected the buffer is

cleared and the cursor placed at the beginning of the buffer line.  Up to 80

characters may be put in each buffer.

<alt><F1> - Loading the Message Buffers

The 10 message buffers may be loaded from disk.  This is accomplished by

pressing <alt><F1>.  The operator is prompted to enter a valid directory-tree

filename which will contain the desired message buffers.  This file is an

ASCII file containing 10 lines of up to 80 characters per line.  They may be

created using CW or a separate word processor program.

<alt><F2> - Saving the Message Buffers

The present message buffer contents may be assigned to a disk file of the

operators choice by pressing <alt><F2>.  The operator is prompted to enter a

directory-tree filename for storage.  Existing files will be overwritten by

this function.

ASCII TRANSMIT/RECEIVE FILES

<F3> - ASCII File Transmit Function

Any valid ASCII file may be transmitted by pressing <F3>.  The operator

is prompted to enter the file name (directory modifiers are ok).  If the file

is not found the operator is returned to the split screen display.  If found,

transmission begins immediately with character display in the transmit window.

CR/LF and other non-Morse-characters are translated to a SPACE for the pur-

poses of generating a correct timing duration.  Transmission may be altered

with the ^P or ^X keys, causing a pause of termination respectively.  If this

key is accidently pressed, respond to the file name prompt with a <RETN>.

<alt><F3> - ASCII Receive File Save Function

The program maintains a receive buffer of 8192 bytes which acts as a

first-in/first-out wrap around memory.  The operator may save the contents of

this buffer from its beginning up to the last character received.  Pressing

the <alt><F3> function key will cause an operator prompt for a filename in

which to save the receive buffer.  If this key is accidently pressed, respond

to the filename prompt with a <RETN>.

AUTOMATIC LOG MAINTENANCE

<F4> - Log Data Entry

CW maintains the file LOG.CW, an ASCII data file containing a log of QSO

activity.  Data may be entered in the log by pressing <F4>.  The operator is

prompted to enter the data for the log in the following sequence:

Date ..............xxxxxxxx

Time ..............xxxxx

Freq ..............xxxxxx

Power .............xxx

Outgoing RST ......xxx

Incoming RST ......xxx

Call sign worked ..xxxxxxxx

Comments [name, etc]

xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx

Default values may be obtained by pressing <RETN> alone.  This is partic-

ularly useful for the TIME and DATE values which are the MS-DOS values corr-

ected to UTC.  All other defaults are blank filled strings.

Each composite of log data forms a single line in the file LOG.CW which

may be subsequently altered with a word processor such as SideKick, Wordstar

or Brief.

<alt><F4> - Saving the LOG

The file LOG.CW may be updated manually by pressing the <alt><F4> function

key.  Log entries made during normal operation will be automatically saved at

the end of the operating session, or when 100 QSO's have been logged, which-

ever comes first.

TRANSMIT/RECEIVE CHARACTER RATES

<F5> - Set Receive Code Speed = Transmit Code Speed

During CW reception the program maintains an estimate of the incoming

code speed.  First order difference equations are used to filter and smoothly

track changes in the detected speed.  During high noise bursts or for rapid

changes in received CW speed it may become necessary to reset the receive

tracking values.  Use this function key to set the current Receive Tracking

Speed equal to the current Transmit Speed.

<alt><F5> - Set Transmit Code Speed = Receive Code Speed

During contests and other rapid CW exchanges it is considered good oper-

ating to transmit at the receive code speed.  Pressing <alt><F5> will immed-

iately force the transmit code speed to the received code speed.

<F6> - Adjusting Transmit Code Speed

The transmitted code speed may be adjusted in 1 WPM increments.  Pressing

<F6> causes an operator prompt for the new value on the status line.  Entries

must be digits 0 ... 9, improper entries will be rejected and the prompt re-

issued.

<alt><F6> - Calibrating the Code Speed

The transmit and receive code speeds are dependent on timing loops used

in TURBO (tm) PASCAL.  They are therefore sensitive to the clock rate of the

computer.  CW can accomodate variations in clock frequencies.  Accurate timing

does require a scope and an external 10 HZ signal generator.  The signal gen-

erator may be a calibrated keyer.

To begin the calibration press <alt><F6> and select the transmit or re-

ceive procedure.

RECEIVE CALIBRATION - Apply a 10 Hz square wave of the proper amplitude

to the input port.  The 10 Hz square wave is the dot rate equivalent of 25

WPM cw.  Pressing the <u> and <d> keys will alter the displayed code speed in

0.1 WPM increments.  Pressing <U> and <D> will alter displayed code speed in

1.0 WPM increments.  Press <Q> when the calibration is complete.

TRANSMIT CALIBRATION - A nominal 10 Hz square is generated at the output

port.  Adjust this signal for proper frequency by pressing the <u/U> and/or

<d/D> keys for the 10 Hz output.  Press <Q> when the calibration is complete.

ALTERNATE CALIBRATION PROCEDURES - The program may be calibrated by 'ear'

and a watch with a sweep second hand.  The ASCII file 'PARIS' contains a seq-

uence of 100 occurences of 'paris '.  This is one of the standard timing words

for text timing as specified in the 1985 issue of the ARRL Handbook.  Proceed

as follows:

<1> Clear the Screen,

<2> Select transmit code speed to be 25 WPM,

<3> Select <F3> (transmit file),

<4> Specify file name of PARIS -- do not press <RETN>,

<5> Press <RETN> - start timing at the first 'p',

<6> At the end of the timed minute press ^X to stop transmission.

Count the number of words 'paris' on the screen.  It should be 25.

If it is not the file INIT.CW can be modified in the Timing Duration

entry.  The new entry can be calculated as:

new = old * #words / 25

That completes the transmit calibration.  You now need to calibrate

an external keyer to 25 wpm.  Some amount of simple hardware will be re-

quired, the purpose of which is to allow both the computer output and the

keyer to simultaneously key an external oscillator.  The external keyer

speed is then adjusted for a zero beat note between the two keying

sources.  Use this calibrated keyer as the input as described in the pri-

mary receive calibration procedure.

SOFTWARE/HARDWARE CONFIGURATION

<F7> - Modifying Configuration

The operator may modify the characteristics of the program with respect

to the transmit code weight, initialization values, internal tone, and screen

colors.  Input/output port assignments and logical levels may be changed.  The

designation of time zone for the purposes of timekeeping and logging is avail-

able.   A previous configuration saved on disk may be loaded for quick changes

in operating characterics.

TRANSMIT WEIGHTING - The operator may select transmit code weighting to

fit his or her individual requirements.  A prompt for the following charac-

teristics is presented:

Dash/Dot (3.00) .....

Letter/Dot (3.00) ...

Word/Dot (7.00) .....

where the values in parenthesis are the present values.  Pressing return at

any one of the three leaves the value unchanged.  Here are some suggestions

for transmit weighting and its intended use.

Dash/Dot    Letter/Dot    Word/Dot       Used for:

3.00        3.00          7.00         Standard code weighting

3.60        3.60          8.00         Moderately heavy code

good for QRN conditions

3.00       10.00         30.00         Practice at 1/2 WPM rate

3.00       17.00         51.00         Practice at 1/3 WPM rate

3.00       24.00         73.00         Practice at 1/4 WPM rate

SET INTERNAL TONE - The internal cw tones may be toggled ON/OFF and the

frequency of the tone modified.  If the 'y' response is made to the prompt:

PC tone on? <Y/N>');

then the operator is requested to enter the desired tone:

Note frequency =  600  change to ....

Pressing <RETN> leaves the frequency unchanged.

SPECIFYING DEFAULT MESSAGE FILE - The operator may specify the start-up

messages which will be loaded on each execution of CW.  The response to the

prompt:

Enter message file name.ext ...

should be an existing \directory\filename.ext string, where the directory is

optional.

SPECIFYING TIME ZONE - The program default for time zone is 0.  That is,

you live in Greenwich, England.  Since most of us are not so fortunate, and

good log keeping stipulates using UTC (GMT for us oldtimers), the operator can

specify his or her own time zone as:

Enter your time zone ...

If a non-zero time zone is entered, the program adjusts the date and time

for the on screen display and the log to accomodate the time zone.  Proper

attention is also made to month and year rollover with the exception of leap

year.  This is a subject of a future change to the program.

CONTROLLING HARDWARE INTERFACE - CW may be configured to access any port

within the 8088 port address space.  The individual bit and its logical sense

can also be specified.  The program default values are for the COM1 port and

uses the RTS, CTS signals for Transmit and Receive respectively.  The operator

will be prompted:

Enter transmit port (1020)...

transmit bit # (1).....

transmit logic (1) ....

receive port (1022)....

receive bit # (4)......

receive logic (1) .....

The values in parenthesis are the present in use, a <RETN> leaves the value

unchanged.  You must enter the port numbers in decimal, the bit number as 0..7,

and the logic level as 1 => active high; 0 => active low.  This may take some

experimenting for your particular hardware.  The author has configured a VISUAL

COMMUTER (IBM clone) and the MFJ 1224, RTTY/CW terminal unit to the LPT1

parallel port and a homebrew interface to the COM1 port.  The homebrew design

is the subject of a forthcoming public domain release.

CHANGING RECEIVE FILTERING - The program is designed to track the charac-

ter rate of the incoming (received) morse code.  Under ideal conditions, no

filtering is required at all, however, in the real world of QRN and QRM it is

an absolute necessity.  The filtering is accomplished using the digital equiv-

alent of a low pass filter.  Just as the response of a low pass RC filter can

be controlled by the resistor-capacitor product, the difference equation

coefficients can be modified to adjust it's response.  The operator is prompt-

ed as follows:

Receive filter coefficient:

fast ... slow

<2> .. <8>

Enter ...

The default value is 5 which seems to work well with the author's Kenwood

599D receiver with an 800 Hz CW filter.  A 200 Hz wide active audio filter is

also used along with a limiter/detector of the author's own design.  This is

definitely an area for experimentation and user feedback.  I simply cannot

afford to buy one of everything to test the programs utility.

LOADING CONFIGURATION - The operator is prompted to enter the name of a

configuration file which is to be loaded.  If the file does not exist or the

<RETN> is pressed with out a file name then the default configuration is

established.

SAVING CONFIGURATION PARAMETERS

<alt><F7> - Save Configuration Parameters

The operator has the option of saving the operating configuration of the

program on a disk file.  The prompt for a file name may be given a <RETN> re-

sponse in which case the default name of 'INIT.CW' is assumed.

The operating characteristics of CW as determined by the configuration

parameters can be saved on the default disk.  The program looks for this file

on initial execution and uses the values contained therein for its presets.

INIT.CW is an ASCII file created by CW when the <alt><F7> function key is

pressed.  It may be observed/changed with any wordprocessor.  A sample of the

INIT.CW file contents are:

Xmt_WPM ............  18

Dash Dot Ratio ..... 3.0

Letter Dot Ratio ... 3.0

Word Dot Ratio ..... 7.0

PC monitor flag ....   1

Monitor note freq .. 600

Time Zone ..........   0

Initial msg file ...NIL

Timing Duration .... 900

Rx Coefficient .....5200

Xmt background .....   7

Xmt foreground .....  14

Rcv background .....   3

Rcv foreground .....  15

Status background ..   4

Status foreground ..  11

Prompt background ..   1

Prompt foreground ..  14

Aux background .....   6

Aux foreground .....  14

Help background ....   2

Help foreground ....  14

Transmit port ......1020

Transmit bit .......   2

Transmit level .....   1

Receive port .......1022

Receive bit ........  16

Receive level ......   1

Filter coefficient..   5

The above values are the equivalent to the internal program configuration

defaults.

TUNING & OTHER STUFF

<F8> - Tune the Transmitter

Pressing the <F8> function key causes the transmitter to be held in the

key down condition.  The key down condition is released when any other key is

pressed.  I suggest the <SPACE> to prevent on the air burbles - hi.

<alt><F8> - Send V's

The character V is used as a standard "test" signal by most CW operators.

The <alt><F8> key causes a string of V's to be transmitted.  Pressing any key

will terminate the sequence of V-s.  The <SPACE> is also appropriate here.

<F9> - Clearing the Screen

The split screen display is cleared of all text, the colors reset, and

the status line refreshed when the <F9> function key is pressed.

<alt><F9> - Reset Receive Buffer Pointer

Clears the screen the same as <F9>, but additionally resets the receive

buffer to begin loading characters at the beginning.  Useful for initializing

the buffer during incoming traffic or bulletins.

<F10> - H E L P !!!

On line help is available and obtained by pressing <F10>.  A command

synopsis is displayed.  Pressing any function-key or alt-function-key sequence

will display the appropriate portion of the HELP.CW file in a central screen

window.  If the file HELP.CW is not on the default drive, no action is taken.

----------------end-of-author's-documentation---------------

This disk copy provided as a service of

The Public (Software) Library

Disks in the library are updated monthly.

For a copy of the latest monthly software library newsletter

and a list of the 700+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705, Rm. F

Houston, TX 77235-5705

(713) 721-6104

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions.