AutoWx.EXE

(C) 1994,95 N5KOA

VER 5.x   03/95

=====

NOTE:

=====  If you are upgrading to a newer version of AutoWx, read

the "Compatibility" notes below.

Earlier versions incorrectly documented the area code for

the ARRL BBS.

AutoWX (Auto-Weather) is a PC compatible packet program that

accepts weather data from the Ultimeter II weather station and

makes the data available on packet by direct connect requests

and/or via automatic beacons (in APRS beacon format).  The

program assumes you are using two serial ports (COM1 and COM2)

and that the U-II is connected to one, the TNC to the other.

Use ALT-H for online help.  The program works very well on an XT

with 256K RAM and a single floppy disk drive.  XT's are often

GIVEN away these days and one could be dedicated full time to

this application.  (If you don't have the serial data cable for

the U-II, see TIPS below.)

Features:

Logs weather data in a binary log file that can be read

by the Data Viewer software sold by the U-II manufacturer

(Peet Bros.)

Displays weather data, including rain, on screen every

five minutes (display options 1 and 2).

Displays Hi/Lo Temperatures and Hi Wind with the time of

day.

Continously reads U-II data port to show highest wind

speed for each 5 minute interval.

Allows packet stations to connect and request current

weather conditions, including rain, and highs and lows

for the date.

Sends weather beacons in APRS format.

Logs weather beacons from other AutoWx or APRS stations.

Creates ASCII text file showing highs and lows for the

date.

Serves as a simple packet terminal program.

Runs on a minimal computer system  (256K, monochrome,

floppy, 2 serial ports).

Thanks to the many users of AutoWx around the country who have

offered suggestions, comments and compliments!

Page 1

Compatibility with earlier versions:

The WX.CFG file contains a logger filename as of version 3.x

Note the contents of your old WX.CFG for reference if you want,

and rename or delete the old file.  Run AutoWx to create a new

configuration file. Read PROGRAM UPDATES section of this

documentation for new features.

As of Version 5.2, the WX.DAT file includes two new entries:

WindChill and WindChill time

If you try to run the newer version with the old WX.DAT file,

you will see an error message indicating "Input Past End of

File" or something to that effect and the program will abort.

Delete your old WX.DAT file before running the new AutoWx or

edit WX.DAT to include entries for WindChill.  If you add the

new data with a text editor, add:

,0,""   (that's comma zero comma quote quote)

STARTUP / CONFIGURATION Parameters

This program is updated often with fixes and new features.  If

you have suggestions, please contact me by packet or U.S. mail.

---------------------------------------------------------------

= THE ULTIITSR PROGRAM SHOULD NOT BE USED WHILE AUTOWX IS RUNNING =

---------------------------------------------------------------

(The TNC ECHO should be ON, AUTOLF OFF and MAXUSERS 1.  If you

are running a very slow 8088 machine, you may have to turn

MONITOR OFF.)

The first time AutoWx is executed, or if the WX.CFG file is not

found, the user is prompted for configuration information.  The

next few paragraphs are in order as needed for configuration.

The COM port of the TNC must be identified as (COM) "1" or "2".

The other COM port will automatically be assigned to the

Ultimeter II.

Supported baud rates for the TNC port are: 1200,2400,4800 & 9600

Your station's latitude and longitude are necessary to beacon in

a format recognized by the Automatic Packet Routing System (APRS

by WB4APR). There is very little error validation in the entry

of the coordinates so be very careful that they are entered in

the format shown at the config prompt.  Null entries at the

Latitude and Longitude prompts will result in "0000.00N" and

"00000.00W".

A temperature adjustment value is included because the Ultimeter

II temperature sensor can be affected if additional lengths of

cable are attached to it.  The default value for the temperature

adjustment is 56 which will be correct if the display on the

U-II is correct.

Page 2

Weather data can be tracked on the AutoWx screen in three

different formats.

If you enter zero (0), the status line at the bottom of

the screen will not show weather data but will show the

last time someone connected and requested weather data,

and how many times weather has been requested since the

program was last started.  (If you start the program

with one of the other status line options and use ALT-S

to change, press ESC to clear the screen.)

A value of one (1) will cause weather data to be printed

on the screen every time an update is performed.

(Usually every 5 minutes - see more information in this

text when the update is discussed.)

A value of two (2) will cause the status line to be

updated with the current reading every time the program

updates weather data.

(VER 4.7b - You can now toggle through the status line

display options with ALT-S.  This does not change the

value in the .CFG file so the program will always start

up in the mode determined by WX.CFG)

Note that when prompted for the Log File Name, your entry should

NOT include an extension.  The extension .LOG is automatically

added.

After the configuration information is entered, it is written to

the ASCII file WX.CFG  The configuration can be changed either

by deleting the file and running AutoWx or by editing the file.

NOTE - If you edit the WX.CFG file, remember the values for Temp

Adjustment, Status Display Format, and Wind Chill are not in

quotes - the others are.  Each element is separated by a comma.

Also, the WX.CFG file must be in the same directory as

AutoWx.EXE.

WX.DAT file

The WX.DAT file is an ASCII file that tracks the high and low

temperatures and high wind speed for the day.  If the file is

not present when AutoWx is executed it will be created.  The

file is updated whenever the Hi's and Lo's are edited (ALT-E)

and when the program is terminated with ALT-Q.  Actually you

must answer the "write data" prompt with "Y" or "Yes" - the

default (No) will not write the file until 50 minutes after the

hour.

The Hi's & Lo's can be edited by typing ALT-E during the

program.  At that point you are prompted to enter  1,2 or 3

depending upon which item you want to change.  If you enter no

value the edit is aborted and the program is back in the main

loop.  If you change a value, enter the new information in the

same format as displayed.

Page 3

If you stop the program and restart it on the same date, the

data in the WX.DAT file will be read and used by the program.

If the date in WX.DAT is different than the system date on your

computer, the data will be ignored and new Hi's and Lo's will be

tracked beginning with the first data read on the wx port.  Hi's

and Lo's are reset at midnite.

The first line of the WX.DAT file is a title included only to

help you read the data when looking at the file with an editor

or list program. The contents inside the quotes on the first

line are totally irrelavent to AutoWx.

UPDATE

An "update" occurs when the program is started, every five

minutes, and when a connected (//WX) request is received.  As of

version 4.x the U-II data is read continuously and logged to

disk every 5 minutes.  Whenever the data is updated, it is also

printed on the screen (if you selected option 1 or 2 in the

configuration of the status line).  This feature can be toggled

off with ALT-U but this will also disable the logging of data to

the log file.  If you are going to disconnect or otherwise

disable the weather input, toggle the updates off with ALT-U to

keep the program from hanging up during the update sequence.

While the updates are toggled OFF, connected requests for

weather (//wx) will receive a message stating that "Weather data

is temporarily unavailable...".

When UPDATE is toggled off, the timer is not reset so when

UPDATE is toggled back ON, if the programmed time (5 min.) has

passed while UPDATE was OFF, the update will be performed

immediately.

If AutoWx is started with the command line parameter "/SHORT",

the weather node response will NOT include hi/lo temps/wind.

Only the current weather data will be sent.  The startup syntax

from DOS would be:             AUTOWX /SHORT

WEATHER NODE (connected requests)

If another packet station connects and sends "//wx", (without

the quotes - upper or lower case), AutoWx will update and send

the current weather data and Hi's and Lo's for the day.

I include instructions in my Connect Text (CTEXT) showing:

"Send //wx for weather data - //bye to disconnect..."

As of version 4 it is not really necessary for a user to request

the weather info more than once because the wind speed will show

the highest windspeed recorded since data was last logged to

disk.  The report is displayed on screen if you enter the //wx

directly from the keyboard but the TNC cmd: 's clutter the

display.

Page 4

While a station is connected, the weather Beacon is suspended.

Also, if UPDATES are OFF a connected user will see the message,

"Weather data temporarily not available...".  The last ten

stations that connected can be viewed by pressing the PgDn key.

If you enter a "greeting" with ALT-G, the message will be sent

to connected users.  You might use this to notify users to check

the mailbox for bulletins, etc.  I use the feature to notify

users of bulletins in my mailbox.  After a storm, I post a

bulletin that includes hi's and low's, time the storm hit, total

rain during the storm, etc.

Users may disconnect by sending //bye.

BEACONS

Automatic beacons can be toggled ON/OFF with ALT-B.  The beacon

will be digi'd through any valid path specified in the UNPROTO

tnc parameter. UNPROTO should be set to APRS or to APRS VIA xxxx

in order to be identified by stations running APRS.

When Beacon is toggled ON, the user is prompted for the interval

(in minutes;  default = 30 minutes) and for any text to be

tagged on the end of the weather data.  The Beacon time will be

in the nearest 5-minute increment regardless of the value

entered;  for example, if you enter 9 minutes, the actual beacon

time will be 10 minutes.  Beacons are transmitted only when the

log file is updated.

The beacon will automatically include the Longitude and Latitude

from the config file.   If you enter text to be appended to the

beacon (Btext), it will be limited to 30 characters.

When the Beacon is ON, it can be forced to xmit immediately by

typing ALT-T.  After using ALT-T, the beacon timer restarts.

The Beacon can be turned off/on remotely with //BEACON.  Make

sure that you have the correct path specified in UNPROTO.  The

default time for beacons turned on remotely is 60 minutes.  You

can change this by starting AutoWx with the /RB switch.  You

will be prompted for the new time.  The new time will remain in

effect until the program is stopped.

If you enable Weather Beacon Logging with ALT-W the program will

capture any monitored APRS-type weather beacons and save them to

an ASCII text file named WX.LOG.  While Weather Beacon Logging

is enabled, an asterisk (*) will appear to the left of the UPDATE

status at the bottom of the screen.  Weather Beacon Logging may

not be compatible with slower 8088 class computers and/or you may

have to use a slower baud rate to the tnc.

Page 5

LOG File

Every 5 minutes the program updates the binary logfile named in

the WX.CFG file.  This file can be read with the Ultimeter II

Data Display program (Vers 1 & 2).  At version 5.2 AutoWx

creates a new log file at 0000 HRS on the first of the month.

The older log file is named MMYY.LOG where MM = month and YY =

year.  A simple file-reading program is also available from

me... see TIPS at the end of this document.   (Log files can be

appended to each other by using the COPY /B command.  Example:

copy /b file1.log+file2.log bigfile.log  See your DOS manual for

details on COPYing binary files.)

NOTE:  Logging is suspended while UPDATE is toggled OFF.

The U-II Data Display program assumes that all log entries are

at five minute intervals.  Therefore, if you suspend logging for

more than a couple of minutes, the Data Display program will be

reading data that was logged after the time displayed in the

Data Display program.  For example:  If you suspended the logger

with ALT-U at 10:28 and did not resume logging until 10:58, the

Display program will show the data logged at 11:00 as if it were

logged at 10:30 and there will be thirty minutes of no data at

the end of the day.  See Log File Notes for more information.

TERMINAL Mode

If you use the program as a packet terminal to connect to other

stations temporarily, turn the Beacon OFF.  If UPDATEs are left

ON, the logger will continue to log data but your screen will

also continue to receive updated data every five minutes and the

program will seem to halt while the U-II data is read (the UII

data stream is not constant and can take several seconds to

complete at times). The program will ignore the //wx request if

you disable it with the ALT-X toggle.  If you plan an extended

QSO, use another program.  AutoWx makes a lousy terminal program

anyway.

TIME SET (ALT-Z)

If you need to reset the system clock because it runs slow or

fast, type ALT-Z and enter the new time in the format HHMMSS.

For example, to set the time to 14:34:15, enter 143415 without

the colons.  This feature does no error-checking so use it

carefully.  The new system time is re-displayed immediately

after you enter a time.  If you press ENTER without entering a

new time, the old time remains intact.  IF YOU ENTER A TIME

EARLIER THAN THE LAST UPDATE, THE PROGRAM WILL NOT UPDATE FOR AT

LEAST 24-HOURS OR UNTIL YOU RESTART THE PROGRAM.

There are several utilities available to "automatically" adjust

your computer's clock if it requires frequent adjustment.  Check

out CLOCKWORK (CWK.EXE) by Pavel Otavsky or the PC MAGAZINE

utility, WTIME.

=====  Always double check the time if you change it!

Page 6

NOTES:

Log File Notes

The binary log file is much like the one created by the

Ultimeter-II Data Logger and is very efficient.  You can

probably get a whole year's data on a 360K floppy.  The AutoWx

program must run for at least ten minutes before any useable

data is logged.  Also, starting and stopping the AutoWx program

before any useable data is logged can cause errors in the

display.  The U-II Data Display program reads a file header in

the log file to determine the starting date and time but all

data entries after the header are assumed to be in five minute

intervals.  The AutoWx program writes a new date/time header

each time it starts but if logging is suspended by the ALT-U

toggle the data may miss the 5 minute interval and therefore be

incorrectly displayed by the U-II Data Display program. If the

logger is suspended for more than 10 minutes, quit the AutoWx

program and restart it to avoid the problem.  Although data may

be missing for the suspended period, the data logged will be

displayed with the correct time if you follow this procedure.

The AutoWx program reads the Ultimeter II port continuously and,

when logging, logs the highest windspeed recorded during the

last 5 minutes. However, the U-II data sends serial data every 4

seconds (on average) but reads sensors about every 1.5 seconds

and there may be differences in the high windspeed recorded on

the U-II display and that recorded by AutoWx.

If you would like to know how to decode the binary log file,

send me a SASE.

Page 7

--------------------------

AutoWx is released for the use by other licensed amateur radio

operators and no payment or registration fees are necessary.

The program is copyright protected and I choose to retain all

rights to the code.  The source code (QuickBasic 4.5) will not

be made available unless somebody convinces me they can

substantially contribute to the project.

Comments and questions can be directed to N5KOA at N5RKN.#WTX

(Odessa, TX).  Please drop me a line if you try the program.

Disk copies are available only when accompanied with enough

green stuff to pay for the postage, disk, etc. DO NOT SEND

DISKETTES!  ($5 or more gets you some utility programs to go

with AutoWx; like WxRead.EXE for reading/converting the binary

log file.)

Ultimeter II weather station sources:

PEET BROS. as advertised in QST

Radio Com Sales  (Larry, N5VFW)

Box 118814

Carrollton, TX  75011

(214) 492-3824

APRS is a map-based packet program by WB4APR, Bob Bruninga,

Annapolis, MD

WXSTAT.EXE - a program to monitor Ultimeter II weather data in

realtime.

Page 8

PROGRAM UPATES

Ver. 1.x - allows only connected weather requests.

2.x - adds Updates to screen and Beacons

3.x - adds data logger

3.3f - allows list of last 10 connected stations with PgDn

CONNECT.LOG remains - see HELP screen

3.3g - tweaked timer routines

3.3h - Fixed config write routine

3.3i - sorts connected stations (listed with PgDn) by Date/Time

4.x  - now reads and logs highest windspeed during the past

five minutes (reads wx data approx.  every 4

seconds) quicker //wx response

4.2  - logging will

synchronize to computer time (XX:X5 or XX:X0)

4.3  - Beacon interval will occur only with logging (5

minute intervals)

4.3c - will abort if no U-II found on indicated port

4.3e - Hi's & Lo's written to ASCII file WX.HST

4.4a - fixes status line display for display option #2

4.5x - Tuned up log entries for rain amounts Flush write

buffer every hour  (at xx:50) Removed ASCII file

CONNECT.LOG Allow null responses to Lat/Long entries

in config

4.5d   Allow serial data starting with #

and allow /KPH startup to correct windspeed  (see

Errors/Bugs text)

4.5e   Fixed rain amounts.

4.5g   Show Hi's & Lo's on status line Fixed Beacon hold

while connected to another station

4.5i   Added ALT-C to copy LOG FILE and WX.HST to floppy

Default = copy to drive A:

Start AutoWX with /B: for drive  B:    8/12/94

4.6a   Fixed rain logging

4.7    Fixed binary logging for dates after 09/30/94

4.7a   Fixed beacon for format & for rain amounts > 9.9 in.

4.7b   Sends bell on error ALT-S toggles through Status

Line Display options

4.8    Weather Beacon

Logging

4.9    Minor tweaking and ALT-Z (TIME set) function added.

5.0    Wind Chill index is calculated

5.1    Lowest Wind Chill shows on status line Fixed error

that reset wind speed to zero at midnite log entry.

12/08/94

5.2    Wind speed direction (Course/CSE) in beacon now

always contains 3-digits.  Windchill is saved in

WX.DAT file.  A new log file is created on the first

of the month. 01/04/95

5.2a   Fixed write of WX.DAT at 50 minute flush.

Previously, if you did not save the WX.DAT file upon

quitting the program, the old file format was saved,

causing an "Input past End of File msg" upon

restarting.  Fixed error that caused Hi/Lo Temps to

be misread upon startup (see additional info below).

5.2b&c Added routines to help keep Beacon status correct

while/after disconnect due to retry/timeout

5.3    Fixed Filename for Monthly log file routine

Page 9

Errors/Bugs and other annoyances...

Somehow, a bug seems to have crept in to Version 5.2 and causes

AutoWx to ignore Hi's and Lo's recorded in the WX.DAT file.

This only occurs if you stop AutoWx and restart on the same date

and has been fixed in Version 5.2a.

If you start AutoWx WITH A FLOPPY DRIVE, make sure the startup

floppy is in it's drive at 50 minutes after the hour.  The write

buffer is flushed at that time and the disk must be found for

the program to continue. Also, do not attempt to use the ALT-C

copy feature if the copy will attempt to copy files onto

themselves.

I have had a couple of requests to allow the assignment of com

ports above COM1 and COM2.  Unfortunately my compile library

does not support COM 3 and above.  If someone ever sends me an

assembly (or C) routine that I can add to my library I'll

attempt to add support for the other com ports.  (Some libraries

do allow you to address the com port by address but allow only

one serial port to be open at a time.  This won't work for the

U-II and a tnc.)

HF transmissions will often interfere with the U-II and bad data

can be passed to the AutoWX program.  The use of an rf filter is

highly recommended.  (A 30 db filter is available from PEET

BROS.  Also, I've seen telephone RF filters at WalMart.  You may

need more than one.) Errors in the binary data file can be

corrected with the DOS DEBUGger program.

All versions after Ver 3.0 log data for the binary log file

every five minutes but data is not physically written to disk

until the write buffer is full (one disk sector).  If power to

the computer is interrupted, any data in the buffer is lost.  As

the program writes only 4 bytes (on average) at a time, 10 hours

or more could be lost.  This problem is characteristic of DOS

I/O and a suitable solution has not been determined yet.  To

write the data every five minutes would result in, what I feel

is, excessive drive wear.  I will continue to look for

alternatives.  In the meantime, the buffer will flush every hour

at 50 minutes after the hour.

The National Weather Service calculates wind speed using a

2-minute average of sustained wind.  AutoWx records the highest

wind speed during a 5-minute period.  The HI WindSpeed indicated

by AutoWx is closer to a "gust" windspeed.

It has been reported that there are two versions of the U-II in

regard to serial data output.  My U-II sends an asterisk (*) at

the beginning of each data stream and there is apparently a

version that sends the pound sign (#).  The (*) is supposed to

indicate wind speed in MPH while the (#) indicates KPH.  If your

U-II sends in KPH (#), start the AutoWx program with the switch

/KPH.  Example:  AUTOWX /KPH

Occasionally the automatic beacon is disrupted when a station

becomes disconnected due to "too many retries".  If that

happens stop/re-start the beacon with ALT-B.

Versions 5.2x did not properly name the new month log file if

the file 0195.LOG existed in your AutoWx directory.  If the

0195.LOG file was renamed or removed, there was no problem.

Page 10

If you find a bug in the program, check for a later version on

the ARRL BBS (203) 666-0578.  If the bug exists in the newest

version please let me know so I can fix it.  I also welcome any

suggestions or comments so please let me know if you like or

dislike the program or if you would like to see other features

added.  I will also consider customizing your version if you

will pay for the disk and postage.

Les Blalock, N5KOA

1711 Pagewood

Odessa, TX  79761

(915) 332-7112 day

332-1942 fax

367-0408 eve  after 6 pm CST

Page 11

TIPS

If you build your own data cable, add an rf choke on the data

lines in the DB25 housing.  (Looking at the 4-wire modular plug,

latch away from you, wire down, the third wire from the left is

the data wire which goes to pin 3 (DB25, pin 2 on DB9) with the

5K resistor connecting pins 3 & 2).   The first wire on the left

is the signal ground (pin 7 DB25 - pin 5 DB9).

If you use the TNC Command that inhibits the display of incoming

characters while you type, turn it off.  Better yet, make sure

you hit the ENTER key the last thing after using your keyboard.

After hitting ENTER the cmd: prompt should appear on the screen

to let you know that there are no characters in the tnc buffer.

Turn off your computer monitor and leave it off when not sitting

at the keyboard.  (The screen will burn in the image of the

weather data over time.)  My monitor stays off for days at a

time although I verify everything's running fine by copying the

data file to floppy and looking at it every day.

Find a HEX-to-BIN/BIN-to-HEX program  (I'll send one if you

can't find one) and use the program to edit log files.  I create

seperate log files for each month and, using the COPY /B

command, I can make one for the year or any other particular

time period.  Also, by working with smaller log files, you don't

have to wait forever when running the Data Display Viewer

program to graph your data.  For example, if you wanted to edit

a log file to include only the data for one month (this is done

automatically as of Version 5.2):

1.  Convert the log file to ASCII:

BIN2HEX ultii.log ultii.hex

2.  Load ULTII.HEX into your favorite ASCII text editor.

3.  Locate the string that represents the first day of

the month  (November 1, 1994 would be "F0 xx xx 41 31").

(xx xx will usually be 00 00 if AutoWx was running

at midnite on that date.)

4.  Select all text prior to F0 xx xx 41 31 and delete it.

5.  Locate the string that represents the first day of

the next month  (December 1, 1994 in this example

would be "F0 xx xx 61 31").

6.  Delete F0 xx xx 61 31 and all text after that.

7.  Save the file (make sure your editor saves in

ASCII).  (The data does not have to be formatted

like the output from BIN2HEX.  Just make sure all

data is in 2-digit hex representation and a space

seperates each one.)

8.  Convert the file back to binary using

HEX2BIN ultii.hex new.log

Page 12

The new binary log file named NEW.LOG now contains only

data for November 1994.  (This also works on any log

files created by Peet Bros.' ULTIITSR.COM program.)

Import WX.HST file entries to a database program and create

custom statistical reports and graphs.  The WX.HST file is easy

to import as a "comma-delimited ASCII file" into most popular

database programs and many spreadsheet programs.

If a BBS in your town supports modem connections, consider

leaving your weather node on the BBS frequency (or a gateway

port) so the node can be accessed by telephone, too.

An ASCII error log file is created if an error is generated by

AutoWx. If this file appears in your directory you can look at

and/or delete it. Just make sure it doesn't get big enough to

eat up disk space.

If your computer has more than 640K RAM you can use status line

display option 1 and scroll back through the data with the PC

Magazine utility PERUSE.COM once in awhile.

A program to read the binary log file and write it to an ASCII

file (WXREAD.EXE) is available for $5 (postage and diskette

included).  Send requests to the address above.

If the Upper RAIN total is zeroed, the program will continue to

log the previous URAIN amount until midnite.

The algorithm used to calculate wind chill is:

Wind Chill = .0817 * (3.71 * SQR(v) + 5.81 - (.25 * v) *

(t-91.4) + 91.4

t = ambient temperature in degrees F

v = wind speed in miles-per-hour

This formula was furnished by the National Weather Service - tnx T.J.!

I have just found some information on the MPX2100AP 15-PSIA

pressure sensor (Motorola) and would like to use it to build a

barometer that could be interfaced with AutoWx.  It seems that

it would be fairly easy to interface to a game card.  Just

think, with barometric pressure, AutoWx could provide basic

forecasts!  If somebody beats me to this project, let me know so

we can work together to include it in AutoWx.

Page 13

AutoWx Quick Guide

ESC   Clear Screen

PgDn   Show last 10 connected stations

ALT - B   BEACON on/off

ALT - C   Copy data files to Floppy

ALT - D   Shell to DOS

ALT - E   Edit/See WX.DAT

ALT - G   Enter GREETING message for additional "connect text"

ALT - Q   Quit

ALT - S   Change Status Display

ALT - T   Xmit Beacon

ALT - U   UPDATES off/on

ALT - W   Wx Beacon logging  (shows * near UPDATE when on)

ALT - X   Wx Node off/on

ALT - Z   Check/Set TIME clock

Startup Options:

/B:   Copy data files to DRIVE B: instead of DRIVE A: when

using ALT-C

/SHORT   Send short, one-line weather info on //WX requests

/KPH   Assume Ultimeter II output is in kilometers per hour

/RB   Change remote beacon time    (default = 30)

Files:

WX.CFG  AutoWx configuration data.  Includes:

TNC port                 TNC baud rate

Latitude                 Longitude

Display option           Logfile filename

WX.DAT  Data file containing current Hi's/Lo's for the date.

Date             Wind Speed               Wind Time

Wind Direction   Hi temp                  Hi temp Time

Lo temp          Lo temp Time             Chill temp

Chill temp Time

WX.HST  Data file showing Hi's/Lo's for each date

WX.ERR  Data file created only if program generates an error.

(This file can be deleted at any time.)

#5  - Illegal function call      #64 - Bad file name

#11 - Division by zero           #68 - Device unavailable

#24 - Device timeout             #69 - Communications-buffer overflow

#55 - File already open          #71 - Disk not ready

#57 - Device I/O error           #72 - Disk media error

#61 - Disk full                  #75 - Path/File access error

#62 - Input past end of file

WX.LOG  Data file of any APRS beacons recorded from other

stations.  (MONITOR must be on)

Page 14