�����������������������������Ŀ

� ABOUT THE AIRFOIL GENERATOR �

�������������������������������

The AIRFOIL GENERATOR program was developed for aviation enthusiasts who

like to experiment with airfoil design, amateur aircraft designers and

model aircraft builders.

To use the program you must enter the name of an ASCI text file that contains

the airfoil ordinate data for a selected airfoil.  Seven (7) sample airfoil

ordinate data files are supplied with this package. You may create as many

different data files as you wish. After selecting the desired airfoil data

file you must enter a proposed chord width in inches. The AIRFOIL GENERATOR

calculates a table of offset values based on the airfoil ordinate data and

the proposed chord width.

You have the option to view and or print the table of offsets that are

generated to create the proposed airfoil.

A graphic representation of the proposed airfoil is displayed on the screen.

If the chord width of the proposed airfoil is less than 10 inches then

the proposed airfoil is displayed at 100%. If the chord width is greater

than 10 inches then the image is scaled to fit the display screen.

The AIRFOIL GENERATOR does not make a hard copy of the airfoil graphic.

The program is designed to take advantage of the graphic capabilities of

many CAD (Computer Aided Design) software packages, including AutoCAD, by

creating a DXF output file of the airfoil graphic. Most CAD packages have

the ability to take input from a DXF output file. The DXF format was

developed by AutoDesk as part the AutoCAD package. It has been adopted as

a standard among most CAD software developers.

After creating the DXF output file you are returned to the main menu for the

option to create a different airfoil.

Exit the AIRFOIL GENERATOR and enter your CAD program. Load the DXF output

file that was created by the AIRFOIL GENERATOR. Once your airfoil is loaded,

it is ready for plotting or further editing with your CAD software.

���������������������Ŀ

� AIRFOIL DATA FILES  �

�����������������������

The airfoil data files contain the ordinate data used to calculate the table

of offsets for a given airfoil chord width. The airfoil data file is an ASCI

text file that contains one(1) STATION , and its corresponding UPPER and

LOWER ORDINATE on each line.  Each STATION must have a corresponding UPPER

and LOWER ORDINATE. All data is in percent of chord width. The data in the

ASCI text file must progress from the leading edge of the airfoil to the

trailing edge. Any number of stations may be entered. Most standard airfoil

data is given for STATIONS at 0, .2, .4, .6, .8, 1.25, 2.5, 5.0, 7.5, 10.0,

15.0, 20.0, 25.0, 30.0, 40.0, 50.0, 60.0, 70.0, 80.0, 90.0, 95.0, and 100

percent of chord width. Use any text editor to create an airfoil data file.

The data file must be saved with a filename and extension of ".AFD".

Review the airfoil data files that were supplied with this package for an

example of the proper data format if you need addition help.

���������������������Ŀ

� CONFIGURATION FILE  �

�����������������������

The AIRFOIL GENERATOR reads a file named "FOILS.CFG" that contains start-up

defaults for the program. The "FOILS.CFG" file is an ASCI text file that has

four (4) variables used by the program. The variables in the "FOILS.CFG" file

are:

1) Graphic Display, which must be either CGA, EGA or VGA

2) Default response for displaying the offset table, which must be a "Y" or "N"

3) Default response for creating a DXF output file, which must be a "Y" or "N"

4) Default setting for line printer, which must be "1" for ON or "2" for OFF

A typical "FOILS.CFG" file might look like:EGA,N,Y,1

The response for displaying the offset table, creating the DXF output file and

toggling the line printer can be changed at any time within the program. The

default responses in the "FOILS.CFG" file are executed if your response to a

prompt is only a Carriage Return (CR). The Graphic display must be set in the

configuration file prior to starting the program.

The configuration file "FOILS.CFG" supplied with this package is set for EGA,

"N" for displaying the offset table, "Y" for creating the DXF output file and

"0", line printer off. If you wish to change these values, use an ASCI text

editor to edit the "FOILS.CFG" file.

�������������������Ŀ

� DXF OUTPUT files  �

���������������������

As stated previously, the AIRFOIL GENERATOR takes advantage of the speed and

precision of a CAD program by creating a DXF output file. The DXF output file

can be loaded into many CAD programs, including AutoCAD and AutoSKETCH. Use

your favorite CAD program and pen plotter or other attached output device to

plot the airfoil. Once the airfoil is a drawing in your CAD program you can

edit it as you wish. Use your imagination! Consult your CAD programs

documentaion to see if it supports the DXF format and how to load a DXF file.

AutoCAD loads a DXF file by the user entering the command:DXFIN at the command

prompt. You then enter the name of the DXF output file that was created with

the AIRFOIL GENERATOR. Within a few seconds the airfoil will appear as a

new drawing.

If you do not own AutoCAD or have access to it, AutoSKETCH Version 2.0 which

retails for $149.95 supports the DXF file format and is a very powerful CAD

program for the money, incorporating many of the features of AutoCAD for a

fraction of the price.

������������Ŀ

� SHAREWARE  �

��������������

The AIRFOIL GENERATOR program is being distributed as SHAREWARE. That doesn't

mean its free! You are being given the opportunity to try a piece of software

without laying out any money.

A lot of thought and time goes into developing one of these packages. Don't

take advantage of a good opportunity by never giving the developer a little

credit for their efforts in bringing use this useful tool.

If you receive a copy of the AIRFOIL GENERATOR, try it, find it useful and

begin to use it in your work or hobby, why not let the author know you

appreciate the time and effort that went into developing the program by sending

him your name, address and a contribution for his effort. This encourages the

development of more software, some of which you might find useful.

Comments and Criticism are always welcome!

Respond to: ���������������������Ŀ

�  TDM Software       �

�  Terry Majewski     �

�  3620 L Street      �

�  Lincoln, Ne. 68510 �

�����������������������