CEAM MODELS AND THE DOS OPERATING ENVIRONMENT

Center for Exposure Assessment Modeling (CEAM)

U.S. Environmental Protection Agency

Environmental Research Laboratory

College Station Road

Athens, Georgia

30613

(404) 546-3123 / (FTS) 250-3123

Prepared by

David W. Disney

Computer Sciences Corporation

January 1988

INTRODUCTION

------------

The Center for Exposure Assessment Modeling is located at EPA's

Environmental Research Laboratory in Athens, Georgia.  For

selected water quality and pollutant loading models, the Center

provides a central support and file distribution point for

computer programs and documentation.  The purpose of this paper

is twofold.  First, the paper should assist users of PC based

versions of CEAM models in installing and executing the models on

their microcomputer system.  Second, the paper should clarify the

relationship between the models and the PC Disk Operating System

(DOS) and selected commands and options in the environment

configuration file.

Most of the information contained in this document is also

available in a printed copy of the "README.1ST" installation and

user support document which is furnished with a copy of a given

model either on diskette or through the CEAM bulletin board

system (referred to as "BBS" in this document).  A copy of the

"README.1ST" file is also on a distribution diskette set.  It is

stored as an ASCII text file which may be displayed on the

monitor screen with the DOS TYPE command or printed using the

PRINT command.

BACKGROUND

----------

The following table lists the language and program development

tools used at CEAM to build the distribution versions of the PC

based model systems:

Computer System:  IBM PC/AT Model 339

Language:         FORTRAN 77

Operating System: PC DOS version 3.21

Compiler:         Ryan-McFarland FORTRAN

RMFORT version 2.42

Link Editor:      Phoenix Technologies LTD.

PLINK86 Plus version 2.23

There are some facts concerning the development and operating

environment for the models that each user should be aware of.

First, each model should execute on the family of IBM

microcomputer systems from an XT through the PS/2 systems,

assuming the presence of a math numerical coprocessor for

selected models, enough disk storage space, and adequate random

access memory (RAM).  More on RAM requirements later.

Second, each model should also execute on a "near 100%" IBM

compatible system such as a COMPAQ or AT&T microcomputer system.

The programs should also work with PC or MS DOS operating systems

with 2.12 and later versions.  HOWEVER, please be aware that we

cannot possibly guarantee this result since we are unable to test

these models in most cases on anything but our own resident IBM

systems.

Third, all executable task images for the models as well as the

source code files, and batch command files to compile and link a

given model system are included on a distribution diskette set or

on our CEAM bulletin board system.  PLEASE NOTE however, that a

FORTRAN compiler and link editor are NOT required to EXECUTE any

portion of a PC based model as received on a distribution

diskette set or through the BBS.

Finally, if the user decides to modify a given model, it is up to

the user to supply an appropriate text editor and/or FORTRAN

program development tools to recompile and link edit any portion

of a given model system.  We cannot support, maintain, and/or be

responsible for modifications that change the functionality of a

given model executable task image.

PC ENVIRONMENT CONFIGURATION

----------------------------

One of the most important operating system files that the user

has access to and control of on a DOS based microcomputer system

is the environment configuration file CONFIG.SYS.  This file is

so named because it may contain a list of commands that set and

control the environment under which your entire PC system will

operate.

This file should reside in the root directory of your "boot" or

start-up diskette or hard disk drive, for example C:\CONFIG.SYS.

Each time you "cold start" (power off/power on) or "warm start"

(<Ctrl><Alt><Del>) your system, DOS searches the root directory

of the drive from which it was started for the file CONFIG.SYS.

If this file is found, DOS reads the file and interprets the

commands within the file.  If the file is not found, DOS assigns

default values for the configuration of the system.

Any text editor program that saves files in ASCII format, such as

DOS EDLIN or the DOS COPY command may be used to edit and/or

create the \CONFIG.SYS file on the "start-up" disk drive.

REMEMBER, these changes will not be in effect until you re-boot

your system (i.e., power off/power on or use the warmstart

procedure by holding down the <Ctrl><Alt> keys and pressing the

<Del> key at the same time).

The sample configuration file shown below lists the recommended

MINIMUM configuration file statements for the implementation and

execution of the CEAM models under MS or PC DOS.  For a full

discussion of all configuration statement commands and options,

please refer to the appropriate DOS Reference Manual.  Make sure

that your IBM PC or compatible system has these statements in the

C:\CONFIG.SYS file and that you have restarted ("warm" or "cold")

your system to implement these changes before attempting to

execute the CEAM models and their related commands and programs.

Sample configuration file C:\CONFIG.SYS:

1)          BREAK=ON

2)          BUFFERS=20

3)          FILES=20

4)          DEVICE=C:\ANSI.SYS

5a)         SHELL=C:\COMMAND.COM C:\ /E:1024 /P

(for PC DOS 3.2x)

-or-

5b)         SHELL=C:\COMMAND.COM C:\ /E:50 /P

(for PC DOS 3.1x and earlier versions)

The following sections discuss each CONFIG.SYS file statement.

Statement 1

-----------

The BREAK command allows you to instruct DOS to check for a

control-break command (i.e., pressing the <Ctrl><Break> or

<Ctrl><C> keys) whenever a program requests DOS to perform ANY

function.  The default value BREAK=OFF means that DOS will check

for a control-break command from the user only during selected

DOS input/output operations.

The BREAK=ON command setting allows you to "break out" of a model

run that requires few, if any, disk, monitor screen, and/or

printer input/output operations.  For example, if a model run is

"compute intensive" or is in an infinite loop because of data

errors, it is important to have a way to stop the program and

return to the DOS control level.

Statement 2

-----------

The BUFFERS command determines the number of disk input/output

buffers that DOS will allocate in memory when it starts.  I

recommend that you use a value between 20 and 32 for the BUFFERS

command in the CONFIG.SYS file--a setting of 20 is usually

adequate.  If this statement is not used, the default value is

"2" ("3" for the IBM AT).

The concept and actual operation of disk "buffering" cannot be

adequately discussed in this paper.  However, the user should be

aware of two things: 1) A disk buffer is a block of RAM memory

that DOS uses to store data being read from or written to disk;

and 2) The use of this configuration command provides the user

with control over the performance and speed of his/her own PC.

In general, the higher the value in the BUFFERS statement, the

greater the speed and overall performance of the PC (up to a

certain limit).

The final consideration in determining the number of buffers to

allocate is the memory size of your computer.  Each additional

buffer unit allocated will decrease the amount of memory

available to the application programs by 528 bytes.  For example,

the statement BUFFERS=20 would use approximately 10,560 bytes of

random access memory (RAM).  Remember, more on RAM requirements

later.

NOTE: Do not set this value too high, for example >64.  Too many

buffers may actually cause some applications to run more slowly,

or not at all, because there is less memory available for the

programs execution code and/or data.  Since there would be less

memory available for the program to hold data, a high BUFFERS

value could result in more frequent reads and writes to the disk

than would otherwise be necessary.

Statement 3

-----------

The FILES command specifies the maximum number of files that can

be opened concurrently by the same application program.  Since

many of the CEAM model programs can have a relatively large

number of files open at once, you should set the FILES parameter

value in the configuration file large enough to accommodate the

appropriate program.  I recommend that you set this value between

20 and 30, with a setting of 20 being adequate for the model

versions currently being distributed.

The user should be aware of three points:

1)  While a value of 20 is adequate for the CEAM models, a higher

value may be required for other commercially developed software

systems to execute properly.

2)  Another consideration in determining the number of files to

have open is the memory size of your computer.  Each additional

file above the default value of 8 will decrease the amount of

memory available to application programs by 48 bytes.  For

example, the statement FILES=20 would use approximately 576

additional bytes of random access memory (RAM).

3)  If this value is not set high enough for a given application

program, the user should see the following Ryan-McFarland FORTRAN

run-time error message displayed on the monitor screen when

attempting to execute a given CEAM model:

Error at line x in pgmname  Error number 3012

where "x" is the line number in the program named "pgmname" where

the error occurred.

For example, the HSPF program opens approximately 20 files

concurrently during execution.  If the FILES parameter is allowed

to default to 8, this error message would appear upon attempting

to open the 9th file.

Statement 4

-----------

The DEVICE statement specifies the name of a file containing a

device driver.  For example, a device can be a printer, disk

drive, monitor, or keyboard.  This version of the DEVICE command

causes DOS to replace the standard input and standard output

support on the keyboard and monitor with extended functions.  For

the CEAM models, I recommend that you specify the "Extended

Screen and Keyboard Control" device driver ANSI.SYS since many of

the CEAM models support such functions as screen clear and cursor

positioning.  For a complete description of the features of this

file, please refer to the appropriate DOS Reference and/or

Technical Reference Manuals.

In the example shown above, it is assumed that the file ANSI.SYS

resides in the root (\) directory of the start up disk drive

(C:).  Make sure you enter the appropriate path indicating the

location of the file on your system.  For example, if you have

the ANSI.SYS file in a root sub-directory C:\DOS, then the

correct DEVICE statement would be:

DEVICE=C:\DOS\ANSI.SYS

Statement 5

-----------

The SHELL command specifies the name and location of the top-

level command processor that the DOS initialization sequence

loads at system start-up.  A command processor is simply a

program that reads and interprets DOS internal commands and batch

command files (*.BAT) and tells the system "what to do".  For

example when you type the DIR (directory listing) command it is

read and executed by the program COMMAND.COM.

This version of the SHELL command indicates to DOS the name of

the standard or default command processor COMMAND.COM.  I

recommend you use this file unless your system runs under a

special command interpreter or processor such as PolyShell.

Again, in the original SHELL statement example

SHELL=C:\COMMAND.COM

it is assumed that the file COMMAND.COM resides in the root (\)

directory of the start-up disk drive (C:).  However, make sure

you enter the appropriate path indicating the location of the

file on your system.  For example, if you have this file in a

root sub-directory C:\SYSTEM, then the correct SHELL command

entry for this option would be:

SHELL=C:\SYSTEM\COMMAND.COM

The next SHELL command parameter indicates to DOS the location of

the command processor in case it needs to be reloaded.  Again,

the example shown in statements 5a and 5b of the sample

configuration file above assumes that the location is in the root

directory (\) of the C: drive.  If the command processor is

located in the root sub-directory C:\SYSTEM, then this portion of

the SHELL command entry would be:

SHELL=C:\SYSTEM\COMMAND.COM C:\SYSTEM\

IMPORTANT NOTE: If the command processor file COMMAND.COM is

in any directory other than the root (\), then the following

command must be added to the file C:\AUTOEXEC.BAT:

SET COMSPEC=C:\subdir\COMMAND.COM

where "subdir" is the root sub-directory (e.g, C:\SYSTEM)

indicating the location of the command processor file.

The SHELL command parameter "/E:" provides a mechanism to modify

the size of the "environment space" under which DOS and

application programs execute.  There is not enough space in this

paper to fully discuss the concept of the DOS environment and its

operation; however, the user should know that several of the CEAM

models, such as HSPF, require a very large environment space in

order to execute correctly.

If you are using DOS version 3.2x, I recommend that this value be

set at 1024 as shown in statement 5a above, or a value of 50 as

shown in statement 5b if you are using DOS 3.1x or an earlier

version.  If this value is set too small, the user should receive

the following error message when attempting to execute a given

CEAM model:

Out of environment space

To verify the version of DOS on your system in order to determine

which form of the SHELL command to use, simply enter the VER

command at the DOS system prompt and press the <Enter> key.  The

version number will appear on the monitor screen.  For example:

C>VER

IBM Personal Computer DOS Version  3.20

While the sizes shown above are required for CEAM models, the

final consideration in determining the size of your system's

environment space is the memory size of your computer.  This

means that the amount of memory (RAM) available for the execution

of application programs will be decreased by the size used for

the environment space.  For DOS version 3.2, the size of the

environment is indicated in actual bytes.  Therefore, statement 5a

shows that 1,024 bytes are used for the environment space.  For

DOS 3.1 and earlier versions, the size of the environment is

indicated in multiples of 16 bytes.  Therefore statement 5b shows

that 800 bytes are used for the environment space.

Finally, the parameter /P indicates that this implementation of

the command processor will be permanent and that DOS will

continue to perform and behave as it always has.

Based on the last SHELL command example in this section and

assuming DOS version 3.2x, the full SHELL statement would be

SHELL=C:\SYSTEM\COMMAND.COM C:\SYSTEM\ /E:1024 /P

RANDOM ACCESS MEMORY (RAM) REQUIREMENTS

---------------------------------------

Random access memory, or "RAM" as it is usually called, is a term

used to describe that portion of your computers storage area that

is used to EXECUTE programs, such as a BASIC program, the CEAM

FORTRAN water quality models, or your favorite word processing or

spread sheet software.  Another commonly known area of memory is

"disk" memory which in general is used to STORE the program code

and data files.

Today, most DOS based microcomputer systems are usually equipped

with either 512k or 640k bytes of RAM.  Older systems may have

only 256k (the "k" is shorthand for the value "1,024") while some

newer systems may have 1,024k of RAM.  So a 640k RAM system, for

example, actually has 655,360 bytes of RAM memory.

Determining the RAM size of your system is as simple as executing

the DOS CHKDSK (check disk) command.  For example, at the DOS

prompt type the following command then press the <Enter> key:

C>CHKDSK

This command should display output on your monitor screen similar

to the following listing:

Volume CEAM_PC_AT  created Oct 20, 1987 10:03a

31768576 bytes total disk space

47104 bytes in 4 hidden files

178176 bytes in 66 directories

20629504 bytes in 1464 user files

10913792 bytes available on disk

655360 bytes total memory

575120 bytes free

The first five lines of the listing give statistics on the disk

storage used and currently available.  The next-to-the-last line

of this listing indicates the total RAM size of your

microcomputer system.  Here "655360 bytes total memory"

translates into a 640k system (i.e. 655360 divided by 1024

bytes).  The last line of this listing indicates how much of this

memory (RAM) is available for the execution of a given program.

In this listing, 575,120 bytes or approximately 562k are

available.

The difference in these two values (total memory minus free

memory) is generally taken up by the memory requirements of the

operating system itself, DOS memory based commands such as PRINT

and MODE, other commercial memory resident software such as

SideKick and SuperKey, the settings chosen for the FILES and

BUFFERS commands in the CONFIG.SYS file, RAM disk or "virtual"

disk drives (not discussed in this paper), and the value chosen

for the environment size in the CONFIG.SYS file SHELL command.

Sometimes in program documentation "memory resident" programs are

referred to as "TSR" programs where TSR stands for "terminate and

stay resident".

It is very important for the user to understand that each CEAM

model has its own unique available memory size requirement for

successful execution.  The actual memory requirement of a given

model should be compared with the last line of the CHKDSK command

output to see if enough RAM is available for the execution of

the model.  This required memory information is listed in the

DEVELOPMENT SYSTEM section of the README.1ST document that is

provided with each model.  For example, the following is a

portion of the DEVELOPMENT SYSTEM section for the QUAL2E and

QUAL2EU models:

Storage Requirements:       (k=1,024; m=1,048,576)

Random Access Memory:  185k bytes needed by QUAL2E program

32k needed for program CACHE overhead

----

217k free bytes needed to execute

QUAL2E program

241k bytes needed by QUAL2EU program

32k needed for program CACHE overhead

----

273k free bytes needed to execute

QUAL2EU program

Use DOS CHKDSK command to verify bytes

free in RAM memory

256k RAM system (minimum) recommended

for QUAL2E

512k RAM system (minimum) recommended

for QUAL2EU

This information indicates that the QUAL2E program requires 217k

of free RAM to execute.  Or put another way, in order to execute

properly, QUAL2E requires a minimum of 217k of available RAM in

addition to what is needed by the operating system and any

resident programs.  Therefore QUAL2E could probably be executed

on a 256k RAM system.  The QUAL2EU program, however, requires

273k of free RAM and therefore needs to be executed on 512k

system.  According to the CHKDSK command listing shown above,

either program could be executed on the system that produced the

listing (i.e., both run-time memory requirements are less than

575,120 bytes or the RAM available on this system).

As another example, the HSPF program requires approximately 595k

of free RAM to execute.  This program could only be executed on a

640k system but could not be executed at this time on the system

that produced the CHKDSK listing above (595k > 575,120).

PLEASE NOTE:  If the available memory requirements of a given

model are not met at execution time, the user will see the

following error message displayed on the monitor screen upon

attempting to execute the program:

Program too big to fit in memory

If this message appears, the user should consider the following

actions to increase the available memory:

1)  remove any "virtual" disk drives

2)  remove non-essential device driver statements (DEVICE) from

the CONFIG.SYS file

3)  remove any commercial memory resident software from the

system (usually found in C:\AUTOEXEC.BAT or CONFIG.SYS files)

4)  adjust the value chosen for the environment size in the SHELL

command in the CONFIG.SYS file

5)  adjust the values chosen for the FILES and/or BUFFERS

commands in the CONFIG.SYS file

6)  restart the system (warm or cold start) to remove memory

resident PRINT and MODE commands and to implement other

changes from 1, 2, 3, 4, and/or 5 above

TECHNICAL CONTACT

-----------------

If you have any questions concerning the installation or

execution of any CEAM model on your microcomputer system, please

contact David Disney in the Athens ERL ADP Section at 404/546-

3123 or (FTS) 250-3123.  Questions concerning model content,

application, and/or theory should be addressed to Tim Wool at the

same number.

DISCLAIMER

----------

Mention of trade names or use of commercial products does not

constitute endorsement or recommendation for use by the United

States Environmental Protection Agency.

####################

DWD - 02/03/88 - C:\MISC\CEAM_DOS.DOC

#. 655360 divided by 1024

bytes).  The last line of this listing indicates