POPDIAL   Version 1.0

(C) 1991   Scanlon Enterprises / Paul Scanlon

POPDIAL is a memory resident (TSR) utility, which will dial a

number, located on the diskplay or one which the user enters.

POPDIAL automatically detects if the cursor is at a usable phone

number, and will dial that number, without further attention from

the user. If the cursor is NOT at a valide phone number, then

POPDIAL will prompt for a number.

POPDIAL is a text based system, and will NOT function correctly

if poped-up in graphics mode. No attempt is made, by POPDIAL to

check the video mode, so the user must assure that POPDIAL is NOT

activated during graphics mode.

POPDIAL can handle dial strings up to 33 characters, and may

include a comma, to produce a pause of 2 seconds (your modem

default). The possible string characters, permitted are, the

numberic keys '0' thru '9', and '#*ABCD,!'. Additionally, the

characters '-() ' are allowed in the string, but are ignored.

The '!' is a special character, used by HAYES compatible modems

to flash, which takes the modem off line momentarily to get a new

dial tone.

Start POPDIAL by entering 'POPDIAL port', where 'port' is a valid

COM port (usually 1-4). Once started, POPDIAL displays the

installation message, which includes the HotKey to use to bring

POPDIAL into play. This special hotkey is 'Control D'.

IE... After installing POPDIAL, press 'control D' to start a dial

sequence.

This version of POPDIAL requires 8K bytes of memory to run.

Each occurance of the ',' (comma) will pause your modem, from

further activity, for about 2 seconds. This is useful for

situations in which you wish to dial a pager, or in which you

must dial a prefix (such as 9) to gain outside line access.

POPDIAL will automatically prefix a long distance number, with

the '1'. A long distance number is a string of numeric characters

which is 10 characters long. If the sting of numbers is, lets

say, 7 characters, then the '1' will NOT be prefixed.

EXAMPLE:

800 222 4582

Will be prefixed with a '1', while

222 4582

Will NOT

POPDIAL requires a HAYES compatible modem.

Registration brings a version which will dial a number, chosen

from a list of 25 numbers, port changing (change COM ports),

select a file containing the phone list (even after activated),

automatic detection of video mode and of course, phone support.

REGISTER:

_______ Copies of POPDIAL @ $10  Ea.                  _________

Shipping :   $ 2 -- US      $3 -- Other               _________

State Tax : California  8%                            _________

Total ----------------------------------------------  _________

Send my order to :  _________________________________

_________________________________

_________________________________

_________________________________

Send your check or money order to :

Scanlon Enterprises, 38354 17th St. E., Palmdale, CA 93550

#####################################################################