ROUTE 1.10

9/17/91

ROUTE is a little program that displays the detailed routing for a

flight between any two of the 26 major airports in subLogic's ATP

(Air Transport Pilot).

ROUTE can help to clarify the exact routing between two airports.

ATC's clearance identifies only the VORs at the intersections of jet

routes; the VORs along a route are not identified.  Sometimes it is

difficult to figure out all of the VORs from the subLogic charts.

For example, I was never able to complete a flight between SAN (San

Diego) and SFO (San Francisco) because I couldn't identify some of

the waypoints from the subLOGIC charts; I always ended up deviating

from the flight plan and getting chided about it by ATC.

ROUTE also provides accurate mileage statistics for any flight

between the major airport pairs and a list of all the VOR, ILS, and

communications frequencies you will need.

| indicates changes in the latest version.

Using ROUTE

------------

To use ROUTE, simply run the program and provide the three-letter

airport codes for the origin and destination airports.  For example,

to see the routing for ORD (Chicago) to PHX (Phoenix), ensure that

the ATP directory is the current directory and type:

route ord phx

The flight plan is displayed on screen; of course, you can print the

report by typing:

route ord phx > prn

or save it in a text file via

route ord phx > ord-phx.txt      (for example)

You can also ask ROUTE to report ALL of the routes in the ATP

database by typing ALL instead of the airport IDs:

route all > routes.txt

(Be forewarned that this requires nearly 360K of disk space.)

The ROUTE report

----------------

The ROUTE display for Chicago to Phoenix would be as follows:

Clearance for ORD -> PHX

Awy    Node   North  East   Freq   Rad  Dist  Rng

---    ----   -----  ----   ----   ---  ----  ---

J35    OBK    17338 16529  113.00  200    44  132

JOT    17043 16415  112.30  244    62  132

J26    BDF    16874 16003  114.70  240   151  132

IRK    16449 15008  114.60  232   107  132

MKC    16110 14326  112.60  254   144  132

J24    SLN    16032 13299  117.10  236   159  132

J18    GCK    15712 12206  113.30  224   254  132

LVS    14973 10548  117.30  232    91  132

ABQ    14819 09916  113.20  240   122  132

SJN    14725 09048  112.30  232   151  132

SRP    14534 07986  115.60  n/a   n/a  132

Total                            1286

ORD ATIS-135.40  Clr-121.60  Gnd-121.90  Twr-126.90  Dep-119.00

PHX ATIS-121.20  App-120.70  Twr-118.70  Gnd-121.90

26L-108.30   8R-108.30

"Awy" is the airway number.  It is shown only for the first node

(waypoint) for each airway.  For example, BDF, IRK, and MKC are all

on airway J26 (SLN is the point of intersection of J26 and J24).

In some cases, the word DIRECT may appear in the airway field.  This

is used when there is no defined airway connecting two nodes.  For

example, part of the report for BOS-MIA is:

J225   BOS    17993 21925  112.70  252    45

...

JFK    17093 21176  115.90  258    57

DIRECT ARD    16869 20835  108.20  234    48

J51    DQO    16588 20644  114.00  232    79

...

There is no jet airway on the chart between ARD and DQO, so the leg

is "direct DQO".

"Node" is a navigation point--usually a VOR.  The north and east

coordinates of the waypoint are also shown.  If the node is a VOR,

the VOR frequency is given.

"Rad" is the radial to follow OUTBOUND from the node.  This may

differ slightly from the radials shown on the ATP charts (it is

always an even number, and the charts show some odd numbers).

"Dist" is the distance from the listed waypoint to the next one.  It

is given in nautical miles (NM).

| "Rng" is the range of the VOR (if the node is a VOR).  This is the

| approximate distance from the node (in nautical miles) at which you

| will pick up the VOR signal.  The ranges of the VORs in the sample

| report all happen to be 132nm, but there are VORs with ranges of

| about 22nm, 70nm, and 182nm.  This information is not enormously

| significant if you fly the built-in ATP flight plans, because you

| will never be out of range of both VORs on any leg.  However, if you

| use our FPED *FlightPlan EDitor) to create your own flight plans,

| this situation CAN occur, and it is useful to get some idea of how

| long you'll be out of touch with your navaids.

The total distance for the clearance is 1286 NM; the actual flight

will likely be longer due to ATC vectoring during approach and

departure.

| Below the routing report is a list of useful frequencies.  The first

| line ("ORD" in the sample) provides the ATIS, clearance, ground,

| tower, and departure frequencies for the departure airport.  The

| second line ("PHX") shows the ATIS, approach, tower, and ground

| frequencies for the destination.  Below this are listed all available

| ILS runways and their ILS frequencies.  This information may not be

| shown if you have a version of ATP that we don't know about (please

| let me know).

Example

-------

Here is a portion of the ORD-PHX routing:

Awy    Node   North  East   Freq   Rad  Dist

---    ----   -----  ----   ----   ---  ----

J26   ...

IRK    16449 15008  114.60  232   107

MKC    16110 14326  112.60  254   144

J24   SLN    16032 13299  117.10  236   159

Theoretically, to navigate from IRK to MKC on airway J26, you'd

follow IRK radial 232 (frequency 114.6) until the DME reads 107.

Ignoring the effect of your altitude on the DME, you should be

approxiamtely over MKC. From MKC (frequency 112.6), follow radial

254 for a distance of 144 NM to reach SLN (the intersection with

J24).

In actual practice, you will not follow the outbound radial all the

way from one VOR to the next; it is too inaccurate.  You will want

to switch navigation to the next VOR when about halfway between two

stations.

The simplest technique, assuming that NAV1 is tuned to 114.6 (IRK),

NAV2 is tuned to 112.6 (MKC), and that you are now at IRK:

1. Set the NAV1 radial to 232; set the autopilot to NAV1.

2. Watch the DME.  Have a couple beers; chat with the pax (only

the ones in first class, of course); complain to the

flight attendants about the food; glare at the first officer,

a young whippersnapper who's after your job.

3. At DME 53 (about halfway), put down the beer for a moment and

adjust the radial on NAV2 until the course deviation

indicator is centered.  This will be somewhere near 232.

4. Switch the autopilot to NAV2.  Also switch the DME, if

necessary (if the plane you're flying has only one DME).

5. Finish your beer and wait for the DME to bottom out and

begin to increase (or, wait for the RDF to swing); you'll

be right at MKC.  Adjust the NAV2 radial to 254 (the

outbound radial from MKC) to begin the leg from MKC

to SLN.

Non-VOR waypoints

-----------------

Some waypoints are not VORs.  For example, here is part of the

routing for BOS-MIA:

Awy    Node   North  East   Freq   Rad  Dist

---    ----   -----  ----   ----   ---  ----

J174   CHS    13485 19484  113.50  216    98

STARY  12854 19181    --    210   138

GNV    11968 18751  116.20  156   245

Note that STARY has no frequency; it is not a VOR.  If you look at

the Southeastern US chart, you'll note a bend in J171 between CHS

and GNV, marked with a small triangle and "STARY".  (This one

happens to be marked on the map; some, such as PACER on J225 between

JFK and HFD, are not marked.)

If you are flying an IRS-equipped aircraft, set a waypoint at STARY

(12854N/19181E) as part of your preflight routine; use the waypoint

for navigation just as though it were a VOR.

If you're not flying an IRS aircraft, you can't directly detect your

passage over STARY; you'll have to do the best you can.  Follow

radial 216 outbound from CHS; set the second nav radio to the

inbound radial shown on the chart for GNV (042+180 = 222).  At a

point approximately 138 DME from CHS, the GNV needle will center;

you are now approximately over STARY.  Switch to the GNV radio for

navigation.

Airport IDs

-----------

For reference, here are the 26 airport IDs:

Albuquerque/Alb. Intl.           ABQ

Atlanta/Hartsfield               ATL

Boston/Logan                     BOS

Chicago/O'Hare                   ORD

Cleveland/Hopkins                CLE

Dallas-Ft. Worth/DFW Intl.       DFW

Denver/Stapleton                 DEN

Detroit/Detroit Metro            DTW

Houston/Houston Intcntl.         IAH

Kansas City (MO)/KC Intl.        MCI

Las Vegas/McCarran               LAS

Los Angeles/LA Intl.             LAX

Miami/Miami Intl.                MIA

Minneapolis/M.-St.Paul Int       MSP

New Orleans/N.O. Intl.           MSY

New York/Kennedy                 JFK

Philadelphia/Phil. Intl.         PHL

Phoenix/Sky Harbor Intl.         PHX

Pittsburgh/Grtr. Pit. Intl.      PIT

Salt Lake City/SLC Intl.         SLC

San Diego/Lindbergh              SAN

San Francisco/SF Intl.           SFO

Seattle/Seattle-Tacoma Int       SEA

St. Louis/Lambert                STL

Tampa/Tampa Intl.                TPA

Washington/Dulles                IAD

Version history

---------------

1.10 9/17/91

Added list of useful COM and ILS frequencies

Added VOR range

1.00 9/12/91

Inital release

****************

Thanks and kudos to Dave Hendrix for excellent detective work in

deciphering most of the format of ROUTES.BIN and NODES.BIN.

****************

ROUTE is donated to the public domain without warranty express or

implied.  Neither the author nor The Cove Software Group are

responsible for any problems, real or imagined, caused by the

program.

Chris Dunford

The Cove Software Group

PO Box 1072

Columbia, MD 21044

301/992-9371

CompuServe 76703,2002