S P -  F O O T I N G

By

Terry W. Towle

A Program to Design Square Spread Footings

With Numerous Column Options

Using ACI 318-89 Code

Terry W. Towle

112 Brentridge Drive

Antioch, TN  37013

Version 1.0

February 1991

Copyright 1991 by Terry W. Towle.  All Rights Reserved.

TABLE OF CONTENTS

1.  LICENSING PROVISIONS................................ Page 1

2.  REGISTRATION/INVOICE FORM............................Page 2

3.  INTRODUCTION.........................................Page 3

4.  HOW TO RUN SP-FOOTING................................Page 4

5.  GENERAL ASSUMPTIONS..................................Page 5

6.  METHODOLOGY..........................................Page 6

7.  REQUIRED INPUT/FUTURE CHANGES........................Page 7

8.  PROBLEM/SUGGESTION FORM..............................Page 8

SP-FOOTING License Provisions                Page 1

SP-FOOTING is made available under the concept of SHAREWARE.  SHAREWARE is

a "Try Before You Buy" approach to marketing and distribution of products.  You

are free to copy this software and distribute it provided that all accompanying

files are distributed intact and unmodified.  Only a reasonable distribution fee

of $10.00 or under may be charged.  No changes to this software may be made.

The author retains ALL RIGHTS to this program.

Why Register Your Copy?

SHAREWARE is not free and it is not Public Domain.  If you find that you

are still using this or any software after a evaluation period, then please

consider supporting the efforts of the author with a small contribution.  A

tremendous amount of manhours has gone into the development of this program.

Using it to design just a few footings or just as a quick method to find the

effective depth (d) will quickly pay for the registration fee.  This contribution

will enable this author to write other civil engineering software programs and

continue to upgrade this one.  Schools, companies and other organizations are

encouraged to write for information about site licensing for multiple copies of

SP-FOOTING.  To register this version of SP-FOOTING, send $18.00 plus $2.00

shipping/handling to:

Terry W. Towle

112 Brentridge Drive

Antioch, TN  37013

615-834-9479

For the registration fee you will recieve:

1.     The registered version of the software on a new disk.

2.     The complete Turbo Pascal source code with comments for the

program.

3.     The latest version of SP-FOOTING should you have an older

version.

4.     A $5.00 credit towards the next updated version of SP-FOOTING and

notification of updates and other items of interest.

5.     The removal of the "UNREGISTERED SOFTWARE" heading on reports and

the various registration prompt screens.

6.     A professionally printed and bound manual including copies of the

hand worked problems used to verify the accuracy of the program.

7.     Good feelings for owning a piece of legal software and supporting

the SHAREWARE concept, which enables the development of reasonable

priced, quality software.

NOTICE - NOTICE - NOTICE

If you paid a SHAREWARE vendor a fee for this program, you paid for copying

and distributing the program only.  The author does not recieve any royalties

or fees of any kind for these sales.  You may evaluate this software, but if you

continue to use it, you should register your copy.

SP-FOOTING REGISTRATION/INVOICE FORM                    Page 2

NAME AND:  __________________________                     P.O. # ___________

ADDRESS

__________________________                   DATE: ____/____/____

__________________________      SP-FOOTING VERSION ______________

__________________________      DISK SIZE ____ 5 1/4  ____ 3 1/2

WHERE DID YOU GET THIS VERSION OF SP-FOOTING?  ____________________________

---------------------------------------------------------------------------

Quantity Ordered   ________       SINGLE ISSUE REGISTRATION  $20.00 Postpaid

(Check in U.S. Funds)  Includes registered

disk, source code, bound manual and hand-

worked verification problems.

TOTAL AMOUNT OF

INVOICE           $________

Mail TO:  Terry W. Towle

112 Brentridge Drive

Antioch, TN  37013

615-834-9479

COMMENTS:________________________________________________________________

_________________________________________________________________________

_________________________________________________________________________

_________________________________________________________________________

OTHER SOFTWARE YOU WOULD LIKE TO SEE:   ____  Retaining Wall Analysis/Design

____  Combination Footings        ____  Concrete Beam or Column Design

____  Earthwork Cut/Fill          ____  Baseplate/Anchor Bolt Design

____  Wood Beam/Column            ____  Other (Explain)

INTRODUCTION                           Page 3

This program designs "typical" interior spread footings with axial loads

only and without moments.  That means that the columns must be centered on the

footing and that no other moment is transferred through the column.  This version

will handle square, rectangular or round columns of concrete, masonry or with

steel baseplates.  It is written according to the latest AMERICAN CONCRETE

INSTITUTE 318-89 code.

LEGALESE

No warranties, guarantees or accuracy are expressed or implied including

but not limited to, the implied warranties of merchantibility or fitness for a

particular purpose.  As is normal with engineering software, you are directed

to thoroughly test this software against known examples of footing design and

use normal good engineering judgement when using this program.  This software

was tested against many examples of footing design and Registered Users will

recieve copies of the hand worked verification problems, but I make no claims

as to the accuracy of the solutions.  However, great effort has gone into the

verification process, including the review by multiple civil engineers.

ACI CODE 318-89 PROVISIONS

This software designs spread footings according to the following code

sections:

Reinforcement spacing                 7.6

Minimum reinforcement cover           7.7.1

Temp & shrink reinforcement           7.12

Footing Load Combinations             9.2

Load Factors                          9.3.2

B1 Factor                             10.2.7

Max. reinforcement ratio              10.3.3

Reinforcement limits in compression   10.9

Bearing on footing                    10.15, AISC J9

1-way shear                           11.12.1.1

2-way shear                           11.12.2

Reinforcement development length      12.2, 12.6

Round columns on footings             15.3

Location of moment                    15.4.2

Location of shear                     15.5

Minimum edge thickness                15.7

HOW TO RUN SP-FOOTING                     Page 4

The SP-FOOTING program consists of 8 files.  They are listed below:

SPFOOT.EXE......The Actual Program

SPFOOT.DOC......This User's Manual

INVOICE.DOC.....A Separate Invoice File

README.AAA......A Short Introduction to the Program

CGA.BGI.........Graphics Driver for CGA

EGAVGA.BGI......Graphics Driver for EGA/VGA

HERC.BGI........Graphics Driver for Hercules

PC3270.BGI......Graphics Driver for 3270

* TO RUN SP-FOOTING ON A FLOPPY DRIVE:  Insert the diskette containing the

SPFOOTING.EXE and all the *.BGI files in either A or B Drive and access that

drive by typing either A: or B: and pressing ENTER.  You then type SPFOOT to run

the program.  Simple.

* TO RUN SP-FOOTING ON A HARD DRIVE:  Make a directory called FOOT by typing

MKDIR FOOT at the C:> prompt.  Insert the floppy disk in drive A or B and type

COPY A:\*.* C:\FOOT or COPY B:\*.* C:\FOOT.  This will copy all the files from

your floppy disk to the hard drive.  To run the program, get into the FOOT

subdirectory by typing CD\FOOT from the C:> prompt.  To run the program type

SPFOOT.   (NOTE: The Graphics Drivers must be in the same directory as SPFOOT.EXE

for the program to run.

The program is then self-explanatory.  You will be prompted for the

necessary input data.  After calculating the footing, you will be shown a summary

report on screen and be given the option of printing one of two reports

containing the analysis and the design of the footing.

GENERAL ASSUMPTIONS                       Page 5

Some assumptions are made by the software.  They are included below:

1.   The minimum effective depth allowed is 8.25".  This allows you to use #3

bar for minimum concrete column reinforcement and obtain the required

development length.  Adding 3" for cover and an average bar diameter of

0.75" gives you a minimum footing thickness of 12".

2.   The smallest footing size allowed is 2.5' x 2.5' or the column size plus

1.5 x effective depth. This takes care of the problem of trying to enter

a small load on a large column and having the program size the footing

smaller than the column for bearing.

3.   All footings are rounded to the nearest 3".  Strengths and forces given are

correct for this size.

4.   The number of column reinforcement dowels are increased and the size of bar

reduced when the development length is inadequate.  The depth of footing

is not increased.  If the number of dowels is excessive, it may be more

economical to increase the footing thickness instead.  Footing thickness

given is the minimum for strength purposes.  It should be rounded up to the

nearest 1".

5.   When you are analyzing a masonry column or steel column with baseplate, the

program checks bearing strength for the footing only.  You must make sure

the masonry bearing pressure and the steel baseplate thickness is

acceptable.  Minimum reinforcement steel area is given for the masonry

column.  If the steel column has unacceptable bearing pressure, it is

flagged on the report.  It is up to you to change concrete strength, column

size or baseplate dimensions.

6.   The number of bars and spacing for bending reinforcement steel is kept

between 6" and 12".  If a different size is preferred, the total area of

steel required is listed.  When going to a larger diameter bar than the one

listed, the footing thickness may have to be increased to handle the

shortened effective depth (d).

7.   Care has been taken to handle most bizarre input scenarios, however if you

input something wild like a 500 kip load on a 5" column with 600 psf bearing

capacity soil - it's your own fault.  Garbage In - Garbage Out.

METHODOLOGY                          Page 6

The design of square spread footings involves the determination of the size

and depth of footing and the amount of main reinforcement and dowels needed to

resist all loads.  It is usually a repetitive process, which lends itself nicely

to computer applications.

STEP 1:

Size the footing area based on the allowable bearing capacity minus the weight

of the footing minus the weight of the overburden soil vs. the loads.  Apply the

load factors and determine the factored soil pressure.

STEP 2:

Determine the depth of the footing based on 2-way shear.  A critical section is

determined based on footing size and column size and type.  This section is

tested with the factored soil pressure.  If the concrete strength is far greater

than the shear applied, the footing thickness can be reduced.  It is an iterative

process.  Check your initial assumption of the weight of the footing when a final

thickness is reached and recalculate if necessary.  Check against 1-way shear.

2-way shear normally controls.

STEP 3:

Check the concrete column load transfer by bearing on the footing.  If OK then

size the minimum steel.  If not OK, then size the necessary transfer steel to

handle the excess load.  Check the development length.  If not OK, increase the

number of bars and reduce their size or increase the footing thickness.  If it's

a masonry or steel column, check footing bearing only.  Use minimum steel for

masonry columns.

STEP 4:

Determine the bending moment at the critical section and calculate the necessary

steel to resist it.  The critical section is determined based on the footing size

and column size and type.  Check the development length.  If not OK, increase

the number of bars and reduce their size.

REQUIRED INPUT                         Page 7

The following data is required for this program to analyze and design a

spread footing:

Concrete Strength                   (psi)

Reinforcement Strength              (ksi)

Column Live Load                    (kips)

Column Dead Load                    (kips)

Soil Bearing Capacity               (psf)

Depth of Footing                    (feet)

Soil Unit Weight                    (pcf)

Column Type                         (Concrete, Masonry, Steel)

Base Plate Size                     (inches, if required)

Column Size                         (inches)

FUTURE CHANGES

If this software is supported by registrations, the following are changes

to be implemented in future versions:

1.  A graphical input system with pull-down menus which show dimensions on a

plan and section graphic and allows changes to any data item after initial

input for easy "What-if" analysis.

2.  The ability to analyze and design pedestals.

3.  The analysis and design of rectangular footings

4.  The capacity to analyze horizontal loads.

5.  The capability to analyze off-center columns which can occur with corner or

edge footings.

6.  Detailed error checking of input data.

7.  The ability to save and retrieve data to and from a file.

8.  The ability to analyze footings with both axial loads and moments.

PROBLEM/SUGGESTION REPORT FORM                 Page 8

If you encounter a problem with SP-FOOTING, I would like to know about it.

Please take the time to fill out this report form.  If a new version is created

because of the problem you encountered, you will recieve a free upgrade with the

problem corrected.  If you decide not to register this program, I would still

like to hear your suggestions for improving this program.

Your Name:__________________________________________________________________

Street Address:_____________________________________________________________

City, State, Zip:___________________________________________________________

Phone Number and Best Time to Call:_________________________________________

What Version of SP-FOOTING Do You Have?_____________________________________

Where Did You Get It?_______________________________________________________

What Brand and Model Computer are you using?________________________________

What type monitor/video card are you using?_________________________________

Describe the problem and what you were doing when it happened.______________

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

ANY OTHER SUGGESTIONS FOR IMPROVING THE PROGRAM?____________________________

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

MAIL TO:  Terry W. Towle

112 Brentridge Drive

Antioch, TN  37013