Tips for running CAD software in Desqview.

Recently I learned that even CAD programs could be run in Desqview

concurrently with various other programs, and even multiple concurrent

CAD programs.  I use DesignCAD 2D and 3D, but I an inclined to imagine

that other CAD programs can be similarly run in DV.

It may be useful to logon to other computers, perhaps to do file

transfers, and to do so at any time without having to stop the CAD

process.  One can, of course, be printing or plotting one drawing in

backgound while working on another in the foreground, and can even run

multiple copies of DC2 and/or DC3 concurrently.

When first trying this, I could not get it to work.  While some of the

necessary options were fairly obvious, others were rather elusive.

This memo is an attempt to summarize some of what seems to help.

Some readers of this may be more familiar with Desqview than others,

and may view this as too detailed while others may regard it as

insufficiently detailed.  My apologies to both.

For openers, I assume a 386 or 386SX with at least 2 meg of ram,

preferably much more.  I also assume Desqview386 which includes QEMM,

which converts extended memory on 386 systems into expanded memory.

Of course when running additional programs, each one gets less of the

cpu, as indicated by the following:

with QEMM and DV, Pantheon.dc3 load, z 0, esc, shade

DC3 in the only DV window 8 mins

DC3 in background with idle program in foreground: 11 mins

DC3 in background while editing this file in foreground: 12 mins

DC3 in background with comm pgm downloading files in foreground: 18 mins

So there's no free lunch, but sometimes it's nice to grab a snack

while continuing to do something else.  BTW, all of the above timings

were while using VGA at 640*480*16.  The current version of DV does

not support SVGA in background, either higher resolution, or more

color at 640*480 resolution.  For now, SVGA in DV would have to stay

in foreground.  However, SVGA in foreground may interfere with

concurrent VGA graphics in background.

Another BTW:  when there are large differences in the amount of

conventional memory used by the various programs that you plan to run

concurrently, it may be best to start a program with the largest

conventional memory requirement first.  On the other hand, programs

that you intend to run for the longest periods of time should be

started first.  Both of these suggestions are to minimize fragmenting

memory and do not matter much if you have more ram than you are likely

to use, although a disk cache can always use any ram you might not

utilize for other purposes.

On to the nitty gritty.  Comments will be placed in quotes in the

following, (those with an * are particularly pertinent), and

alterations would be appropriate for various hardware/software

combinations.  My versions of:

DOS: 3.3, QEMM: 5.0, DV: 2.26, DC2: 4.2, and DC3: 3.1.

config.sys

stacks=0,0 "DOS stacks are not needed in DV and waste memory"

device=qemm.sys ma=6 ha=16 ram ta=40 extmem=256 I=0C00-0FFF I=F000-F7FF

"* A crucial option is the ta=40.  In QEMM 5.0 a number larger than 40

seems to be ignored, reverting to some small number, and a small

number will not do the trick.  The other options may vary depending on

your configuration and applications."

device=loadhi.sys c:\dmdrvr.bin /2  "load some drivers in high memory"

device=loadhi.sys c:\pg\hyperdkx.exe xs oj c:768 s:+ "A disk cache"

device=loadhi.sys vdisk.sys size=256 sector=512 entries=80 /e:4

device=loadhi.sys g:\ati\mouse.sys

buffers=1  "Not to worry, more of these will be setup in autoexec.bat"

files=5

autoexec.bat

LOADHI /B BUFFERS 5 "Only a few of these are needed with the disk cache"

LOADHI /B FILES 40

C:\A\XCOPY C:\A\*.* K: > NUL "load stuff onto ramdisk"

CD \DV

BREAK OFF

XDV

"Following are various panels of the DV setup program"

Advanced Setup: Performance

Task Processing Time (in Clock Ticks)

Foreground:    3  "Small is beautiful, especially for modems"

Background:    3  "Each background window gets this many"

Memory Usage (in K)

Common Memory:      13  "More than minumum seems to be a waste"

DOS Buffer for EMS:  0  "Set to zero unless on a LAN"

Optimize communications?    (Y/N): Y  "Unless no modem"

Allow swapping of programs? (Y/N): Y

Manage printer contention?  (Y/N): N

Advanced Setup: Video Monitor

Do you want text & graphics displayed

at the same time? (Y/N):  Y  "* A must for background graphics"

Advanced Setup: Mouse

Mouse Type: 0  "Personal preference.  I tell DV that I don't have

a mouse. DC will find it anyway.  The mouse driver

may be loaded in the DC window, or it may be loaded

outside of DV for use with multiple concurrent

programs.  Some mouse drivers may be 'loaded high',

otherwise if they are loaded outside of DV, they

reduce the maximum window memory size."

"Following are the particular panels in which DV is told about DC"

Change a Program

Program Name............: DesignCAD

Keys to Use on Open Menu: DC               Memory Size (in K): 580

"This memory size does not include EMS and may differ on your system.

If the mouse driver is loaded outside of DV, but not loaded high,

this will be smaller."

�������������������������������������������������������������

Program...:                  "Here is where to load the mouse driver

* unless using the mouse with several concurrently running programs.

Otherwise, you might want to load your CAD program here."

Parameters: "Options for whatever program you load on the previous line"

Directory.: g:\dc

��������������������������������������������������������������

Options:

Writes text directly to screen.......: [Y] "* These 3

Displays graphics information........: [Y]  must be

Virtualize text/graphics (Y,N,T).....: [Y]  set to Y"

Change a Program Advanced Options

System Memory (in K)....:   0 Maximum Program Memory Size (in K)..:

Script Buffer Size....:   640 Maximum Expanded Memory Size (in K):  512

"If you leave 'maximum expanded memory' blank, DC will grab it all.

However, you may want more than 512K for larger drawings."

Text Pages: 1  Graphics Pages: 1   Initial Mode:  Interrupts: 00 to FF

���������������������������������������������������������������

Window Position:

Maximum Height:  30       Starting Height:  30  Starting Row...:   0

"* The Height(s) of 30 seem required for multiple concurrent DC"

Maximum Width.:  80       Starting Width.:  80  Starting Column:   0

���������������������������������������������������������������

Close on exit (Y,N,blank).....: [N] Uses its own colors............ [Y]

Allow Close Window command....: [Y] Runs in background (Y,N,blank). [Y]

Uses math coprocessor.........: [Y] Keyboard conflict (0-F)........ [0]

Share CPU when foreground.....: [Y] Share EGA when foreground/zoomed[Y]

Can be swapped out (Y,N,blank): [N] Protection level (0-3)......... [0]

"For those who may be new to DV (including QEMM) let me mention that

many hardware configurations have their idiosyncracies, and many

programs do also.  DV tries to squeeze the most out of your hardware,

and tries to run multiple programs, many of which may misbehave even

when nothing else is running on the PC at the same time.  Sometimes,

things go bump in the ram.  For example, if some other program has its

DV window defined as 'Writes directly to screen:  Y', and 'Virtualize

text/graphics:  N', then apparently while that program is in the

foreground, background 640*480*16 graphics programs will simply stop

and wait until that program is not in foreground.  You may need to

tinker to get things running smoothly, but the payoff can be worth the

effort.

If all else fails, look for help in the DV conference of your favorite

BBS network."

Jerry Brennan