These files were originally created to let me execute Bitfax

under DV-386, but should come in handy for any program that

terminates without cleaning up its interrupt vectors behind it.

Bitfax kept crashing my system, sometimes to the point of wiping

the CMOS. Through trial and error I narrowed down the problem to

something in the Bitsched TSR, which is required for Bitfax to run.

From the thoughness of each crash, I figured the vector tables

were screwed up. I took a look with Manifest, and found both

programs grabbing a whole bunch of vectors, some of which I couldn't

even find documentation for.

The programs labeled INT??.SHP were the results of my efforts to

"shotgun" the problem by writing these little reentrant blocks of

assembler code that do little more than grab and save each vector

before Bitfax gets to them, and then restoring those values when

Bitfax and Bitsched terminate. Each module is stand-alone, handling

just the interrupt indicated in its name.

Evidently this approach worked, since I can now run

Bitfax/Bitsched under DV-386 with no problems. I still don't know

which vector(s) caused the problem, but since finding out would

require some messy trial-and-error with me rewriting my CMOS on each

error I'm not very inclined to find out.

The source code was written in MASM 5.0 and assembled and linked

to execute under CODEVIEW. If you want to create other vector

grabbers, simply take one of the source files and change the

interrupt code, then reassemble.

NEXT PROJECT: Bitfax is still giving me a few problems; such as

not letting the hardware operate at 9600bps. Could DV-386 be slicing

away from Bitfax during negotiations? The obvious answer would be to

set up a grabber that handles INT-0B/0C and sets up a chain to it

thru its own code. On interrupt, it would poke DV with a

BEGIN_CRITICAL message, then chain to the original interrupt. On the

interrupt's return the program would send DV an END_CRITICAL then

return itself.

The problem with this is there aren't very many comm programs out

there that don't bypass the serial port interrupts and talk directly

to the hardware. So here I sit, scratching my head.... Comments?

-------->Mike.