Supplemental Instructions for using DESQview with Micro

Display Systems GENIUS

The following information applies to DESQview version 2.24

used with a Micro Display Systems GENIUS, GENIUS plus or GENIUS

squared display adapter and monitor.

FEATURES

DESQview when used with GENIUS displays, will give you use of

the full screen display for your program windows.  On the

GENIUS or GENIUS plus, DESQview can arrange the screen to view

25, 41, 66, or 82 lines by 80 columns.  On the GENIUS�,

DESQview can arrange the screen rows and columns to: 25 x 80 or

66 x 80 or 66 x 127.  DESQview window sizes can be set to use

the entire screen or view several full normal applications

screens (25 lines x 80 rows) simultaniously.

DESQview also supports applications which use GENIUS

graphics modes.

DESQview can be used with the GENIUS VHR or G2 utilities which

allows programs to set the number of screen lines, background

color or split the screen. (Such as LOTUS 1-2-3 dual monitor,

Microsoft Word.)

INSTALLATION  -  GENIUS or GENIUS plus (Model 420)

So DESQview can take full advantage of your GENIUS or GENIUS plus

display, note the following changes to the normal DESQview

installation as described in the DESQview manual Chapter 1:

Install the VHR_ANSI.SYS file (found on the GENIUS UTILITIES

diskette) in the CONFIG.SYS file used to boot DOS on your

PC.  (This will only be needed if you wish to use the VHR

program in windows which "Writes directly to screen".)

If you are using QEMM version 4.2 or higher, be sure to exclude

the video graphics area 0A000 - 0C7FF.  The following is an

example of a CONFIG.SYS:

DEVICE=C:\GENIUS\VHR_ANSI.SYS

DEVICE=C:\QEMM.SYS X=A000-C7FF

FILES=20

BUFFERS=15

When the DESQview Setup screen appears, select "Enter" for

ADVANCED setup procedure. Then select "V" for Video Monitor.

In the first field type 5 for other.  In the Current

Graphics driver field type "GRFGNS.DVR".  Press Enter to

continue with DESQview's installation.

INSTALLATION  -  GENIUS� (Model 1000)

So DESQview can take full advantage of your GENIUS squared

display, note the following changes to the normal DESQview

installation as described in the DESQview manual Chapter 1:

Install the G2_ANSI.SYS file (found on the GENIUS UTILITIES

diskette) in the CONFIG.SYS file used to boot DOS on your

PC.  Make sure to run the G1LOAD command with G1_MDS.OUT.

(Recommended in your AUTOEXEC.BAT file.)

If you are using QEMM version 4.2 or higher, be sure to exclude

the video graphics area 0A000 - 0B7FF.  The following is an

example of a CONFIG.SYS:

DEVICE=C:\GENIUS2\G2_ANSI.SYS

DEVICE=C:\QEMM.SYS X=A000-B7FF

FILES=20

BUFFERS=15

When the DESQview Setup screen appears, select "Enter" for

ADVANCED setup procedure. Then select "V" for Video Monitor.

In the first field type 5 for other.  In the Current

Graphics driver field type "GRFGNS2.DVR".  Press Enter to

continue with DESQview's installation.

REARRANGE VIDEO OPTIONS MENU

To have DESQview change the number of lines displayed

on the screen, from the DESQview menu press "R" for the Rearrange

menu, then press "V" for Video options menu.

The GENIUS or GENIUS plus options:

1 thru 4 to select the number of display lines you want

DESQview to display, 5 - 7 to select the foreground /

background text intensity of the display.

The GENIUS� Model 1000 options:

1 thru 3 to select the number of display rows and columns

you want DESQview to display, 4 and 5 to select the

foreground / background text intensity.

CHANGING A PROGRAM'S INFORMATION

It is useful to customize your applications with DESQview for the

GENIUS displays.  For programs that run in "small" windows (do NOT

write directly to the screen or text applications and QEMM) note the

following on the advanced options screen of Change a Program:

"Writes directly to screen" option must be set to "N".

"Virtualize text/graphics" option must be set to "T" or "N".

MAXIMUM HEIGHT, MAXIMUM WIDTH, STARTING HEIGHT, and STARTING

ROW can use numbers from 1 to 82.  (Normally this would be 1 to

25.)  For Lotus 1-2-3 version 2 and DESQview companions DOS

Services or DESQview Notepad, use this feature to look at 82

lines at one time!

"Text Pages" must be set to 1 when the Maximum Height is not

25.

For applications that use that write directly to the screen using

the MDS graphics mode (e.g. Ventura Publisher, PageMaker, AutoCAD)

note the following for Change a Program:

"Writes directly to screen" option must be set to "Y".

"Virtualize text/graphics" option must be set to "T" or "N".

"Text Pages" should be set to 1.

"Graphics Pages" must be set to 1.

Note DESQview requires 128k or 256k of memory to save and

restore a graphics application screen image.  DESQview will use

Expanded memory if available, or Vdisk or hard disk to save the

screen image.  When the ALT key is pressed from a graphics

application, DESQview will "curtain" the screen then present

the DESQview menu.  The graphics screen will be restored when

you switch back to the application.

For applications that use that write directly to the screen using

the DUAL mode (e.g. LOTUS 1-2-3) note the following for Change a

Program:

"Writes directly to screen" option must be set to "Y".

"Displays Graphics information" should be set to "N".

"Virtualize text/graphics" option must be set to "N".

"Text Pages" must be set to 5 and Maximum Height to 25.

When the ALT key is pressed, the graphics portion will

disappear, but be restored when you switch back to the

application.

The VHR utility should be run from a batch file as in

the following example:

VHR DUAL/N

123 DUAL.SET

DESQview is a trademark of Quarterdeck Office Systems.  Genius

is a trademark of Micro Display Systems, Inc.  AutoCAD is a trademark

of Autodesk, Inc.  Lotus 1-2-3 is a trademark of Lotus Devlopment

Corporation.  Microsoft Word is a trademark of Microsoft Corporation.

PageMaker is a trademark of Aldus Corporation.  Ventura Publisher

is a trademark of Ventura Software, Inc.