CHKVESA  (Check VESA Extended Video BIOS)

Command

----------------------------------------------------------------------

Version 1.1, Copyright 1991 Gerald A. Monroe

----------------------------------------------------------------------

Purpose:    A diagnostic utility which checks for the presence of a

VESA BIOS Extension (VBE), and reports various statistics

about supported video modes.

Format:     CHKVESA

Remarks:    CHKVESA requires the presence of a VESA BIOS Extension.

VBEs may either be permanently fixed in the video ROM, or

loaded in the form of a memory-resident software driver.

"VESA Version" indicates the level of the official VESA specification

to which the active VBE complies.

"BIOS identification string" may identify the adapter manufacturer,

the adapter model, and/or the revision number of the VBE driver.

"Mode" is the hexadecimal BIOS mode number.  Modes numbered less than

100h can be set using the standard IBM mode set function, INT 10h/

AH=0/AL=Mode.  Modes numbered 100h and higher can be set only by using

the VBE's mode set function, INT 10h/AX=4F02h/BX=Mode.

"Type" indicates whether the mode supports color or monochrome only.

"Text" here indicates a character-based mode.  "APA" (for "all points

addressable") indicates a graphic mode.

"Resolution" reports the number of addressable horizontal-by-vertical

pixels on the display for graphic modes, or the number of character

cells (rows-by-columns) for text modes.  Some VBE drivers incorrectly

report text-mode resolutions in pixel units rather than in characters.

"Colors" indicates the number of colors that may be displayed on the

screen at one time in that mode.  Monochrome modes return "n/a" here.

"Bits/Planes" reports, for graphic modes, the total number of bits per

pixel, and the number of bit planes.  Text modes report "n/a" here.

"Memory Model" indicates the type of video memory organization scheme

employed by the mode.  "Four-plane" here means that the memory layout

closely resembles the one used by the IBM-standard mode 10h (640 x

350 x 16 colors), while "Packed-pixel" modes resemble IBM's mode 13h

(320 x 200 x 256 colors).  "Defined by OEM" means the memory layout

is unique to this particular graphics adapter.

"BTS" (for "BIOS text support") indicates whether the video BIOS sup-

ports the basic IBM-standard character output functions in this mode.

"CRT" indicates whether the mode can be displayed on the monitor that

is currently attached to the adapter, given the monitor's capabilities

and the amount of video memory installed on the adapter.  You may see

a few "No's" in this field if the adapter is not fully populated with

RAM chips or is attached to a medium-performance monitor.

A question mark ("?") in any field indicates that the VBE was unable

to provide the requested information for that particular mode.

YOUR LICENSE TO USE CHKVESA:

-----------------------------

CHKVESA.EXE and CHKVESA.DOC, Version 1.1, are Copyright 1991 by

Gerald A. Monroe.  All rights are reserved except those expressly

granted here.

These files may be used and distributed freely, but on non-commercial

terms only, and only if they are distributed together and neither is

modified in any way.

NOTICE TO DISTRIBUTORS AND PUBLISHERS: CHKVESA may not be distributed,

either alone or together with other software, on ANY medium for which

ANY fee or premium is charged, without the express written permission

of the copyright owner.  Any corporation or organization wishing to

distribute CHKVESA on such terms should apply in writing to:

Gerald A. Monroe

1241 Bunts Road

Lakewood, Ohio 44107

U.S.A.

CompuServe ID # 72321,1257