FONTHT.DOC

---------

Font Height Utility

version 1.0

(c) E. Meyer, 19 Mar 1991

Requires: EGA or VGA compatible video adapter; DOS 2.x or higher

USAGE:        C>FONTHT            gives instructions, version/date

=====         C>FONTHT nn         changes to font height nn (7-20)

C>FONTHT 0 or OFF   reverts to normal for your system

C>FONTHT ?          displays current font information

Many people are aware that EGA/VGA systems can provide both a "standard"

25 line text screen and a "compressed" mode (43 lines on EGA, 50 on VGA).  But

these video adapters are actually capable of a wide range of font sizes, of

which these are only two examples.  Here is a table of the possibilities

offered by FONTHT; the character height is given in "points" or scan lines,

and the two "standard" combinations are marked with asterisks:

FONT       TEXT LINES               FONT       TEXT LINES

HEIGHT    ON EGA  ON VGA            HEIGHT    ON EGA  ON VGA

7        50      57                14        25 *    28

8        43 *    50 *              15        23      26

9        38      44                16        21      25 *

10        35      40                17        20      23

11        31      36                18        19      22

12        29      33                19        18      21

13        26      30                20        17      20

It's often useful to see as much text as possible on screen at once; the

7-point font can display an entire page full.  On the other hand, larger

characters, such as the 20-point font, are very easy on the eyes.  You can

determine which size is the best compromise for your computing tasks.

Of course, the usefulness of FONTHT depends on the ability of your

favorite software to adapt to nonstandard screen sizes.  Some older MSDOS

programs simply assume the screen has its standard 25 lines; they won't work

well with fewer lines, and will ignore any extra.  Programs that (re)set the

video mode will wipe out any font selected with FONTHT.  You will simply have

to experiment to determine compatibility with other software.

Even DOS itself will show a few quirks; for example, the TYPE and CLS

commands will not use any screen lines beyond 25.  If you have an ANSI.SYS

driver installed, you can fix the CLS problem simply by adding "$e[J" to your

PROMPT.

When FONTHT changes fonts, the overlapping portion of the screen is

preserved, but some text will have disappeared or been added at the bottom,

which may make the display slightly confusing at first.

FONTHT and its documentation are (c)1991 E.Meyer, all rights reserved.

They may be freely distributed, but not modified or sold for profit without my

written consent.  The user takes full responsibility for any damages resulting

from the use of this program.  Please report any problems encountered to the

author.

Eric Meyer

3541 Smuggler Way              CompuServe [74415,1305]

Boulder, CO  80303  USA