����������������������������������ķ

� HIBACK.EXE v1.1 (c) 1991 D.Huras �

����������������������������������Ľ

BACKGROUND

����������

The VGA adaptor supports either High Intensity Backgrounds

or Blinking Foregrounds.  When blinking foregrounds are

enabled, you are limited to 8 background colours.  The

background colour choices can be increased to 16 (8 normal +

8 high intensity) if you are willing to forego blinking

foregrounds.

USAGE

�����

This program interrogates the bit which controls high

intensity backgrounds/blinking foregrounds and will report

its current state and/or set the bit depending on the

arguments passed to it.

Arguments (only one argument can be passed).

���������

-none-   � enable High Intensity Backgrounds

/B       � enable Blinking Foregrounds

/T       � toggle the status of the bit

/R       � report the status of the bit; do NOT set the bit

Errorlevels returned

��������������������

If valid arguments are passed, the errorlevel returned will

always be the current state of the bit:

0 - High Intensity Backgrounds are enabled

1 - Blinking Foregrounds are enabled

99 - Invalid arguments

Notes

�����

� The current status of the bit is always checked before

any interrupts are called; no interrupts will be called

if the bit is already in the requested state.  A message

will be displayed to this effect.

� The /T (toggle) option, checks the current state of the

bit and toggles it (ie. if high intensity backgrounds are

enabled they will be disabled and blinking foregrounds

will be enabled.

� This program can be compressed with an .EXE compressor

which supports .COM files.  A 25% space saving should

be realized.

Considerations

��������������

The normal state of this bit is Blinking Foregrounds.  After

running the program to enable High Intensity Backgrounds you

will not notice anything until you run a program which

displays text with flashing foregrounds.  This text will now

appear with high intensity backgrounds (unless the program

forces blinking foregrounds).

If you use a program which allows you to customize the

colours it displays including blinking foregrounds; the

blinking foregrounds will change to high intensity

backgrounds and you will see some interesting new colour

combinations.

Some programs toggle the BIT and do not restore it to its

previous state. You may notice after running certain

programs that foreground blinking has been disabled.  You

can run HIBACK with the /R option to report the status of

the bit before and after the suspected program to determine

if it is changing the bit and not restoring it.  Run HIBACK

with the /B argument to restore blinking foregrounds.

Some programs enable High Intensity Backgrounds, but if you

shell out to DOS and run another program which forces

foreground blinking and doesn't restore the bit, you may end

up with a blinking screen when you EXIT the shell.  Once

again, run HIBACK to restore High Intensity Backgrounds

before you return to the program you shelled out of.

HISTORY

�������

Ver.   Release Date  Changes

���    ������������  ���������������������������������������

1.0      May  1991   Initial Release

1.1      Jul. 1991   � Check status of bit before calling int.

� /R (report) option added

� /T (toggle) option added

� Errorlevels 1,0,99 added

� changed from .EXE to .COM file

DONATIONS WELCOME!

������������������

If you find the program useful, a SMALL donation would be

appreciated($5 suggested). Thank-you.

David Huras CIS: 72767,472

2745 Islington Ave. Apt. 701

Rexdale, Ont  M9V 5C1

CANADA

JULY 1991

�������������������������������������������������������������