B's Wave Centre   (BWC)

Version 1.06

Copyright 1990,1991

B & J Corporation

Kim Heino

Arttu Kontkanen

All rights reserved.

===============================================================================

Contents of this manual:

1       General information

2       Commands

2.1     Command line parameters

2.2     Commands in editor

2.2.1   Infolines

2.2.2   Menus, keys

2.3     Hotkeys for commands

3       Setup

4       Technical information

5       Last words, version information

5.1     Last words

5.2     Version information

===============================================================================

1       General information

BWC is the best sample editor program available for IBM PC and

compatibles. Only requirements are EGA or EGA-Mono display

adapter and 256kB of memory. BWC supports JDAC (see JDAC.DOC),

Sound Blaster card and the PC internal speaker.

The best result is obtained by a JDAC connected to your stereo

set. You can easily make the JDAC with the instructions included

in this package. It is a simple 8 bit D/A-converter to be

connected to your parallel port. Also a product named Covox

Speech Thing seems to be compatible with the JDAC. It is

possible to use it with BWC.

Sound Blaster's I/O address and IRQ number can be configured

with CFGBWC. The default settings are IRQ 7 and I/O address

0220h (SB 1.5). The internal speaker routine plays the samples

only with 6 bits. It is IBM's fault! Another internal speaker

playing routines, like RealSound, plays the samples just like we

do. But we convert the 8 bit data to 6 bit in real-time to get

the best result. Note that you can easily hang your computer by

playing samples with too high digirates.

BWC supports MSMOUSE-compatible mouse. Remember to load your

driver before starting BWC.

BWC identifies automatically following sample formats: RAW, IFF,

RealSound (Steve), Sound Blaster's (Voxkit) format, Akai

Professional Sampling Keyboard (see part 4 of this document) and

of course our own format.

Don't panic if you find some undocumented features in BWC. Just

report them to us and they will be mentioned in the future

versions.

2       Commands

2.1     Command line parameters

Usage of BWC is following:

BWC [filename] {filename2} {/I | /1 | /2 | /3 | /4 | /S} {/M}

{/P{hz}} {/Vvol}

Filenames are the name(s) of the sample(s) you want to load in

the startup. You must specify one, the other is optional. You

can specify full pathname, or just the name. See Defaul Dir

setting in Setup for more information.

"/I", "/1".."/4" and "/S" are the output device used. This

overrides the Setup setting.

/I = Internal Speaker

/1 = JDAC (or other D/A-convertor) in LPT1

/2 = JDAC (or other D/A-convertor) in LPT2

/3 = JDAC (or other D/A-convertor) in LPT3

/4 = JDAC (or other D/A-convertor) in LPT4

/S = Sound Blaster

"/M" must be given if you are working on EGA-Mono display. The

default setting is EGA-Color. BWC does not automatically detect

your screen type.

With "/V" you can give start-up volume for sample-player or

editor. Default is 100%.

When "/P" is given BWC just plays the sample and exit. So BWC

can also be used as a sample-player. You can specify the playing

rate by writing it right after "/P". No spaces or other

characters. Just like "/P22000". This parameter replaces the

value detected in load.

2.2     Commands in editor

After the sample(s) are loaded the main screen will appear. The

upper area is the first sample in graphical type. There is also

the sample's infoline. Below it are the same things for the

other sample. Below them there is general information about

memory, editing, buffers, markings, volume and zoom. And in the

bottom of the screen is the menu of available commands.

2.2.1   Infolines

The sample's infoline is:

??? Size: ??????B Lenght:???????s Digirate:?????Hz Type:????

And the general info is:

Editing:  ?????      Marked: ??????B, ???????s    Volume: ???

Free mem: ??????B    Buffer: ??????B, ???????s    Zoom:   ?????

In sample's infoline the first word tells you the name of the

sample. DOS's limits are used, 12 character name and 3 character

extension. Size lets you know about the size of the sample as

bytes. Lenght does the same, though informing it as seconds. You

can see the digirate of the sample after word Digirate. It is

showed as hertzs. Type shows the type of the sample. It is

processed as bits. Here is the list of available types:

0000 = RAW sample, loops off.

0001 = RAW sample, loops on.

0080 = IFF sample, loops off.

0081 = IFF sample, loops on.

In general info editing tells the sample you are editing. Is it

"Upper", "Lower" or "Both". Marked shows the size of the area

marked with the mark lines in bytes and seconds. If the mark

lines are off, the size is zero. The volume shows the percental

volume rate of playing. It is handled real-time, so you can test

which would be the best volume. Free mem informs about the

available memory. You can't process with bigger sample than you

have free memory. Buffer tells you the size of the copy buffer

in bytes and seconds. If you have Both-function On, the buffer

functions are directed to the both samples, upper and lower.

Therefore the number of free memory for buffer must be bigger

than marked area. Zoom lets you know the ratio of the zoomed

area. If it is "2", the ratio is 1:2, one byte in screen means

two bytes in memory.

2.2.2   Menus, keys

To move between the upper or lower samples press the spacebar

key. Also if you are in Both mode, press the spacebar to set it

off.

You can choose the desired function in the command line by

pressing the button which is lighted in the command word. By

using ESC the program returns to the main menu. You can also use

F1-F9 to jump directly between the menus.

If you try to do some uncomfortable command suggestions to the

program or there are not enough memory available, BWC will

answer to you by saying BEEP.

File menu:

Save:   Save saves the sample to the disk and asks the new name

of the sample. The default is the original name. If you

don't change the name the original sample will be

replaced. The sample will be saved to our own format.

There are not any possibilities to save the sample in

some other format in BWC 1.06. When you have saved the

sample, the new name of the sample will replace the

original sample name in the sample's infoline.

Load:   Loads sample from disk to the memory. A little window

appears to screen. Write the desired sample path and

name to load the sample. You can also specify the sub

directory for the samples in Setup, and when loading

something, you don't have to give it again, you just

choose the sample. The sample always will be loaded to

lower sample. Copy buffer will also be cleared.

Exit:   By the function Exit you quit the program. There is not

any kind of Dos Shell in BWC. Also there isn't any "Are

you sure" -prompts.

Play menu:

All:    Plays the whole sample. You can stop playing at the any

time just pressing any key. Note that you can't play

under multitasking.

Marked: Plays the marked part of sample. If the mark lines are

off, the command is disabled.

Loop:   Starts playing the sample from begin. But repeats the

part marked as a loop until key is pressed. Loops must

be on for this command.

Mark:   This is the only command in main menu which has not its

own sub menu. This command turns mark lines on and off.

You can choose particularly area of the sample just by

moving the mark lines with the cursor keys. One push in

cursor key will move the mark line one byte. Cursor key

and Ctrl together moves it 15 bytes in the same time.

There is two mark lines, begin and end. To change the

selected mark line press the Tab-key. The area between

the lines is marked. Use the Mark function to specify

the area to copy, delete, play etc. You can also move

the mark lines with mouse. If you for example have

80000B sample in the screen, which showes only 640B in

one sight, the mark lines and all the other functions

works only 80000/640 resolution (the zoom ratio). This

means that the mark line jumps always 125B when the mark

line is been moved only one byte in the screen. Of

course the situation will be different, if the sample is

short or zoomed in.

Buffer menu:

Cut:    Saves the marked part of sample to copy buffer, and

deletes it from sample. The previous content of the copy

buffer will be deleted. If Both is on, this and most of

the other commands works with both samples.

Copy:   Like Cut, but doesn't delete the marked part from

sample. Copy buffer will be replaced.

Paste:  Makes space in sample, and copies the content of the

copy buffer. The place must be specified by the left

mark line.

Over:   Just like paste, but copies the buffer replacing the

corresponding part of the sample.

Delete: Deletes marked area of the sample.

Clear:  Clears the contents of the copy buffer.

Zoom menu:

In:     By moving mark lines you can define the part of the

sample and zoom it to screen.

Out:    Zoom out is an opposite command to Zoom in. Zoom out

will automatically zoom to the previous Zoom in

situation. For example if you first zoom in to 20987B

and then to 14063B, and then use the function Zoom out,

the screen shows the previous zoom situation (in this

case 20987B). Only ten latest zoom positions will be

saved to the memory.

Half:   Zooms in. New zoom factory is half of the original. The

middle of the screen is the part zoomed in.

All:    Zoom all shows the whole sample in the screen. This is

the same situation as in the startup.

Screen: Marks the whole screen, but doesn't change the zoom

factory.

Envelope:

The envelope diagram shows the sample's level

distribution. There is four parts in an instrument

sample, A-D-S-R. Attack determines how quickly the

instrument starts to sound in its top level. Delay is

the time the sample slows down to its normal volume

level. Sustain is the time sample's so called normal

level lasts. Release time is the slowing down to zero-

time.

Envelope Both:

Envelope Both -command shows the envelope of the samples

in the same window. The envelopes will be in different

colors. The command zooms the envelopes in by the larger

sample. So the other sample might leave a bit short

propotioned to the other, depending of course the ratio

of the lenghts of the samples. You can't edit the

envelopes but you can edit the samples and the

corresponding changes you can see in the envelope. It is

really useful to know sample's ADSR when mixing them. If

the envelope seems to be little weird (up and down

lines), try Envelope Fix setting to fix it.

Mix menu:

Back:   Backwards turns around the whole sample or the marked

part of it. After turning, the sample will be redrawn to

screen.

Zero:   Zero function zeroes desired part of the sample data.

You can specify the part of the sample to zero with mark

lines.

Echo:   Version 1.06 of BWC contains only two kind of echoes.

First you must specify the delay of the echo in bytes.

If delay is smaller than length of the sample, the echo

type one will be used. It mixes the original sample and

the echo together. You can also specify the volume of

the echo. If delay is bigger than length of the sample,

the echo type two is done. You can specify the number of

echoes, volume of first echo, and sub of volume in next

echoes.

Two2one:

Stereo to mono mixs the left and right channel of the

samples to one mono sample. You can also mix two

different samples together. The sample you get can be

processed like one mono sample. But before you mix two

samples together, be sure that the samples have been

digitized in the same digitizing rate. If not, resample

them to the same digirate.

One2two:

This command doubles the sample which is in the upper

window. It makes a complete copy of upper sample to the

lower sample. This is an easy and fast way to make a

backup copy of sample you are editing. You can save the

editing situation, when there is very critical moment

going on. And then use the Swap up/lo to get the "saved"

sample back to editing. The copy buffer will be deleted

when you use this command.

Swap:   This command swaps the samples. This is useful when you

are editing a sample and you want to digitize some other

piece of sound while you have one sample already in the

screen and it is in the lower window. And because

digitized sample always replaces the lower window, the

original sample can be moved to safe in the upper

window. This command clears the copy buffer.

Add space:

Add space adds space. :-) By using this command you can

get a desired quantity of work space to your editing.

The size of the sample will changed. You can specify the

size of extra space as bytes. The space will be

displayed to the place you have specifyed with the mark

lines.

Resample:

If you have digitized a sample for example rate 25 kHz

and you think it is a bit too much for your needs, and

the sample takes too much disk space, you can soft

resample the sample and make it smaller, or bigger. The

volume can also be changed. By raising this volume you

can raise the level (amplitude) of the sample. Original

sample volume is 100 (percent). With resample you can

also change the format of the sample, IFF or RAW. If you

have mark lines on you can only change the volume of the

marked area.

Loop menu:

Loop on/off:

Turn loops on and off. The loop is shown in the samples

graphical view with three lines.

Mark begin:

Marks loop's begin. You can specify it with the left mark

line.

Mark end:

Marks end of the loop, which can be specified with the

right mark line.

Seek forward:

This function finds the suitable place for loop end.

First you must specify both start and end lines. Then

using this function the end line will be moved forward

to the suitable place. Function checks out the sample

diagram's angle in the start point of loop and seeks for

equivalent place in the end of it. The best result is

given by choosing the angle of the start points near by

�45 degrees.

Seek backward:

Same as above but the end line will be moved backwards

so looped area will be shorter.

Digitize menu:

Record: The only function of Digitize is record. BWC's first

versions supports only the Sound Blaster's digitizer. To

record: Connect the sound source to your Sound Blaster.

Press the record on. The program asks the digitizing

rate. Put the source on and when desired part of sound

is playing, press Y. Your available memory will be used

as a buffer for your digitizing. You can stop digitizing

by pressing any key. After digitizing you can use the

sample as any other samples. The digitized sample will

replace the lower sample window. This also clears the

copy buffer.

Setup menu:

Volume: Raising the number of the volume you are able to hear

the sample playing louder. The volume raises the level

of the playing, not the sample level. Volume is working

as real-time volume and is showed as percents. The

original sample volume corresponds rate 100 %, and you

can raise or decrease it between 1-250 %.

Max volume:

BWC will examine the sample data (or both) for maximum

peak level and then adjusts it's volume to maximum. This

is a good thing to do always after you have digitized

something. Remember to resample the volume to the sound

data too!

Type:   If the sample doesn't look or hear like it should be,

there might be some differences between the sample

format and the setupped format. BWC identifies

automatically most of the sample formats, but if it

doesn't, you must change it. Only IFF and RAW is

supported by this command.

Hertz:  You can specify the digirate of the sample, if BWC does

not identify it by itself. It is given in hertz.

Envelope fix:

Envelope diagram might get a bit strange with some

samples. You can fix it up by making some changes to the

numeric rate (0-9) of the envelope fix. This is the

fixing resolution of the envelope diagram.

Both:   Both allows you to edit two samples in the same time

just operating the another sample. So you don't have to

make same changes twice when you are working with

stereosamples or you want to edit two mono samples in

the same time. You can't use the copy buffer first in

the Both mode and then paste it in the mono mode. For

example if you copy 602B area to the copy buffer from

both samples, the copy buffer size will look like there

is 1204B, like there is, in a way. Then you turn to mono

mode and try to Paste the content of the copy buffer to

the one sample. Well, it's not gonna work. Marked area

indicator shows copy buffer size as bytes of one sample.

You can turn both off by pressing the spacebar.

2.3     Hotkeys for commands

This is the complete list of hotkeys:

Command         Key             Command            Key

---------------------------------------------------------

Main menu       ESC, Enter      Play               0

File menu       F1              Play at  6000Hz    1

Play menu       F2              Play at  8000Hz    2

Mark on/off     F3              Play at  9000Hz    3

Buffer menu     F4              Play at 11000Hz    4

Zoom menu       F5              Play at 13000Hz    5

Mix menu        F6              Play at 15000Hz    6

Loop menu       F7              Play at 17000Hz    7

Digitize menu   F8              Play at 21000Hz    8

Setup menu      F9              Play at 25000Hz    9

Both on         F10             Cut                -

Fast move/scr   Ctrl-Cursor     Copy               *

Move/scroll     Cursor          Paste              +

Upper/lower     Space           Zoom in            .

Change markl.   TAB             Zoom out           ,

Load            Alt-L           Loop on/off        Alt-P

Save            Alt-S           Loop begin         Alt-3

Envelope        Alt-E           Loop end           Alt-4

Resample        Alt-R           Hertz              Alt-H

One2two         Alt-1           Output             Alt-O

Two2one         Alt-2           Type               Alt-T

Add space       Alt-A           Volume             Alt-V

Swap            Alt-W           Zero               Alt-Z

Back            Alt-B           Echo               Alt-C

Loop seek <-    Alt-5           Zoom half          Alt-F

Loop seek ->    Alt-6

3       Setup

To change colors, output device or default directory, there is a

file called CFGBWC.EXE. This is a config-program for BWC. First

it asks the filename to configure. It is not wise to config your

only copy of BWC. Always have the original backup. If you just

press the Enter-key to some question, it will not be changed.

First there is the output device, then the colors. You can check

the meaning of the color numbers from DOS's manual, or almost

any other manual. Last question before saving the values is

default dir. If you give filename without path or drive, this

will be added to the begin of the name. So you can easily have

BWC main files and samples in the different directories.

4       Technical information

BWC's samplefile format is following:

Offset  Meaning

0000    The header, always letters "BJH"

0003    (word) Digirate of the sample in hertz.

0005    (word) Type of the sample.

0007    The sampledata starts here. Or, if (type and 1)=1 then

0007    (word) Loop begin offset.

0009    (word) Loop end offset.

0011    Begin of real sample.

Akai Professional Sampling Keyboard -fileformat is a concat of

two files transmitted by S700 and X7000. First there is sample

parameters -file (319 bytes) and then sample dump -file. BWC can

read this file and convert the data to 8 bit resolution. This

procedure is quite slow.

Formats are:

- SAMPLE PARAMETERS (as recognised and transmitted by S700 and X7000)

BYTE    RANGE   TYPE  DESCRIPTION

0       0F0h    B     System exclusive code

1       47h     B     AKAI identifier

2       0-15    B     MIDI Channel no.

3       10      B     SPRM, Function code for sample parameters.

4       65      B     S700, X7000 identifier.

5       0-5     B     Sample number.

6       0       B     Reserved (must be 0).

Next are 32 sets of sample parameters, one for each program.

Each consists of 4 data elements:

7,8     0-99    DB    KEYL    Lowest key in keyrange for this sample

in first program.

9,10    0-99    DB    KEYH    Highest key in keyrange.

11,12   0-255   DB    TRNSP   7 LSBs are signed key transpose, +/-60.

MSB is set for constant pitch.

13,14   0-255   DB    ATPTCH  Audio trigger. MSB set to disable,

7 LSBs give MIDI note.

15-262    31 further blocks of four elements for 2nd to 32nd

programs. Each block coded as bytes 7-14 above.

263,264 X       DB            undefined.

265,266 1,2,4,9 DB    RPMODE  1=One shot, 2=Looping, 4=Alternating,

9=Drum triggered one shot.

267-270 0-32552 DW    SSTRT   Start point.

271-274 X       DW            undefined.

275-278 200-32752 DW  SEND    End point + 1, must be >= SSTRT + 200.

279-282 0-32752 DW    SLOOP   Loop point, must be smaller than SEND.

283-286 X       DW            undefined.

287,288 0-99    DB    LFSP    LFO speed, 0 = 2 Hz, 99 = 8 Hz

289,290 0-99    DB    LFDP    LFO depth, 99 = +/- semitone.

291,292 0-99    DB    LFDL    LFO delay, 99 = slowest buildup.

293,294 0-99    DB    FLTFRQ  Filter frequency, 99 means maximum.

295,296 0-99    DB    RLEASE  Release time, 99 means slowest.

297,298 0-99    DB    LDNSS   Loudness, 99=max. LSB is 0.375 dB.

299,300 0-255   DB    SFNTUN  Fine tuning, signed. LSB=1/16 semitone.

301,302 24-96   DB    RECNP   Nominal recording pitch, coding as for

MIDI KEY in semitones (69=A3 440Hz).

303,304 36-77   DB    RECBW   Recording bandwidth, defines original

sample rate. 69 means bandwidth of

10.56 KHz which corresponds to sample

rate of 26400 Hz. Other rates are

computed according to

((RECBW-69)/12)

Rate = 26400 * 2

If RECNP=RECBW and SFNTUN=0 and TRNSP=0

then samples made with 60 points per

cycle will come out at concert pitch

(A3 440Hz).

305,306 0,255   DB    RVRSE   0=Forward, 255=Reversed. Used only for

display.

307,308 0-99    DB    VELFIL  Velocity/filter interaction.

309,310 0-99    DB    VELLDN  Velocity/loudness interaction.

311,312 0-99    DB    VELRLS  Velocity/release interaction.

313,314 X       DB            undefined.

315,316 X       DB            undefined.

317     0-127   B     Checksum, XOR of bytes 7-316.

318     0F7h    B     End-Of-Exclusive.

- SAMPLE DUMP FORMAT (as recognised and transmitted by S700 and X7000)

BYTE   RANGE   TYPE  DESCRIPTION

0      0F0h    B     System-exclusive code

1      07Eh    B     Common non-realtime identifier

2      1       B     SD, Sample dump code

3      N       B     LS, Sample number. Incoming sample will replace

X7000's sample numbered N+1 where N is 0 to 5.

if N>5, then the sample being edited will be

replaced.

4      0       B     Reserved (must be 0).

5      8-16    B     Bits per sample word. X7000 transmits 12, but can

accept 8 to 14.

6-8    0-127   TB    Sampling period in nS. X7000 accepts 15259 to

500000. X7000 assumes the nominal pitch of

samples is C3 (MIDI 60) unless a subsequent

set of sample parameters specifies it otherwise.

9-11   0-127   TB    Total words in sample. X7000 accepts 200 to

32752.

12-14  0-127   TB    Loop start point. If it is >= Total word - 5,

assume non-looping.

15-17  0-127   TB    Loop end. X7000 takes this as sample end point.

It takes the start point as 0, unless subse-

quent set of sample parameters specify it

otherwise.

18     0-1     B     Mode, 0=looping, 1=alternating (One shot, if

loop length < 5).

Next follows a variable number of sample blocks. Each has the

same format:

0      0-127   B     Block number LSByte.

1,2    0-4095  SW    Sample data.

3-120  0-4095  SW    59 more sample data points.

121    0-127   B     Checksum, XOR of previous 120 bytes.

A final byte follows the last block:

??     0F7h    B     End-Of-Exclusive.

- Coding of types B, DB, TB, DW and SW:

- B  represents a byte transmitted as a single MIDI byte.

- DB represents a 8bit value transmitted as 2 MIDI bytes:

first  byte: 0 d6 d5 d4 d3 d2 d1 d0

second byte: 0  0  0  0  0  0  0 d7

- TB represents a 21bit value transmitted as 3 MIDI bytes:

first  byte: 0  d6  d5  d4  d3  d2  d1  d0

second byte: 0 d13 d12 d11 d10  d9  d8  d7

third  byte: 0 d20 d19 d18 d17 d16 d15 d14

- SW represents a 12bit value transmitted as 2 MIDI bytes:

first  byte: 0 d11 d10  d9  d8  d7  d6  d5

second byte: 0  d4  d3  d2  d1  d0   0   0

- DW represents a 16bit value transmitted as 4 MIDI bytes:

first  byte: 0  d6  d5  d4  d3  d2  d1  d0

second byte: 0   0   0   0   0   0   0  d7

third  byte: 0 d14 d13 d12 d11 d10  d9  d8

fourth byte: 0   0   0   0   0   0   0 d15

5       Last words, version information

5.1     Last words

This program has no copy protection. Only thing is your name

displayed after exiting. It can be changed, but remember, it is

not the only one. It is crypted all over the EXE file too.

If you have ideas, have found some bugs, or anything, contact

us. See REGISTER.DOC for more information.

Thanks to...

Patrick Aalto for ideas, testing and Akai-help.

Komatsu for ideas, testing and spreading.

To keep lawyers happy...

IBM PC, XT, AT are registered trademarks of International

Business Machines Corp.

RealSound is a registered trademark of Steve Vitzel.

Sound Blaster is a registered trademark of Creative Labs, Inc.

Akai Professional Sampling Keyboard X7000 is a registered

trademark of Akai Electric Co, Ltd (Japan).

Akai Professional Sampling Keyboard S700 is a registered

trademark of Akai Electric Co, Ltd (Japan).

DESQview is a registered trademark of Quarterdeck Office

Systems.

5.2     Version information

v1.00   This is the first released version. No bugs, we hope.

v1.01   Change volume parameter added to Resample command.

Both-mode and mark lines with Resample command.

Sound Blaster configurable via CFGBWC.

More free memory (64kB).

Documents fixed.

New version of JDAC (didn't work with all amplifiers).

v1.05   Support for Akai Professional Sampling Keyboard files.

Zoom Half command.

Max Volume command.

v1.06   Volume for command line.

Loop seek backward command.

Loop seek forward command.

-------------------------------------------------------------------------------

Brought to you by Philip Dorant

-------------------------------------------------------------------------------