�LM0��RM255�

CROPLINE.ZIP �� R.J.Holmgren 3/3/90

Print Crop Marks on LaserJet Pages

**WARNING** -- DO *NOT* PRINT THIS DOCUMENT TO A LASER PRINTER!!  (It's full

=======          ===                    of sample printer control strings)

CROPLINE.ZIP consists of three files:

CROPLINE.DOC� this file

CROP.MAC    � a macro downloaded ONCE to the laser printer (before printing)

MACROLOD.PM � a macro set-up PrograM that DL's CROP.MAC + other instructions

Two basic approaches are discussed:  print crop marks derived from characters

in a font, or print a raster graphics (font-independent) crop-mark image.

Both approaches follow the same general procedure (H-P compatible PCL commands

are assumed throughout):

1) push current cursor position

2) print crop marks in four (or two) positions on the page, determined by

absolute (X,Y) references to the PCL "logical page" coordinate grid

3) pop current cursor position

To print crop marks on every page, the instructions are placed in a Running

Header or Running Footer.

Non-specialists might think that its easiest to derive marks from a font.  In

fact, raster graphics are much more flexible, using a printer Macro; the only

real difficulty lies in making the raster graphics image; but since a sample

-- CROP.MAC -- is included with this ZIP, you're ready to go.  Do **NOT** load

CROP.MAC into XyWrite or any other editor which is not 100% Ascii!

If you don't like CROP.MAC, make your own image (I used ALTERFNT.COM [in

ALTRFT26.ZIP] to create CROP.MAC's image and load it into a font, and then

DEBUG.COM to write nnn bytes from the font into CROP.MAC; the whole procedure

took about 10 minutes).

Method #1 -- Use Raster Graphics

--------------------------------

For purposes of this demo, all files are assumed to reside in the current

directory (modify MACROLOD.PM as required).  Filename DLMACRO.TMP is

*reserved* for the use of MACROLOD.PM (any pre-existing file named DLMACRO.TMP

will be erased).

MACROLOD.PM is RUN first, and only *once*, prior to printing, to download

the crop macro (CROP.MAC) into the printer's permanent memory; neither

CROP.MAC nor MACROLOD.PM are used again (unless you turn off the printer).

Note that the purpose of MACROLOD.PM is to establish a Macro ID# (range

0-32767) prior to downloading the raster graphics image, to download the macro

itself, and finally to make it "permanent".  Modify MACROLOD to select the

macro ID number you want to use; the default is an ID of�201.

If you want to hard-code an ID number within CROP.MAC, and thereby

render MACROLOD.PM superfluous, write an ID command in a DUMMY1.FIL

(*no* terminating Carriage Return!):

#&f#Y     [replace "#" with ID number]

and write in DUMMY2.FIL an instruction to make macro CROP.MAC permanent,

so that it survives a printer reset (no terminating <cr>):

#&f#y10X  [replace "#" with ID number]

Then concatenate DUMMY1.FIL, CROP.MAC, and DUMMY2.FIL (from DOS):

copy/B dummy1.fil/A+crop.mac/A+dummy2.fil/A<cr>

and REName DUMMY1.FIL as CROP.MAC.  Now you only need to download

CROP.MAC to the printer *once*, before printing; do it from the XyWrite

CMline:

dos /c copy/B crop.mac LPT1:<cr>

CROP.MAC prints a simple cross on the page, at absolute positions determined

by Printer Insert (�AEPI�AF) instructions in a Running Header or Footer

(�AERH�AF��AERF�AF).  These positions are plotted on a 300dpi (dot per inch) "X"x"Y"

grid.  XyWrite formatting commands are *irrelevant* & have *no impact* on

these positions (which makes things easy for you).  A good "starter" �AEPI�AF

instruction is:

�RFA�PI#&f0S#*p0x0Y#&f1y2X#*p2320x0Y#&f1y2X#*p0x2925Y#&f1y2X#*p2320x2925Y#&f1y2X#&f1S��

Note that, in XyWrite v3.53-, this single �AEPI�AF instruction must be divided

into two �AEPI�AF commands not exceeding 80 characters each.

#&f0S   - push the current cursor position

#*p0x0Y - locate the cursor at page position 0X,0Y (upper-leftmost corner)

#&f1y2X - select Macro ID# 1 (#&f1Y) and execute last-ID'd Macro (#&f2X)

...     - same routine for the other three corners

#&f1S   - pop the cursor position

A cross prints at the four corners of the page, as shown in the diagram

below.  You must make sure that text fits *within* the crop marks, using

XyWrite formatting controls (LM, RM, IP, OF, etc).  If your text already

contains a Running Footer, insert the �AEPI�AF instructions at the *end* of the

Footer.

The cursor positions where the crosses print can be altered by modifying the

"#*p#x#Y" instructions in �AEPI�AF.  Current defaults represent the diagrammed

positions:

������������������Ŀ

0,0� .              . �2320,0  (2320 [2400]x0=8.00"x0")

�                  �

�                  �

�                  �  marginal areas cannot be

�                  �  used by the LaserJet

�                  �

�                  �

�                  �

�                  �

0,2925� .              . �2320,2925  (2320 [2400]x2925=8.00"x9.75")

��������������������

Method #2 -- Use Font Character(s)

----------------------------------

This method has several approaches, but all require that an appropriate font

(either internal, cartridge, or downloaded) be defined as a �AEPT�AF number in the

*same* .PRiNter file that you use to print your documents.  The internal font

MEDIUM-COURIER is handiest for LaserJet users; other US-ASCII fonts also work.

If you have MEDIUM-COURIER defined in your .PRN file (as, e.g., �AEPT1�AF), you

can simply call �AEPT1�AF immediately before the �AEPI�AF strings in your Running

Header/Footer.  FYI, the precise printer select string for MEDIUM-COURIER

portrait (if you need to define MEDIUM-COURIER in your .PRN file) is:

#&l0O#(10U#(s0p10h12v0s0b3T

N.B.:  If your XyWrite .PRN file defines the Symbol Set for MEDIUM-COURIER as

"#(8U", make a new �AEPT�AF and new "FO:" font definition with identical

information, but change "8U" to "10U" �� otherwise, Roman-8 characters will

print (no crop marks!).

This character arrangement makes a fairly good crop mark:  ���

A simple �AERF�AF might be (ASSUMPTIONS:  MEDIUM-COURIER is �AEPT1�AF, your default

writing font is �AEPT5�AF):

�RFA�PT1��PI#&f0S#*p0x0Y���#*p2310x0Y���#*p0x3040Y���#*p2310x3040Y���#&f1S��PT5��

A more complicated (but elegant) �AERF�AF follows; it manipulates:

���    ���

�        �

�        �

���    ���

�RFA�PT1��PI#&f0S#*p0x0Y���#*p0x50Y�#*p2310x0Y���#*p2370x50Y�#*p0x3010Y�#*p0x3040Y���#*p2370x3010Y�#*p2310x3040Y���#&f1S��PT5��

What do these �AERF�AFs contain?

�AEPT1�AF   - switch to MEDIUM-COURIER

#&f0S   - push the current cursor position

#*p0x0Y - locate the cursor at page position 0X,0Y (upper-leftmost corner)

���     - print crop mark

...     - same routine for the other three corners

#&f1S   - pop the cursor position

�AEPT5�AF   - return to default writing font

Enjoy!

�PG�