Program           : MIC (Module Integrity Check) Version 1.0

-------

Release Date      : July 1990

------------

Purpose           : To provide easy to use "protection" * from Computer Viruses

-------

Positive Features : - simple to install

-----------------   - easy to use

- runs fast!

Suggested         : Run this program AT LEAST once a day.

---------

Quick Start       :

-----------

1. Copy MIC.EXE to your hard drive (preferably into a

subdirectory that is along your PATH)

2. From the DOS prompt, type MIC and hit <enter>

(there are NO switches or command line options!).

3. Depending on the size of your hard disk, and the number of

programs and overlays on it, it should complete in about

20 seconds to 1 minute. (The bottom line on the screen

will give you a feel for how long it is progressing, where

it reports "Processing xx out of yy", where xx is the current

file, and yy is the total number of files to be processed).

4. MIC will create 4 files in your root directory:

MIC.ADD - lists program files added since the last MIC run.

MIC.DEL - lists program files deleted since the last MIC run.

MIC.CHG - lists program files changed since the last MIC run.

MIC.NEW - lists size, date, checksum, and other info about all

program files from the most current run.

5. After the first time, running MIC will also rename the existing

MIC.NEW file to MIC.OLD, for comparison purposes.

6. Upon completion, type or browse the file MIC.CHG.

Any entries in this file indicate a POSSIBILITY of VIRUS

ACTIVITY. Please read the full description that follows for more

information.

Full Description :

----------------

As mentioned above, MIC was written to offer a degree of protection

from Computer Viruses. Actually, it CANNOT PREVENT infection from a

virus - rather it tries to DETECT the presence of a virus.

If you can detect that your system has been invaded by a virus

early enough, then there is a good change you can get rid of it

without suffering any loss of programs or data.

Other Anti Virus products:

-------------------------

There are several other programs out there that offer virus

protection, either by Prevention, Detection, or Identification.

One fine program, mentioned below, is SCAN. It can IDENTIFY KNOWN

viruses by scanning all your program files for specific patterns

(of already identified viruses). The only problem is that it can

only detect those viruses that it was programmed for - so if newer

viruses come along, it would not detect them. However, SCAN is a

SHAREWARE program, and registered users will be kept informed of

updates to detect newer viruses as they are discovered.

Another program, also shareware, is FLU_SHOT+. It offers actual

PROTECTION from viruses, as well as detection.

FLU_SHOT + will require a little effort to install. It will flag you

when programs attempt to become memory resident, or try to write

to certain files. It can also check for changes made to your

BOOT SECTOR. This is definitely a quality piece of work, and for

the $14 registration fee, not a bad deal. However, it will take some

effort to set up the program to meet your needs.

Flu_Shot is available from :

Ross M. Greenberg

Software Concepts Design

594 Third Avenue

New York, New York 10016

While FLU_SHOT + is technically FAR AHEAD of this program, I think

that MIC is attractive because it is EASY to use, runs quickly without

any inconvenience, and does not take up memory (is not a TSR).

My feeling is that if something is too cumbersome to use, or even

just PERCEIVED as being cumbersome to use, then a lot of people

will simply not use it (even if it is a technical masterpiece,

even if it has a nice interface, and even though the threat from

viruses would justify putting up with any inconvenience caused

by using these products).

Compared with this Program:

--------------------------

So this program was written for basically those people that presently

are using NO ANTI-VIRUS software.

Yes, there are several programs out there that are BETTER than this.

I encourage you to look into them. Get one. Use it. But meanwhile,

if you have nothing else, then running this program at least once

a day will give you at least some "protection" * for the least amount of

effort.

*   I put the word PROTECTION in quotes because MIC will NOT protect you

from viruses - it will only alert you to possible viral activity

AFTER you are infected. However, if you detect a virus soon enough after

being infected, you may be able to eradicate it without losing any

programs or data.

How it works:

------------

MIC scans the entire disk that it is running on, and keeps a record

on all files ending in COM, EXE, SYS, BIN, and OVR.

For each file, it writes the file size, date and time last saved,

checksum, attribute, and for EXE files, the program Entry Point.

This information is stored in a text file in your root directory

called MIC.NEW. However, before creating MIC.NEW, the program first

renames the existing MIC.NEW (from the last time you ran MIC) to

MIC.OLD.

It then compares the two files, and anything in the new file, but

not in the old file is written to MIC.ADD (files added since last run).

Anything in the old file but not in the new file is written to MIC.DEL

(files deleted since last run). Any file where either the size,

date, time, checksum, or entry point has changed is written to MIC.CHG.

Explanation of Checksum :

-----------------------

A checksum is a method of adding up the values of all the bytes in a

file to come up with a total, called the checksum. But simply adding

the values of each byte is not reliable, since if you took a file

and replaced some of the A's with E's, and an equal number of the E's

with A's, you would get the SAME checksum.

So rather than simply adding the byte values, the checksum algorithm

usually performs some manipulation on each byte, like rotating the bits

a certain number of times, and logically ANDING or ORING the bits to

something else. The exact procedure varies by position of each byte,

so that if you replaced an "EA" with an "AE", you will come up with

a different checksum.

MIC calculates a checksum on the files it selects. However, to

checksum each entire file made the program run too slowly - about 20

minutes on my 16MHZ 386 SX with 180 program files.

So to speed it up, it performs a checksum only on part of the files -

the part or parts that a virus would most likely infect. What was

running 20 minutes now takes about 40 seconds!

This means, though, that it is possible for a virus to change an

executable program, and MIC would not detect the change, though, in

theory, you could the next time you ran the program, because it

would behave differently.

This means that MIC is NOT FOOLPROOF; it is LIMITED insurance that if

your PC does contract a virus, there is a good change that MIC will

alert you to the POSSIBILITY of an infection.

What to do if a change is detected:

----------------------------------

The MIC.CHG file will list any programs where the filesize, date,

time, checksum, or entry point has changed since the last time

you ran MIC.

IT will show both the OLD and the NEW values.

If a file has changed, IT DOES NOT NECESSARILY MEAN THAT YOU HAVE A

VIRUS.

There are several things that can cause a change:

- IF you wrote a program, then later made a change and recompiled it.

This would change probably all of the values recorded for it.

- If you ran an install program  and changed something like

monitor type, directory name, or any of the defaults. Some programs

store all the defaults in an Config file; others store the

information DIRECTLY in the program file. (One of my other

shareware programs, NUSTUFF.EXE does it. You can change the

default colors with another program called NUCOLOR, and it writes

the colors directly into NUSTUFF.EXE). This probably will not

change the filesize or date and time stamp, but could change

the checksum.

- If you ran "TOUCH.COM" or something similar. This is a little

utility to update the current date and time into the file's date

and time stamp. This however, should not cause a change to the

checksum.

- If you were "hacking around" with DEBUG or a hex file editor, or

a sector write tool, and made a change to the file.

In essence, the only thing this program will do is inform you if

something about a program file has changed. You will have to

determine the cause. And that may be difficult.

But if each time you run a newly downloaded program, and you run

MIC immediately before and after, and some totally unrelated COM

or EXE File has a new checksum, then you'd better be mighty suspicious.

That's the time to load one of those TSR programs that monitor all disk

accesses.

For added security:

------------------

You could set up a batch file to run MIC, which would basically involve

using a floppy disk to store the MIC.NEW and MIC.OLD files.

Then upon completion, copy these files back to the floppy.

This could protect you from the far-fetched possibility that someone would

write a virus to scan the MIC.NEW and change it to cover up

the dirty work that it does. As I said, far-fetched.

If you think you have a virus:

-----------------------------

As mentioned above, there is an excellent shareware program called

"SCAN", which scans your program files looking for any of about 60

known viruses. The same author also provides a program to remove

these viruses. These programs may be obtained from :

McAfee Associates

4423 Cheeney Street

Santa Clara, CA 95054

408 988 3832

BBS: 408 988 4004

PC Magazine in their Feb 13, 1990 issue, describes their own Data

Integrity Toolkit, with extensive instructions on how to get it for

free, how to install it, how to use it, and what to do if you have

a virus.

Their toolkit is much more sophisticated than this program and offers

a greater degree of protection; however, it also requires a bit more

effort to install and use.

In a worst case scenario, you should always be prepared to delete all

program files from your disk, then restore them from the ORIGINAL,

WRITE PROTECTED floppies. If there is still a problem, you may have to

do a low level format of your disk, then restore all data files from

backups (you do back up your data files regularly, right?), and then

restore all program files from the ORIGINAL WRITE PROTECTED floppies.

A FINAL WARNING:

---------------

A virus could theoretically defeat the protection offered by this program.

I won't explain how. Likewise, even an already identified virus could

(in theory) elude SCAN; and a virus could also bypass the protection

mechanisms of FLU_SHOT +. Therefore, you are NEVER 100% safe!

It's like putting a lock on your house. At some point the cost of

security becomes more than the value of the stuff that you're trying to

protect (or the amount of the insurance deductable!).

So take reasonable steps to protect yourself. Use MIC, and one or two

OTHER VIRUS protection type programs. But give yourself an out

for the worst case scenario. Back your data up regularly. And keep

your original program disks someplace safe, write protected. And be

prepared to have to do a hard disk format.

Limitations:

-----------

1. This program does not detect any changes to the BOOT Sector.

Some viruses infect the BOOT Sector, but unfortunately, this

program would not detect that. (A future revision may, though!)

2. Some viruses "go TSR". This program does not check memory, or

detect TSR type viruses.

3. This program does not detect all changes to a program file.

That is because the checksum routine does not check EACH and

EVERY byte in the program file. But it does check the areas

of a program file that a virus must change (like the area around

the entry point, the file header, etc) if it is to be able to

reproduce itself.

Warranty:

---------

There is no warranty, period.

Steve Leonard specifically disclaims all warranties, expressed or implied.

Use this product at your own risk.

In no event shall Steve Leonard be liable for any loss of profit, or any other

damages or loss incurred because of this product.

Distribution of this program:

----------------------------

This program may be used by anyone FREE of charge. Additionally, it may

be distributed by Shareware companies for a reasonable fee.

The only requirement is that this DOC file is included, and not altered.

Anyone who finds this program of value is encouraged to make a VOLUNTARY

DONATION to the author; those sending $10 or more will receive a free update

IF ONE IS WRITTEN, but there is no guarantee that an update will ever be

written.

However, even if you do not make a donation you are still free to use

this program as you see fit.

Steve Leonard

260 Dunbar Rd.

Hilton, NY 14468

P.S. Anyone having any real life dealings with viruses - I'd like to

hear from you. How did you know you had a virus, and how did you

get rid of it? I'm always trying to learn more.