Future Technology Trends

3M First to Release High Capacity 4 Megabyte Diskettes

St. Paul, Mn...3M has become the first major supplier to enter the commercial

market for high-capacity 4 megabyte 3.5-inch diskettes with the introduction

of its ED product.

3M's 3.5 ED (Extra Density) diskettes are based on a new barium ferrite media

formulation that will serve as the platform for a new generation of high

capacity diskette products that could eventually go up to 100 megabytes in a

3.5-inch design.

3.5 ED diskettes are compatible with Toshiba's models PD-211 and PD-212

diskette drives which employ subsaturation recording made possible by the use

of a pre-erase head.  Barium ferrite coating provides the ability to achieve

very high capacities using a conventional recording head.  This is due to the

inherent capability of the hexagonal barium ferrite platelets to lay flat on

the coating and to retain magnetism much more efficiently than needle-shaped

particles, such as conventional gamma ferric oxide.  The result is an advanced

high-capacity media and drive system with the reliability and cost benefits of

existing designs.

Additionally, barium ferrite media offers improved performance by creating a

smoother recording surface for improved head-to-media contact and a more

consistent signal compared to conventional floppy disk pigments.

3M's newest micro diskettes are visually and mechanically differentiated from

previous products.  The diskette shell is a dark charcoal color, compared to

the light gray 1 megabyte and black 2 megabyte products.  A distinctive 4

megabyte logo is also silk screened onto the shutter of each diskette, and is

complimented with a bold "ED" on the diskette shell.  Physically, the new

disks are characterized by a new sensing hole pattern that allows the drive to

distinguish the 4 megabyte disks from other 3.5-inch products.

Although the new 3.5 ED diskettes provide twice the capacity of the previous

product generation (4MB unformatted with a recording density of 35,000 bits

per inch), barium ferrite has the potential to support significantly higher

capacities, said Dave Tynan, 3M Market Development Manager for diskettes.

These diskettes represent the first in a new generation of products, rather

than a simple enhancement to existing diskette technology, Tynan said.

"The industry has reached a performance plateau with conventional oxide

media," said Tynan.  "Barium ferrite provides a new technology platform to

catapult capacities to significantly higher levels."

Due to the strength of the fundamental attributes of barium ferrite

technology, 3M has made an announcement of the Extra Density diskette.  "We

believe by doing this, we can help the industry accept this product as the new

high capacity standard."  "This is a significant product endorsement from 3M,"

said Tynan, "It demonstrates our confidence in the long-term viability of

barium ferrite and 3M's commitment to advancing diskette technology to the

next level.

3M's barium ferrite 3.5 ED diskettes are now available through 3M distributors

at a suggested list price of $32.50.

3M is the world's largest supplier of flexible magnetic recording media.  The

company supplies world-wide markets with a full line of diskettes, 1/4-inch

data cartridges, 1/2-inch computer tape, IBM and DEC-compatible 1/2-inch

cartridge media as well as CD-ROM and rewritable optical disks.

Helpful Information About Diskettes

The Truth About Diskette Erasure!

---------------------------------

(c) 3M 1990

Magnetic media products are reliable and offer good stability of the

recorded data.  Once recorded, the magnetic field strength of the bits will

last for a long time, unless they are altered by an external magnetic field or

worn or damaged by other causes.

The following points are intended to cover facts about diskette erasure,

what causes it and what does not.

1.  Magnetic fields of high intensity can erase the data on a diskette.  Some

magnetic fields drop off by the cube of the distance from the source.  This

means you have to be very close to the magnetic field to cause erasure.   A

spacing of only a few inches is usually all that is necessary to totally

protect a diskette from even extremely strong magnetic fields.

2.  Utility magnets and "refrigerator" magnets are strong enough to erase data

from a diskette if direct contact is made between the magnet and diskette.

3.  A magnet such as described in #2 when affixed to a file cabinet, with

diskettes inside the cabinet, would not be able to cause any erasure to the

diskettes.

4.  Diskettes cannot "conduct" the magnetism applied by a permanent magnet.

If a stack of diskettes were contacted directly by a magnet, only the closest

proximity areas could be erased, and not the entire stack.

5.  Heat cannot cause erasure until about 850 degrees F.  This normally won't

be encountered, but high heat can distort the polyester base of the diskette

media and case  causing readback problems.

6.  Static electricity discharge cannot cause diskette erasure.  There is no

magnetic field associated with static electricity, and extensive testing has

shown no erasure. Lightning discharge in close proximity to a diskette through

a pipe could generate a magnetic field strong enough to erase a diskette (and

probably destroy it as well).  Static discharge into a computer with a

diskette in it could cause incorrect information to be recorded, thus static

strips and mats are recommended.

7.  Radar and microwave transmissions can not affect magnetic media unless

they are directly in front of the antenna (this would also be a burn hazard to

humans).

8.  X-Rays can not erase magnetic media. Theoretically this is impossible

because X-Rays do not have any magnetic fields associated with them, and

extensive testing has not found any erasure evident with high level exposures.

9.  Metal detectors in airports do not have a high enough field to erase

magnetic media.  These devices will require posting if they evolve to higher

level fields that could affect magnetic media, hearing aids, pacemakers, etc.

We hope this has provided you the proper facts about diskette erasure.

Thank You!

3M Data Storage Products Division