DOS 5 UPDATE FOR 800K DISKETTE FORMATTING UTILITY

-------------------------------------------------

Original program Copyright by Alan D. Jones.

Update Copyright by David Lerner 7-13-91.

This  update  has  been created  without  the  permission  of the

original  copyright  holder.  It  may be  freely  used  for  non-

commercial purposes,  as allowed  by the original  copyright. All

rights to the update are assigned to Alan D. Jones.

The format  program  produces  a boot  sector  of  the  formatted

diskette  by copying the boot  sector from the  default drive and

inserting  new  media  parameters,  in  accordance  with   MS-DOS

standards. This operation will  fail when the default drive  is a

disk partition larger than  32 Mb. Such partitions are  supported

by both DOS 4 and DOS 5. The modified program (800KFMT.COM)  uses

a modified call on interrupt 25 that is supported by  DOS 3.0 and

higher. This call supports both large and small partitions.

The  program  720KFMT.COM is  also  provided  to  format  720  Kb

diskettes in a  1.2 Mb drive.  This allows 5.25" diskettes  to be

used  to backup the 3.5"  diskettes that are  frequently used for

software distribution. The formatted diskettes have been found to

be compatible  with the DOS DISKCOPY function for copying between

5.25" and 3.5" media. This  is especially convenient for software

installation where the 1.2  Mb drive is A:, and  the installation

program does not support installation from drive B:. The AMI BIOS

in the  author's system  also allows  the system to  boot from  a

720/800 Kb  diskette,  without  the  800K.COM  program  described

below.

The programs provided by  Alan D. Jones also include  800K.COM to

permit the use of 720K/800K diskettes with systems where the BIOS

causes track skipping for  any double density diskette used  in a

1.2 Mb drive. The  800K.COM program must be manually  disabled or

uninstalled to allow the  use of 360K diskettes in a  drive where

800K has been enabled.

Assembler  source  programs  are  included  for   all  referenced

programs.

David Lerner

Vienna, VA.