COLSET Version 1.1.4d

(c) Copyright 1990, Raymond P. Tackett

COLSET is a resident utility which maintains text mode screens at your

preferred color setting and mode.  It detects and defeats attempts to reset

your text colors to white on black.  The program can be customized to maintain

color settings in any 16 color text mode, such as Paradise modes 54h and 55h

with 132 characters per line.

COLSET can replace ANSI escape sequences for color setting and allows you to

get rid of simple color setting escape sequences in your DOS prompt.  You may

even be able to delete the "device = ansi.sys" line from your config.sys file

(unless you also map your function keys).

Version 1.1.4d is designed to add functionality if your command processor is

4DOS by JP Software.  If you use 4DOS, COLSET will check your video mode every

time you return to the DOS prompt.  If it was changed by a previous

application and not reset, your screen will be reset to your chosen video mode

and colors automatically.  Version 1.1.4d is not downward compatible with

version 1.1 and the two should not be mixed.

Hardware requirements:

PC or AT (or clone) with color video adapter and monitor.  Systems with two or

more monitors, any one of which is monochrome should not use COLSET.  DO NOT

ATTEMPT TO RUN COLSET ON A SYSTEM WITH A MONOCHROME MONITOR!  If your stored

system configuration shows a monochrome display as your primary adapter,

COLSET is supposed (but is not guaranteed) to abort.  If you try to run COLSET

on a monochrome monitor, the monitor hardware may be damaged if the controller

switches to a color mode.

Software requirements:

DOS 2.0 or greater

4DOS version 3.0 or greater recommended

Getting started:

The command syntax is:

COLSET [color] [minimum custom mode] [maximum custom mode] [set mode]

All command line arguments may be omitted.  The default will be gray text on

blue background with no custom mode enhancements.  All arguments are in

hexadecimal, two digits (one byte) each separated by one or more blanks.

If your screen colors change immediately to gray on blue or your customized

choice and the message "COLSET is now resident" appears, COLSET is resident

and will do its job until you power off or reboot.  If your screen does not

blank or change colors, the load failed.  You should see the message

"Monochrome monitor detected.  COLSET not installed".  If you see only a

copyright notice, the program hung, probably due to incompatibility with your

video card.

[color]

Consists of two hexadecimal digits constructed as described under "Color

setting help", below.  For example "colset 2E" will give you yellow

characters on a green background.

[minimum custom mode] and [maximum custom mode]

These options must be used together or not at all.  Some video adapters

offer custom text modes.  For example, a Paradise Plus video adapter uses

modes 54 and 55 for 132x43 and 132x25 text modes, respectively.  If you

type "colset 17 54 55", your gray on blue color settings will also apply

to the two Paradise 132 column modes in addition to mode 3.  The Trident

chip set, used in several controllers, offers various text modes in the

range 50 to 5A.  "colset 17 50 5A" would cause colset to work in all modes

from 50 to 5A, inclusive as well as mode 3.

If you specify a mode range which includes a mode not having 16 colors

available, results are unpredictable when that mode is entered.

Mode range checking can be disabled by using "ff 00" as minimum and

maximum, respectively, or any minimum greater than the maximum.

These parameters can be used only if [color] is specified.

[set mode]

COLSET will switch your video adapter to the mode specified, whether or

not it's in the min/max mode range.  The mode is not likely to remain set

for long, however.  The DOS MODE command and most application packages

reset your screen to mode 03 via hardware, not BIOS calls.  Direct

hardware mode changes are not detected in order to avoid interference with

applications that don't know about oversize screens.  BIOS mode set calls

to set modes 2, 3, or 7 will be switched to your chosen mode.  [set mode]

can be used only if all previous command line options are used. If 4DOS is

the command processor at the current shell level, COLSET will check the

BIOS video mode and reset it if necessary upon return to the prompt line.

Command line parsing is extremely simplistic and does not check for errors.

Garbage in, garbage out!  Blanks are the only valid delimiters between

parameters.  Any number of blanks may be used.  The letter (A-F) parsing is

case-insensitive.

Colset may be run as many times as desired.  Only one copy will remain

resident.  Subsequent runs will just change colors and mode selection.  Every

run after the first will be acknowledged with the message "COLSET parameters

updated".

What it does:

COLSET intercepts and examines BIOS calls which manipulate the video setup

(INT 10h).  Programs which write directly to video RAM or write adapter

registers do not fall under the influence of COLSET.

Only certain calls are "interesting" to COLSET.  "Uninteresting" calls

are passed through to the system almost immediately.  Your DOS level

default text mode is mode 3, which is set by such commands as "mode

co80".  Only calls which set mode 3 (and/or your customized choices) or

perform certain functions in one of your selected modes are "interesting".

Following is a list of "interesting" functions and COLSET's action:

Set Video Mode (3) -- Mode 3 set is allowed to proceed and is followed by

a quick screen erase to the correct background color.

Set Video Mode (2) -- Mode 2 is forced into mode 3 to keep the chroma

burst alive on composite monitors (relatively rare).

Set Video Mode (7) -- Mode 7 (monochrome) is forced into mode 3.  This is

hazardous to monochrome monitor hardware, hence the strong warning

against using COLSET with a monochrome monitor.

The following actions take place only if the current video mode is 3 or

one of your chosen modes.

Scroll up or initialize screen (function 6) -- Background color requested

is checked for black.  If it's black, COLSET's color settings are

substituted for the caller's.

Scroll down or initialize screen (function 7) -- Same as above

Write character and attribute at cursor position (function 9) --

background color is checked for black.  If it's black, COLSET's

background color is substituted.  If the substitution would make the text

and the background the SAME color (i.e., hide the text), the text color

is inverted to maintain readability.

Colset also hooks the "return to prompt" interrupt issued by 4DOS as a

signal to check the current video mode and reset it if necessary.

Side effects:

A few programs which use text for graphics may show your chosen

background color in places that used to be black.  The effect may be

spotty, depending on the behavior of the program.

Only colors with black backgrounds in mode 3 and selected text modes are

affected.  Various programs may set non-black background colors.  COLSET will

have no effect on the operation of such programs.

If a black background is set in a non-selected video, COLSET will have no

effect.

Some screen blankers can't get past COLSET to blank the screen.  With

today's shorter persistence phosphors, burn-in isn't the problem it

used to be.  Try your own blanker and make your own decision.

If you set 43 or 50 line text modes, the initial color setting may or

may not fill the entire screen.  PCMagazine's EGA50, for example, does

not seem to set the correct video page size in the BIOS area.  Their

EGA43 utility is compatible with COLSET, however.

ANSI.SYS and its various clones also set screen colors.  If both ANSI.SYS and

COLSET are loaded, you may get strange screen color combinations which can

only be cleared by a command to the correct utility.  For example, Nels

Anderson's MAHJONGG issues an ANSI escape sequence on exit.  If ANSI.SYS is

loaded before COLSET (the usual case), COLSET cannot control the screen colors

which result.  COLSET does not respond to ANSI escape sequences.

Color setting help:

Binary color setting chart -- use 1 for on, 0 for off in each selection:

Background                    Foreground

Blink  Red  Green  Blue     Bright    Red Green Blue

|     |     |     |         |        |    |    |

|     |     |     |         |        |    |    Blue used in text color

|     |     |     |         |        |    Green used in text color

|     |     |     |         |        Red used in text color

|     |     |     |         Foreground color in high intensity

|     |     |     Blue used in background color

|     |     Green used in background color

|     Red used in background color

Text will blink (can be annoying)

Example:  The default is 17 hexadecimal.  In binary that's 0001 0111.

Here are the eight bits (ones and zeroes) broken down as in the chart:

0     0     0     1         0        1    1    1

You can see that the background is blue, the text will NOT blink, the

intensity will be low, and the text will use red, green, and blue.  The

total effect of red, green, and blue together is white, which in low

intensity looks gray.

To convert binary to hexadecimal, break the eight bits into two groups

of four.  For example, 0 0 0 1 0 1 1 1  should be written as 0001 0111.

Convert each group of four bits to a character according to the

following table:

0000 = 0   0001 = 1   0010 = 2   0011 = 3   0100 = 4   0101 = 5  0110 = 6

0111 = 7   1000 = 8   1001 = 9   1010 = A   1011 = B   1100 = C  1101 = D

1110 = E   1111 = F

In the above example 0001 0111 = 17.

Color mixes:  The three color bits; red, green, blue; give you eight possible

colors.  The foreground can be high or low intensity, giving sixteen possible

text color effects.  The color combinations are:

0000 black

0001 blue  (blue only is on)

0010 green (green only is on)

0011 cyan  (green + blue)

0100 red   (red only is on)

0101 magenta (red + blue)

0110 brown (dim yellow -- green+red)

0111 gray  (dim white -- red+green+blue)

The following are possible in foreground (right digit only):

1000 black

1001 bright blue

1010 bright green

1011 bright cyan

1100 bright red

1101 bright magenta

1110 yellow

1111 white

Disclaimer:

This program has been tested to the extent of my ability to do so and works

correctly on my machines and does not seem to interfere with PC Paintbrush,

Lotus 1-2-3, or Microsoft Windows 286.  There is no warranty expressed or

implied as to whether it will work correctly or work at all on your machine.

You use it entirely at your own option and risk, including any and all

possible damage which may arise.

All trademarks found in this document are the property of the respective

owners.

COLSET Distribution Policy:

(c) Copyright 1990, Raymond P. Tackett

The entire work is my copyrighted property.  You may use and distribute it

freely in its original, unmodified form provided:

1.  The original, unmodified .ZIP file is the distribution form.

2.  No fee is charged for the package itself.  However, reasonable and

customary duplicating, download, or connect time charges may be made.

3.  The work is not bundled with any goods or services.

Support:

There is no registration. I am not charging for the program because I cannot

afford the time to provide proper support.  I can be reached via CompuServe

(76416,276) to answer questions and receive comments.

The source code is written for Borland's Turbo Assembler.  I haven't made a

final decision regarding source availability.  Please let me know of any

interest in having the source along with the reasons for the interest.

Revision history:

Version 1.1  Mode setting command line option added.

Parameter change acknowledgement message added.

Version 1.1.4D Added hook into multiplex interrupt (27h) to allow "seeing"

4DOS return to prompt.  Interrupt is still hooked but has

no effect if the command processor is not 4DOS.

Fixed bug with 40X25 as primary monitor being reported as

monochrome.

Version 1.1.4D-Beta  Took out keystuffing of carriage return due to

change in 4DOS 3.01 beta version.  If used with 4DOS 3.0,

your prompt will disappear if the mode has to be reset.#########################################################