DEVALIAS.SYS

This unique utility allows alternate names to be used for DOS character devices.

It would be useful, for example, if you had several devices on your system

connected to various COM ports and/or LPT ports, and wanted a way to give them

mnemonic names.  For example, we use it as a way of encoding the printer type

so that our print spooler can format the text properly before sending it to

the printer.

The utility is used inside CONFIG.SYS with a line like

device=devalias.sys START OLDNAME NEWNAME

where OLDNAME is the name of an already existing device, and NEWNAME is the

name you desire to give it.  Note that OLDNAME is not removed: the device can

be accessed through either name.  START is the name of a device which precedes

OLDNAME in the DOS device linked list.  This is necessary due to the segmented

x86 architecture.  This can be the name of a user device such as

the expanded memory driver EMMXXXX0, but you can also use CON, although if you

do use CON, then you cannot alias CON.  You can use a previous alias as the

START name.  The resident portion of DEVALIAS uses only 32 bytes of

conventional memory per alias.  I have tested DEVALIAS only on DOS 3.3, but I

have no reason to believe that it won't work any any version from 2.0 on.

--> Example 1:

device=devalias.sys con lpt1 lj$

device=devalias.sys con com1 plot$

In this example, we install the name 'LJ$' as an alias for LPT1, and 'PLOT$'

as an alias for COM1.  This presumably would mean that there is a LaserJet

connected to LPT1 and a plotter connected to COM1.  You can send data to these

devices with commands like

copy text lj$

or, if you are inside a program which plots, for example, you can specify the

'PLOT$' as the output file and then plot to the 'file'.

--> Example 2:

device=devalias.sys con com1 modem$

device=devalias.sys modem$ con tty

In this example, we see how to use an alias as a START name.  First, we alias

COM1 as MODEM$, then use that name as a way of aliasing CON as the standard

UNIX name for the user's terminal, TTY (a.k.a. /dev/tty).  Notice that you

could also use a name which already exists as NEWNAME--this will make the old

use of the name inaccessible (e.g., 'device=devalias con lpt2 com4' would make

COM4 an alias for LPT2, and the original COM4 name would no longer be useable).

--> A word on device names in DOS.

First, here is the table of the standard DOS character devices in their standard

order:

(NUL)

CON

AUX

PRN

CLOCK$

COM1

LPT1

LPT2

LPT3

COM2

COM3

COM4

When a device is opened, DOS searches this list in order, starting with NUL.

Of these names, NUL can never be aliased with this method.  Any or all of

the others can be.  New names are added after NUL and before CON, in

most-recent-first order.  For example, after Example 2 above, the start of

the list would become:

(NUL)

TTY

MODEM$

CON

...

The other thing which must be kept in mind about DOS device names is that

when files are opened in DOS, the path and suffix are stripped from the name

and the resulting basename is searched for in the device table.  If it is

found, the device is used instead of the file.  Thus, if you have a file

called \src\include\tty.h on your system, it will become inaccessible once

you have a device (either an actual device or an alias) named TTY.  For this

reason, it is probably a good idea to use names which are unlikely to exist

elsewhere on the system, such as names ending in $.

By the way, it is a useful convention to refer to device files using the

directory \DEV as a path: i.e., \DEV\COM1 instead of just COM1.  This makes it

explicit that you are referring to a device instead of a plain file.  DOS

special-cases this directory and will find the device even if there is no such

directory on the disk.  Unfortunatly, except in some versions of DOS 2.X, this

doesn't allow files bearing the same basename as a device to be accessed.

--> DEVALIAS is shareware.

I want people to use this utility if they need it, and I don't have the energy

to get into marketing or anything like that.  I will support DEVALIAS on CIS,

either through the DOS/DOS UTILS section of the IBMSYS forum, through e-mail

to me at my CIS account 71061,1324, or through US mail at the address below.

If you like it and use it for more than a month or so, I request that you

indicate this by sending $5 to me for each machine on which you use it.  My

address is

Greg Shenaut

PO Box 414

Vacaville CA 95696

Please indicate that you are registering DEVALIAS.  Also, you are free to

share DEVALIAS with your friends: please give them this README file and

explain that it is shareware, not freeware.  Thank you.