Sydex

EAGLE16

A Device Driver for Eagle 1600 tm  MS-DOS Diskettes

EAGLE16

A Device Driver for Eagle 1600 MS-DOS Diskettes

Copyright 1990, Sydex

All Rights Reserved

Version 1.0 January, 1990

Sydex

153 North Murphy Ave.

Sunnyvale, CA 94086

(408) 739-4866

NOTICE

THIS IS NOT FREE  SOFTWARE!    If  you  paid  a  "public

domain" vendor for this program,  you paid for the serv-

ice of copying the program,  and not for the program it-

self.    Rest  assured  that  nothing  ever  gets to the

originators of this product from such a sale.   You  may

evaluate  this product,  but if you make use of it,  you

must register your copy.

We offer several inducements  to  you  for  registering.

First  of  all,  you receive the most up-to-date copy of

the program that we have--and we do update  the  product

on  a  regular  basis.    You  also  receive support for

EAGLE16--which can be quite  valuable  at  times.    And

finally,  we include a few utilities not included in the

evaluation package.  Make no mistake however--we do  not

"cripple"  our  evaluation software--our sample programs

are fully functional and complete.

REGISTRATION INFORMATION

The  non-commercial  single-user  registration  fee  for

EAGLE16  is  $15.00 US.   The commercial and multisystem

site fee is $50.00.

Send a check or company P.O.  for the appropriate amount

to:

SYDEX

153 North Murphy Ave.

Sunnyvale, CA  94086

(408) 739-4866

Major Credit Cards Accepted

Please indicate what product you are ordering and if you

have  a  requirement  for 3.5" media.   We normally ship

5.25" 360K diskettes,  but we  will  furnish  3.5"  720K

media  upon request.   Orders placed outside of the U.S.

and Canada should include an additional $5.00  for  air-

mail.  We can accept U.S. funds only, please.

DISTRIBUTION NOTICE

This  is  "user-supported"  software.   You  are  hereby

granted a license by Sydex to distribute this evaluation

copy of EAGLE16 and its documentation,  subject  to  the

following conditions:

1.   EAGLE16 may be distributed freely without charge in

evaluation form only.

2.   EAGLE16 may not be sold, licensed, or a fee charged

for  its  use.    If a fee is charged in connection

with EAGLE16,  it must cover the cost of copying or

dissemination  only.   Such charges must be clearly

identified as such by the originating party.  Under

no circumstances may the purchaser be given the im-

pression that he is buying EAGLE16 itself.

3.   EAGLE16 must be presented as a complete  unit,  in-

cluding this documentation.   EAGLE16's  documenta-

tion may be not amended or altered in any way.

4.   In granting you the right to distribute the evalua-

tion form of EAGLE16,  Sydex does  not  acknowledge

you as the owner of EAGLE16 in any form.

Any  other  use  or distribution of EAGLE16 is expressly

forbidden without the written consent of SYDEX.

________________________________________________________________

"IBM",  "PC DOS" and "Personal System/2" are trademarks of Inter-

national Business Machines Corporation.   "MS-DOS" is a trademark

of Microsoft Corporation.

What is EAGLE16?

EAGLE16 is a device driver which supports reading and writing  of

MS-DOS diskettes from  Eagle 1600 tm  series computers on PC XT- or

AT-compatible  machines.    Under DOS 3.2,  the utilities FORMAT,

DISKCOPY and DISKCOMP are supported,  as well as normal DOS  read

and write operations.

To use EAGLE16, your computer must be PC AT-, XT- or IBM Personal

System/2  compatible.    Computers  such as the Tandy 2000 or the

Zenith Z-150 are not suitable because of  their  relative  incom-

patibility.   Further, your computer must be equipped with either

a 5.25" high-density  (1.2  Megabyte)  or  5.25"  "quad"  density

drive,  such  as the Teac 55F.  The diskette adapter on your com-

puter must match the diskette drive characteristics.  That is,  a

high-density  drive  must  be  used  with a high-density adapter.

Many manufacturers, such as Western Digital, produce high-density

diskette adapters for XT-style computers,  and these may be  used

successfully with EAGLE16.   Currently, we support only the first

(or primary) diskette adapter in a system.

And finally,  your computer needs to be running PC-DOS or  MS-DOS

Version  2.0 or later.   DOS 3.2 or later is preferable,  but not

required.   Version 2 DOS users may find that some EAGLE16  func-

tions, such as formatting, are not supported.

Installing and Using EAGLE16

Installation of EAGLE16 is straightforward.  First, find the file

CONFIG.SYS  in  the root directory of the disk that your computer

boots (loads DOS) from.   It may be possible that you do not have

a  CONFIG.SYS file,  in which case the next few steps will create

one.   If you are unfamiliar with the CONFIG.SYS file,  take time

out  now and read the section in your DOS manual dealing with it,

and in particular, the DEVICE= statement.

You'll need to add a line to CONFIG.SYS of the following  general

form:

DEVICE=EAGLE16.SYS  /D:unit  /F:format

This  can  be  done with the program EDLIN which is supplied with

DOS, or with a text editor of your choosing.   If you use a word-

processing editor, make sure that you indicate that CONFIG.SYS is

a nondocument file.   If you didn't find a CONFIG.SYS,  the above

line will be the only line in the file which is created.

The value of unit can range from 0 to 3 and represents the physi-

cal diskette unit to be used to read and  write  the  Eagle  1600

diskettes.   If this unit is normally called A: on your computer,

then the value of unit is 0;  if B:,  then unit is 1.    External

drives are generally units 2 and 3.

Page 1

The  value  of  format depends on the type of diskette drive that

you're using.  If it's a high-density 1.2 megabyte unit, then the

value of format is 1.   If you're using a "quad"  density  drive,

such as the Teac 55F, then format is 2.

If /D:unit is omitted in the configuration line,  unit is assumed

to be 1. If /F:format is omitted, format is assumed to be 1 also.

The last step is to copy  the  file  EAGLE16.SYS  into  the  root

directory of your system or "boot" drive.   Finally, re-boot your

computer.   If your computer is configured with a revision of DOS

3, the following message will be displayed:

Eagle 1600 diskette driver, Copyright 1990, Sydex

All Rights Reserved.

Eagle 1600 DOS diskette unit x is accessible as drive y:

The  letters x and y will take on values which depend on the con-

figuration of your computer.

If your computer is configured with a revision of DOS 2, the fol-

lowing message will be displayed:

Eagle 1600 diskette driver, Copyright 1990, Sydex

All Rights Reserved.

Eagle 1600 DOS diskette unit x installed.

In this case,  the drive letter which is used to read Eagle  1600

diskettes  will generally be one greater than the last drive let-

ter in use before you modified CONFIG.SYS.  For example, if drive

C: was the last drive on your computer,  EAGLE16 will  use  drive

letter D:.

As an example of the whole process described above,  suppose that

we wished to add EAGLE16 to a PC-AT "clone" which has  one  high-

density  diskette drive and a hard disk.   Our CONFIG.SYS file on

drive C: might look like this after we modified it using EDLIN:

FILES = 30

BUFFERS = 10

DEVICE = EAGLE16.SYS  /D:0  /F:1    (This line is added)

We then copy EAGLE16.SYS to drive C:...

COPY A:EAGLE16.SYS  C:\

We then press the  keys  Ctrl,  Alt  and  Del  simultaneously  to

"reboot"  our  computer.    After  the rebooting process has com-

pleted, the following message is displayed:

Eagle 1600 diskette driver, Copyright 1989, Sydex

All Rights Reserved.

Eagle 1600 DOS diskette unit 0 is accessible as drive D:

Page 2

We then find that Eagle 1600 diskettes placed in our A: drive can

be read and written if we refer to that drive as drive D:.

FINAL COMMENTS...

EAGLE16 was written in response to inquiries from  customers  who

had  our  CP/M  diskette transfer package,  but who had no way to

easily handle the Eagle 1600 DOS diskettes.   We hope  that  this

situation is now remedied.  If you experience problems in getting

EAGLE16 to work for you, or if you have questions about any Sydex

product, please give us a call.

Page 3

The Sydex Product Line

AnaDisk  -- The compleat diskette utility.   Nothing like it

anywhere else;  scan,  edit,  repair and copy just about any

kind of diskette.  $25.00 ($150.00 site)  registration fee.

Con>Format  --  Concurrent  "background" diskette formatter.

Features  "pop-up"  operation  and  "hot  key"   activation.

You've  got to see it to believe.   Supports all current DOS

formats. $15.00 ($50.00 site) registration fee.

22DISK -- Transfer files, format, examine and erase files on

"foreign" CP/M diskettes on your PC.   Includes tips on sup-

porting  8"  and  5.25" single-density diskettes.   Contains

definitions for over 200 different formats.  $25.00 ($100.00

site) registration fee.

22NICE -- A CP/M 2.2 emulation package.   Supports  the  NEC

V-series  chips  or  performs emulation by software for both

the 8080 and Z80 processors.   Includes  terminal  emulation

and  diskette  handling  for common CP/M systems.   Includes

22DISK.  $40.00 ($150.00 site) registration fee.

TeleDisk -- Turn any diskette into a  compressed  data  file

and vice-versa.   Allows you to send and receive entire dis-

kettes via modem.   Even works  with  some  "copy-protected"

diskettes.  $20.00 ($150.00 site) registration fee.

COPYQM -- Mass diskette duplicator.  Format, copy and verify

multiple  diskettes  from  a single master.   Implements "no

keyboard" interaction mode and drive "round  robin"  servic-

ing.    Supports  all standard DOS formats.   $15.00 ($50.00

site) registration fee.

FORMATQM -- Mass diskette formatter - format a box  of  dis-

kettes at a single sitting.  Implements "no keyboard" inter-

action mode and drive "round robin" servicing.  Supports all

standard  DOS  formats.    $10.00 ($40.00 site) registration

fee.

...And, soon to be released:

InterDisk -  The  universal  Diskette  Interchange  utility.

Features  user-programmable  media conversion for those for-

mats not covered by 22DISK.  Available 1Q90.

Information on any of these products can be obtained  from  Sydex

by  calling (408) 739-4866 (voice) or by leaving a request on our

Bulletin Board System (408) 738-2860 (1200-9600 bps, N81).

Page 4