FASTSE -- Speedup Utility for the Toshiba T1000SE.

Alan J. Fridlund, Ph.D.

August 1, 1990

FASTSE acts on the Toshiba T1000SE's memory circuitry to

speed up the "refresh" rate of the computer's dynamic RAM. As

a result, the speed of main memory accesses is increased, and

the SE runs 8-9 % faster.

To verify the speed difference:

1) Run the included SETEST.EXE program and note the number

of seconds it takes to complete an integer 1-5000 loop

(16.75 sec on my SE).

2) Run FASTSE.COM.

3) Re-run SETEST.EXE, and note the new time. On my SE, it

now takes only about 15.30 sec. This represents a speed

increase of 8.5 %,  small but often noticeable.

FASTSE.COM is designed conservatively to work on most SE's.

There is a chance that on some units, a slow memory chip will

prevent proper operation after running FASTSE.COM. If your SE

misbehaves after running FASTSE, absolutely no damage is done.

Re-booting will restore normal operation immediately.

* * *

CAVEAT:

Verifying proper operation of your SE after running FASTSE

is * your * responsibility; I assume no liability should your

unit behave improperly after running FASTSE.COM.

TO INSURE THAT YOUR UNIT IS OPERATING CORRECTLY:

Make sure that your Hard RAM is backed up.  Run FASTSE.COM

and then run all of your most frequently used software to verify

proper operation. If the SE misbehaves, just reboot and every-

thing will return to normal (you may have to restore the contents

of your hard RAM in some cases).

* * *

If your SE operates properly after running FASTSE.COM, you

can include it in your AUTOEXEC.BAT file so that it runs auto-

matically on boot-up.  The program is non-resident, which means

that it takes up no memory after it's run.

FASTSE may also work on Toshiba's other than the SE; however,

I have not had the opportunity to try it on them.

FASTSE isn't free. If it works for you, please help defray

my time and effort by sending $5.00 to:

Alan J. Fridlund, Ph.D.

797 Sequoia Walk #J

Goleta CA 93117

CompuServe [71511,3702]

FASTSE.COM is based on program QFRESH by Brian K. Roemmele.

Happy Computing!