Rom capture utilities for PC and AT

This is a set of utilities that allows the user to capture the contents

of the rom sets in either the PC or the AT.  It allows the content of the rom

to be written to a file named by the user for the XT or the AT.  A utility

called romsplit is provided for the AT that allows the atrom file to be split

into two files that represent the even and odd bytes and which are called

<inputfile>.evn and <inputfile>.odd, respectively.  Note that this is not

necessary for the XT.  Another utility called lsplit is provided to allow the

large files produced by romsav to be split into files that will fit into the

desired destination eproms.  For prom burners that accept data format in the

Motorola S record format, a utility called binex is provided that converts

a binary file into standard S record format.  A typical session is described

below for an AT.

1.   Run the command "getatrom" yourfilename.  This will produce a file called

"yourfilename".

2.   Run the command "romsplit" yourfilename.  This will produce two files

called "yourfilename.evn" and "yourfilename.odd".

3.   If the destination eproms are 27256 then it is not necessary to split the

files into smaller ones, otherwise run the command "lsplit <epromsize>

<yourfilename.evn>, where <epromsize> is the decimal size of the destination

file in bytes (e.g. "lsplit 16384 yourfilename.evn" to make <yourfilename.evn>

into two files called "F0" and "F1" that will fit into 27128 eproms.  Rename

the files to some new name so that they will not be overwritten at the next

step.  Now run the command "lsplit <epromsize> <yourfilename.odd> to produce

files for the odd byte eproms.

4.   If you are producing S records run "binex <sourcefile> <destinationfile>"

on each file that you wish to convert to S record format.

5.   Feed them to a prom burner and keep track of which eproms were even and

odd and the order in which they were burned to allow you to install them

properly in the target system.

THE ABOVE PROGRAMS WERE COMPILED WITH TURBO C ver 2.0