RM60

LITES.COM

Command

Jeff Prosise			1989 No. 5 (Utilities)

Purpose:	Replaces an RS-232 breakout box by

displaying the status of the DTR, DSR, RTS, CTS, DCD, and RI

pins; also indicates the data rate, parity, number of data

bits, and number of stop bits of the serial port being

monitored.

Format:	LITES [comport] [U]

Remarks:	LITES is a memory-resident utility and may

be loaded either at the DOS prompt or as part of an

AUTOEXEC.BAT file.  If no communications port is specified,

the default comport monitored is COM1.  After loading,

alternately pressing Alt-L (the default hotkey) pops up and

cancels the onscreen display, which is located (by default)

in the upper right corner of the screen.  LITES can be

deinstalled by entering it with the optional u (or U)

parameter, which  simplifies changing the comport to be

monitored.  (Do not precede the U with a slash.)

The various default options of LITES can be changed

with DEBUG.COM.  The procedure is explained in the printed

article, which also contains full tables of scan codes,

shift masks, colors, and the like.  The available patch

points, bytes allocated, and default values are listed

below.

Function	                Address	  Bytes     Default

Hotkey ID String	            0155h	    8	        "Alt-L"

Hotkey Scan Code	            0160h	    1	         26h

LITES Video Attribute	       0161h	    1	         70h

LITES Display Row Number	    0162h	    1	         0

LITES Display Column Offset	 0163h	    1	         0

Refresh Frequency Counter	   0164h	    1	         2

Default COM Port Designator	 0165h	    1	         0

Hotkey Shift Mask	           01A0h	    1	         8

Available for downloading from PC MagNet (see the

LITES by Modem sidebar), LITES.COM is already compiled and

ready to run.  LITES.BAS will automatically create LITES.COM

when run once in BASIC.  To create LITES.COM from the

LITES.ASM source code requires use of a macro assembler (IBM

or Microsoft, Version 2 or later) and the following

commands:

MASM LITES;

LINK LITES;

EXE2BIN LITES LITES.COM;