Sydex

RainDOS

A Device Driver for DEC Rainbow tm  MS-DOS Diskettes

RainDOS

A Device Driver for DEC Rainbow MS-DOS Diskettes

Copyright 1989, 1990, Sydex

All Rights Reserved

Version 1.1 January, 1990

Sydex

153 North Murphy Ave.

Sunnyvale, CA 94086

(408) 739-4866

NOTICE

THIS IS NOT FREE  SOFTWARE!    If  you  paid  a  "public

domain" vendor for this program,  you paid for the serv-

ice of copying the program,  and not for the program it-

self.    Rest  assured  that  nothing  ever  gets to the

originators of this product from such a sale.   You  may

evaluate  this product,  but if you make use of it,  you

must register your copy.

We offer several inducements  to  you  for  registering.

First  of  all,  you receive the most up-to-date copy of

the program that we have--and we do update  the  product

on  a  regular  basis.    You  also  receive support for

RainDOS--which can be quite  valuable  at  times.    And

finally,  we include a few utilities not included in the

evaluation package.  Make no mistake however--we do  not

"cripple"  our  evaluation software--our sample programs

are fully functional and complete.

REGISTRATION INFORMATION

The  non-commercial  single-user  registration  fee  for

RainDOS  is  $15.00 US.   The commercial and multisystem

site fee is $50.00.

Send a check or company P.O.  for the appropriate amount

to:

SYDEX

153 North Murphy Ave.

Sunnyvale, CA  94086

(408) 739-4866

Major Credit Cards Accepted

Please indicate what product you are ordering and if you

have  a  requirement  for 3.5" media.   We normally ship

5.25" 360K diskettes,  but we  will  furnish  3.5"  720K

media  upon request.   Orders placed outside of the U.S.

and Canada should include an additional $5.00  for  air-

mail.  We can accept U.S. funds only, please.

DISTRIBUTION NOTICE

This  is  "user-supported"  software.   You  are  hereby

granted a license by Sydex to distribute this evaluation

copy of RainDOS and its documentation,  subject  to  the

following conditions:

1.   RainDOS may be distributed freely without charge in

evaluation form only.

2.   RainDOS may not be sold, licensed, or a fee charged

for  its  use.    If a fee is charged in connection

with RainDOS,  it must cover the cost of copying or

dissemination  only.   Such charges must be clearly

identified as such by the originating party.  Under

no circumstances may the purchaser be given the im-

pression that he is buying RainDOS itself.

3.   RainDOS must be presented as a complete  unit,  in-

cluding this documentation.   RainDOS's  documenta-

tion may be not amended or altered in any way.

4.   In granting you the right to distribute the evalua-

tion form of RainDOS,  Sydex does  not  acknowledge

you as the owner of RainDOS in any form.

Any  other  use  or distribution of RainDOS is expressly

forbidden without the written consent of SYDEX.

________________________________________________________________

"DEC" and "Rainbow" are trademarks of Digital Equipment  Corpora-

tion.   "IBM", "PC DOS" and "Personal System/2" are trademarks of

International Business  Machines  Corporation.    "MS-DOS"  is  a

trademark of Microsoft Corporation.

What is RainDOS?

RainDOS  is a device driver which supports reading and writing of

MS-DOS diskettes from Digital Equipment  Corporation's  Rainbow tm

series computers on PC XT- or AT-compatible machines.   Under DOS

3.2,  the utilities FORMAT,  DISKCOPY and DISKCOMP are supported,

as well as normal DOS read and write operations.

To use RainDOS, your computer must be PC AT-, XT- or IBM Personal

System/2  compatible.    Computers  such as the Tandy 2000 or the

Zenith Z-150 are not suitable because of  their  relative  incom-

patibility.   Further, your computer must be equipped with either

a 5.25" high-density  (1.2  Megabyte)  or  5.25"  "quad"  density

drive,  such  as the Teac 55F.  The diskette adapter on your com-

puter must match the diskette drive characteristics.  That is,  a

high-density  drive  must  be  used  with a high-density adapter.

Many manufacturers, such as Western Digital, produce high-density

diskette adapters for XT-style computers,  and these may be  used

successfully with RainDOS.   Currently, we support only the first

(or primary) diskette adapter in a system.

And finally,  your computer needs to be running PC-DOS or  MS-DOS

Version  2.0 or later.   DOS 3.2 or later is preferable,  but not

required.  Version 2 DOS users may find that some  RainDOS  func-

tions, such as formatting, are not supported.

Installing and Using RainDOS

Installation of RainDOS is straightforward.  First, find the file

CONFIG.SYS in the root directory of the disk that  your  computer

boots (loads DOS) from.   It may be possible that you do not have

a CONFIG.SYS file,  in which case the next few steps will  create

one.   If you are unfamiliar with the CONFIG.SYS file,  take time

out now and read the section in your DOS manual dealing with  it,

and in particular, the DEVICE= statement.

You'll  need to add a line to CONFIG.SYS of the following general

form:

DEVICE=RAINDOS.SYS  /D:unit  /F:format

This can be done with the program EDLIN which  is  supplied  with

DOS,  or with a text editor of your choosing.  If you use a word-

processing editor, make sure that you indicate that CONFIG.SYS is

a nondocument file.   If you didn't find a CONFIG.SYS,  the above

line will be the only line in the file which is created.

The value of unit can range from 0 to 3 and represents the physi-

cal  diskette  unit to be used to read and write the Rainbow dis-

kettes.   If this unit is normally called A:  on  your  computer,

then  the value of unit is 0;  if B:,  then unit is 1.   External

drives are generally units 2 and 3.

Page 1

The value of format depends on the type of  diskette  drive  that

you're using.  If it's a high-density 1.2 megabyte unit, then the

value  of  format is 1.   If you're using a "quad" density drive,

such as the Teac 55F, then format is 2.

If /D:unit is omitted in the configuration line,  unit is assumed

to be 1. If /F:format is omitted, format is assumed to be 1 also.

The  last  step  is  to  copy  the file RAINDOS.SYS into the root

directory of your system or "boot" drive.  Finally,  re-boot your

computer.   If your computer is configured with a revision of DOS

3, the following message will be displayed:

Rainbow diskette driver, Copyright 1990, Sydex

All Rights Reserved.

Rainbow DOS diskette unit x is accessible as drive y:

The letters x and y will take on values which depend on the  con-

figuration of your computer.

If your computer is configured with a revision of DOS 2, the fol-

lowing message will be displayed:

Rainbow diskette driver, Copyright 1990, Sydex

All Rights Reserved.

Rainbow DOS diskette unit x installed.

In this case, the drive letter which is used to read Rainbow dis-

kettes  will  generally be one greater than the last drive letter

in use before you modified CONFIG.SYS.  For example,  if drive C:

was the last drive on your computer,  RainDOS will use drive let-

ter D:.

As an example of the whole process described above,  suppose that

we  wished  to add RainDOS to a PC-AT "clone" which has one high-

density diskette drive and a hard disk.   Our CONFIG.SYS file  on

drive C: might look like this after we modified it using EDLIN:

FILES = 30

BUFFERS = 10

DEVICE = RAINDOS.SYS  /D:0  /F:1    (This line is added)

We then copy RAINDOS.SYS to drive C:...

COPY A:RAINDOS.SYS  C:\

We  then  press  the  keys  Ctrl,  Alt  and Del simultaneously to

"reboot" our computer.   After the  rebooting  process  has  com-

pleted, the following message is displayed:

Rainbow diskette driver, Copyright 1989, Sydex

All Rights Reserved.

Rainbow DOS diskette unit 0 is accessible as drive D:

Page 2

We then find that Rainbow diskettes placed in our A: drive can be

read and written if we refer to that drive as drive D:.

FINAL COMMENTS...

RainDOS  was  written in response to inquiries from customers who

had our CP/M diskette transfer package,  but who had  no  way  to

easily  handle  the  Rainbow  DOS  diskettes.   We hope that this

situation is now remedied.  If you experience problems in getting

RainDOS to work for you, or if you have questions about any Sydex

product, please give us a call.

Page 3

The Sydex Product Line

AnaDisk -- The compleat diskette utility.   Nothing like  it

anywhere else;  scan,  edit,  repair and copy just about any

kind of diskette.  $25.00 ($150.00 site)  registration fee.

Con>Format -- Concurrent  "background"  diskette  formatter.

Features   "pop-up"  operation  and  "hot  key"  activation.

You've got to see it to believe.   Supports all current  DOS

formats. $15.00 ($50.00 site) registration fee.

22DISK -- Transfer files, format, examine and erase files on

"foreign" CP/M diskettes on your PC.   Includes tips on sup-

porting 8" and 5.25"  single-density  diskettes.    Contains

definitions for over 200 different formats.  $25.00 ($100.00

site) registration fee.

22NICE  --  A CP/M 2.2 emulation package.   Supports the NEC

V-series chips or performs emulation by  software  for  both

the  8080  and Z80 processors.   Includes terminal emulation

and diskette handling for common  CP/M  systems.    Includes

22DISK.  $40.00 ($150.00 site) registration fee.

TeleDisk  --  Turn  any diskette into a compressed data file

and vice-versa.   Allows you to send and receive entire dis-

kettes  via  modem.    Even works with some "copy-protected"

diskettes.  $20.00 ($150.00 site) registration fee.

COPYQM -- Mass diskette duplicator.  Format, copy and verify

multiple diskettes from a single  master.    Implements  "no

keyboard"  interaction  mode and drive "round robin" servic-

ing.   Supports all standard DOS formats.    $15.00  ($50.00

site) registration fee.

FORMATQM  --  Mass diskette formatter - format a box of dis-

kettes at a single sitting.  Implements "no keyboard" inter-

action mode and drive "round robin" servicing.  Supports all

standard DOS formats.   $10.00  ($40.00  site)  registration

fee.

...And, soon to be released:

InterDisk  -  The  universal  Diskette  Interchange utility.

Features user-programmable media conversion for  those  for-

mats not covered by 22DISK.  Available 1Q90.

Information  on  any of these products can be obtained from Sydex

by calling (408) 739-4866 (voice) or by leaving a request on  our

Bulletin Board System (408) 738-2860 (1200-9600 bps, N81).

Page 4