Update on 6/20/90 version

-------------------------

References to f000:0000 are erroneous, replace them with ffff:0000.

Rebooting your machine in software [for TURBO-XT 3.01 BIOS]

-----------------------------------------------------------

I have been following a thread on this subject for a while, and got

interested in the experiment.  So, I tried the techniques that were

listed on the thread, and they are: [all numbers in HEX]

call interrupt 19

or

jump to ffff:0000

For either, setting the word at 40:72 to 1234 will cause a warm boot,

otherwise, a cold boot is obtained.

A warm boot means skipping the memory test [an irritating process on

some machines]. A cold boot is what you get when you turn the machine

on.

So, I went ahead an tried calling the interrupt, and my machine got

hung regardless of what I did to 40:72, and I had to use the power

switch. Also, this interrupt call skipped more than the memory test;

i.e.: it also skipped the screen clearing, and the display of the

BIOS manufacturer's logo.

Thus, I went and tried the jmp ffff:0000 alternative, and I got a

cold boot no matter what I did to 40:72.

Finally, I used DEBUG, and disassembled to code at ffff:0000, and

found that it is only a jump instruction to f000:e05b.  At that

location, I found code that clears 40:72 [sets it to 0], and then

resumes control at f000:e066.

Thus, my next experiment was to jump straight to f000:e066 after I

set up 40:72, and this worked fine.  I could choose between cold and

warm boots by setting 40:72 to 0 or 1234 respectively.  On warm

boots, only the memory test was skipped, and the machine never hung.

The programs

------------

Three programs are included: reboot.exe, reb.com, and reb-ta.com.

Reboot.exe is the Turbo Pascal 5.5 prototype that I used to find out

how this stuff worked; it is easily convertible to a unit or a

procedure [source: reboot.pas].

Reb.com is the Chasm [Cheap assembler] implementation.  Reb.asm is

the source.  Note that the jumps into the bios had to be hand

assembled to get the program to work.  Also note that the program is

one byte longer than the Turbo Assembler program [reb-ta.com]; this

is due to Chasm's insertion of segment overrides where none are

necessary (examine the listings around "mov bx, 0040h").

Reb-ta.com is the Turbo Assembler implementation.  It looks more

decent, and I found a way to avoid hand assembling the jumps into the

bios.

The C library

-------------

For the sake of exercise, I have also hacked a C library for the

reboot procedures.  These are in reblib-s.* and reblibl.*, and a

sample tiny C program is used to demonstrate the usage.

Portability

-----------

These programs have been tested on my machine [Turbo-XT 3.01], with

a hard disk controller card.  I do not know whether the techniques I

have presented are immediately applicable to non Turbo-XT BIOS's, nor

the cause of the non-standard behavior [controller or plain BIOS?].

Finally

-------

These programs are released in the Public Domain as reading material.

You are entirely on your own if you want to execute any of these

programs or any of their parts, or if you want to use the information

provided for any purpose.

File list

---------

reb.asm

reb.com       Chasm source, listing, and binary.

reb.lst

reb-ta.asm

reb-ta.com    Turbo Assembler source, listing, binary, and map file.

reb-ta.lst

reb-ta.map

reboot.exe    Turbo Pascal 5.5 source and binary.

reboot.pas

inputil.pas   A Turbo Pascal unit for a simple user interface.

inputil.tpu

reblib-s.asm  Turbo Assembler source for a tiny/small/compact reboot

library for Turbo C programs.

reblib-s.obj  Object form of above.

reblib-s.lst  Listing file for above.

reblibl.asm   Turbo Assembler source for a medium/large/huge reboot

library for Turbo C programs.

reblibl.obj   Object form of above

reblibl.lst   Listing file.

c-reboot.c    C source file to try the reboot library.

c-reboot.exe  Large model executable of above.

c-reb-s.com   Tiny model executable of above.

makefile      Makefile for compiling and linking the C program.

readme.doc    This file.

Contact

-------

Antoun Kanawati

email: tonyk@cowin.ccs.northeastern.edu

or

on Channel-1 [Cambridge, MA]

Programming Languages conference

user name: Tony Kanawati.