Copyright (c) 1988-1991  Thomas G. Hanlin III

A configurable background-operating time display utility

VIDCLOCK keeps a constant time display on the screen.  You can select the

location the time will be displayed, what color it will be, whether it will

be normal or military (24-hour) format, whether the seconds should be

displayed, whether flicker-free video is required, and how often to update

the display.  You can also remove VIDCLOCK entirely from memory if you so

desire.  For help, just type VIDCLOCK without any parameters.

VIDCLOCK.COM works with any computer.

VCLOCK.COM does not work with 8088 or 8086 systems.  It is for computers

which use NEC V20, 80186, 80286, 80386, or 80486 processors.  In return for

this limitation, it is more efficient in operation and requires slightly less

memory (although the file is slightly larger, due to additional installation

code which is discarded when installation is complete).

This utility may be freely distributed according to these rules:

1) VCLOCK.ASM, VCLOCK.COM, VIDCLOCK.ASM, VIDCLOCK.COM, VIDCLOCK.DOC, and

VIDCLOCK.NEW must all be distributed together, in unmodified form.

2) Advertising files MAY NOT be included with the above files.  This applies

to BBS ads in specific.

3) No fee may be charged, other than a disk duplication / handling fee

which may not exceed $6.00.

VIDCLOCK Options:

/I      Install the clock (turns on the clock)

/R      Remove the clock (turns off the clock)

/F      Flicker-free (slow) video, for cheap CGAs

/M      Military time format (24-hour clock)

/S      Seconds off (don't display seconds)

/C=f,b  Color settings for clock.  Foreground and background colors are

selectable, using numbers as follows:

Number    Color system

0         Black

1         Blue

2         Green

3         Cyan

4         Red

5         Magenta

6         Brown

7         White

Add eight to foreground for intense or bright colors.

Add eight to background for blinking colors (not recommended).

For a Monochrome display, use the following pairs:

Numbers   Mono System

7,0      Normal

1,0      Underlined

0,7      Inverse video

/L=r,c  Location of clock.  Row and Column should be specified.  The

upper left corner of the screen is 1,1.  The default is around

1,71 (depending on screen mode and /F, /S options).  It is

recommended that the clock go on the top line of the screen, or

past the 25th line (for EGA/VGA in the proper modes), since

normal scrolling will otherwise tend to messily duplicate the

time value on the screen.

/U=t    Update frequency: how often to update the clock display, in

18ths of seconds.  This may range from 1-255, with the default

at 9 (update every half-second).

General Information:

VIDCLOCK is designed to work with any text-mode screen.  It will work on

Monochrome, CGA, EGA, VGA, and Hercules displays.  Any text mode can be used:

40x25, 80x25, 80x43, 80x50.  Only IBM and compatible machines are supported--

if it doesn't work, your computer isn't compatible enough.  You may be able

to fix this by making sure you have the latest version of your BIOS ROM and a

recent version of DOS (3.0 or later).

VIDCLOCK is a TSR (Terminate and Stay Resident) program.  It takes up a tiny

656 bytes of memory until you remove it or reboot the computer.

The /F option should only be used if VIDCLOCK causes flickering on your

screen (which will only happen on some low-quality CGAs).  It will stop the

flickering, at the cost of some speed.  It may also tend to cause

interference with any serial communications being done.

The /U option controls how frequently the clock display is updated, in 18ths

of seconds.  Smaller values make the clock update more frequent and larger

values make the clock update less frequent.  There probably isn't any reason

to change this from the default value.  If your application erases the clock

frequently, however, you may wish to have the clock updated more frequently.

Note that small values may tend to cause interference with communications.

Large values may result in an unevenly-updated clock display, so you will

probably want to turn off the seconds display (see /S option) if you choose a

value of 18 or above.

Technical Notes:

VIDCLOCK was designed for minimum size and maximum speed.  For this reason,

it uses a number of techniques which might otherwise be shunned, such as

self-modifying code.  Since a clock display like this has the potential of

slowing down the entire computer system, I felt it desirable to avoid the

usual niceties in favor of a more practical approach.

VIDCLOCK correctly handles any normal text screen mode.  It does so on a

static basis, going by what was in effect when it was installed.  Any display

may be used, with any display page (if applicable), in 40x25, 80x25, 80x43,

and 80x50 text modes on MDA, Hercules, CGA, EGA, and VGA systems.  It is

assumed that screen flicker or "snow" is not a problem, since that's true in

most cases, and it makes for much faster displays.  Flicker-free display

techniques can be selected manually if required.  If the screen is placed in

graphics mode, VIDCLOCK will cease updating the display until text mode is

resumed.

VIDCLOCK's copy of the environment space is discarded before installation, to

keep the size to a minimum.  VIDCLOCK can detect whether it has been

installed in a manner resistant to disruption by other TSRs.  When VIDCLOCK

is turned off, it restores the original interrupt vectors and removes itself

from memory.  In operation, VIDCLOCK uses direct machine access for speed and

to avoid the reentrancy problem with BIOS and DOS.

The timer interrupt, INT 8, is used to tell when to update the clock.  This

reduces likelihood of conflict with other TSRs, which tend to use INT 1Ch

for timing purposes.  The actual time is calculated from the BIOS time

data area.

The video interrupt, INT 10h, is hooked to allow VIDCLOCK to update the

screen after any action which may erase the clock.  It isn't foolproof, as

many applications bypass INT 10h, but on the whole it allows for a much

steadier clock display.

Interrupt INT 2Fh, the "multiplex" interrupt, is also hooked in, for use in

determining whether the clock is installed and in removing the clock.  The

VIDCLOCK device identifier is 0DAh.  You can use the following method to see

if VIDCLOCK is installed (assembly language):

MOV   AX,0DA00h     ; query status for I.D. 0DAh

INT   2Fh           ; use multiplex interrupt

;-------------------------

; Returns:

;    AL=0    not installed

;    AL=0FFh installed

Note that INT 2Fh is not always available.  Old computers (say, before 1985

or so) and systems running old DOS versions (before DOS 3.0) may not properly

initialize this interrupt.  In that case, trying to use it will most probably

cause a system crash.  A more robust test would get the vector for INT 2Fh

and test it for 0000:0000, which would be the most likely value if the

interrupt was not initialized.