ANSI.MOD Documentation

May 1989

Version 1.24

Copyright 1988 by John H. Brooks.

All rights reserved.

Permission is granted to quote or

distribute copies of this documentation separately,

and the entire, unaltered POWERKIT II package.

DISCLAIMER: This software is sold "as is."  No warranty is given, either

express or implied, that any specific POWERKIT feature will work on any

particular machine.  The manufacturer will not be liable for any damage

caused by the use of POWERKIT.

The names of hardware and software products, and companies mentioned in

this document are trademarks or service marks of the respective companies.

FORWARD: Welcome, let me tell you what this ANSI.MOD documentation does NOT

contain.  Although ANSI.MOD supports most of the ANSI X3.64 and VT100

"escape" sequences, not all of them will be described here.  Just remember

that ANSI.MOD will do the job for you, should you have an application pro-

gram that requires VT100 emulation, or ANSI.SYS be loaded.  Also, ANSI.MOD

has many helpful routines that can be used by other programs, and these

will not be mentioned here either.  Instead, for those of you who might

wish to write your own "module", or to delve into the esoteric mysteries of

ANSI "escape" sequences, there is a technical reference manual available to

all registered users of POWERKIT II.

I have placed the "module" format for executable TSR (and device driver,

ANSI.MOD is the console device driver*) files in the public domain, and

any software developer, commercial or shareware, may use it, free of royal-

ties.  You may also take advantage of ANSI.MOD's superior routines in your

programs.  There are some restrictions, and you MUST contact me for

approval, and for the specifics of this offer.

WHAT IS ANSI.MOD?  ANSI.MOD performs three major tasks (the keystroke buffer,

with view-port, is a separate program).  First, it is a replacement for

the rather inadequate MS/PC-DOS console that controls your screen and

keyboard.  The DOS console's inadequacies are the reason every major

application package handles its own I/O to/from the screen and key-

board.  Since each program handles this I/O a bit differently, you can

* - For you "techies", ANSI.MOD (when active, and it's simple to turn it on and

off) is the highest "CON" link in the device driver chain, is carried in

the "list of lists" as the console driver, takes over INT 29h duties, and

is the "file" that answers to "handles" 0, 1, and 2.

Page 2

blame the poorly designed DOS console for the fact that some of your

favorite pop-up TSR's (terminate, but stay resident programs) don't

work inside some of your applications, and why some TSR's don't work

very well with other TSR's.  Thus, a major goal of the POWERKIT II

design is, at least, to get a special class of TSR's, called "modules",

to COEXIST amicably.  To this end, ANSI.MOD provides a set of keyboard

handling and screen updating routines, that are quite simply the best

in the business.  ANSI.MOD can replace an entire 80x25 screen, 40 times

a second, on a standard 4.77 Mhz PC.

Second, ANSI.MOD is a replacement for the equally laughable ANSI.SYS

that came in your DOS package.  It has a far larger set of the ANSI

X3.64 and VT100 escape sequences, plus a few that ANSI never heard of.

The commands represented by the escape sequences work under much more

diverse conditions than do the corresponding ANSI.SYS commands, and in

many instances, the same command does many more things than its ANSI-

.SYS counterpart.

To cite an example, the old ANSI "keyboard redefine" sequence did

indeed replace one character by one or more other characters, giving

ANSI.SYS a "MACRO" capability, sort of.  But, the key redefinition

acted ONLY at the DOS prompt (well OK, in DEBUG and EDLIN too).  When

you entered an application, the key reverted to its former self.  ANSI-

.MOD produces true MACROS that are available at the touch of a key, all

the time.

Furthermore, you can attach a comment or description to each of your

MACROS, to remind you of some important fact about them.  There is a

way provided to show this comment, and also a means to move this MACRO

to a different key (you'll see why later).

Another feature unique to ANSI.MOD's MACROS is that ANSI escape

sequences can be embedded in them.  Thus, MACROS can be used to com-

mand your communications port (dial the phone, for example), or

printer, or even to define other MACROS (or delete!!, even themselves).

Finally, MACROS can be defined at any time, because ANSI.MOD pro-

vides "hot" key access to the ANSI sequence parser, replacing "PROMPT

$e [" (ALT-ESC is the default, and FYI, all hot keys for POWERKIT II

modules are just special types of MACROS, the help screens for the mod-

ules are the above-mentioned comments, and the hot key can be moved

just like any other MACRO).  The old, unhandy "PROMPT $e [ ... " method

is still available for batch file use, etc (but don't put "ECHO OFF" in

that batch file, use CTTY NUL, then CTTY CON to restore the display).

Oops, I almost forgot, an ANSI.MOD MACRO can contain sub-macros

(to 10 levels deep), and these are expanded when the outer MACRO is

being called, not when the outer MACRO is being defined.  This not

only saves space in the MACRO buffer, but the sub-macro(s) can be

ALTERED between calls to the outer MACRO.  I'm sure you will discover

many uses for this sophisticated capability.

Speaking of space, ANSI.MOD MACROS use only a single byte for most key-

strokes, except "extended" ASCII codes (these are two byte codes whose

first byte is 0, or E0h).  "Problem" codes (such as control codes) are

handled in software.  This results in a significantly larger program,

but the reward is, ANSI's buffer will hold almost twice as much inform-

ation, in the same space, as some other "KEY" or "ANSI type" programs.

Page 3

This brings us to a second important goal of POWERKIT II, STANDARD-

IZATION!!!.  All good word processors, spreadsheets, data "basers",

provide many of the same helpful operations, like SEARCH, REPLACE,

COPY, MOVE, DELETE, to name a few.  The only trouble is, they each use

different keystrokes to accomplish these tasks, and YOU have to REMEM-

BER which keystroke combination goes with which program.  NO MORE, just

create a batch file for each of your programs.  The batch file will

create MACROS that move these operations to the same keys in all the

programs -- call the program -- then "uncreate" the MACROS (yep, I

forgot that too, you can delete MACROS) after exiting the program.

Lastly, ANSI.MOD is the traffic cop that mediates potential conflicts

between POWERKIT II modules, recognizes hot keys and calls the associ-

ated module, disables one when another pops up on top of it (to 4 lev-

els deep), interprets their "command" keys for them and calls the

proper routine, sends them keystrokes when asked, pops up their help

screens when F1 is pressed, provides many other useful routines (PKII-

.MOD is several hundred bytes smaller because of these routines).

Now for perhaps THE most AMAZING fact about ANSI.MOD.  In its smallest

configuration, with a 256 byte MACRO / ANSI sequence buffer, ANSI.MOD

requires slightly over 5k bytes! and this leads us naturally to a third

important goal of the POWERKIT II design, SAVING SPACE !!!  The space

savings provided by the remarkable module design is described in

PKII.DOC, but ANSI.MOD also plays its part.

The three goals mentioned above are intermediate to the FINAL goal,

which is to SAVE MS/PC-DOS!!! (from itself, mainly).  It's just too

LOUSY an operating system not to need a great deal of help to survive.

And survive it SHOULD; consider the alternatives; multi-megabyte,

multi-user, multi-tasking UNIX and OS/2!  Sure, they're powerful oper-

ating systems, but I question whether even most "power users" need

them.  What I believe IS needed is a communications program, a smallish

one, that will operate in the background, to process your Email, UL

and DL files, and such (MIRROR II does a good job, but consumes 150k+,

and THE INVISIBLE LINK has the right idea, does a partial job in only

29k of memory, could use more fine tuning); a print spooler to drive

your printer in the background (PRINT comes with DOS, and is one of the

best there is); a small, not too sophisticated foreground/background

dual-tasker to handle a compute-intensive process (like a spreadsheet

re-calc) in the background, and an I/O intensive task (like a word pro-

cessor) in the foreground; ANSI, PKII.MOD, and a few other tiny, inex-

pensive, indispensable, POWERKIT II modules to help you cope with DOS.

Well, I've spoken my piece, now let me hear from YOU (please send money

too).  Does this approach to Personal Computing make any sense at all

to you?

INSTALLATION: Though it is far from being "just another module", ANSI.MOD can

be loaded (and unloaded) like all other modules, by typing:

LOAD C:\PKII\ANSI.MOD (instructions to ANSI.MOD may go here)

Page 4

Of course, this assumes you have previously created the \PKII directory

on the "C" (or "A") drive, and copied ANSI.MOD into it.  The easiest

way to do this is to run POWERKIT II's "INSTALL" program (see the in-

structions in README.1ST).

IMPORTANT NOTE:  ANSI.MOD MUST BE LOADED BEFORE MOST OTHER MODULES !!!

ANSI.MOD has two switches that may be set at load time (if you don't

like the default values, otherwise don't do a thing).  For example:

LOAD C:\PKII\ANSI.MOD /m100/j0

^ blank space

will set aside 25,600 bytes for MACROS and ANSI sequence parsing

(the default is 2048 bytes), and eliminate ANSI.MOD's help screen.

* /Mn, n = 1 to 237 (approx.), tells ANSI how many 256 byte sections of

memory to reserve for MACROS and ANSI escape sequence parsing.

* /Jn, n = 0 or 1, turns ANSI.MOD 's help screen off or on.

NOTE: You must reserve space, in ANSI.MOD's MACRO buffer, for module help

screens.  If the screen for PKII.MOD is attached, in /Mn, n must be 4,

or higher.  ANSI comes to you with the default set at 8 (i.e., 2k of

MACRO buffer space), so there's no need to worry when you first load

ANSI.MOD and PKII.MOD (2k will hold both help screens).

ANSI COMMANDS: All ANSI escape sequence commands begin with the "escape" char-

acter, which is a decimal 27.  The next character is almost always a

left bracket (a "[").  The real "meat" of the command then follows

those preliminaries.  As was stated earlier, ANSI commands can be ac-

tivated from the keyboard, at any time (not just at the DOS prompt), by

pressing ALT-ESC (press the "ESC" key, while holding down the "ALT"

key).  ALT-ESC acts as the "27" and the"[".  The "meat" is either a

series of numbers, separated by semicolons (like 1;2;33;44), or a se-

ries of characters, enclosed by single or double quotes (like 'abc' or

"abc"), plus a final upper or lower case character (must be outside of

any quote marks).  That final character of the sequence tells the ANSI

command interpreter (hereafter referred to simply as ANSI) what to do.

MACRO RELATED COMMANDS: To a Greek (and a select few others), "macro" means

"large".  In computerese, a MACRO is a string of one or more charac-

ters, that are sent to a display, a file, printed on a printer, or sent

to a modem, after a (single) key is pressed.  At this very moment, I'm

using MACROS to enter the words "ANSI.MOD" (I press ALT-2), "ALT-ESC"

(I press ALT-3), and "MACRO" (I press ALT-4), into this document.

CREATE A MACRO: The final character (mentioned above) that tells ANSI to

create a MACRO is lower case "p" (upper case "P" means something en-

tirely different to ANSI).  Creating a MACRO with ANSI.MOD is almost as

easy as (or easier than) it is with those well-known (and costly, in

PRICE and MEMORY) "KEY" and "ANSI" programs:

The "quote" key is the key between the semicolon/colon key, and the

ENTER (RETURN) key.

Page 5

1. Press ALT-ESC (you should hear a beep)

2. Press the "quote" key (shifted or not)

3. Press the key to which the MACRO will be attached

4. Press the sequence of keys you wish the redefined key to represent

5. Press the "quote" key again (shifted or not, but the same as in step

two above)

6. Press lower case p

That's all there is to it.  You will notice ANSI's view-port will open

to show you the keystrokes you are typing (well not quite, it cannot

show non-printing codes).

If you need one or the other of the "quote" keystrokes in your MACRO,

use the other 'quote' as the delimiter.  If you need them both, end

one "quote" sequence, press ";", press the other 'quote', press the

"quote" you need, and continue.

If you wish an ANSI escape sequence to be part of your redefinition,

press ALT-ESC, followed by the ANSI sequence.  Make sure you are

inside the "quote" delimiters (see note below). Outside "quote"

delimiters, you must use "0;1" to represent "ESC [".

IMPORTANT NOTE: Pressing ALT-ESC a second time ABORTS an ANSI escape

sequence, but not if you are inside the "quote" delimiters.

DELETE A MACRO: To delete a MACRO, press ALT-ESC, "quote", the key to

which the MACRO is attached, "quote", then lower case p.  This causes

the key to revert to its original function.   Sorry, the only way to

delete all MACROS, is to UNLOAD, then re-LOAD, ANSI.  The reason for

this, is that all module "hot" keys are MACROS too (including ANSI.-

MOD's), and it wouldn't be wise to delete them.

MOVE A MACRO: To move a MACRO to a different key, press ALT-ESC, "quote",

the "old" key, the "new" key, "quote", then lower case k.  Since module

hot keys are MACROS, this procedure will also move a module's hot key.

MACRO DESCRIPTION: You can include a description of what the MACRO does,

when you define the MACRO, if you wish.  After defining the MACRO, and

pressing the "quote" key, but before pressing "p", press the "left

single quote" key, enter your description, press the "left single

quote" again, then press "p".  Include carriage returns (the ENTER or

RETURN key), when you want to start a new line, in your description.

SHOW DESCRIPTION: To look at this description, press ALT-ESC, "quote", the

MACRO's key, "quote", then lower case q.  The description will pop-up

on the screen, then disappear as soon as you press another key (the

SHIFT key, for example).

NOTE: This can be used to produce helpful information for you at the touch

of a key.  Create a "view-only" MACRO by putting the text you would

like to see, in the description field, the MACRO's activation key, en-

closed in quotes, then lower case q, in the execution field of the

MACRO.  Now, when this MACRO's key is pressed, the text will pop up on

the screen, and disappear when the next key is pressed.

TURN MACROS OFF/ON:  Press ALT-ESC, "0", lower case "t" to disable all MAC-

ROS, except ANSI.MOD's hot key.  ALT-ESC, "1", "t", turns them back on.

Page 6

IMPORTANT NOTE: The fact that ANSI.MOD waits until an outer MACRO is being

called, before expanding sub-macros, creates a problem for which there

is no completely satisfactory solution.  Consider this: Suppose you at-

tached the MACRO "a" to "a".  In other words, define the key "a" to be

itself.  When "a" is pressed, ANSI.MOD will look at it, decide that it

was redefined, and look up its redefinition.  The characters in the re-

definition will then be checked to see if any are themselves redefini-

tions, AND DISCOVER THAT "a" IS.  What has been created, is an infin-

ite loop.  To stop this, ANSI.MOD checks the stack of sub-macros cur-

rently being expanded, and if it finds a sub-macro already on the stack

that is the same as the one being checked, it will NOT expand the one

being checked.

FAST SCREEN MODE: ANSI.MOD is far from being a slow poke in putting characters

on your screen.  But sometimes, it can be even faster.  DOS does some

checking between sending characters to the screen.  For instance, DOS

checks for CTRL-C, which tells DOS to stop doing whatever it is doing.

This checking can be turned off, thus speeding up screen writes.  True,

you will lose CTRL-C and CTRL-BREAK, but PKII.MOD's KEYPAD 5 (or SCROLL

LOCK), ESC will still get you out of trouble.

To enable ANSI.MOD's fast screen update mode, press ALT-ESC, then 1,

then lower case "r".  To disable this mode, press ALT-ESC, 0, lower

case "r".

CHANGE TEXT AND GRAPHICS MODES: Press ALT-ESC, n, lower case h, where n is the

number of the text or graphics mode you wish to use.  I use this mostly

when some ignorant programmer has decided he knows more than I, about

what mode I want MY display in, puts MY display into 80x25 color mode,

when I want it to be in 80x25 Black & White.

Note that n = 7 is not a text or graphics mode, but rather a "wrap"

mode.  If you don't wish characters to "wrap" at the end of a line,

press ALT-ESC, 7, lower case L (l looks like 1).  The option, n = 43,

is not part of ANSI, but if you have an EGA (or VGA), it will put your

display into an 80x43 (50 for VGA) text mode.

The possible values for "n" are:

0 - 40x25 black & white

1 - 40x25 color

2 - 80x25 black & white

3 - 80x25 color

4 - 320x200 color graphics

5 - 320x200 black & white graphics

6 - 640x200 black & white graphics

7 - Automatic CR LF at end of line

8 - 160x200 color graphics (PCjr and Tandy 1000)

9 - ?

10 - ?

11 - ?

12 - ?

13 - 320x200 EGA color graphics

14 - 640x200 EGA color graphics

15 - 640x350 EGA B & W graphics

16 - 640x350 EGA color graphics

Page 7

17 to 42 - ?

43 - Forty-three (50) line TEXT mode for EGA (VGA) (text only)

? - Means "perhaps" that number does something nice to your machine.

If not, entering it may cause your machine to LOCK UP.

CHANGE SCREEN COLORS, ETC.: Press ALT-ESC, P1;P2; ... ;Pk, lower case m, where

the Pi's are numbers listed below.  In other words, you are able to

select more than one of the attributes listed below, in the same call

to ANSI.  Put semicolons between each number.  Again, this is used when

some stupid application messes up your screen.  It always works after

you exit the program, but some programs won't permit you to change

colors while inside them.  Note that option 9 lets you supply an attri-

bute byte, thus can do the work of (almost all of) the other options.

no param - makes current screen (low intensity) White, on Black

0 - return to White on Black background, no Bold, no Blink

1 - high intensity (foreground and border only)

2 - normal intensity, no color change, just dimmer foreground

4 - underline (monochrome adaptors only)

5 - blinking (in graphics modes, characters are XOR'ed)

7 - reverse video (XOR background & foreground, blink, bold

unchanged)

8 - concealed characters (foreground = background, in color)

9 - The number entered after the 9 replaces current attribute

byte (background & foreground colors, blink, and intensity)

*     10 - Turn off cursor in graphics modes (this is the default)

*     11 - Turn on cursor in graphics modes (VERY SLOW, use sparingly)

*     12 - Turn off (detach) ANSI.MOD.  MACROS, and many other things,

still work.  ANSI.MOD can still support the other modules.

*     13 - Turn ANSI.MOD back on.

21 - Remove boldness

22 - Remove blinking and boldness (like option 0, but it works

on color screens)

24 - Remove underlining (MDA only)

25 - Remove blinking

27 - Reverse video (same as option 7)

28 - Remove concealed characters (restore foreground)

foreground colors

30 - black

31 - red

32 - green

33 - yellow

34 - blue

35 - magenta

36 - cyan

37 - white

38 - ERROR

39 - ERROR

* - NOTE: At release time (1/31/89), options 10 through 13 sometimes had to be

entered twice before they "took".

Page 8

40-47 - Background colors (in same order as 30-37)

48 - ERROR

49 - ERROR

50-81 - Border colors (in same order as 30-37, except 51-blue,

53-cyan, 54-red, 56-yellow.  58-65 are high intensity.)

COPY TO PRINTER OR MODEM: Use ALT-ESC, n, i, where n is a number listed below.

These commands will usually be used in MACROS.  For example, suppose

your boss is too cheap to buy you a demon dialing telephone, and you

MUST call your stockbroker quickly and often.  If your modem has auto-

dial capability, you can set up a MACRO to do this.  Let's attach this

MACRO to F10.  Press ALT-ESC, "quote", F10, ALT-ESC (again), 2, i,

ATDT5551212 (or whatever your broker's number is), ALT-ESC (yes,

again), 6, i, "quote", p.  Note: If your boss is too cheap even to

install touch tone lines, change the "ATDT" to "ATDP".

What did we do here?  The first ALT-ESC gets ANSI's attention.  The

"quote" mark says that characters, rather than numbers, are to follow.

F10 is the MACRO's key.  The characters before the next "quote" will be

explained later.  The "quote" marks the end of the characters, and the

"p" tells ANSI to attach those characters to the F10 key.

Now here's what happens when you press F10.  ALT-ESC starts the ANSI

sequence to turn on copy to the modem.  The "2" selects the modem.  The

"i" completes the ANSI sequence.  From here on, everything that would

normally go to the screen, goes to the modem instead.  Thus, the "ATDT-

5551212" is sent to the modem, and the modem dials the number.  The

final ALT-ESC alerts ANSI again; the "6" deselects the modem; the "i"

completes the command, and ANSI directs output back to the screen.

The possible values for "x" are:

0 - Copy to printer (LPTn)

2 - Copy to modem (COMn)

3 - Copy from modem (COMn)

4 - Turn off copy to printer

6 - Turn off copy to modem

7 - Turn off copy from modem

x = 8 thru 11 - Select LPTn & COMn, n = x - 7 (i.e., 9 selects LPT2 & COM2)

FAKE KEYSTROKES: How many times have you wished you could send instructions to

an application you call from a batch file?  Now you can.  In the batch

file that calls the application, on a line before the call, put:

prompt $e["instruction 1";13;"instruction 2";13; ... etc. ... ;v

Those "13"'s are carriage returns to separate the individual

instructions (sometimes not needed).

STANDARDIZATION: Suppose you have two word processors, one named "EDITOR", and

one named "ED".  One you use to dash off notes and memos, the other,

for more serious work (not that your memos aren't serious).  For the

sake of simplicity, suppose the two have identical command structures,

except that ED uses F8 to call up the "search" capability, while EDITOR

uses F9.  To get both programs to use the same keys for the same opera-

tions, you must call one using a batch file.  Let's call ED using the

batch file, EDIT.BAT.  Use either ED or EDITOR to create the following

EDIT.BAT, batch file (note the CTTY command, in place of ECHO OFF.

ECHO OFF disables the "PROMPT" command):

Page 9

ctty nul             #- diverts output from display

prompt $e[0;67;0;66p #- turns F9 into F8

ed                   #- calls the editor

prompt $e[0;67p      #- after ED finishes, turns F9 back into F9

prompt $n$g          #- resets your "DOS prompt"

ctty con             #- sends output back to display

Now, when you wish to use ED, just type EDIT, and the F9 key will call

the search facility.

Perhaps some of you are thinking "what's this screwy `prompt $e[' no-

tation?"  Well, it's the screwy way DOS has provided you to access

ANSI.  I'll bet you recognized the "p" at the end of the first line,

and guessed that a MACRO was being defined there.  The "prompt $e[" is

DOS's way of sending an escape (27, remember?),and a "[", to ANSI.  The

"0;67" is the (extended) ASCII code for the F9 key, and "0;66" the

ASCII code for the F8 key.  They have to be represented in this awkward

manner (when using the PROMPT command only), because they are nonprint-

ing characters (for printing characters, you would simply put in a

"quote", then the characters, then another "quote").   Thus, the first

line of EDIT.BAT turns F9 into F8.  The second line calls ED.  The

third line isn't executed until you have exited ED, and it deletes the

MACRO attached to F9.  The fourth line tells DOS to make a prompt

consisting of the current drive letter, followed by the symbol ">"

(look familiar?).

The following does the same thing, but without using "prompt ...", and

also suppresses the irritating display of "echo off" when the batch

file starts.  Most editors will let you hold down ALT, then press

keypad "2", then "7", to make the "#" (escape) character:

#[8m              #- make foreground = background (and save foreground)

echo off          #- suppress display of batch file actions

echo #[28m        #- restore foreground color

echo #[0;67;0;66p #- turns F9 into F8

ed                #- calls the editor

echo #[0;67p      #- after ED finishes, turns F9 back into F9

echo on           #- restore display

This method has the added advantage of not destroying your DOS prompt.

I hope this (admittedly oversimplified) example gives some idea of how

to lessen the memory circuit overload many of us endure, because every

program we own does the same thing differently.  Most DOS manuals have

a table of ASCII codes in the back, for you to refer to in this endeav-

or.  Yes, these ANSI sequences are kind of tough to construct, but you

need to create a batch file only once, for each of your most-used

programs.