SCROLOCK program and documentation Copyright (c) 1990-91 Daniel

Schwartz

SCROLOCK.COM (v1.3) is a small memory-resident (TSR) utility

for IBM and compatable PC's that does one thing: makes the

SCROLL LOCK key functional.

It does this by intercepting the video BIOS interrupt

(INT 10h), checking the status of the SCROLL LOCK key when-

ever it is called.  If the key is not engaged, processing

continues as usual.  If the key IS engaged, the system will wait

until it is released before continuing.  Only the foreground

application is halted by SCROLOCK, and only when it is updating

the screen.

To activate SCROLOCK, simply type SCROLOCK at the DOS

prompt, or include the line in your AUTOEXEC.BAT or other batch

file.  If you do not wish the sign-on message to be displayed,

you may use the form SCROLOCK >NUL to suppress it.  This will not

result in the loss of a "file handle".

SCROLOCK is useful in particular with programs and languages

(such as GWBASIC) that do not recognize a "control-S" keypress to

halt a long screen listing.  The only way to pause a long BASIC

listing is normally to use the PAUSE key.  This often has the

undesirable effect of pausing a screen scroll halfway through,

causing one line to appear twice, generally with part of the

preceding line attached to it.  Because of the way it works,

SCROLOCK will always pause the screen BETWEEN the output of two

characters, not DURING the output of a single character (such as

a carraige return, which in BASIC can scroll the screen),

eliminating this potential source of confusion.  (Note: a similar

utility from PC Magazine DOES sometimes freeze the screen in the

middle of a scroll.  It also disables the LED indicator for the

SCROLL LOCK key; this program leaves the indicator active.)

This version of the program, unlike a previous release, can be

safely used in BASIC even while PEN ON event trapping is in

effect.

This utility represents my first attempt at "publishing" a

program.  Feel free to use it and to give copies to whomever you

wish, but please also include this documentation file with it.

If you upload it to a BBS system, upload the two files

(program and documentation) together as a .ZIP or .ARC or similar

combined file.

I do not DEMAND that you pay me money if you find the

program useful, but I would APPRECIATE it if you could contribute

$5.00 or so, or whatever you think the program is worth.  It

would certainly encourage me to attempt other, more ambitious

projects in the future.

Send any contributions to:

Daniel Schwartz

75 E. 190 St. #1D

Bronx, NY 10468

Note: the program should be loaded only once during a session at

the computer.  If you invoke it a second time, you will just eat

up another 224 bytes or so of memory for no good purpose.  You

can use a TSR manager like INSTALL/REMOVE or MARK/RELEASE or

POPDROP disable the program and reclaim the small amount of

memory that it used.