EMS 4.0 DOCUMENTATION

The EMS (Expanded Memory Specification) Version 4.0 device

driver is a standard MS DOS device driver that gets loaded at boot

time by the CONFIG.SYS file. Because EMS 4.0 is a device driver, it

should look like this in the CONFIG.SYS file:

DEVICE = EMM.SYS

Without anything else included in this command line, the EMS

driver will get loaded with the default hardware values of the I/O

address that enables the EMS and the memory address used for EMS

paging. These default values are dependent on the NEAT implementation

of EMS.

To change these default values, as well as add a few extra

features, the EMS device driver command line can look like this:

DEVICE = EMM.SYS -Ix -My -Pzz -D

where,

I = I/O address to enable EMS pages

x = 0 ==> 208h

= 1 ==> 218h

= 5 ==> 258h

= 6 ==> 268h

= A ==> 2A8h

= B ==> 2B8h

= E ==> 2E8h

M = page frame address (address of 64K window)

y = 1 ==> C0000h

= 2 ==> C4000h

= 3 ==> C8000h

= 4 ==> CC000h

= 5 ==> D0000h

= 6 ==> D4000h

= 7 ==> D8000h

= 8 ==> DC000h

= 9 ==> E0000h

P = maximum number of open processes

(defaults to 64)

zz = number of processes (64-256)

D = enable EMS diagnostics

The command line parameters can be in either upper or lower case.

When the EMS driver is first executed, a title and copyright notice

will appear on the screen as follows:

" EMS Expanded Memory Device Driver  Ver. 4.0 "

" Copyright (c) Chips and Technologies inc., 1987"

If the EMS driver detects that a previous EMS driver has been loaded, it will

put this message on the screen:

" An Expanded Memory Manager has already been installed. "

The EMS driver will then not get loaded into the system.

When the EMS driver gets loaded by MS DOS, an internal check is

done on the EMS configuration in hardware. If the driver detects any

conflicts that would cause EMS to not function properly, the message:

" The EMS setup has been incorrectly specified. No EMS is available. "

will appear on the screen after the EMS copyright notice and EMS will be

disabled. If the EMS hardware is configured properly and the I/O address

is overridden by the command line parameter, the screen will show this

message:

" The EMS I/O address has been changed. "

If the EMS page frame address is being overridden from the command line,

the following message will appear:

" The EMS Frame Segment has been changed. "

If the user specifies an invalid parameter(s), the default values, set up by

the BIOS or the setup program, will be used.

If the diagnostics option is selected, this message will appear on the

screen:

" Testing EMS Expanded Memory Page Number: NNN "

The NNN represents the page number currently under test. If the diagnostics

fail, the EMS will be disabled and the following message appears on the

screen:

" Expanded Memory FAILED diagnostics test. "

If the diagnostics pass, or if diagnostics were not specified on the command

line, the EMS driver has been loaded correctly and EMS is enabled. The final

message to appear on the screen is:

" There are XXX pages, or YYYY Kbytes of EMS Expanded Memory on the system. "

The system can now be used to support Lotus/Intel/Microsoft EMS 4.0

in the same manner as standard EMS cards.