UMB_DRVR.DOC	   Version 5.0		    09/01/91		 Page 1

UMB_DRVR.SYS Device Driver

UMB provider for DOS 5.0

Copyright (C) 1991, All Rights Reserved

Christopher Blum	     CompuServe: 76625,1041

1022 East Wayne Avenue		    BIX: cblum

Wooster, Ohio 44691

IMPORTANT INFORMATION

This program carries no warranties expressed or implied.  It is

solely up to the user to determine its suitability for use on his

machine.

This program is made available on a 'try before you buy' basis.

It is not crippled in any way, has no 'advertising', and future versions

will be made available via these same channels as they are developed.

Personal use license is $10.00 ( U.S. funds ) which should be

mailed to the above address if the program is used after a reasonable

trial period.

Corporate users must contact me for corporate rate or site license

arrangements.

This program is distributed as a self-extracting file containing

the device driver and its associated documentation. Copying and

redistribution is encouraged, but must be the original, unmodified

file containing this documentation, and the transfer must not carry

any fee or charge specific to this program: i.e. general BBS access

or line charges are OK, but no 'download fee' or similar charge. This

means that BBS operators may post this file for download, but

'Distribution houses' which charge for copies of disks are expressly

forbidden to distribute without prior written permission from me.

The most current version will be available on CompuServe in the

IBM forum ( 'go ibmsys' ).

The best way to contact me is via CompuServe Email ( EasyPlex )

as 76625,1041 - I monitor my mail almost every day, and I will try to

respond to requests for assistance as best I can. Do NOT leave me

messages in a forum. I do not regularly scan them, and it may never

reach me. I also monitor my Email on BIX - address it to 'cblum'.

The U.S. Postal Service is also OK ( address above ), but not as timely.

UMB_DRVR.DOC	    Version 5.0 	     09/01/91		  Page 2

INTRODUCTION

UMB_DRVR.SYS is a DOS 5.0 device driver that will use the shadow

memory capability of the memory controller portion of various chip sets

to:

A) expand DOS base memory beyond 640KB if possible, and

B) provide UMBs ( Upper Memory Blocks ) to DOS for loading

programs and device drivers into 'high memory'

while NOT using ANY resident memory ( mapped out or otherwise ).

A by-product of this is that many if not all other device drivers

and resident programs may be loaded 'high' including HIMEM.SYS and

EMM386.EXE ( even though the DOS documentation says not! ). Also,

device drivers that cannot be loaded high when EMM386.EXE is providing

UMBs via the 'ram' or 'noems' parameter, due to their use of DMA

transfers, will probably work with UMB_DRVR.SYS ( EMM386.EXE remaps

ram using Virtual-86 mode and the memory management on the 386+ chips,

and the virtual address is not the same as the real address for the

area mapped as UMBs, whereas UMB_DRVR.SYS uses the hardware external

to the cpu, remaining in real mode with virtual=real mapping ).

Another advantage is that programs that must be able to use protected

mode, such as Borland's Turbo Debugger in 386 mode ( TD386 ) can

operate as intended.

CURRENT CHIP SET SUPPORT

## Name

** *************************************************************

01 Chips and Technologies NEAT chip set for 286/386SX

** *************************************************************

If your chip set is not in the preceeding list, I can include

support for it if you can supply me with documentation ( e.g. hardware

spec sheet ) on the registers, features, etc. it has. New versions of

the program will be posted as other chip sets are supported.

INSTALLATION

The driver is installed ( preferably as the first driver ) using

the CONFIG.SYS line:

DEVICE=UMB_DRVR.SYS /C=nn [/M=ssssssssssssssssssss]

The /C= parameter is required - nn is the chip set from the list above.

UMB_DRVR.DOC	    Version 5.0 	     09/01/91		  Page 3

The /M= parameter is optional and is used to override default

action. The contents of the /M= parameter are positional characters

corresponding to 16K memory blocks at the following addresses:

/M=ssssssssssssssssssss

|||||||||||||||||||`EC000

||||||||||||||||||`E8000

|||||||||||||||||`E4000

||||||||||||||||`E0000

|||||||||||||||`DC000

||||||||||||||`D8000

|||||||||||||`D4000

||||||||||||`D0000

|||||||||||`CC000

||||||||||`C8000

|||||||||`C4000

||||||||`C0000

|||||||`BC000

||||||`B8000

|||||`B4000

||||`B0000

|||`AC000

||`A8000

|`A4000

`A0000

s = '.' Allow default use of this memory block

'-' Force this memory block to be unused by the driver

'+' Force this memory block to be used for UMB's or

to expand DOS memory > 640K ( as appropriate )

The /M= parameter must always be supplied as all 20 characters, using

the '.' character to fill any positions not forced on or off. For

example, on a VGA system using video memory from A0000 to BFFFF, if

it is in CGA 80 x 25 mode, the only video memory in use is B8000-BFFFF.

In this case, the memory from A0000-B7FFF may be used to expand DOS >

640K by using the parameter:

/M=++++++..............

Of course, with the system configured like this, if you change the video

mode, undefined ( read as disaster city! ) results will occur.

A second example would be a system where you wish to use EMM386.EXE

to supply EMS memory and wish to use E0000-EFFFF as the EMS base address

range. To accomplish this, use the parameter:

/M=................----

to reserve this memory for EMM386.EXE to use. You will also need to use

the I= parameter for EMM386.EXE to include E0000-EFFFF for its use. Do

not use the 'ram' or 'noems' parameters for EMM386.EXE - these cause

it to remap memory to supply UMBs and we already have done that!

UMB_DRVR.DOC	    Version 5.0 	     09/01/91		  Page 4

DRIVER OPERATION

The driver must be installed BEFORE HIMEM.SYS is installed. It is

an XMS 2.0 server providing UMBs to DOS via that standard. The chip

set parameter is processed and the proper routine called to remap the

unused shadow ram to DOS-useable memory. Available memory starting at

A0000 is used to expand DOS base memory beyond 640K, and other available

memory ( i.e. above the video memory ) is used for UMBs ( the areas

DOS uses for DEVICEHIGH or LOADHIGH commands ).

The driver by default will not use any areas used for video memory

or ROM areas ( shadowed or not ). On the C&T Neat chip set, it will

also recognize whether EMS hardware support is active and will not use

the EMS base area.

The driver should be loaded as the first driver if possible. This

allows following drivers and resident programs to be loaded high - even

HIMEM.SYS and EMM386.SYS ( DOS documentation says they can't, but it

works! ). It will initialize, supply UMBs, and go away leaving no

resident memory anywhere.

Appropriate status and error messages are issued during processing

and a map of the driver's action is displayed.

SYSTEM SETUP EXAMPLE

One of my systems is a 386SX using the Chips and Technologies NEAT

chip set. It has 4MB of RAM and a Hercules Monochrome Graphics card.

I configure 2MB of EMS memory with a base address of E0000-EFFFF using

the NEAT hardware and their supplied software driver RMS386N.SYS, and

shadow the BIOS at F0000-FFFFF. I load DOS into the HMA using HIMEM.SYS,

and also load Borland's Turbo Debugger device driver TDH386.SYS for 386

virtual mode ( TD386 ) and ANSI.SYS. My CONFIG.SYS looks like this:

BREAK=ON

FILES=20

BUFFERS=40

STACKS=0,0

SHELL=C:\DOS\COMMAND.COM C:\DOS\ /P

DEVICE=C:\UTILITY\UMB_DRVR.SYS /C=01

DEVICEHIGH=C:\DOS\HIMEM.SYS

DEVICEHIGH=C:\BORLANDC\BIN\TDH386.SYS

DEVICEHIGH=C:\UTILITY\RMS386N.SYS

DEVICEHIGH=C:\DOS\ANSI.SYS

DOS=HIGH,UMB

This gives me 704K base memory for DOS, a maximum executable program

size of almost 690K, and 128K in one UMB at C0000-DFFFF with over 95K

still free in that upper memory block for a mouse driver, network

driver, TSR, etc. I consider that a pretty good cure for 'RAM cram'!