The file OLYMPIAS.FXF, used together with FX.COM, allow the symbol wheel of

the Olympia Compact 2 (wheel number 787-304) to be emulated on the Epson FX

printers. This allows a large report containing mathematical symbols to be

printed on the FX for proofreading, before being printed on the Olympia

(which takes forever!).

The following differences between the Olympia and the FX will exist after

OLYMPIAS.FXF is downloaded.

- The alternate character set is activated and deactivated on the Olympia by

changing type wheels; on the FX, by the ESC % 01H and ESC % 00H escape

sequences. Word processors like MS WORD can handle this with an appropriate

printer definition file.

- The Olympia has a "special symbol area" which on the symbol wheel contains

six characters, four for constructing square brackets, a dagger, and an

accent. These are not included in OLYMPIAS.FXF.

- Some of the characters on the symbol wheel are designed to be combined with

half-line roll up and roll down movements to make large symbols such as

brackets, integration signs, and the like. These won't work well on the Epson

because characters seperated vertically by one line can't be made to overlap.

Acknowledgement is due to Wheatland Design Laboratory, originators of program

WXMX80. This character set was constructed using the GREEK.CHR file included

with the program WSMX80 on PC SIG 526 as a starting point.  Most of the

characters have been modified to adapt to the FX printer.

Jack Dodds

Geo Acoustics Inc.

Box 772

Aurora Ontario Canada

L4G 4J9